3GPP TSG RAN WG1 #106bis-e	                                         	    R1-2110030

e-Meeting, October 11th – 19th, 2021

Agenda Item:	8.3.3

Source:	Apple Inc.

Title:	Design considerations on Rel-17 intra-UE multiplexing/prioritization

Document for:	Discussion/Decision

1	Introduction	2

2	Discussion on Rel-17 UCI multiplexing flow	4

3	PUCCH resource selection	7

3.1	Review on Rel-15/16 PUCCH resource set selection, PUCCH resource selection, PRB number adjustment, and CSI omission rule	7

3.2	PUCCH resource set selection procedure	10

3.3	PUCCH resource selection procedure	10

3.4	PRB number adjustment procedure	10

3.5	CSI omission procedure	17

4	PUCCH resource set selection in Rel-17	18

5	Rel-17 PRB number adjustment/interlace number adjustment design	18

5.1	PRB number adjustment	18

5.1.1	Multiplexing of different UCI parts on the same modulation symbol	19

5.2	Interlace number adjustment	19

5.3	Summary	20

6	Omission rules in Rel-17	20

6.1	Design for UCI omission	20

6.2	Summary	21

7	Ordering of UCIs in UCI part 1 and UCI part 2	21

7.1	Discussion on CSI Part I and CSI Part II	22

8	Discussion on CSI feedback’s priority	23

9	Discussion on UCI multiplexing over PUCCH with more than 2 UCI bits	23

9.1	Separate encoding HP HARQ-ACK and LP HARQ-ACK	25

10	PUCCH resources for SPS HARQ-ACK	28

11	UCI multiplexing over PUSCH	30

11.1	Mapping of UCIs to UCI Parts over PUSCH	31

12	PUSCH resource determination (α and β) for UCI multiplexing	35

12.1	Rel-15/16 design review	35

12.1.1	BetaOffsets	35

12.1.2	BetaOffsets for CG PUSCH and DG PUSCH	35

12.1.3	Summary on Rel-15/16 design for BetaOffsets	37

12.1.4	Summary on Rel-15/16 design for alpha	37

12.2	Rel-17 design for loading beta offsets	38

13	Power control aspects	41

13.1	Design history in NR	41

13.2	Design considerations on PUCCH power control for Rel-17	42

13.3	Proposed change for PUCCH power control	44

14	Discussion on PHY prioritization of overlapping dynamic grant PUSCH and configured grant PUSCH	46

15	Simultaneous PUCCH and PUSCH transmissions	46

16	Conclusion	48

17	References	50

18	Appendix Specification Text for power control formulas	51

7.2.1	UE behaviour	51

19	Appendix Reflecting r1 and r2 in PUCCH resource selection	56

19.1	Determination of r1 and r2	57

19.2	Summary	57

20	Appendix: Solution to fix inconsistent design in PUCCH resource set selection and UCI omission rule	58

20.1	Inconsistent design in Rel-15/16	58

20.1.1	Issue 1	58

20.1.2	Issue 2	58

20.2	Joint PUCCH resource set selection and PUCCH resource selection	58

20.2.1	Joint PUCCH resource set selection and PUCCH resource selection tackling Issue 1 and Issue 2	58

20.2.2	Joint PUCCH resource set selection and PUCCH resource selection without tackling Issue 1 and Issue 2	59

# Introduction

At RAN1 #104bis, the following agreements were agreed:

Agreements:

For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, when the total number of LP and HP HARQ-ACK bits is more than 2, support separate coding for the two HARQ-ACKs.

- FFS for HP HARQ-ACK or LP HARQ-ACK of 1-2 bit(s).

- (working assumption) Drop CSI (including part 1 and part2, if exist) if CSI would multiplex on a PUCCH which has HP A/N.

  - FFS Strive to let HP A/N reuse the encoder, rate matching equation, and RE mapping rules in Rel-15 for A/N+CSI-1.

  - FFS Strive to let LP A/N reuse the encoder, rate matching equation, and mapping rules in Rel-15 for CSI-2.

Agreement:

For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUSCH in R17, support separate coding for the two HARQ-ACKs.

- It is understood that it is intended that the number of encoding chains for all UCI multiplexing combinations in Rel-17 should not exceed that in Rel-15/16.

At RAN1 #105-e, the following agreements were reached:

Agreement:

For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, when the total number of LP and HP HARQ-ACK bits is more than 2,

- For HP HARQ-ACK or LP HARQ-ACK of 1-2 bit(s), support separate coding. Down-select from the two options:

  - Option 1: Reuse R15 TS 38.212 Clause 5.3.3.1 for 1-bit. Reuse R15 TS 38.212 Clause 5.3.3.2 for 2-bit.

  - Option 2: Reuse R15 TS 38.212 Clause 5.3.3.3, i.e., padding to 3 bits and using RM coding.

- For HP HARQ-ACK or LP HARQ-ACK >2 bit(s), HP HARQ-ACK and LP HARQ-ACK are separately encoded according to R15 TS 38.212 Clause 5.3.3.3 or Clause 5.3.1.

- FFS rate matching equation and RE mapping rules for PF2/3/4. Rel-15 is baseline if available.

Agreement:

For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, when the total number of LP and HP HARQ-ACK bits is 2, treat the two bits as HARQ-ACK bits with High priority.

·           Rel-15 design (for PF0 and PF1) is baseline.

·           Note: QC has strong concern on above scheme. The scheme cannot provide unequal error protection between the HP bit and LP bit hence could suffer from performance degradation for the HP bit. QC accept the scheme for the sake of progress in RAN 1 with the concern on the performance reserved.

In this contribution, continuing the discussion from [10], we provide our views on UCI multiplexing design and related design from HARQ/CSI enhancements.

On UCI multiplexing over PUCCH:

- In Section 2, we discuss the UCI multiplexing flow in Rel-16 and Rel-17.

- In section 3, we review the Re-15/Rel-16 design in PUCCH resource set selection, PUCCH resource selection, PRB number adjustment and CSI omission. In Sections 4/5/6, we give our proposals on PRB number adjustment and UCI omission in Rel-17. Some design issues are identified in Section 8 also, a joint PUCCH resource set/PUCCH resource selection is proposed overt here to address the raised issues.

- The order UCIs in UCI parts are treated in Section 7. And separate encoding design for PF2 is treated in Section 9. UCI multiplexing with a small payload size is treated in Section 10.

- SPS HARQ related issues are discussed in Section 10.

On UCI multiplexing over PUSCH:

- In Section 11, we discuss alternatives in where to map LP-HARQ ACK over PUSCH.

- In Section 12, We discuss how beta/alpha are loaded for PUSCH.

In Section 13, we discuss power control for PUCCH formats 2/3/4 to support UCIs of mixed physical priorities.

In Section 14, we discuss CG/DG PUSCH cancellation, and in Section 15, we discuss simultaneous PUCCH/PUSCH transmission.

# Discussion on Rel-17 UCI multiplexing flow

From the past RAN1 meetings on Rel-17 eIIoT/URLLC, we can see there can be some UE features coming out of the Rel-17 WI. As UCI multiplexing is complicated even in Rel-15, we need to ensure decision taken on each of them won’t in the end lead to big problems.

- In terms of the transmission of UCIs

  - sim Tx of PUCCH/PUSCH

  - PUCCH carrier switching

- In terms of UCI multiplexing

  - inter-priority multiplexing

- In terms of HARQ codebook construction

  - SPS HARQ deferral

  - retransmission of cancelled HARQ

  - Type 1 sub-slot HARQ codebook enhancement

- In terms of the physical channel design of PUCCH:

  - PUCCH sub-slot repetition

  - PUCCH F0/F2 inter-sub-slot repetition

Fundamental to UCI multiplexing in Rel-17 is the decision whether the Rel-16  L1 priority-specific UCI multiplexing design is used as the baseline or a L1 priority-agnostic UCI multiplexing design similar to that in Rel-15 is used as the baseline.

Considering L1 priority based UCI prioritization and inter-L1 priority UCI multiplexing may need to be supported at the same time:

- The current agreements on intra-UE multiplexing cover HARQ/SR multiplexing only, CSI is excluded in the consideration hence UCI-type specific treatment with prioritization or multiplexing is assumed to be necessary

- Under some condition inter-L1 priority multiplexing may be infeasible due to processing time

- If the processing flow for multiplexing is to a large degree shared with that of prioritization, then both specification effort and implementation/test effort can be reduced.

With these considerations, we propose to study the UCI processing flow in Rel-17. Note some items included there (e.g. PUCCH carrier switching, Type 1 codebook enhancement, etc) are for inspection on their effect on the UCI multiplexing processing flow, their inclusion here does not suggest we find them to be essential to the Rel-17 eIIoT/URLLC work.

At RAN1 #106-e, the following was reached:

Working Assumption

For handling overlapping PUCCHs/PUSCHs with different priorities in R17

- Step 1: Resolve overlapping PUCCHs and/or PUSCHs with the same priority

- Step 2: Resolve overlapping PUCCHs and/or PUSCHs with different priorities

Note: Avoid recursive pseudo-code to implement this procedure

Note: It is expected that Rel-15 intra-UE UCI multiplexing timeline will be applicable

Figure 1 Proposed Rel-17 UCI multiplexing flow

We propose to agree the UCI multiplexing flow as shown in Figure 1. More details of the processing flow are included below:

- In Step 1-1: intra-L1 priority PUCCH multiplexing is conducted as in Rel-16

  - For a given L1 priority, the Set Q is collected, and PUCCH resources Z are generated. The PUCCHs at another priority are treated as non-existent for PUCCH multiplexing at the given L1 priority.

- In Step 1-2:

  - For a given L1 priority, intra-L1 priority PUCCH/PUSCH multiplexing is conducted over PUCCH/PUSCH over the given L1 priority.

    - The PUCCHs/PUSCHs at another priority are treated as non-existent.

    - If simultaneous PUCCH/PUSCH is not supported by the UE or inter-band simultaneous PUCCH/PUSCH transmission is not configured by the gNB,

      - The PUSCH selection rule is the same as in RAN1 #97 clarification;

    - Otherwise

    - Only PUSCH(s) of the same L1 priority as PUCCH’s which reside on a CC in the same band of the PUCCH cell’s are candidates for UCI multiplexing over PUSCH.

- In Step 2-1:

  - inter-L1 priority PUCCH multiplexing is performed.

  - Scanning from the earliest HP PUCCH resource Z to the latest HP PUCCH resource Z, for a HP PUCCH resource Z, overlapping LP PUCCH resource(s) Z are identified.

  - To avoid recursive processing, after inter-L1 priority UCI multiplexing, the resulted PUCCH ’s “footprint” does not change with respect to HP PUCCH resource Z, which is shown in Figure 2. In Figure 3, the consequence of not keeping the same foot print for the resulted HP PUCCH is seen: if HP PUCCH (denoted as “HP PUCCH 1A”) overlaps with HP PUSCH, then somehow Step 1-2 needs to be run again, which is clearly undesired.

    - In our view, UCI multiplexing of HP HARQ-ACK, HP SR, LP HARQ-ACK and LP SR over HP PUCCH should be supported.

- In Step 2-2:

  - Inter-L1 priority PUCCH/PUSCH multiplexing

  - For HP PUCCH and LP PUSCH:

    - If simultaneous PUCCH/PUSCH is not supported by the UE or inter-band simultaneous PUCCH/PUSCH transmission is not configured by the gNB,

      - LP PUSCHs on all CCs are candidates for UCI multiplexing over PUSCH, and the PUSCH selection rule is the same as in RAN1 #97 clarification;

    - Otherwise

      - Only LP PUSCH(s) which reside on a CC(s) at the same band of the PUCCH cell’s are candidates for UCI multiplexing over LP PUSCH.

  - For LP PUCCH and HP PUSCH:

    - If simultaneous PUCCH/PUSCH is not supported by the UE or inter-band simultaneous PUCCH/PUSCH transmission is not configured by the gNB,

      - HP PUSCHs on all CCs are candidates for UCI multiplexing over PUSCH, and the PUSCH selection rule is the same as in RAN1 #97 clarification;

    - Otherwise

      - Only HP PUSCH(s) which reside on a CC(s) at the same band of the PUCCH cell’s are candidates for UCI multiplexing over HP PUSCH.

- Finally apply SFI (dynamic & semi-static) to remove colliding PUCCHs/PUSCHs colliding with any DL symbol.

Figure 2 Step 2-1 inter-L1 PUCCH multiplexing by keeping HP PUCCH's footprint

Figure 3 Step 2-1 inter-L1 PUCCH multiplexing by keeping HP PUCCH's footprint

Proposal 2-1: Step 2 consists of two steps:

In step 2-1, inter-L1 PUCCH multiplexing is performed.

HP PUCCH resources Z are arranged according to starting time. The earliest unprocessed HP PUCCH resource Z is scanned first, and any overlapping LP PUCCH resources Z are identified and the resulted HP PUCCH is the same as the PUCCH resource Z.

In Step 2-2: inter-L1 priority PUCCH/PUSCH multiplexing is performed.

  - For HP PUCCH and LP PUSCH:

    - If simultaneous PUCCH/PUSCH is not supported by the UE or inter-band simultaneous PUCCH/PUSCH transmission is not configured by the gNB,

      - LP PUSCHs on all CCs are candidates for UCI multiplexing over PUSCH, and the PUSCH selection rule is the same as in RAN1 #97 clarification;

    - Otherwise

      - Only LP PUSCH(s) which reside on a CC(s) at the same band of the PUCCH cell’s are candidates for UCI multiplexing over LP PUSCH.

  - For LP PUCCH and HP PUSCH:

    - If simultaneous PUCCH/PUSCH is not supported by the UE or inter-band simultaneous PUCCH/PUSCH transmission is not configured by the gNB,

      - HP PUSCHs on all CCs are candidates for UCI multiplexing over PUSCH, and the PUSCH selection rule is the same as in RAN1 #97 clarification;

    - Otherwise

      - Only HP PUSCH(s) which reside on a CC(s) at the same band of the PUCCH cell’s are candidates for UCI multiplexing over HP PUSCH.

# PUCCH resource selection

## Review on Rel-15/16 PUCCH resource set selection, PUCCH resource selection, PRB number adjustment, and CSI omission rule

In both Rel-15 and Rel-16, multiple procedures are involved in PUCCH resource selection and PUCCH resource size determination. In the figure below, the PUCCH resource set selection, PUCCH resource selection, PRB number adjustment/interlace number adjustment, and CSI omission are shown.

Figure 3 Rel-15/16 NR design

Example of RRC configuration is shown in Tables to help discussion below.

Table 1 Example of PUCCH resource set configuration

|  | PUCCH resource set 0 | PUCCH resource set 1 | PUCCH resource set 2 | PUCCH resource set 3 |
| --- | --- | --- | --- | --- |
| PUCCH resource set index | pucch-ResourceSetId = 0 | pucch-ResourceSetId = 1 | pucch-ResourceSetId = 2 | pucch-ResourceSetId = 3 |
| PUCCH resources in a PUCCH resource set | resourceList (0,1,2,…,7) | resourceList (32,33,..,39) | resourceList (40,41,..,47) | resourceList (48,49,…,55) |
| maxPayloadSize |  | N2=maxPayloadSize1 | N3= maxPayloadSize2 |  |
| Applicable range | OUCI≤2 | 2<OUCI≤N2 | N2<OUCI≤N3 | N3<OUCI≤1706 |

Table 2 Example of PUCCH resources configurations indicated by PRI=0.

|  | PUCCH resource 0 | PUCCH resource 32 | PUCCH resource 40 | PUCCH resource 48 |
| --- | --- | --- | --- | --- |
| pucch-ResourceId | 0 | 32 | 40 | 48 |
| startingPRB | 10 | 0 | 2 | 6 |
| intraSlotFrequencyHopping |  | enabled | enabled | enabled |
| secondHopPRB |  | 80 | 50 | 76 |
| PUCCH format | PUCCH-format0: - initialCyclicShift: 6; - nrofSymbols: 2 - startingSymbolIndex: 11 | PUCCH-format4: - nrofSymbols: 4 - occ-Length: n4 - occ-Index: n3 - startingSymbolIndex: 0 | PUCCH-format2: - nrofPRBs: = 8 - nrofSymbols: 2 - startingSymbolIndex: 12 | PUCCH-format3: - nrofPRBs: = 10 - nrofSymbols: 4 - startingSymbolIndex: 9 |

Table 3 Example of PUCCH resource configurations indicated by PRI = 1

|  | PUCCH resource 1 | PUCCH resource 33 | PUCCH resource 41 | PUCCH resource 49 |
| --- | --- | --- | --- | --- |
| pucch-ResourceId | 1 | 33 | 41 | 49 |
| startingPRB | 11 | 1 | 10 | 16 |
| intraSlotFrequencyHopping |  | enabled | enabled | enabled |
| secondHopPRB |  | 81 | 58 | 86 |
| PUCCH format | PUCCH-format0: - initialCyclicShift: 6; - nrofSymbols: 2 - startingSymbolIndex: 11 | PUCCH-format4: - nrofSymbols: 4 - occ-Length: n4 - occ-Index: n3 - startingSymbolIndex: 0 | PUCCH-format3: - nrofPRBs: = 2 - nrofSymbols: 4 - startingSymbolIndex: 7 | PUCCH-format3: - nrofPRBs: = 8 - nrofSymbols: 5 - startingSymbolIndex: 8 |

Table 4 Example of maxCodeRate configurations at PF2/PF3/PF4 for HP PUCCH-Config

|  |  | PUCCH-Config (for HP PUCCH) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | format1 | format2 | format3 | format4 |
| With PUCCH-FormatConfig, Each parameter can be provided for each PUCCH format separately | interslotFrequencyHopping |  |  |  |  |
|  | additionalDMRS |  |  |  |  |
|  | maxCodeRate | N/A | zeroDot08 | zeroDot25 | zeroDot15 |
|  | nrofSlots |  |  |  |  |
|  | pi2BPSK |  |  |  |  |
|  | simultaneousHARQ-ACK-CSI |  |  |  |  |

In the example, PUCCH resource 40 and PUCCH resource 41 are under PUCCH resource set 2, note they can be configured with different maxCodeRate (0.08 and 0.25 respectively in the example).

Comparing Table 4 and Table 5, it is seen also that maxCodeRate for a given PUCCH format can be configured separately for each PUCCH-Config.

Table 5 Example of maxCodeRate at PF2/PF3/PF4 for LP PUCCH-Config

|  |  | PUCCH-Config (for LP PUCCH) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | format1 | format2 | format3 | format4 |
| With PUCCH-FormatConfig, Each parameter can be provided for each PUCCH format separately | interslotFrequencyHopping |  |  |  |  |
|  | additionalDMRS |  |  |  |  |
|  | maxCodeRate | N/A | zeroDot15 | zeroDot35 | zeroDot25 |
|  | nrofSlots |  |  |  |  |
|  | pi2BPSK |  |  |  |  |
|  | simultaneousHARQ-ACK-CSI |  | true | true | true |

## PUCCH resource set selection procedure

- Input: OUCI, maxPayloadSize for the second PUCCH resource set, maxPayloadSize for the third PUCCH resource set

Note: at a given physical layer priority, with PUCCH-Config (i.e. there is one PUCCH-Config for LP PUCCHs, and another PUCCH-Config for HP PUCCHs), A UE can be configured with up to 4 PUCCH resource sets for for UCI feedback including HARQ-ACK in response to a grant based PDSCH or a SPS release.

There are up to 32 PUCCH resources under the first PUCCH resource set, and up to 8 PUCCH resources under each of the configured PUCCH resource set. The second and the third PUCCH resource sets can be configured with a “maxPayloadSize".

Necessary simplification is made to make the NR design easier to understand, the full details can be found in Clause 9.2.1 of TS 38.213. We denote PUCCH format x as PFx, e.g. PF1 for PUCCH Format 1, PF3 for PUCCH format 3, etc.

- Process (refer to Clause 9.2.1 of T 38.213 for details):

For PUCCH resource set selection, OUCI bits (not including CRC bits) is compared with 2, maxPayloadSize for the 2nd PUCCH resource set, maxPayloadSize for the 3rd PUCCH resource set to select a PUCCH resource set. Essentially, 2, maxPayloadSize for the 2nd PUCCH resource set, maxPayloadSize for the 3rd PUCCH resource set and 1706 define 4 ranges of UCI bits: and each range corresponds to a PUCCH resource set.

- Output: selected PUCCH resource set

## PUCCH resource selection procedure

- Input: 1) Selected PUCCH resource set, 2) PRI included a latest DL DCI

- Process: the PRI included the DL DCI is used to select a PUCCH resource within the selected PUCCH resource set.

- Output: selected PUCCH resource

## PRB number adjustment procedure

Since in Rel-15, only PUCCH format 2 and PUCCH format 3 can have more than one PRB, to use PUCCH resource in a parsimonious a way, the number of PRBs in PUCCH format 2 and PUCCH format 3 can be adjusted according to the payload size, note the CRC bits are included in the consideration of PRB number adjustment, and further the allowable PRB number for PUCCH format 3 is limited to a number with prime factor(s) from 2, 3 and 5 as DFT-S-OFDM is used for PUCCH format 3.

- Input: the configured PRB number of the selected PUCCH resource (if it is at PF2 or PF3), OUCI, OCRC, maxCodeRate according to the PUCCH format of the selected PUCCH resource

Note 1: all PUCCH resources of the same PUCCH format (PF2/PF3/PF4) under the same PUCCH-Config have the same maxCodeRate in Rel-15/16.

Note 2: OUCI, OCRC consist of two parts if two part CSI is reported. More details are provided in the next.

- Process: Choose the smallest PRB number which allows transmission of OUCI+OCRC with no greater coding rate than the PUCCH specific maxCodeRate according to the PUCCH format of the selected PUCCH resource. For PF3, the number of PRBs is restricted to have prime factor(s) out of 2, 3 and 5.

- Output: NRB,minPUCCH (the smallest PRB number meeting the transmission requirement over the selected PUCCH resource

To guide the Rel-17 design, we examine cases in Figure 6 one by one:

- Case 1:

  - The PRB number adjustment for HARQ-ACK only over PF2/PF3 procedure (Clause 9.2.3 of TS 38.213 v16.4.0) is captured below.

  - For interlaced transmission of PF2/PF3 as introduced in NR-U, interlace number adjustment is performed in a similar way to PRB number adjustment.

Interlace number adjustment for PF2/PF3 with interlaced transmission as introduced in NR-U.

- Case 2:

  - The PRB number adjustment procedure for HARQ-ACK+SR (Clause 9.2.5.1 of TS 38.213)  over PF2/PF3 is given below.

  - For interlaced transmission of PF2/PF3 as introduced in NR-U, interlace number adjustment is performed in a similar way to PRB number adjustment.

- Case 3:

  - For {HARQ-ACK+SR+WB/SB CSI over PF2, or HARQ-ACK+SR+WB CSI over PF3/PF4}, Clause 9.2.5.2 of TS 38.213, the description is more complex than Case 1 and Case 2.

  - First, parameters/notations are defined in Clause 9.2.5.2, which are common for Case 3 and Case 4. Note

    - OCSI and OCRC consist of two parts;

    - r  is the maxCodeRate associated with the PUCCH format of the selected PUCCH resource.

PRB number adjustment/interlace number adjustment (highlighted in yellow) and CSI omission rules (highlighted in cyan) are described in Clause 9.2.5.2:

- Case 4:

  - The text for HARQ-ACK+SR+SB CSI over PF3/PF4, Clause 9.2.5.2 of TS 38.213, is captured below.

  - Note the parameters are defined in the text excerpt under Case 3:

    - OCSI and OCRC consist of two parts;

    - r  is the maxCodeRate associated with the PUCCH format of the selected PUCCH resource.

In Rel-16, interlaced transmission of PUCCH is supported from the NR-U work item. For interlaced transmission of PF3, the interlace number adjustment and CSI omission are captured below:

From the review, we have

Observation 3-1: in Rel-16, for PRB number adjustment/interlace number adjustment, the configured PRB number of the  selected PUCCH resource (if it is at PF2 or PF3), OUCI, OCRC, maxCodeRate according to the PUCCH format of the selected PUCCH resource,  OUCI, OCRC, maxCodeRate according to the PUCCH format of the selected PUCCH resource are used as inputs, and the same coding rate is used for two CSI parts.

## CSI omission procedure

- Condition for applying CSI omission:

* Omission Case 1: If CSI consists of WB CSI or Part 1 only, CSI omission is performed for Part I for a PUCCH resource at PF2/PF3/PF4. The priority value formula in Clause 5.2.5 in TS 38.212 is used.

* Omission Case 2: If CSI consists of two Parts, then CSI omission is performed for Part I and Part II for a PUCCH resource for PF3/PF4. The priority value formula in Clause 5.2.5 in TS 38.212 is used, and Table 5.2.3-1 Priority reporting levels for Part 2 CSI, Clause 5.2.3 of TS 38.212 is used.

- Input: selected PUCCH resource (if it is at PF2 or PF3, PF4), OUCI, OCRC, maxCodeRate according to the PUCCH format of the selected PUCCH resource

- Process:

* For Omission case 1: discard Part 1 of CSI report(s) until the the remaining payload and its corresponding CRC bits can be transmitted with no greater a coding rate than the PUCCH format specific maxCodeRate according to the PUCCH format of the selected PUCCH resource.

* For Omission case 2: discard Part 2 of CSI report(s), and if necessary, Part 1 of CSI report(s) until the the remaining payload and its corresponding CRC bits can be transmitted with no greater a coding rate than the PUCCH format specific maxCodeRate according to the PUCCH format of the selected PUCCH resource.

- Output:

* For Omission case 1: NCSIreported (the number of CSI reports surviving CSI omission).

* For Omission case 2: NCSI-part2reported (the number of CSI reports with un-omitted CSI part 2) or NCSI-part1reported (the number of CSI reports with un-omitted CSI part 1 when part 2 of all CSI reports are omitted.

For omission of Part II of CSI reports, the following two conditions are used in TS 38.213:

n=1NCSI-part2reported‍OCSI-part2,n+OCRC,CSI-part2,N≤MRBPUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH-⌈OACK+OSR+n=1NCSItotal‍OCSI-part1,n+OCRC,CSI-part1/Qm⋅r⌉⋅Qm⋅rn=1NCSI-part2reported+1‍OCSI-part2,n+OCRC,CSI-part2,N+1>MRBPUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH-⌈OACK+OSR+n=1NCSItotal‍OCSI-part1,n+OCRC,CSI-part1/Qm⋅r⌉⋅Qm⋅r

Those two conditions can be reformulated as

n=1NCSI-part2reported‍OCSI-part2,n+OCRC,CSI-part2,N/(Qm⋅r)+⌈OACK+OSR+n=1NCSItotal‍OCSI-part1,n+OCRC,CSI-part1/Qm⋅r⌉≤MRBPUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCHn=1NCSI-part2reported+1‍OCSI-part2,n+OCRC,CSI-part2,N+1/(Qm⋅r)+⌈OACK+OSR+n=1NCSItotal‍OCSI-part1,n+OCRC,CSI-part1/Qm⋅r⌉>MRBPUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH

From the reformulated conditions, it can be seen roughly the same coding rate is required for part I and part II.

Observation 3-2:for CSI omission, selected PUCCH resource (if it is at PF2 or PF3, PF4), OUCI, OCRC, maxCodeRate according to the PUCCH format of the selected PUCCH resource are used as inputs, the same coding rate is used for two CSI parts.

# PUCCH resource set selection in Rel-17

At RAN1 #106-e, PUCCH resource set selection was discussed. The alternative routes to decide PUCCH resource selection can be found in Section . In the end, with the understanding different code rates can be still reflected in PRB number adjustment and UCI omission rules and for the sake of progress, we did not insist on the proposal to reflect r1 and r2 in PUCCH resource selection.

# Rel-17 PRB number adjustment/interlace number adjustment design

## PRB number adjustment

In Rel-15, PRBs for a PUCCH are contiguous in the frequency domain. In Rel-16, under NR-U (NR Unlicensed Spectrum Access), interlaced transmission of PUCCH where PRBs for a PUCCH may not be contiguous. When two interlaces are configured for PUCCH for unlicensed spectrum access, the PRBs under one PUCCH may actually be contiguous if both interlaces are utilized. However, the resource adjustment is at interlace level, which is different from the PRB number adjustment as specified in Rel-15.

In Clause 9.2.5.2 of TS 38.213 (v16.5.0), two conditions (trigger condition & stop condition) are used for PRB number adjustment:

If (OACK+OSR+OCSI+OCRC)≤MRBPUCCH⋅Nsc,ctrlRBNsymb-UCIPUCCH⋅Qm⋅r, the UE transmits the HARQ-ACk, SR and the NCSItotal CSI report bits by selecting the minium number MRB,minPUCCH of PRBs from the MRBPUCCH PRBs satifying (OACK+OSR+OCSI+OCRC)≤MRB,minPUCCH⋅Nsc,ctrlRBNsymb-UCIPUCCH⋅Qm⋅r as described in Clasuses 9.2.3 nand 9.2.5.1

According to the notations established in Clause 9.2.5.2 of TS 38.213,

• OCSI=n=1NCSItotal‍OCSI-part1,n+OCSI-part2,n, where OCSI-part1,n is a number of Part 1 CSI reports bits for CSI report with priority value n, OCSI-part2,n is a number of Part 2 CSI report bits, if any, for CSI report with priority value n [6, TS 38.214], and NCSItotal is a number of CSI reports that include overlapping CSI reports.

• OCRC=OCRC,CSI-part1+OCRC,CSI-part2, where OCRC,CSI-part1 is a number of CRC bits, if any, for encoding HARQ-ACK, SR and Part 1 CSI report bits and OCRC,CSI-part2 is a number of CRC bits, if any, for encoding Part 2 CSI report bits.

Then the condition to trigger PRB number adjustment in Rel-16 can be reformulated as

(OACK+OSR+n=1NCSItotal‍OCSI-part1,n+OCRC,CSI-part1)/(Qm⋅r)+(n=1NCSItotal‍OCSI-part2,n+OCRC,CSI-part2)/(Qm⋅r)≤MRBPUCCH⋅Nsc,ctrlRBNsymb-UCIPUCCH, essentially the sum of the number of modulation symbols required for UCI Part I and the number of modulation symbols frequired for UCI Part II does not exceed exceed the number of modulation symbols with the maximum available number PRBs on the selected PUCCH.

We propose to consider different coding rates for UCI Part I and UCI Part II in Rel-17 for PRB number adjustment.

Then for Rel-17 design, the condition to trigger PRB number adjustment can be given by

n=1NUCI-part2total‍OUCI-part2,n+OCRC,UCI-part2/(Qm⋅r2)+n=1NUCI-part1total‍OUCI-part1,n+OCRC,UCI-part1/Qm⋅r1≤MRBPUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH

And similarly, the stop condition for PRB number adjustment can be modified in Rel-17 as follows:

n=1NUCI-part2total‍OUCI-part2,n+OCRC,UCI-part2/(Qm⋅r2)+n=1NUCI-part1total‍OUCI-part1,n+OCRC,UCI-part1/Qm⋅r1≤MRB,minPUCCH⋅Nsc,ctrlRB,min⋅Nsymb-UCIPUCCH

### Multiplexing of different UCI parts on the same modulation symbol

In Rel-15/16, CSI parts are not multiplexed on the same modulation symbol.

In Rel-17, even though logically it is also possible to allow different UCI parts to be carried on the same modulation symbol, it is good to keep the same design principle such that different UCI parts are not multiplexed on the same modulation symbol.

To be more consistent with the UCI omission rules to be discussed in the next, the condition for PRB number adjustment, can be tightened a little bit by taking the ceiled value of the second item:

n=1NUCI-part2total‍OUCI-part2,n+OCRC,UCI-part2/(r2)+⌈n=1NUCI-part1total‍OUCI-part1,n+OCRC,UCI-part1/r1⌉≤MRBPUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH⋅Qm

And the stop condition for PRB number adjustment can be modified in Rel-17 as follows:

n=1NUCI-part2total‍OUCI-part2,n+OCRC,UCI-part2/(r2)+⌈n=1NUCI-part1total‍OUCI-part1,n+OCRC,UCI-part1/r1⌉≤MRB,minPUCCH⋅Nsc,ctrlRB,min⋅Nsymb-UCIPUCCH⋅Qm

## Interlace number adjustment

For interlace number adjustment, instead of performing PRB number adjustment for potentially many PRB number candidates, only two PRB number candidates are examined: one corresponds to the number of PRBs in the first interlace, another corresponds to the number PRBs in the first and second interlace. The same consideration provided in the previous section applies.

Further details can be found below:

If a UE is provided a first interlace of MInterlace,0PUCCH PRBs by interlace0 in InterlaceAllocation, the UE has HARQ-ACK, SR and wideband or sub-band CSI reports to transmit, and the UE determines a PUCCH resource with PUCCH format 2, or the UE has HARQ-ACK, SR and wideband CSI reports to transmit and the UE determines a PUCCH resource with PUCCH format 3, or the UE has HARQ-ACK, SR and sub-band CSI reports to transmit, and the UE determines a PUCCH resource with PUCCH format 3, where

- the UE determines the PUCCH resource using the PUCCH resource indicator field in a last of a number of DCI formats with a value of a PDSCH-to-HARQ_feedback timing indicator field indicating a same slot for the PUCCH transmission, from a PUCCH resource set provided to the UE for HARQ-ACK transmission, and after the UE determines the PUCCH resource set

- if

n=1NUCI-part2totalOUCI-part2,n+OCRC,UCI-part2/Qm⋅r2+⌈n=1NUCI-part1totalOUCI-part1,n+OCRC,UCI-part1/Qm⋅r1⌉≤MInterlace,0PUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH,

the UE transmits the HARQ-ACK, SR, and CSI reports bits in a PUCCH over the first interlace

- else, if the UE is provided a second interlace of MInterlace,1PUCCH PRBs by interlace1 and if

n=1NUCI-part2totalOUCI-part2,n+OCRC,UCI-part2/Qm⋅r2+⌈n=1NUCI-part1totalOUCI-part1,n+OCRC,UCI-part1/Qm⋅r1⌉≤MInterlace,0PUCCH+MInterlace,1PUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH,

the UE transmits the HARQ-ACK, SR, and CSI reports bits in a PUCCH over both the first and second interlaces

- else, the procedure is same as the corresponding one when the UE is provided PUCCH-ResourceSet by replacing MRBPUCCH with MInterlace,0PUCCH, or, if the UE is provided interlace1, by MInterlace,0PUCCH+MInterlace,1PUCCH.

A joint PUCCH resource set selection and PUCCH resource selection can be also conducted.

## Summary

In this section, we treat PRB number adjustment in Rel-17. We have

Proposal 5-1: considering coding rates of UCI parts in the PRB number adjustment/interlace number adjustment.

Proposal 5-2: The condition to trigger PRB number adjustment is given by

n=1NUCI-part2total‍OUCI-part2,n+OCRC,UCI-part2/(Qm⋅r2)+n=1NUCI-part1total‍OUCI-part1,n+OCRC,UCI-part1/Qm⋅r1≤MRBPUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH

And  the stop condition for PRB number adjustment  is as follows:

n=1NUCI-part2total‍OUCI-part2,n+OCRC,UCI-part2/(Qm⋅r2)+n=1NUCI-part1total‍OUCI-part1,n+OCRC,UCI-part1/Qm⋅r1≤MRB,minPUCCH⋅Nsc,ctrlRB,min⋅Nsymb-UCIPUCCH

# Omission rules in Rel-17

## Design for UCI omission

Assume different coding rate targets for UCI part I and UCI part II, instead of a single coding rate target r, we have r1 for the maximum coding rate for UCI part I, r2 for the maximum coding rate for UCI part II, the following conditions are used for UCI omission:

n=1NUCI-part2reported‍OUCI-part2,n+OCRC,UCI-part2,N/(Qm⋅r2)+⌈n=1NUCI-part1total‍OUCI-part1,n+OCRC,UCI-part1/Qm⋅r1⌉≤MRBPUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCHn=1NUCI-part2reported+1‍OUCI-part2,n+OCRC,UCI-part2,N+1/(Qm⋅r2)+⌈n=1NUCI-part1total‍OUCI-part1,n+OCRC,UCI-part1/Qm⋅r1⌉>MRBPUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH

If all UCI part II is dropped, then the the following is examined to determine remaining UCIs in UCI part I:

n=1NUCI-part1reported‍OUCI-part1,n+OCRC,UCI-part1,,N/(Qm⋅r1)≤MRBPUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCHn=1NUCI-part1reported+1‍OUCI-part1,,n+OCRC,UCI-part1,N+1/(Qm⋅r1)>MRBPUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH

## Summary

We have

Proposal 6-1: Consider different coding rates for UCI parts in UCI omission rule.

Proposal 6-2: With two UCI parts, the omission rule is as follows:

the following conditions are used for UCI omission:

n=1NUCI-part2reported‍OUCI-part2,n+OCRC,UCI-part2,N/(Qm⋅r2)+⌈n=1NUCI-part1total‍OUCI-part1,n+OCRC,UCI-part1/Qm⋅r1⌉≤MRBPUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCHn=1NUCI-part2reported+1‍OUCI-part2,n+OCRC,UCI-part2,N+1/(Qm⋅r2)+⌈n=1NUCI-part1total‍OUCI-part1,n+OCRC,UCI-part1/Qm⋅r1⌉>MRBPUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH

If all UCI part II is dropped, then the the following is examined to determine remaining UCIs in UCI part I:

n=1NUCI-part1reported‍OUCI-part1,n+OCRC,UCI-part1,,N/(Qm⋅r1)≤MRBPUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCHn=1NUCI-part1reported+1‍OUCI-part1,,n+OCRC,UCI-part1,N+1/(Qm⋅r1)>MRBPUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH.

# Ordering of UCIs in UCI part 1 and UCI part 2

Here we consider CSI reporting is for either HP or LP. The order of UCI in a UCI part defines the order by which UCIs are omitted.

For the case where HP CSI is present:

On UCI part 1:

HP HARQ-ACK (including potentially more than one HARQ codebook), HP SR, HP CSI-Part I

On UCI part 2:

HP CSI-part II, LP-HARQ-ACK, (LP-SR)

Hence on UCI Part 2, (LP-SR) if supported by specification and present, is omitted first, LP-HARQ-ACK (including potentially more than one HARQ codebook) is omitted second (if more than one HARQ codebook is present, then the last placed HARQ codebook is omitted/compacted first for example), then HP CSI-part II can be omitted in a similar way as CSI part II in Rel-16.

For the case LP CSI is present:

On UCI part 1:

HP HARQ-ACK (including potentially more than one HARQ codebook), HP SR, LP CSI-Part I

On UCI part 2:

LP-HARQ-ACK, (LP-SR), LP CSI-part II

## Discussion on CSI Part I and CSI Part II

In Rel-15, the introduction of two part CSI feedback was motivated by the payload size variation for the reported rank: CSI Part I provides sufficient information for gNB to determine the size of CSI part II, so blind detection with respect to different ranks is avoided. Note in Rel-15, single part CSI reports are included in CSI Part I even though for a single part CSI report there is no CSI part I in the CSI report. Considering in Rel-17 the coding rate for UCI part I can be different from that for UCI part II, hence there may be a motivation to move all non-essential UCIs to UCI part II. Specifically, if a CSI report consists of a single part, then it can be carried to UCI Part II. However, if the single part CSI report is at HP, then it may be still desirable to carry the HP CSI report in UCI Part I.

Assume there are cases where the UCI payload may exceed the capacity of a PUCCH, if the following priority rule is used (note not all UCI types may be present in a particular scenario, nor all the combinations need to be supported by specification):

HP HARQ-ACK > HP SR > (HP CSI) > LP HARQ-ACK > (LP SR) > (LP CSI)

With the above prioritization rule, then CSI omission rule as captured in TS 38.214 may need to run twice: it is first run for LP CSI, and after omitting or dropping all LP CSI, LP SR and LP HP-HARQ-ACK, part of HP CSI needs to be omitted secondly. In Figure 5-14, it shows that the CSI priority rule is applied separately to HP CSI and LP CSI, then (parts of) CSI reports within H-CSI and CSI reports within L-CSI are separately dropped, so the CSI priority rules are run twice, one for HP CSI another for LP CSI.

Alternatively, LP CSI is dropped when HP CSI is present to avoid running CSI omission rules twice: there are two choices: with choice 1, LP CSI is dropped on UCI Part II only, with choice 2, LP CSI is dropped from UCI part I and UCI II all together. Then for UCI Part II or for both UCI Parts I and II,

when HP CSI is present, only the following UCIs are carried in UCI part I and part II:

HP HARQ-ACK > HP SR > (HP CSI) > LP HARQ-ACK > (LP SR)

When HP CSI is absent, only the following UCIs are carried UCI part I and part II

HP HARQ-ACK > HP SR > LP HARQ-ACK > (LP SR) > LP CSI

Figure 8-1 separate CSI treatment to physical layer priority

Figure 8-2 separate CSI treatment

We have:

Proposal 7-1

when HP CSI is present, only the following UCIs are carried in UCI part I and part II:

HP HARQ-ACK > HP SR > (HP CSI) > LP HARQ-ACK > (LP SR)

When HP CSI is absent, only the following UCIs are carried UCI part I and part II

HP HARQ-ACK > HP SR > LP HARQ-ACK > (LP SR)

# Discussion on CSI feedback’s priority

In Rel-16, periodic and semi-persistent CSI feedback are at low physical layer priority. As for aperiodic CSI feedback, the CSI feedback takes the same physical layer priority as the PUSCH’s, i.e. determined by the “physical layer priority” indication in the DCI.

In Rel-17, 4 bits sub-band CQI was discussed and agreed at RAN1 #106-e. In principle, the 4 bit subband CQI can be used for a periodic CSI report, SP CSI report or an aperiodic CSI report. One potential discussion point is whether periodic CSI report or an SP CSI report with 4 bit subband CQI should be treated as high priority or not – it is clear for the proponent companies the main use case for 4 bit subband CQI is URLLC. Hence we have

Proposal 8: clarify whether periodic CSI report or an SP CSI report with 4 bit subband CQI should be treated as high priority or not.

# Discussion on UCI multiplexing over PUCCH with more than 2 UCI bits

In NR design, UCI multiplexing over PUCCH considers reliability of OFDM symbols in PUCCH, OFDM symbols are grouped in up to 3 groups, then depending on whether the size of UCI part I and UCI part II, the resources are determined.

Figure 9-1 UCI mapping for UCI part I in Rel-15

Given the complex nature of the design, it is not desirable to reopen the design on all fronts, and then Rel-15 design can be should retained when possible.

Figure 9-2 Rel-15 design

In Rel-15:

- In no CSI report is not of two parts, UCI multiplexing for HARQ-ACK, SR and CSI are jointly encoded.

- If at least one CSI report is of two parts, then CSI part 2 is separated encoded, and the rest are jointly encoded.

We see the key design principle to follow for Rel-17 URLLC design is to keep the design as close as possible to the Rel-15 design.

Figure 9-3 UCI mapping over PUCCH for two-part CSI in Rel-15/16

The mapping rules for CSI Part I and CSI part II over PUCCH are specified in TS 38.212 in Rel-15/16. To facilitate the discussion in the following, we denote the resources taken by CSI Part I and CSI part II as UCI Part I and UCI part II respectively. Then by creating mapping rules as proposed subsequently, it is intended to a large degree the mapping rules for CSI Part I, CSI part II as captured in Clause 6.3.1 of TS 38.212 can be reused for UCI part I and UCI Part II.

Note in Clause 6.3.1.1.3 of TS 38.212, two sequences are defined for the case with two part CSI for the case shown in the red box in Figure 2:

“If at least one of the CSI reports for transmission on a PUCCH is of two parts, two UCI bit sequences are generated,  and , according to the following, where  and ”.

With proposed solutions below,  and  are populated differently.

Similarly, one UCI bit sequence is generated, for cases shown in the blue box in Figure 2, then the discussion is above how to populate the UCI bit sequence.

In NR, 5 PUCCH formats are specified, the support of joint encoding and separate encoding of UCIs is shown below.

Table 9-1 Separate encoding and joint encoding with PUCCH formats in Rel-16

|  | PUCCH Format 2 | PUCCH Format 3 | PUCCH Format 4 |
| --- | --- | --- | --- |
| Joint encoding of UCIs | X | X | X |
| Separate encoding of UCIs |  | X | X |

With PUCCH Format 0, joint encoding of HARQ-ACK and SR is supported.

With PUCCH Format 1, channel selection is used in Rel-15/16 to support the signaling of SR and HARQ-ACK, yet on the PUCCH itself, only HARQ-ACK is carried. Partly for that reason, there has been diverse view to support HP/LP UCIs over PUCCH Format 1.

Over PUCCH Format 2, two part CSI is not supported in Rel-15, consequently in Rel-15/16, only joint encoding of UCIs is supported over PUCCH format 2.

## Separate encoding HP HARQ-ACK and LP HARQ-ACK

With separate encoding, the following mapping can be considered.

Figure 9-4 Separate encoding for HP HARQ-ACK and LP HARQ-ACK

As noted previously, in Rel-15 UCI part I and UCI part II roughly have the same coding rate. One argument for separate encoding is that different coding rates can be applied to HP HARQ-ACK and LP HARQ-ACK.

Note in a particular case or the final design, not all the UCIs are present or can be present. CSI feedback can be assumed at LP. If Rel-17 inter-L1 priority UCI multiplexing is limited to HARQ-ACK and SR, then CSI part I and CSI part II are both absent in the UCI multiplexing as shown in the figure below.

Figure 9-5 Separate encoding for HARQ-ACK/SR only

1

EUCI-part1=min(Etot,⌈(OH-ACK+OH-SR+L)/r1/Qm⌉⋅Qm)

EUCI-part2=Etot-min(Etot,⌈(OH-ACK+OH-SR+L)/r1/Qm⌉⋅Qm)

EUCI-part1=min(Etot,⌈(OH-ACK+OH-SR+OH-CSI+L)/r1/Qm⌉⋅Qm)

EUCI-part2=Etot-min(Etot,⌈(OH-ACK+OH-SR+OH-CSI+L)/r1/Qm⌉⋅Qm)

EUCI-part1=min(Etot,⌈(OH-ACK+OH-SR+OCSI-part1+L)/r1/Qm⌉⋅Qm)

EUCI-part2=Etot-min(Etot,⌈(OH-ACK+OH-SR+OCSI-part1+L)/r1/Qm⌉⋅Qm)

1

EUCI-part1=min(Etot,⌈(OH-ACK+OH-SR+L)/r1/Qm⌉⋅Qm)

EUCI-part2=Etot-min(Etot,⌈(OH-ACK+OH-SR+L)/r1/Qm⌉⋅Qm)

EUCI-part1=min(Etot,⌈(OH-ACK+OH-SR+OH-CSI+L)/r1/Qm⌉⋅Qm)

EUCI-part2=Etot-min(Etot,⌈(OH-ACK+OH-SR+OH-CSI+L)/r1/Qm⌉⋅Qm)

EUCI-part1=min(Etot,⌈(OH-ACK+OH-SR+OCSI-part1+L)/r1/Qm⌉⋅Qm)

EUCI-part2=Etot-min(Etot,⌈(OH-ACK+OH-SR+OCSI-part1+L)/r1/Qm⌉⋅Qm)

1

EUCI-part1=min(Etot,⌈(OH-ACK+OH-SR+L)/r1/Qm⌉⋅Qm)

EUCI-part2=Etot-min(Etot,⌈(OH-ACK+OH-SR+L)/r1/Qm⌉⋅Qm)

EUCI-part1=min(Etot,⌈(OH-ACK+OH-SR+OH-CSI+L)/r1/Qm⌉⋅Qm)

EUCI-part2=Etot-min(Etot,⌈(OH-ACK+OH-SR+OH-CSI+L)/r1/Qm⌉⋅Qm)

EUCI-part1=min(Etot,⌈(OH-ACK+OH-SR+OCSI-part1+L)/r1/Qm⌉⋅Qm)

EUCI-part2=Etot-min(Etot,⌈(OH-ACK+OH-SR+OCSI-part1+L)/r1/Qm⌉⋅Qm)

1

EUCI-part1=min(Etot,⌈(OH-ACK+OH-SR+L)/r1/Qm⌉⋅Qm)

EUCI-part2=Etot-min(Etot,⌈(OH-ACK+OH-SR+L)/r1/Qm⌉⋅Qm)

EUCI-part1=min(Etot,⌈(OH-ACK+OH-SR+OH-CSI+L)/r1/Qm⌉⋅Qm)

EUCI-part2=Etot-min(Etot,⌈(OH-ACK+OH-SR+OH-CSI+L)/r1/Qm⌉⋅Qm)

EUCI-part1=min(Etot,⌈(OH-ACK+OH-SR+OCSI-part1+L)/r1/Qm⌉⋅Qm)

EUCI-part2=Etot-min(Etot,⌈(OH-ACK+OH-SR+OCSI-part1+L)/r1/Qm⌉⋅Qm)

Figure 9-6(a) UCI mapping for separate encoding of HP/LP HARQ-ACK (1)

Figure 9-6(b) UCI mapping for separate encoding of HP/LP HARQ-ACK (2)

Proposal 9-1: leverage the Rel-15 design, LP HARQ-ACK is mapped to UCI Part II in separate encoding, adopt the UCI mapping in Figures 9-6a/9-6b.

At RAN1 #104bis, the following agreements were agreed:

Agreements:

For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, when the total number of LP and HP HARQ-ACK bits is more than 2, support separate coding for the two HARQ-ACKs.

- FFS for HP HARQ-ACK or LP HARQ-ACK of 1-2 bit(s).

- (working assumption) Drop CSI (including part 1 and part2, if exist) if CSI would multiplex on a PUCCH which has HP A/N.

  - FFS Strive to let HP A/N reuse the encoder, rate matching equation, and RE mapping rules in Rel-15 for A/N+CSI-1.

  - FFS Strive to let LP A/N reuse the encoder, rate matching equation, and mapping rules in Rel-15 for CSI-2.

As in Rel-15/16, PUCCH Format 2 does not support two part CSI, only joint encoding of UCIs is supported.  We have the following proposal regarding the support of HP HARQ-ACK/LP HARQ-ACK multiplexing over PUCCH format 2:

Generating two encoded sequences for HP-ACK (with r1) and LP-ACK (with r2) separately and then concatenating those two encoded sequences into one encoded sequence , UCI mapping is not changed for PUCCH Format 2.

We have

Proposal 9-2:

Consider for PUCCH format 2 to support multiplexing of HP UCI(s) and LP UCI(s):

generating two encoded sequences for HP-ACK (with r1) and LP-ACK (with r2) separately and then concatenating those two encoded sequences into one encoded sequence, UCI mapping is not changed for PUCCH Format 2.

# PUCCH resources for SPS HARQ-ACK

Figure 10-1 Feedback resource determination for SPS HARQ-ACK in Rel-15 and Rel-16

- In Figure 10-1, SPS HARQ-ACK only multiplexing over PUCCH for CSI is shown, the flow chart is generated according to TS 38.213.

- The blue box shows there is no PRB # adjustment for PF2/PF3 when a single CSI report’s PUCCH is selected for SPS HARQ multiplexing.

- The yellow box shows there is no PRB # adjustment for PF2/PF3, but CSI omission rules are used.

Figure 10-2 Proposed design for PUCCH resource determination SPS HARQ-ACK

We can consider a few use cases for SPS HARQ-ACK feedback:

- Use case 1:

  - If S/SP CSI targeting URLLC traffic as discussed currently in Rel-17, can be motivated & specified, then HP S/SP CSI’s PUCCH can be also considered as container for cross-priority HARQ-ACK/SR multiplexing. In this case, cross-priority HARQ multiplexing over PUCCH for CSI can happen:

    - LP SPS HARQ-ACK + HP S/SP CSI

- User case 2:

  - Even if S/SP CSI is still always at LP in Rel-17, the payload size range may swing more widely in Rel-17 than in Rel-16, with SPS HARQ-ACK deferral.

  - There is no agreement yet in RAN1 on the PUCCH resource for SPS HARQ-ACK deferral:

    - Several options can be considered:

      - DG HARQ-ACK’s PUCCH resource sets/PUCCH resources, but the lack of PRI indication may make such a design less attractive from network operation of view;

      - SPS HARQ-ACK’s own PUCCH resources, then depending on the presence/absence of SPS HARQ-ACK(without deferral) and SPS HARQ-ACK with deferral due on the current slot, the combined payload size can swing widely.

      - Dedicated PUCCH resources for SPS HARQ-ACK deferral.

With the considered use cases, we propose to consider the flow chart in Figure 10-2.

- SPS HARQ deferral is treated as a new input for the UCI multiplexing.

- The blue box can be modified to use CSI omission rules and HARQ compaction rules.

- The yellow box already supports CSI omission rules:

  - Consider the payload size fluctuation in Rel-17 is much more than in Rel-15, over-provision of PF2/PF3 may be necessary, from that PRB # adjustment may become necessary also as when SPS HARQ-ACK deferral is not present, the payload can be smaller.

Proposal 10-1: consider PRB # adjustment and CSI omission/HARQ compaction for the PUCCH resource under multi-CSI-PUCCH-ResourceList and pucch-CSI-ResourceList.

# UCI multiplexing over PUSCH

UCI multiplexing over PUSCH in Rel-15 is quite complex already:

- Depending on the number of HARQ-ACK bits, puncturing or rate matching is used for HARQ-ACK bits.

- As PUSCH does not carry SR, the UCI types are limited to HARQ-ACK and CSI. When both HARQ-ACK and CSI feedback are present, then a three-part UCI multiplexing scheme is used.

Given the complexity of the existing NR design, it is beneficial to reuse the existing design as much as possible. One can treat the CSI encoding/multiplexing almost as functions in a programming language, then with proper adaption of the inputs to these functions, the Rel-17 design can be obtained. Note however, some details still need to be worked out.

Figure 11-1 Mapping of UCIs to 3 parts in UCI multiplexing over PUSCH in Rel-15/16

The mapping rules for HARQ-ACK, CSI Part I and CSI part II over PUSCH are specified in TS 38.212 in Rel-15/16, e.g. in Clause 6.2.7 (Data and control multiplexing, TS 38.212).

The inputs to the mapping rules are defined in Clause 6.2.7:

“Denote the coded bits for UL-SCH as .

Denote the coded bits for HARQ-ACK or jointly coded bits for HARQ-ACK and CG-UCI when the high layer parameter cg-UCI-Multiplexing is configured, if any, as .

Denote the coded bits for CSI part 1, if any, as .

Denote the coded bits for CSI part 2, if any, as .

Denote the coded bits for CG-UCI without HARQ-ACK, if any, as g0CG-UCI,  g1CG-UCI,  g2CG-UCI, g3CG-UCI, …, gGCG-UCI-1CG-UCI.

Denote the multiplexed data and control coded bit sequence as . “

We denote the resources taken by HARQ-ACK and/or CG-UCI ( or ) , CSI Part I () and CSI part II () as UCI Part 0, UCI Part I and UCI part II respectively. Then by creating mapping rules subsequently, it is intended to a large degree the mapping rules for HARQ-ACK, CSI Part I, CSI part II as captured as a 6 step procedure in Clause 6.2.7 of TS 38.212 can be reused for UCI Part 0, UCI part I and UCI Part II.

To avoid cumbersome description, we may drop mention of CG-UCI in the following. It is understood that CG-UCI is mapped to UCI Part 0.

With proposed solutions below, UCIs in UCI Part 0 populate a sequence in replacement of , UCIs in UCI Part I populate a sequence in replacement of , UCIs in UCI Part II populate a sequence in replacement of .

## Mapping of UCIs to UCI Parts over PUSCH

Figure 11-2 Separate encoding of HP HARQ-ACK and LP HARQ-ACK

With the goal to reuse the Rel-15 design, LP HARQ-ACK can be mapped to UCI Part I or UCI Part II:

- If mapped to UCI Part I (shown by the blue solid line), then the encoding sequence is CSI Part I > LP HARQ-ACK if CSI Part I contains HP CSI, otherwise the encoding sequence is LP HARQ-ACK> CSI Part I.

- If mapped to UCI Part II (shown by the red dashed line), then the encoding sequence is CSI Part II > LP HARQ-ACK if CSI Part II contains HP CSI, otherwise LP HARQ-ACK> CSI Part II can be used.

Note even on LP PUSCH, LP HARQ-ACK is bumped to Part I or Part II once HP HARQ-ACK is present.

To avoid complicated CI omission rules, with a consideration similar to that on UCI multiplexing on PUCCH, LP CSI can be dropped once HP CSI is present.

Figure 11-3 Separate UCI multiplexing choice

LP HARQ-ACK mapping Alt. 1:

Benefit of mapping LP HARQ-ACK to UCI part I:

It is reasonable to assume UCI part I tends to be better protected than UCI part II. Mapping LP HARQ-ACK to UCI Part I gives better protection to LP HARQ-ACK than mapping LP HARQ-ACK to UCI Part II.

LP HARQ-ACK mapping Alt. 2:

Benefit of mapping LP HARQ-ACK to UCI part II:

- Since in Rel-15 CSI omission is applied on UCI PART II, to minimize specification impact and change in implementation for Rel-17, LP HARQ-ACK can be mapped to Part II all the time irrespective of whether LP PUSCH or HP PUSCH carries mixed UCIs including LP-HARQ-ACK. With that, tasks of CSI omission and HARQ-ACK omission/compaction can be conducted on a single UCI part instead of being spread to multiple UCI parts. Hence it may happen HP HARQ-ACK is mapped to Part 0, Part I is empty, LP HARQ-ACK is over Part II, which should not be a problem from specification or implementation point of view.

- Considering there may be important HP AP CSI carried in UCI part I, mapping LP HARQ-ACK into UCI Part II may have less impact to HP UCIs than mapping LP HARQ-ACK into UCI Part I.

The treatment of UCI multiplexing over HP PUSCH is given in Table 11-1 and illustrated in Figure 11-4. Similar to the discussion on UCI ordering for PUCCH, the order by which UCIs are concatenated in a UCI Part may be relevant to UCI omission rules. For example, for the combination highlighted in purple in Table 11-1, LP-HARQ-ACK would be omitted first before any component from  HP CSI Part II is omitted.

Table 11-1 UCI multiplexing over HP PUSCH

|  | HP CSI is present |  | HP CSI is absent |  |
| --- | --- | --- | --- | --- |
|  | HP PUSCH (Alt. 1) | HP PUSCH (Alt. 2) | HP PUSCH (Alt. 1) | HP PUSCH (Alt. 2) |
| UCI Part 0 | HP HARQ-ACK | HP HARQ-ACK | HP HARQ-ACK | HP HARQ-ACK |
| UCI Part I | HP CSI Part I, LP HARQ-ACK | HP CSI Part I | LP HARQ-ACK |  |
| UCI Part II | HP CSI Part II | HP CSI Part II, LP-HARQ-ACK |  | LP-HARQ-ACK |
| Note 1: if HP CSI consists of a single part, then HP CSI part II is absent Note 2: If CG-UCI is present, it is carried in UCI Part 0 |  |  |  |  |

Figure 11-4 Separate UCI multiplexing (LP HARQ-ACK over HP PUSCH)

The treatment of UCI multiplexing over LP PUSCH is given in Table 11-1 and illustrated in Figure 11-5.

Table 11-2 UCI multiplexing over LP PUSCH

|  | HP CSI is present |  | Both HP CSI and LP CSI are absent |  | HP CSI is absent, and LP CSI is present. |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | LP PUSCH (Alt. 1) | LP PUSCH (Alt. 2) | LP PUSCH (Alt. 1) | LP PUSCH (Alt. 2) | LP PUSCH (Alt. 1) | LP PUSCH (Alt. 2) |
| UCI Part 0 | HP HARQ-ACK | HP HARQ-ACK | HP HARQ-ACK | HP HARQ-ACK | HP HARQ-ACK | HP HARQ-ACK |
| UCI Part I | HP CSI Part I, LP HARQ-ACK | HP CSI Part I | LP HARQ-ACK |  | LP HARQ-ACK, LP CSI Part I | LP CSI Part I |
| UCI Part II | HP CSI Part II | HP CSI Part II, LP-HARQ-ACK (discard LP-HARQ-ACK first) |  | LP-HARQ-ACK | LP CSI Part II | LP-HARQ-ACK, LP CSI Part II, (discard LP CSI first) |
| Note 1: if HP CSI consists of a single part, then HP CSI part II is absent. Note 2: If CG-UCI is present, it is carried in UCI Part 0 |  |  |  |  |  |  |

Figure 11-5 Separate UCI multiplexing choice (HP HARQ-ACK over LP PUSCH)

We have

Proposal 11-1: Between two options of mapping LP HARQ-ACK into UCI Part I and UCI Part II, RAN1 select one of them consistently across multiplexing scenarios.

Proposal 11-2: Adopt Alt. 1 or Alt. 2 design from Tables 11-1 and 11-2.

Proposal 11-3: For a UCI part, UCI omission/compaction is applied to the right-most UCI first among UCIs in that UCI part as in Tables 11-1 and 11-2. Before all the later-placed UCIs are omitted, an early UCI is not omitted or compacted.

In previous meetings, there were proposals to drop CSI Part II and keep CSI part I for multiplexing HP and LP HARQ-ACK over PUSCH. In our view, such a treatment is neither motivated by implementation complexity nor justified by specification development effort. It is well known the multiplexing of HARQ-ACK , CSI part I and CSI part II is routinely conducted over PUCCH. We don’t see any fundamental difference for that between PUCCH and PUSCH. We have

Observation 11-1: multiplexing of CSI part I and HARQ-ACK is supported over PUCCH and there is no fundamental difference in terms of implementation complexity between PUCCH and PUSCH.

It is worthwhile to recall details on UCI part1/UCI part2, UCI components for Type II codebook, as captured in a table from the rapporteur’s summary on Rel-16 NR eMIMO are shown below. In Rel-15/16, MIMO delegates from different companies worked hard to come up with CSI omission rules, so in the case there is a payload shortage (rank 2 is preferred yet base station gives enough resource for rank 1 feedback only), CSI omission is applied to UCI part 2, and the resulted CSI is useful to gNB scheduler. The same goes for Rel-15 UCI part 2 omission rule.

As the Rel-15 Type II UCI part 2 omission rule is easier to explain, we can use that as an example, roughly speaking for Rel-15 Type II CSI omission for UCI part 2,  PMIs for subbands of even indices are kept and PMIs for subbands of odd indices are discarded.  Assuming frequency selective fading is not too severe, gNB can still use interpolation to recover the odd-indexed subbands’ PMIs, so the UCI with omission remains useful to gNB scheduler. When UCI part 2 is dropped in total, the whole PMI information is gone, which totally defeats the purpose of Type II CSI feedback. Note by TS 38.214, wideband CQI and subband CQI are derived under the PMI assumption, once information about PMI is gone, then widebandCQI and subband CQI are meaningless. Also the “ # NZ coefficients” is for the PMI definition, since PMI information in UCI part II is gone, it is also meaningless. We have

Observation 11-2: If CSI part II is dropped in total by design when accommodating HP/LP HARQ-ACK multiplexing, then the CSI feedback is useless.

In general, AP CSI is critical to gNB’s scheduling decision, it would be wrong to spend time & effort to develop a feature where CSI feedback is sacrificed for no reason so HP/LP HARQ-ACK multiplexing can be supported, which would make feature selection problematic and go against the goal of Rel-17 enhancement.

We have

Proposal 11-4: LP HARQ-ACK can be multiplexed to either CSI part 1 or CSI part 2, CSI part 2 is dropped by design due to the presence of LP HARQ-ACK on PUSCH.

Table  UCI parameter list for Type II codebook

| Parameter | Location | Details/description |
| --- | --- | --- |
| # NZ coefficients | UCI part 1 | RI ({1,…, RIMAX}) and KNZ,TOT (the total number of non-zero coefficients summed across all the layers, where KNZ,TOT {1,2,…, 2K0} are reported in UCI part 1 |
| Wideband CQI | UCI part 1 | Same as R15 |
| Subband CQI | UCI part 1 | Same as R15 |
| Bitmap per layer | UCI part 2 | RI=1-2: for layer l, size-2LM For RI=3-4, bitmaps, each with size-2LMi (i=0,1,…, RI-1, where i denotes the i-th layer) are reported in UCI part 2 |
| Strongest coefficient indicator (SCI) | UCI part 2 | RI=1: the existing agreement on SCI: “A log2KNZ-bit indicator for the strongest coefficient index l*,m*” RI>1: See Table below |
| SD basis subset selection indicator | UCI part 2 | SD basis subset selection indicator is a -bit indicator. Details follow Rel.15 |
| FD basis subset selection indicator | UCI part 2 | See Table below |
| LC coefficients: phase | UCI part 2 | Quantized independently across layers |
| LC coefficients: amplitude | UCI part 2 | Quantized independently across layers (including reference amplitude for weaker polarization, for each layer) |
| SD oversampling (rotation) factor q1, q2 | UCI part 2 | Values of q1, q2 follow Rel.15 |

# PUSCH resource determination (α and β) for UCI multiplexing

## Rel-15/16 design review

### BetaOffsets

The RRC configuration for BetaOffsets from TS 38.331 is shown below:

BetaOffsets ::= SEQUENCE {

betaOffsetACK-Index1 INTEGER(0..31) OPTIONAL, -- Need S

betaOffsetACK-Index2 INTEGER(0..31) OPTIONAL, -- Need S

betaOffsetACK-Index3 INTEGER(0..31) OPTIONAL, -- Need S

betaOffsetCSI-Part1-Index1 INTEGER(0..31) OPTIONAL, -- Need S

betaOffsetCSI-Part1-Index2 INTEGER(0..31) OPTIONAL, -- Need S

betaOffsetCSI-Part2-Index1 INTEGER(0..31) OPTIONAL, -- Need S

betaOffsetCSI-Part2-Index2 INTEGER(0..31) OPTIONAL-- Need S

}

BetaOffsets field descriptions:

betaOffsetACK-Index1

Up to 2 bits HARQ-ACK (see TS 38.213 [13], clause 9.3). When the field is absent the UE applies the value 11.

betaOffsetACK-Index2

Up to 11 bits HARQ-ACK (see TS 38.213 [13], clause 9.3). When the field is absent the UE applies the value 11.

betaOffsetACK-Index3

Above 11 bits HARQ-ACK (see TS 38.213 [13], clause 9.3). When the field is absent the UE applies the value 11.

betaOffsetCSI-Part1-Index1

Up to 11 bits of CSI part 1 bits (see TS 38.213 [13], clause 9.3). When the field is absent the UE applies the value 13.

betaOffsetCSI-Part1-Index2

Above 11 bits of CSI part 1 bits (see TS 38.213 [13], clause 9.3). When the field is absent the UE applies the value 13.

betaOffsetCSI-Part2-Index1

Up to 11 bits of CSI part 2 bits (see TS 38.213 [13], clause 9.3). When the field is absent the UE applies the value 13.

betaOffsetCSI-Part2-Index2

Above 11 bits of CSI part 2 bits (see TS 38.213 [13], clause 9.3). When the field is absent the UE applies the value 13.

### BetaOffsets for CG PUSCH and DG PUSCH

ConfiguredGrantConfig ::= SEQUENCE {

...

uci-OnPUSCH SetupRelease { CG-UCI-OnPUSCH } OPTIONAL, -- Need M

...

betaOffsetCG-UCI-r16 INTEGER (0.. 31) OPTIONAL, -- Need R

...

}

CG-UCI-OnPUSCH ::= CHOICE {

dynamic SEQUENCE (SIZE (1..4)) OF BetaOffsets,

semiStatic BetaOffsets

}

Note on “uci-OnPUSCH”

- Selection between and configuration of dynamic and semi-static beta-offset.

- For Type 1 UL data transmission without grant, uci-OnPUSCH should be set to semiStatic.

betaOffsetCG-UCI

Beta offset for CG-UCI in CG-PUSCH, see TS 38.213 [13], clause 9.3

PUSCH-Config ::= SEQUENCE {

...

uci-OnPUSCH SetupRelease { UCI-OnPUSCH} OPTIONAL, -- Need M

...

uci-OnPUSCH-ListForDCI-Format0-2-r16 SetupRelease

{ UCI-OnPUSCH-ListForDCI-Format0-2-r16} OPTIONAL, -- Need M

...

uci-OnPUSCH-ListForDCI-Format0-1-r16 SetupRelease

{ UCI-OnPUSCH-ListForDCI-Format0-1-r16 } OPTIONAL, -- Need M

}

Note on uci-OnPUSCH-ListDCI-0-1, uci-OnPUSCH-ListDCI-0-2:

- Configuration for up to 2 HARQ-ACK codebooks specific to DCI format 0_1/0_2.

- The field uci-OnPUSCH-ListDCI-0-1 applies to DCI format 0_1 and the field uci-OnPUSCHListDCI-0-2 applies to DCI format 0_2 (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13] clause 9.3).

UCI-OnPUSCH ::= SEQUENCE {

betaOffsets CHOICE {

dynamic SEQUENCE (SIZE (4)) OF BetaOffsets,

semiStatic BetaOffsets

} OPTIONAL, -- Need M

scaling ENUMERATED { f0p5, f0p65, f0p8, f1 }

}

UCI-OnPUSCH-ListForDCI-Format0-2-r16 ::= SEQUENCE (SIZE (1..2))

OF UCI-OnPUSCH-ForDCI-Format0-2-r16

UCI-OnPUSCH-ListForDCI-Format0-1-r16 ::= SEQUENCE (SIZE (1..2))

OF UCI-OnPUSCH

UCI-OnPUSCH-ForDCI-Format0-2-r16 ::= SEQUENCE {

betaOffsetsForDCI-Format0-2-r16 CHOICE {

dynamicForDCI-Format0-2-r16 CHOICE {

oneBit-r16 SEQUENCE (SIZE (2)) OF BetaOffsets,

twoBits-r16 SEQUENCE (SIZE (4)) OF BetaOffsets

},

semiStaticForDCI-Format0-2-r16 BetaOffsets

} OPTIONAL, -- Need M

scalingForDCI-Format0-2-r16 ENUMERATED { f0p5, f0p65, f0p8, f1 }

}

### Summary on Rel-15/16 design for BetaOffsets

For DG PUSCH:

For the non-fallback DCIs (0-1/0-2), if physical layer priority is configured (priorityIndicatorDCI-0-1/priorityIndicatorDCI-0-2) , then there can be one set of beta offsets for semi-static signaling of beta offsets, and there can be up to 4 sets of betaoffsets for dynamic signaling of beta offsets. Note each set of beta offsets can include 3 beta offsets for HARQ-ACK (for 3 ranges: up to 2 bits, 3 bits to 11 bits, and more than 11 bits), 2 offsets for CSI Part I (for 2 cases: up to 11 bits, and more than 11 bits), and 2 offsets for CSI Part II (for 2 cases: up to 11 bits, and more than 11 bits).

For CG PUSCH:

Similar to DG PUSCH, UCIs over CG PUSCH can be supported, and its signaling design in Rel-15 is similar to that for DG’s. Further CG-UCI’s beta offsets can be configured through RRC signaling.

The coding rate for UCI Part 0 (the part taken by HARQ-ACK) is determined according to the configured beta offsets set(s) and dynamic signaling if present:

For UCI part 0, the determination of βoffsetPUSCH can be summarized as below:

• HARQ-ACK only over PUSCH with UL-SCH or without UL-SCH (Clause 6.3.2.4.1.1):  βoffsetPUSCH=βoffsetHARQ-ACK;

• CG-UCI over PUSCH with UL-SCH (6.3.2.4.1.4 CG-UCI ):  βoffsetPUSCH=βoffsetCG-UCI;

• HARQ-ACK and CG-UCI over PUSCH with UL-SCH (6.3.2.4.1.5 HARQ-ACK and CG-UCI)

βoffsetPUSCH=βoffsetHARQ-ACK;

For UCI part 1, the determination of βoffsetPUSCH is as follows:

• CSI part I over PUSCH with UL-SCH or without UL-SCH:(Clause 6.3.2.4.1.2):  βoffsetPUSCH=βoffsetCSI-part1;

For UCI part 2, the determination of βoffsetPUSCH is as follows:

• CSI part II over PUSCH with UL-SCH or without UL-SCH (6.3.2.4.1.3 CSI part 2):  βoffsetPUSCH=βoffsetCSI-part2;

note βoffsetPUSCH is NOT used for CSI part II over PUSCH without UL-SCH.

### Summary on Rel-15/16 design for alpha

The determination of α is according to  scaling  in the RRC signaling.

The information bits or payload for UCI part 0 (corresponding to HARQ-ACK or CG-UCI or HARQ-ACK+CG-CGI in Rel-16), UCI part I (corresponding to CSI part I in Rel-16), UCI part II (corresponding to CSI part II in Rel-16).

Figure 12-1 Example of UCI multiplexing over PUSCH

## Rel-17 design for loading beta offsets

If there is no inter-L1 priority UCI multiplexing, the Rel-16 behavior should be retained. If there is inter-L1 priority UCI multiplexing, α and/or β can be modified accordingly:

Now with the UCI mapping discussed in our contribution to RAN1 #104bis (R1-2103106), UCI part II may have only 1 bit (e.g. 1 bit for LP HARQ-ACK), in this case, it may be possible to develop a new coding scheme, e.g. by using the PCFICH code design from LTE, or through padding or repetition: with padding, [X00] for 1 bit [XY 0] for 2 bits (X and Y are HARQ-ACK bits) can be fed to the Reed-Muller code. Through repetition is also possible: [XXX], or [XX], or [X0X] or [...X0..X..] (mapping two occurrence of X’s to two positions for example) can be considered. A new channel coding scheme can be also considered.

In this case, irrespective of how 1 or 2 bits for HARQ-ACK is handled over UCI Part I and UCI Part II is handled, it may be necessary to introduce a new β offset for up to Z bits for UCI Part I and/or UCI part II (Z = 1 or Z= 2 for example), then an IE under BetaOffsets can be introduced for up to Z bits such as

betaOffsetCSI-Part1-Index3 INTEGER(0..31) OPTIONAL– Need S

for LP HARQ-ACK mapping Alt. 1; or

betaOffsetCSI-Part2-Index3 INTEGER(0..31) OPTIONAL– Need S

for LP HARQ-ACK mapping Alt. 2.

With LP HARQ-ACK mapping Alt. 1, then the revised BeatOffsets set is as follows:

BetaOffsets ::= SEQUENCE {

betaOffsetACK-Index1 INTEGER(0..31) OPTIONAL, -- Need S

betaOffsetACK-Index2 INTEGER(0..31) OPTIONAL, -- Need S

betaOffsetACK-Index3 INTEGER(0..31) OPTIONAL, -- Need S

betaOffsetCSI-Part1-Index1 INTEGER(0..31) OPTIONAL, -- Need S

betaOffsetCSI-Part1-Index2 INTEGER(0..31) OPTIONAL, -- Need S

betaOffsetCSI-Part1-Index3 INTEGER(0..31) OPTIONAL, -- Need S

betaOffsetCSI-Part2-Index1 INTEGER(0..31) OPTIONAL, -- Need S

betaOffsetCSI-Part2-Index2 INTEGER(0..31) OPTIONAL-- Need S

}

betaOffsetCSI-Part1-Index3

1-2 bits of UCI part 1 bits (see TS 38.213 [13], clause 9.3). When the field is absent the UE applies the value 13.

Note up to Rel-16, α or scaling is semi-statically configured (separately for HP PUSCH and LP PUSCH).

In the discussion below, we assume the encoding of 1- or 2-bits UCI in Part I or Part II is with a solution already.

Then the UCI multiplexing resources can be fully determined once (β0,β1,β2,α) and the UCI encoding sequences are known which is treated above, where the beta offset for HARQACK/CG-UGI/(HARQ-ACK+CG-UCI) or UCI part 0 in Rel-16 corresponds to β0, and β1 is the beta offset for UCI part I, and β2 is the beta offset for UCI part II.

To make it easier to incorporate the specification change, we can categorize HARQ-ACK into Group-1-HARQ-ACK and Group-2-HARQ-ACK, basically Group-1-HARQ-ACK is transmitted over UCI Part 0, Group-2-HARQACK is transmitted over UCI Part I or UCI part II.

Essentially, we need to the adapt beta offsets and alpha for different scenarios:

Legacy scenarios:

If there is no inter-L1 priority UCI multiplexing and the UCIs are of the same physical layer priority as PUSCH, the Rel-16 design is reused, which can be also formulated as

(β0,β1,β2) = (betaOffsetHARQ−ACK or betaOffsetCG−UCI, betaOffsetCSI−part1, betaOfsetCSI−part2)

HP UCI only over LP PUSCH:

for alpha, a separate alpha can be semi-statically configured for such a case. It is also possible directly to assume α = 1 in this case and using up all the resources for HP UCI should be a viable choice.

As for beta offsets, there are two options:

Option 1: (β0,β1,β2) for HP UCIs over HP PUSCH can be re-used

Option 2: up to 4 new beta offset sets can be specifically introduced for the case with HP UCI(s) over LP PUSCH.

If the LP PUSCH is a DG (dynamic Grant) PUSCH:

beta_offset indicator in the DCI scheduling the LP PUSCH can be used to look up the set in either case. HP HARQ-ACK is Group-1-HARQ-ACK.

LP UCIs only over HP PUSCH:

a separate set of (β0,β1,β2,α) which can be different from the set for HP UCIs over HP PUSCH can be used. for alpha, a separate alpha can be semi-statically configured for such a case. It is also possible directly to assume α = 0.5 or even a smaller value.

As for beta offsets, there are two options:

Option 1: (β0,β1,β2) for LP UCIs over LP PUSCH can be re-used

Option 2: up to 4 new beta offset sets can be specifically introduced for the case with LP UCI(s) over HP PUSCH

and beta_offset indicator can be used to look up the set in either case. LP HARQ-ACK is Group-2-HARQ-ACK, which can be mapped to UCI part I or UCI part II.

Mixed UCIs over LP PUSCH:

the consideration can be similar to what we have for the case with HP UCIs only over LP PUSCH. In addition, we can also consider updating betaOffsets for the UCI part(s) which carry HP UCIs, e.g. β0 is used for the HP HARQ-ACK over LP PUSCH, which is different from that configured for LP HARQ-ACK over LP PUSCH, but β1 and β2 remain the same as for LP CSIs over PUSCH if UCI part I and UCI part II don’t carry any HP UCI. HP-ACK is Group-1-HARQ-ACK for rate matching. LP-ACK is Group-2-HARQ-ACK for rate-matching.

Mixed UCIs over HP PUSCH: α can remain unchanged compared to the case with HP UCIs only over HP PUSCH, and beta offsets can remain unchanged also compared to the case with HP UCIs only over HP PUSCH. HP HARQ-ACK is Group-1-HARQ-ACK for rate matching.

LP HARQ-ACK is Group-2-HARQ-ACK, which can be mapped to UCI part I or UCI part II.

Figure 4 betaOffset set selection at a PUSCH with a given physical layer priority

Note beta_offset indicator, the physical layer priority, and the types of UCI multiplexing (HP UCIs or LP UCIs or mixed UCIs, HARQ-ACK’s presence, etc) can be used to look up the beta offset set so the PRI field size does not need to be expanded. With that, 4 groups of beta Offsets sets can be supported:

- Group 1: beta Offsets sets for LP UCIs (all UCIs are at LP) over LP PUSCH as in the Rel-16 legacy design

- Group 2: beta Offsets sets for HP UCIs (all UCIs are at HP) over HP PUSCH as in the Rel-16 legacy design

- Group 3: beta Offsets sets for UCI(s) with at least one  UCI at HP over LP PUSCH

- Group 4: beta Offsets sets for UCI(s) with at least one  UCI at LP over HP PUSCH

Then on a LP PUSCH, with the presence of a HP UCI, Group 3 instead of Group 1 is selected. Similarly, on a HP PUSCH, with the presence of a LP UCI, Group 4 instead of Group 2 is selected.

For DG PUSCH triggered by dynamic signaling with DCI format 0-2, two groups of beta offset sets are already supported in Rel-16 w.r.t. physical layer priority, then the number of groups can be extended to 4 in Rel-17:

UCI-OnPUSCH-ListForDCI-Format0-2-r17 ::= SEQUENCE (SIZE (1..4))

OF UCI-OnPUSCH-ForDCI-Format0-2-r17

A similar extension can be considered for DG PUSCH triggered by dynamic signaling with DCI format 0-1, and the number of groups is extended to 4:

UCI-OnPUSCH-ListForDCI-Format0-1-r17 ::= SEQUENCE (SIZE (1..4))

OF UCI-OnPUSCH-r17

Note that UCI-OnPUSCH-r17 and UCI-OnPUSCH-ForDCI-Format0-2-r17 can have beta value less than 1 compared to their Rel-16 counterparts.

The semi-static beta Offset set is shown as Set 0 and Set 0A in Figure 20.  For the β1 set,  3 values are given in Figure 20 to reflect the choice to map LP HARQ-ACK to UCI Part 1 and creating a beta offset for 1~2 LP HARQ-ACK bits as discussed above. The adaptation of mapping LP HARQ-ACK to UCI part II can lead to adding one value to the β2 set. To reflect the situation that no all UCIs mapped to UCI part I or UCI part II are CSIs, the IE names can be modified, e.g. from “betaOffsetCSI-Part1-Index1” to “betaOffsetUCI-Part1-Index1”, …, from “betaOffsetCSI-Part2-Index1” to “betaOffsetUCI-Part2-Index1”, etc.

However, if the beta_offset indicator field size can be expanded, then the selection of a beta set group can be explicitly indicated.

For a configured grant configuration, as the physical layer priority is configured as part of configured grant configuration, two groups of beta Offsets sets can be configured:

- One group for the case where all UCI(s) are of the same physical layer priority as the configured grant PUSCH

- One group for the case where at one UCI is of a different physical layer priority as the configured grant PUSCH

Proposal 12-1: a beta offset set can be looked up according to physical layer priority, beta offset selection, and the presence of mixed UCIs.

# Power control aspects

In NR, important decisions on PUCCH power control were taken at RAN1 #92, which leveraged LTE’s design. By reviewing the discussion history, it can be seen much simulation work is required to arrive at an optimized design. Given the diverse issues to tackle just under UCI multiplexing, reopening all the design choices is not desirable. In Section 13.1, we first review the discussion history on power control in NR. In Section 13.2, we provide general design consideration for Rel-17. In Section 13.3, we provide our concrete proposal for power control.

## Design history in NR

In Rel-15, for PUCCH formats 2/3/4, evaluations on the candidates for ΔPUCCH_TF,c(i) were conducted. It was assumed that

  - For  2 < OUCI ≦11 bits, When RM code is used, the coding gain is considered linear.

  - For  12 ≦ OUCI ≦19 bits,  Polar code with 6-bit CRC is used.

  - For  19 bits <  OUCI ,  Polar code with 11-bit CRC is used.

It is clear that for small UCI payload size, Reed-Muller code was the target for curve-fitting, and for larger UCI payload size, polar code was the target for curve-fitting.

In Rel-15/16, only UCIs of the same L1 priority can be multiplexed in a PUCCH. The power control rules for PUCCH formats are captured in TS 38.213. For PF0/PF1, the payload size is not considered in the power control formula. For PF2/3/4, the payload size is considered. For PF2/3/4, joint encoding is supported, and for PF3/4, separate encoding is supported. It can be seen from the agreement in RAN1 #92, when the agreements on power control formula were reached, it seems joint encoding was assumed in the specification work, and separate encoding was not considered.

At RAN1 #92, the following were agreed for NR:

Agreement:

- For PUCCH format 2, 3, and 4, for the case of small UCI payload size (less than or equal to 11)

  - ΔPUCCH_TF,c(i) = 10log10(K2∙BPRES(i)) where

    - BPRES(i) = OUCI(i)/NRE(i)

    - OUCI(i) is the actual number of UCI bits transmitted in i excluding the known bits as in LTE

    - NRE(i) = MPUCCH,c(i) x number of subcarriers per PRB x number of DFT-s-OFDM/CP-OFDM symbols excluding DMRS symbols/tones

    - FFS: K2 (there is no new RRC parameter introduced)

Agreement:

For the case of large UCI payload size (greater than 11), ΔPUCCH_TF,c(i) is equal to ΔPUCCH_TF,c(i) = 10log10(2K1∙BPRE(i) – 1)

- BPRE(i) = OUCI(i)/NRE(i)

- OUCI(i) is the number of UCI bits including CRC bits in i

- NRE(i) = MPUCCH,c(i) x number of subcarriers per PRB x number of DFT-s-OFDM/CP-OFDM symbols excluding DMRS symbols/tones

- FFS: K1 (there is no new RRC parameter introduced)

While the use of OUCI(i) is more straightforward, the derivation of  can be found from the specification text below:

is a number of HARQ-ACK information bits that the UE determines as described in Clause 9.1.2.1 for Type-1 HARQ-ACK codebook and as described in Clause 9.1.3.1 or 9.1.3.3 for Type-2 HARQ-ACK codebook.is the same as  as described in Clause 9.1.4 for Type-3 HARQ-ACK codebook. If the UE is not provided any of pdsch-HARQ-ACK-Codebook, pdsch-HARQ-ACK-Codebook-r16, or pdsch-HARQ-ACK-OneShotFeedback,  if the UE includes a HARQ-ACK information bit in the PUCCH transmission; otherwise,

And tracing back the discussion history in LTE, nHARQ (which is the LTE counterpart of nHARQ-ack(i) in NR) was introduced in Rel-10. The discussion in LTE (e.g. [8]) first led to an agreement in RAN1 #63:

Agreement (RAN1 #63)

n_harq based on:

-  number of received TBs (also including SPS release)

And the calculation of  nHARQ was discussed and agreed in [9] from the email discussion in May 2011 after the RAN1 #65 meeting “[65-09] R1-111886 36213CR0299 CR on power control for HARQ-ACK transmission on PUCCH”, the calculation was incorporated in Clause 10.1 of TS 36.213v10.2.0.

One consideration of using nHARQ instead of the HARQ-ACK size is to avoid un-necessarily high transmission power of LTE PUCCH format 3 when the number of received PDSCHs is actually small.

We note  is intended to be used for the case with Reed-Muller code.

## Design considerations on PUCCH power control for Rel-17

In previous sections, we provide our view on PUCCH resource set selection, PUCCH resource determination, omission rules, etc. We can see it is necessary to reflect the payloads at different coding rates for them. The coding rates can be associated with UCI parts, e.g. UCI part 1, UCI part2, the coding rates can be also associated with UCI types including the nature of the UCI payload, e.g. whether it is for HARQ-ACK, SR or CSI, further the L1 priority (HP or LP) can be used for further associations. As discussed in Section 4.1, there are a number of choices (4.1-option 1 to 4.1-option 4) to derive coding rates (r1 and r2).  It can be seen in the Rel-16 power control formula, the payload size at PF 2/3/4 is reflected in the power adjustment. Hence we propose necessary updates to support power control for UCIs at two L1 priorities.

Here we reproduce the figures in Section 9 to make it easier to follow the discussion below.

With separate encoding, the following mapping can be considered.

Figure 9-4 Separate encoding for HP HARQ-ACK and LP HARQ-ACK

Figure 9-5 Separate encoding for HARQ-ACK/SR only

Figure 5 Delta factors for Reed-Muller code and Polar code

(Note: per symbol is equivalent to per RE in this case)

In one example, it may happen the number of UCI bits for UCI part 1 is no larger than 11, and the number of UCI bits for UCI part 2 is no larger than 11, hence each UCI part is protected through a Reed-Muller code. However, if the sum of the UCI bits from two parts exceeds 11, then the power adjustment term follows that for polar code following the current specification.

In another example, it may also happen that HP HARQ-ACK in UCI part 1 has a few bits, yet LP HARQ-ACK in UCI part 2 has tens of bits. If the Rel-16 design is used, then according to the total number of UCI bits, the power adjustment for Polar code is used, which leads to a much lower Tx power than that for Reed-Muller code if the code rate is below 2/3 with QPSK as shown in Figure 12. In that case, the protection of the HP HARQ-ACK can be lacking.

To avoid reopening the design for Rel-17, and we can derive the adjustment factors for two UCI parts separately. Purely from protecting HP UCIs’ point of view, the adjust factor for UCI part 1 should be the determining factor.

## Proposed change for PUCCH power control

Since UCI part 1 and UCI part 2 are separately encoded, the delta factors can be determined from UCI part 1 and UCI part 2. We can consider the delta factor from UCI part 1 only to prioritize the transmission of HP UCI(s) in UCI part 1. To avoid power spectral density change in the frequency domain and/or time domain, then the determined delta factor is applied for both UCI parts. To derive the delta factor for UCI part 1, its payload size and the number of used RE resource elements for UCI part 1 need to be identified. Instead of using NREi, which is the total number of resource elements to carry both UCI parts, a UCI part specific resource element parameter can be used for UCI part 1.

As the rate matching procedure can guarantee on one resource element, only coded bits from a single UCI part are present. Then we can use the number of coded bits for UCI part 1 to calculate the number of resource elements taken by UCI part 1.

1

EUCI-part1=min(Etot,⌈(OH-ACK+OH-SR+L)/r1/Qm⌉⋅Qm)

EUCI-part2=Etot-min(Etot,⌈(OH-ACK+OH-SR+L)/r1/Qm⌉⋅Qm)

EUCI-part1=min(Etot,⌈(OH-ACK+OH-SR+OH-CSI+L)/r1/Qm⌉⋅Qm)

EUCI-part2=Etot-min(Etot,⌈(OH-ACK+OH-SR+OH-CSI+L)/r1/Qm⌉⋅Qm)

EUCI-part1=min(Etot,⌈(OH-ACK+OH-SR+OCSI-part1+L)/r1/Qm⌉⋅Qm)

EUCI-part2=Etot-min(Etot,⌈(OH-ACK+OH-SR+OCSI-part1+L)/r1/Qm⌉⋅Qm)

1

EUCI-part1=min(Etot,⌈(OH-ACK+OH-SR+L)/r1/Qm⌉⋅Qm)

EUCI-part2=Etot-min(Etot,⌈(OH-ACK+OH-SR+L)/r1/Qm⌉⋅Qm)

EUCI-part1=min(Etot,⌈(OH-ACK+OH-SR+OH-CSI+L)/r1/Qm⌉⋅Qm)

EUCI-part2=Etot-min(Etot,⌈(OH-ACK+OH-SR+OH-CSI+L)/r1/Qm⌉⋅Qm)

EUCI-part1=min(Etot,⌈(OH-ACK+OH-SR+OCSI-part1+L)/r1/Qm⌉⋅Qm)

EUCI-part2=Etot-min(Etot,⌈(OH-ACK+OH-SR+OCSI-part1+L)/r1/Qm⌉⋅Qm)

1

EUCI-part1=min(Etot,⌈(OH-ACK+OH-SR+L)/r1/Qm⌉⋅Qm)

EUCI-part2=Etot-min(Etot,⌈(OH-ACK+OH-SR+L)/r1/Qm⌉⋅Qm)

EUCI-part1=min(Etot,⌈(OH-ACK+OH-SR+OH-CSI+L)/r1/Qm⌉⋅Qm)

EUCI-part2=Etot-min(Etot,⌈(OH-ACK+OH-SR+OH-CSI+L)/r1/Qm⌉⋅Qm)

EUCI-part1=min(Etot,⌈(OH-ACK+OH-SR+OCSI-part1+L)/r1/Qm⌉⋅Qm)

EUCI-part2=Etot-min(Etot,⌈(OH-ACK+OH-SR+OCSI-part1+L)/r1/Qm⌉⋅Qm)

1

EUCI-part1=min(Etot,⌈(OH-ACK+OH-SR+L)/r1/Qm⌉⋅Qm)

EUCI-part2=Etot-min(Etot,⌈(OH-ACK+OH-SR+L)/r1/Qm⌉⋅Qm)

EUCI-part1=min(Etot,⌈(OH-ACK+OH-SR+OH-CSI+L)/r1/Qm⌉⋅Qm)

EUCI-part2=Etot-min(Etot,⌈(OH-ACK+OH-SR+OH-CSI+L)/r1/Qm⌉⋅Qm)

EUCI-part1=min(Etot,⌈(OH-ACK+OH-SR+OCSI-part1+L)/r1/Qm⌉⋅Qm)

EUCI-part2=Etot-min(Etot,⌈(OH-ACK+OH-SR+OCSI-part1+L)/r1/Qm⌉⋅Qm)

Figure 13-1 UCI mapping for separate encoding of HP/LP HARQ-ACK (1).

Figure 13-2 UCI mapping for separate encoding of HP/LP HARQ-ACK (2).

In the tables above, L is the number of CRC bits which can be zero, Qmis the modulation order. Etot and  EUCI-part1 are the number of coded bits for both UCI parts and UCI part 1 respectively,  NREi and  NRE-part1  are the number of resource elements for both UCI parts and the number of resource elements for UCI part 1 respectively.

For PUCCH formats 2/3/4, the delta factor ΔTF,b,f,ci is determined from UCI part 1:

- The number of resource elements for UCI part 1 NRE-part1(i)=EUCI-part1EtotNRE(i),  where   EUCI-part1 is the number of coded bits for UCI part 1

- If  n=1NUCI-part1total‍OUCI-part1,n  is smaller or equal to 11,

  - ΔTF,b,f,c,part1i=10⋅log10(K1⋅n=1NUCI-part1total‍OUCI-part1,n/NRE-part1i)

  - If a HARQ-ACK codebook with OUCI-part1,n'  bits is included in UCI part 1,  is used instead of OUCI-part1,n'  for the HARQ-ACK codebook:

    - ΔTF,b,f,c,part1i=10⋅log10(K1⋅n=1NUCI-part1total‍OUCI-part1,n-OUCI-part1,n' +nHARQ-ACK(i)/NRE-part1i)

  - If more than one HARQ-ACK codebooks are included in UCI part 1 (e.g. one due to SPS HARQ deferral, another for HARQ feedback for dynamic grant PDSCH(s)), then replacement of the number of HARQ-ACK codebook size by the associated nHARQ-ACK can be applied to each HARQ-ACK codebook.

    - The subscript i in  is  dropped as there are multiple nHARQ-ACK.

- otherwise

  - ΔTF,b,f,c,part1i=10log10(2K2⋅BPREi-1)  where

    - and

    - BPREi=n=1NUCI-part1total‍OUCI-part1,n+OCRC,UCI-part1/NRE-part1i

- And ΔTF,b,f,ci = ΔTF,b,f,c,part1i is applied to both UCI parts.

We have

Proposal 13:  For PUCCH formats 2/3/4, the delta factor ΔTF,b,f,ci is determined from UCI part 1:

- The number of resource elements for UCI part 1 NRE-part1(i)=EUCI-part1EtotNRE(i),  where   EUCI-part1 is the number of coded bits for UCI part 1

- If  n=1NUCI-part1total‍OUCI-part1,n  is smaller or equal to 11,

  - ΔTF,b,f,c,part1i=10⋅log10(K1⋅n=1NUCI-part1total‍OUCI-part1,n/NRE-part1i)

  - If a HARQ-ACK codebook with OUCI-part1,n'  bits is included in UCI part 1,  is used instead of OUCI-part1,n'  for the HARQ-ACK codebook:

    - ΔTF,b,f,c,part1i=10⋅log10(K1⋅n=1NUCI-part1total‍OUCI-part1,n-OUCI-part1,n' +nHARQ-ACK(i)/NRE-part1i)

  - If more than one HARQ-ACK codebooks are included in UCI part 1 (e.g. one due to SPS HARQ deferral, another for HARQ feedback for dynamic grant PDSCH(s)), then replacement of the number of HARQ-ACK codebook size by the associated nHARQ-ACK can be applied to each HARQ-ACK codebook.

- otherwise

  - ΔTF,b,f,c,part1i=10log10(2K2⋅BPREi-1)  where

    - and

    - BPREi=n=1NUCI-part1total‍OUCI-part1,n+OCRC,UCI-part1/NRE-part1i

- And ΔTF,b,f,ci = ΔTF,b,f,c,part1i is applied to both UCI parts.

Looking back on Rel-15 design, for PUCCH formats 3 / 4 with two UCI parts, it might be possible to determine adjustment factors for two UCI parts first, similar to what is proposed for UCI part 1 here, then define a rule (e.g., taking the higher adjustment factor between them) to arrive at ΔTF,b,f,c,part1i. Given existing deployments and implementation, it is not desirable to reopen the discussion for Rel-15/16. With that said, given the difference can clearly see from Figure 12, the Rel-15/16 practice should not be used to block Rel-17 design.

# Discussion on PHY prioritization of overlapping dynamic grant PUSCH and configured grant PUSCH

The WI has tasked RAN1 to “Specify PHY prioritization of overlapping dynamic grant PUSCH and configured grant PUSCH of different PHY priorities on a BWP of a serving cell including the related cancelation behavior for the PUSCH of lower PHY priority, taking the solution developed during Rel-16 as the baseline”.

At RAN1 103-e, the following was agreed:

Agreements:

Support PHY prioritization of overlapping high-priority dynamic grant PUSCH and low-priority configured grant PUSCH on a BWP of a serving cell in R17.

- FFS the related cancelation behavior for the PUSCH of lower PHY priority and other details.

  - First clarify what is the scope of this feature, e.g. if overlapping between more than 2 channels is considered.

- FFS the timeline requirements.

  - First clarify what is the behavior of Rel-16 UE in case of DG/CG/UCI overlapping, with and without uplink skipping enabled.

- FFS UE capability for this feature.

- Note: The main bullet has been agreed in the WID by RAN Plenary.

In Rel-15, when timeline conditions are met, a DG PUSCH can be scheduled over a CG transmission occasion. While the specification for DG/CG with single slot transmission is rather clear, we note that the Rel-15 behavior concerning CG/DG with slot aggregation was only clarified at RAN1 101-e.

With the introduction of multiple CGs over a BWP, PUSCH repetition Type B, and physical layer priority, the Rel-16 behavior concerning CG/DG is quite complicated and in need of clarification, we have discussed them in a paper [3] in at RAN1 #106-e. In our view, bringing clarity to the Rel-16 behavior is the first step to define the Rel-17 solution, and it is important to identify what transmission strategies are already supported by the Rel-16 specification and what are not; this also fulfills the requirement in WID “taking the solution developed during Rel-16 as the baseline”. It is quite clear from the Rel-16 maintenance discussion, the Rel-16 baseline is far from being clarified. Even though companies can still strive to finish CG/DG PUSCH cancellation & replacement in Rel-17, we are not optimistic of its feasibility.

We have

observation 14-1: Clarifying the Rel-16 UE behavior concerning DG/CG transmission is a pre-condition for any Rel-17 normative work on DG/CG cancellation & replacement.

# Simultaneous PUCCH and PUSCH transmissions

The following was agreed:

Agreements:

Support simultaneous PUCCH/PUSCH transmissions on different cells at least for inter-band CA.

- FFS how to trigger this function.

- FFS for intra-band CA.

Subject to UE capability, simultaneous PUCCH and PUSCH transmissions for inter-band CA can be enabled; the enabling mechanism should be through RRC configuration. Note at RAN1 #97, the UCI multiplexing behavior was clarified, the note below has been captured in the Chairman’s notes of RAN1 #97:

The Rel-15 and Rel-16 NR design mandates that UCI which would be carried over a PUCCH overlapping with PUSCH(s), be multiplexed over one of those PUSCHs. With simultaneous PUCCH/PUSCH transmissions on different cells for inter-band CA, Step 2 is executed for PUCCH and PUSCHs on for CCs in the same band as the PUCCH-residing CC, and PUSCHs on CCs at other bands are not considered in Step 2. simultaneous PUCCH/PUSCH transmissions for intra-band CA is supported, then Step 2 is executed for PUCCH and PUSCHs on for CCs in the same CC, and PUSCHs on other CCs are not considered in Step 2.

Simultaneous PUCCH/PUSCH transmission for intra-band CA leads to Tx discontinuity and large Tx power back-off. More specifically, phase discontinuity is a concern if different durations of the PUSCH and PUCCH are used.  To avoid phase discontinuity, then PUSCH duration and PUCCH duration have to be the same. However, for transmission at different physical layer priorities, mandating the same transmission duration for PUCCH/PUSCH severely limits cases where Simultaneous PUCCH/PUSCH transmission is used. With such strong restriction on PUSCH/PUCCH, it is not clear whether Simultaneous PUCCH/PUSCH transmission for intra-band CA provides much benefit.

Figure 6 Phase discontinuity at PUCCH

Proposal 15-1: Simultaneous PUCCH/PUSCH transmission for intra-band CA is not supported if phase discontinuity problem cannot be addressed.

In NR, PTRS (Phase Tracking Reference Signal) has been introduced for PDSCH and PUSCH to handle phase noise in FR2. Essentially by inserting reference symbols in the time domain, the receiver can track the phase change from one OFDM symbol to another OFDM symbol, and with the pre-DFT insertion of PTRS with DFT S-OFDM even tracking phase change at sub-OFDM symbol is also possible. PTRS can be also used to track frequency offset, e.g. front-load DMRS and PTRS can be used together as an alternative to inserting DMRS at multiple locations in PUSCH. Similarly, PTRS can be used to handle phase discontinuity for simultaneous PUCCH/PUSCH transmission:

- PTRS in PUSCH can be configured so phase jump due to the start or the end of PUCCH on another CC can be handled.

- PTRS in PUCCH can be introduced so phase jump due to the start or the end of PUSCH on another CC can be handled.

  - Note: an often-cited example for simultaneous PUCCH/PUSCH transmissions is a short PUSCH on CC1 and a long PUCCH on CC2.

For PUCCH format 3, PTRS can be introduced in a similar way as that for DFT-S-OFDM PUSCH.

Proposal 15-2: consider the feasibility of introducing PTRS for PUCCH to handle phase discontinuity problem in simultaneous PUCCH/PUSCH transmissions for intra-band CA.

Simultaneous PUCCH/PUSCH transmission on the same cell also leads to phase discontinuity issue. When the separation of PUCCH and PUSCH in the frequency domain is large, an even more serious issue is inter-modulation interference, which leads to a big challenge in UE implementation.  We have

Proposal 15-3: Simultaneous PUCCH/PUSCH transmission on the same CC is not supported.

# Conclusion

In this contribution, we provide our views on UCI multiplexing design and other design issues in Rel-17. We have

Proposal 2-1: Step 2 consists of two steps:

In step 2-1, inter-L1 PUCCH multiplexing is performed.

HP PUCCH resources Z are arranged according to starting time. The earliest unprocessed HP PUCCH resource Z is scanned first, and any overlapping LP PUCCH resources Z are identified and the resulted HP PUCCH is the same as the PUCCH resource Z.

In Step 2-2: inter-L1 priority PUCCH/PUSCH multiplexing is performed.

  - For HP PUCCH and LP PUSCH:

    - If simultaneous PUCCH/PUSCH is not supported by the UE or inter-band simultaneous PUCCH/PUSCH transmission is not configured by the gNB,

      - LP PUSCHs on all CCs are candidates for UCI multiplexing over PUSCH, and the PUSCH selection rule is the same as in RAN1 #97 clarification;

    - Otherwise

      - Only LP PUSCH(s) which reside on a CC(s) at the same band of the PUCCH cell’s are candidates for UCI multiplexing over LP PUSCH.

  - For LP PUCCH and HP PUSCH:

    - If simultaneous PUCCH/PUSCH is not supported by the UE or inter-band simultaneous PUCCH/PUSCH transmission is not configured by the gNB,

      - HP PUSCHs on all CCs are candidates for UCI multiplexing over PUSCH, and the PUSCH selection rule is the same as in RAN1 #97 clarification;

    - Otherwise

      - Only HP PUSCH(s) which reside on a CC(s) at the same band of the PUCCH cell’s are candidates for UCI multiplexing over HP PUSCH.

Observation 3-1: in Rel-16, for PRB number adjustment/interlace number adjustment, the configured PRB number of the  selected PUCCH resource (if it is at PF2 or PF3), OUCI, OCRC, maxCodeRate according to the PUCCH format of the selected PUCCH resource,  OUCI, OCRC, maxCodeRate according to the PUCCH format of the selected PUCCH resource are used as inputs, and the same coding rate is used for two CSI parts.

Observation 3-2:for CSI omission, selected PUCCH resource (if it is at PF2 or PF3, PF4), OUCI, OCRC, maxCodeRate according to the PUCCH format of the selected PUCCH resource are used as inputs, the same coding rate is used for two CSI parts.

Proposal 5-1: considering coding rates of UCI parts in the PRB number adjustment/interlace number adjustment.

Proposal 5-2: The condition to trigger PRB number adjustment is given by

n=1NUCI-part2total‍OUCI-part2,n+OCRC,UCI-part2/(Qm⋅r2)+n=1NUCI-part1total‍OUCI-part1,n+OCRC,UCI-part1/Qm⋅r1≤MRBPUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH

And  the stop condition for PRB number adjustment  is as follows:

n=1NUCI-part2total‍OUCI-part2,n+OCRC,UCI-part2/(Qm⋅r2)+n=1NUCI-part1total‍OUCI-part1,n+OCRC,UCI-part1/Qm⋅r1≤MRB,minPUCCH⋅Nsc,ctrlRB,min⋅Nsymb-UCIPUCCH

Proposal 6-1: Consider different coding rates for UCI parts in UCI omission rule.

Proposal 6-2: With two UCI parts, the omission rule is as follows:

The following conditions are used for UCI omission:

n=1NUCI-part2reported‍OUCI-part2,n+OCRC,UCI-part2,N/(Qm⋅r2)+⌈n=1NUCI-part1total‍OUCI-part1,n+OCRC,UCI-part1/Qm⋅r1⌉≤MRBPUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCHn=1NUCI-part2reported+1‍OUCI-part2,n+OCRC,UCI-part2,N+1/(Qm⋅r2)+⌈n=1NUCI-part1total‍OUCI-part1,n+OCRC,UCI-part1/Qm⋅r1⌉>MRBPUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH

If all UCI part II is dropped, then the the following is examined to determine remaining UCIs in UCI part I:

n=1NUCI-part1reported‍OUCI-part1,n+OCRC,UCI-part1,,N/(Qm⋅r1)≤MRBPUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCHn=1NUCI-part1reported+1‍OUCI-part1,,n+OCRC,UCI-part1,N+1/(Qm⋅r1)>MRBPUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH.

Proposal 7-1:

when HP CSI is present, only the following UCIs are carried in UCI part I and part II:

HP HARQ-ACK > HP SR > (HP CSI) > LP HARQ-ACK > (LP SR)

When HP CSI is absent, only the following UCIs are carried UCI part I and part II

HP HARQ-ACK > HP SR > LP HARQ-ACK > (LP SR) > LP CSI

Proposal 8-1: consider joint PUCCH resource set selection and PUCCH resource selection.

Proposal 8-2: if joint PUCCH resource set selection and PUCCH resource selection is supported, r1 and r2 are configured per PUCCH format or per PUCCH resource.

Proposal 9-1: leverage the Rel-15 design, LP HARQ-ACK is  mapped to UCI Part II in separate encoding, adopt the UCI mapping in Figures 9-6a/9-6b.

Proposal 9-2:

Consider for PUCCH format 2 to support multiplexing of HP UCI(s) and LP UCI(s):

generating two encoded sequences for HP-ACK (with r1) and LP-ACK (with r2) separately and then concatenating those two encoded sequences into one encoded sequence, UCI mapping is not changed for PUCCH Format 2.

Proposal 10-1: consider PRB # adjustment and CSI omission/HARQ compaction for the PUCCH resource under multi-CSI-PUCCH-ResourceList and pucch-CSI-ResourceList.

Proposal 11-1: Between two options of mapping LP HARQ-ACK into UCI Part I and UCI Part II, RAN1 select one of them consistently across multiplexing scenarios.

Proposal 11-2: Adopt Alt. 1 or Alt. 2 design from Tables 11-1 and 11-2.

Observation 11-1: multiplexing of CSI part I and HARQ-ACK is supported over PUCCH and there is no fundamental difference in terms of implementation complexity between PUCCH and PUSCH.

Observation 11-2: If CSI part II is dropped in total by design when accommodating HP/LP HARQ-ACK multiplexing, then the CSI feedback is useless.

Proposal 11-3: For a UCI part, UCI omission/compaction is applied to the right-most UCI first among UCIs in that UCI part as in Tables 11-1 and 11-2. Before all the later-placed UCIs are omitted, an early UCI is not omitted or compacted.

Proposal 11-4: LP HARQ-ACK can be multiplexed to either CSI part 1 or CSI part 2, CSI part 2 is dropped by design due to the presence of LP HARQ-ACK on PUSCH.

Proposal 12-1: a beta offset set can be looked up according to physical layer priority, beta offset selection, and the presence of mixed UCIs.

Proposal 13:  For PUCCH formats 2/3/4, the delta factor ΔTF,b,f,ci is determined from UCI part 1:

- The number of resource elements for UCI part 1 NRE-part1(i)=EUCI-part1EtotNRE(i),  where   EUCI-part1 is the number of coded bits for UCI part 1

- If  n=1NUCI-part1total‍OUCI-part1,n  is smaller or equal to 11,

  - ΔTF,b,f,c,part1i=10⋅log10(K1⋅n=1NUCI-part1total‍OUCI-part1,n/NRE-part1i)

  - If a HARQ-ACK codebook with OUCI-part1,n'  bits is included in UCI part 1,  is used instead of OUCI-part1,n'  for the HARQ-ACK codebook:

    - ΔTF,b,f,c,part1i=10⋅log10(K1⋅n=1NUCI-part1total‍OUCI-part1,n-OUCI-part1,n' +nHARQ-ACK(i)/NRE-part1i)

  - If more than one HARQ-ACK codebooks are included in UCI part 1 (e.g. one due to SPS HARQ deferral, another for HARQ feedback for dynamic grant PDSCH(s)), then replacement of the number of HARQ-ACK codebook size by the associated nHARQ-ACK can be applied to each HARQ-ACK codebook.

- otherwise

  - ΔTF,b,f,c,part1i=10log10(2K2⋅BPREi-1)  where

    - and

    - BPREi=n=1NUCI-part1total‍OUCI-part1,n+OCRC,UCI-part1/NRE-part1i

- And ΔTF,b,f,ci = ΔTF,b,f,c,part1i is applied to both UCI parts.

Proposal 14-1: Clarify the Rel-16 UE behavior concerning DG/CG transmission.

Proposal 15-1: Simultaneous PUCCH/PUSCH transmission for intra-band CA is not supported if phase discontinuity problem cannot be addressed.

Proposal 15-2: consider the feasibility of introducing PTRS for PUCCH to handle phase discontinuity problem in simultaneous PUCCH/PUSCH transmissions for intra-band CA.

Proposal 15-3: Simultaneous PUCCH/PUSCH transmission on the same CC is not supported.

# References

- RP-193233, “New WID on enhanced Industrial Internet of Things (IoT) and URLLC support”, Nokia, Nokia Shanghai Bell, RAN #86, December 2019

- RP-201310, “Revised WID: Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR”, Nokia, Nokia Shanghai Bell, RAN #88e, June 2020

- R1-2008434, “Maintenance of configured grant design in Physical Layer Enhancements for NR URLLC”, Apple, RAN1 #103-e, October 2020

- R1-2103083, UCI multiplexing and PUSCH skipping design in URLLC, Apple, RAN1 #104bis-e, April 2021

- R1-1801462, Remaining details of UL power control design	Huawei, HiSilicon, RAN1 #92, Feb. 2018

- R1-1801746, Correction on NR PUCCH Power Control Formula	CATT, RAN1 #92, Feb. 2018

- R1-1801994, On Delta_TF for PUCCH Power Control	Samsung, RAN1 #92, Feb. 2018

- R1-106020, PUCCH Format 3 Transmission Power for DL CA, Samsung, RAN1 #63, November 2011.

- R1-112009, CR on power control for HARQ-ACK transmission on PUCCH, ZTE, CATT, Samsung, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, Huawei, HiSilicon, RAN1 #65, May 2011.

- R1-2106120, Design of Rel-17 intra-UE multiplexing/prioritization, RAN1 #105-e, May 2021

- R1-2107735, Design of Rel-17 intra-UE multiplexing/prioritization, RAN1 #106-e, August 2021

# Appendix Specification Text for power control formulas

If the UE is configured with a SCG, the UE shall apply the procedures described in this clause for both MCG and SCG.

-	When the procedures are applied for MCG, the term 'serving cell' in this clause refers to serving cell belonging to the MCG.

-	When the procedures are applied for SCG, the term 'serving cell' in this clause refers to serving cell belonging to the SCG. The term 'primary cell' in this clause refers to the PSCell of the SCG.

If the UE is configured with a PUCCH-SCell, the UE shall apply the procedures described in this clause for both primary PUCCH group and secondary PUCCH group.

-	When the procedures are applied for the primary PUCCH group, the term 'serving cell' in this clause refers to serving cell belonging to the primary PUCCH group.

-	When the procedures are applied for the secondary PUCCH group, the term 'serving cell' in this clause refers to serving cell belonging to the secondary PUCCH group. The term 'primary cell' in this clause refers to the PUCCH-SCell of the secondary PUCCH group. If pdsch-HARQ-ACK-Codebook-secondaryPUCCHgroup-r16 is provided, pdsch-HARQ-ACK-Codebook is replaced by pdsch-HARQ-ACK-Codebook-secondaryPUCCHgroup-r16.

### 7.2.1	UE behaviour

If a UE transmits a PUCCH on active UL BWP  of carrier  in the primary cell  using PUCCH power control adjustment state with index , the UE determines the PUCCH transmission power  in PUCCH transmission occasion  as

[dBm]

where

-	 is the UE configured maximum output power defined in [8-1, TS 38.101-1], [8-2, TS38.101-2] and [8-3, TS38.101-3] for carrier  of primary cell  in PUCCH transmission occasion

-	 is a parameter composed of the sum of a component , provided by p0-nominal, or  dBm if p0-nominal is not provided, for carrier  of primary cell  and, if provided, a component  provided by p0-PUCCH-Value in P0-PUCCH for active UL BWP  of carrier  of primary cell , where .  is a size for a set of  values provided by maxNrofPUCCH-P0-PerSet. The set of  values is provided by p0-Set. If p0-Set is not provided to the UE, ,

-	If the UE is provided PUCCH-SpatialRelationInfo, the UE obtains a mapping, by an index provided by p0-PUCCH-Id, between a set of pucch-SpatialRelationInfoId values and a set of p0-PUCCH-Value values. If the UE is provided more than one values for pucch-SpatialRelationInfoId and the UE receives an activation command [11, TS 38.321] indicating a value of pucch-SpatialRelationInfoId, the UE determines the p0-PUCCH-Value value through the link to a corresponding p0-PUCCH-Id index. The UE applies the activation command in the first slot that is after slot  where  is the slot where the UE would transmit a PUCCH with HARQ-ACK information for the PDSCH providing the activation command and  is the SCS configuration for the PUCCH

-	If the UE is not provided PUCCH-SpatialRelationInfo, the UE obtains the p0-PUCCH-Value value from the P0-PUCCH with p0-PUCCH-Id value equal to the minimum p0-PUCCH-Id value in p0-Set

-	 is a bandwidth of the PUCCH resource assignment expressed in number of resource blocks for PUCCH transmission occasion  on active UL BWP  of carrier  of primary cell  and  is a SCS configuration defined in [4, TS 38.211]

-	is a downlink pathloss estimate in dB calculated by the UE using RS resource index  as described in Clause 7.1.1 for the active DL BWP  of carrier  of the primary cell  as described in Clause 12

-	If the UE is not provided pathlossReferenceRSs or before the UE is provided dedicated higher layer parameters, the UE calculates  using a RS resource obtained from an SS/PBCH block with same SS/PBCH block index as the one the UE uses to obtain MIB

-	If the UE is provided a number of RS resource indexes, the UE calculates  using RS resource with index , where .  is a size for a set of RS resources provided by maxNrofPUCCH-PathlossReferenceRSs. The set of RS resources is provided by pathlossReferenceRSs. The set of RS resources can include one or both of a set of SS/PBCH block indexes, each provided by ssb-Index in PUCCH-PathlossReferenceRS when a value of a corresponding pucch-PathlossReferenceRS-Id maps to a SS/PBCH block index, and a set of CSI-RS resource indexes, each provided by csi-RS-Index when a value of a corresponding pucch-PathlossReferenceRS-Id maps to a CSI-RS resource index. The UE identifies a RS resource in the set of RS resources to correspond either to a SS/PBCH block index or to a CSI-RS resource index as provided by pucch-PathlossReferenceRS-Id in PUCCH-PathlossReferenceRS

-	If the UE is provided pathlossReferenceRSs and PUCCH-SpatialRelationInfo, the UE obtains a mapping, by indexes provided by corresponding values of pucch-PathlossReferenceRS-Id, between a set of pucch-SpatialRelationInfoId values and a set of referenceSignal values provided by PUCCH-PathlossReferenceRS. If the UE is provided more than one values for pucch-SpatialRelationInfoId and the UE receives an activation command [11, TS 38.321] indicating a value of pucch-SpatialRelationInfoId, the UE determines the referenceSignal value in PUCCH-PathlossReferenceRS through the link to a corresponding pucch-PathlossReferenceRS-Id index. The UE applies the activation command in the first slot that is after slot  where  is the slot where the UE would transmit a PUCCH with HARQ-ACK information for the PDSCH providing the activation command and  is the SCS configuration for the PUCCH

-	If PUCCH-SpatialRelationInfo includes servingCellId indicating a serving cell, the UE receives the RS for resource index  on the active DL BWP of the serving cell

-	If the UE is provided pathlossReferenceRSs and is not provided PUCCH-SpatialRelationInfo, the UE obtains the referenceSignal value in PUCCH-PathlossReferenceRS from the pucch-PathlossReferenceRS-Id with index 0 in PUCCH-PathlossReferenceRS where the RS resource is either on the primary cell or, if provided, on a serving cell indicated by a value of pathlossReferenceLinking

-	If the UE

-	is not provided pathlossReferenceRSs, and

-	is not provided PUCCH-SpatialRelationInfo, and

-	is provided enableDefaultBeamPL-ForPUCCH, and

-	is not provided coresetPoolIndex value of 1 for any CORESET, or is provided coresetPoolIndex value of 1 for all CORESETs, in ControlResourceSet and no codepoint of a TCI field, if any, in a DCI format of any search space set maps to two TCI states [5, TS 38.212]

the UE determines a RS resource index qd providing a periodic RS resource configured with with qcl-Type set to 'typeD' in the TCI state or the QCL assumption of a CORESET with the lowest index in the active DL BWP of the primary cell. For a PUCCH transmission over multiple slots, a same qd applies to the PUCCH transmission in each of the multiple slots.

-	The parameter  is a value of deltaF-PUCCH-f0 for PUCCH format 0, deltaF-PUCCH-f1 for PUCCH format 1, deltaF-PUCCH-f2 for PUCCH format 2, deltaF-PUCCH-f3 for PUCCH format 3, and deltaF-PUCCH-f4 for PUCCH format 4, if provided; otherwise ΔF_PUCCHF=0.

-	 is a PUCCH transmission power adjustment component on active UL BWP  of carrier  of primary cell

-	For a PUCCH transmission using PUCCH format 0 or PUCCH format 1,  where

-	 is a number of PUCCH format 0 symbols or PUCCH format 1 symbols for the PUCCH transmission as described in Clause 9.2.

-	 for PUCCH format 0

-	 for PUCCH format 1

-	 for PUCCH format 0

-	 for PUCCH format 1, where  is a number of UCI bits in PUCCH transmission occasion

-	For a PUCCH transmission using PUCCH format 2 or PUCCH format 3 or PUCCH format 4 and for a number of UCI bits smaller than or equal to 11, , where

-

-	 is a number of HARQ-ACK information bits that the UE determines as described in Clause 9.1.2.1 for Type-1 HARQ-ACK codebook and as described in Clause 9.1.3.1 or 9.1.3.3 for Type-2 HARQ-ACK codebook.is the same as  as described in Clause 9.1.4 for Type-3 HARQ-ACK codebook. If the UE is not provided any of pdsch-HARQ-ACK-Codebook, pdsch-HARQ-ACK-Codebook-r16, or pdsch-HARQ-ACK-OneShotFeedback,  if the UE includes a HARQ-ACK information bit in the PUCCH transmission; otherwise,

-	 is a number of SR information bits that the UE determines as described in Clause 9.2.5.1

-	 is a number of CSI information bits that the UE determines as described in Clause 9.2.5.2

-	 is a number of resource elements determined as , where  is a number of subcarriers per resource block excluding subcarriers used for DM-RS transmission, and  is a number of symbols excluding symbols used for DM-RS transmission, as defined in Clause 9.2.5.2, for PUCCH transmission occasion  on active UL BWP  of carrier  of primary cell

-	For a PUCCH transmission using PUCCH format 2 or PUCCH format 3 or PUCCH format 4 and for a number of UCI bits larger than 11, , where

-

-

-	 is a number of HARQ-ACK information bits that the UE determines as described in Clause 9.1.2.1 for Type-1 HARQ-ACK codebook and as described in Clause 9.1.3.1 or 9.1.3.3 for Type-2 HARQ-ACK codebook, or as described in Clause 9.1.4 for Type-3 HARQ-ACK codebook. If the UE is not provided any of pdsch-HARQ-ACK-Codebook, pdsch-HARQ-ACK-Codebook-r16, or pdsch-HARQ-ACK-OneShotFeedback,  if the UE includes a HARQ-ACK information bit in the PUCCH transmission; otherwise,

-	 is a number of SR information bits that the UE determines as described in Clause 9.2.5.1

-	 is a number of CSI information bits that the UE determines as described in Clause 9.2.5.2

-	 is a number of CRC bits that the UE determines as described in Clause 9.2

-	 is a number of resource elements that the UE determines as , where  is a number of subcarriers per resource block excluding subcarriers used for DM-RS transmission, and  is a number of symbols excluding symbols used for DM-RS transmission, as defined in Clause 9.2.5.2, for PUCCH transmission occasion  on active UL BWP  of carrier  of primary cell .

-	For the PUCCH power control adjustment state  for active UL BWP  of carrier  of primary cell  and PUCCH transmission occasion

-	 is a TPC command value included in a DCI format scheduling a PDSCH reception for active UL BWP  of carrier  of the primary cell  that the UE detects for PUCCH transmission occasion , or is jointly coded with other TPC commands in a DCI format 2_2 with CRC scrambled by TPC-PUCCH-RNTI [5, TS 36.212], as described in Clause 11.3

-	 if the UE is provided twoPUCCH-PC-AdjustmentStates and PUCCH-SpatialRelationInfo and  if the UE is not provided twoPUCCH-PC-AdjustmentStates or PUCCH-SpatialRelationInfo

-	If the UE obtains a TPC command value from a DCI format scheduling a PDSCH reception and if the UE is provided PUCCH-SpatialRelationInfo, the UE obtains a mapping, by an index provided by p0-PUCCH-Id, between a set of pucch-SpatialRelationInfoId values and a set of values for closedLoopIndex that provide the  value(s). If the UE receives an activation command indicating a value of pucch-SpatialRelationInfoId, the UE determines the value closedLoopIndex that provides the value of  through the link to a corresponding p0-PUCCH-Id index

-	If the UE obtains one TPC command from a DCI format 2_2 with CRC scrambled by a TPC-PUCCH-RNTI, the  value is provided by the closed loop indicator field in DCI format 2_2

-	 is the current PUCCH power control adjustment state  for active UL BWP  of carrier  of primary cell  and PUCCH transmission occasion , where

-	The  values are given in Table 7.1.2-1

-	 is a sum of TPC command values in a set  of TPC command values with cardinality  that the UE receives between  symbols before PUCCH transmission occasion  and  symbols before PUCCH transmission occasion  on active UL BWP  of carrier  of primary cell  for PUCCH power control adjustment state, where  is the smallest integer for which  symbols before PUCCH transmission occasion  is earlier than  symbols before PUCCH transmission occasion

-	If the PUCCH transmission is in response to a detection by the UE of a DCI format,  is a number of symbols for active UL BWP  of carrier  of primary cell  after a last symbol of a corresponding PDCCH reception and before a first symbol of the PUCCH transmission

-	If the PUCCH transmission is not in response to a detection by the UE of a DCI format,  is a number of  symbols equal to the product of a number of symbols per slot, , and the minimum of the values provided by k2 in PUSCH-ConfigCommon for active UL BWP  of carrier  of primary cell

-	If the UE has reached maximum power for active UL BWP  of carrier  of primary cell  at PUCCH transmission occasion  and , then

-	If UE has reached minimum power for active UL BWP  of carrier  of primary cell  at PUCCH transmission occasion  and , then

-	If a configuration of a  value for a corresponding PUCCH power control adjustment state  for active UL BWP  of carrier  of primary cell  is provided by higher layers,

-

If the UE is provided PUCCH-SpatialRelationInfo, the UE determines the value of  from the value of  based on a pucch-SpatialRelationInfoId value associated with the p0-PUCCH-Id value corresponding to  and with the closedLoopIndex value corresponding to ; otherwise,

-	Else,

-	, where , and  is

-	the TPC command value indicated in a random access response grant corresponding to a PRACH transmission according to Type-1 random access procedure, or in a random access response grant corresponding to MsgA transmissions according to Type-2 random access procedure with RAR message(s) for fallbackRAR, or

-	the TPC command value indicated in a successRAR corresponding to MsgA transmissions for Type-2 random access procedure, or

-	the TPC command value in a DCI format with CRC scrambled by C-RNTI or MCS-C-RNTI that the UE detects in a first PDCCH reception in a search space set provided by recoverySearchSpaceId if the PUCCH transmission is a first PUCCH transmission after 28 symbols from a last symbol of the first PDCCH reception,

and, if the UE transmits PUCCH on active UL BWP  of carrier  of primary cell ,

;

otherwise,

where  is provided by higher layers and corresponds to the total power ramp-up requested by higher layers from the first to the last preamble for active UL BWP  of carrier  of primary cell , and  corresponds to PUCCH format 0 or PUCCH format 1

Table 7.2.1-1: Mapping of TPC Command Field in a DCI format to accumulated  values

| TPC Command Field | Accumulated [dB] |
| --- | --- |
| 0 | -1 |
| 1 | 0 |
| 2 | 1 |
| 3 | 3 |

# Appendix Reflecting r1 and r2 in PUCCH resource selection

In this section and the next section, we document one design choice could be taken for PUCCH resource selection, note all the solution details can be found in our RAN1 #106-e contribution (R1-2107735, “Design considerations on Rel-17 intra-UE multiplexing/prioritization”). It is clear a complete solution was provided and we did not insist on its adoption for the sake of progress and also reflecting r1 and r2 in PRB number adjustment and UCI omission is still  possible.

From the review on Rel-16 design, it can be seen that 1) the coding rate of UCI payload plus CRC bits is not a factor considered in PUCCH resource set selection, 2) all PUCCH resources of the same PUCCH format (PF2 or PF3 or PF4) under the same PUCCH-Config have the same maxCodeRate in Rel-15/16.

Considering a key motivation to support separate encoding is to provide differentiated protection to HP HARQ-ACK and LP HARQ-ACK. With that, q1 HP HARQ-ACK pluse  q2  LP HARQ-ACK can require very different PUCCH resource in terms of number of coded bits compared to the case with q2 HP HARQ-ACK plus q1 LP HARQ-ACK. Hence the coding rates for them should be reflected in PUCCH resource set selection.

PUCCH resource set selection Alt. 1:

For PUCCH resource set selection, the following can be considered:

Ocombined=OUCI-part1+OUCI-part2⋅r1/r2,

where r1 is the maximum coding rate for UCI part I, and r2 is the maximum coding rate for UCI part II.

With the revised metric to look up PUCCH resource set, it is also clarified in Rel-17 the maxPayloadSize under PUCCH-ResourceSet corresponds to one of the configured coding rates, e.g. r1.

PUCCH resource set selection Alt. 2:

It is also possible to treat the configuration PUCCH resources/PUCCH resource sets as with a reference coding rate, e.g. rref, then depending on some condition(s), r1 and r2 are determined (e.g. r1 and r2 can be loaded with different values, e.g. depending on UCI combination, PUCCH format, number of PRBs in the PUCCH resource, PUCCH resource set, etc.), and assume the required numbers of coded bits don’t exceed the number of coded bits available at the PUCCH, i.e.

OUCI-part11r1+OUCI-part2⋅1r2≤Mref1rref,

where Mref is the maximum number of UCI bits supported over PUCCH resource set j, e.g. Mref is 2, N2=maxPayloadSize1, N3=maxPayloadSize2, 1706. Alternatively, Mref can be configured for all or some PUCCH resource set(s).

Ocombined=OUCI-part1rrefr1+OUCI-part2⋅rrefr2,

is used to look up a PUCCH resource set.

In one choice, the reference coding rate is configured for all or some PUCCH resource set(s).

In another choice, the reference coding rate is an order statistics of the maxCodeRate for PUCCH resources under a PUCCH resource set or PUCCH formats for PUCCH resources under a PUCCH resource set. In one example, the reference coding rate is the minimum/maximum/second smallest/second largest/... of all the maxCodeRate for PUCCH resources under a PUCCH resource set or PUCCH formats for PUCCH resources under a PUCCH resource set.

Then a generic formulation can be given as

Ocombined=OUCI-part1/r1+OUCI-part2/r2,

where r1=r1/rref, r2=r2/rref.

For both alternatives, Ocombined is checked with the range [1,2], [3,N2], (N2,N3], (N3,1706] for PUCCH resource set selection.

## Determination of r1 and r2

Now Multiple options are provided for determining r1 and r2:

Option 4-1) r1 and r2 can be configured for UCI Part 1 and UCI part 2 under the high priority PUCCH-Config;

Option 4-2a) r1 and r2 can be configured for UCI Part 1 and UCI part 2 for a PUCCH resource Set under the high priority PUCCH-Config; different PUCCH resource Sets may have different pairs of r1 and r2;

Option 4-2b) r1 and r2 can be configured for UCI Part 1 and UCI part 2 for a PUCCH format under the high priority PUCCH-Config;

Option 4-2c) r1 and r2 can be configured for UCI Part 1 and UCI part 2 for a PUCCH resource under the high priority PUCCH-Config;

## Summary

We have

Proposal 18-1: Ocombined=OUCI-part1+OUCI-part2⋅r1/r2  is used for PUCCH resource set selection.

# Appendix: Solution to fix inconsistent design in PUCCH resource set selection and UCI omission rule

## Inconsistent design in Rel-15/16

### Issue 1

It can be seen that in Rel-15/16, PUCCH resource set selection design and CSI omission design are not consistent: CRC bits are not used in PUCCH resource set selection but CRC bits are considered in the CSI omission rule.

Hence it may happen PUCCH resource set 2 is selected according to OUCI bits, then later it is found that CSI omission is triggered over the indicated PUCCH resource within PUCCH resource set 2; it may happen a counterpart PUCCH in PUCCH resource set 3 has enough capacity to carry the UCI bits plus CRC bits without triggering CSI omission. If the CRC bits were considered in the first place in PUCCH resource set selection, then PUCCH resource set 3 could have been selected and then un-necessary CSI omission could be avoided in Rel-15/16.

One could argue such a CSI omission issue does not amount to much problem in Rel-16: as aperiodic CSI can be also triggered if gNB needs fresh CSI. However, from the agreement reached in RAN1 #104bis-e, LP HARQ-ACK can be carried over UCI part II, it would be undesirable to drop/or compact LP HARQ-ACK if the inconsistent design from Rel-15/16 is inherited in Rel-17.

### Issue 2

UCI payload’s Coding rate is not a factor in PUCCH resource set, but it is a factor in PRB number adjustment and CSI omission.

The variation in PUCCH capacity within a PUCCH resource set is not accounted in PUCCH resource set selection.

## Joint PUCCH resource set selection and PUCCH resource selection

In Rel-15/16, PUCCH resource takes two steps: in the first step, OUCI bits is used to look up a PUCCH resource set. Actually the capacity of PUCCH resources within a PUCCH resource set varies, e.g. PUCCH-11 and PUCCH-12 are both under PUCCH resource set 1, but PUCCH-11 supports up to 10 bits for payload while PUCCH-12 supports up to 20 bits, such variation in capacity may not be reflected by the PUCCH resource set’s maxPayloadSize; in the second step, PRI from a DL DCI is used to select a PUCCH resource within the selected PUCCH resource set in the first step.

From two issues discussed above, and also from the consideration on rref and Mref under PUCCH resource set selection Alt. 2, where additional RRC signaling or rules to determine rref and Mref are needed, a simplified solution is proposed which does joint PUCCH resource set selection and PUCCH resource selection, and does not require introduction of new RRC parameters for rref and Mref.

A simpler approach, compared to the solution provided in Sections 4 to Section 6, would be to use the PRI to look up a PUCCH resource matching the PRI from each PUCCH resource set, then among those PUCCH resources with a capacity no smaller than OUCI or OUCI+OCRC, then the the PUCCH resource with the least capacity is chosen; in the case where none of those PUCCH resources has a capacity no smaller than OUCI or OUCI+OCRC, then the PUCCH resource with the largest capacity is chosen.

### Joint PUCCH resource set selection and PUCCH resource selection tackling Issue 1 and Issue 2

And for selecting a PUCCH resource from a number of PUCCH resource sets, in each PUCCH resource set, the PUCCH resource of an index as indicated by the PRI is checked one by one with the following condition:

⌈n=1NUCI-part2total‍OUCI-part2,n+OCRC,UCI-part2/(Qm⋅r2)⌉+⌈n=1NUCI-part1total‍OUCI-part1,n+OCRC,UCI-part1/Qm⋅r1⌉≤MRBPUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH

or

n=1NUCI-part2total‍OUCI-part2,n+OCRC,UCI-part2/(Qm⋅r2)+⌈n=1NUCI-part1total‍OUCI-part1,n+OCRC,UCI-part1/Qm⋅r1⌉≤MRBPUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH

Note in either case, the CRC bit length is considered on PUCCH resource determination. To avoid chicken-egg problem, it can be also assumed for information bits no smaller than 360, then L=11 is assumed.

### Joint PUCCH resource set selection and PUCCH resource selection without tackling Issue 1 and Issue 2

To be more consistent with the Rel-16 design, another choice is to exclude CRC bit length in PUCCH resource determination and the formulas can be modified by removing OCRC,UCI-part2 and/or OCRC,UCI-part1

⌈n=1NUCI-part2total‍OUCI-part2,n/(Qm⋅r2)⌉+⌈n=1NUCI-part1total‍OUCI-part1,n/Qm⋅r1⌉≤MRBPUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH

or

n=1NUCI-part2total‍OUCI-part2,n/(Qm⋅r2)+⌈n=1NUCI-part1total‍OUCI-part1,n/Qm⋅r1⌉≤MRBPUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH

From the discussion above, it can be seen with joint PUCCH resource set selection and PUCCH resource selection, there is no need to introduce PUCCH resource set-specific coding rates for UCI part I and UCI Part II. As in Rel-15/16, the maxCodeRate is configured per PUCCH format, then the enhancement in Rel-17 can be the introduction of two maxCodeRates per PUCCH format or per PUCCH resource (i.e. the two options provided in Section 4: 4-2b and 4-2C).

We have

Proposal 19-1: consider joint PUCCH resource set selection and PUCCH resource selection.

Proposal 19-2: if joint PUCCH resource set selection and PUCCH resource selection is supported, r1 and r2 are configured per PUCCH format or per PUCCH resource.