3GPP TSG RAN WG1 #106b-e			R1-2110033

e-Meeting, October 11th – 19th, 2021

Agenda Item:	8.4.3

Source:	Apple

Title:	Discussion on HARQ Enhancements for NR NTN

Document for:	Discussion/Decision

# Introduction

One of the RAN1-lead objectives in the NTN WID [1] is HARQ enhancements for NTN. A list of topics related to HARQ enhancements was provided [2]. The following agreements have been made in RAN1 #106-e meeting [3].

Agreement:

For enhancement on the HARQ process indication, extend the HARQ process ID field up to 5 bits for DCI 0-1/1-1 when the maximum supported HARQ processes number is configured as 32.

Agreement:

- For Type-1 HARQ codebook, if DCIs carrying the feedback disabled and feedback-enabled HARQ processes are detected by UE, one of following options should be supported:

  - Option-1: The UE will report NACK only for the feedback disabled HARQ process regardless of decoding results of corresponding PDSCH

  - Option-2: The UE will report NACK/ACK for the feedback disabled HARQ process depending on the decoding results of corresponding PDSCH

- FFS: Other cases, e.g., if only DCI carrying feedback disabled HARQ process is detected by UE

Agreement:

For enhancement on the HARQ process indication, one of following options for DCI 0-0/1-0 can be considered:

- Option 2: Reusing one bit from other bit field

- Option 4: No enhancement

Agreement:

For Type-1 HARQ codebook, if only DCI carrying feedback disabled HARQ process is detected by UE, one of following options should be supported:

- Option-1: The UE’s behavior is same as the case if DCIs carrying the feedback disabled and feedback-enabled HARQ processes are detected by UE

- Option-2: The UE should skip the codebook feedback at least when the feedback is carried by PUCCH

  - FFS: the case that feedback is carried by PUSCH.

Agreement:

The maximum number of supported aggregation factor (i.e., pdsch-AggregationFactor) for DL PDSCH is [X]

- FFS: X = 8, 16 or 32

Agreement:

For the DCI of PDSCH with feedback disabled HARQ processes, only one of following is supported for Type-2 codebook:

- Option-1: The C-DAI and T-DAI are the count of feedback-enabled processes, despite they are not incremented, and are taken into account by the UE for type 2 codebook generation.

- Option-2: The C-DAI and T-DAI are ignored by the UE regardless of the value for Type 2 codebook generation.

In this contribution, we provide our views on HARQ process number indication, enhancement on HARQ-ACK codebook constructions, and mechanism for disabling HARQ feedback.

# Discussion

## HARQ process number indication

It was agreed [4] that 32 is taken as the maximal supported HARQ process number for both uplink and downlink. Since the maximum bit field size for HARQ process number in existing DCI formats is 4 bits, the HARQ process number indication needs to be enhanced.

It was agreed [5] that for enhancement on the HARQ process indication, extend the HARQ process ID field up to 5 bits for DCI 0-2/1-2. It was agreed [3] that for enhancement on the HARQ process indication, extend the HARQ process ID field up to 5 bits for DCI 0_1/1_1.

For fallback DCI, it was agreed [3] to consider between two options for enhancement on the HARQ process indication. One option is to reuse one bit from other bit field, and the other option is no enhancement is needed for DCI 0_0/1_0.

The second option (i.e., no enhancement for DCI 0_0/1_0) implies that scheduling with up to 32 HARQ processes is not allowed when fallback DCI formats are used. Note that a UE is not always configured to receive non-fallback DCI. If a UE needs to use 32 HARQ processes, then the UE has to be configured to receive non-fallback DCI. This increases the blind decoding efforts at UE, which is not preferrable.

Hence, we prefer the first option (i.e., reuse one bit from other bit field), which allows a UE to be scheduled with up to 32 HARQ processes while monitoring only fallback DCI.

Regarding which bit from other DCI bit field to be re-interpreted to represent HARQ process ID, our preference is that 1 redundancy version (RV) bit is used to indicate the MSB of HARQ process number, and only RV0 and RV3 are used when the supported HARQ process number is 32. One benefit of this scheme is that the soft buffer size per HARQ process is reduced by restricting to RV0 and RV3 at high code rates. Hence, the total soft buffer size over all HARQ processes may not increase even if the number of HARQ processes is increased to 32.

Note that the limitation of RV version is already supported in NR release 16. For example, in DCI formats 0_2 and 1_2 for URLLC feature, one bit to indicate RV is supported, where only RV0 and RV3 are used (cf. Table 7.3.1.2.3-1 in [6]). In another example, in DCI format 0_1 for NR-U feature, one bit to indicate RV is supported, where only RV0 and RV2 are used (cf. Table 7.3.1.1.2-34 in [6]).

Note the discussion above should be restricted to fallback DCI with CRC scrambled by C-RNTI, CS-RNTI or MCS-C-RNTI. For the fallback DCI with CRC scrambled by P-RNTI, SI-RNTI, RA-RNTI, MsgB-RNTI, TC-RNTI, we could restrict the usage of 16 HARQ processes.

Proposal 1: Enhanced HARQ process number indication is supported for DCI 0_0/1_0 by reusing one bit from another DCI bit field (e.g., RV field).

## Enhancement on HARQ-ACK codebook

Type-1 HARQ-ACK codebook is semi-statically constructed. The size of Type-1 HARQ-ACK codebook depends on the total number of candidate PDSCH reception occasions, rather than whether or not there is actually scheduled PDSCH. If no PDSCH is received in a candidate PDSCH reception occasion due to not detecting a corresponding DCI, then NACK is generated. Here, overhead reduction of Type-1 HARQ-ACK codebook is not the main design consideration.

It was agreed [3] that for Type-1 HARQ-ACK codebook, if DCIs carrying the feedback disabled and feedback enabled HARQ processes are detected by UE, two options should be down selected. In the first option, UE reports NACK for the feedback disabled HARQ process regardless of decoding results of corresponding PDSCH. In the second option, UE reports ACK/NACK for feedback disabled HARQ process depending on decoding results of corresponding PDSCH.

The reporting of ACK/NACK for feedback disabled HARQ process does not help for HARQ retransmission. This information may be useful at gNB side for scheduling of high layer retransmission.

On the other hand, one benefit of the first option is that some HARQ feedback bits are pre-known at gNB side, which could improve the decoding performance. For example, in polar code successive cancellation list decoding, the known bits could enhance the reliability of the subsequent bit decoding. Also, the first option relaxes the decoding timeline at UE side, since the UE does not have to finish PDSCH decoding associated with feedback disabled HARQ process at the time of HARQ-ACK codebook construction. UE only needs to finish the PDSCH decoding before the reception of next PDCCH scheduling the same HARQ process number.

Proposal 2: For Type-1 HARQ-ACK codebook, if DCIs carrying the feedback disabled and feedback enabled HARQ processes are detected by UE, the UE always reports NACK for the feedback disabled HARQ process.

It was agreed [3] that for Type-1 HARQ-ACK codebook, if only DCI carrying feedback disabled HARQ process is detected by UE, two options should be down selected. In the first option, UE sends the HARQ-ACK codebook, i.e., the same behavior as the case if DCI carrying the feedback disabled and feedback enabled HARQ processes are detected by UE. In the second option, UE skips the HARQ-ACK codebook feedback.

In our view, since UE does not know whether it will detect only DCI carrying feedback disabled HARQ process until the last DCI is received, UE continues preparing the HARQ-ACK codebook. Hence, skipping the codebook feedback in the second option does not significantly simplify UE’s operations. The PUCCH resource for HARQ-ACK codebook feedback is anyway allocated and skipping codebook feedback does not improve uplink resource utilization.

On the other hand, the case where UE only detects DCIs carrying feedback disabled HARQ processes is not common, and it is not worth to optimize for this corner case. This also leads to inconsistent UE behavior, which is not preferred.

Furthermore, this optimization is against the principle of semi-static codebook construction, which indicates the HARQ-ACK codebook size depends on the DCI decoding results. The main motivation of introducing feedback disabled HARQ processes in NTN is to avoid the HARQ stalling issue due to large propagation delay in NTN. Skipping HARQ-ACK codebook feedback is irrelevant to this main motivation.

Proposal 3: For Type-1 HARQ-ACK codebook, if only DCI carrying feedback disabled HARQ process is detected by UE, the UE’s behavior is same as the case if DCIs carrying the feedback disabled and feedback enabled HARQ processes are detected by UE.

Type-2 HARQ-ACK codebook construction for NTN has been discussed in the past RAN1 meetings. It was agreed [7] that the Type-2 HARQ codebook size is reduced to only include HARQ-ACK of PDSCH with feedback enabled HARQ processes. It was agreed [5] that for DCI of PDSCH with feedback enabled HARQ process, the C-DAI and T-DAI are the count of only feedback enabled processes.

It was agreed [3] that for DCI of PDSCH with feedback disabled HARQ process, the C-DAI and T-DAI signaling is to be down selected from two options. In the first option, the C-DAI and T-DAI are the count of feedback enabled processes. In the second option, the C-DAI and T-DAI fields are reserved and ignored by UE.

The second option has the benefit of saving the DCI bit fields of C-DAI and T-DAI. It also has less specification impact.

The first option has the advantage of indicating the total number of HARQ-ACK bits, which benefits in addressing the last DCI missing issue. The UE behavior for the first option is open. In our view, when a UE receives a DCI of PDSCH with feedback disabled HARQ process, it only updates the C-DAI and T-DAI values indicated in the DCI but does not generate HARQ-ACK bits subsequently.

Proposal 4: In Type-2 HARQ-ACK codebook construction, for the DCI of PDSCH with feedback disabled HARQ processes, C-DAI and T-DAI are given the count of feedback enabled processes.

The Type-2 HARQ-ACK codebook construction for SPS PDSCH is open. The similar issue is also open for Type-1 HARQ-ACK codebook only for SPS PDSCH reception.

In release 16 terrestrial network, a SPS configuration may include multiple HARQ processes. The construction of HARQ-ACK bits for SPS PDSCH follows the “serving cell index first, SPS configuration index second, and slot index with SPS PDSCH reception third” rule.

In NTN, enabling/disabling on HARQ feedback for downlink transmission is configurable per HARQ process. It is understood this agreement is applicable to the HARQ processes in a SPS configuration. In other words, the HARQ processes in a SPS configuration can have different HARQ feedback settings. If so, then the construction rule of HARQ-ACK bits for SPS PDSCH may be modified to “serving cell index first, SPS configuration index second, and slot index with SPS PDSCH reception with feedback enabled HARQ processes third” rule. Specifically, if a SPS configuration includes feedback disabled HARQ processes, then the HARQ-ACK codebook does not include the ACK/NACK bits for those feedback disabled HARQ processes.

On the other hand, it is preferred that a SPS configuration is composed of HARQ processes with the same feedback setting (i.e., enabled or disabled HARQ feedback), since a SPS configuration is aimed to serve a certain type of data. For this purpose, a SPS configuration can additionally include a setting on HARQ feedback enabling/disabling. This setting could overwrite the per HARQ process configuration of HARQ feedback enabling/disabling.

Proposal 5: For Type-1 HARQ-ACK codebook only for SPS PDSCH and for Type-2 HARQ-ACK codebook for SPS PDSCH, if a SPS configuration includes feedback disabled HARQ processes, then the HARQ-ACK codebook does not include the ACK/NACK bits for those feedback disabled HARQ processes.

- Strive for a mechanism where all HARQ processes in a SPS configuration have the same feedback setting.

## Mechanism for disabling HARQ feedback

It was agreed [3] that the maximum number of supported aggregation factor (i.e., pdsch-AggregationFactor) for PDSCH is X, where X is to be determined by 8, 16, 32.

In NTN, the pathloss is large due to long distance between satellite and UE. The large pathloss results in the low SNR. The NTN coverage needs to be improved in general, which is also considered as a potential objective for Rel-18 NTN evolution.

In order to enhance the reliability of downlink transmission, especially for the feedback disabled HARQ processes, we think the maximum number of supported aggregation factor for PDSCH should be increased from 8. This extension applies to both dynamic grant PDSCH or SPS PDSCH.

Proposal 6: Support to increase the aggregation factor for both dynamic grant PDSCH and SPS PDSCH.

In SPS configuration, a single value of aggregation factor (i.e., pdsch-AggregationFactor) is configured between 1, 2, 4 and 8. In NGSO scenario, satellite keeps moving with a high speed, which continuously changes the distance between UE and satellite. This results in continuously varying pathloss and SNR on the channel between satellite and UE.

A single value of aggregation factor in a SPS configuration may not fit with the real-time distance between UE and satellite. For example, if the distance between UE and satellite is long, a larger aggregation factor is used for SPS PDSCH transmission. If the distance between UE and satellite is short, a smaller aggregation factor is used for SPS PDSCH transmission.

To avoid the frequent resetting a SPS configuration, we think multiple values of aggregation factor could be configured in a SPS configuration. The selection of which aggregation factor to use may depend on the distance between UE and satellite. This distance may be reflected by UE specific Koffset value, which is aligned between gNB and UE. For example, for a large UE specific Koffset, a large configured aggregation factor is used in SPS PDSCH transmission. For a small UE specific Koffset, a small configured aggregation factor is used in SPS PDSCH transmission. Through this way, the SPS configuration does not need to be frequently modified in the NGSO scenario.

Proposal 7: Consider an adjustable aggregation factor in SPS PDSCH transmission.

# Conclusion

In this contribution, we provided our views on HARQ enhancements for NTN. Our proposals are as follows:

Proposal 1: Enhanced HARQ process number indication is supported for DCI 0_0/1_0 by reusing one bit from another DCI bit field (e.g., RV field).

Proposal 2: For Type-1 HARQ-ACK codebook, if DCIs carrying the feedback disabled and feedback enabled HARQ processes are detected by UE, the UE always reports NACK for the feedback disabled HARQ process.

Proposal 3: For Type-1 HARQ-ACK codebook, if only DCI carrying feedback disabled HARQ process is detected by UE, the UE’s behavior is same as the case if DCIs carrying the feedback disabled and feedback enabled HARQ processes are detected by UE.

Proposal 4: In Type-2 HARQ-ACK codebook construction, for the DCI of PDSCH with feedback disabled HARQ processes, C-DAI and T-DAI are given the count of feedback enabled processes.

Proposal 5: For Type-1 HARQ-ACK codebook only for SPS PDSCH and for Type-2 HARQ-ACK codebook for SPS PDSCH, if a SPS configuration includes feedback disabled HARQ processes, then the HARQ-ACK codebook does not include the ACK/NACK bits for those feedback disabled HARQ processes.

- Strive for a mechanism where all HARQ processes in a SPS configuration have the same feedback setting.

Proposal 6: Support to increase the aggregation factor for both dynamic grant PDSCH and SPS PDSCH.

Proposal 7: Consider an adjustable aggregation factor in SPS PDSCH transmission.

# References

- RP-211557, “Solutions for NR to support non-terrestrial networks (NTN),” Mar. 2021.

- R1-2108511, “Summary#2 of AI 8.4.3 for HARQ in NTN,” Aug. 2021.

- Chairman’s Notes, 3GPP TSG RAN1 WG1 #106-e Meeting, Aug. 2021.

- Chairman’s Notes, 3GPP TSG RAN1 WG1 #103-e Meeting, Nov. 2020.

- Chairman’s Notes, 3GPP TSG RAN1 WG1 #105-e Meeting, May 2021.

- 3GPP TS 38.212, NR; Multiplexing and channel coding, v16.5.0, Mar. 2021.

- Chairman’s Notes, 3GPP TSG RAN1 WG1 #104-e Meeting, Feb. 2021.