3GPP TSG-RAN WG1 #106 bis-e		R1-2110041

e-Meeting, October 11th-19th, 2021

Source: 	Apple Inc.

Title:                     Duplex Operation for Redcap

Agenda item:	8.6.1.2

Document for:	Discussion and Decision

# 1 Introduction

In the RAN1 #106 e-meeting, good progress was made and the following was agreed for HD-FDD operation for Redcap [1]:

Agreement: - For Case 5 of SSB overlaps with in configured UL transmission, re-use the existing collision handling principles of Rel-15/16 for NR TDD that SSB is prioritized over configured UL transmission - The configured UL transmission includes CG-PUSCH, or SRS - FFS: Confirm that PUCCH is included Agreement - For Case 5 of SSB overlaps with configured UL transmission, the configured UL transmission includes PUCCH transmission configured by higher layers - Note: The UL transmission indicated by DCI is supposed to be dynamic UL transmission. Agreement - For Type-A HD-FDD UEs, all ROs applicable to RedCap UEs are valid, and for the case of SSB overlapping with valid RO from cell specific point of view, leave it to UE implementation whether to receive SSB or transmit PRACH - No support of differentiating of ROs for Type-A HD-FDD Redcap UEs and FD FDD RedCap UEs Agreement - For Case 8 of valid RO overlapping with PDCCH in Type 0/0A/1/2 CSS set, leave it to UE implementation whether to receive configured PDCCH or transmit PRACH - Note: For valid RO intended for PRACH triggered by PDCCH order, it has been covered in Case 2. Agreement - For Case 8 of valid RO overlapping with UE-dedicated configured DL reception (e.g., PDCCH in USS, SPS PDSCH, CSI-RS or DL PRS), leave it to UE implementation whether to receive the DL or transmit PRACH - Note: For valid RO intended for PRACH triggered by PDCCH order, it has been covered in Case 2. Agreement - For Case 5 of dynamically scheduled UL transmission vs. SSB, one or both of the following options to be determined till next meeting: - Option 1: Dynamically scheduled UL transmission is prioritized over SSB - Option 2: Reuse the existing collision handling principles of Rel-15/16 for NR TDD that SSB is prioritized over dynamically scheduled UL transmission Agreement - For Case 8 of valid RO overlapping with dynamically scheduled DL reception, downselect one of following options in next meeting - Option 2: Leave to UE implementation whether to receive the dynamically scheduled DL or transmit PRACH - Option 3: Follow the handling of Case 1 (dynamically scheduled DL reception vs. semi-statically configured UL transmission) - Option 4: Valid RO is prioritized over dynamic DL reception

In this contribution, we discuss the remaining issues of half-duplex operation focusing on how to handle collision between DL/UL for HD-FDD capable UE according to the RAN1 106e agreements.

# 2. Discussions

In RAN1 104 e-meeting, seven collision cases were identified as listed in Table 1. A general principle of collision handling for HD-FDD Redcap devices was agreed to reuse existing collision handling principles in Rel-15/16 NR for operation on a single carrier /single cell in an unpaired spectrum.

Table 1 summarized the latest status of collision handling rule defined for HD-FDD UE according to the agreement made in RAN1 106 e-meeting and the leftover open issues:

Table 1: Overview of Collision Handling Rules for identified Use Cases and Remaining Issues

| # | Description | Agreement | FFS aspects |
| --- | --- | --- | --- |
| Case 1 | Dynamically scheduled DL reception vs. semi-statically configured UL transmission (e.g., dynamic PDSCH or CSI-RS collides with configured SRS, PUCCH, or CG PUSCH) | Reuse Rel-15/16 single TDD CC collision handling rule | None |
| Case 2 | Semi-statically configured DL reception vs. dynamically scheduled UL transmission (e.g., PDCCH or SPS PDSCH collides with dynamic PUSCH or PUCCH) | Reuse Rel-15/16 single TDD CC collision handling rule | None |
| Case 3 | Semi-statically configured DL reception vs. semi-statically configured UL transmission | Reuse Rel-15/16 single TDD CC collision handling rule in general. Collision handling related to SSB or RO are to be treated in case 5 and case 8. | None |
| Case 4 | Dynamically scheduled DL reception vs. dynamic scheduled UL transmission | Reuse Rel-15/16 single TDD CC collision handling rule | None |
| Case 5 | Configured SSB vs. dynamically scheduled or configured UL transmission (e.g., PUSCH, PUCCH, PRACH, SRS) | Prioritize SSB for overlapping with CG UL transmission. | FFS on Dynamically scheduled UL Transmission |
| Case 8 | Dynamic or semi-static DL vs. valid RO | Left for UE implementation for RO colliding with SSB/CSS/SPS DL reception. | FFS Dynamic DL vs. Valid RO |
| Case 9 | Collision due to direction switching |  | FFS: whether/how to account for Tx/Rx switching time before and after the set of SSB symbols |

## 2.1 Case 5: SSB vs. Configured and Dynamically scheduled Uplink transmission

RAN1#106 e-meeting reached the following agreement for Case 5 [1]:

Agreement: - For Case 5 of SSB overlaps with in configured UL transmission, re-use the existing collision handling principles of Rel-15/16 for NR TDD that SSB is prioritized over configured UL transmission - The configured UL transmission includes CG-PUSCH, or SRS - FFS: Confirm that PUCCH is included Agreement - For Case 5 of SSB overlaps with configured UL transmission, the configured UL transmission includes PUCCH transmission configured by higher layers - Note: The UL transmission indicated by DCI is supposed to be dynamic UL transmission. Agreement - For Case 5 of dynamically scheduled UL transmission vs. SSB, one or both of the following options to be determined till next meeting: - Option 1: Dynamically scheduled UL transmission is prioritized over SSB - Option 2: Reuse the existing collision handling principles of Rel-15/16 for NR TDD that SSB is prioritized over dynamically scheduled UL transmission

The following is the collision handling defined in Rel-15/16 for TDD operation for Case 5, which essentially prioritizes the SSB transmission over any overlapped UL transmissions including both dynamically scheduled UL transmission and configured UL transmission:

For operation on a single carrier in unpaired spectrum, for a set of symbols of a slot indicated to a UE by ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon, for reception of SS/PBCH blocks, the UE does not transmit PUSCH, PUCCH, PRACH in the slot if a transmission would overlap with any symbol from the set of symbols and the UE does not transmit SRS in the set of symbols of the slot.

Figure 1: Collision handling between SSB and dynamically scheduled UL Transmissions

We prefer Opt.2 above i.e., reusing the existing rule for HD-FDD Redcap UEs to handle the SSB collisions with other channels for Case 5 (i.e., prioritize SSB over dynamically scheduled UL transmission). In our view, the Opt.1 has clear drawback, which mandates HD-FDD UE to first check the presence of any overlapped DG-PUSCH and then accordingly select SSB for RRM measurement or RLM. However, the SSB/SMTC(s) selection for RRM measurement, at least for the serving cell, is fully left for UE implementation as long as the RRM/RLM requirements defined in [2] are met. For example, it is allowed to conduct the SSB-based RRM/RLM by the hardware without any interruption/interaction with L1 control block such that the complexity and power consumption can be reduced at UE side. However, Opt.1 prohibits reusing the existing hardware-centric RRM/RLM implementation for HD-FDD UE. One concern raised on Opt.2 is impact on scheduling flexibility for HD-FDD UE.  However, it should be kept in mind that HD-FDD device never targets for high peak data rate and low latency from the beginning. In our view, it remains unclear what exactly Opt.1 is trying to optimize for HD-FDD device and why it is important. On the quite contrary, the Opt.1 adds complexity at the HD-FDD Redcap without proper justification. Nevertheless, if the scheduling flexibility is indeed seen as a problem of Opt.2, one possible option to make progress is to support both options with separate UE capabilities, which at least provides the flexibility for HD-FDD implementation to choose between the complexity reduction (Opt.2) or potential throughput/latency benefit (Opt.1)

Proposal 1: For Case 5 of dynamically scheduled UL transmission vs. SSB, support either Opt.2 (i.e., prioritize SSB over DG UL transmission) or support both options with separate UE capabilities.

## 2.2 Case 8: Dynamic or semi-static DL vs. valid RO

RAN1#106-e reached the following agreements for Case 8:

Agreement - For Type-A HD-FDD UEs, all ROs applicable to RedCap UEs are valid, and for the case of SSB overlapping with valid RO from cell specific point of view, leave it to UE implementation whether to receive SSB or transmit PRACH - No support of differentiating of ROs for Type-A HD-FDD Redcap UEs and FD FDD RedCap UEs Agreement - For Case 8 of valid RO overlapping with PDCCH in Type 0/0A/1/2 CSS set, leave it to UE implementation whether to receive configured PDCCH or transmit PRACH - Note: For valid RO intended for PRACH triggered by PDCCH order, it has been covered in Case 2. Agreement - For Case 8 of valid RO overlapping with UE-dedicated configured DL reception (e.g., PDCCH in USS, SPS PDSCH, CSI-RS or DL PRS), leave it to UE implementation whether to receive the DL or transmit PRACH - Note: For valid RO intended for PRACH triggered by PDCCH order, it has been covered in Case 2. Agreement - For Case 8 of valid RO overlapping with dynamically scheduled DL reception, down-select one of following options in next meeting - Option 2: Leave to UE implementation whether to receive the dynamically scheduled DL or transmit PRACH - Option 3: Follow the handling of Case 1 (dynamically scheduled DL reception vs. semi-statically configured UL transmission) - Option 4: Valid RO is prioritized over dynamic DL reception

In general, there are four sub-cases of valid RO overlapping with different DL channels

  - Case 8-1: Valid RO overlaps with SSB

  - Case 8-2: Valid RO overlaps with cell-specific PDCCH in Type 0/0A/1/2 CSS set

  - Case 8-3: Valid RO overlaps with UE-dedicated configured DL reception (e.g., PDCCH in USS, SPS PDSCH, CSI-RS or DL PRS)

  - Case 8-4: Valid RO overlaps with dynamically scheduled DL reception

Per agreements made in RAN1 the #106 e-meeting, a unified solution (i.e., left for UE implementation) is adopted for all sub-cases except Case 8-4, which offers advantage in terms of DL resource utilization when a UE does not select valid RO for preamble transmission.

Regarding Case 8-4, our preference is to apply the same rule as other sub-cases to leave for UE implementation. Opt.3 may cause a waste of dynamically scheduled DL reception and increase RACH procedure latency when the scheduled UE has been out-of-sync in UL and unknown by gNB. It shoulde be noted that the consequence of deprioritizing RO in Case 8-4 is totally different with deprioritizing CG-based UL transmission in Case 1 e.g., PUSCH or PUCCH. Although Opt.4 is also a feasible solution, we slightly prefer Opt.2 to apply a single rule for all sub-cases under Case 8 and avoids case-by-case optimization.

Proposal 2: For a valid RO overlaps with dynamical PDSCH, leave for UE implementation whether to receive the dynamically scheduled DL or transmit PRACH (Opt.2).

## 2.3 Case 9: Collision due to direction switching

RAN1#104bis-e reached the following working assumptions [3]

Working assumption: - For HD-FDD, reuse the same principle as Rel-15/16 UE not capable of full-duplex communication - A HD-FDD UE is not expected to transmit in the uplink earlier than [NRX-TX Tc] after the end of the last received downlink symbol in the same cell - A HD-FDD UE is not expected to receive in the downlink earlier than [NTX-RX Tc] after the end of the last transmitted uplink symbol in the same cell - FFS NTX-RX and NRX-TX - FFS: how it jointly works with the agreement for other collision cases

Two possible back-to-back UL/DL cases exist:

- Case 1: A UL transmission follows a DL reception, without a gap or with a gap that is shorter than the DL-to-UL switching time

- Case 2: A DL reception that follows an UL transmission, without a gap or with a gap that is shorter than the UL-to-DL switching time

Table 2: Collision Handling for HD-FDD UEs

| Cases | Channel#1 | Channel #2 | Comments |
| --- | --- | --- | --- |
| 1 | Dynamically scheduled DL reception | Semi-statically configured UL transmission | Ensuring switching gap by properly selecting dynamically scheduled DL or UL resource |
| 2 | Dynamically scheduled UL transmission | Semi-statically configured DL reception |  |
| 3 | Semi-statically configured DL reception | Semi-statically configured UL transmission | Managed by RRC signalling setting. |
| 4 | Dynamically scheduled DL reception | Dynamically scheduled UL transmission | Ensuring switching gap by properly selecting dynamically scheduled DL and UL resource |
| 5-1 | SSB | CG uplink transmission | Managed by higher layers signalling. |
| 5-2 | SSB | Dynamically scheduled UL transmission | Ensuring switching gap by properly selecting dynamically scheduled UL resource |
| 8-1 | SSB | Valid RO | Redcap UE is not expected to handle both channels, instead of selecting one of them. |
| 8-2 | Type 0/0A/1/2 CSS set |  |  |
| 8-3 | UE-dedicated configured DL reception (e.g., PDCCH in USS, SPS PDSCH, CSI-RS or DL PRS) |  |  |
| 8-4 | Dynamically scheduled DL reception |  | Ensuring switching gap by properly selecting dynamic scheduled DL resource |
| Note | Channel #1 is prioritized unless the collision is error case (e.g., Case 3 and Case 4) |  |  |

As summarized in Table 2, it is observed that the switching gap NTX-RX and NRX-TX can be managed by network through proper gNB scheduler implementation for cases involving dynamic scheduling (Case 1/2/4/8-4 in Table 2) or semi-static scheduling (Case 3/5-1 in Table 2).

On Case 8-1/8-2/8-3 of back-to-back cell-specific DL and UL resources, it can be supported to maintain same flexibility as in legacy FDD system and avoid impact on non-Redcap UEs. However, if the DL channel and RO has a shorter gap than the DL-to-UL switching time NTX-RX and NRX-TX, a Redcap UE should be allowed to select one of them, instead of both. Our understanding is that it is aligned with working assumption.

We therefore propose to confirm the wording assumption:

Proposal 3: Confirm the working assumption with some modifications:

- For HD-FDD, reuse the same principle as Rel-15/16 UE not capable of full-duplex communication

  - A HD-FDD UE is not expected to transmit in the uplink earlier than [NRX-TX Tc] after the end of the last received downlink symbol in the same cell

  - A HD-FDD UE is not expected to receive in the downlink earlier than [NTX-RX Tc] after the end of the last transmitted uplink symbol in the same cell.

  - The case of the “back-to-back” non-overlapping UL/DL without sufficient gap is NOT allowed except the Case 8-1/8-2/8-3.

  - For Case 8-1/8-2/8-3, the case of the “back-to-back” non-overlapping UL/DL without sufficient gap is allowed and leave it to UE implementation to ensure the switching time is satisfied.

# 3. Conclusion

In this contribution, we have presented our views on the support of HD-FDD operation for Redcap UEs. Based on the discussions, we proposed the following:

Proposal 1: For Case 5 of dynamically scheduled UL transmission vs. SSB, support either Opt.2 (i.e., prioritize SSB over DG UL transmission) or support both options with separate UE capabilities.

Proposal 2: For a valid RO overlaps with dynamical PDSCH, leave for UE implementation whether to receive the dynamically scheduled DL or transmit PRACH (Opt.2).

Proposal 3: Confirm the working assumption with some modifications:

- For HD-FDD, reuse the same principle as Rel-15/16 UE not capable of full-duplex communication

  - A HD-FDD UE is not expected to transmit in the uplink earlier than [NRX-TX Tc] after the end of the last received downlink symbol in the same cell

  - A HD-FDD UE is not expected to receive in the downlink earlier than [NTX-RX Tc] after the end of the last transmitted uplink symbol in the same cell.

  - The case of the “back-to-back” non-overlapping UL/DL without sufficient gap is not allowed except the Case 8-1/8-2/8-3.

  - For Case 8-1/8-2/8-3, the case of the “back-to-back” non-overlapping UL/DL without sufficient gap is allowed and leave it to UE implementation to ensure the switching time is satisfied.

# References

- RAN1 106 e-meeting Chairman notes

- TS 38.133: 5G NR, Requirement

- RAN1 104bis e-meeting Chairman notes