3GPP TSG RAN WG1 #106bis-e			R1-2110057

e-Meeting, October 11th – 19th, 2021

Agenda item:	8.12.2

Source:	Apple Inc.

Title:	Discussion on MBS reliability improvement for RRC_CONNECTED UEs

Document for:	Discussion/Decision

# Introduction

In RAN#86 meeting, new work item on NR Multicast and Broadcast Services [1] was agreed. Some of objectives of this study item are showing below,

- Specify RAN basic functions for broadcast/multicast for UEs in RRC_CONNECTED state [RAN1, RAN2, RAN3]:

  - Specify a group scheduling mechanism to allow UEs to receive Broadcast/Multicast service [RAN1, RAN2]

    - This objective includes specifying necessary enhancements that are required to enable simultaneous operation with unicast reception.

  - Specify support for dynamic change of Broadcast/Multicast service delivery between multicast (PTM) and unicast (PTP) with service continuity for a given UE [RAN2, RAN3]

  - Specify support for basic mobility with service continuity [RAN2, RAN3]

  - Assuming that the necessary coordination function (like functions hosted by MCE, if any) resides in the gNB-CU, specify required changes on the RAN architecture and interfaces, considering the results of the SA2 SI on Broadcast/Multicast (SP-190625) [RAN3]

  - Specify required changes to improve reliability of Broadcast/Multicast service, e.g. by UL feedback. The level of reliability should be based on the requirements of the application/service provided.[RAN1, RAN2]

  - Study the support for dynamic control of the Broadcast/Multicast transmission area within one gNB-DU and specify what is needed to enable it, if anything [RAN2, RAN3]

Some MBS reliability related agreements were made in RAN1#106-e meeting [2]. In this contribution, we discuss the MBS reliability enhancement aspects, including the HARQ-ACK feedback mechanism, Type 1 HARQ codebook, etc.

# MBS reliability improvements for RRC_CONNECTED UEs

Priority index in first DCI format

Regarding the priority index in first DCI format, two alternatives were proposed in the last RAN1 meeting. According to the discussion, the first DCI format is assumed to be the fallback DCI, which is differentiate from the second DCI format with the configurable fields. Following the similar design as Rel.16 unicast, low priority is assumed always, no new field is introduced in the first DCI format.

Agreement:[2] The priority index is, - for the second DCI format for GC-PDCCH, optionally configured to be included in the DCI format. If not configured, the priority index is not included in the DCI format and is low priory by default. - for the first DCI format for GC-PDCCH, down-select from: - Alt1: Optionally configured to be included in the DCI format. If not configured, the priority index is not included in the DCI format and is low priory by default. - Alt2: Always low priority, i.e., the priority index is not included in the DCI format.

Proposal 1: The priority index is not included in the first DCI format for GC-PDCCH, i.e., PDSCH scheduled by first DCI foams is always low priority.

NACK-only based HARQ-ACK feedback

According to last RAN1 meeting agreement, PUCCH format 0 and format 1 are supported for NACK-only based HARQ-ACK feedback. As these resources are shared among the users, different users have different BWP configuration in frequency domain. To serve the purpose of resource saving, PUCCH resources need to be independently configured in a common frequency region (CFR), the resources could not be shared with unicast and ACK/NACK based HARQ-ACK feedback.

Agreement:[3] - Support PUCCH format 0 and format 1 for NACK-only based HARQ-ACK feedback for multicast. Conclusion: PUCCH resource for NACK-only can be shared by UEs transmitting the NACK-only based HARQ-ACK feedback.

Proposal 2: PUCCH resources for NACK-only are independently configured in UL CFR.

For the PUCCH resource configuration for NACK-based feedback, the network can provide multiple MBS sessions, and one UE could just subscribe one or several MBS sessions, different users could subscribe different MBS sessions. In addition, one MBS session could be scheduled twice in one HARQ feedback window. PUCCH format 0/1 can only provide up to two HARQ-ACK bits. To better use the PUCCH resources, from network side, gNB could provide a PUCCH resources pool for all the MBS session combination, one PUCCH resource in the resource pool is associated with two MBS sessions combination. The users in the group just select the right PUCCH resource for HARQ-ACK feedback according to the MBS sessions and MBS PDSCH scheduling.

Proposal 3: PUCCH resource pool is defined for NACK-based feedback, UE select the PUCCH resource according to the subscribed MSB sessions.

As the HARQ payload size is limited to two bits for NACK-based feedback, there are several cases that the required HARQ-ACK feedback could be larger than 2bits, such as PDSCHs for a MBS service are scheduled over twice in the feedback window, the user subscribed multiple MBS services, the HARQ feedback for unicast and multicast PDSCHs is over 2bits. For the discussed alternatives, Alt2 and Alt 3have the limitation of total reported bits smaller than 2bits for each sub-slot based PUCCH or each slot based PUCCH, and unicast could not multiplexing on NACK-based PUCCH resources. For Alt 5, different user subscribe different MBS services, the bundled HARQ-ACK would have different meanings, then the NACK-based feedback PUCCH resource could not be shared among users. Alt 4 would require more PUCCH resource to be reserved for different combinations. Comparing these alternatives, it seems the Alt1 is simple and straightforward.

Agreement:[2] - When more than one NACK-only based feedback are available for transmission in the same PUCCH slot, down-select from the following alternatives: - Alt1: Support UE multiplexing the HARQ-ACK bits by transforming NACK-only into ACK/NACK HARQ bits. - Alt2: Support sub-slot based PUCCH for this case. - Alt3: Support UE transmitting more than one slot-based PUCCHs in the same PUCCH slot. - Alt4: Define combination of NACK-only which corresponds to a specific sequence or a PUCCH transmission. - Alt5: NACK-only bundling

Proposal 4: If the reported HARQ-ACK bits are larger than PUCCH payload size, UE switches to ACK/NACK based HARQ-ACK feedback mode.

HARQ-ACK feedback resource indication

One of the left issues related the PUCCH resource is how to indicate the PUCCH resource.

- Alt.1: the last DCI for unicast

The benefit of this Alternative is the unicast DCI could directly indicate UE specific PUCCH resource for HARQ-ACK feedback for users in the group. But it seems not a complete solution, if there is no unicast PDSCH is scheduled in the feedback window, then no PUCCH resource could be indicated for feedback.

- Alt.2: the last DCI across unicast and multicast;

For this Alternative, there were concerns that one PRI indicating resource to all users could cause PUCCH resource collision or reserve more PUCCH resources. Generally, PUCCH resources for MBS could not be shared among users, due to same k1 value is applied. So if the last DCI is multicast,  it’s up to gNB to configure orthogonal PUCCH resources among users within the same group. If the PUCCH overhead is the concern, HARQ-NACK based feedback mode can be configured.

Agreement:[4] For multiplexing the ACK/NACK-based HARQ-ACK feedback for multicast and unicast, determining the PUCCH resources for transmission is based on the PRI indicated in the “last DCI”, where the “last DCI” refers to, down-select the following alternatives: - Alt.1: the last DCI for unicast; - Alt.2: the last DCI across unicast and multicast;

Proposal 5: For multiplexing the ACK/NACK-based HARQ-ACK feedback for multicast and unicast, the PRI in last DCI across the unicast and multicast is used for PUCCH resource indication.

Type-1 HARQ codebook

For Type 1 HARQ-ACK codebook, one open issue is how to determine the K1 set in codebook construction for unicast and multicast to be multiplexed in the same PUCCH resource.

- Alt 1 separately handling K1 set for unicast and multicast

- This Alternative can be seen as an optimization on top of Rel.16 codebook construction. The union of the TDRA tables of unicast and multicast is only applied to intersection slots of both K1 sets. Otherwise, the unicast TDRA tables are applied to slots of unicast K1 set, and multicast TDRA tables are applied to the slots of multicast K1 set. In this way, the number of reported HARQ-ACK bits are reduced, or the redundant HARQ-ACK bits in Alt 2 are removed.

- Alt 2  Joint K1 set of unicast and multicast

- Alt 2 is to reuse Rel.16 defined mechanism. The union of TDRA tables are applied to all the slots of joint K1 set of unicast and multicast. The benefit of Alt 2 is standard impact is small, the drawback is more HARQ bits are unnecessarily required for feedback.

In short, the Alt 1 provide the better performance and save the PUCCH resource, the specification impacts are acceptable.

Agreement: For TDM-ed unicast and multicast, for Type-1 HARQ-ACK codebook construction for ACK/NACK-based unicast and multicast to be multiplexed in the same PUCCH resource, determining PDSCH reception candidate occasions is based on down-selecting one of the two alternatives as follows: - Alt 1: - for slot timing values in the intersection of set for unicast (termed set A) and set for multicast (termed set B), based on union of the PDSCH TDRA sets, - for slot timing values in set A but not in set B, based on PDSCH TDRA set for unicast, and - for slot timing values in set B but not in set A, based on PDSCH TDRA set for multicast. - Alt 2: for slot timing values in the union of set for unicast and set for multicast, based on the union of the PDSCH TDRA sets. - Companies are encouraged to continue discussion of pros and cons for each alternative for further down-selection in the next meeting.

Proposal 6: For Type-1 HARQ-ACK codebook construction for TMD-ed unicast and multicast multiplexed in the same slot, K1 set is in the intersection K1 set of unicast and multicast is based on union of the PDSCH TDRA sets, otherwise PDSCH TDRA set of unicast K1 set or multicast K1 set is applied.

# Summary

In this contribution, we discuss MBS HARQ feedback mechanism and potential reliability improvement techniques, and have the following proposals:

Proposal 1: PUCCH resources for NACK-only are independently configured in UL CFR.

Proposal 2: PUCCH resource pool is defined for NACK-based feedback, UE select the PUCCH resource according to the subscribed MSB sessions.

Proposal 3: if the reported HARQ-ACK bits are larger than PUCCH payload size, UE switches to ACK/NACK based HARQ-ACK feedback mode.

Proposal 4: For multiplexing the ACK/NACK-based HARQ-ACK feedback for multicast and unicast, the PRI in last DCI across the unicast and multicast is used for PUCCH resource indication.

Proposal 5: For Type-1 HARQ-ACK codebook construction for TMD-ed unicast and multicast multiplexed in the same slot, K1 set is in the intersection K1 set of unicast and multicast is based on union of the PDSCH TDRA sets, otherwise PDSCH TDRA set of unicast K1 set or multicast K1 set is applied.

# References

- RP-193248, “New Work Item on NR Multicast and Broadcast Services”, Huawei, HiSilicon

- R1-2108692, “Report of RAN1#106-e meeting”, ETSI MCC

- R1-2106402, “Report of RAN1#105-e meeting”, ETSI MCC

- R1-2104151, “Report of RAN1#104bis-e meeting”, ETSI MCC