3GPP TSG RAN WG1 #106bis-e	                                         	                                     R1-2110067

e-Meeting, October 11th – 19th, 2021

Agenda Item:	8.17.3

Source:	Apple Inc.

Title:	Views on UE features for Rel-17 enhanced IIoT and URLLC

Document for:	Discussion/Decision

# Introduction

In this contribution we provide our initial views on UE features for Rel-17 enhanced IIoT and URLLC as proposed in [1].

# Discussion on UE features for Rel-17 enhanced IIoT and URLLC

Given many details for Rel-17 design are still to be agreed, in general all the component descriptions should be enclosed in brackets to avoid premature discussion on them.

As for UE capability’s granularity, URLLC features tend to be complicated from the perspective of UE implementation. The “per UE” designation can lead to UE implementation efforts for no apparent use cases, so its use should be adequately justified.

In case that the discussion on feature components and capability granularity need to take place now, we also provide comments for the proposed UE features.

As for two proposed features below:

25-14	PHY prioritization of overlapping low-priority DG-PUSCH and high-priority CG-PUSCH

25-15	PHY prioritization of overlapping high-priority DG-PUSCH and low-priority CG-PUSCH

It was discussed in RAN #93-e for possible downscoping on them. We note the relevant Rel-16 maintenace work is essentially at a deadlock, and it is unlikely the Rel-16 maintenance work can be finshed in one meeting (November 2021) and at the same time the design details on DG/CG PUSCH proritization can be finalized. Hence we suggest to remove 25-14 and 25-15.

For CG enhancements on unlicensed spectrum, the following two features were agreed but not included in the feature list: disabling of CG retransmission timer/CG-UCI/CG-DFI and PUSCH repetition Type B.

- For disabling of CG retransmission timer/CG-UCI/CG-DFI (where the UE falls back to CG operation on licensed spectrum), a new FG would need to be defined because in Rel-16 these features are always enabled.

- For PUSCH repetition Type B, since the corresponding Rel-16 FG is defined as per FS, the signaling can be directly reused to cover the support on unlicensed spectrum.

Proposal 1: Define a new FG for the support of disabling CG retransmission timer/CG-UCI/CG-DFI on unlicensed spectrum.

Proposal 2: The Rel-16 capability signaling for PUSCH repetition Type B is reused for unlicensed spectrum.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25. NR_IIOT_URLLC_enh | 25-1 | SPS HARQ-ACK deferral in case of TDD collision | 1. Idenfify HARQ-ACK bits of active SPS configurations for deferral in the initial PUCCH slot 2. Determination of the target PUCCH slot for SPS HARQ-ACK deferral 3. Multiplexing and transmission of deferred SPS HARQ-ACK information in the target PUCCH slot | 5-18 | Yes | N/A |  | Per UE Apple: per-band preferred A stronger use case is with Rel-16 multiple SPS configurations (12-2) rather than 5-18. Since 12-2 is per band, per band is also preferred for 25-1. | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-2 | Repetitions for PUCCH format 0, and 2 over multiple slots with K = 2, 4, 8 | Repetitions for PUCCH format 0 and 2 over multiple slots with K = 2, 4, 8 | 4-23 Apple: Rel-15 feature is for PF 1/3/4, the dependence should be removed. | Yes | N/A |  | Per UE Apple: note 25-2 should follow 25-3’s designation as 25-2 was agreed simply to avoid fragmentation of specification. | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-3 | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots with configured K = 2, 4, 8 | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots with RRC configured repetition factor K = 2, 4, 8 | 4-23 11-3 | Yes | N/A |  | Per UE Apple: Since sub-slot URLLC feature in Rel-16 (11-3) is per FS, this should be per FS | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-3a | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots using dynamic repetition indication | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots based on dynamic repetition indication. | 25-3 30-X | Yes | N/A |  | Per UE Apple: Since sub-slot URLLC feature in Rel-16 (11-3) is per FS, this should be per FS | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-4 | One-shot HARQ ACK feedback triggered by DCI format 1_2 | 1. Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_2 scheduling a PDSCH 2. Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_2 without scheduling a PDSCH using a reserved FDRA value | 10-16 11-1 | Yes | N/A |  | Per UE Apple: the support of 1_2 is per UE, yet the support of one-shot CB (Type 3) is per band, so this should be per band | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-5 | PHY priority handling for one-shot HARQ ACK feedback | Support transmission of type 3 HARQ-ACK codebook using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI | 10-16 11-4 | Yes | N/A |  | Per UE Apple: dci-DL-PriorityIndicator-r16 in Rel-16 is per UE, so it is okay | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-6 | Enhanced type 3 HARQ-ACK codebook feedback | 1. Support feedback of enhanced type 3 HARQ-ACK codebook, triggered by a DCI 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2, 11-1) 2. Support configuration of up to X enhanced type 3 HARQ-ACK codebooks. 3. Support feedback of a dynamically selected enhanced type 3 HARQ-ACK codebook based on triggering information in DCI 1_1 and DCI 1_2 (for a UE supporting DCI format 1_2, 11-1) 4. Support transmission of enhanced type 3 HARQ-ACK codebook using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4) | 10-16 | Yes | N/A |  | Per UE Apple: Per band is prefeerred, following the logic for Rel-16 Type 3 CB | No | No | N/A | For component 2, the UE indicates its capability in the number of enhanced type 3 HARQ-ACK codebooks: {1,...,X} For component 3, the dynamic indication is only supported if the UE for component 2 supports more than one enhanced type 3 HARQ-ACK codebook to be configured | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-7 | Triggered HARQ-ACK codebook re-transmission | 1. Support HARQ-ACK re-transmission from an earlier PUCCH slot based on the triggering information in DCI format 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2, 11-1) 2. Support the related PHY priority handling in terms of HARQ-ACK codebook selection and the applicable PUCCH configuration (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4) |  | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-8 | Semi-static HARQ-ACK codebook for sub-slot PUCCH | Semi-static (Type 1) HARQ-ACK codebook for sub-slot based PUCCH configuration | 4-11 11-3 | Yes | N/A |  | Per UE Apple: Per sub-slot URLLC feature in Rel-16 is per FS , this should be per FS | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-9 | Semi-static PUCCH cell (FFS or PUCCH carrier) switching | Semi-static PUCCH cell (FFS or PUCCH carrier) switching using configured time-domain domain pattern of applicable PUCCH cell / carrier |  | Yes | N/A |  | Per UE Apple: Since this is for inter-band TDD, per band per band combination is more reasonable. | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-10 | PUCCH cell (FFS or PUCCH carrier) switching based on dynamic indication | PUCCH cell (FFS or PUCCH carrier) switching based on dynamic indication in the DCI scheduling the PUCCH |  | Yes | N/A |  | Per UE Apple: Since this is for inter-band TDD, per band per band combination is more reasonable. | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-11 | 4-bits subband CQI | Subband CQI reporting with 4 bits per subband |  | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-12 | UE initiating a semi-static channel occupancy with configurations dependent on gNB semi-static channel access configurations | Support initiating a semi-static channel access occupancy by the UE where the corresponding period is the same as, integer multiple of, or inter-factor of the period configured for a semi-static channel occupancy that can be initiated by gNB. | 10-1a | Yes | N/A |  | Per band | N/A | N/A | N/A | The signaling is per band but is only expected for a band where shared spectrum channel access must be used | Optional with capability signaling [This FG is a part of basic operation for following scenarios defined in TS38.300 Scenario A2, B, C, D and E with semi-static channel access mode] |
| 25. NR_IIOT_URLLC_enh | 25-13 | UE initiating a semi-static channel occupancy with independent configurations from gNB semi-static channel access configurations | Support initiating a semi-static channel access occupancy by the UE where the corresponding period is independently configured from the period configured for a semi-static channel occupancy that can be initiated by gNB. | 10-1a | Yes | N/A |  | Per band | N/A | N/A | N/A | The signaling is per band but is only expected for a band where shared spectrum channel access must be used | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-14 | PHY prioritization of overlapping low-priority DG-PUSCH and high-priority CG-PUSCH | Support PHY prioritization for the case where low-priority DG-PUSCH collides with high-priority CG-PUSCH | 12-1 | Yes | N/A |  | Per band Apple: Premature to discuss 25-14 | N/A | N/A | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-15 | PHY prioritization of overlapping high-priority DG-PUSCH and low-priority CG-PUSCH | Support PHY prioritization of overlapping high-priority dynamic grant PUSCH and low-priority configured grant PUSCH on a BWP of a serving cell | 12-1 | Yes | N/A |  | Per band Apple: Premature to discuss 25-15 | N/A | N/A | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-16 | HARQ-ACK with different priorities multiplexing using a PUCCH | 1. Support multiplexing a high-priority HARQ-ACK and a low-priority HARQ-ACK into a PUCCH. Support separate coding for the two HARQ-ACKs. 2. [Support multiplexing a low-priority HARQ-ACK and a high-priority SR into a PUCCH for some HARQ-ACK/SR PF combinations (FFS applicable combinations).] 3. [Support multiplexing a low-priority HARQ-ACK, a high-priority HARQ-ACK and a high-priority SR into a PUCCH.] | 11-3 | Yes | N/A |  | Per UE Apple: Note the cancellation behavior is defined at FS level, this should be at FS level also. Premature to discuss the components | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-17 | HARQ-ACK piggybacked on a PUSCH of a different priority | 1. Support multiplexing a low-priority HARQ-ACK in a high-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination. 2. Support multiplexing a high-priority HARQ-ACK in a low-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination. 3. Support multiplexing a low-priority HARQ-ACK a high-priority HARQ-ACK and/or CSI in a high-priority PUSCH. Support separate beta_offset values for this priority combination. 4. Support multiplexing a high-priority HARQ-ACK, a low-priority HARQ-ACK and/or CSI in a low-priority PUSCH. Support separate beta_offset values for this priority combination. | 11-3 12-1 | Yes | N/A |  | Per UE Apple: Note the cancellation behavior is defined at FS level, this should be at FS level also. Premature to discuss the components | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-18 | Parallel PUCCH and PUSCH transmission across CCs in inter-band CA | Support simultaneous PUCCH/PUSCH transmissions on different cells [at least] for inter-band CA. |  | Yes | N/A |  | Per UE Apple: Should be per FS Putting PUCCH on a particular band | No | No | N/A |  | Optional with capability signaling |

# References

- R1-2108679, “Preliminary RAN1 UE features list for Rel-17 NR”, Moderators (AT&T, NTT DOCOMO, INC.), RAN1 #16-e, August, 2021