3GPP TSG-RAN WG1 #106 bis-e		R1-2110069

e-Meeting, October 11th-19th, 2021

Source: 	Apple Inc.

Title:                     Views on UE Features for Redcap

Agenda item:	8.17.6

Document for:	Discussion and Decision

# 1 Introduction

This contribution discusses the UE capabilities for Rel-17 Redcap WI listed in [1].

# 2. Discussions

## FG 28-2

In RAN1 105-e meeting, the following was agreed:

Agreement: For UE capability signalling, the number of Rx branches for RedCap is implicitly indicated by the corresponding capability parameter maxNumberMIMO-LayersPDSCH in the existing UE capability framework. - Detailed signalling is up to RAN2

In RAN2 115 e-meeting, the following was agreed in offline discussions over email:

- Do not introduce capability signalling on the supported Rx number for RedCap UE since the number of Rx branches for RedCap is implicitly indicated by the corresponding capability parameter maxNumberMIMO-LayersPDSCH in the existing UE capability framework;

Based on the RAN2 agreement, it is our view that FG 28-2 is not needed. In addition, the existing FG 2-3 maxNumberMIMO-LayersPDSCH is defined as per FSPC, instead of ‘per band’, which should be kept for Redcap UEs.

Proposal 1: Remove FG 28-2 in [1] and use the capability parameter maxNumberMIMO-LayersPDSCH (FG2-3) in the existing UE capability framework to indicate MIMO layers and Rx branches as agreed by RAN1 and RAN2.

Proposal 2: Add a note in 38.306 for maxNumberMIMO-LayersPDSCH IE that is also used to indicate the number of Rx branches supported by the Redcap UE.

## FG 28-4

In Rel-15, support of 256QAM for PDSCH is mandatory feature with capability for FR1 and is optional with capability signalling for FR2. Hence, the capability IE pdsch-256QAM-FR1 (FG1-4) was introduced since Rel-15 and can be used for Redcap UE naturally. What needs to be modified for Redcap is to change it from ‘Mandatory’ to ‘optional’, which can be achieved by adding a note in TS 38.306. Note that, creating a new IE FG28-4 cannot avoid adding ‘note’ for legacy FG 1-4 for Redcap as ‘mandatory with capability for FR1’ is not valid for Redcap anymore. Instead adding note to revoke the existing FG 1-4 for Redcap in FR1 and add a new FG 28-4, it is a simpler and clean way to add a note for FG 1-5 without introducing new FG 28-4. For FR2, the FG 1-4 can be reused as it has been optional feature even for eMBB UEs in Rel-15.

Proposal 3: Remove FG 28-4 in [1] and add a note for the existing FG1-4 (pdsch-256QAM-FR1) to be optional for Redcap

## FG 28-5

Similarly, support of 256QAM for PUSCH is an optional feature for Rel-15 eMBB UEs, which is indicated by FG 1-5 (pusch-256QAM). The FG 1-5 can be fully reused for Redcap without differentiating from legacy UEs on this aspect.

Proposal 4: Remove FG 28-5 in [1].

## FG 28-x

As discussed in our companion paper [2], we propose to introduce an optional UE feature FG 28-x to indicate the support of DL BWP without SSB in RRC_CONNECTED state, analogous to legacy FG 6-1A.

Proposal 5: Introduce a new FG 28-x that is used to indicate the support of a UE-specific DL BWP without SSB when UE is in RRC_CONNECTED state.

## Others

Our view on early Indication functionality for Redcap devices is that it can be defined and captured in RAN2-led UE features since the entire feature is led by RAN2 as captured in WID. Hence, it is reasonable not to include it in the RAN1 FG list.

# 3. Conclusion

Based on the discussions in the previous sections, we made the following proposal:

Proposal 1: Remove FG 28-2 in [1] and use the capability parameter maxNumberMIMO-LayersPDSCH (FG2-3) in the existing UE capability framework to indicate MIMO layers and Rx branches as agreed by RAN1 and RAN2.

Proposal 2: Add a note in 38.306 for maxNumberMIMO-LayersPDSCH IE that is also used to indicate the number of Rx branches supported by the Redcap UE.

Proposal 3: Remove FG 28-4 in [1] and add a note for the existing FG1-4 (pdsch-256QAM-FR1) to be optional for Redcap

Proposal 4: Remove FG 28-5 in [1].

Proposal 5: Introduce a new FG 28-x that is used to indicate the support of a UE-specific DL BWP without SSB when UE is in RRC_CONNECTED state.

# References

- R1-2108679  	Preliminary RAN1 UE features list for Rel-17 NR, 	AT&T, NTT DOCOMO, INC.

- RP-211574, “Support of reduced capability NR devices,” Ericsson.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 28. NR_redcap | 28-1 | RedCap UE | 1. Maximum FR1 RedCap UE bandwidth is 20 MHz. 2. Maximum FR2 RedCap UE bandwidth is 100 MHz. |  | Yes |  | Impact on UE complexity | Per UE | No | [No] |  | RedCap UEs do not support carrier aggregation or dual connectivity. | Optional with capability signaling |
| 28. NR_redcap | 28-2 | Number of UE Rx branches and DL MIMO layers for RedCap UE | 1. For a RedCap UE with 1 Rx branch, 1 DL MIMO layer is supported. 2. For a RedCap UE with 2 Rx branches, 2 DL MIMO layers are supported. | 28-1 | Yes |  | Impact on UE complexity | [Per band] | No | [No] |  | For UE capability signalling, the number of Rx branches for RedCap is implicitly indicated by the corresponding capability parameter maxNumberMIMO-LayersPDSCH in the existing UE capability framework. Detailed signalling is up to RAN2. | Optional with capability signaling |
| 28. NR_redcap | 28-3 | Half-duplex FDD operation for RedCap UE | 1. Half-duplex FDD operation (instead of full-duplex FDD operation) for RedCap UE | 28-1 | Yes |  | Impact on UE complexity | [Per band] | FDD only | FR1 only |  |  | Optional with capability signaling |
| 28. NR_redcap | 28-4 | DL 256QAM support for RedCap UE | 1. Support of 256QAM for PDSCH for RedCap UE 2. Support of 256QAM MCS table (Table 5.1.3.1-2 in TS 38.214) for PDSCH for RedCap UE 3. Support of CQI table 2 (Table 5.2.2.1-3 in TS 38.214) for RedCap UE | 28-1 | Yes |  | Impacts on UE complexity and DL link performance at high SNR | [Per band] | No | [No] |  | For RedCap UEs, the 256QAM MCS table for PDSCH and CQI table 2 are only supported if the UE supports 256QAM for PDSCH. | Optional with capability signaling |
| 28. NR_redcap | 28-5 | UL 256QAM support for RedCap UE | 1. Support of 256QAM for PUSCH for RedCap UE 2. Support of 256QAM MCS table (Table 5.1.3.1-2 in TS 38.214) for PUSCH for RedCap UE | 28-1 | Yes |  | Impact on UE complexity and UL link performance at high SNR | [Per band] | No | [No] |  | For RedCap UEs, the 256QAM MCS table for PUSCH is only supported if the UE supports 256QAM for PUSCH. | Optional with capability signaling |