3GPP TSG RAN WG1 #106bis-e		R1-2110070

e-Meeting, October 11th – 19th, 2021

Agenda Item:	8.17.7

Source:	Apple Inc.

Title:	Views on UE features for Rel-17 UE power saving

Document for:	Discussion/Decision

# Introduction

An initial UE feature list is provided in R1-2108679. We provide our views and proposals for R17 UE power saving features in this contribution.

# R17 UE Power Saving Features

The following UE features have been proposed for R17 UE power saving.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | 29-1 | Paging enhancement | 1. Support paging early indication 2. Support UE subgroup indication |  | N |  | High idle/inactive mode UE power consumption if NR SA networks | Per UE | N | N | N |  | Optional without capability signalling |
| 29. NR_UE_pow_sav_enh | 29-2 | TRS resources for idle/inactive UEs | TRS occassions for idle/inactive UEs 1. Support reading TRS configuration from SIB 2. Support receving L1 indication for TRS availability |  | N |  | Lose of power saving gain on AGC, time/frequency tracking in idle/inactive mode | Per UE | N | N | N |  | Optional without capability signalling |
| 29. NR_UE_pow_sav_enh | 29-3 | PDCCH monitoring adaptation within an active BWP | - Support of PDCCH monitoring adaptation behaviour 1/1A - Support of PDCCH monitoring adaptation behaviour 2/2A/[2B] |  | Y |  | PDCCH monitoring adaptation within an active BWP is not supported | Per UE | N | N | N | FFS: Support of PDCCH monitoring adaptation behaviour 1/1A/2/2A/2B | Optional |

For paging enhancements, there are two features: paging early indication and UE subgroup indication. These two are different functions, we should define two separate FGs for paging early indication and UE subgroup indication.

Regarding whether some of the FGs need to be reported to the network, we think the FGs for paging enhancements should be reported to the gNB.

- For paging early indication, it is not absolutely necessary for the UE to report. However, if it is not reported, the gNB would always need to transmit PEI, even if the UE does not support it. So it is beneficial for the UE to report the capability, so that the gNB can decide whether to transmit PEI based on the capability of UEs to be paged.

- For subgroup indication, the UE capability needs to be reported to the gNB so that the gNB can forward it to the core network. When there is paging for a UE, the core network knows whether the UE supports subgroup indication, and if yes, the core network can send the corresponding information to the gNB so that the gNB can deliver subgroup indication properly.

- In addition, for subgroup indication, it may be better to leave it to RAN2 to decide how to define the FG. According to the RAN2 LS R2-2108917, CN-based subgrouping and UEID-based subgrouping have been agreed. It is better to leave it to RAN2 to decide whether separate reporting is needed for the two approaches.

For FG 29-3, there are 3 types of behaviours:

- Behaviour 1/1A (PDCCH skipping)

- Behaviour 2/2A (SSSG switching)

- Behaviour 2B (using empty SSG to emulate PDCCH skipping)

These 3 types of behaviours are independent, and the support should be separately indicated.

# Conclusion

Based on the discussions, we propose the following changes to R17 UE power saving features.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | 29-1a | Paging early indication | 1. Support paging early indication |  | Y |  | High idle/inactive mode UE power consumption in NR SA networks | Per UE | N | N | N |  | Optional with capability signalling |
| [29. NR_UE_pow_sav_enh] | [29-1b] | [Paging subgroup indication] | [1. Support UE subgroup indication] |  | [Y] |  | [Higher power consumption due to false paging wakeup for IDLE/INACTIVE UEs] | [Per UE] | [N] | [N] | [N] |  | [Optional with capability signalling] |
| 29. NR_UE_pow_sav_enh | 29-2 | TRS resources for idle/inactive UEs | TRS occasions for idle/inactive UEs 1. Support receiving TRS configuration being indicated to idle/inactive UEs from a new SIB 2. Support receiving L1 indication for TRS availability |  | N |  | Lost of power saving gain on AGC, time/frequency tracking in idle/inactive mode | Per UE | N | N | N |  | Optional without capability signalling |
| 29. NR_UE_pow_sav_enh | 29-3 | PDCCH monitoring adaptation within an active BWP | - Support of PDCCH monitoring adaptation behaviour 1/1A - Support of PDCCH monitoring adaptation behaviour 2/2A - [Support of PDCCH monitoring adaptation behaviour 2B] |  | Y |  | PDCCH monitoring adaptation within an active BWP is not supported | Per UE | N | N | N | Component 1: {support, not support} Component 2: {support, not support} [Component 3: {support, not support}] FFS: Support of PDCCH monitoring adaptation behaviour 1/1A/2/2A/2B | Optional with capability signalling |

# Reference