3GPP TSG RAN WG1 #106bis-e	   	 	  	                	      R1- 2110079

e-Meeting, Oct 11th – 19th, 2021

Agenda item:	8.1.2.2

Source:	LG Electronics

Title:	Enhancements on Multi-TRP inter-cell operation

Document for:	Discussion and Decision

Introduction

In this contribution, we discuss potential QCL/TCI-related enhancement to enable inter-cell multi-TRP operations, assuming multi-DCI based multi-PDSCH reception.

Discussion

The maximum number of additional RRC-configured PCI is still under discussion. In our view, 3 or 7 can be supported other than 1. Regarding the detail reporting mechanism, two alternatives were discussed in the last meeting and we prefer to report single value, rather than optimizing X depending on SSB configuration.

Proposal #1: Support X = {1,3,7} and report single value as UE capability for any possible SSB time domain position and periodicity.

Another remaining FFS point is about association between PCI and CORESETPoolIndex when switching between intra-cell mTRP and inter-cell mTRP. In our view, this is not an urgent issue since the scope of this agenda is to support inter-cell mDCI based mTRP operation. Also, this switching is already supported with RRC reconfiguration. So, we prefer to finalize inter-cell MTRP operation first. If dynamic switching needs to be supported, one approach is to increase the number of CORESET pools configured by RRC, e.g. 4, and activate up to two CORESET pools by MAC-CE. With this MAC-CE based CORESET pool activation, inter-cell and intra-cell switching can be done and we can keep one to one mapping between TRP and CORESET pool.

Proposal #2: Deprioritize dynamic switching enhancement between intra-cell mTRP and inter-cell mTRP.

In the last meeting, it was briefly discussed whether PDSCH/PDCCH from one MTRP cell is rate matched around the other MTRP cell SSB. Given that the interference from MTRP non-serving cell PDSCH/PDCCH to serving cell SSB is much stronger than non-serving cell PDSCH/PDCCH which does not participate in MTRP operation, non-serving cell PDSCH/PDCCH should be rate matched around serving cell SSB. Also, since UE needs to derive QCL parameters of MTRP, serving cell PDSCH/PDCCH should be rate matched around the non-serving cell SSB.

Proposal #3: PDSCH/PDCCH from serving cell should be rate matched around non-serving cell SSB and PDSCH /PDCCH from non-serving cell should be rate matched around serving cell SSB.

Conclusion

In this contribution, we discuss potential QCL/TCI-related enhancement to enable inter-cell multi-TRP operations, assuming multi-DCI based multi-PDSCH reception and our proposal is as follow:

Proposal #1: Support X = {1,3,7} and report single value as UE capability for any possible SSB time domain position and periodicity.

Proposal #2: Deprioritize dynamic switching enhancement between intra-cell mTRP and inter-cell mTRP.

Proposal #3: PDSCH /PDCCH from serving cell should be rate matched around non-serving cell SSB and PDSCH /PDCCH from non-serving cell should be rate matched around serving cell SSB.