3GPP TSG RAN WG1 #106bis-e		                     R1- 2110085

e-Meeting, October 11th – 19th, 2021

Agenda item:	8.4.2

Source:	LG Electronics

Title:	Discussions on UL time and frequency synchronization enhancements in NTN

Document for:	   Discussion and Decision

- Introduction

In the RAN1 #106-e meeting [1], the following agreements and conclusions were made for UL time and frequency synchronization in NTN.

Agreement: - A validity duration configured by the network for satellite ephemeris data indicates the maximum time during which the UE can apply the satellite ephemeris without having acquired new satellite ephemeris. - FFS: Associated UE behaviour if the UE does not read the ephemeris within the validity duration. - FFS: Whether the same validity duration can be applied for Common TA. Agreement: Confirm the working assumption on non-extension of TAC 12-bit field in msg2 (or msgB) and that the UE follows the requirements on UL time pre-compensation for Msg1/MsgA transmission as defined by RAN4. Agreement: In NTN, to avoid that the UE over pre-compensates its TA during RACH procedure, down-select one option from below: - Option 1: PRACH transmission is delayed by minCP2,GP2 - Option 2: TA margin can be considered, and it is explicitly indicated to the UE - Option 3: TA margin can be considered, and it is included within the Common TA - Option 4: UE handles it via implementation Agreement: In NR NTN, NTA update based on TA Command field in msg2/msgB and MAC CE TA command is used for UL timing alignment correction as follows: - When TAC (TA) in msg2/msgB is received, UE receives the first adjustment and NTA is updated as follows: NTA=NTA_old+TA. 16.642μ , FFS: the value of NTA_old, where, TA is the TAC field in msg2/msgB - When TACs (TA) provided within the MAC CE is received, NTA is updated as follows: NTA_new=NTA_old+TA-31.16.642μ , Where, TA is the TAC field receivd in MAC CE command Agreement: Serving satellite ephemeris Epoch time is implicitly known as a reference time defined by the starting time of a DL slot and/or frame. - FFS: Whether this starting time is given by predefined rule or it is indicated by the Network Conclusion: Indication of common post-compensation frequency offset for Uplink is not needed. Working assumption: Common TA may include parameter(s) indicating timing drift. - The UE will apply common TA according to the parameters provided by the network (if any). No offset between the common TA according to the parameters provided by the network and the actual feeder link RTT is considered when defining UE UL timing error requirements.

In this contribution, we discuss on the enhancement on UL time and frequency synchronization in NTN.

- Discussion

Enhancement on UL time synchronization

2.1.1. Regarding common TA tracking

In RAN1 #104b-e meeting, the equation of timing advance for NR NTN UE, which includes common TA and UE specific TA was agreed. The remaining FFS point is the signaling details of common TA. Also, in the last meeting, working assumptions that common TA may include parameters indicating timing drift was made. Basically, it is reasonable that the common TA is provided via higher layer signaling (e.g., SIB1 and/or dedicated RRC signaling). If only common TA value is provided by the network, the UE may not be able to apply appropriate common TA value as time changes. So, the network needs to provide information on reference time corresponding to the common TA.

First of all, in a simple way, the network can explicitly indicate the reference time corresponding to the common TA via the same higher layer singling. In other ways, if the common TA is provided by periodically, the reference time of common TA can be predefined. For example, if the common TA is provided via SIB1, the reference time corresponding to the common TA can be defined as the starting slot timing of SIB1 reconfiguration period. After then, the UE can accurately predict the current common TA using both common TA value and reference time.

Moreover, it can be considered that series of common TA value would be provided by the network in order to apply proper common TA according to time changes at UE side. Also, it could be considered that the network can provide the series of common TA as much as a predefined period (e.g., SIB reconfiguration period). Alternatively, it can be also considered that common TA drift rate can be provided in common TA parameters. If it is provided, the UE can also apply appropriate common TA due to time changes.

Regarding the series of common TA parameter, in RAN1 #105-e meeting, [2] suggested that the series of common TA parameter could be considered in order to reduce the frequent signaling from gNB. Also, according to comments from several companies during the e-mail discussion in the last meeting, we think it is necessary to discuss because it can improve the accuracy and increase validity time duration (or indication period).

Proposal 1. Support additional signaling by the network in order to apply proper common TA according to time changes at UE side. Potential solutions can include

- Alt 1) providing the reference time corresponding to common TA

- Alt 2) providing series of common TA and common TA drift rate

2.1.2. Regarding TA margin for the TA estimation uncertainty

When the UE estimates the UE specific TA by autonomous manner, an inter-symbol-interference (ISI) may affect to the OFDM symbol of previous slot (or to the PRACH preamble of the previous RO) if the estimation error occurs (e.g., over estimation). Therefore, in order to reduce the effect of this interference, the TA margin can be considered.

In the RAN1 #106-e meeting, the following agreement was made for TA margin.

Agreement: In NTN, to avoid that the UE over pre-compensates its TA during RACH procedure, down-select one option from below: - Option 1: PRACH transmission is delayed by minCP2,GP2 - Option 2: TA margin can be considered, and it is explicitly indicated to the UE - Option 3: TA margin can be considered, and it is included within the Common TA - Option 4: UE handles it via implementation

Among the above options, it is not desirable to support option 1. The reason is that the CP and the GP are different for each PRACH format, and even, PRACH format A1, A2, and A3 do not have GP. Hence, if gNB needs to use the TA margin with option 1, the PRACH format A1, A2, and A3 cannot be configured to UEs. In addition, the GP is not defined in current specification.

As a result, we prefer the option 2 in which gNB can explicitly indicate the TA margin. For example, gNB may configure appropriate TA margin in the pre-defined TA margin set. Alternatively, the option 3 that TA margin is handled by gNB implementation is also fine.

Proposal 2. Support option 2 in the agreement in RAN1 #106-e regarding TA margin.

- Option 2: TA margin can be considered, and it is explicitly indicated to the UE

2.1.3. Regarding report of UE specific TA in initial access procedure

When the UE estimates the TA by autonomous manner, gNB does not know the TA value estimated by the UE. For example, if gNB does not know the UE specific TA, gNB may provide a maximum K-offset value that can be used for the transmission timing of Msg. 3 PUSCH, or for the transmission timing of ACK for Msg. 4 PDSCH. Consequently, the maximum K-offset value increases the time required for RACH procedure and the time required for the UE to access the serving cell.

Regarding this issue, it was discussed that the UE specific TA can be reported to gNB via Msg. 3 PUSCH. If the Msg. 3 PUSCH is received at gNB properly, gNB can determine suitable K-offset value, and if this value is applicable for indication of transmission timing of ACK for Msg. 4 PDSCH, the time required for RACH procedure can be reduced. However, if the Msg. 3 PUSCH is not received at gNB properly, gNB can use maximum K-offset value for the re-transmission for Msg. 3 PUSCH. In addition, this TA reporting approach can increase the signaling overhead at UE side.

On the other approach, if the UE specific TA can be reported for gNB during transmission/reception of Msg. 1 preamble and if suitable K-offset value determined based on TA report is applied for transmission timing of Msg. 3 PUSCH, and thus the time required for RACH procedure can be further reduced. As one solution of this approach, it can be considered that the different UE specific TA (or the range of UE specific TA) can be mapped to different ROs (or RO groups). Subsequently, the UE can select the RO if the TA (or the range of TA) mapped to the RO including the TA estimated by the UE. Then, the UE can transmit a PRACH preamble in the selected RO. After that, gNB can identify the TA estimated by UE based on the TA (or the range of TA) of the RO where the PRACH preamble was transmitted.

Proposal 3. Support implicit reporting of UE specific TA estimated by the UE.

- The different TA (or the range of TA) can be mapped to different ROs (or RO groups).

Details on signaling, reporting and validity duration

2.2.1. Regarding signaling and reporting

In cases when the additional information (e.g., common TA, satellite ephemeris, etc.) is provided by gNB, it is necessary to be discussed which DL signals and/or channels will provide them. First of all, in case when the UE is in RRC_IDLE and/or RRC_INACTIVE states, it is reasonable to provide the information via semi-static signaling (e.g., SIB).

However, in case when the UE is in RRC_CONNECTED states, it can be considered that the information is provided by dynamic signaling (e.g., GC PDCCH). Alternatively, it can also be considered that the initial information is provided by semi-static signaling (e.g., SIB or RRC signaling) and after that, the updated information is provided by dynamic signaling (e.g., GC PDCCH).

Proposal 4.

- At least for the case when the UE is in RRC_IDLE and/or RRC_INACTIVE states, it is reasonable to provide the additional information via semi-static signaling.

- In case when the UE is in RRC_CONNECTED states, it can be considered that the information is provided by dynamic signaling.

Moreover, in case when the UE is in RRC_CONNECTED states, the UE specific TA (or K-offset) is necessary to be updated since the UE position and/or satellite ephemeris can be changed. Generally, both the UE initiated update procedures and the network initiated update procedures could be considered. For the first example, the UE can trigger the update of UE specific TA when it deviates from a predefined metric (e.g., UE specific TA threshold), it means that the UE position and/or satellite ephemeris may be changed. For another example, the network can provide the UE specific TA update period, or can indicate to update the UE specific TA via DCI (e.g., DCI for PDCCH order RACH procedure).

Also, it is desirable to report the updated UE specific TA to the network, in order to calculate and/or estimate the appropriate K-offset at network side. As an example of how to report, it can be defined to report every time when the UE updates the UE specific TA, or the network can indicate the reporting periodicity for UE specific TA. Also, the network can trigger to report the UE specific TA via DCI (e.g., DCI for PDCCH order RACH procedure).

Proposal 5. RAN1 should discuss how to report the UE specific TA in case when the NTN UE is in RRC_CONNECTED states.

2.2.2. Regarding validity duration

In the RAN1 #106-e meeting, the following agreement was made regarding validity duration.

Agreement: - A validity duration configured by the network for satellite ephemeris data indicates the maximum time during which the UE can apply the satellite ephemeris without having acquired new satellite ephemeris. - FFS: Associated UE behaviour if the UE does not read the ephemeris within the validity duration. - FFS: Whether the same validity duration can be applied for Common TA.

According to FFS on the first sub-bullet, it can be further discussed whether to support the associated UE behavior in case when the UE does not read the satellite ephemeris within the validity duration. Basically, it is desirable for the UE to read and receive the satellite ephemeris within the validity duration. If the UE succeeds to decode the satellite ephemeris within the validity duration, it is desirable to set the validity duration to re-start.

However, if the UE fails to read the satellite ephemeris during the validity duration (for some reason) and the validity duration is expired as it is, the UE can determine that the existing satellite ephemeris is no longer valid and stop UE specific TA updating/reporting. Thereafter, the UE may receive new satellite ephemeris through the SIB. If the UE succeeds to receive new satellite ephemeris, the UE may perform UE specific TA updating/reporting again, and in addition, the validity duration can be re-started.

Observation 1.

- It is desirable for the UE to read and receive the satellite ephemeris within the validity duration.

- If the UE fails to read the satellite ephemeris during the validity duration, the UE can determine that the existing satellite ephemeris is no longer valid and stop UE specific TA updating/reporting.

According to FFS on the second main-bullet, it is necessary to determine whether to use the same validity duration for both common TA and satellite ephemeris. We think it is desirable that the validity duration for common TA is provided separately from the validity duration for satellite ephemeris. The reason is that the appropriate update duration for common TA and the appropriate update duration for satellite ephemeris may not be the same as the purpose of signaling of common TA and satellite ephemeris is different.

Moreover, if the same validity duration is considered for both common TA and satellite ephemeris, gNB has no choice but to determine the validity duration based on the specific parameter that need to be updated more frequently, and accordingly, the other parameter will be updated more often than necessary.

Proposal 6. Support independent validity durations for common TA and satellite ephemeris in Rel-17 NTN.

Combination of open and closed loop TA control

Before discussing the details for combination of open loop TA control and closed loop TA control, it is necessary to discuss whether open loop control and/or closed loop control will be applied to which TA, i.e., UE specific TA and/or common TA.

First of all, it could be considered to determine the TA control methods depending on the type of TA. For example, the closed loop TA control may be appropriate to common TA (i.e., NTA,common), because the common TA can be updated based on the parameter signalled from network (e.g., (series of) common TA (and/or common TA drift rate)). Here, the parameters may be indicated via SIB when the NTN UE is in RRC idle/inactive state, or they may be indicated via SIB and/or dedicated RRC signalling when the NTN UE is in RRC connected state.

For another example, regarding UE specific TA (i.e., NTA,UE-specific), although the satellite ephemeris are provided from the network, but the open loop TA control may be appropriate to UE specific TA, because it can be updated based on both UE position and satellite ephemeris. Here, similarly as above, the satellite ephemeris may be indicated via SIB when the NTN UE is in RRC idle/inactive state, or they may be indicated via SIB and/or dedicated RRC signalling when the NTN UE is in RRC connected state.

Proposal 7: Define the appropriate TA control method for each TA as follows:

- Closed loop TA control is applied to common TA (i.e., NTA,common)

- Open loop TA control is applied to UE specific TA (i.e., NTA,UE-specific)

If the closed loop TA control is applied to common TA, and if the open loop TA control is applied to UE specific TA, each TA can be updated by network and/or UE through independent loops. Here, there may be no problem if open loop TA control and closed loop TA control only update the corresponding TA. Therefore, it is beneficial to support independent TA update for each TA using independent TA control methods.

Proposal 8: Support independent TA update for each TA using independent TA control methods.

However, if the network is allowed to update the total TA using closed loop TA control, unintended conflicts can occur between open loop TA control and closed loop TA control. In such case, the following ways can be considered to solve the confliction.

As a first way, it may be considered that the network indicates the timing window (e.g., period and/or duration, etc.) for updating of corresponding TA. At this point, the timing window may be TA specific, and it should be configured independently. Alternatively, after the UE is indicated to update the TA by closed loop TA control (e.g., common TA and/or TAC), the open loop TA control (e.g., UE specific TA) can be disabled during predefined/configured time duration.

As another way, priority rule can be considered for each TA control method. For example, regarding the priority between open loop TA control (e.g., UE specific TA) and closed loop TA control (e.g., common TA and/or TAC), the closed loop TA control may have a higher priority. Because, basically, it is desirable to follow the indication of network first.

Observation 2: It is desirable to set open loop TA control and closed loop TA control as non-conflicting. If not, the conflicts between both TA control methods may occur, and additional UE behaviour might be necessary.

As mentioned before, the UE specific TA can be autonomously updated by UE. So, we think it is reasonable to leave over UE implementation how to update the UE specific TA. However, it is necessary to discuss when to apply the updated UE specific TA for UL signal/channel transmission. For example, in case when the UE is about to transmit new UL signal/channel, the updated UE specific TA should be applied for total TA. In other words, the latest updated UE specific TA may be applied from the first slot of new UL signal/channel transmission. On the other hand, in case when the UE is about to transmit new UL signal/channel, if the UE specific TA is not required to update, the latest applied UE specific TA can be used for new UL signal/channel transmission.

Proposal 9. RAN1 should discuss when to apply the updated UE specific TA for UL signal/channel transmission.

Satellite ephemeris format

In RAN1 #105-e meeting, it was agreed that two satellite ephemeris formats are supported in specifications. In this agreement, two satellite ephemeris formats are considered as follows:

- Format 1: Ephemeris format based on satellite state vectors.

- Format 2: Ephemeris format based on orbital elements.

Agreed detail parameters regarding Format 1 are 3-demensional position and velocity vectors, i.e., (x, y, z, vx, vy, vz). In this option, the network needs to provide additional epoch time (i.e., reference time) information along with the above six elements. It may be desirable to provide these information frequently for accurate satellite ephemeris prediction.

Another agreed detail parameters regarding Format 2 are Keplerian Orbit Elements (i.e., α, e, ω, Ω, i, M0). In this option, some elements (e.g., α, e, ω, Ω, i) are semi-static (or time invariant) and some elements are not. So, the network can semi-statically configure element such as {α, e, ω, Ω, i}, and provide time variant element such as {M0} periodically.

Even though two satellite ephemeris formats are supported in specifications, this agreement does not enforce that they are mandatory features for UE implementation. Therefore, to reduce the UE implementation complexity, we prefer that one of these formats can be mandatory feature and the other can be optional feature, and this issue can be further discussed in the UE capability discussion.

Observation 3. Regarding two satellite ephemeris formats, to reduce the UE implementation complexity, it is preferred that one of these formats can be a mandatory feature and the other can be optional feature.

In the RAN1 #106-e meeting, a validity duration for satellite ephemeris information was supported. Moreover, since two satellite ephemeris formats are supported in specifications, it can be considered that gNB can provide both formats at the same time. In this situation, it is reasonable to consider to introduce independent validity durations for each format. This is because, the appropriate propagator model may be different based on each format. Also, since each format may have different accuracy for satellite ephemeris information, so independent values may be required for each validity duration. Therefore, we prefer to support independent validity durations for two different satellite ephemeris formats.

Proposal 10. Support independent validity durations for two different satellite ephemeris formats in Rel-17 NTN.

- Conclusion

In this contribution, we discussed on the enhancement on UL time and frequency synchronization in NTN. Based on the above discussion, we have following observations and proposals:

Proposal 1. Support additional signaling by the network in order to apply proper common TA according to time changes at UE side. Potential solutions can include

- Alt 1) providing the reference time corresponding to common TA

- Alt 2) providing series of common TA and common TA drift rate

Proposal 2. Support option 2 in the agreement in RAN1 #106-e regarding TA margin.

- Option 2: TA margin can be considered, and it is explicitly indicated to the UE

Proposal 3. Support implicit reporting of UE specific TA estimated by the UE.

- The different TA (or the range of TA) can be mapped to different ROs (or RO groups).

Proposal 4.

- At least for the case when the UE is in RRC_IDLE and/or RRC_INACTIVE states, it is reasonable to provide the additional information via semi-static signaling.

- In case when the UE is in RRC_CONNECTED states, it can be considered that the information is provided by dynamic signaling.

Proposal 5. RAN1 should discuss how to report the UE specific TA in case when the NTN UE is in RRC_CONNECTED states.

Observation 1.

- It is desirable for the UE to read and receive the satellite ephemeris within the validity duration.

- If the UE fails to read the satellite ephemeris during the validity duration, the UE can determine that the existing satellite ephemeris is no longer valid and stop UE specific TA updating/reporting.

Proposal 6. Support independent validity durations for common TA and satellite ephemeris in Rel-17 NTN.

Proposal 7: Define the appropriate TA control method for each TA as follows:

- Closed loop TA control is applied to common TA (i.e., NTA,common)

- Open loop TA control is applied to UE specific TA (i.e., NTA,UE-specific)

Proposal 8: Support independent TA update for each TA using independent TA control methods.

Observation 2: It is desirable to set open loop TA control and closed loop TA control as non-conflicting. If not, the conflicts between both TA control methods may occur, and additional UE behaviour might be necessary.

Proposal 9. RAN1 should discuss when to apply the updated UE specific TA for UL signal/channel transmission.

Observation 3. Regarding two satellite ephemeris formats, to reduce the UE implementation complexity, it is preferred that one of these formats can be a mandatory feature and the other can be optional feature.

Proposal 10. Support independent validity durations for two different satellite ephemeris formats in Rel-17 NTN.

Reference

[1] RAN1 #106-e Chairman’s note

[2] ZTE, R1-2105190, “Discussion on UL synchronization for NR-NTN,” RAN1#105-e, May 2021.