3GPP TSG RAN WG1 #106bis-e    			       	 R1-2110090

e-Meeting, Oct 11th – Oct 19th, 2021

Agenda Item:	8.5.3

Source: 	LG Electronics

Title: 	Discussion on accuracy improvement for DL-AoD positioning

Document for:   Discussion and decision

Introduction

This contribution discusses Rel-17 NR positioning enhancements specifically in the DL-AoD (Angle of Departure) based positioning solution.

Downlink positioning technique enhancement

Reporting of first arrival path (FAP)

Agreement: For definition of the path PRS RSRP, consider the following options until RAN1#106b-e: - Option 1: the measured path PRS RSRP correspond to the power of the channel impulse response, at a certain path delay, over which the DL PRS is received. - Option 2: the path PRS RSRP correspond to the accumulated power of the channel impulse response over which the DL PRS is received, over a time duration corresponding to the given path delay - FFS: whether/how is the window conveyed to the UE (i.e., fixed in specification or configured in measurement request or determined by the UE) - FFS on relationship with the UE DL PRS measurement bandwidth. - FFS: normalization of the path RSRP measurement with DL PRS RSRP (i.e. RSRP for all path as defined in Rel-16) could be included in the measurement definition. - FFS: Further details of the definition, e.g. definition of the certain path delay - Up to RAN4 to define any test/requirement for the measurement.

To improve the accuracy of positioning measurement, path-specific reporting has been introduced. The channel parameters to be estimated are the CIR and the FAP. The estimated CIR can be composed of several taps. In general, each tap represents a transmission route and the corresponding propagation delay. If the propagation delay between two adjacent taps is not crucial factor for accuracy performance, we don’t need to report all of them. In this respective, introducing the window to calculate path PRS RSRP considering multiple taps within the short time duration is helpful and necessary.

Proposal 1:

- RAN1 should adopt a window to calculate path PRS RSRP for the case that the propagation delay between two adjacent taps within a very short time duration is not crucial factor for accuracy performance.

In terms of normalization of power, we think there might be ambiguity point based on the fact that there are many methods for estimating the CIR for OFDM system. For example, the comb-type is generally used for resource mapping for reference signal such as PRS. In that case, UE can apply power normalization to either all taps or partial taps. We think how to normalize the power seems to be discussed.

Adjacent beam reporting

Regarding adjacent beam reporting, the followings were agreed in the previous meeting [2].

Agreement: For UE-assisted DL-AOD positioning method, select one or more of the following to enhance the signaling to the UE for the purpose of PRS resource(s) measurement and reporting: - Option 1: the LMF explicitly identify adjacent beams in the assistance data (AD) - Option 2: the LMF send the beam information in the AD with an order of priority of PRS resources. - Option 3: the LMF includes boresight direction information for each PRS resource in the assistance data. - Option 4: the LMF send the beam information in the AD with indicated subset of PRS resources. - FFS: Detailed signaling and procedure - FFS: How to define adjacent beams

If LMF knows about Tx beam information at gNB, LMF can have a knowledge on which adjacent beams are allocated with PRS resources. That means LMF can make a list of the subset of PRS resources that are related with adjacent beams. If LMF provides the information to UE, UE can report measurement results about the PRS resource corresponding to the best RSRP and measurement results related to PRS resources that are included in the subset. From this perspective, option #4 has the advantage in terms of signaling overhead because the information is provided by subset, not by resource. In the case of other options, the information of adjacent beams needs to be delivered directly or indirectly per resource

Observation 1:

- For an adjacent beam reporting, using the subset of PRS resources (option 4) has an advantage in terms of signaling overhead reduction because the information is provided by subset, not by resource.

Proposal 2:

- For UE-assisted DL-AOD positioning method, select option 4 (‘the LMF send the beam information in the AD with indicated subset of PRS resources’)

Rx beam index information

In the DL-AoD positioning technique, LMF would estimate UE’s location by using information on PRS resource(s) and the corresponding RSRP measurement values for one or multiple TRP(s). Conceptually, the UE’s relative direction from a TRP could be found by one or two PRS resource index(es) which shows the maximum RSRP among a set of PRS resources. In case of the DL-AoD positioning, it would be reasonable to use a fixed reception beam across multiples PRS resources to estimate variation of RSRP measurements for the different PRS resources. Thus, for the effective location estimation, it was introduced that the UE can report its reception beam index for two or more DL PRS measurements if the PRS measurements were made with the same reception beam where the index of reception beam is up to 8. In the current phase, however, it is unclear how the LMF utilizes the reception beam index to find UE’s location and the location server cannot know the reception beam direction corresponding to the RX beam index, even if the UE reports the reception beam index. However, this feature needs to be further discussed since the reception beam highly affects RSRP measurement values.

Regarding the technical feature on when the reception beam index reporting can be used, the following cases can be considered. 1) The UE can use the same reception beam for the multiple PRS resources unless those PRS resources have a different source of QCL type-D RS. 2) The UE uses the same reception beam for the multiple PRS resources even if those PRS resources have a different source of QCL type-D RS regardless of the QCL type-D configuration. In our understanding, the second case is also feasible by implementation. However, even if the reception beam selection is up to the UE, further restriction might be required so that the UE uses a proper reception beam to ensure the positioning accuracy or to avoid worst case of reception beam selection. For example, the UE's fixed reception beam for the PRS measurements with regard to a specific TRP should be the best reception beam in terms of the RSRP.

In addition, there is another issue on how to interpret the same reception beam index reported in the different reporting time instances. Although the UE reports the same RX beam index at two different reporting time instances, the same reception beam index does not guarantee the same beam direction due to the variation of UE’s position and/or rotation. In Rel-17 NR positioning, it is necessary to discuss how to utilize the reception beam index to improve the location estimation, such as finding UE’s location when the UE is located between the transmission beams. Also, we need discussions on how to inform LMF of the reception beam information corresponding to the reported reception beam index, if needed.

Proposal 3:

- A further restriction would be required so that the UE uses a reception beam to avoid worst case of the reception beam selection, even if the UE can ignore QCL type-D configuration of the PRS resources to use a fixed reception beam for DL-AoD technique.

Proposal 4:

- Need discussions on how to utilize the reception beam index for the accuracy improvements of DL-AoD based positioning, such as finding UE’s location when the UE is located between the transmission beams.

Two-state PRS beam sweeping

Considering beam resolution, the followings are agreed in the last meeting [2].

Agreement: - For both UE-based and UE-assisted DL methods, at least for two-stage PRS beam sweeping, study further at least the following: - Enhancements in the association between resources belonging to two DL PRS resource sets of the same TRP - Companies are encouraged to evaluate whether other potential enhancements in this subagenda or other subagendas (e.g. additional beam information, on-demand PRS framework) could be used to enable this feature (potentially by implementation). - Note: Two-stage PRS beam sweeping corresponds to different DL PRS resource sets

For the accuracy of the DL-AoD positioning technique, it would be necessary to discuss on how to estimate the UE’s location when the UE is located between two different transmission beams. As a simple example, a UE is located in a certain region between two transmission beams represented by PRS resource #1 and PRS resource #2, respectively, depicted in Figure 1. In this example, it can be envisioned that, by using the reception beam direction oriented to PRS resource#2, the RSRP measurement of the PRS resource #2 could be the maximum among overall PRS resources transmitted from the TRP. Based on the reported RSRP measurements, the LMF would estimate UE’s location on the direction of transmission beam of PRS resource #2.

Figure 1 An illustrative example of the UE’s location between two transmission beams.

Thus, Rel-17 NR positioning enhancements should discuss how to address the accuracy degradation according to the resolution problem. As a possible solution, we could consider the following two steps. Firstly, each TRP can transmit the PRS resources with the relatively wide beams as depicted on the left-hand of Figure 2 and the UE obtains PRS RSRP measurements with a fixed reception beam. Then, the LMF may roughly estimate that the UE is located in a certain area/direction. Secondly, after finding the rough location of the UE, the TRP can transmit the PRS resources with narrow beams where each beam covers relatively small area, and the UE needs to measure each PRS resource with multiple reception beams in order to find the best Rx/Tx beam pair. The best beam pair would be a combination of reception and transmission beam maximizing RSRP or maximizing the received signal power of the first arrival signal path.

In consideration of the current TS 37.355, at least, it is necessary to support that the UE needs to report the maximum RSRP for each PRS resource considering multiple reception beams.

Figure 2 An illustrative example of the DL-AoD positioning based on two-step method.

In terms of utilization of PRS resources, each PRS resource set can be used for each steps. In case of the first PRS resource set, it can be composed of multiple PRS resources and they are associated with wide beams. And then, the multiple PRS resources that are in the second PRS resource set can be associated with narrow beams. LMF can configure associated PRS resources based on the measurement report in the first step.

Proposal 5:

- Regarding 2-stage PRS beam sweeping, RAN1 should consider the following procedure for 2-stage beam reporting:

  - In case of the first PRS resource set, it can be composed of multiple PRS resources and they are associated with wide beams.

  - And then, the multiple PRS resources that are in the second PRS resource set can be associated with narrow beams. LMF can configure associated PRS resources based on the measurement report in the first step.

In addition, we also think that RAN1 needs to consider appropriate measurement report for 2-stage beam sweeping. Considering the intention of 2-stage beam sweeping, since beam width used for 2nd state is narrower than beam width used for 1st state, we think that the resolution and reporting range for measured quantity value could be different. For example, 1dB that is already used for resolution is applied for UL SRS RSRP reporting in 1st stage and the smaller value of resolution like 0.5dB can be used in 2nd stage. We think that approaching measurement reporting in this way is help for LMF to calculate UE’s location more precisely. From this point of view, RAN1 needs to consider applying different resolution and range for measured quantity value in each stage respectively.

Observation 2:

- Considering the intention of 2-stage beam sweeping, the resolution and reporting range for measured quantity value could be different since beam width used for 2nd state is narrower than beam width used for 1st state.

Proposal 6:

- RAN1 needs to consider applying different resolution and range for measured quantity value in each stage respectively.

Conclusion

In this contribution, we have discussed potential enhancements for DL-AOD positioning in Rel-17, and our proposals are summarized below.

Reporting of first arrival path

Proposal 1:

- RAN1 should adopt a window to calculate path PRS RSRP for the case that the propagation delay between two adjacent taps within a very short time duration is not crucial factor for accuracy performance.

Adjacent beam reporting

Observation 1:

- For an adjacent beam reporting, using the subset of PRS resources (option 4) has an advantage in terms of signaling overhead reduction because the information is provided by subset, not by resource.

Proposal 2:

- For UE-assisted DL-AOD positioning method, select option 4 (‘the LMF send the beam information in the AD with indicated subset of PRS resources’)

Rx beam index information

Proposal 3:

- A further restriction would be required so that the UE uses a reception beam to avoid worst case of the reception beam selection, even if the UE can ignore QCL type-D configuration of the PRS resources to use a fixed reception beam for DL-AoD technique.

Proposal 4:

- Need discussions on how to utilize the reception beam index for the accuracy improvements of DL-AoD based positioning, such as finding UE’s location when the UE is located between the transmission beams.

Two-state PRS beam sweeping

Proposal 5:

- Regarding 2-stage PRS beam sweeping, RAN1 should consider the following procedure for 2-stage beam reporting:

  - In case of the first PRS resource set, it can be composed of multiple PRS resources and they are associated with wide beams.

  - And then, the multiple PRS resources that are in the second PRS resource set can be associated with narrow beams. LMF can configure associated PRS resources based on the measurement report in the first step.

Observation 2:

- Considering the intention of 2-stage beam sweeping, the resolution and reporting range for measured quantity value could be different since beam width used for 2nd state is narrower than beam width used for 1st state.

Proposal 6:

- RAN1 needs to consider applying different resolution and range for measured quantity value in each stage respectively.

Reference

- 3GPP RAN1, Chair’s Notes RAN1#106-e meeting.

- 3GPP RAN1, Chair’s Notes RAN1#105-e meeting.