3GPP TSG-RAN WG1 #106bis-e		 	R1- 2110113

e-Meeting, October 11th – 19th, 2021

Agenda item:	8.2.5

Title:	PDSCH design consideration for NR from 52.6 GHz to 71 GHz

Source:	Convida Wireless

Document for:	Discussion

# Introduction

Agreement:

For TDRA in a DCI that can schedule multiple PDSCHs (or PUSCHs),

- A row of the TDRA table can indicate PDSCHs (or PUSCHs) that are in consecutive or non-consecutive slots, by configuring {SLIV, mapping type, scheduling offset K0 (or K2)} for each PDSCH (or PUSCH) in the row of TDRA table.

- Note: Whether and how to reduce RRC overhead is left to RAN2.

Agreement:

- For single TRP operation, for 480/960 kHz SCS,

  - FFS: A UE does not expect to be scheduled with more than one PDSCH in a slot, by a single DCI or multiple DCIs.

  - FFS: A UE does not expect to be scheduled with more than one PUSCH in a slot, by a single DCI or multiple DCIs.

- For single TRP operation, for 120 kHz SCS (same as current specification for FR2-1 for PUSCH),

  - Subject to UE capability, a UE can be scheduled with more than one PDSCH in a slot, by a single DCI or multiple DCIs.

  - Subject to UE capability, a UE can be scheduled with more than one PUSCH in a slot, by a single DCI or multiple DCIs.

- FFS for multi-TRP operation

- Note: The optimization of HARQ codebook size for Type 1 or Type 2 codebook design is considered as a low priority in Rel-17 (this does not preclude HARQ ACK bundling in time domain).

- The agreement made in RAN1#105-e is revised as follows.

Agreement: (RAN1#105-e) For enhancements of generating type-1 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs, the set of candidate PDSCH reception occasions corresponding to a UL slot with HARQ-ACK transmission is determined based on a set of DL slots and a set of SLIVs corresponding to each DL slot belonging to the set of DL slots. - The set of DL slots contains all the unique DL slots determined by considering all combinations of the configured K1 values and the configured rows of the TDRA table. - The set of SLIVs corresponding to a DL slot (belonging to the set of DL slots) contains all the SLIVs for that slot determined by considering all combinations of the configured K1 values and the configured rows of the TDRA table. - The Rel-16 procedure is reused for determining the candidate PDSCH reception occasions for the set of SLIVs corresponding to each DL slot belonging to the set of DL slots - Note: The Rel-16 procedure already handles pruning of multiple SLIVs corresponding to a DL slot, for both UEs that are and are not capable of receiving multiple PDSCHs per slot - FFS impact of time domain bundling, if supported

In RAN 1 meeting #106-e, the following agreements for supporting NR from 52.6 GHz to 71 GHz were made [1]:

In this contribution, we discuss several aspects of the PDSCH enhancement for supporting NR on frequencies from 52.6 GHz to 71 GHz.

# Supporting NR Operation from 52.6 GHz to 71 GHz

Frequency bands above 52.6 GHz have several advantages such as limited interference, wider bandwidth, high-gain beamforming, multi-beam systems, etc. Therefore, use cases like eMBB with high data rate, short range D2D communications with high data rate, wireless display transfer for AR/VR, etc. can be enabled.

To support NR from 52.6 GHz to 71 GHz, operation on both licensed and unlicensed spectrum with a range of ultra-wide-bandwidth frequency bands needs to be considered. For example, the entire 57 GHz – 71 GHz frequency range is available in USA, providing 14 GHz of contiguous bandwidth. In addition, the 66 GHz to 71 GHz band was recently opened for licensed use in international telecommunications union (ITU) Regions 1 and 3.

In RAN1 #103e meeting, subcarrier spacings (SCS) of 120 KHz, 240 KHz, 480 KHz and 960 KHz (i.e., μ= 3, 4, 5 and 6 respectively) for NR operation for both licensed and unlicensed bands from 52.6 GHz to 71 GHz have been agreed. When the larger SCSs/numerologies are introduced, the duration of slot in a subframe will be decreased accordingly. Due to the limited PDCCH processing capability, the number of monitored PDCCH candidates and the number of non-overlapped CCE per slot are expected to be decreased for the higher SCSs/numerologies (e.g., SCS = 480 KHz, 960 KHz, etc.) scenarios in frequency range greater than 52.6 GHz. In addition, the decreased number of PDCCH candidates and CCEs per slot may limit the scheduling flexibility. Besides, monitoring every slot for PDCCH becomes too frequent and may consume too much UE power in higher frequency range. Therefore, a single DCI scheduling multiple PDSCH/PUSCH could benefit the scheduling efficiency when the limited number of PDCCH/DCI can be scheduled within a monitoring occasion.

In the following sections, we discuss the PDSCH enhancement for NR operation from 52.6 GHz to 71 GHz.

## TDRA bit field for single DCI schedules multiple PDSCH/PUSCHs

Single DCI can schedule multiple PDSCH/PUSCHs for NR from 52.6 GHz and above. In this way, the PDCCH monitoring frequency could be reduced, thus it can reduce PDCCH decoding efforts for a UE. There are several design factors to be considered for single DCI scheduling multi-PDSCH. First, the number of scheduled PDSCH may vary case by case. To support flexible scheduling and reduce BD efforts, the single DCI format could schedule both a single PDSCH and multiple PDSCHs. In RAN 1 meeting #104bis-e [2], it has agreed that TDRA bit field in single DCI scheduling multi-PDSCH can be used for the indication of the number of scheduled PDSCH(s). Also, each PDSCH has a separate SLIV and mapping type such that continuous or non-contiguous (time-domain) transmission of PDSCH can be supported. TDRA table is extended such that each row indicates up to 8 multiple PDSCHs (that can be non-continuous in time-domain). Each PDSCH has a separate SLIV, K0/K2s and mapping type. The number of scheduled PDSCHs is implicitly indicated by the number of indicated valid SLIVs in the row of the TDRA table signalled in DCI. Especially, the number of scheduled PDSCH(s) is one of the key factors for the determination of the type-1 HARQ-ACK codebook generation.

In Rel-15/16, the candidate slot for PDSCH reception is determined by UL slot N (where HARQ-ACK codebook is transmitted) and K1 set. If Rel-15/16 rule is applied for single DCI scheduling multi-PDSCH, then K1 set requires some modification for handling the multi-PDSCH HARQ-ACK timing. To simplify the type-1 codebook HARQ-ACK generation and unify the framework between Rel-16 and Rel-17, we prefer no more than one scheduled PDSCHs are received in a same slot especially for SCS = 480/960 KHz. In other words, no overlapped SLIVs in the same and different DL slot. Therefore, type1 codebook ARQ-ACK generation can be determined by the (extended) K1 set and the set of candidate PDSCH reception occasions (i.e., slots).

Proposal 1. For type-1 codebook HARQ-ACK generation, it is preferred to use the extension of K1 set and the set of candidates PDSCH reception occasions/slots for reducing specification impact for single DCI scheduling multi-PDSCH.

Proposal 2. To simplify type-1 codebook HARQ-ACK generation in Rel-17, receiving more than one PDSCH for 480/960 KHz in a slot is not considered.

# Conclusions

In this contribution, we discussed the fundamental aspects of PDCCH monitoring for supporting NR from 52.6 to 71 GHz. The following proposals were made:

Proposal 1. For type-1 codebook HARQ-ACK generation, it is preferred to use the extension of K1 set and the set of candidates PDSCH reception occasions/slots for reducing specification impact for single DCI scheduling multi-PDSCH.

Proposal 2. To simplify type-1 codebook HARQ-ACK generation in Rel-17, receiving more than one PDSCH in a slot is not considered.

# References

- Chairman Notes, RAN1 #106-e, August 16th – 27th, 2021.

- Chairman Notes, RAN1 #104bis-e, April 12th – 20th, 2021.