3GPP TSG RAN WG1 Meeting #106-e						            R1-2110117

e-Meeting, October 11th – 19th, 2021

Agenda Item:	8.11.1.2

Title:	Discussion on Inter-UE Coordination for NR SL Mode 2 Enhancement

Source:	Convida Wireless

Document for:	Discussion

# Introduction

In RAN1 #106e e-meeting [6], the following were concluded:

Agreement

For scheme 1, the following inter-UE coordination information signalling from UE-A is supported. FFS details including condition(s)/scenario(s) under which each information is enabled to be sent by UE-A and used by UE-B.

- Set of resources preferred for UE-B’s transmission

- Set of resources non-preferred for UE-B’s transmission

Agreement

For scheme 2, the following inter-UE coordination information signalling from UE-A is supported. FFS details including condition(s)/scenario(s) under which each information is enabled to be sent by UE-A and used by UE-B

- Presence of expected/potential resource conflict on the resources indicated by UE-B’s SCI

  - FFS: UE behaviour when the presence of expected/potential resource conflict is detected by the transmitter

- FFS: Whether to additionally support the presence of detected resource conflict on the resources indicated by UE-B’s SCI

Agreement

- In scheme 1, the following is supported for UE(s) to be UE-A(s)/UE-B(s) in the inter-UE coordination information transmission triggered by an explicit request in Mode 2:

  - A UE that sends an explicit request for inter-UE coordination information can be UE-B

  - A UE that received an explicit request from UE-B and sends inter-UE coordination information to the UE-B can be UE-A

  - Working assumption At least a destination UE of a TB transmitted by UE-B can be UE A

  - The above feature can be enabled or disabled or controlled by (pre-)configuration

  - FFS: Details on how to support this, including (pre-)configuration signaling granularity

  - FFS: Additional details and conditions on UE-A and UE-B

- Working Assumption In scheme 1, the following is supported for UE(s) to be UE-A(s)/UE-B(s) in the inter-UE coordination information transmission triggered by a condition other than explicit request reception in Mode 2:

  - A UE that satisfies the condition mentioned in the main bullet and sends inter-UE coordination information is UE-A

  - A UE that received inter-UE coordination information from UE-A and uses it for resource (re-)selection is UE-B

  - The above feature can be enabled or disabled or controlled by (pre-)configuration

    - FFS: Details on how to support this, including (pre-)configuration signaling granularity

  - FFS: Additional details and conditions on UE-A and UE-B

Agreement

In scheme 2, at least the following is supported for UE(s) to be UE-A(s)/UE-B(s) in the inter-UE coordination transmission triggered by a detection of expected/potential resource conflict(s) in Mode 2:

- A UE that transmitted PSCCH/PSSCH with SCI indicating reserved resource(s) to be used for its transmission, received inter-UE coordination information from UE-A indicating expected/potential resource conflict(s) for the reserved resource(s), and uses it to determine resource re-selection is UE-B

- A UE that detects expected/potential resource conflict(s) on resource(s) indicated by UE-B’s SCI sends inter-UE coordination information to UE-B, subject to satisfy one of the following conditions, is UE-A

  - Working assumption At least a destination UE of one of the conflicting TBs, i.e., TBs to be transmitted in the expected/potential conflicting resource(s)

    - Whether a non-destination UE of a TB transmitted by UE-B can be UE-A is (pre-)configured

  - FFS: Additional details and condition(s) on UE-A and UE-B

- The above feature can be enabled or disabled or controlled by (pre-)configuration

  - FFS: Details on how to support this, including (pre-)configuration signaling granularity

- FFS: Definition of expected/potential resource conflict(s) and other details (if any)

Agreement

In scheme 2, the following UE-B’s behavior in its resource (re)selection is supported when it receives inter-UE coordination information from UE-A:

- UE-B can determine resource(s) to be re-selected based on the received coordination information

  - UE-B can reselect resource(s) reserved for its transmission when expected/potential resource conflict on the resource(s) is indicated

    - FFS: Other details (if any)

Agreement

In scheme 1, at least following UE-B’s behavior in its resource (re-)selection is supported when it receives inter-UE coordination information from UE-A:

- For preferred resource set, the following two options are supported:

  - Option A): UE-B’s resource(s) to be used for its transmission resource (re-)selection is based on both UE-B’s sensing result (if available) and the received coordination information

    - UE-B uses in its resource (re-)selection, resource(s) belonging to the preferred resource set in combination with its own sensing result

      - UE-B uses in its resource (re-)selection, resource(s) not belonging to the preferred resource set when condition(s) are met

        - FFS: Details of condition(s)

      - This option is supported when UE-B performs sensing/resource exclusion

      - FFS: Other details (if any)

  - Option B): UE-B’s resource(s) to be used for its transmission resource (re-)selection is based only on the received coordination information

    - UE-B uses in its resource (re-)selection, resource(s) belonging to the preferred resource set

      - This option is supported at least when UE-B does not support sensing/resource exclusion

        - FFS: Whether the support is conditional or UE capability

      - FFS: Other details (if any)

  - FFS: Other option(s), and other details (if any)

- For non-preferred resource set,

  - UE-B’s resource(s) to be used for its transmission resource (re-)selection is based on both UE-B’s sensing result (if available) and the received coordination information

    - UE-B excludes in its resource (re-)selection, resource(s) overlapping with the non-preferred resource set

      - FFS: Details including

        - Whether/how UE-B can use in its resource (re-)selection, resource(s) overlapping with the non-preferred resource set, definition of the overlap, and other details (if any)

        - When UE-B excludes in its resource (re-)selection, resource(s) overlapping with the non-preferred resource set

    - FFS: UE-B reselects in its resource (re-)selection, resource(s) to be used for its transmission when the resource(s) are fully/partially overlapping with the non-preferred resource set

  - FFS: Other option(s), and other details (if any)

Agreement

In scheme 2, at least the following is supported to determine inter-UE coordination information:

  - Among resource(s) indicated by UE-B’s SCI, UE-A considers that expected/potential resource conflict occurs on the resource(s) satisfying at least one of the following condition(s):

  - Condition 2-A-1:

    - Other UE’s reserved resource(s) identified by UE-A are fully/partially overlapping with resource(s) indicated by UE-B’s SCI in time-and-frequency

    - FFS: Other details (if any)

    - FFS: Whether/how to specify additional criteria and other details (if any) including signaling details of conflict indication

  - (Working Assumption) Condition 2-A-2:

    - Resource(s) (e.g., slot(s)) where UE-A, when it is intended receiver of UE-B, does not expect to perform SL reception from UE-B due to half duplex operation

      - FFS: Other details (if any)

  - FFS: Other condition(s)

  - FFS: Other details (if any)

Agreement

In scheme 1, at least the following is supported to determine inter-UE coordination information of preferred resource set:

  - UE-A considers any resource(s) satisfying all the following condition(s) as set of resource(s) preferred for UE-B’s transmission

    - Condition 1-A-1:

      - Resource(s) excluding those overlapping with reserved resource(s) of other UE identified by UE-A whose RSRP measurement is larger than a RSRP threshold

        - FFS: Other details (if any)

    - FFS: Condition 1-A-2:

      - Resource(s) excluding slot(s) where UE-A, when it is intended receiver of UE-B, does not expect to perform SL reception from UE-B

        - FFS: Other details (if any)

    - FFS: Condition 1-A-3:

      - Resource(s) satisfying UE-B’s traffic requirement (if available)

        - FFS: Other details (if any)

    - FFS: Other condition(s)

  - FFS: Other details (if any)

Agreement

In scheme 1, at least the following is supported to determine inter-UE coordination information of non-preferred resource set:

- UE-A considers any resource(s) satisfying at least one of the following condition(s) as set of resource(s) non-preferred for UE-B’s transmission

  - Condition 1-B-1:

    - Reserved resource(s) of other UE identified by UE-A from other UEs’ SCI (including priority field) and RSRP measurement

      - FFS: Other details (if any)

  - FFS: Condition 1-B-2:

    - Resource(s) (e.g., slot(s)) where UE-A, when it is intended receiver of UE-B, does not expect to perform SL reception from UE-B

      - FFS: Other details (if any)

  - FFS: Other condition(s)

- FFS: Other details (if any)

In RAN1 #104bis e-meeting [1], the following were concluded:

Agreements:

- Support the following schemes of inter-UE coordination in Mode 2:

  - Inter-UE Coordination Scheme 1:

    - The coordination information sent from UE-A to UE-B is the set of resources preferred and/or non-preferred for UE-B’s transmission

      - FFS details including a possibility of down-selection between the preferred resource set and the non-preferred resource set, whether or not to include any additional information other than indicating time/frequency of the resources within the set in the coordination information

    - FFS condition(s) in which Scheme 1 is used

  - Inter-UE Coordination Scheme 2:

    - The coordination information sent from UE-A to UE-B is the presence of expected/potential and/or detected resource conflict on the resources indicated by UE-B’s SCI

      - FFS details including a possibility of down-selection between the expected/potential conflict and the detected resource conflict

    - FFS condition(s) in which Scheme 2 is used

Agreements:

- Study further to determine the conditions for UEs to be UE-A(s)/UE-B(s) for inter-UE coordination:

  - Details include applicable scenario(s)/inter-UE coordination scheme(s)

  - E.g., only UE(s) among the intended receiver(s) of UE-B can be a UE-A, any UE can be a UE-A, high-layer configured, etc.

    - Including the possibility of being subject to certain conditions and/or capability

Agreements:

- When UE-B receives the inter-UE coordination information from UE-A, consider at least one of the following options (with details FFS including possibly down-selecting/merging one or more of the options below, applicable scenario(s)/condition(s) for each option, UE behavior) for UE-B’s to take it into account in the resource (re)-selection for its own transmission

  - For scheme 1:

    - Option 1-1: UE-B’s resource(s) to be used for its transmission resource (re)-selection is based on both UE-B’s sensing result (if available) and the received coordination information

    - Option 1-2: UE-B’s resource(s) to be used for its transmission resource (re)-selection is based only on the received coordination information

    - Option 1-3: UE-B’s resource(s) to be re-selected based on the received coordination information

    - Option 1-4: UE-B’s resource(s) to be used for its transmission resource (re)-selection is based on the received coordination information

  - For scheme 2:

    - Option 2-1: UE-B can determine resource(s) to be re-selected based on the received coordination information

    - Option 2-2: UE-B can determine a necessity of retransmission based on the received coordination information

At RAN #91e [4], the scope of the resource allocation enhancements for mode 2 was limited to inter-UE coordination. In this contribution, we consider and discuss the aspects of resource allocation for NR sidelink enhancement. Particularly, we consider the aspects of resource allocation mode 2 enhancement.

# Reliability and Latency Consideration for Resource Allocation Mode 2

In Rel-16, NR sidelink resource allocation mode 2 performs autonomous resource selection. Such autonomous resource selection is based on resource sensing procedure. The resources that are sensed not to be available (e.g., the sensed resources that have higher SL-RSRP and higher priority) could be excluded from the resource selection during resource selection procedure while the resources that are sensed to be available could be included for the resource selection during resource selection procedure. For example, the sensed resources that have lower SL-RSRP and lower priority could be included in resource selection during resource selection procedure for the Tx UE. Different from resource allocation mode 1, the performance of resource allocation mode 2 are also determined by the performance of resource sensing. If resource sensing performance is not reliable enough, the total reliability of resource allocation mode 2 could be negatively affected. Therefore, as compared to resource allocation mode 1, additional enhancement for resource allocation mode 2 should be considered.

When resource sensing procedures result in resource collisions for resource allocation mode 2, eventually such collisions result in overall NR sidelink performance degradation for mode 2. If PSCCH decoding performance suffers, this could result in that UE not decoding PSCCH successfully. PSCCH decoding is part of resource sensing procedure. Failure of PSCCH decoding may cause inaccurate sensing results, which in turn may cause inaccurate resource selection. Inaccurate or incorrect resource selection may result in resource collisions. This could cause additional problem for reliability for resource allocation mode 2. In order to support URLLC-like use cases, reliability for resource allocation mode 2 should be considered and enhancement for resource allocation mode 2 should be supported.

NR sidelink resource allocation mode 2 performs autonomous resource selection which is based on resource sensing procedure. If resource sensing is not perfect or not accurate enough, collisions for resources during resource selection procedure may occur. When Tx UE selects the resources that are not appropriate for Rx UE, interference at the Rx UE could be high and detection of PSCCH and PSSCH could be poor and even fail. If too many collisions occur, not only reliability for NR sidelink communication using resource allocation mode 2 could be affected, but also there would be impact on latency for NR sidelink communication using resource allocation mode 2 as well. When resource collision occurs, it could cause that PSCCH and/or PSSCH are not received successfully. This could have impact on the detection of the 2nd stage SCI as well as PSSCH reception. This could result in additional retransmissions for HARQ procedure or failure of blind retransmission. Therefore, resource allocation mode 2 should be evaluated whether there is significant impact on the latency especially for supporting URLLC-like latency. Latency for NR sidelink resource allocation mode 2 should be considered. Reduction of latency for NR sidelink resource allocation mode 2 should be supported considering URLLC-type sidelink use cases for low latency application.

Proposal 1: Reliability and latency enhancement for NR SL resource allocation mode 2 should be supported.

# Inter-UE Coordination for Resource Allocation Mode 2

Since NR sidelink resource allocation mode 2 needs to perform additional procedures such as resource sensing procedures for resource (re-)selection and scheduling, there could be performance impact on either reliability, latency or both as described previously. Some methods may be needed to mitigate such impact on reliability or latency. Inter-UE coordination may be a way to mitigate the impact on reliability and latency. Inter-UE coordination could allow collaboration between UEs for resource sensing procedures and resource selection procedures as well as scheduling between UEs. It could potentially reduce the resource collisions and mitigate incorrect resource selection. It was concluded that no WID update is necessary [4]. WGs continue specifying inter-UE coordination. It is noted that enhancements other than inter-UE coordination is not pursued in the scope of the objective “Study the feasibility and benefit of solution(s) on the enhancement(s) in mode 2 for enhanced reliability and reduced latency in consideration of both PRR and PIR defined in TR37.885 (by RAN#91),and specify the identified solution(s) if deemed feasible and beneficial.” In one case, Tx UE and Rx UE may have different interferences from other surrounding UEs or nodes, this may yield different resource sensing results. Therefore, inter-UE coordination may mitigate such interferences with the help or assistance of the other UE(s). For example, Rx UE may provide assistance information to Tx UE, or Tx UE may provide assistance information to Rx UE. By doing so, resource sensing performance may be improved, and resource selection performance could be enhanced.

Tx UE could schedule other UEs for transmitting or reception. If a UE can schedule another UE or UEs, e.g., a group of UEs, then there would be no collision at least between the UEs within that group. This is because UEs can be scheduled not to collide with one another within that group. In general, this could result in better reliability and lower latency for resource allocation mode 2 for NR sidelink. This could also result in lower power consumption for UEs since UEs that are scheduled may not need to perform sensing. Therefore, inter-UE coordination should be considered for performance enhancement for reliability and/or latency as well as reduction for power consumption.

For inter-UE coordination a set of resources determined at UE-A could be sent to UE-B in mode 2. UE-B can take this information into account in the resource selection for its own transmission. We may consider a set of resources that is preferred for UE-B’s transmission, e.g., resource set which is preferred for UE-A’s reception. In addition, we may also consider resource set which is preferred not to be used by UE-B’s transmission, e.g., resource set that is not preferred for UE-A’s reception. In case of groupcast we may also need to consider a set of resources that is preferred for other intended receivers of UE-B’s transmission. In addition, we may also consider resource set that is not preferred for other intended receiver(s) of UE-B’s transmission. The set of resources could be carried either at the physical layer or at higher layers. How to determine which/what channel to carry the information for a set of resource could depend on the payload size, priority and/or latency. For example, if payload size of the information for a set of resource is small, the set of resources could be carried at the physical layer, e.g., in SCI. On the other hand, if payload size is large, the set of resources could be carried at the higher layer. Alternatively, if payload size is small, the set of resources could be carried in PSCCH. If payload size is large, the set of resources could be carried in PSSCH.

Periodic, aperiodic or semi-persistent transmission of the set of resources could be considered. The set of resources may be requested by UE B. UE B may request different types of resources or information. For example, UE B may request the set of resources that is good to avoid hidden-node problem, exposed-node problem, half duplex problem, or combination of them, etc. The coordination information may be requested by UE B. UE B may request different types of coordination information. For example, different types of coordination information could include scheduling type or assistance type information.

UE-B could take a set of resources into account in the sensing and resource (re-)selection procedures for its own transmission. Other assistance and/or coordinating information other than a set of resources could be supported if needed. UE B may request different type of information including the set of resources with and without other assistance information.

Inter-UE coordination can be supported in all cast types including unicast, groupcast and broadcast to enable the enhancement for reliability and latency for different cast types. At least unicast and groupcast should be supported. Different coordination signaling or information could be used for different cast types. A unified coordination signaling or information could also be considered and developed. When a set of resources determined at UE-A is sent to UE-B in mode 2, UE-B can take into account the information in the sensing and/or resource selection for its own transmission. UE-A could send information of a set of resources to UE-B based on explicit signaling, e.g., trigger or request signaling. UE-A could also send information of a set of resources to UE-B based on implicit indication, e.g., based on a pre-defined or (pre)configured triggering condition(s). Combination of both implicit indication and explicit signaling could also be considered. UE A could send coordination signaling or coordination information based on triggering, request signal and/or predefined or (pre-)configured conditions.

Coordination signaling or coordination information may contain a set of resources and other assistance information. For example, UE A could send coordination signaling or coordination information including information for a set of resources periodically e.g., in PSCCH, aperiodically e.g., in PSSCH, or semi-persistently. Whether to use periodic or aperiodic could depend on payload size of coordination signaling or coordination information. For example, if payload size of coordination signaling or coordination information is large, then aperiodic transmission or reporting of coordination signaling or coordination information could be used. On the other hand, if payload size of coordination signaling or coordination information is small, then periodic transmission or reporting of coordination signaling or coordination information could be used.

Coordination information could be of different type(s) including scheduling type info or assistance type info for inter-UE coordination for mode 2. If assistance type inter-UE coordination is used, assistance type coordination information could include a set of resource that is provided by UE A and could be used for sensing and/or resource (re-)selection at UE B for inter-UE coordination. If scheduling type inter-UE coordination is used, scheduling type coordination information could include the exact resource that is used for scheduling UE B for inter-UE coordination. To support and enable scheduling type inter-UE coordination, an indicator could be included in SCI to distinguish between transmission and reception for scheduling the UE. If assistance type inter-UE coordination is used, assistance type coordination information could include other assistance info if needed in addition to a set of resources for inter-UE coordination.

When UE-B receives the inter-UE coordination information from UE-A, UE-B could take coordination information into account in the resource (re)-selection for its own transmission. For scheme 1, UE-B’s resource(s) could be used for its transmission resource (re)-selection and could be based on both UE-B’s sensing result and the received coordination information, or UE-B’s resource(s) to be used for its transmission resource (re)-selection could be based only on the received coordination information. UE-B’s resource(s) could be re-selected based on the received coordination information. Or UE-B’s resource(s) to be used for its transmission resource (re)-selection could be based on the received coordination information. For scheme 2, UE-B could determine resource(s) to be re-selected based on the received coordination information. Alternatively, UE-B could determine a necessity of retransmission based on the received coordination information. For scheme 1, our preference is that UE-B’s resource(s) could be used for its transmission resource (re)-selection and could be based on both UE-B’s sensing result if available and the received coordination information. UE-B’s resource(s) to be used for its transmission resource (re)-selection could be based only on the received coordination information in case that scheduling type coordination is used. For scheme 2, our preference is that both possibiulities that UE-B could determine resource(s) to be re-selected based on the received coordination information and UE-B could determine a necessity of retransmission based on the received coordination information could be supported.

In scheme 1, at least following UE-B’s behavior in its resource (re-)selection is supported when it receives inter-UE coordination information from UE-A. For preferred resource set, the following two options can be supported. Option A): UE-B’s resource(s) to be used for its transmission resource (re-)selection is based on both UE-B’s sensing result if available and the received coordination information. UE-B uses in its resource (re-)selection, resource(s) belonging to the preferred resource set in combination with its own sensing result. UE-B uses in its resource (re-)selection, resource(s) not belonging to the preferred resource set when condition(s) are met. This option is supported when UE-B performs sensing/resource exclusion. Option B): UE-B’s resource(s) to be used for its transmission resource (re-)selection is based only on the received coordination information. UE-B uses in its resource (re-)selection, resource(s) belonging to the preferred resource set. This option is supported at least when UE-B does not support sensing/resource exclusion. Whether the support is conditional or UE capability can be for further studied. Whether other option or other details are needed could be further studied.

For non-preferred resource set, UE-B’s resource(s) to be used for its transmission resource (re-)selection is based on both UE-B’s sensing result if available and the received coordination information. UE-B excludes in its resource (re-)selection, resource(s) overlapping with the non-preferred resource set. Whether and how UE-B can use in its resource (re-)selection, resource(s) overlapping with the non-preferred resource set, definition of the overlap, and other details could be further studied. When UE-B excludes in its resource (re-)selection, resource(s) overlapping with the non-preferred resource set. Whether UE-B reselects in its resource (re-)selection, resource(s) to be used for its transmission when the resource(s) are fully/partially overlapping with the non-preferred resource set could be for further study. Whether other option or other details are needed could be further studied.

Proposal 2: Inter-UE operation with coordination or assistance information should be supported.

Proposal 3: Coordination information could be of different type(s) including scheduling type and assistance type information for inter-UE coordination for resource allocation mode 2.

Proposal 4: Assistance type information could include a set of resource that is provided by UE A and could be used for sensing and/or resource (re-)selection at UE B for inter-UE coordination.

Proposal 5: Assistance type information could include other assistance information if needed in addition to a set of resources for inter-UE coordination.

Proposal 6: Scheduling type information could include the exact resource that is used for scheduling UE B for inter-UE coordination.

Proposal 7: Which channel to carry a set of resources could be based on at least payload size of coordination information.

Proposal 8: Whether to use periodic or aperiodic to transmit a set of resources could be based on at least payload size of coordination information.

Proposal 9: An indicator could be included in SCI to distinguish between transmission and reception for scheduling of UE(s).

Proposal 10: Different cast types including unicast, groupcast and broadcast should be considered for inter-UE coordination.

Proposal 11: For scheme 1, UE-B’s resource(s) could be used for its transmission resource (re)-selection and could be based on both UE-B’s sensing result and the received coordination information. UE-B’s resource(s) to be used for its transmission resource (re)-selection could be based only on the received coordination information.

Proposal 12: For scheme 2, both possibilities that UE-B could determine resource(s) to be re-selected based on the received coordination information and UE-B could determine a necessity of retransmission based on the received coordination information could be considered.

Proposal 13: For scheme 1, both Option A: UE-B’s resource(s) to be used for its transmission resource (re-)selection is based on both UE-B’s sensing result (if available) and the received coordination information and Option B: UE-B’s resource(s) to be used for its transmission resource (re-)selection is based only on the received coordination information are supported.

Proposal 14: For scheme 1 Option B, UE-B’s resource(s) to be used for its transmission resource (re-)selection is based only on the received coordination information and UE A scheduling UE B should be supported.

# Conclusions

In this contribution, we have the following proposals:

Proposal 1: Reliability and latency enhancement for NR SL resource allocation mode 2 should be supported.

Proposal 2: Inter-UE operation with coordination or assistance information should be supported.

Proposal 3: Coordination information could be of different type(s) including scheduling type and assistance type information for inter-UE coordination for resource allocation mode 2.

Proposal 4: Assistance type information could include a set of resource that is provided by UE A and could be used for sensing and/or resource (re-)selection at UE B for inter-UE coordination.

Proposal 5: Assistance type information could include other assistance information if needed in addition to a set of resources for inter-UE coordination.

Proposal 6: Scheduling type information could include the exact resource that is used for scheduling UE B for inter-UE coordination.

Proposal 7: Which channel to carry a set of resources could be based on at least payload size of coordination information.

Proposal 8: Whether to use periodic or aperiodic to transmit a set of resources could be based on at least payload size of coordination information.

Proposal 9: An indicator could be included in SCI to distinguish between transmission and reception for scheduling of UE(s).

Proposal 10: Different cast types including unicast, groupcast and broadcast should be considered for inter-UE coordination.

Proposal 11: For scheme 1, UE-B’s resource(s) could be used for its transmission resource (re)-selection and could be based on both UE-B’s sensing result and the received coordination information. UE-B’s resource(s) to be used for its transmission resource (re)-selection could be based only on the received coordination information.

Proposal 12: For scheme 2, both possibilities that UE-B could determine resource(s) to be re-selected based on the received coordination information and UE-B could determine a necessity of retransmission based on the received coordination information could be considered.

Proposal 13: For scheme 1, both Option A: UE-B’s resource(s) to be used for its transmission resource (re-)selection is based on both UE-B’s sensing result (if available) and the received coordination information and Option B: UE-B’s resource(s) to be used for its transmission resource (re-)selection is based only on the received coordination information are supported.

Proposal 14: For scheme 1 Option B, UE-B’s resource(s) to be used for its transmission resource (re-)selection is based only on the received coordination information and UE A scheduling UE B should be supported.

# References

- “RAN1 Chairman’s Notes”, RAN1 #104bis-e, April 12th – April 20th, 2021

- “RAN1 Chairman’s Notes”, RAN1 #104-e, January 25th – February 5th, 2021

- “RAN1 Chairman’s Notes”, RAN1#103-e.

- RP-202846, WID revision: NR sidelink enhancement

- “RAN1 Chairman’s Notes”, RAN1 #105-e, May 10th – May 27th, 2021

- “RAN1 Chairman’s Notes”, RAN1 #106-e