3GPP TSG RAN WG1 #106b-e		                                    		   R1-2110134

e-Meeting, Oct. 11th – Oct. 19th, 2021

Agenda Item:	8.5.2

Source:	InterDigital, Inc.

Title:	Enhancements for UL-AoA positioning solutions

Document for:	Discussion and Decision

# Introduction

In RAN1#106e, the agreements regarding  enhancements for UL-AoA positioning methods were made[1]. In this contribution, we make a proposal on RSRP reporting in the presence of multiple paths in the channel.

# Enhancements for UL-AoA positioning methods

The following agreement is made in RAN1#106e. The agreement below was made to enhance reporting for DL & UL based positioning in the presence of multiple paths in the channel.

Agreement: Reporting of one gNB Rx-Tx time difference and multiple UL-AOAs measurements for the first arrival path per SRS resource for positioning in a single gNB report to LMF is supported - The above measurements are associated with SRS resource ID which is also reported to LMF - FFS: Reporting of RSRP for the first arrival path

Reporting of RSRP for the first arrival path will be beneficial to determine quality of the measurement. If multiple UL-AOAs are observed for the path, if the gNB is capable, the RSRP per UL-AOA should also be reported to the LMF from the gNB to improve resolution of the measurement. In addition, the number of UL-AOAs that can be measured by the gNB and corresponding number of RSRPs should be agreed. Thus the following proposal is made :

Proposal 1: RSRP per measured UL-AOA for the first arrival path can be reported by the gNB to the LMF

# Conclusion.

In this contribution, the following proposal is made.

Proposal 1: RSRP per UL-AOA for the first arrival path can be reported by the gNB to the LMF

# Reference

[1] RAN1 Chairman’s notes, RAN1#106e, Aug. 2021.