3GPP TSG RAN WG1 #106bis-e	R1-2110142

e-Meeting, Oct 11th – 19th, 2021

Source:	Ericsson

Title:	Reduced Latency SCell Activation

Agenda Item:	8.13.2

Document for:	Discussion and Decision

# Introduction

In this contribution we discuss further details of SCell activation procedure using ‘temporary RS’ including triggering of ‘temporary RS’ and aspects related to Option 1a including MAC CE-related design.

# Discussion

In RAN1#106-e, agreements were made on timeline design for receiving temporary RS, and the information that is to be indicated via the MAC CE, including identification of possible options to convey the triggering information via the MAC CE. We discuss below some of the remaining aspects.

The first issue is related to what information is conveyed by the MAC and how it is conveyed, including any related RRC parameters.

We now discuss what information about the temporary RS(s) should be carried within the MAC CE(s). Below agreements were made in previous RAN1 meeting.

Agreement

For efficient activation of Scells, the triggered temporary RS is aperiodic.

Agreement

To trigger temporary RS for efficient activation of SCells, the contents of the triggering MAC-CE(s) in a single PDSCH provide at least the following information (explicitly or implicitly):

- Whether or not temporary RS is triggered

- FFS detailed Information of temporary RS, e.g.:

  - Resources used for triggered Temporary RS

  - Triggering time offset of triggered Temporary RS

  - QCL source for triggered Temporary RS

Agreement

To trigger temporary RS,

- MAC-CE at least provides the following information:

- temporary RSs are to be triggered on X out of Y (Y≥X) to-be-activated SCells, respectively, while no temporary RS is to be triggered on the other to-be-activated SCells.

- The following information can be provided by RRC for temporary RS for each SCell

- The number of RS bursts and the gap length between the RS bursts (Opt 2.3.3)

- Triggering offset of temporary RS (Opt 2.3.4)

    - Triggering offset can be provided, e.g., by reusing existing CSI-RS framework

- QCL information (Opt 2.3.5)

    - Triggering QCL information can be provided, e.g., by reusing existing CSI-RS framework

- A unique temporary RS configuration index

- FFS: the maximum number of temporary RS per cell/per UE

Note: Reusing A-TRS triggering framework is not precluded.

- Information for 0, 1, or more temporary RS can be provided for each configured SCell

The following was agreed on potential alternatives based on which the triggering option would be selected.

Agreement

- For triggering temporary RS, down-select based on the following alternatives, or let RAN2 be aware the status of this discussion

- Alt 1: Bitmap approach in MAC-CE similar to SCell activation

    - Every Z-bit block in the bitmap corresponds to a SCell, Z>=0

    - A Z-bit block indicates the temporary RS [configuration index], and a value zero indicated by the bit block means no RS resource transmitted.

    - The to-be-activated SCell is indicated via the C values in the legacy SCell activation/de-activation MAC CE or in the new MAC-CE

- Alt 2: Reuse A-TRS triggering framework

    - A trigger state is indicated by the MAC-CE explicitly

    - The association between a trigger state and aperiodic temporary RS for one or multiple SCells is configured by RRC according Rel-16 A-TRS triggering framework

      - SCell ID is configured as a part of the temporary RS configuration. Some SCell IDs derived from the trigger state triggered by the new MAC-CE may not refer to to-be-activated SCells that are indicated by the new MAC-CE or the legacy SCell activation/de-activation MAC-CE

    - FFS: The value zero of the MAC-CE indication means no temporary RS is triggered by the MAC-CE for all to-be-activated SCells

- Note: The down-selection targets at a RAN1 consensus on MAC-CE functionality and the list of RRC parameters for this feature. Any MAC-CE signaling design above are reference concept, its final MAC-CE signaling design is up to RAN2.

We start with Rel-15/16 approach first, since Rel-15/16 already supports triggering of aperiodic TRS for one or more cells. This is done via a single DCI field (e.g. CSI request field in DCI 0_1). The triggering information included in the DCI corresponds to a trigger State ID which looks up a A-CSI-RS trigger state list configured by the RRC to identify the cells on which TRS is triggered and the associated details of those TRSs (NZP CSI-RS resource, QCL information, aperiodic triggering offset, etc).

Observation 1

- In Rel-15/16 DCI-based aperiodic TRS triggering, a single field within a DCI (e.g., 0_1, 1_1) can trigger aperiodic TRS(s) on one or more cells using the A-CSI-RS trigger-state-list based approach.

Therefore, our view is that existing RRC structure for A-TRS triggering should be reused as much possible and specifying new and potentially duplicate RRC structures (e.g. the temporaryRS-Config shown in the RRC parameter spreadsheet discussed during RAN1#106-e [4]) should be avoided.

For example, to trigger single ‘burst’ of A-TRS during SCell activation, the RRC framework of ‘CSI-AperiodicTriggerStateList’ can be fully reused. Trigger state ID ‘n’ indicated by MAC CE would correspond to nth configured CSI-AperiodicTriggerState in the list. This should be starting point for the discussion and any changes/optimizations relative to this can be discussed with related justifications.

For multiple burst case also (e.g. as indicated in LS from RAN4 in [3]), the existing structure can be reused (e.g., a CSI-AperiodicTriggerState including two separate resourceSet each with different aperiodicTriggeringOffset).

Above approach is based on Alt 2 in the RAN1#106-e agreement and this is therefore more efficient and simpler. The main additional specification would be the RS trigger will be sent in MAC CE.

Observation 2

- For MAC-CE-based aperiodic TRS triggering in Rel-17, a single field within the MAC CE  is sufficient to trigger aperiodic TRS(s) on one or more cells using the A-CSI-RS trigger-state-list based approach.

Observation 3

- For MAC-CE-based aperiodic TRS triggering in Rel-17, reusing the existing A-CSI-RS trigger-state-list based approach is simple solution that avoids new RRC structures with duplicated functionality.

Proposal 1

- A-CSI-RS trigger-state-list based approach is reused for configuration of temporary RS, and the trigger state ID is carried in MAC CE to indicate the triggered A-TRS.

The existing Rel16 A-CSI-RS trigger state list can be reused with some trigger states configured for A-TRS on different configured SCells and other states for A-CSI reporting, with the inclusion of the burst information within the resource set configuration. Then in terms of optimizations beyond reusing existing RRC parameters, optional configuration of a separate list with trigger states just for A-TRS can be considered e.g. TRS-AperiodicTriggerStateList with same structure as CSI-AperiodicTriggerStateList but only containing trigger states that trigger A-TRS (this to limit impact on number of entries in CSI-AperiodicTriggerStateList which is also used for DCI based A-CSI triggering).

Proposal 2

- If existing Rel16 A-CSI-RS trigger state list is deemed insufficient, a new A-TRS trigger state list (with same framework as A-CSI-RS trigger state list) for SCell activation is configured by higher layers, and the trigger state ID corresponding to the separate A-TRS trigger state list is carried in MAC CE to indicate the triggered A-TRS.

# RRC parameter discussion

Per the proposals in section 2, the trigger states are configured via RRC, and the configuration for each trigger state includes all the information needed for the triggered A-TRS, including cell identifier, the TRS resource information (for aperiodic TRS), QCL information, as well as the aperiodic triggering offset, e.g. as below. Then, the MAC CE indicates the trigger state ID.

Figure 1. Rel-15/16 Trigger state list mechanism.

As mentioned in previous section, the RRC parameter structure from Rel-15/16 A-CSI-trigger-state-list mechanism can be reused, and relevant IEs are shown here for reference.

-- ASN1START

-- TAG-CSI-APERIODICTRIGGERSTATELIST-START

CSI-AperiodicTriggerStateList ::=   SEQUENCE (SIZE (1..maxNrOfCSI-AperiodicTriggers)) OF CSI-AperiodicTriggerState

CSI-AperiodicTriggerState ::=       SEQUENCE {

associatedReportConfigInfoList      SEQUENCE (SIZE(1..maxNrofReportConfigPerAperiodicTrigger)) OF CSI-AssociatedReportConfigInfo,

...

}

CSI-AssociatedReportConfigInfo ::=  SEQUENCE {

reportConfigId                      CSI-ReportConfigId,

resourcesForChannel                 CHOICE {

nzp-CSI-RS                          SEQUENCE {

resourceSet                         INTEGER (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig),

qcl-info                            SEQUENCE (SIZE(1..maxNrofAP-CSI-RS-ResourcesPerSet)) OF TCI-StateId

OPTIONAL  -- Cond Aperiodic

},

csi-SSB-ResourceSet                 INTEGER (1..maxNrofCSI-SSB-ResourceSetsPerConfig)

},

csi-IM-ResourcesForInterference     INTEGER(1..maxNrofCSI-IM-ResourceSetsPerConfig)               OPTIONAL, -- Cond CSI-IM-ForInterference

nzp-CSI-RS-ResourcesForInterference INTEGER (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)          OPTIONAL, -- Cond NZP-CSI-RS-ForInterference

...

}

-- TAG-CSI-APERIODICTRIGGERSTATELIST-STOP

-- ASN1STOP

-- ASN1START

-- TAG-NZP-CSI-RS-RESOURCESET-START

NZP-CSI-RS-ResourceSet ::=          SEQUENCE {

nzp-CSI-ResourceSetId               NZP-CSI-RS-ResourceSetId,

nzp-CSI-RS-Resources                SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceId,

repetition                          ENUMERATED { on, off }                                                  OPTIONAL,   -- Need S

aperiodicTriggeringOffset           INTEGER(0..6)                                                           OPTIONAL,   -- Need S

trs-Info                            ENUMERATED {true}                                                       OPTIONAL,   -- Need R

...,

[[

aperiodicTriggeringOffset-r16       INTEGER(0..31)                                                          OPTIONAL   -- Need S

]]

}

-- TAG-NZP-CSI-RS-RESOURCESET-STOP

-- ASN1STOP

-- ASN1STOP

# Conclusions

In this document we discuss fast SCell activation for Rel17 NR CA and make the following proposals.

Observation 1

- In Rel-15/16 DCI-based aperiodic TRS triggering, a single field within a DCI (e.g., 0_1, 1_1) can trigger aperiodic TRS(s) on one or more cells using the A-CSI-RS trigger-state-list based approach.

Observation 2

- For MAC-CE-based aperiodic TRS triggering in Rel-17, a single field within the MAC CE is sufficient to trigger aperiodic TRS(s) on one or more cells using the A-CSI-RS trigger-state-list based approach.

Observation 3

- For MAC-CE-based aperiodic TRS triggering in Rel-17, reusing the existing A-CSI-RS trigger-state-list based approach is simple solution that avoids new RRC structures with duplicated functionality.

Proposal 1

- A-CSI-RS trigger-state-list based approach is reused for configuration of temporary RS, and the trigger state ID is carried in MAC CE to indicate the triggered A-TRS.

Proposal 2

- If existing Rel16 A-CSI-RS trigger state list is deemed insufficient, a new A-TRS trigger state list (with same framework as A-CSI-RS trigger state list) for SCell activation is configured by higher layers, and the trigger state ID corresponding to the separate A-TRS trigger state list is carried in MAC CE to indicate the triggered A-TRS.

# References

- R1-2102300 “Reply LS on temporary RS for efficient SCell activation in NR CA”, RAN4 -> RAN1 LS, RAN1#104b-e, April 2021.

- R1-2106427	Reply LS on temporary RS for efficient SCell activation in NR CA	RAN4, Huawei, Aug 2021.

- R1-2108708	Reply LS on temporary RS for efficient SCell activation in NR CA	Huawei, HiSilicon	RAN4, Oct 2021

- R1-2108674	Summary of email discussion [Post-106-e-Rel17-RRC-14] on efficient SCell activation/de-activation mechanism of NR CA	Moderator (Huawei), Aug 2021

- R1-2108679	Preliminary RAN1 UE features list for Rel-17 NR	Moderators (AT&T, NTT DOCOMO, INC.), Aug 2021