3GPP TSG RAN WG1 #106bis-e	    	     			R1-2110145

e-Meeting, Oct 11th -19th, 2021

Agenda Item:	8.17.7

Source:	Ericsson

Title:	UE features for UEPS

Document for:	Discussion/Decision

# 1	Introduction

This document discusses UE features related to Rel-17 UE power savings work item.

# 2	Discussion and proposals

Below are some of the main changes proposed for the FGs (using track changes in below table).

- For FGs 29-1 (PEI) and 29-2 (TRS occasions), allow optional UE capability signalling as it is useful for NW to know when to turn on these features. Alternatively, the last column can be left blank and discussed later.

- The ‘consequence if feature is not supported by the UE’ should be left empty’ and making generalized statements about deployments, etc should be avoided. It is clear that all features being developed in the WI are for UE power savings.

- For FG(29-3), component 3) should be added as per the WA from RAN1#106-e.

Proposal 1: Update the UE feature list for UEPS as shown in below Table.

- NR_UE_pow_sav_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | 29-1 | Paging enhancement | 1. Support paging early indication 2. Support UE subgroup indication |  |  |  |  | Per UE | N | N | N |  | Optional with capability signalling |
| 29. NR_UE_pow_sav_enh | 29-2 | TRS occasions for idle/inactive UEs | TRS occasions for idle/inactive UEs 1. Support reading TRS configuration from SIB 2. Support receiving L1 indication for TRS availability |  |  |  |  | Per UE | N | N | N |  | Optional with capability signalling |
| 29. NR_UE_pow_sav_enh | 29-3 | PDCCH monitoring adaptation within an active BWP | - Support of PDCCH monitoring adaptation behaviour 1/1A - Support of PDCCH monitoring adaptation behaviour 2/2A/[2B] - Support of PDCCH monitoring adaptation behaviour 1/1A/2/2A |  |  |  |  | Per UE | N | N | N | FFS: Support of PDCCH monitoring adaptation behaviour 1/1A/2/2A/2B | Optional |

# References

[1] R1-2108679, Preliminary RAN1 UE features list for Rel-17 NR, Moderators (AT&T, NTT DOCOMO, INC.), RAN WG1 #106-e, Aug 2021.