3GPP TSG RAN WG1 Meeting #106bis-e    	            R1-2110162

E-meeting, October 11th – 19th, 2021

Source:	Qualcomm Incorporated

Title:	Discussion on RAN4 LS on scell dropping issue of CA

Agenda Item:	5

Document for: 	Discussion and Decision

# Introduction

The work item description has been revised in RP-212527 to include a new objective for UL-CA [1]:

- 5) Specify the solution that prevent transmission power dropping on the cell with lower priority order for FR1 and FR2 CA - The solution applies for both intra-band UL CA and inter-band UL CA within the same frequency range. - Update RAN1, RAN2 specification on the impact of the solution if any

Related to this, RAN4 approved an LS that asks RAN1 following questions [2].

In TS 38.213, the transmission priority rule for CA is specified based on physical channel, and also with: “In case of same priority order and for operation with carrier aggregation, the UE prioritizes power allocation for transmissions on the primary cell of the MCG or the SCG over transmissions on a secondary cell”. Thus, the UE is always required to drop the power on cell with lower priority when the configured power on cell with high priority occupies all power ability of the UE. In order to advance the above discussions, RAN4 would like to ask RAN1 following questions: Question 1: Whether UE drop Scell power according to the priority rule defined in 38.213 is considered as an issue from RAN1 perspective. Question 2: Whether UE drop Scell power according to the priority rule defined in 38.213 has been addressed from 16 or 17? If not, what expected solution(s) are? Question 3: If the problem above is solved in RAN4 specifications with solution by higher layer configuration, e.g. introduce additional UE-specific configuration of power limits on Pcmax,f,c for each CC to prevent SCell dropping (see e.g. R4-2112826 or R4-2114551 for details), is there any expected RAN1 spec impact or possible conflict with UE behaviour defined in RAN1 specifications?

In this contribution, we provide our views on this RAN4 LS.

# Discussion

RAN1 spec specifies the UE behavior for UL-CA power-limited handling in the following way:

- If the total power would exceed PCMAX for a given FR,

  - the UE allocates the transmission power to the concurrent UL transmissions according to the priority order specified in 38.213 Section 7.5, where:

  - the total power in the FR after this power allocation according to the priority order does not exceed PCMAX.

There are some cases where power allocation to UL transmissions having lower priorities is not guaranteed when the total power would exceed PCMAX.

The simplest solution to make sure that the power is available for UL transmissions having lower priorities is to configure smaller values of PEMAX,c for UL carriers. However, this constantly reduces the max available power of the UL carriers and hence degrades the performance. The “additional UE-specific configuration of power limits on Pcmax,f,c for each CC to prevent SCell dropping” described in the RAN4 LS would have the same issue – the UE-specific configuration applies to the transmissions on the UL carriers irrespective of whether there is/are concurrent transmissions, whether the UE is in power-limited, and whether the concurrent transmissions have same/different priorities.

Observation 1:

- Configuring smaller values of PEMAX,c for UL carriers constantly reduces the available power of the UL carriers and hence degrades the performance.

- The solution “additional UE-specific configuration of power limits on Pcmax,f,c for each CC to prevent SCell dropping” would have the same problem as for smaller values of PEMAX,c.

Having said that, RAN1 discussion is necessary for the issue of UL-CA power-limited handling. Possible RAN1 spec impact has to be considered.

Proposal 1:

- RAN1 should discuss the issue of UL-CA power-limited handling.

As for the solution, we propose to introduce a higher-layer parameter for relative power offset for UL-CA power-limited handling. This offset is to reflect the priority difference for concurrent UL transmissions when the power-limited issue is resolved.

Proposal 2:

- Introduce a higher-layer parameter for relative power offset to reflect the priority difference for concurrent UL transmissions in case of power-limited.

- For UL-CA power-limited handling, the UE allocates transmission powers to the concurrent transmissions taking into account the total available power and the relative power offset.

Suppose a UE has two concurrent UL transmissions, U1 and U2, where U1 has higher priority than U2. The configured relative power offset is assumed to be D=3dB. The available power for each transmission and total available power for the UE are assumed to be all 23dBm. Then, the power-limited handling can be as follows.

- Case 0: If the total transmit power would not exceed 23dBm, the procedure does not apply.

- Case 1: If the total transmit power would exceed 23dBm and if the difference between the requested powers for U1 and U2 is not larger than D=3dB, the UE allocates power to U1 and U2 such that (1) total power does not exceed 23dBm and (2) the power difference between U1 and U2 equals to the configured offset, 3dB.

  - I.e., the offset between transmission powers for U1 and U2 is set to D=3dB and then the transmission powers for U1 and U2 are scaled to meet the total maximum power limitation.

- Case 2: If the total transmit power would exceed 23dBm and if the difference between the requested powers for U1 and U2 is larger than D=3dB, the UE allocates power to U1 and U2 such that total power does not exceed 23dBm.

  - I.e., the transmission powers for U1 and U2 are just scaled to meet the total maximum power limitation without using the offset.

(a) Case 1

(b) Case 2

Figure 1. UL-CA power-limited handling with relative power offset (Option 1).

The procedure of Option 1 in Fig. 1 can be expressed as follows.

- Find K that satisfies K×Preq_U1+Ptmp_U2≤PCMAX

  - where Ptmp_U2=minPreq_U2,   δ×Preq_U1, and

  - Preq_U1 and Preq_U2 are the requested transmission powers for U1 and U2 before power-limited handling, respectively

- PU1=minPreq_U1,   K×Preq_U1

- PU2=minPreq_U2,  PCMAX-PU1

Alternatively, following also works; the relative offset D dB is applied to U2 if the difference between Pmax⁡_U2 and Preq_U2 is not more than D dB, where Pmax⁡_U2 is the maximum available power for U2 transmission when there is no concurrent transmission(s) (e.g., PCMAX,f,c). The resulting power allocation of this option is quite similar to that of the previous option.

(a) Case 1

(b) Case 2

Figure 2. UL-CA power-limited handling with relative power offset (Option 2).

The procedure of Option 2 in  Fig. 2 can be expressed as follows.

- Find K that satisfies K×Preq_U1+Ptmp_U2≤PCMAX

  - where Ptmp_U2=minPreq_U2,   δ×Pmax⁡_U2, and

  - Preq_U1 and Preq_U2 are the requested transmission powers for U1 and U2 before power-limited handling, respectively, and Pmax⁡_U2 is the maximum available power for U2 when there is no concurrent transmission(s) (e.g., PCMAX,f,c)

- PU1=minPreq_U1,   K×Preq_U1

- PU2=minPreq_U2,  PCMAX-PU1

The priority order of power allocation in the RAN1 spec can be re-used for the priority determinations for the offset, i.e., in the above example, U1 and U2 can be determined based on the existing priority order. Furthermore, this solution can easily be extended to various other cases, e.g., more than two concurrent transmissions with two or more different priorities. We propose to adopt this as the solution for UL-CA power-limited handling.

Proposal 3:

- Adopt either of the following UL-CA power-limited handling:

  - Option 1:

    - Find K that satisfies K×Preq_U1+Ptmp_U2≤PCMAX

      - where Ptmp_U2=minPreq_U2,   δ×Preq_U1, and

      - Preq_U1 and Preq_U2 are the requested transmission powers for U1 and U2 before power-limited handling, respectively

    - Transmit power for U1 is given by PU1=minPreq_U1,   K×Preq_U1

    - Transmit power for U2 is given by PU2=minPreq_U2,  PCMAX-PU1

  - Option 2:

    - Find K that satisfies K×Preq_U1+Ptmp_U2≤PCMAX

      - where Ptmp_U2=minPreq_U2,   δ×Pmax⁡_U2, and

      - Preq_U1 and Preq_U2 are the requested transmission powers for U1 and U2 before power-limited handling, respectively, and Pmax⁡_U2 is the maximum available power for U2 when there is no concurrent transmission(s) (e.g., PCMAX,f,c)

    - Transmit power for U1 is given by PU1=minPreq_U1,   K×Preq_U1

    - Transmit power for U2 is given by PU2=minPreq_U2,  PCMAX-PU1

# Conclusion

In this contribution we provided our views on SCell dropping issue of CA. Our observation and proposals are following.

Observation 1:

- Configuring smaller values of PEMAX,c for UL carriers constantly reduces the available power of the UL carriers and hence degrades the performance.

- The solution “additional UE-specific configuration of power limits on Pcmax,f,c for each CC to prevent SCell dropping” would have the same problem as for smaller values of PEMAX,c.

Proposal 1:

- RAN1 should discuss the issue of UL-CA power-limited handling.

Proposal 2:

- Introduce a higher-layer parameter for relative power offset to reflect the priority difference for concurrent UL transmissions in case of power-limited.

- For UL-CA power-limited handling, the UE allocates transmission powers to the concurrent transmissions taking into account the total available power and the relative power offset.

Proposal 3:

- Adopt either of the following UL-CA power-limited handling:

  - Option 1:

    - Find K that satisfies K×Preq_U1+Ptmp_U2≤PCMAX

      - where Ptmp_U2=minPreq_U2,   δ×Preq_U1, and

      - Preq_U1 and Preq_U2 are the requested transmission powers for U1 and U2 before power-limited handling, respectively

    - Transmit power for U1 is given by PU1=minPreq_U1,   K×Preq_U1

    - Transmit power for U2 is given by PU2=minPreq_U2,  PCMAX-PU1

  - Option 2:

    - Find K that satisfies K×Preq_U1+Ptmp_U2≤PCMAX

      - where Ptmp_U2=minPreq_U2,   δ×Pmax⁡_U2, and

      - Preq_U1 and Preq_U2 are the requested transmission powers for U1 and U2 before power-limited handling, respectively, and Pmax⁡_U2 is the maximum available power for U2 when there is no concurrent transmission(s) (e.g., PCMAX,f,c)

    - Transmit power for U1 is given by PU1=minPreq_U1,   K×Preq_U1

    - Transmit power for U2 is given by PU2=minPreq_U2,  PCMAX-PU1

# Reference

- RP-212527, ‘WID revision: RF requirements enhancement for NR frequency range 1 (FR1)’, 	Huawei, HiSilicon

- R1-2108701, ‘LS on scell dropping issue of CA’, RAN4, Huawei