3GPP TSG RAN WG1 Meeting #106bis-e    	            R1-2110214

E-meeting, October 11th – 19th, 2021

Source:	Qualcomm Incorporated

Title:	Efficient activation/de-activation mechanism for SCells in NR CA

Agenda Item:	8.13.2

Document for: 	Discussion and Decision

# Background

At the RAN1#105-e meeting, following agreements have been made [1]:

Agreement For efficient SCell activation, the earliest slot for a UE to receive a triggered temporary RS is the reference slot (i.e., the last DL slot of the to-be-activated Scell overlapping with slot n+k as defined in 38.213 sub-clause 4.3). Agreement For to-be-activated SCell, if any BWP ID is configured as part of temporary RS(s) configuration, the value of the BWP ID is expected to be equal to firstActiveDownlinkBWP-Id; Agreement To trigger temporary RS, - MAC-CE at least provides the following information: - temporary RSs are to be triggered on X out of Y (Y≥X) to-be-activated SCells, respectively, while no temporary RS is to be triggered on the other to-be-activated SCells. - The following information can be provided by RRC for temporary RS for each SCell - The number of RS bursts and the gap length between the RS bursts (Opt 2.3.3) - Triggering offset of temporary RS (Opt 2.3.4) - Triggering offset can be provided, e.g., by reusing existing CSI-RS framework - QCL information (Opt 2.3.5) - Triggering QCL information can be provided, e.g., by reusing existing CSI-RS framework - A unique temporary RS configuration index - FFS: the maximum number of temporary RS per cell/per UE Note: Reusing A-TRS triggering framework is not precluded. - Information for 0, 1, or more temporary RS can be provided for each configured SCell Agreement - For triggering temporary RS, down-select based on the following alternatives, or let RAN2 be aware the status of this discussion - Alt 1: Bitmap approach in MAC-CE similar to SCell activation - Every Z-bit block in the bitmap corresponds to a SCell, Z>=0 - A Z-bit block indicates the temporary RS [configuration index], and a value zero indicated by the bit block means no RS resource transmitted. - The to-be-activated SCell is indicated via the C values in the legacy SCell activation/de-activation MAC CE or in the new MAC-CE - Alt 2: Reuse A-TRS triggering framework - A trigger state is indicated by the MAC-CE explicitly - The association between a trigger state and aperiodic temporary RS for one or multiple SCells is configured by RRC according Rel-16 A-TRS triggering framework - SCell ID is configured as a part of the temporary RS configuration. Some SCell IDs derived from the trigger state triggered by the new MAC-CE may not refer to to-be-activated SCells that are indicated by the new MAC-CE or the legacy SCell activation/de-activation MAC-CE - FFS: The value zero of the MAC-CE indication means no temporary RS is triggered by the MAC-CE for all to-be-activated SCells - Note: The down-selection targets at a RAN1 consensus on MAC-CE functionality and the list of RRC parameters for this feature. Any MAC-CE signaling design above are reference concept, its final MAC-CE signaling design is up to RAN2.

Meanwhile, RAN4 has provided their feedback on temporary RS structures for different cases:

[R1-2102300] - SCell to be activated is known and belongs to FR1 - If SCell measurement cycle is equal to or smaller than 160ms - 1 burst (2-slot with four CSI-RS resources) is necessary for time/frequency tracking - If SCell measurement cycle is larger than 160ms - 1 burst (2-slot with four CSI-RS resources) is necessary for AGC - 1 separate burst (2-slot with four CSI-RS resources) is necessary for time/frequency tracking in addition to the one for AGC - Minimum gap between the RS symbol(s) for AGC and the RS symbols for time/frequency acquisition is, - Option 1: 2 slots - Option 2: 2 ms - SCell is unknown and belongs to FR1 - When SCell is contiguous to an active serving cell in the same band (Intra-band continuous CA) - 1 burst (2-slot with four CSI-RS resources) is necessary for AGC when the power difference in serving cell and to be activated Scell is smaller than or equal to 6dB - No cell detection provided the conditions specified for intra-band contiguous CA case in TS38.133 section 8.3.2 are satisfied; - 1 burst (2-slot with four CSI-RS resources) is necessary for time-frequency tracking - SCell belongs to FR2 - If there is at least one active serving cell on that FR2 band and temporary RS for the target SCell is provided - 1 burst (2-slot with four CSI-RS resources) is necessary for time-frequency tracking - If there is no active serving cell on that FR2 band, and the SCell to be activated is known to UE - 1 burst (2-slot with four CSI-RS resources) is necessary for time-frequency tracking - If the SCell being activated is unknown and there is no active serving cell on that FR2 band, - Temporary RS cannot be used for AGC while no conclusion on temporary RS use for time/frequency tracking

[R1-2106427] - SCell to be activated is known and belongs to FR1 and if SCell measurement cycle is larger than 160ms, It is confirmed in [R4-2104067] [R4-2105799] that - Temporary RS can be used for AGC - 1 burst (2-slot with four CSI-RS resources) is required - Temporary RS can be used for time/frequency tracking - 1 separate burst (2-slot with four CSI-RS resources) is required in addition to the one burst required for AGC - Minimum gap between the RS symbol(s) for AGC and the RS symbols for time/frequency acquisition is needed to account for UE AGC application time delay. In this meeting RAN4 further discussed the minimum gap length and still no consensus was achieved. RAN4 will continue the discussion and provide feedback to RAN1 if there is conclusion. - SCell is unknown and belongs to FR1 - When SCell to be activated is non-contiguous to an active serving cell in the same band, or - When SCell to be activated and active serving cell are in the different band - It is not a target scenario for temporary RS based SCell activation latency optimization. - The agreement above applies based on RAN1 working assumptions on temporary RS design provided in the LS R1-2009798 - SCell to be activated belongs to FR2 - If the SCell being activated is unknown and there is no active serving cell on that FR2 band, - It is not a target scenario for temporary RS based SCell activation latency optimization. - The agreement above applies based on RAN1 working assumptions on temporary RS design provided in the LS R1-2009798

[R4-2115370] - SCell to be activated is known and belongs to FR1 and if the measurement period of the SCell being activated is larger than [2400ms]. It is confirmed in previous LS reply that - Temporary RS can be used for AGC - 1 burst (2-slot with four CSI-RS resources) is required - Temporary RS can be used for time/frequency tracking - 1 separate burst (2-slot with four CSI-RS resources) is required in addition to the one burst required for AGC - Minimum gap between the RS symbol(s) for AGC and the RS symbols for time/frequency acquisition is needed to account for UE AGC application time delay. In this meeting RAN4 further discussed and agreed that the minimum gap length is - For 15kHz and 30kHz SCS: 2 slots - For 60kHz SCS: 3 slots In addition, RAN4 would like to inform RAN1 an information that for the case where SCell to be activated is known and belongs to FR1, the condition of “SCell measurement cycle is larger than/less than/equal to 160ms” which was used in previous reply LS is replaced by the condition “the measurement period of the SCell being activated is larger than/less than/equal to [2400ms]” in RAN4.

In this contribution, we provide our views on detailed aspects of efficient SCell activation using temporary RS.

# Signalling and configuration for Temporary RS

According to the RAN1’s WA and RAN4’s feedback, a temporary RS can be formed by one or two bursts of tracking RS. Since the structure of a tracking RS has already been specified in TS38.214 Section 5.1.6.1.1 and all necessary configuration/signalling are available in ASN.1, all the necessary building blocks for constructing a temporary RS are already in the spec. The only necessary change would be an interpretation of a parameter as follows:

- Indication for the slot(s) where a temporary RS is transmitted

  - For legacy aperiodic CSI-RS, aperiodicTriggeringOffset or aperiodicTriggeringOffset-r16 configured for each NZP-CSI-RS resource set indicates the slot-level offset from the slot where the triggering DCI is detected to the slot where the CSI-RS according to the NZP-CSI-RS resource set is transmitted.

  - However, for temporary RS based on option 1b, there will be no triggering DCI – instead, a MAC-CE triggers it. Besides, it was agreed that the reference slot for triggering offset of temporary RS is the last DL slot of the to-be-activated SCell overlapping with slot n+k as defined in 38.213 sub-clause 4.3. Therefore, the triggering offset indication should be based on this reference slot.

  - For this, the current configuration framework can be maintained with re-interpreting the value of aperiodicTriggeringOffset or aperiodicTriggeringOffset-r16 as the number of slots from the last DL slot of the to-be-activated SCell overlapping with slot n+k as defined in 38.213 sub-clause 4.3. Note that it is not possible to indicate minus value by aperiodicTriggeringOffset or aperiodicTriggeringOffset-r16.

It was agreed that a temporary RS is an aperiodic RS. Since triggering/activation of aperiodic CSI-RS has already been specified in TS38.214 Sections 5.2.1.5.1 and 5.2.1.5.1a and all necessary configuration/signalling are available in ASN.1, all the necessary building blocks for triggering a temporary RS are already in the spec. In order to support multiple bursts of tracking RS forming a temporary RS, support of a larger number of NZP-CSI-RS-Resource associated with a triggering state is required (compared to legacy tracking RS). Nevertheless, this does not require spec change.

According to the legacy CSI framework, the carrier(s) where aperiodic CSI-RS is triggered is configured by carrier in the associated CSI-ReportConfig linked to the CSI-ResourceConfig for the aperiodic CSI-RS. This parameter can be re-used for temporary RS for SCell activation to configure on which carrier(s) it is triggered. For a given carrier, the DL BWP where the aperiodic CSI-RS is transmitted is configured by bwp-id in each CSI-ResourceConfig. Considering that the first active DL BWP for a to-be-activated SCell is configured by firstActiveDownlinkBWP-id in the ServingCellConfig for the carrier, this bwp-id in each CSI-ResourceConfig for temporary RS for SCell activation is indeed redundant. However, it is not necessary to change the spec – network can ensure that for triggering temporary RS, bwp-id in each CSI-ResourceConfig is not provided or is aligned with firstActiveDownlinkBWP-id in the ServingCellConfig. Overall, for aperiodic triggering a temporary RS, the only necessary change compared to legacy is to use a MAC-CE instead of a DCI.

Fig.1 Diagram of ASN.1 signalling structure of aperiodic RS triggering

Proposal 1: Re-use the existing CSI-RS configuration framework with the following change

- A burst on a serving cell is a tracking RS as specified in TS38.214 5.1.6.1.1

  - Four NZP-CSI-RS resources over two consecutive slots form one or multiple NZP-CSI-RS resource sets that forms a burst

  - For each NZP-CSI-RS resource in a NZP-CSI-RS resource set:

    - Time-domain locations of four CSI-RS resources in two consecutive slots for a burst is {4, 8}, {5, 9}, or {6, 10}

    - Single antenna port is configured

    - Density p=3 is configured

  - For each NZP-CSI-RS resource set:

    - Higher layer parameter trs-Info is configured

    - Offset parameter (e.g., aperiodicTriggeringOffset) is provided where the slot with offset = 0 is the last DL slot of the to-be-activated Scell overlapping with slot n+k as defined in 38.213 sub-clause 4.3

- One or multiple bursts form a temporary RS on a serving cell

Proposal 2: Re-use the existing aperiodic triggering framework for a temporary RS with the following change

- A triggering state list that includes one or multiple triggering state is configured

  - Each triggering state is associated with an indication by the MAC-CE

  - Each triggering state configuration includes a list of {resourceSet, qcl-Info}, where:

    - resourceSet = one of NZP-CSI-ResourceSet provided by a CSI-RS-ResourceConfig associated with a CSI-RS-ReportConfig

      - carrier in the CSI-ReportConfig informs on which carrier the CSI-RS is transmitted

      - bwp-id in the CSI-ResourceConfig should be either (1) not configured or (2) same as firstActiveDownlinkBWP-Id in the ServingCellConfig for the to-be-activated SCell

    - qcl-Info = list of TCI-StateId to inform the QCL source/type for each NZP-CSI-RS-Resource in the NZP-CSI-RS-ResourceSet configured as resourceSet

Based on the proposals 1 and 2, the only open issue is whether the CSI-AperiodicTriggerStateList for triggering temporary RS via MAC-CE is the common/shared list as for general/existing aperiodic RS triggering by UL DCI. We are OK to consider a separate/dedicated list for temporary RS triggering, subject to the understanding that the UE reports maximum numbers of trigger states and maximum number of temporary RS configurations per trigger state.

Proposal 3: FFS whether a separate list of aperiodic trigger of temporary RS is necessary other than the existing CSI-AperiodicTriggerStateList

# Conclusion

In this contribution we proposed designs of temporary RS structure and its triggering signalling as follows.

Proposal 1: Re-use the existing CSI-RS configuration framework with the following change

- A burst on a serving cell is a tracking RS as specified in TS38.214 5.1.6.1.1

  - Four NZP-CSI-RS resources over two consecutive slots form one or multiple NZP-CSI-RS resource sets that forms a burst

  - For each NZP-CSI-RS resource in a NZP-CSI-RS resource set:

    - Time-domain locations of four CSI-RS resources in two consecutive slots for a burst is {4, 8}, {5, 9}, or {6, 10}

    - Single antenna port is configured

    - Density p=3 is configured

  - For each NZP-CSI-RS resource set:

    - Higher layer parameter trs-Info is configured

    - Offset parameter (e.g., aperiodicTriggeringOffset) is provided where the slot with offset = 0 is the last DL slot of the to-be-activated Scell overlapping with slot n+k as defined in 38.213 sub-clause 4.3

- One or multiple bursts form a temporary RS on a serving cell

Proposal 2: Re-use the existing aperiodic triggering framework for a temporary RS with the following change

- A triggering state list that includes one or multiple triggering state is configured

  - Each triggering state is associated with an indication by the MAC-CE

  - Each triggering state configuration includes a list of {resourceSet, qcl-Info}, where:

    - resourceSet = one of NZP-CSI-ResourceSet provided by a CSI-RS-ResourceConfig associated with a CSI-RS-ReportConfig

      - carrier in the CSI-ReportConfig informs on which carrier the CSI-RS is transmitted

      - bwp-id in the CSI-ResourceConfig should be either (1) not configured or (2) same as firstActiveDownlinkBWP-Id in the ServingCellConfig for the to-be-activated SCell

    - qcl-Info = list of TCI-StateId to inform the QCL source/type for each NZP-CSI-RS-Resource in the NZP-CSI-RS-ResourceSet configured as resourceSet

Proposal 3: FFS whether a separate list of aperiodic trigger of temporary RS is necessary other than the existing CSI-AperiodicTriggerStateList

# Reference

- 3GPP RAN1#105-e, Chairman’s note.

- R1-2102300, Reply LS on temporary RS for efficient SCell activation in NR CA, RAN4

- R1-2106427, Reply LS on temporary RS for efficient SCell activation in NR CA, RAN4

- R4-2115370, Reply LS on temporary RS for efficient SCell activation in NR CA, RAN4