3GPP TSG-RAN WG1 Meeting #106-bis-e	R1-2110219

e-Meeting, October 11th – October 19th, 2021

Agenda item:	8.17.1

Source: 	Qualcomm Incorporated

Title: 	Discussion on FeMIMO UE features

Document for:	Discussion/Decision

- Introduction

In R1-2108679, preliminary RAN1 UE features for Rel-17 NR provided from WI rapporterurs are provided. It is understood that the list provided in R1-2108679 is only for information for preparation of RAN1#106bis-e and all formal discussion/decision on Rel-17 UE features will start from RAN1#106bis-e.

In this contribution, we discuss our views on various FeMIMO UE features.

- Multi-beam Operation

In the preliminary UE feature list (R1-2108679), a single capability is defined for unified TCI for both intra-BM and inter-cell BM. Ideally, they belong to two different features and should have separate capabilitites, e.g. UE supporting inter-cell BM should additionally support QCL source RS from non-serving SSB as well as L1-RSRP measurement on non-serving SSB.

Proposal 1-1: The unified TCI for intra-cell BM and inter-cell BM should have separate capabilities, e.g. FG 23-1-1 and FG 23-1-2.

For intra-cell BM, we also have the following proposals

Proposal 1-2: As in FG 23-1-1, supporting intra-cell BM implies supporting the following basic sub-features

- Support of joint DL/UL TCI state

- Support of TCI-independent UL PC settings except for PL RS

- Support individual BWP/CC based TCI pool configuration as in R15/16

- Support of MAC-CE based beam indication

- Support of MAC-CE+DCI based beam indication with DL assignment

  - Only for UE supporting more than 1 active TCI state

Proposal 1-3: UE supporting intra-cell BM may optionally support the following sub-features

- Support of separate DL/UL TCI state (FG 23-1-1a)

- Support of TCI-specific UL PC settings except for PL RS (FG 23-1-1b)

- Support of beam misalignment for PL RS and associated UL beam (FG 23-1-1c)

- Support of MAC-CE + DCI based beam indication without DL assignment (FG 23-1-1d)

- Support of a common TCI state ID activated/selected for a set of BWPs/CCs (FG 23-1-1f)

- Support a TCT state pool configured on a reference BWP/CC shared by a set of BWPs/CCs (FG 23-1-1h)

Proposal 1-4: UE supporting intra-cell BM should clarify the following capabilities

- Minimum time duration between beam indication DCI and TCI application time (FG 23-1-1e)

- Maximum number of configured TCI state pools across BWPs/CCs (FG 23-1-1h)

- Maximum number of activated and configured TCI states (FG 23-1-1i)

| 23. NR_FeMIMO | 23-1-1 | Unified TCI for intra-cell beam management | - Support of joint DL/UL TCI state - Including QCL rules, applicable source and target signals - Support of TCI-independent UL PC settings except for PL RS - Support individual BWP/CC based TCI pool configuration as in R15/16 - Support of MAC-CE based beam indication - Support of MAC-CE + DCI based beam indication with DL assignment - Including DCI format 1_1 and 1_2 |  |  |  |  | Per band |  |  |  | Note: Component 4 is only supported by UE supporting more than 1 active TCI state | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-1-1a | Unified TCI for intra-cell beam management with separate DL/UL TCI | - Support of separate DL/UL TCI state - Including QCL rules, applicable source and target signals | FG 23-1-1 |  |  |  | Per band |  |  |  |  | Optional |
| 23. NR_FeMIMO | 23-1-1b | Unified TCI for intra-cell beam management with TCI-specific UL PC settings except for PL RS | - Support of TCI-specific UL PC settings except for PL RS | FG 23-1-1 |  |  |  | Per band |  |  |  |  | Optional |
| 23. NR_FeMIMO | 23-1-1c | Unified TCI for intra-cell beam management with beam misalignment for PL RS | - Support of beam misalignment for PL RS and associated UL beam | FG 23-1-1 |  |  |  | Per band |  |  |  | Note: | Optional |
| 23. NR_FeMIMO | 23-1-1d | Unified TCI for intra-cell beam management with DCI based beam indication without DL assignment | - Support of MAC-CE + DCI based beam indication without DL assignment - Including DCI format 1_1 and 1_2 | FG 23-1-1 |  |  |  | Per band |  |  |  | Note: | Optional |
| 23. NR_FeMIMO | 23-1-1e | Time duration between DCI and TCI application time | - Minimum time duration, Xi, between the last symbol of the beam indication DCI and the 1st slot to apply the TCI indicated by the DCI - Xi is the number of OFDM symbols, i is the index of SCS, l=1,2,3,4 corresponding to 60,120,480, 960 kHz SCS. | FG 23-1-1 |  |  |  | Per band |  |  |  | Note: Applicable only to FR2 | Candidate value set for Xi is {…}, |
| 23. NR_FeMIMO | 23-1-1f | Unified TCI for intra-cell beam management with common TCI ID selected for a group of CCs | - Support of a common TCI state ID activated or selected for a set of BWPs/CCs | FG 23-1-1 |  |  |  | Per band |  |  |  | Note: Supported TCI type is indicated in FG 23-1-1 and FG 23-1-1a | Optional |
| 23. NR_FeMIMO | 23-1-1g | Unified TCI for intra-cell beam management with configured TCI pool shared by a group of CCs | - Support a TCI state pool configured on a reference BWP/CC shared by a set of BWPs/CCs | FG 23-1-1 |  |  |  | Per band |  |  |  | Note: Supported TCI type is indicated in FG 23-1-1 and FG 23-1-1a | Optional |
| 23. NR_FeMIMO | 23-1-1h | Maximum number of configured TCI pool across CCs | - Maximum number of configured TCI state pools across BWPs/CCs | FG 23-1-1 |  |  |  | Per band |  |  |  | Note: Supported TCI type is indicated in FG 23-1-1 and FG 23-1-1a | Optional |
| 23. NR_FeMIMO | 23-1-1i | Maximum number of activated and configuredTCI states | - Maximum number of activated TCI states per BWP/CC - Maximum number of configured TCI states per BWP/CC - Maximum number of activated TCI states across BWPs/CCs in a band - Maximum number of configured TCI states across BWPs/CCs in a band | FG 23-1-1 |  |  |  | Per band |  |  |  | Note 1: Supported TCI type is indicated in FG 23-1-1 and FG 23-1-1a Note 2: Maximum number in each component is across all supported TCI types | Component -1: Candidate value set: {1, 2, 4, 8} Component-2: candidate value set: {4, 8, 16, 32, 64, 128} Component -3: Candidate value set: {…} Component-4: candidate value set: {…} |

Proposal 1-5: As in FG 23-1-2, supporting inter-cell BM implies supporting SSB from non-serving cell as direct/indirect beam indication RS in unified TCI state

- This should be a separate capability from intra-cell BM

| 23. NR_FeMIMO | 23-1-2 | Unified TCI for intra-cell beam management | 1. Support of SSB from non-serving cell as direct/indirect QCL source RS and/or spatial relation RS in unified TCI state | FG 23-1-1 |  |  |  | Per band |  |  |  | Note: | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 1-6: As in FG 23-1-3, the support of inter-cell measurement/reporting comprises the following 3 components

- Maximum number of configured non-serving cells for L1-RSRP measurement (Component 1 in FG 23-1-3)

- Maximum number of reported DL RSs in a beam report (Component 2 in FG 23-1-3)

- Maximum number of time overlapped SSBs for L1-RSRP measurement (Component 3 in FG 23-1-3)

| 23. NR_FeMIMO | 23-1-3 | Inter-cell measurement and reporting (for inter-cell BM and mTRP) | 1. Maximum number of configured non-serving cells for L1-RSRP measurement 2. Maximum number of reported DL RSs in a beam report with measured DL RSs from both serving and non-serving cells 3. Maximum number of time overlapped SSBs from different cells for L1-RSRP measurement | FG 23-1-2, FG 23-4 |  |  |  | Per band |  |  |  |  | Component -1: Candidate value set: {0,1} Component-2: candidate value set: {0,1,2,3,4} Component -3: Candidate value set: {0,1} |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 1-7: As in FG 23-1-4, the support of MPE mitigation can be indicated by the maximum number of additionally reported P-MPR values per CC in the enhanced P-MPR report.

| 23. NR_FeMIMO | 23-1-4 | MPE mitigation | 1. Maximum number of additionally reported P-MPR value(s) per CC in enhanced P-MPR report | FG 23-1-1a |  |  |  | Per band |  |  |  |  | Candidate value set: {0,1,2,3} |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 1-8: As in FG 23-1-5, the support of multiple SRS resource sets with different SRS antenna port number includes the following components

- Maximum number of SRS resource sets with different number of SRS antenna ports (Component 1 in FG 23-1-5)

- Maximum number of SRS resources per set with different number of SRS antenna ports (Component 2 in FG 23-1-5)

Proposal 1-9: As listed in FG 23-1-5a, UE supporting FG 23-1-5 can further indicate whether the reported SRS antenna port number can be applied to a set of BWPs/CCs when the associated reported DL RS indicates a common UL beam for the set of BWPs/CCs

| 23. NR_FeMIMO | 23-1-5 | Multiple SRS resource sets with different number of SRS antenna ports | 1. Maximum number of SRS resource sets with different number of SRS antenna ports 2. Maximum number of SRS resources per set with different number of SRS antenna ports |  |  |  |  | Per band |  |  |  |  | Component -1: Candidate value set: {0,1,2} Component-2: candidate value set: {1,2,3,4} |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-1-5a | Reported SRS antenna port number applicable to multiple BWPs/CCs | 1. Support of reported SRS antenna port number applicable to a set of BWPs/CCs when the associated reported DL RS indicates a common UL beam for the set of BWPs/CCs | FG 23-1-5 |  |  |  | Per band |  |  |  |  | Optional |

- mTRP PDCCH

In Rel-15, PDCCH monitoring capability is defined for different cases including Case 1-1 for basic capability (FG 3-1), Case 1-1 for single occasion monitoring (FG 3-2), and Case 2 for PDCCH monitoring with span gap (FG 3-5b). Furthermore, Rel-16 introduced additional capabilities (FG 11-2 family). The support of Rel-17 mTRP PDCCH repetition should be separately indicated for each of these PDCCH monitoring capabilities. In other words, the basic FG for Rel-17 mTRP PDCCH repetition should only include the basic PDCCH monitoring capability. This is because PDCCH repetition can significantly increase the UE complexity and some of the advanced PDCCH monitoring features that the UE is able to support in the absence of PDCCH repetition need a separate capability when combined with PDCCH repetition.

Proposal 2-1: The basic FG for Rel-17 mTRP PDCCH repetition (FG 23-2-1) should only include the basic PDCCH monitoring capability. PDCCH repetition for more advanced PDCCH monitoring capabilities need a separate capability.

Furthermore, for span-based PDCCH monitoring (either Rel-15 version or Rel-16 version), the UE processing complexity and memory requirements are different depending on whether PDCCH repetitions are within the same span or in different spans. Hence, UE capability for such indication is needed.

Proposal 2-1: For span-based PDCCH monitoring (either Rel-15 version or Rel-16 version), UE can indicate if PDCCH repetitions can be intra-span or inter-span.

Based on the above discussions, we propose the following FGs for mTRP PDCCH:

Proposal 2-3: Adopt the following for Rel-17 mTRP PDCCH UE features.

| 23. NR_FeMIMO | 23-2-1 | Multi-TRP PDCCH repetition | 1. Support of PDCCH repetition (based on two linked SS sets associated with corresponding CORESETs) 2. Support of reporting one [or more] number(s) as required number of BDs for the two PDCCH candidates 3. Support of whether the individual candidate is monitored or not when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET 4. Maximum number of such overlaps in 3 per scheduled cell per slot |  |  |  |  | Per FS |  |  |  | Note: For PDCCH monitoring capability, the following is assumed: For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, the monitoring occasion is within the first 3 OFDM symbols of a slot. Component2: {2, 3, [{2,3}]} Component3: {monitored, not monitored} Component4: {2,4,6,10,20,40} | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-2-1a | Multi-TRP PDCCH repetition with PDCCH monitoring on any span of up to 3 consecutive OFDM symbols of a slot | Support of PDCCH repetition for PDCCH monitoring on any span of up to 3 consecutive OFDM symbols of a slot | FG 3-2 FG 23-2-1 |  |  |  | Per Band |  |  | FR1 only |  | Optional |
| 23. NR_FeMIMO | 23-2-1b | Multi-TRP PDCCH repetition for Case 2 PDCCH monitoring with a span gap | 1. Support of PDCCH repetition for PDCCH monitoring of any occasions with span gap as defined in FG 3-5b. 2. Supported mode of PDCCH repetition | FG 3-5b FG 23-2-1 |  |  |  | Per FS |  |  |  | This capability is necessary for each SCS. Component2: {intra-span, inter-span, both} | Optional |
| 23. NR_FeMIMO | 23-2-1c | Multi-TRP PDCCH repetition for Rel-16 PDCCH monitoring | 1. Support of PDCCH repetition with Rel-16 PDCCH monitoring capability as defined in FG 11-2 family. 2. Supported mode of PDCCH repetition 3. Maximum number of overlaps between individual candidates and linked candidates per scheduled cell per span | FG 11-2 FG 23-2-1 |  |  |  | Per FS |  |  |  | This capability is signalled for SCS 15 kHz and 30 kHz. Component2: {intra-span, inter-span, both} Component3: {1,2,3,5,10,20} | Optional |
| 23. NR_FeMIMO | 23-2-2 | Two QCL TypeD for PDCCH repetition | Support of determining two QCL-TypeD for overlapping CORESETs in the same CC or for intra-band CA when UE is configured with PDCCH repetition | FG 23-2-1 |  |  |  | Per Band |  |  | FR2 only |  | Optional |
| 23. NR_FeMIMO | 23-2-3 | PDCCH repetition for Type3 CSS | Support of PDCCH repetition for Type3 CSS | FG 23-2-1 |  |  |  | Per Band |  |  |  |  | Optional |

- mTRP PUSCH and PUCCH

The following aspects need to be considered when designing UE capability for mTRP PUSCH repetitions:

- Separate capabilities are required for codebook-based and non-codebook based mTRP PUSCH.

  - For codebook-based mTRP PUSCH, the structure of Rel-15 capability (FG 2-14) including indication on max number of SRS resources need to be taken in to account, and should be extended for the case that two SRS resource sets are configured.

  - For non-codebook based mTRP PUSCH, the structure of Rel-15 capability (FGs 2-15, 2-15a, 2-15b) including indication on max number of SRS resources and various capabilities for associated CSI-RS (for precoding calculation) need to be taken in to account, and should be extended for the case that two SRS resource sets / two associated CSI-RS resources are configured.

- Separate capabilities for mTRP PUSCH repetition are required for repetition Type A and repetition Type B.

- Capabilities are needed for transmitting AP/SP-CSI on two PUSCH repetitions.

- Per TRP default power control pramaters or per TRP OLPC set indication features require UE capability.

Based on the above discussions, we propose the following FGs for mTRP PUSCH:

Proposal 3-1: Adopt the following for Rel-17 mTRP PUSCH UE features.

| 23. NR_FeMIMO | 23-3-1a | Multi-TRP PUSCH repetition for codebook based | 1. Support of codebook-based multi-TRP PUSCH repetition for repetition Type A 2. Support of two SRS resource sets with usage set to codebook 3. Supported number of SRS resources in one SRS resource set 4. Supported number of SRS resources across two SRS resource sets | FG 2-14 |  |  |  | Per FSPC |  |  |  | Note: For more than 2 repetitions, sequential beam mapping is supported as part of this FG. Component 3: {1,2,4} Component 4: {2,3,4,5,6,8} | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-3-1b | Multi-TRP PUSCH repetition for non-codebook based | 1. Support of non-codebook-based multi-TRP PUSCH repetition for repetition Type A 2. Support of two SRS resource sets with usage set to noncodebook 3. Supported number of SRS resources in one SRS resource set 4. Supported number of SRS resources across two SRS resource sets | FG 2-15 |  |  |  | Per FSPC |  |  |  | Note: For more than 2 repetitions, sequential beam mapping is supported as part of this FG. Component 3: {1,2,3,4} Component 4: {2,3,4,5,6,7,8} | Optional |
| 23. NR_FeMIMO | 23-3-1b-1 | Two associated CSI-RS resources | Support of up to two CSI-RS resources associated with the two SRS resource sets for non-codebook-based mTRP PUSCH | FG 2-15a FG 23-3-1b |  |  |  | Per Band |  |  |  |  | Optional |
| 23. NR_FeMIMO | 23-3-1b-2 | CSI-RS processing framework for SRS with two associated CSI-RS resources | 1. Maximum number of periodic SRS resources associated with first or second CSI-RS per BWP 2. Maximum number of aperiodic SRS resources associated with first or second CSI-RS per BWP 3. Maximum number of semi-persistent SRS resources associated with first or second CSI-RS per BWP 4. UE can process Y SRS resources associated with first or second CSI-RS resources simultaneously in a CC. Includes P/SP/A SRS. | FG 2-15a FG 23-3-1b |  |  |  | Per Band |  |  |  | Component 1: {1 to 8} Component 2: {1 to 8} Component 3: {1 to 8} Component 4: {1 to 16} | Optional |
| 23. NR_FeMIMO | 23-3-1c | Multi-TRP PUSCH repetition for repetition Type B | Support of multi-TRP PUSCH repetition for repetition Type B | FG 11-5 FG 23-3-1a or 23-3-1b |  |  |  | Per FS |  |  |  |  | Optional |
| 23. NR_FeMIMO | 23-3-1d | Two TPC fields in UL DCI | Support of second TPC field for per TRP closed-loop power control for PUSCH with DCI formats 0_1 / 0_2 | FG 8-8 FG 23-3-1a or 23-3-1b |  |  |  | Per Band |  |  |  |  | Optional |
| 23. NR_FeMIMO | 23-3-1e | Cyclic beam mapping | Support of cyclic mapping when the number of repetitions is larger than 2 | FG 23-3-1a or 23-3-1b |  |  |  | Per Band |  |  |  |  | Optional |
| 23. NR_FeMIMO | 23-3-1f | Two PHRs per CC | Support of reporting two PHRs per CC (for a CC configured with mTRP PUSCH repetition) | FG 23-3-1a or 23-3-1b |  |  |  | Per Band |  |  |  |  | Optional |
| 23. NR_FeMIMO | 23-3-1g | Multi-TRP PUSCH repetition for configured grant | 1. Support of multi-TRP PUSCH repetition for configured grant 2. Supported type of configured grant for multi-TRP PUSCH repetition | FG 23-3-1a or 23-3-1b |  |  |  | Per Band |  |  |  | Component 2: {Type1, Type2, both} | Optional |
| 23. NR_FeMIMO | 23-3-1h | AP-CSI on two PUSCH repetitions | Support of transmitting AP-CSI on two PUSCH repetitions | FG 23-3-1a or 23-3-1b |  |  |  | Per Band |  |  |  |  | Optional |
| 23. NR_FeMIMO | 23-3-1i | SP-CSI on two PUSCH repetitions | Support of transmitting SP-CSI on two PUSCH repetitions | FG 23-3-1a or 23-3-1b |  |  |  | Per Band |  |  |  |  | Optional |
| 23. NR_FeMIMO | 23-3-1j | Per TRP OLPC set configurations | Support configurations of two (per TRP) p0-PUSCH-SetList | FG 11-8 FG 23-3-1a or 23-3-1b |  |  |  | Per Band |  |  |  |  | Optional |
| 23. NR_FeMIMO | 23-3-1k | Per TRP default power control parameters | Support of two (per TRP) sets of default power control parameters when SRI fields are absent from the DCI | FG 23-3-1a or 23-3-1b |  |  |  | Per Band |  |  |  |  | Optional |

The following aspects need to be considered when designing UE capability for mTRP PUCCH repetitions:

- Separate capabilities are needed for mTRP PUCCH Scheme 1 versus Scheme 3

  - Supported PUCCH format(s) need to be indicated for each of the Scheme 1 and Scheme 3

  - For the details of the capability for Scheme 3, we need to consider how the capability of sub-slot based PUCCH repetition with sTRP in Rel-17 IIoT AI is indicated.

- Updating two beams / power control parameters for a group of PUCCH resources requires a UE capability

Proposal 3-2: Adopt the following for Rel-17 mTRP PUCCH UE features.

| 23. NR_FeMIMO | 23-3-2 | Multi-TRP PUCCH repetition | 1. Support of multi-TRP PUCCH repetition scheme 1 (inter-slot repetition) 2. Support of updating two Spatial Relation Info’s / two sets of power control parameters for a PUCCH resource by MAC-CE 3. Supported PUCCH formats for this scheme |  |  |  |  | Per FS |  |  |  | Note: For more than 2 repetitions, sequential beam mapping is supported as part of this FG. Component 3: A bit map of size 5 for PUCCH format 0-4. At least one value of the bitmap should be set to 1. | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-3-2a | Multi-TRP PUCCH repetition with sub-slots | 1. Support of multi-TRP PUCCH repetition scheme 3 (sub-slot based repetition) 2. Supported PUCCH formats for this scheme | FG 23-3-2 |  |  |  | Per FS |  |  |  | Note1: For more than 2 repetitions, sequential beam mapping is supported as part of this FG. Note2: The details of this FG should be decided after FG(s) related to sTRP sub-slot based PUCCH repetition in Rel-17 IIoT AI. Component 3: A bit map of size 5 for PUCCH format 0-4. At least one value of the bitmap should be set to 1. | Optional |
| 23. NR_FeMIMO | 23-3-2b | Two TPC fields in DL DCI | Support of second TPC field for per TRP closed-loop power control for PUCCH with DCI formats 1_1 / 1_2 | FG 8-9 FG 23-3-2 |  |  |  | Per Band |  |  |  |  | Optional |
| 23. NR_FeMIMO | 23-3-2c | Cyclic beam mapping | Support of cyclic mapping when the number of repetitions is larger than 2 | FG 23-3-2 |  |  |  | Per Band |  |  |  |  | Optional |
| 23. NR_FeMIMO | 23-3-2d | Updating two Spatial relation or two sets of power control parameters for PUCCH group | Support of updating two Spatial Relation Info’s / two sets of power control parameters for a group of PUCCH resources in a CC by MAC-CE | FG 16-1b-3 FG 23-3-2 |  |  |  | Per Band |  |  |  |  | Optional |

- Inter-cell mTRP

The following aspects need to be considered when designing UE capability for Inter-cell mTRP

- Aligned and unaligned SSBs need to have separate capabilities due to different UE complexity

  - The max number of additional RRC-configured PCIs should be separately indicated for each of the two scenarios above

Proposal 4-1: Adopt the following for Rel-17 inter-cell mTRP UE features.

| 23. NR_FeMIMO | 23-4 | IntCell-mTRP | Support of additional RRC-configured PCI(s) that a TCI state/QCL information is associated with | FG 16-2a |  |  |  | Per FS |  |  |  | Note1: Only 1 additional PCI can be associated with the active TCI States, Note2: One PCI associated with one or more of activated TCI states for PDSCH/PDCCH is associated with one CORESETPoolIndex , another PCI associated with one or more of activated TCI states for PDSCH/PDCCH is associated with another CORESETPoolIndex | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-4b | Aligned SSBs | 1. Support the case where SSBs of the serving PCI and SSBs of additional RRC-configured PCI(s) have the same time domain position and periodicity 2. The maximum number of additional RRC-configured PCI(s) | FG 23-4 |  |  |  | Per Band |  |  |  | Component 2: {1 … TBD} | Optional |
| 23. NR_FeMIMO | 23-4c | Unaligned SSBs | 1. Support the case where SSBs of the serving PCI and SSBs of additional RRC-configured PCI(s) do not have the same time domain position and periodicity 2. The maximum number of additional RRC-configured PCI(s) | FG 23-4 |  |  |  | Per Band |  |  |  | Component 2: {1 … TBD} | Optional |

- mTRP BM

Proposal 5-1: As in FG 23-5-1, the support of enhanced group-based beam report can be indicated by the maximum number of supported beam groups.

Proposal 5-2: As in FG 23-5-1a, UE supporting FG 23-5-1 can indicate whether supporting the 2 reported beams in a report from different cells

- This requires UE to support inter-cell mTRP in FG 23-4.

| 23. NR_FeMIMO | 23-5-1 | Enhanced group-based beam report | 1. Maximum number of supported beam groups |  |  |  |  | Per band |  |  |  |  | Candidate value set: {0,1,2,3,4} |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-5-1a | Support 2 reported beams per group from different cells | 1. Support of 2 reported beams in a reported beam group from different cells | FG 23-4 |  |  |  | Per band |  |  |  |  | Optional |

Proposal 5-3: As listed in FG 23-5-2, the support of mTRP BFR can be indicated by the maximum number of supported BFD-RS resources per set.

| 23. NR_FeMIMO | 23-5-2 | Maximum number of BFD-RS resources per set for MTRP BFR | 1. Maximum number of supported BFD-RS resources per set |  |  |  |  | Per band |  |  |  |  | Candidate value set: {0,1,2} |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- mTRP HST-SFN

In [1], four FGs (FG 23-6-1 to FG 23-6-4) have been listed for HST-SFN enhancement. Regarding FG 23-6-1 of the support SFN scheme A (scheme 1), we believe it can be split into two sub features. The main features should describe UE capability for supporting SFN scheme 1 for both PDDCH and PDSCH or only PDSCH. This is based on the RAN1 agreement of the supported trannissmision schemes between PDCCH and PDSCH where either same SFN mode is used for both PDCCH and PDSCH in addition to single TRP PDCCH and SFN scheme 1 PDSCH.

Agreement Support the following combination of the transmission schemes - Single-TRP PDCCH + Rel-17 Scheme 1 PDSCH - Single-TRP PDCCH + Rel-17 TRP-based pre-compensation PDSCH - FFS: Other combinations of the transmission scheme Note: The PDSCH corresponds to the PDSCH scheduled by DCI formats 1_1 and 1_2. Agreement - UE does not expect to be configured different SFN schemes (scheme 1 or TRP pre-compensation) for both PDCCH and PDSCH.

Proposal 6-1: Split FG 23-6-1 into main FG for the support of scheme A for PDDCH/PDSCH and FG 23-6-1A for the support of dynamic switching between single TRP PDSCH and SFN PDSCH scheme A.

Proposal 6-2: FG 23-6-1 should have two componeonets:

- Support of Scheme A for both PDSCH and PDCCH

- Support of Scheme A for PDSCH only, where PDCCH is single TRP.

A similar discussion is applicable to SFN scheme B, where based on RAN1 agreements SFN scheme B is applicable on to HST deployment. FG 23-6-2 can be split into main FG for the support of SFN scheme B for both PDCCH and PDSCH or PDSCH only where PDCCH is single TRP.

Proposal 6-3: Split FG 23-6-2 into main FG for the support of scheme B for PDDCH/PDSCH and FG 23-6-2A for the support of dynamic switching between single TRP PDSCH and SFN PDSCH scheme B.

Proposal 6-4: FG 23-6-2 should have two componeonets:

- Support of Scheme B for both PDSCH and PDCCH

- Support of Scheme B for PDSCH only, where PDCCH is single TRP.

Regarding FG 23-6-3, it should be prerequist on the support of SFN PDCCH scheme A or scheme B and should be a separate UE feature than Rel-16 FG 16-1b-1.

Proposal 6-5: The support of SFN PDDCH (FG 23-6-1 or FG 23-6-1) should be prerequist to support FG 23-6-3 of simmultenous activation of two TCI states for PDCCH across multiple CCs

Regarding FG 23-6-4, it could be split into multiple FGs, the main one on the default beam for PDSCH, add FG FG 23-6-4A for UE capality of two QCL TypeD properories for multiple overlapping CORESETs when a CORESET is activated with two TCI as per RAN1-agreement below. Also, add component 23-6-4b for default beam for AP-CSIRS and compoenent 23-6-4c for default spatial relation and pathloss RS.

Agreement When a CORESET is activated with two TCI states which overlaps with another CORESET, support extension of Rel-15 prioritization rule for PDCCH monitoring of PDCCH candidates in overlapping monitoring occasions with different QCL-TypeD - FFS: Prioritization rule considers CORESETs indicated with 1 and/or 2 TCI states - Supports identifying two QCL-TypeD properties for multiple overlapping CORESETs - UE capability is introduced - FFS other details - FFS: Strive to have same / similar solution as discussed under AI 8.1.2.1

Proposal 6-6: Split FG 23-6-4 into main FG for the support of default beam for SFN PDSCH, and

- FG 23-6-4A: Two QCL TypeD properties for overlapping CORESETs

- FG 23-6-4B: Default beam for AP-CSI in SFN mode

- FG 23-6-4C: Default spatial relation and pathloss RS

Regarding componenet 2 of supporting PDSCH reception when tci filed not present, we believe that this component should be either removed as it has been agreed only for DCI format 1_0 which already doesn’t have tci-field or explcilty clarified in the description that DCI refers to DCI format 1_0.

Proposal 6-7:  Remove compoenent 2 of FG 23-6-4 or clarify the component description that DCI refer for DCI format 1_0.

Proposal 6-8: Adopt the following for Rel-17 HST-SFN UE features.

| 23. NR_FeMIMO | 23-6-1 | SFN scheme A (scheme 1) | 1. Support of scheme A for both PDCCH and PDSCH. 2. Support of scheme A for PDSCH only, where PDCCH is single TRP. |  |  |  |  | Per band |  |  |  | For each component: {Support, Not support} | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-6-1A | SFNschem A PDSCH dynamic switching | Support of dynamic switching between single-TRP and SFN PDSCH scheme A for DCI formats 1_1, 1_2 | FG 23-6-1 |  |  |  | Per band |  |  |  | {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-6-2 | SFN scheme B (TRP based pre-compensation) | 1. Support of scheme B for both PDCCH and PDSCH 2. Support of scheme B for PDSCH only, where PDCCH is single TRP. |  |  |  |  | Per band |  |  |  | For each component: {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-6-2A | SFNschem B PDSCH dynamic switching | Support of dynamic switching between single-TRP and SFN PDSCH scheme B for DCI formats 1_1, 1_2 | FG 23-6-2 |  |  |  | Per band |  |  |  | {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-6-3 | Simultaneous activation of two TCI states for PDCCH across multiple CCs (HST) | Support of simultaneous activation of two TCI states in all BWPs across set of configured component carriers | FG 23-6-1 (component 1 PDCCH), or FG 23-6-2 (component 1 PDCCH) |  |  |  | Per UE |  |  |  | {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-6-4 | Default beam setup for HST | 1. Support of PDSCH reception using default beam for Rel-17 enhanced SFN scheme when PDSCH is scheduled with offset less than threshold 2. Support PDSCH reception using default beam for Rel-17 enhanced SFN scheme when TCI field is not present in DCI for DCI format 1_0. |  |  |  |  | Per band |  | FR2 only |  | For each component {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-6-4a | Two QCL TypeD properties for SFN PDCCH | Supports identifying two QCL-TypeD properties for multiple overlapping CORESETs when a CORESET is activated with two TCI | FG 23-6-1 (PDCCH), or FG 23-6-2 (PDCCH) |  |  |  | Per Band |  | FR2 only |  | {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-6-4b | Default QCL assumption for A-CSI-RS in SFN mode | Support aperiodic CSI-RS reception using default beam for Rel-17 enhanced SFN scheme when scheduling offset is less than threshold | FG 23-6-1 (PDCCH), or FG 23-6-2 (PDCCH) |  |  |  | Per Band |  | FR2 only |  | {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-6-4c | Default spatial relation and pathloss RS | 1. Support of single-TRP PUCCH transmission using default beam and PL-RS when enhanced SFN PDCCH transmission scheme is configured 2. Support of single-TRP PUSCH transmission using default beam and PL-RS when enhanced SFN PDCCH transmission scheme is configured 3. Support of single-TRP SRS resource transmission using default beam and PL-RS when enhanced SFN PDCCH transmission scheme is configured | FG 23-6-1 (PDCCH), or FG 23-6-2 (PDCCH) |  |  |  | Per Band |  | FR2 only |  | For each component: {Support, Not support} | Optional |

- SRS enhancement

In [1], egiht FGs (FG 23-8-1 to FG 23-8-8) have been listed for SRS enhancement. In general, we are fine with the listed UE features. On FG 23-8-4, we think that two notes should be added to clarity the supported number of sets for periodic and semi-persisent SRS sets when the UE doesn’t support this feature. Regarding FG 23-8-6, the description filed should clarity that the feature is supported when SRS frequency hopping is enabled. Also, FG 2-52 (basic SRS) should be listed as prerequist for the FG 23-8-1 and FG 23-8-2.

Proposal 7-1: For UE features on R17 SRS enhancements, suggest the following edits

- Add two Notes for clarifying the number of sets for periodic and semi-persistent SRS when the UE doesn’t support FG 23-8-4

- Clarify FG 23-8-6 with regards to SRS frequency hopping configuration.

- Add FG 2-52 (basic SRS) as prerequisite for FG 23-8-1 and FG 23-8-2

- Add FG 2-55 (SRS Tx switch) as prerequisite for FG 23-8-3 and further discuss extension of Rel-16 of combo SRS Tx-switch.

- Add FG 2-53 (SRS resources) as prerequisite for FG 23-8-4.

- Add FG10-11 (SRS transmission at any symbol) as prerequisite for FG 23-8-5.

| 23. NR_FeMIMO | 23-8-1 | SRS triggering offset enhancement | Support of determining aperiodic SRS location based on available slot for DCI format 1_1, 1_2, 0_1 and 0_2 | FG 2-52 |  |  |  | Per FS |  |  |  | {Support, Not support} | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-8-2 | Triggering SRS only in DCI 0_1/0_2 | Support of triggering A-SRS in DCI 0_1/0_2 without data and without CSI | FG 2-52 |  |  |  | Per FS |  |  |  | {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-8-3 | SRS Antenna switching for >4Rx | Support of SRS antenna switching xTyR with y>4 | FG 2-55 |  |  |  | Per BC |  |  |  | {1T6R, 1T8R, 2T6R, 2T8R, 4T6R, 4T8R} | Optional |
| 23. NR_FeMIMO | 23-8-4 | Maximum 2 SP SRS sets for antenna switching | Support of maximum 2 SP SRS resource sets for antenna switching | FG 2-53 |  |  |  | Per FS |  |  |  | {Support, Not support} Note1: For xTyR where y>4, if UE does NOT support this feature, support maximum one SRS resource set for semi-persistent SRS. Note2: For xTyR where y<=4, follow Rel-15 on the number of resource sets for periodic and semi-persistent SRS. | Optional |
| 23. NR_FeMIMO | 23-8-5 | Increased repetition for SRS | Support of increased repetition patterns (8, 10, 12, 14 symbols) for SRS resource | FG 10-11 |  |  |  | Per FS |  |  |  | {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-8-6 | Partial frequency sounding of SRS | Support of partial frequency sounding for peridocs and semi-persisent SRS, only when freq. hopping enabled. |  |  |  |  | Per Band |  |  |  | {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-8-7 | Start RB location hopping for partial frequency SRS | Support of start RB location hopping in partial frequency sounding with frequency hopping | FG 23-8-6 |  |  |  | Per Band |  |  |  | {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-8-8 | Comb-8 SRS | Support of comb-8 for MIMO SRS |  |  |  |  | Per FS |  |  |  | {Support, Not support} | Optional |

- mTRP CSI

The following aspects need to be considered when designing UE capability for mTRP CSI:

- The basic FG need to only include the minimum that is needed for NCJT CSI.

  - The FG needs to be consistent with the Rel-15 and Rel-16 CSI capability framework.

  - Due to complexity for UE to calculate two PMIs jointly for NCJT, a list of vectors for NCJT capabilities and sTRP capabilities are needed.

- For NCJT CSI reporting, UE should be able to indicate whether option 1, option 2, or both are supported.

  - For Option 1, UE should be able to indicate which values of X (X=0,1,2) are supported.

  - Default capability for Option 1 is X=0.

Proposal 5-1: Adopt the following for Rel-17 inter-cell mTRP UE features.

| 23. NR_FeMIMO | 23-7-1 | CSI Enhancement for Multi-TRP | 1. Support of two CMR groups in a CSI-RS resource set for Multi-TRP CSI 2. Support of one pair of CMRs from the two CMR groups (Nmax=1) 3. Supported option(s) for CSI reporting 4. A list of vectors {max # of ports per resource for NCJT, max # of total NCJT hypotheses, max # of total ports of CMRs for NCJT, max # of ports per resource for sTRP, max # of resources for sTRP, max # of total ports for sTRP} 5. Supported codebook modes 6. A list of (Y1,Y2): UE can process Y1 NCJT CSI report(s) and Y2 sTRP CSI report(s) simultaneously in a CC 7. A list of (X1,X2): UE can process X1 NCJT CSI report(s) and X2 sTRP CSI report(s) simultaneously across all CCs |  |  |  |  | Per Band Per Band Combination |  |  |  | Note: This feature is applicable to ‘typeI-SinglePanel’ codebook Component 3: {Option 1, Option 2, both} If UE supports ‘Option 1’ or ‘both’, X=0 CSIs associated with single-TRP measurement hypotheses is assumed Compenent 4: The list can have maximum of 16 vectors - Max # of Tx ports in one resource for NCJT: {2, 4, 8, 12, 16, 24, 32} - Max # of total NCJT hypotheses: {1 to 32} - Max # total ports of CMRs for NCJT: {4 to 256} - Max # of Tx ports in one resource for sTRP: {4,8,12,16,24,32} - Max # resources for sTRP: {1 to 64} - Max # total ports for sTRP: {4 to 256} Note: Component 4 is also reported per band combination (in addition to per band). Compenent 5:{ mode 1, both mode 1 and mode 2} Compoentn 6: The list can have maximum of 16 pairs. - Y1: {1 to 4} - Y2: {1 to 6} Compoentn 7: The list can have maximum of 16 pairs. - X1: {2 to 16} - X2: {1 to 28} Note: When no NCJT CSI is configured/active, sTRP CSI capabilities are based on Rel-15/16 capabilities. | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-7-2 | Supported value of X for Option 1 | Supported values of X (reported CSIs associated with single-TRP measurement hypotheses) for Option 1 | FG 23-7-1 |  |  |  | Per Band |  |  |  | Note: UE supporting this feature indicates ‘Option 1’ or ‘both’ in component 4 of FG 23-7-1. Candidate values: {1, 2, 1and2} | Optional |
| 23. NR_FeMIMO | 23-7-3 | More than two resources in a resource set for Multi-TRP CSI | Supported maximum # of CSI-RS resources in a resource set for Multi-TRP CSI (value Ks,max ) | FG 23-7-1 |  |  |  | Per Band |  |  |  | Candidate values: {3…8} | Optional |
| 23. NR_FeMIMO | 23-7-4 | Support two pairs of CMRs | Support of two pairs of CMRs from the two CMR groups (Nmax=2) | FG 23-7-1 |  |  |  | Per Band |  |  |  |  | Optional |
| 23. NR_FeMIMO | 23-7-5 | CMR sharing | Support a CMR from a pair of CMRs configured for a Multi-TRP CSI measurement hypothesis to be used for Single-TRP measurement hypotheses | FG 23-7-1 |  |  |  | Per Band |  |  |  |  | Optional |

- FDD CSI

For Rel-17 port-selection Type II codebook, it is preferred to reuse Rel-16 UE capability framework/mechanism as much as possible. For codebook concurrency, since Type I codebook is typically used for basic periodic CSI reporting, it is reasonable to introduce codebook currency of Type I and Rel-17 port-selection Type II codebook. Hence, we propose

Proposal 9: For Rel-17 port-selection Type II codebook, following aspects need to be considered

- Supporting Rel-17 port-selection Type II codebook with reported CSI-RS capability in terms of {max # ports per resource, max # resource, max # total ports}

  - Per band capability and per band-combination capability

- Supporting M > 1 FD bases with additional UE capability signaling (previous agreement)

  - Report {support, not support}

  - Support per band capability

- Supporting rank 3 and rank 4 as additional UE capability signaling (previous agreement)

  - Report {support, not support}

  - Support per band capability

- Supporting R > 1 with additional UE capability signaling (previous agreement, but whether having R > 1 is TBD)

  - Supporting R > 1 with reported CSI-RS capailbity in terms of {max # ports per resource, max # resource, max # total ports}

  - Per band capability and per band combination capability

- Introduce codebook combination {Type I, Rel-17 port-selection Type II} and report CSI-RS capability in terms of {max # ports per resource, max # resource, max # total ports}

Example UE feature list is provided below.

| 23. NR_FeMIMO | 23-9-1 | Port-selection FeType II | 1. {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support port selection eType-II for R=1 2. Parameter combinations related to M = 1 3. Support of rank 1,2 | FG 2-35 |  |  |  | Per band Per BC |  |  |  | Candidate values for component 1: - Maximum 16 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256} | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-9-2 | More than 1 FD basis | Support parameter combinations related to M > 1 | FG 23-9-1 |  |  |  | Per band |  |  |  | {support, not support} | Optional |
| 23. NR_FeMIMO | 23-9-3 | Rank 3 and 4 | Support rank 3 and 4 | FG 23-9-1 |  |  |  | Per band |  |  |  | {support, not support} | Optional |
| 23. NR_FeMIMO | 23-9-4 | Support PMIs in one subband with R > 1 (if agreed in MIMO session) | 1. {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support port selection eType-II for R > 1 | FG 23-9-1 |  |  |  |  |  |  |  | Candidate values for component 1: - Maximum 16 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256} |  |
| 23. NR_FeMIMO | 23-9-5 | Active CSI-RS resources and ports for mixed codebook types in any slot for R17 | 1. Report a list of codebook combinations as {codebook 1, codebook 2} 2. For each codebook combination, report a list of {max number of ports per resource, max number of resources, max number of total ports} | 2-36/40 in Rel-15, and 23-9-1/4 in Rel-17 |  |  |  | Per band Per BC |  |  |  | Component-1 candidate values: Codebook 1 = {Type I SP, Type I MP} Codebook 2 = R17 port-slection FeType II Note 3：if a UE reports one or more codebook combinations in 23-9-5, then usage of active CSI-RS resources and ports for multiple codebooks in any slot is allowed only within those combinations Note 4: For coexisting of mixed codebooks in any slot, gNB need to honor 23-9-5 and per-codebook capability 2-36/40 and 23-9-1/23-9-4 Component-2 candidate values: - Maximum 16 triplets for each codebook combination - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256} | Optional |

- References

- R1-2108679, Preliminary RAN1 UE features list for Rel-17 NR, Moderators (AT&T, NTT DOCOMO, INC.).