3GPP TSG RAN WG1 #106bis-e			R1-2110227

e-Meeting, October 11th – 19th, 2021

Agenda item:	8.17.9

Source: 	Qualcomm incorporated

Title: 	UE features for NB-IoT and LTE-MTC enhancements

Document for:	Discussion / Decision

# Introduction

In this contribution, we present our views on the UE features for NB_IOTenh4_LTE_eMTC6. Track changes are used with respect to the version in R1-2108678.

- NB_IOTenh4_LTE_eMTC6

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | [Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs)] | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Capability interpretation for mixture of FDD/TDD | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. NB_IOTenh4_LTE_eMTC6 | 1-1 | 16-QAM for unicast NPDSCH | 1. Reception of unicast NPDSCH modulated with 16-QAM 2. Downlink power allocation for 16-QAM | Category NB-2 | Yes | N/A | The network cannot schedule a unicast NPDSCH modulated with 16-QAM for the UE | Per Band | N/A | N/A | FFS whether separate capabilities are indicated for different deployment modes. | Optional with capability signaling |
| 1. NB_IOTenh4_LTE_eMTC6 | 1-1a | CQI report to support 16-QAM modulation | 1. CQI report to support 16-QAM modulation | 1-1 | Yes | N/A | The network cannot rely on CQI report for scheduling 16-QAM | Per Band | N/A | N/A |  | Optional with capability sigaling |
| 1. NB_IOTenh4_LTE_eMTC6 | 1-2 | 16-QAM for unicast NPUSCH | 1. Transmission of unicast NPUSCH modulated with 16-QAM 2. New term in the UE’s transmit power control equation. | Category NB-2 | Yes | N/A | The network cannot schedule a unicast NPUSCH modulated with 16-QAM for the UE | Per Band | N/A | N/A |  | Optional with capability signaling |
| 1. NB_IOTenh4_LTE_eMTC6 | 1-3 | 14 HARQ processes for PDSCH for HD-FDD Cat. M1 UEs | 1. Support of 14 DL HARQ processes for unicast in HD-FDD in CE mode A in RRC_CONNECTED 2. PDSCH scheduling delay 3. HARQ-ACK delay solution with Alt-1 and/or Alt-2e | 1. Category M1 2. HD-FDD 3. CE Mode A [4. Support of 10 DL HARQ processes in HD-FDD in CE mode A] [5. HARQ-ACK bundling support in HD-FDD in CE mode A] | Yes | N/A | The network cannot enable 14 HARQ processes for the UE | Per Band | FDD only | N/A | - PDSCH scheduling delay: - 2 BL/CE DL subframes. - 1 BL/CE DL subframe + 1 subframe + 3 BL/CE UL subframes + 1 subframe + 1 BL/CE DL subframe. - 1 subframe + 3 BL/CE UL subframes + 1 subframe + 2 BL/CE DL subframes. - HARQ-ACK delay: - Alt-1: The HARQ-ACK delay is determined through an expression consisting of different subframe types (Using a similar principle as the PDSCH scheduling delay). - Alt-2e: The HARQ-ACK delay is determined following the legacy approach. That is, the “HARQ-ACK delay” is kept expressed in terms of “absolute subframes”. | Optional with capability signaling Values (which apply to support of component 5): {alt1, alt2-e, both} |
| 1. NB_IOTenh4_LTE_eMTC6 | 1-4 | A maximum DL TBS of 1736 bits for HD-FDD Cat. M1 UEs in CE mode A only | 1. Support of 1736 bits max DL TBS for unicast in HD-FDD in CE mode A in RRC_CONNECTED | 1. Category M1 2. HD-FDD 3. CE Mode A | Yes | N/A | The network cannot schedule a PDSCH with TBS larger than 1000 bits for Cat. M1 UEs | Per Band | FDD only | N/A |  | Optional with capability signaling |