3GPP TSG RAN WG1 #106bis-e			R1-2110229

October 11th – 19th, 2021

Agenda item:	8.17.11

Source: 	Qualcomm

Title: 	UE Features for Sidelink Enhancements

Document for:	Discussion/Decision

- Introduction

In this contribution, we propose a set of UE features for NR Sidelink Enhancements based on the preliminary feature list provided in [1].

In general, many of the new features are impacted by the currently supported featureset, e.g. the ability to process the additional information required for inter-UE coordination could be impacted by support of operations in other bands that would increase the baseband processing effort. Similarly transmitting or receiving such information would be impacted by other reception and transmission. Therefore, we propose that the feature be defined per featureset.

Proposal 1: UE features for sidelink enhancements are defined per featureset.

A second general note is that the new features are enhancements that could be implemented separately to a large degree. The UE ability to utilize inter-UE coordination information is not related to its ability to perform partial sensing for example. Therefore, we propose to have all the features as optional without being required for a UE that supports NR sidelink. Furthermore, a Release-16 UE that supports NR sidelink but not any of the Release-17 features is still capable of successfully performing sidelink communications and should not be impacted by the introduction of those new features.

Proposal 2: UE features for sidelink enhancements are optional and not required of a UE that supports NR sidelink.

Release-16 required any sidelink UE capable of transmission in Mode 2 to also be able to perform random resource selection for transmission in the exceptional resource pool. In Release-17, random selection is introduced as a resource selection mode for regular resource pools as well. Any Release-17 UE that can transmit sidelink using Mode 2 should be able to use random selection since it is the simplest form of resource selection. This could be captured directly in specifications without UE capability signaling. Then, there is no need to mention random resource selection as a consequence of not supporting full sensing or partial sensing.

Proposal 3: Any Release-17 UE that is capable of Mode 2 sidelink transmission supports random resource selection.

Some sidelink-related FGs were included for LTE in [2]. In our view, the LTE sidelink feature list can be discussed after the NR FGs are finalized.

Proposal 4: Release-17 sidelink related FGs in the LTE UE feature list can be discussed after the NR FGs are finalized.

The rest of this section provides discussion for some of the changes in the feature list included in the next section.

In [1], FG 32-2 combined support for receiving PSFCH and S-SSB. However, the two serve different purposes and have different implementation requirements. Therefore, there is no need to group them. We propose to separate them into two FGs. While the gNB would benefit from knowing whether a UE can receive PSFCH or not, the same does not hold for S-SSB reception.

FG 32-3 in [1] already exists as FG 15-3 in the UE feature list. There is no need to duplicate the feature.

It was already agreed to support reevaluation and pre-emption checking. Two new FGs are introduced for these features.

A single FG is used for all inter-UE coordination features in [1]. However, the ability to utilize preferred resource information is unrelated to the ability to utilize non-preferred resources. Therefore, the FGs for Scheme 1 with preferred resource indication, for Scheme 1 with non-preferred resource indication, and Scheme 2 are separated. Moreover, a UE that utilizes coordination information does not necessarily need to be able to generate coordination information. Hence, those two aspects are listed in separate FGs for each variant.

- NR_SL_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_SL_enh | 32-1 | [Receiving NR sidelink of PSCCH/PSSCHPSFCH/S-SSB] | 1) UE can receive NR PSCCH/PSSCH/PSFCH/S-SSB. | None | [Yes] | [No] |  | [Per FS] | N.A. | N.A. | N.A. |  | Optional with capability signalling. |
| 32. NR_SL_enh | 32-2a | Receiving NR sidelink of PSFCH | 1) UE can receive NR PSFCH. | None | [Yes] | [Yes] |  | [Per FS] | N.A. | N.A. | N.A. |  | Optional with capability signalling. |
| 32. NR_SL_enh | 32-2b | Receiving NR sidelink of S-SSB | 1) UE can receive NR S-SSB. | None | [No] | [No] |  | [Per FS] | N.A. | N.A. | N.A. |  | Optional with capability signalling. |
| 32. NR_SL_enh | 32-4 | Transmitting NR sidelink mode 2 with partial sensing | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 with partial sensing configured by NR Uu or preconfiguration. 2) UE can perform periodic-based partial sensing and resource allocation operation. 3) UE can perform contiguous partial sensing and resource allocation operation. | None | [No] | [Yes] | UE does not support trasmissoin according to the partial sensing and resource allocation | [Per FS] | N.A. | N.A. | N.A. |  | Optional with capability signalling. |
| 32. NR_SL_enh | 32-6 | Reevaluation of selected resources | - UE can perform reevaluation checking for selected but not reserved resources. | None | No | No |  | Per FS | N.A. | N.A. | N.A. |  | Optional with capability signalling |
| 32. NR_SL_enh | 32-7 | Preemption checking for reserved resources | - UE can perform preemption checking for selected resources. | None | No | Yes |  | Per FS | N.A. | N.A. | N.A. |  | Optional with capability signalling |
| 32. NR_SL_enh | 32-5a | Inter-UE coordination preferred resource transmission | 1) UE can generate and transmit inter-UE coordination information of preferred resource set in NR sidelink mode 2. 2) UE can receive an explicit request for inter-UE coordination information | None | Yes | Yes |  | Per FS | N.A. | N.A. | N.A. |  | Optional with capability signalling |
| 32. NR_SL_enh | 32-5b | Inter-UE coordination preferred resource reception | 1) UE can receive inter-UE coordination information of preferred resource set and use the received information in its own resource (re-)selection in NR sidelink mode 2. 2) UE can transmit an explicit request for inter-UE coordination information | None | Yes | Yes |  | Per FS | N.A. | N.A. | N.A. |  | Optional with capability signalling |
| 32. NR_SL_enh | 32-5c | Inter-UE coordination non-preferred resource transmission | 1) UE can generate and transmit inter-UE coordination information of non-preferred resource set in NR sidelink mode 2. | None | Yes | Yes |  | Per FS | N.A. | N.A. | N.A. |  | Optional with capability signalling |
| 32. NR_SL_enh | 32-5d | Inter-UE coordination preferred nonresource reception | 1) UE can receive inter-UE coordination information of non-preferred resource set and use the received information in its own resource (re-)selection in NR sidelink mode 2. | None | Yes | Yes |  | Per FS | N.A. | N.A. | N.A. |  | Optional with capability signalling |
| 32. NR_SL_enh | 32-5e | Inter-UE coordination expected conflict indication transmission | 1) UE can detect the presence of expected/potential resource conflict and transmit an indication for inter-UE coordination in NR sidelink mode 2. | None | Yes | Yes |  | Per FS | N.A. | N.A. | N.A. |  | Optional with capability signalling |
| 32. NR_SL_enh | 32-5f | Inter-UE coordination expected conflict indication reception | 1) UE can receive inter-UE coordination information indicating the presence of expected/potential resource conflict and use the received information in its own resource re-selection in NR sidelink mode 2. | None | Yes | Yes |  | Per FS | N.A. | N.A. | N.A. |  | Optional with capability signalling |

# Conclusion

In addition to the changes in Section 23, we proposed the following:

Proposal 1: UE features for sidelink enhancements are defined per featureset.

Proposal 2: UE features for sidelink enhancements are optional and not required of a UE that supports NR sidelink.

Proposal 3: Any Release-17 UE that is capable of Mode 2 sidelink transmission supports random resource selection.

Proposal 4: Release-17 sidelink related FGs in the LTE UE feature list can be discussed after the NR FGs are finalized.

# References

[1]  R1-2108679, “Preliminary RAN1 UE features list for Rel-17 NR,” Moderators  (AT&T, NTT DOCOMO, INC.), RAN1 #106-e.

[2]  R1-2108678, “Preliminary RAN1 UE features list for Rel-17 LTE,” Moderators  (AT&T, NTT DOCOMO, INC.), RAN1 #106-e.