3GPP TSG RAN WG1 #106bis-e			R1-2110230

e-Meeting, October 11th – 19th, 2021

Agenda item:	8.17.12

Source: 	Qualcomm Incorporated

Title: 	UE features for MBS

Document for:	Discussion and Decision

- 1. Introduction

This contribution includes our views on the RAN1 UE features for Rel-17 NR MBS. The tracking changes are used with respect to the version in R1-2108679.

Given the potential UE testing differentiation among licensed, unlicensed, and NTN band, the type for a UE feature should be ‘per FSPC’ for Rel-17 NR MBS, or at least ‘per Band’.

Besides the track changes with respect to the version in R1-2108679, it needs to be noted that the total number of RNTIs (including MCCH-RNTI, G-RNTI(s) for broadcast, G-RNTI(s) for multicast, G-CS-RNTI for multicast SPS) per UE should be further discussed.

- NR_MBS

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-1 | Broadcast | - Support of group-common PDCCH/PDSCH with CRC scrambled by MCCH-RNTI. - Support of [Nbroadcast>=1 (FFS: value of Nbroadcast)] group-common PDCCH/PDSCH with CRC scrambled by G-RNTI(s). - Support of CFR configuration for broadcast. - Support of CORESET and common search space in a CFR for broadcast. - Support of DCI format 1_0 with CRC scrambled with G-RNTI/MCCH-RNTI for broadcast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH (for broadcast) in different slots. - Support of MCCH change notification indication via DCI (FFS: via a new RNTI or DCI bits). - FFS: Support of TRS for GC-PDCCH/PDSCH configured in a broadcast CFR |  | Yes |  |  | [Per FSPC] | N/A | N/A |  |  | Optional without capability signalling |
| 33. NR_MBS | 33-2 | Dynamic scheduling for multicast | - Support of [Nmulticast_dynamic >=1 (FFS: value of Nmulticast_dynamic)] group-common PDCCH/PDSCH with CRC scrambled by G-RNTI(s) for multicast. - Support of CFR configuration for multicast. - Support of CORESET and common search space configuration in a CFR for multicast. - Support of DCI format 1_0 with CRC scrambled with G-RNTI for multicast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH (for multicast) in different slots. |  | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-2a |  | - Support of DCI format 1_1 with CRC scrambled with G-RNTI for multicast | 33-2 | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-2b |  | - Support of enabling/disabling ACK/NACK-based HARQ-ACK feedback for dynamically scheduled multicast by RRC signaling - Support of PTM retransmission for dynamically scheduled multicast based on G-RNTI | 33-2 | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-2c |  | - FFS: Support enabling/disabling ACK/NACK based HARQ-ACK feedback configured per G-RNTI by RRC signaling | 33-2b | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-2d |  | - Support of PTP retransmission for dynamically scheduled multicast based on C-RNTI | 33-2b | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-3-1 | Semi-static slot-level repetition for dynamically scheduled group-common PDSCH | - Support of semi-static slot-level repetition for dynamically scheduled group-common PDSCH for multicast. | 33-2 | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-3-1a | Dynamic slot-level repetition for dynamically scheduled group-common PDSCH | 1. Support of dynamic slot-level repetition for dynamically scheduled group-common PDSCH for multicast. | 33-2 | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-3-2 | FDM-ed unicast PDSCH and group-common PDSCH | - Support of FDM between one unicast PDSCH and one group-common PDSCH in a slot. | 33-1, 33-2 | Yes |  |  | Per FSPC | N/A | N/A |  | FFS: maximum data rate of (GC-)PDSCHs in a slot per CC. | Optional with capability signalling |
| 33. NR_MBS | 33-3-2a | Feedback multiplexing for FDM-ed unicast PDSCH and group-common PDSCH | 1. Support of FDM-ed ACK/NACK-based HARQ-ACK codebook for unicast and multicast in a slot. | 33-2b, 33-3-2 | Yes |  |  | Per FSPC | N/A | N/A |  | Values = {Type-1 HARQ-ACK codebook only, Type-2 HARQ-ACK codebook only, both} | Optional with capability signalling |
| 33. NR_MBS | 33-3-3 | Intra-slot TDM-ed unicast PDSCH and group-common PDSCH | - Support of TDM between one unicast PDSCH and one group-common PDSCH in a slot. | 33-1, 33-2 | Yes |  |  | Per FSPC | N/A | N/A |  | FFS: maximum data rate of (GC-)PDSCHs in a slot per CC. | Optional with capability signalling |
| 33. NR_MBS | 33-3-3a | Feedback multiplexing for intra-slot TDM-ed unicast PDSCH and group-common PDSCH | 1. Support of TDM-ed ACK/NACK-based HARQ-ACK codebook for unicast and multicast in a slot. | 33-2b, 33-3-3 | Yes |  |  | Per FSPC | N/A | N/A |  | Values = {Type-1 HARQ-ACK codebook only, Type-2 HARQ-ACK codebook only, both} | Optional with capability signalling |
| 33. NR_MBS | 33-3-4 | Intra-slot TDM-ed multiple unicast PDSCHs and/or multiple group-common PDSCHs | - Support TDM between M (M>1) TDMed unicast PDSCHs and one group-common PDSCH in a slot per CC - Support TDM among N (N>1) group-common PDSCHs in a slot per CC - Support TDM between K (K>1) TDMed unicast PDSCHs and L (L>1) TDMed group-common PDSCHs in a slot per CC - The UE maximum number of TDMed PDSCH receptions capability in a slot per CC is kept as for Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b. - Note: Group-common PDSCH(s) are counted as unicast PDSCH(s). | 33-1, 33-2 | Yes |  |  | Per FSPC | N/A | N/A |  | FFS: maximum data rate of (GC-)PDSCHs in a slot per CC. | Optional with capability signalling |
| 33. NR_MBS | 33-3-4a | Feedback multiplexing for Intra-slot TDM-ed multiple unicast PDSCHs and/or multiple group-common PDSCHs | 1. Support of TDM-ed ACK/NACK-based HARQ-ACK codebook for multiple unicasts and/or multiple multicasts in a slot. | 33-2b, 33-3-4 | Yes |  |  | Per FSPC | N/A | N/A |  | Values = {Type-1 HARQ-ACK codebook only, Type-2 HARQ-ACK codebook only, both} | Optional with capability signalling |
| 33. NR_MBS | 33-3-5 | Feedback multiplexing for inter-slot TDM-ed unicast PDSCH and group-common PDSCH | 1. Support of TDM-ed ACK/NACK-based HARQ-ACK codebook for unicast and one multicast in different slots | 33-2b | Yes |  |  | Per FSPC | N/A | N/A |  | Values = {Type-1 HARQ-ACK codebook only, Type-2 HARQ-ACK codebook only, both} | Optional with capability signalling |
| 33. NR_MBS | 33-4 | NACK-only based HARQ-ACK feedback for multicast | - Support of NACK-only based HARQ-ACK feedback for dynamically scheduled multicast. - Support PTM retransmission for dynamically scheduled multicast based on G-CS-RNTI | 33-2 | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-4a | More than one NACK-only based HARQ-ACK feedback for multicast in a PUCCH resource | 1. Support of more than one NACK-only based HARQ-ACK feedback for dynamically scheduled multicast in a PUCCH resource | 33-4 | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1 | SPS group-common PDSCH for multicast | - Support of one SPS group-common PDSCH configuration in a CFR for multicast - Support of [Nmulticast_SPS >=1 (FFS: value of Nmulticast_SPS)] group-common PDCCH/PDSCH with CRC scrambled by G-CS-RNTI(s) for multicast - Support of DCI format 1_0 with CRC scrambled with G-CS-RNTI for multicast. - FFS: ACK/NACK-based HARQ-ACK feedback for multicast SPS activation PDCCH and multicast SPS release PDCCH. | 33-2 | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1a |  | - Support of DCI format 1_1 with CRC scrambled with G-CS-RNTI for multicast SPS transmission. | 33-5-1 | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1b |  | - Support of enabling/disabling ACK/NACK-based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling. - Support of PTM retransmission for SPS group-common PDSCH based on G-CS-RNTI | 33-5-1 | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1c |  | - FFS: Support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signaling for SPS group-common PDSCH without PDCCH scheduling per G-CS-RNTI | 33-5-1b | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1d |  | - Support PTP retransmission for SPS group-common PDSCH based on CS-RNTI | 33-5-1b | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1e |  | - Support of NACK-only-based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling. - Support of PTM retransmission for SPS group-common PDSCH based on G-CS-RNTI | 33-5-1 | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1f | Semi-static slot-level repetition of SPS group-common PDSCH for multicast | - Support of semi-static slot-level repetition for SPS group-common PDSCH | 33-5-1 | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1h | Dynamic slot-level repetition of SPS group-common PDSCH for multicast | - Support of dynamic slot-level repetition for SPS group-common PDSCH | 33-5-1 | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-2 | Multiple SPS group-common PDSCH configuration | - Support [N>1] SPS group-common PDSCH configuration in a CFR for multicast Note: The total number of SPS configurations supported by a UE currently defined for unicast is not increased due to additionally supporting MBS. | 33-2, 33-5-1 | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-2a | Multiple activated SPS group-common PDSCH configurations | 1. Support [M>1] activated SPS group-common PDSCH configurations among the N SPS group-common PDSCH configurations per CFR for multicast. | 33-5-2 | Yes |  |  | Per FSPC | N/A | N/A |  | M is no larger than N | Optional with capability signalling |
| 33. NR_MBS | 33-6-1a | DL priority of multicast ACK/NACK-based feedback for dynamically scheuled multicast | 1. Support of priority configured for multicast ACK/NACK-based HARQ-ACK feedback of dynamically scheuled multicast Notes: - Two priority indexes are introduced for multicast, with index 0 meaning low priority and index 1 meaning high priority. - The priority of multicast is the same as the priority of unicast for the same priority index of HARQ-ACK at least for ACK/NACK based feedback. | 33-2b | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-6-1b | DL priority of multicast NACK-only-based feedback for dynamically scheuled multicast | 1. Support of priority configured for multicast NACK-only-based HARQ-ACK feedback of dynamically scheuled multicast Notes: - Two priority indexes are introduced for multicast, with index 0 meaning low priority and index 1 meaning high priority. - The priority of multicast for NACK-only based feedback is the same as the priority of unicast for the same priority index of HARQ-ACK. | 33-4 | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-6-1c | DL priority indication for dynamically scheduled multicast in DCI | 1. Support of priority indicator field configured in DCI formats 1_1 with CRC scrambled with G-RNTI for dynamically schedule multicast. | 33-2a, 33-6-1a or 33-6-1b | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-6-1d | DL priority of multicast ACK/NACK-based feedback SPS multicast | 1. Support of priority configured for multicast ACK/NACK-based HARQ-ACK feedback of SPS multicast | 33-5-1b | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-6-1e | DL priority of multicast NACK-only-based feedback SPS multicast | 1. Support of priority configured for multicast NACK-only-based HARQ-ACK feedback of SPS multicast | 33-5-1e | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-6-1f | DL priority indication for SPS multicast in activation DCI | 1. Support of priority indicator field configured in DCI format 1_1 with CRC scrambled with G-CS-RNTI for SPS multicast. | 33-5-1a, 33-6-1d or 33-6-1e | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-6-2 | Two ACK/NACK-based HARQ-ACK codebooks simultaneously constructed for supporting HARQ-ACK codebooks with: different priorities for multicast or for unicast and multicast at a UE. | 1. Supports of two ACK/NACK-based HARQ-ACK codebooks with different priorities to be simultaneously constructed different priorities for multicast or for unicast and multicast at a UE. | 33-2b, 33-6-1c | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-6-3 | PUCCH resource configuration for ACK/NACK-based multicast feedback for dynamically scheduled multicast | 1. Support of a PUCCH-Config for multicast ACK/NACK-based HARQ-ACK feedback, separate from that of unicast configurations | 33-2b | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-6-3a | Up to 2 PUCCH resources configuration for ACK/NACK-based multicast feedback for dynamically scheduled multicast | 1. Support of a PUCCH-ConfigurationList for multicast ACK/NACK-based HARQ-ACK feedback, separate from that of unicast configurations | 33-2b, 33-6-1c | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-6-4 | PUCCH resource configuration for NACK-only-based multicast feedback for dynamically scheduled multicast | 1. Support of a PUCCH-Config for multicast NACK-only-based HARQ-ACK feedback, separate from that of multicast ACK/NACK-based configurations if configured | 33-4 | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-6-4a | Up to 2 PUCCH resources configuration for NACK-only-based multicast feedback for dynamically scheduled multicast | 1. Support of a seperate PUCCH-ConfigurationList for multicast NACK-only-based HARQ-ACK feedback, separate from that of multicast ACK/NACK-based configurations if configured | 33-4 | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-6-5 | PUCCH resource configuration for ACK/NACK-based multicast feedback for SPS GC-PDSCH | 1. Support of a SPS-PUCCH-AN-List for multicast ACK/NACK-based HARQ-ACK feedback of all SPS configuration(s), separate from that of SPS unicast configurations | 33-5-1b | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-6-6 | More than one PUCCH for ACK/NACK-based HARQ-ACK transmission for multicast or for unicast and multicast within a slot | 1. Support of two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast or for unicast and multicast with different priorities in a slot. | 33-6-2, 33-6-3a | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-7 | Supporting group-common DCI indicating the enabling/disabling ACK/NACK based HARQ-ACK feedback for dynamically scheduled group-common PDSCH | 1. Support of the function of group-common DCI indicating the enabling/disabling ACK/NACK based HARQ-ACK feedback for dynamically scheduled group-common PDSCH | 33-2b | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |