3GPP TSG RAN WG1 #106bis-e		R1-2110238

e-meeting, October 11th – 19th, 2021

Source:	Lenovo, Motorola Mobility

Title: 	Enhancements for TB processing over multi-slot PUSCH

Agenda Item:   8.8.1.2

Document for:	Discussion

# 1	Introduction

In RAN#91-e, revised work item for NR coverage enhancements in Rel-17 has been approved with following objectives for in RP-210855 [1]:

- Specification of PUSCH enhancements [RAN1, RAN4]

  - Specify the following mechanisms for enhancements on PUSCH repetition type A [RAN1]

    - Increasing the maximum number of repetitions up to a number to be determined during the course of the work.

    - The number of repetitions counted on the basis of available UL slots.

  - Specify mechanism(s) to support TB processing over multi-slot PUSCH [RAN1]

    - TBS determined based on multiple slots and transmitted over multiple slots.

  - Specify mechanism(s) to enable joint channel estimation [RAN1, RAN4]

    - Mechanism(s) to enable joint channel estimation over multiple PUSCH transmissions, based on the conditions to keep power consistency and phase continuity to be investigated and specified if necessary by RAN4 [RAN1, RAN4]

      - Potential optimization of DMRS location/granularity in time domain is not precluded

    - Inter-slot frequency hopping with inter-slot bundling to enable joint channel estimation [RAN1]

- Specification of PUCCH enhancements [RAN1, RAN4]

  - Specify signaling mechanism to support dynamic PUCCH repetition factor indication [RAN1]

  - Specify mechanism to support DMRS bundling across PUCCH repetitions [RAN1, RAN4]

    - When applicable, based on similar mechanism(s) for enabling joint channel estimation for PUSCH

- Specify mechanism(s) to support Type A PUSCH repetitions for Msg3 [RAN1]

In this contribution, we provide our views on following aspects for TB processing over multi-slot PUSCH:

- RV and rate-matching with TBoMS

- Retransmissions with TBoMS

- Repetitions with TBoMS

- Configuration/Indication of TBoMS

# 2	Enhancements for RV and rate-matching with TBoMS

In RAN1#106-e [2], following working assumption related to RV design and rate-matching with TBoMS have been made:

Working Assumption

Single TBoMS structure of Option 3 is selected

- Option 3: Multiple TOTs are determined for a TBoMS. The TB is transmitted on the multiple TOTs using a single RV.

  - FFS: how the single RV is rate matched across single or multiple TOTs, e.g., rate matched for each TOT, rate matched for all the TOTs, rate matched for each slot and so on.

With option 3, a single RV is applied to the entire duration of TBoMS regardless of whether all the slots are contiguous or if there are multiple disjointed set of contiguous slots. One of the main issues for option 3 is if there are potential collisions with latter part of the TBoMS, then it needs to be considered also to determine TBS, rate-matching for the earlier part of TBoMS as well. However, in our understanding, applying single RV doesn’t really imply such a restriction. One potential solution to deal with the issue is that the coded bits based on single RV can be split up into multiple segments, where each segment can have potentially separate rate-matching, if needed. For example, if RV0 is applied across entire TBoMS, then the first slot or TOT will start with the coded bits from the starting position of RV0 and end at bit, say b0, in the circular buffer without rate-matching. Then for next slot or TOT, the coded bits will be applied from the position subsequent to the end of the first slot or TOT (bit b0+1) and so on. Therefore, any issue or limitation with single RV over entire TBoMS can be avoided.

Proposal 1: For PUSCH coverage enhancements in NR Rel-17 with TBoMS, working assumption for supporting option 3 should be confirmed.

Proposal 2: For PUSCH coverage enhancements in NR Rel-17 with TBoMS, for option 3, different starting points (to apply coded bits) from a single RV should be considered for different slots or contiguous set of slots.

Furthermore, regarding rate-matching, three options are currently under consideration. If similar behaviour as in Rel-15/16 needs to be followed where UCI multiplexing to UL-SCH is done on slot basis, then the most straightforward way is to perform rate-matching per slot as described in option a in the agreement from RAN1#105-e.

Proposal 3: For PUSCH coverage enhancements in NR Rel-17 with TBoMS, option a should be adopted for rate-matching i.e., the rate-matching is performed per slot basis.

# 3	Retransmissions with TBoMS

Another important aspect related to TBoMS is retransmissions. It is expected that TBoMS can span quite a long duration over multiple contiguous and/or non-contiguous slots. However, it is possible that only a portion of a TBoMS transmission might need to be retransmitted. It is not optimal to transmit entire TB over multiple slots in this case. Even for smaller TB size, multiple slots could be scheduled for transmission and therefore, CBG based PUSCH retransmissions might not always work. Therefore, enhancements to handle PUSCH retransmissions when TBoMS is applied should be considered.

Proposal 4: For PUSCH coverage enhancements in NR Rel-17 with TBoMS, retransmission procedure and signaling should be enhanced to support retransmission of only partial slots from the TBoMS.

One potential method could be to support a time unit for a portion of the TBoMS which can be re-transmitted. Since configuration of the time unit could either be independently configured to the UE or could be possibly associated with other design factors such as TOT duration, RV, rate-matching, etc. In Figure 1 and Figure 2, two illustrations are shown for retransmissions of only portions of TBoMS for the two cases of single set of contiguous slots (one TOT) and multiple disjointed sets of contiguous slots (multiple TOTs). The main benefits of applying such methods is to allow retransmission of smaller portions instead of entire TBoMS and also with configuration/indication about portion size and/or number of portions, the retransmission overhead (indication bits) can be controlled to be not too large. The time unit of portions can be smaller or larger depending on the overall duration of TBoMS. For example, if the duration is not too long, then the portion can be equal to 1 slot and retransmission can be configured on slot basis. On the other hand, if the duration is too long, then the portion can be equal to some multiples of slots or even a single TOT and then retransmission can be configured accordingly.

Proposal 5: For PUSCH coverage enhancements in NR Rel-18 with TBoMS, if retransmission for duration shorter than the overall duration of TBoMS is supported, then implicit/explicit configuration of the portion (duration) should be supported with portion indication in the retransmission DCI. Exact duration of the portion can be as follows:

- Explicitly configured to the UE

- Implicitly determined by UE depending on the duration of TBoMS, number of TOTs, duration of TOTs

Figure 1: Example for retransmission of shorter portions when TBoMS is applied, and only single TOT exists

Figure 2: Example for retransmission of shorter portions when TBoMS is applied, and multiple TOTs exist

# 4	Repetitions with TBoMS

In RAN1#106-e [2], following agreement related to repetitions of a single TBoMs has been made:

Agreement

Repetitions of a single TBoMS are supported, where:

- The number of configured repetitions is denoted by M, i.e., the total number of allocated slots for TBoMS repetition is M*N.

  - Note: M*N is no more than the max number of repetitions agreed for repetition Type A enhancement in agenda 8.8.1.1

- Available slot determination is according to existing agreements.

- The number and location of allocated symbols within an allocated slot for TBoMS transmission are the same among all repeated single TBoMS.

- FFS other aspects of TBoMS repetitions, e.g.:

  - Details of time domain resource indication.

  - Supported values for the number of TBoMS repetitions.

  - How to indicate the number of TBoMS repetitions.

  - Interactions with frequency hopping and precoder cycling across the M groups of N allocated slots for each single TBoMS repetition.

  - Whether RV indices should be cycled across the M groups of N allocated slots for each single TBoMS repetition.

  - Details of TBoMS retransmissions.

  - Potential MAC layer impact, but should be decided by RAN2

Note: No additional dropping rule optimization will be introduced other than dropping rules for single TBoMS transmission.

Further details related to repetition of TBoMS include:

- whether to support PUSCH repetition type-A or type-B or both

- how to handle indication of repetition factor along with indication of number of slots for TBoMS

In our view, TBoMS spans multiple slots and latency is not expected to be a key factor, therefore we don’t see a need to consider PUSCH repetition type B for TBoMS repetition. The issue of indication of repetition factor, we discuss in section 5 along with configuration/indication of TBoMS.

Proposal 6: For PUSCH coverage enhancements in NR Rel-17 with TBoMS, if repetition of TBoMS is supported, then only PUSCH repetition type A should be considered.

# 5	Configuration/Indication of TBoMS

Although PUSCH transmission with TBoMS has design features derived from PUSCH repetitions, however, this should be considered as an independent and new feature for PUSCH. Therefore, separate configuration of TBoMS should be supported to enable/disable TBoMS and to distinguish between configuration/indication of PUSCH repetitions and PUSCH transmission with TBoMS.

Proposal 7: For PUSCH coverage enhancements in NR Rel-17 with TBoMS, semi-static and/or dynamic configuration of TBoMS feature for PUSCH should be supported, and independent from PUSCH repetition.

In RAN1#105-e [3], following agreement related to indication of number of slots for TBoMS has been made:

Agreement:

Number of slots allocated for TBoMS is determined by using a row index of a TDRA list, configured via RRC.

- FFS: details.

For repetitions of TBoMS, both the indication of the number of slots for TBoMS and repetition factor will need to be supported. In this case, repetition factor in the TDRA table can be utilized only for one purpose, but not both. For this scenario, two possibilities can be considered:

- Introduce indication for number of slots for TBoMS in addition to repetition factor via TDRA row index

- Only support dynamic indication for number of slots for TBoMS via TDRA, but the repetition factor is indicated only via RRC configuration (based on Rel-15 indication)

Proposal 8: For PUSCH coverage enhancements in NR Rel-17 with repetition of TBoMS, following two methods can be considered to indicate the number of slots for TBoMS and repetition factor for TBoMS repetition:

- Introduce indication for number of slots for TBoMS in addition to repetition factor via TDRA row index

- Only support dynamic indication for number of slots for TBoMS via TDRA, but the repetition factor for TBoMS repetition is indicated only via RRC configuration

# 6	Conclusion

In this section, we summarize the observation/proposals from above section:

Proposal 1: For PUSCH coverage enhancements in NR Rel-17 with TBoMS, working assumption for supporting option 3 should be confirmed.

Proposal 2: For PUSCH coverage enhancements in NR Rel-17 with TBoMS, for option 3, different starting points (to apply coded bits) from a single RV should be considered for different slots or contiguous set of slots.

Proposal 3: For PUSCH coverage enhancements in NR Rel-17 with TBoMS, option a should be adopted for rate-matching i.e., the rate-matching is performed per slot basis.

Proposal 4: For PUSCH coverage enhancements in NR Rel-17 with TBoMS, retransmission procedure and signaling should be enhanced to support retransmission of only partial slots from the TBoMS.

Proposal 5: For PUSCH coverage enhancements in NR Rel-18 with TBoMS, if retransmission for duration shorter than the overall duration of TBoMS is supported, then implicit/explicit configuration of the portion (duration) should be supported with portion indication in the retransmission DCI. Exact duration of the portion can be as follows:

- Explicitly configured to the UE

- Implicitly determined by UE depending on the duration of TBoMS, number of TOTs, duration of TOTs

Proposal 6: For PUSCH coverage enhancements in NR Rel-17 with TBoMS, if repetition of TBoMS is supported, then only PUSCH repetition type A should be considered.

Proposal 7: For PUSCH coverage enhancements in NR Rel-17 with TBoMS, semi-static and/or dynamic configuration of TBoMS feature for PUSCH should be supported, and independent from PUSCH repetition.

Proposal 8: For PUSCH coverage enhancements in NR Rel-17 with repetition of TBoMS, following two methods can be considered to indicate the number of slots for TBoMS and repetition factor for TBoMS repetition:

- Introduce indication for number of slots for TBoMS in addition to repetition factor via TDRA row index

- Only support dynamic indication for number of slots for TBoMS via TDRA, but the repetition factor for TBoMS repetition is indicated only via RRC configuration

# 7	References

[1]	3GPP RP-210855, “Revised WID on NR coverage enhancements”, China Telecom

[2] 	3GPP RAN1#106-e. “Chairman Notes”

[3] 	3GPP RAN1#105-e. “Chairman Notes”