3GPP TSG RAN WG1 #106bis	R1-2110265

e-Meeting, 11 – 19 October, 2021

Agenda item:		8.17.2

Source:	Nokia, Nokia Shanghai Bell

Title:	On UE features for supporting NR from 52.6 GHz to 71 GHz

Document for:		Discussion and Decision

# Introduction

Initial drafts on NR and LTE UE features have been made available already on RAN1 reflector [1, 2]. In this contribution we present our views on the UE features for supporting NR from 52.6 GHz to 71 GHz.

# Discussion

Detailed comments on UE features are listed below. For reference, the feature list is available in the Annex.

- In general, it seems that many capabilities are missing still, e.g. related to beam management and PUCCH formats.

- Regardless of the eventual FG type decision, it should be noted which FGs are exclusively applicable to FR2-2 bands.

- 24-1:

  - SSB support for non-initial access should be mentioned in this FG as well

- 24-2:

  - Component description should be 120kHz

  - FG name mentions stand-alone support, but the functionality is relevat to PSCell operation as well. It would be better to merge this FG with FG 24-1.

- 24-3:

  - Similar comment as above, this is relevant to PSCell as well, not only stand-alone. It would be better to merge this FG with FG 24-4.

- FG on Cat 3 or Cat 4 LBT support for FR2-2 unlicensed operation: Agree that more clarity is needed before defining the details of the FG.

Proposal: Consider the observations and modifications proposed above for the next version of the corresponding RAN1 UE features list.

# Conclusion

In this contribution we have presented our views on UE features for supporting NR from 52.6 GHz to 71 GHz, with the following proposal:

Proposal: Consider the observations and modifications proposed above for the next version of the corresponding RAN1 UE features list.

# References

- R1-2108679, ”Preliminary RAN1 UE features list for Rel-17 NR”, Moderators (AT&T, NTT DOCOMO, INC.), Aug. 2021

- R1-2108678, ”Preliminary RAN1 UE features list for Rel-17 LTE”, Moderators (AT&T, NTT DOCOMO, INC.), Aug. 2021

# Annex

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-1 | General FR2-2 support | 1. Support 120KHz SCS transmission and reception 2. Support multi-RB PUCCH format 0/1/4 3. PRACH with 120KHz SCS and length 139/571/1151 |  |  |  |  |  |  |  |  |  |  |
| 24. NR_ext_to_71GHz | 24-2 | 120KHz SSB based stand-alone support | 1. Support 480KHz SSB for initial access | 24-1 |  |  |  |  |  |  |  |  |  |
| 24. NR_ext_to_71GHz | 24-3 | 480KHz SSB based stand-alone support | 1. Support 480KHz SSB for initial access | 24-1, 24-2, 24-4 |  |  |  |  |  |  |  | From WID: - In addition to 120kHz, support 480 kHz SSB for initial access with support of CORESET#0/Type0-PDCCH configuration in the MIB with following constraints: - Note: 480 kHz is an optional SSB numerology for initial access for the UE. A UE supporting a band in 52.6-71 GHz must at least support 120 kHz SCS (for initial access and after initial access) |  |
| 24. NR_ext_to_71GHz | 24-4 | 480KHz SCS support | 1. 480KHz SCS for UL transmission 2. 480KH SCS for DL reception 3. 480KHz for SSB monitoring 4. Multiple-slot PDCCH monitoring for 480KHz with X=4 5. PRACH with 480KHz and length 139/[571] |  |  |  |  |  |  |  |  | From WID: In addition to 120kHz SCS, specify new SCS, 480kHz and 960kHz, and define maximum bandwidth(s), for operation in this frequency range for data and control channels and reference signals, only NCP supported. |  |
| 24. NR_ext_to_71GHz | 24-5 | 960KHz SCS support | 1. 960KHz SCS for UL transmission 2. 960KH SCS for DL reception 3. 960KHz for SSB monitoring 4. Multiple-slot PDCCH monitoring for 960KHz with X=8 5. PRACH with 960KHz and length 139 |  |  |  |  |  |  |  |  | From WID - In addition to 120kHz SCS, specify new SCS, 480kHz and 960kHz, and define maximum bandwidth(s), for operation in this frequency range for data and control channels and reference signals, only NCP supported. |  |
| 24. NR_ext_to_71GHz | 24-? | Cat 3 or Cat 4 LBT support for FR2-2 unlicensed operation | 1. Cat 3 or 4 LBT support (not agreed yet if CW is supported, so it can be either Cat 3 or Cat 4 LBT for now. Will update when we have agreement) | 24-1 |  |  |  |  |  |  |  |  |  |
| 24. NR_ext_to_71GHz | 24-? | Cat 2 LBT support for FR2-2 unlicensed operation | 1. Support Cat 2 LBT | 24-1 |  |  |  |  |  |  |  |  |  |