3GPP TSG RAN WG1 #106bis	R1- 2110273

e-Meeting, 11 – 19 October, 2021

Agenda item:		8.17.11

Source:	Nokia, Nokia Shanghai Bell

Title:	On UE features for NR sidelink enhancement

Document for:		Discussion and Decision

# Introduction

Initial drafts on NR and LTE UE features have been made available already on RAN1 reflector [1, 2]. In this contribution we present our views on the UE features for NR sidelink enhancement.

# Discussion

Detailed comments on UE features are listed below. For reference, the feature list is available in the Annex.

- In general it is unclear how the current Rel-16 FGs relate to the proposed Rel-17 FGs. For several Rel-16 FGs it is noted in 38.306 that ”Support of this feature is mandatory if UE supports NR sidelink”. In principle this applies to Rel-17 supporting SL as well, and hence this needs to be taken into account in the proposed FGs under this WID. At the same time, Rel-17 defines functionality that is simplified compared to Rel-16 V2X, and hence there may be a need for differentiation on UE FGs too.

- 32-1:

  - It is unclear what is the value of adding this particular FG on top of Rel-16 FGs, in particular FG15-1.

- 32-2:

  - This FG is currently defined in isolation, not being a pre-requisite to any other FG. It is unclear what functionality is enabled by such FG. Moreover, the considerations above on Rel-16 FGs needs to be considered here as well, as in the current form this FG is essentially an incapability in that context, as it defines what the UE is unable to do compared to a Rel-16 UE supporting SL.

- 32-3, 32-4:

  - At least FG15-3 should be added as pre-requisite.

Proposal: Consider the observations and modifications proposed above for the next version of the corresponding RAN1 UE features list.

# Conclusion

In this contribution we have presented our views on UE features for NR sidelink enhancement, with the following proposal:

Proposal: Consider the observations and modifications proposed above for the next version of the corresponding RAN1 UE features list.

# References

- R1-2108679, ”Preliminary RAN1 UE features list for Rel-17 NR”, Moderators (AT&T, NTT DOCOMO, INC.), Aug. 2021

- R1-2108678, ”Preliminary RAN1 UE features list for Rel-17 LTE”, Moderators (AT&T, NTT DOCOMO, INC.), Aug. 2021

# Annex

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_SL_enh | 32-1 | [Receiving NR sidelink of PSCCH/PSSCHPSFCH/S-SSB] | 1) UE can receive NR PSCCH/PSSCH/PSFCH/S-SSB. | None | [Yes] | [No] |  | [Per band] | N.A. | N.A. | N.A. |  | Optional with capability signalling. FFS: For UE supports NR sidelink, UE must indicate this FG is supported. |
| 32. NR_SL_enh | 32-2 | [Receiving NR sidelink of PSFCH/S-SSB only] | 1) UE can receive NR PSFCH/S-SSB only. | None | [Yes] | [No] |  | [Per band] | N.A. | N.A. | N.A. |  | Optional with capability signalling. FFS: For UE supports NR sidelink, UE must indicate this FG is supported. |
| 32. NR_SL_enh | 32-3 | Transmitting NR sidelink mode 2 with full sensing | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 with full sensing configured by NR Uu or preconfiguration. 2) UE supports the sensing and resource allocation operation as specified in Rel-16. | [32-1] | [Yes] | [No] | [UE can perfom random resource selection only] | [Per band] | N.A. | N.A. | N.A. |  | Optional with capability signalling. FFS: For UE supports NR sidelink, UE must indicate this FG is supported. |
| 32. NR_SL_enh | 32-4 | Transmitting NR sidelink mode 2 with partial sensing | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 with partial sensing configured by NR Uu or preconfiguration. 2) UE can perform periodic-based partial sensing and resource allocation operation. 3) UE can perform contiguous partial sensing and resource allocation operation. | [32-1], [32-3] | [Yes] | [No] | UE does not support trasmissoin according to the partial sensing and resource allocation | [Per band] | N.A. | N.A. | N.A. |  | Optional with capability signalling. FFS: For UE supports NR sidelink, UE must indicate this FG is supported. |
| 32. NR_SL_enh | 32-5 | Inter-UE coordination in NR sidelink mode 2 | 1) UE can transmit and receive inter-UE coordination information of preferred resource set/non-preferred resource set and use the received information in its own resource (re-)selection in NR sidelink mode 2. 2) UE can transmit and receive inter-UE coordination information of presence of expected/potential resource conflict and use the received information in its own resource re-selection in NR sidelink mode 2. 3) UE can transmit and received an explicit request for inter-UE coordination information of [FFS: preferred resource set only or both preferred resource set and non-preferred resource set]. | [32-1] | [Yes] | [Yes] | UE does not support inter-UE coordination in NR sidelink mode 2. | [Per band] | N.A. | N.A. | N.A. |  | Optional with capability signalling. FFS: For UE supports NR sidelink, UE must indicate this FG is supported. |