3GPP TSG RAN WG1 #106b	R1-2110280

e-Meeting, Oct 11th – 19th, 2021

Agenda item:		8.6.1.3

Source:	Nordic Semiconductor ASA

Title:					On other aspects related to RedCap UE

Document for:		Discussion and Decision

# Introduction

In RAN1#106 the following options were discussed in [1]

- Please share your views on the following for handling of scaling factors for RedCap UEs: - Opt. 1: Scaling factors for peak DL/UL rates with existing values {0.4, 0.75, 0.8, 1} are available to RedCap UEs, with the same constraint on the minimum value of the product vLayers(j)⋅Qmj⋅fjas applicable for single carrier NR SA operation. - No change to current specs for RedCap. - Opt. 2: Scaling factors for peak DL/UL rates with existing values {0.4, 0.75, 0.8, 1} are available to RedCap UEs, with the removal of the constraint on the minimum value of the as applicable for single carrier NR SA operation. - Opt. 3: Scaling factors for peak DL/UL rates with existing values {0.4, 0.75, 0.8, 1} and new smaller values from one or more of: {0.1, 0.2} are available to RedCap UEs, with the relaxation/removal of the constraint on the minimum value of the product vLayers(j)⋅Qmj⋅fjas applicable for single carrier NR SA operation. - Opt. 4: Scaling factors for peak DL/UL rates are NOT available to RedCap UEs. Other options are not precluded.

# Max TBS size

We suggest that legacy framework is fully re-used with relaxation of minimum value from 4->2. This means that RedCap UE should support at least TBS size scheduled with 16QAM and code-rate of 0.5. Reduction from 4->2 is due to reduction of 2-layer to 1-layer for Redcap UEs with 1Rx.

Proposal-1: Scaling factors for peak DL/UL rates with existing values {0.4, 0.75, 0.8, 1} and new smaller values from one or more of: {0.1, 0.2} are available to RedCap UEs, with the relaxation of the constraint on the minimum value of the product vLayers(j)⋅Qmj⋅fjas applicable for single carrier NR SA operation.

- minimum value is relaxed from 4 to 2

# L2 Buffer

L2 buffer size when not MR-DC or NR-DC (i.e. RedCap) is calculated in TS38.306 by MaxDLDataRate * RLC RTT + MaxULDataRate * RLC RTT.

RLC RTT for NR cell group is defined in Table 4.1.4-1

Table 4.1.4-1: RLC RTT for NR cell group per SCS

| SCS (kHz) | RLC RTT (ms) |
| --- | --- |
| 15KHz | 50 |
| 30KHz | 40 |
| 60KHz | 30 |
| 120KHz | 20 |

The max data rate is given in TS38.306 as:

Therefore, from capability point of view, data rate depends on vLayers(j)⋅Qmj⋅fjsize scaling which we believe should be reduced from 4 to 2 for RedCap UEs. Therefore, if Proposal 1 is agreed it results automatically to reduction of minimum L2 buffer size for RedCap UE.

Observation-1: If Proposal-1 is agreed, then minimum L2 buffer size for RedCap UE will reduce to half.

# References

- R1-2108316,	 “FL summary #1 on other aspects of UE complexity reduction for RedCap”,	Moderator (Intel Corporation)