3GPP TSG RAN WG1 #106bis	R1-2110284

e-Meeting, Oct 11th – 19th, 2021

Agenda item:		8.7.1.2

Source:	Nordic Semiconductor ASA

Title:					On TRS design for idle/inactive UEs

Document for:		Discussion and Decision

# Introduction

So far there has been good progress with Idle TRS design. In this contribution, we focus on the following aspects

- How to validate Idle TRS

- Indication design

- Reduction of configuration overhead

- How to further reduce power consumption with Idle TRS

# Validation of a periodic iTRS occasions

Agreement L1 based availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs is valid for a time duration starting from a reference point, where - the time duration can be determined based on at least one from the following (to be down-selected): - Alt-1: configured by higher layer - Alt-2: a predefined/configured window - Alt-3: value indicated by the availability indication, where the value is one of multiple configured time duration(s) - Alt-4: until when the UE receives another availability indication - A combination of alternatives or other alternatives is not precluded. - the reference point can be determined as at least one from the following (to be down-selected): - Alt-1: start of next PO or DRX cycle - Alt-2: time location where UE receives the indication - Note: the time location is subject to application delay if agreed - Alt-3: start of current PO or DRX cycle where UE receive the indication - Alt-4: a time location which is configured by higher layer - A combination of alternatives or other alternatives is not precluded.

In our opinion, single validation period would be sufficient, there is no need for dynamic indication of a duration (i.e. Alt-1). Such period could be configured in frames/paging-cycles and should be SCS agnostic.

When it comes to reference point, from a UE point of view, it would be sufficient to know whether one TRS before its next PO is validated or not. However, different UEs may have different (e)DRX cycles and thus this approach would be too complex. Therefore, to us it seems a better solution if validation indication received in paging cycle X applies from paging cycle X+1 (i.e. Alt-1).

Proposal-1: Down-select Alt-1 for both time duration and reference point.

Working assumption: Support paging PDCCH based availability indication of TRS/CSI-RS occasions for idle/inactive UEs. Support PEI based availability indication of TRS/CSI-RS occasions for idle/inactive UEs at least if PDCCH-based PEI is down-selected. - FFS whether and how to enable/disable L1 based availability indication configurable by SIB Agreement: Support at least L1 based signaling for the availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs. - FFS details, including paging DCI and/or PEI for L1 based signaling - FFS SIB-based signaling/configuration - Note: It is RAN1 understanding that existing SI update procedure is used for SIB based signalling

When it comes to indication of which TRS resources are to be triggered, as previously mentioned, it would be sufficient to trigger all TRS for all transmitted SSB beams, especially if Idle TRS burst is defined. However, if network vendors see benefit in triggering separate groups, we suggest that codepoint are configured, each codepoint toggles one or more TRS resource-sets. For example, one code-point may trigger all resource sets, while the other code-point only a subset.

Agreement Support at least one of the following alternatives - Alt1: L1 availability indication at an occasion provides availability/unavailability information only for RS resources with the same QCL reference as the L1 availability indication occasion. - Alt2: L1 availability indication at an occasion can provide availability/unavailability information for RS resources with QCL references not confined to be the same as for the L1 availability indication occasion Note: The occasion mentioned above refers to a signal/channel monitoring occasion (e.g. a paging PDCCH or PEI monitoring occasion) to provide the L1 availability indication. Note: a RS resource is a RS from configured TRS/CSI-RS occasion(s) for idle/inactive UEs., where the configuration for TRS/CSI-RS occasion(s) for idle/inactive UEs is based on periodic TRS only.

Alt1 will clearly reduce the PDCCH overhead and will enable gNB to indicate TRS presence independently per beam.  When considering mobility, if UE wakes-up from DRX and realizes that due to mobility beam/cell is lost, it would not be able to figure out whether TRS are available for a new beam/cell, this being an issue. However, in this case, anyway UE would need perform re-selection of beam/cell before starting to listen to paging. Therefore, our preference is Alt.1.

Proposal-2: A gNB may configure X codepoints, up to [8], each codepoint indicating validity/invalidity for a subset of all configured iTRS resource sets.

- DCI field is present in PEI PDCCH (if configured), otherwise in Paging DCI.

- L1 availability indication at a monitoring occasion provides availability/unavailability information for RS resources, of the subset of iTRS resource set(s), having the same QCL reference as the monitoring occasion.

# Configuration of iTRS

## On set of configurable parameters

Feature lead summarized all the parameters for connected CSI-RS configuration. After RAN1#105 it seems that applicable RRC parameters has been finalized. This is summarized below

| # | Parameters | Need and details | Overhead |
| --- | --- | --- | --- |
| 1 | bwp-Id | Idle TRS agreed not to be part of any BWP | 0 |
| 2 | resourceType {aperiodic, semiPersistant, periodic} | Field is not needed, as only periodic TRS are supported based on previous agreements. | 0 |
| 3 | repetition {on, off} | Not applicable to TRS | 0 |
| 4 | aperiodicTriggeringOffset | Field not needed, as only periodic TRS are supported | 0 |
| 5 | trs-Info {true} | Not applicable to TRS | 0 |
| 7 | powerControlOffset | Not applicable to TRS | 0 |
| 8 | powerControlOffsetSS | Agreed {-3, 0, 3, 6} | 2 |
| 9 | scramblingID | Agreed 0..1023 | 10 |
| 10 | periodicityAndOffset | Agreed: {10, 20, 40, 80}ms + offsets max for 120kHz SCS, this means 640+320+160+80=1200 entries | 11 |
| 11 | qcl-InfoPeriodicCSI-RS | Agreed: 0..63 | 6 |
| 12 | frequencyDomainAllocation {row1, row2, row4, others} | Agreed: {0,1,2,3} | 2 |
| 13 | nrofPorts | Not applicable to TRS | 0 |
| 14 | firstOFDMSymbolInTimeDomain | Agreed: 0-9 | 4 |
| 15 | firstOFDMSymbolInTimeDomain2 | Not applicable to TRS | 0 |
| 16 | cdm-Type | Not applicable to TRS | 0 |
| 17 | density | Not applicable to TRS | 0 |
| 18 | startingRB | Agreed: 0 to 274 | 9 |
| 19 | nrofRBs | Agreed: 24 to 276 | 8 |
| 20 | subcarrierSpacing (this is not part of CSI-RS resource configuration) | RAN1 agreed that SCS is the one of CORESET#0 | 0 |
| 21 | Others. (please provide any missing/additional parameters) |  | 0 |
|  |  | Total | Total 52bits |

Observation-1: The overhead per Idle TRS resource is 52bits

## On reduction of configuration and system overhead

It turns out that configuring resources for 64beams results in overhead of 3328bits and even if separate SIBx would be used to carry such information overhead is larger than maximum TBS of 2976 for SI message. Therefore, RAN1 should send LS to RAN2 that includes the above set of agreed parameters and ask whether RAN1 should reduce overhead by setting some parameters to be common to multiple NZP-CSI-RS resources.

Proposal-3: Send LS to RAN2 that includes the above set of agreed parameters and ask whether RAN1 should reduce overhead by making some parameters common to multiple NZP-CSI-RS resources.

## On trade-off between TRS overhead and UE power consumption

While agreed iTRS periodicities are 20,40,80,160ms. SSB periodicity would be typically 20ms, which is default periodicity assumption for SSB bursts. On one side, gNB would prefer to configure for iTRS large periodicity such as 40,80ms to reduce overhead. On the other hand, if periodicity of TRS is large, this results in unnecessary increased power consumption for the Idle UE. For example, if TRS happen to be e.g. 60ms before paging frame, UE would not benefit from such TRS, since it would need to wake-up from deep sleep at least three SSBs before PO. Therefore, for the case when TRS periodicity is larger than SSB periodicity, RAN1 should consider enabling shift/delay PF from its nominal position (PO R15) to one frame after TRS (PO R17) as shown in Figure 2. This would increase the size of paging groups and group paging rate, but such increased paging rate could be reduced by means of R17 sub-grouping. On the other hand, increased paging rate can also be reduced by shifting/delaying PF of a UE further away from TRS position, if its nominal PF position is right after TRS and the UE is known to be stationary and having good coverage so that it does not need TRS for resynchronization.

Figure 2 Delaying Paging frame after TRS

Observation-2: When TRS periodicity is larger than SSB periodicity, UEs with TRS location being far ahead of PF nominal location will have unnecessarily large power consumption.

Proposal-4: For the case when TRS periodicity is larger than SSB cycle, consider delaying UE’s PF from nominal location to frame after TRS, in order to facilitate power saving.

# Conclusions

In this contribution we discussed iTRS design and we had the following observations and proposals:

Proposal-1: Down-select Alt-1 for both time duration and reference point.

Proposal-2: A gNB may configure X codepoints, up to [8], each codepoint indicating validity/invalidity for a subset of all configured iTRS resource sets.

- DCI field is present in PEI PDCCH (if configured), otherwise in Paging DCI.

- L1 availability indication at a monitoring occasion provides availability/unavailability information for RS resources, of the subset of iTRS resource set(s), having the same QCL reference as the monitoring occasion.

Observation-1: The overhead per Idle TRS resource is 52bits

Proposal-3: Send LS to RAN2 that includes the above set of agreed parameters and ask whether RAN1 should reduce overhead by making some parameters common to multiple NZP-CSI-RS resources.

Observation-2: When TRS periodicity is larger than SSB periodicity, UEs with TRS location being far ahead of PF nominal location will have unnecessarily large power consumption.

Proposal-4: For the case when TRS periodicity is larger than SSB cycle, consider delaying UE’s PF from nominal location to frame after TRS, in order to facilitate power saving.