3GPP TSG RAN WG1#106-bis-e	           R1-2110302

e-Meeting, 11th –19th October 2021

Agenda item:	8.5.6

Source: 	Lenovo, Motorola Mobility

Title: 	Discussion on On-Demand PRS and RRC_INACTIVE Positioning

Document for:	Discussion

# Introduction

During the previous RAN1#106-e meeting [1], the following agreements related to On-demand PRS and RRC_INACTIVE positioning were reached:

Agreement: - The following lists of on-demand DL-PRS parameters are discussed/prepared by RAN1 and provided as input to RAN2: - List#1: List of parameters for UE-initiated on-demand DL PRS request - List#2: List of parameters for LMF-initiated on-demand DL PRS request - For the following lists of on-demand DL-PRS parameters, send an LS to RAN2 to check whether RAN2 would like RAN1 to send the list of parameters and request feedback as early as possible: - List #3: List of parameters for UE-initiated on-demand DL PRS request associated with pre-configured set of on-demand DL PRS configurations - List #4: List of parameters for LMF-initiated on-demand DL PRS request associated with pre-configured set of on-demand DL PRS configurations R1-2108383 LS to RAN2 with update on RAN1 discussion for on-demand DL PRS Agreement: From RAN1 perspective, it is feasible to support transmission of SRS for positioning by UEs in RRC _INACTIVE state for UL and DL+UL positioning under certain validation criteria - FFS: Type(s) of SRS for positioning (i.e., periodic, semi-persistent, aperiodic) - FFS: Details of validation criteria which may also be discussed in RAN2 - Send LS to RAN2 informing them of this agreement Agreement: Open loop power control defined in Rel.16 for transmission of SRS for positioning by RRC_CONNECTED UEs is applicable for RRC_INACTIVE UEs. Agreement: Spatial relation defined in Rel.16 for transmission of SRS for positioning by RRC_CONNECTED UEs is applicable for RRC_INACTIVE UEs. Conclusion: It is up to RAN2 to define TA procedures for SRS for positioning transmission by RRC_INACTIVE UEs. R1-2108563 Draft LS to RAN2 on SRS for Positioning Transmission by UEs in RRC_INACTIVE State Moderator (Intel Corporation) R1-2108564 LS to RAN2 on SRS for Positioning Transmission by UEs in RRC_INACTIVE State RAN1, Intel Corporation Agreement: At least the following list of on-demand DL PRS parameters is supported for UE-initiated and LMF-initiated on-demand DL PRS requests 1. DL PRS Periodicity 2. DL PRS resource bandwidth 3. DL PRS QCL information Conclude on remaining parameters at RAN1#106-bis-e

This contribution discusses further related issues for both on-demand PRS and RRC_INACTIVE positioning.

# On-demand PRS

An initial set of on-demand PRS parameters have been agreed upon for LMF-initiated and the UE-initiated On-demand PRS procedures, which include DL PRS Periodicity, DL PRS Resource bandwidth and DL PRS QCL information. The remaining parameters and issues are discussed the following Sections.

## UE-initiated and LMF-initiated On-demand PRS List

An aspect of discussion from the previous RAN1#106-e meeting was the provision of pre-configured on-demand PRS lists to RAN2 as reflected in List#3 and List#4 [2]. Based on the latest reply LS by RAN2 [3], the intention of the original LS [4] was to receive feedback on the list of physical layer parameters that can be dynamically adjusted for UE-initiated and LMF-initiated DL-PRS. Therefore, a reply LS can be sent to RAN2 containing only List#1 and List#2 comprising of UE-initiated and LMF-initiated on-demand DL-PRS parameters, once the lists have been finalized.

Proposal 1: Send a reply LS to RAN2 containing List#1 (UE-initiated) and List#2 (LMF-initiated) DL-PRS, after the on-demand parameters have been agreed upon.

Table 1 shows the supported UE-initiated on-demand PRS parameters list from our perspective:

Table 1: Supported UE-initiated and LMF-initiated on-demand PRS parameters

| Parameter | Example parameter definition | UE-initiated | LMF-Initiated |
| --- | --- | --- | --- |
| Start/end time of DL PRS transmission | Time interval / time window in slots recommended for DL PRS transmissions within SFN cycle. Note: It may contain one or more DL PRS occasions/periods which periodicity and length are indicated separately | Yes | Yes |
| Number of DL PRS frequency layers | Recommended number of DL PRS frequency layers | Yes | Yes |
| DL PRS frequency layer indicator | Recommended DL PRS frequency layers | Yes | - |
| DL-PRS Resource Bandwidth | Number of PRBs recommended for the DL-PRS Resource (allocated DL-PRS bandwidth) in multiples of 4 PRBs. | Agreed in RAN#106-e | Agreed in RAN#106-e |
| DL-PRS Subcarrier Spacing | Subcarrier spacing of the DL-PRS Resource | - | - |
| DL-PRS Start PRB | Start PRB index defined as offset with respect to reference DL-PRS Point A for the Positioning Frequency Layer | Yes | Yes |
| DL-PRS Point A | Absolute frequency of the reference resource block for the DL-PRS | - | - |
| DL-PRS CombSizeN | Resource element spacing in each symbol of the DL-PRS Resource | Yes | Yes |
| DL-PRS Cyclic Prefix | Cyclic Prefix length of the DL-PRS Resource | - | - |
| DL PRS resource ID(s) | List of recommended DL PRS Resource ID(s) per DL PRS Resource Set | Yes | Yes |
| DL PRS resource set ID(s) | List of DL PRS Resource Set ID(s) per DL PRS frequency layer per TRP | Yes | Yes |
| DL PRS Periodicity and ResourceSetSlotOffset | Recommended periodicity of DL-PRS allocation in slots configured per DL-PRS Resource Set and the slot offset with respect to SFN #0 slot #0 | Agreed in RAN#106-e | Agreed in RAN#106-e |
| DL PRS Resource Repetition Factor | Recommended number of DL-PRS Resource repetitions for a single instance of the DL-PRS Resource Set | Yes | Yes |
| DL PRS Resource Time Gap | Recommended offset in units of slots between two repeated instances of a DL-PRS Resource corresponding to the same DL-PRS Resource ID within a single instance of the DL-PRS Resource Set | Yes | Yes |
| Number of DL PRS Resource Symbols per DL PRS resource | Recommended number of symbols per DL-PRS Resource within a slot | Yes | Yes |
| DL PRS Resource Power | Recommended average EPRE of the resource elements that carry the PRS in dBm that is used for PRS transmission. The UE assumes constant EPRE is used for all REs of a given DL-PRS resource. | - | - |
| DL PRS Muting Option 1 | Recommended DL-PRS muting configuration of the TRP for the Option-1 | - | Yes |
| DL PRS Muting Option 2 | Recommended DL-PRS muting configuration of the TRP for the Option-2 | - | Yes |
| DL PRS QCL information (DL-PRS-QCL-Info) | QCL indication with other DL reference signals for serving and neighbouring cells per DL PRS Resource | Agreed in RAN#106-e | Agreed in RAN#106-e |
| Number of TRPs | Recommended number of TRPs [total or per frequency layer] | Yes | Yes |
| Number of DL PRS resources per DL PRS resource set | Recommended number of DL PRS resources per DL PRS resource set [per frequency layer] | Yes | Yes |
| Beam directions | Recommended beam directions | Yes | Yes |
| ON/OFF indicator of the DL PRS | Recommended indication to switch ON/OFF DL PRS transmission [per frequency layer or per TRP or per DL PRS resource set or per DL PRS resource] | Yes, per TRP | Yes, per TRP |
| dl-PRS-ID(s) | Along with a DL-PRS Resource Set ID and a DL-PRS Resources ID to uniquely identify a DL-PRS Resource. This ID can be associated with multiple DL-PRS Resource Sets associated with a single TRP | - | - |
| nr-PhysCellID | The physical cell identity of the associated TRP | - | - |
| nr-CellGlobalID | Identifies the globally unique identity of a cell in NR, of the associated TRP | - | - |
| nr-ARFCN | Specifies the NR-ARFCN of the TRP | - | - |
| Priority Order of TRPs and PRS resource sets | This field enables the UE to process TRPs and PRS resource sets according to a configured explicit priority, which is requested by the UE/configured by the LMF. | Yes | - |
| TRP Indication/ Group TRP Indication | This field specifies which TRPs should transmit DL PRS based on the on/off indicator parameter described by “ON/OFF indicator of the DL PRS” | Yes | Yes |

Proposal 2: Support the indicated UE-initiated and LMF-initiated parameters as shown in Table 1.

The UE measurement accuracy or radio conditions experienced at the UE can be attributed to the quality of the DL-PRS resources transmitted over a particular TRP. The UE may prioritize certain TRPs (and hence DL-PRS resources) based on certain factors such as:

- Location of the UE

- Radio link conditions of a TRP/set of TRPs

- Depending on different UE requirements and capabilities

This can be especially applicable for UE-initiated on-demand PRS, where the UE requests a certain priority based on e.g., RS measurements.  Therefore, the priority order of the of TRPs and associated resource sets should be at least considered for UE-initiated on-demand DL-PRS.

Proposal 3: Support TRP Priority order indication as part of UE-initiated On-demand PRS.

The “number of TRPs” parameter is not sufficient as it only indicates the recommended number of TRPs carrying DL-PRS. It would also be beneficial to collectively switch off adjacent beams carrying DL-PRS, which are not relevant for the calculation of the UE’s position estimate.

In the case of UE-initiated on-demand DL-PRS, the UE (or group of UEs) may request the LMF to switch on/off certain TRPs based on the measurement quality and in some cases the UE’s mobility pattern. Furthermore, TRPs/links with consistently poor quality may result in frequent beam failure and may not be recommended for DL-PRS transmission. Such recommended feedback on which TRPs are suitable/recommended for DL PRS transmissions can only be provided by the UE.

For the case of LMF-initiated on-demand DL-PRS, the LMF may have prior knowledge of the UE’s rough/course location and therefore can provide a recommendation to the serving gNB to switch on/off DL-PRS transmissions on applicable TRPs.

Observation 1: “Number of TRPs” only indicates the recommended total number of TRPs transmitting DL-PRS and does not indicate which of those TRPs are best suited for UE measurement.

This can be achieved by individually signalling which Tx beams carrying DL-PRS resources to switch on/off, which may incur signalling overhead for multiple beams/TRPs.  Alternatively, an optimized signalling approach would be for the LMF to group adjacent beams which may be relevant or not relevant for the UE, via a single TRP group indication ID to switch on/off multiple beams efficiently and collectively.

Proposal 4: Support ON/OFF indicator of the DL PRS per TRP and associated TRP indication/Group TRP indication for signalling which TRPs to switch on/off.

## Two-stage PRS beam sweeping

Two-stage PRS beam sweeping offers the opportunity to refine the beams carrying DL-PRS resources for improved accuracy measurement at the UE. This feature can be somewhat supported in the existing specification, where two DL-PRS resource sets are configured per TRP. The first DL-PRS resource set could correspond to transmissions over a wider beam, while the second DL-PRS resource set over a narrower beam. However, this specification is ambiguous in enabling this aspect and should be clarified.

In addition, the association of 2 DL PRS resource sets belonging to the same TRP can be associated by a grouping these PRS resource sets for the purposes of 2 stage beam sweeping. Different pairs of resource sets can be differentiated by different Group IDs. If a same grouping ID is associated to 2 different PRS resource set IDs, then the UE is not expected to receive the corresponding DL-PRS transmissions (from respective Tx beams) using the same spatial filter at the receiver, enabling UE reception with different beam widths.

Proposal 5: Association between resources belonging to two DL PRS resource sets of the same TRP can be enabled by a grouping ID and can be signalled in the assistance data.

The 2-stage beam sweeping procedure can also supported by the on-demand PRS procedure, where the UE might request the LMF to configure an updated set of DL-PRS resources to be transmitted using a narrower beam, which can assist the LMF better resolve the DL-AoD for improved positioning accuracy.

Proposal 6: Two-stage PRS beam sweeping can be enabled by the on-demand PRS mechanism.

# RRC_INACTIVE Positioning

## DL-based Positioning Configuration and Capabilities

During the RAN1#105-e meeting, it was agreed to update the DL PRS and DL RSTD measurements in TS 38.215 to be supported in RRC_INACTIVE state. A remaining issue was to identify any remaining additional impacts.

The applicability of the DL-PRS physical layer configuration in addition to UE behavior and capability of performing DL-only measurements in RRC_INACTIVE state is yet to be finalized. RRC_CONNECTED state offers a higher degree of freedom in which to perform the applicable positioning measurements in terms of periodicity and number of occasions, which may differ from positioning measurement performed in RRC_INACTIVE state, which can affect the achievable accuracy. More specifically, if the measurement requirements are different between RRC_CONNECTED and RRC_INACTIVE states, the LMF may need to adapt the DL-PRS configuration accordingly, e.g., based on the UE DRX configuration, which impacts the energy consumption.

Observation 2: DL-only measurement durations and periods may be different between RRC_CONNECTED and RRC_INACTIVE states due to e.g., UE DRX configuration, which may affect the overall location accuracy and power consumption.

Proposal 7: RAN1 to consider the DL-PRS configuration impact on measurement accuracy in RRC_INACTIVE state. FFS solutions to address this gap, e.g., separate DL-PRS configurations for RRC_INACTIVE UEs, sharing of the UE DRX configuration with the LMF for optimal RRC_INACTIVE measurements, etc.

It is also intuitive that the capability to perform DL-based/UL-based RRC_INACTIVE positioning will differ from RRC_CONNECTED UEs. A separate capability for RRC_INACTIVE UEs will also need to be considered.

Proposal 8: RAN1 to support separate capabilities of UEs performing RRC_INACTIVE positioning.

# Conclusion

The following observation were noted:

Observation 1: “Number of TRPs” only indicates the recommended total number of TRPs transmitting DL-PRS and does not indicate which of those TRPs are best suited for UE measurement.

Observation 2: DL-only measurement durations and periods may be different between RRC_CONNECTED and RRC_INACTIVE states due to e.g., UE DRX configuration, which may affect the overall location accuracy and power consumption.

Based on the discussion, the following proposals are summarized as follows:

Proposal 1: Send a reply LS to RAN2 containing List#1 (UE-initiated) and List#2 (LMF-initiated) DL-PRS, after the on-demand parameters have been agreed upon.

Proposal 2: Support the indicated UE-initiated and LMF-initiated parameters as shown in Table 1.

Proposal 3: Support TRP Priority order indication as part of UE-initiated On-demand PRS.

Proposal 4: Support ON/OFF indicator of the DL PRS per TRP and associated TRP indication/Group TRP indication for signalling which TRPs to switch on/off.

Proposal 5: Association between resources belonging to two DL PRS resource sets of the same TRP can be enabled by a grouping ID and can be signalled in the assistance data.

Proposal 6: Two-stage PRS beam sweeping can be enabled by the on-demand PRS mechanism.

Proposal 7: RAN1 to consider the DL-PRS configuration impact on measurement accuracy in RRC_INACTIVE state. FFS solutions to address this gap, e.g., separate DL-PRS configurations for RRC_INACTIVE UEs, sharing of the UE DRX configuration with the LMF for optimal RRC_INACTIVE measurements, etc.

Proposal 8: RAN1 to support separate capabilities of UEs performing RRC_INACTIVE positioning.

# References

- R1-2108605, “Session Notes for 8.5 (NR Positioning Enhancements)”, Ad-Hoc Chair (Ericsson), 3GPP RAN1#106-e, Aug. 2021.

- R1-2108294, “Feature Lead Summary#3 for E-mail Discussion [106-e-NR-ePos-06]”, Moderator (Intel), 3GPP RAN1#106-e, Aug. 2021.

- R1-2108711, “Reply LS to RAN1 on on-demand DL PRS parameters”, RAN2#115-e, Aug. 2021.

- R1-2106412, “LS to RAN1 on parameters for on-demand PRS”, RAN2#114-e, May 2021.