3GPP TSG-RAN WG1 Meeting #106bis-e	R1-2110318

e-Meeting, October 11th – 19th, 2021

Agenda Item:	8.9.3

Source:	Ericsson

Title:	On the support of 16-QAM for unicast in UL and DL for TDD NB-IoT

Document for:	Discussion

# 1	Introduction

In the Work Item (WI) on “Rel-17 enhancements for NB-IoT and LTE-MTC” [1], one of the objectives is to specify the following enhancement for NB-IoT:

- Specify 16-QAM for unicast in UL and DL, including necessary changes to DL power allocation for NPDSCH and DL TBS. This is to be specified without a new NB-IoT UE category. For DL, increase in maximum TBS of e.g. 2x the Rel-16 maximum, and soft buffer size will be specified by modifying at least existing Category NB2. For UL, the maximum TBS is not increased. [NB-IoT] [RAN1, RAN4] - Extend the NB-IoT channel quality reporting based on the framework of Rel-14—16, to support 16-QAM in DL. [NB-IoT] [RAN2, RAN1, RAN4]

While discussing the UE Feature list, a question was raised on whether there is a “Need of FDD/TDD differentiation”, and in this contribution we intent to find out whether the agreements we have reached to support 16-QAM hold for TDD and what would be the impact in other Working Groups.

# 2	Rel-17 RRC parameters and Specification impacts

Envisioning the finalization of Rel-17, discussions on the UE parameter list have been started. The template of the UE parameter lists contains a question on the “Need of FDD/TDD differentiation”. On this matter, we have the following observations:

The WID left open the applicability of 16-QAM for unicast in UL and DL for both FDD and TDD, hence it needs to be investigated whether the agreements we have reached to support 16-QAM hold for TDD and what would be the impact in other Working Groups.

A preliminary assessment on RAN1 aspects touching upon TBS/MCS tables, DCI designs, DL power allocation, UL power control term, channel quality reporting, etc, seem to indicate that there won’t be any incompatibility issue with TDD.

Nonetheless, it seems that from a RAN4 perspective there might be a need to add another Test Model (in the BS conformance specs) and NPDSCH demodulation requirements for supporting 16-QAM in TDD.

Operators, UE vendors, Network vendors and other interested parties should discuss the relevance of supporting 16-QAM for TDD NB-IoT, especially if supporting it results in an extra impact in any of the Working Groups involved.

Thus, we have the following proposals:

Discuss whether the agreements we have reached to support 16-QAM in UL and DL hold for TDD and what are the impacts in other Working Groups.

Decide whether 16-QAM for unicast in UL and DL is applicable for both FDD and TDD.

# 6	Conclusion

Based on the discussion in the previous section we made the following observations:

Observation 1	The WID left open the applicability of 16-QAM for unicast in UL and DL for both FDD and TDD, hence it needs to be investigated whether the agreements we have reached to support 16-QAM hold for TDD and what would be the impact in other Working Groups.

Observation 2	A preliminary assessment on RAN1 aspects touching upon TBS/MCS tables, DCI designs, DL power allocation, UL power control term, channel quality reporting, etc, seem to indicate that there won’t be any incompatibility issue with TDD.

Observation 3	Nonetheless, it seems that from a RAN4 perspective there might be a need to add another Test Model (in the BS conformance specs) and NPDSCH demodulation requirements for supporting 16-QAM in TDD.

Observation 4	Operators, UE vendors, Network vendors and other interested parties should discuss the relevance of supporting 16-QAM for TDD NB-IoT, especially if supporting it results in an extra impact in any of the Working Groups involved.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Discuss whether the agreements we have reached to support 16-QAM in UL and DL hold for TDD and what are the impacts in other Working Groups.

Proposal 2	Decide whether 16-QAM for unicast in UL and DL is applicable for both FDD and TDD.

# References

RP-201306, “WID revision: Additional enhancements for NB-IoT and LTE-MTC”, RAN #88e, Electronic Meeting, June 29th – July 3rd, 2020.