3GPP TSG RAN WG1 #106bis-e	R1-2110342

e-Meeting, October 11th – 19th, 2021

Agenda Item:	8.17.11

Source:	Ericsson

Title:	UE features for NR sidelink enhancement

Document for:	Discussion, Decision

# 1	Introduction

In this paper, we discuss the UE features for NR Rel-17 that are included in the draft document R1-2108678 and R1-2108679 and the potential addition of other FGs in order to be aligned with the RAN1 agreements.

# 2	Feature group discussion

In this section, we provide some comments and modifications to the feature groups considered by the moderator in the document R1-2108678 and R1-2108679 for LTE and NR, respectively [1, 2].

As a general comment for this RAN1 meeting, we propose to focus only on the addition and the clear definition of the FG to be added in Rel-17 and avoid going into discussions regarding other details of the FGs which can be done once the list of FGs is stable.

- In order to avoid discussion regarding details that can be discussed in further meetings, we suggest focusing on the addition and definition of the needed FGs.

## 2.1	LTE UE Feature List (R1-2108678)

The list of UE features which are studied and potentially introduced in Rel-17 are only relevant for UEs which use NR Rel-17 technology/procedures. Therefore, there is no need to include or update any of the FG in the LTE feature list.

There is no need to include or update any of the FG in the LTE feature list. Therefore, we propose to remove all the SL_enh feature groups included in R1-2108678.

## 2.2	Feature group 32-1 and 32-2

The feature group 32-1 indicates the behaviour of a UE which is capable of receiving all the SL signalling, i.e., similar capability as a Rel-16 UE. On the other hand, the feature group 32-2 indicates a UE with a reduced SL reception capability, i.e., only able to receive the PSFCH and S-SSB. We are supportive of both FG since they are aligned with the agreements and conclusion taken in RAN1.

Support the inclusion of FG 32-1 and FG 32-2 including its general description.

## 2.3	Feature group 32-3 and 32-4

The feature group 32-3 indicates the full sensing operation that a UE performs similar to NR SL Rel-16 procedure. For the feature group 32-4, the UE is capable of performing partial sensing operation as defined in RAN1 agreements. Following a similar approach as in LTE Rel-14, we support to have separate FGs for full sensing and partial sensing procedures.

Support the inclusion of FG 32-3 and FG 32-4. FFS details and wording of the FG.

## 2.4	New feature group for UEs transmitting SL mode 2 without sensing

The feature groups 32-3 and 32-4 define the sensing behaviour of the UE when performing mode 2 resource allocation for sidelink. In our view, we need to also include a new feature group to indicate that mode 2 RA for sidelink can be performed without performing sensing. The inclusion of this feature group is also aligned with the following agreement reached in the inter-UE coordination discussion for scheme 1:

Agreement In scheme 1, at least following UE-B’s behavior in its resource (re-)selection is supported when it receives inter-UE coordination information from UE-A: - For preferred resource set, the following two options are supported: - Option A): UE-B’s resource(s) to be used for its transmission resource (re-)selection is based on both UE-B’s sensing result (if available) and the received coordination information - UE-B uses in its resource (re-)selection, resource(s) belonging to the preferred resource set in combination with its own sensing result - UE-B uses in its resource (re-)selection, resource(s) not belonging to the preferred resource set when condition(s) are met - FFS: Details of condition(s) - This option is supported when UE-B performs sensing/resource exclusion - FFS: Other details (if any) - Option B): UE-B’s resource(s) to be used for its transmission resource (re-)selection is based only on the received coordination information - UE-B uses in its resource (re-)selection, resource(s) belonging to the preferred resource set - This option is supported at least when UE-B does not support sensing/resource exclusion - FFS: Whether the support is conditional or UE capability - FFS: Other details (if any)

As indicated in the text marked in yellow, a UE that it is not capable of performing sensing/resource exclusion should indicate this capability to the other UEs in order to be participant of the inter-UE coordination under certain circumstances. Therefore, we propose to include an FG indicating this capability.

Include a new FG to indicate that a UE can perform transmission in NR sidelink mode 2 without sensing.

## 2.5	Feature group 32-5

For this feature group, we propose to split the feature into two different groups due to the different schemes and potential requirements as agreed for inter-UE coordination framework in RAN1:

- The requirements that are needed for each of the Inter-UE coordination schemes agreed in RAN1, i.e., Scheme 1 and Scheme 2, can be different.

### 2.5.1	Feature group 32-5-1: Inter-UE coordination in NR sidelink mode 2 scheme 1

We propose to include the following FG 32-5-1 to include the capability for the inter-UE coordination scheme 1.

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32-5-1 | Inter-UE coordination in NR sidelink mode 2 scheme 1 | 1) UE can transmit and receive inter-UE coordination information of preferred resource set/non-preferred resource set and use the received information in its own resource (re-)selection in NR sidelink mode 2. 2) UE can transmit and received an explicit request for inter-UE coordination information of [FFS: preferred resource set only or both preferred resource set and non-preferred resource set]. | [32-1] | [Yes] | [Yes] | UE does not support inter-UE coordination scheme 1 in NR sidelink mode 2. | Per UE | N.A. | N.A. | N.A. |  | Optional with capability signalling. FFS: For UE supports NR sidelink, UE must indicate this FG is supported. |

### 2.5.2	Feature group 32-5-2: Inter-UE coordination in NR sidelink mode 2 scheme 2

We propose to include the following FG 32-5-2 to include the capability for the inter-UE coordination scheme 2.

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32-5-2 | Inter-UE coordination in NR sidelink mode 2 scheme 2 | UE can transmit and receive inter-UE coordination information of presence of expected/potential resource conflict and use the received information in its own resource re-selection in NR sidelink mode 2. | [32-1] [32-2] | [Yes] | [Yes] | UE does not support inter-UE coordination scheme 2 in NR sidelink mode 2. | Per UE | N.A. | N.A. | N.A. |  | Optional with capability signalling. FFS: For UE supports NR sidelink, UE must indicate this FG is supported. |

The motivation of the proposed changes (in red in the tables) is as follows:

- In our view, making mandatory that in case a UE supports the inter-UE coordination mechanism, it has to support both schemes is not feasible. Therefore, we propose to divide the FG into two different groups, one for Scheme 1 and another for Scheme 2.

- For Scheme 1 since the coordination message is likely to be included in MAC CE/PC5-RRC or PSSCH, it requires to support the group 32-1 which indicates that the UE is able to receive all of the SL signals.

- For Scheme 2, since the coordination message is likely to be included within the PSFCH resources, i.e., coexisting with the HARQ-ACK resources, then the UE can support this feature by supporting either 32-1 or 32-2.

- The FGs for inter-UE coordination mechanism can be defined per UE.

Divide the 32-5 FG into two different FG as defined above to indicate whether the UE supports either Scheme 1 and/or Scheme 2 as agreed in RAN1.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	In order to avoid discussion regarding details that can be discussed in further meetings, we suggest focusing on the addition and definition of the needed FGs.

Observation 2	The requirements that are needed for each of the Inter-UE coordination schemes agreed in RAN1, i.e., Scheme 1 and Scheme 2, can be different.

Based on the discussion in the previous sections we propose the following:

Proposal 1	There is no need to include or update any of the FG in the LTE feature list. Therefore, we propose to remove all the SL_enh feature groups included in R1-2108678.

Proposal 2	Support the inclusion of FG 32-1 and FG 32-2 including its general description.

Proposal 3	Support the inclusion of FG 32-3 and FG 32-4. FFS details and wording of the FG.

Proposal 4	Include a new FG to indicate that a UE can perform transmission in NR sidelink mode 2 without sensing.

Proposal 5	Divide the 32-5 FG into two different FG as defined above to indicate whether the UE supports either Scheme 1 and/or Scheme 2 as agreed in RAN1.

# 4	References

R1-2108678, “Preliminary RAN1 UE features list for Rel-17 LTE”, Moderators (AT&T, NTT DOCOMO, INC.), e-Meeting, August 16th- 27th, 2021

R1-2108679, “Preliminary RAN1 UE features list for Rel-17 NR”, Moderators (AT&T, NTT DOCOMO, INC.), e-Meeting, August 16th- 27th, 2021