3GPP TSG-RAN WG1 Meeting #106-bis-e	R1- 2110347

e-Meeting, October 11th – 19th, 2021

Agenda Item:	8.17.5

Source:	Ericsson

Title:	Views on NR positioning Enhancements UE features

Document for:	Discussion, Decision

- Introduction

This contribution provides our view on the list of UE features for NR positioning in release17, based on the initial email discussion outcome in R1-2108679.

- UE Features for NR positioning enhancements

## (27x1) Mitigation of UE Rx timing delays

For UE Tx TEG reporting, the following agreement was made RAN1#104bis-e::

Agreement: - Support the following for mitigating TRP Tx timing errors and/or UE Rx timing errors for DL TDOA - Support a UE to provide the association information of RSTD measurements with UE Rx TEG(s) to the LMF when the UE reports the RSTD measurements to the LMF if the UE has multiple TEGs - Support a TRP providing the association information of DL PRS resources with Tx TEGs to the LMF if the TRP has multiple TEGs - Support the LMF to provide the association information of DL PRS resources with Tx TEGs to a UE for UE-based positioning if the TRP has multiple TEGs - FFS: the details of the signaling, procedures, and UE capability - Send an LS to RAN4 to check if there is any issue to support the above enhancements

From the above agreement, UE Rx TEG reporting is only relevant if the UE has multiple Rx TEGs.  Hence, when the UE reports the capability for ‘(27x1) Mitigation of UE Rx timing delays’, there is no need to include the value of 1 for the maximum number of UE Rx TEGs.  Hence, we suggest to remove ‘FFS: whether to have a value=1 to indicate UE Rx timing errors is well calibrated’.

Furthermore, the reporting of maximum number of UE Rx TEGs is applicable to multi-RTT positioning as well.  Hence, we suggest to add multi-RTT positioning to the top level component description (i.e., The maximum number of UE-RxTEG per UE, which is supported and reported by UE for DL TDOA and/or Multi-RTT positioning)

As for capability reporting of the maximum number of UE Rx TEGs, we do not see the need to report different values for DL TDOA and Multi-RTT positioning.  Hence, we suggest to remove ‘FFF: whether to have separate values for DL TDOA and/or Multi-RTT positioning’

Our suggested changes are summarized below:

| 27. NR_pos_enh | 27-x1 | Mitigation of UE Rx timing delays | The maximum number of UE-RxTEG per UE, which is supported and reported by UE for DL TDOA and/or Multi-RTT positioning FFS: the values (>1). |
| --- | --- | --- | --- |

## (27x2) Mitigation of UE Tx timing delay

The following was agreed in RAN1#106e:

Agreement: Support the following for mitigating UE Tx timing errors and/or TRP Rx timing errors for UL TDOA - Support a TRP to provide the association information of RTOA measurements with TRP Rx TEG(s) to the LMF when the TRP reports the RTOA measurements to the LMF if the TRP has multiple Rx TEGs - Support a UE to provide under capability the association information of UL SRS resources for positioning with Tx TEGs to the LMF if the UE has multiple Tx TEGs - FFS: Whether to support a UE to provide the association information of UL SRS resources for MIMO with Tx TEGs to the LMF if the UE has multiple Tx TEGs - FFS: Whether the association information is sent directly from UE to LMF, or is first provided to gNB and then forwarded to LMF - FFS: the details of the Signaling, procedures, and UE capability

From the above agreement, UE Tx TEG association with UL SRS is only relevant if the UE has multiple Tx TEGs.  Hence, when the UE reports the capability for ‘(27x2) Mitigation of UE Tx timing delays’, there is no need to include the value of 1 for the maximum number of UE Tx TEGs.  Hence, we suggest to remove ‘FFS: whether to have a value=1 to indicate UE Tx timing errors is well calibrated’

As for capability reporting of the maximum number of UE Tx TEGs, we do not see the need to report different values for DL TDOA and Multi-RTT positioning.  Hence, we suggest to remove ‘FF: whether a UE supports different values for UL TDOA and/or Multi-RTT positioning’

| 27. NR_pos_enh | 27-x2 | Mitigation of UE Tx timing delays | The maximum number of UE-TxTEG per UE, which is supported and reported by UE for UL TDOA and/or Multi-RTT positioning FFS; the values (>1). |
| --- | --- | --- | --- |

### (27-v2) Multipath  reporting

The following agreement was made at RAN1#106e:

Agreement: - For up to N>2 additional paths, support reporting relative timing (to the first detected path) in the measurement reports from UE to LMF for at least DL-TDOA and multi-RTT - FFS: Definition of additional paths for N>2 - FFS: Whether power is additionally reported and if reported whether power is relative to first detected path or total power - Support one of the following options for maximum value of N at RAN1#106-b (any further criteria for selection to be discussed during RAN1#106): - Option 1: N = 4 - Option 2: N = 8 - Option 3: N = 16 - Option 4: N = 32

Since the maximum number of additional path is a UE capability, the following feature is proposed to be added:

| 27. NR_pos_enh | 27-v2 | Multipath reporting | UE’s capability to support up to N>2 additional path relative timing reporting for DL-TDOA and multi RTT FFS: value of N FFS: whether to have separate capability component for DL-TDOA and UE multi-RTT additional path relative timing reporting time difference measurements. FFS: additional capability to also include power reporting. |
| --- | --- | --- | --- |

### (27-w2)support of SRS in RRC_INACTIVE

The following agreement was made at RAN1#106e:

Agreement: From RAN1 perspective, it is feasible to support transmission of SRS for positioning by UEs in RRC _INACTIVE state for UL and DL+UL positioning under certain validation criteria - FFS: Type(s) of SRS for positioning (i.e., periodic, semi-persistent, aperiodic) - FFS: Details of validation criteria which may also be discussed in RAN2 - Send LS to RAN2 informing them of this agreement

The transmission of the SRS in RRC_INACTIVE should be a UE capability, since it requires a specific RRC release message to be supported. The following feature is proposed to be added:

| 27. NR_pos_enh | 27-w2 | Support of SRS in RRC_INACTIVE state | UE’s capability to support transmission of the SRS for positioning in RRC_INACTIVE state FFS: which SRS type (periodic, aperiodic, semi persistent) |
| --- | --- | --- | --- |

# Conclusion

We have proposed to modify/append 4 UE features for NR positioning enhancements. A full UE feature list in tracked changes is provided in section 4.

- References

- R1-2108679, Preliminary RAN1 UE features list for Rel-17 NR, Moderators (AT&T, NTT DOCOMO, INC.), RAN1#106e

- UE Feature List for NR_pos_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_pos_enh | 27-x1 | Mitigation of UE Rx timing delays | The maximum number of UE-RxTEG per UE, which is supported and reported by UE for DL TDOA and/or Multi-RTT positioning FFS: the values (>1). |  | No |  | Mitigation of UE Rx timing delays is not supported | Per UE | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 27. NR_pos_enh | 27-x2 | Mitigation of UE Tx timing delays | The maximum number of UE-TxTEG per UE, which is supported and reported by UE for UL TDOA and/or Multi-RTT positioning FFS; the values (>1). |  | FFS |  | Mitigation of UE Tx timing delays is not supported | FFS: Per UE or per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. FFS: whether gNB needs to know if the feature is supported. | Optional with capability signaling |
| 27. NR_pos_enh | 27-x3 | Mitigation of UE RxTx timing delays | The maximum number of UE-RxTxTEG per UE, which is supported and reported by UE for Multi-RTT positioning FFS; the values (>1) FFS: whether to have a value=1 to indicate UE RxTx timing errors is well calibrated |  | No |  | Mitigation of UE RxTx timing delays is not supported | Per UE | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 27. NR_pos_enh | 27-x4 | The maximum Number of UE Rx TEGs for measuring the same DL PRS resource | The maximum number of different UE-RxTEGs that a UE can support to measure the same DL PRS of a TRP. FFS; The values (>1). | 27-x1 | No |  | Mitigation of UE Rx timing delays by using different Rx TEGs are not supported | Per UE | n/a | n/a | n/a | Need for location server to know if the feature is supported. |  |
| 27. NR_pos_enh | 27-z1 | Support of PRS RSRP of the first path | UE’s capability to support providing the PRS RSRP of the first path |  | No |  |  | Per UE | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 27. NR_pos_enh | 27-z2 | Support of DL PRS RSRP reporting for more than 8 measurements. | UE capability to support reporting K> 8 DL PRS RSRP measurements per TRP. FFS: the values of K •Note: Multiple RSRPs corresponding to same or different Rx Beam index should be able to be reported for a given PRS resource for different timestamps. Note: Additional capability may be added: •FFS: Limit the maximum number of DL PRS RSRP associated with the same Rx beam index |  | No |  | UE report of more than 8 DL PRS-RSRP is not supported. | Per UE | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 27. NR_pos_enh | 27-u1 | Support of M-sample measurements. | The capability to support providing a measurement based on measuring M samples (instances) of a DL PRS resurce set M=[1, 4]. FFS: other values. If the UE does not provide the capability, the UE is assume to support M=4 only. |  | No |  |  | Per UE | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 27. NR_pos_enh | 27-u5 | PRS measurement outside MG | 1. Supported PRS processing types subject to the UE determining that DL PRS to be higher priority for PRS measurement outside MG. Candidate values: {Type 1A, Type 1B, Type 2}. Note: - Type 1A refers to DL PRS being prioritized over other DL signals/channels in all OFDM symbols within the PRS processing priority window. The DL signals/channels from all DL CCs (per UE) are affected. - Type 1B refers to DL PRS being prioritized over other DL signals/channels in all OFDM symbols within the PRS processing priority window. The DL signals/channels from certain DL CCs are affected. - Type 2 refers to DL PRS being prioritized over other DL signals/channels only in DL PRS symbols within the PRS processing priority window. Note: When the UE determines higher priority for other DL signals/channels over the PRS measurement/processing, the UE is not expected to measure/process DL PRS which is applicable to all of the above capability options. |  | No |  |  | FFS: Per UE or per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 27. NR_pos_enh | 27-u6 | PRS Processing Capability outside MG | 1. DL PRS buffering capability: Type 1 or Type 2 a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering 2. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. a) Type 1 – sub-slot/symbol level buffering b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms 3. Max number of DL PRS resources that UE can process in a slot under it a) FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b) FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz | 27-u5 | No |  |  | Per band | n/a |  |  | FFS |  |
| 27. NR_pos_enh | 27-v1 | LOS/NLOS Indicator | UE’s capability to support reporting LoS/NLoS indicator to LMF for RSTD and UE Rx-Tx time difference measurements to LMF for DL and DL+UL positioning. FFS: whether to have separate capability component for RSTD and UE Rx-Tx time difference measurements. |  | No |  |  | Per UE | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 27. NR_pos_enh | 27-v2 | Multipath reporting | UE’s capability to support up to N>2 additional path relative timing reporting for DL-TDOA and multi RTT FFS: value of N FFS: whether to have separate capability component for DL-TDOA and UE multi-RTT additional path relative timing reporting time difference measurements. FFS: additional capability to also include power reporting. |  |  |  |  |  |  |  |  |  |  |
| 27. NR_pos_enh | 27-w1 | Support of on-demand PRS | UE’s capability to support UE-initiated on-demand PRS. |  | No |  |  | Per UE | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 27. NR_pos_enh | 27-w2 | Support of SRS in RRC_INACTIVE state | UE’s capability to support transmission of the SRS for positioning in RRC_INACTIVE state FFS: which SRS type (periodic, aperiodic, semi persistent) |  | No |  |  | Per UE | n/a | n/a | n/a | Need for gNB and location server to know if the feature is supported. | Optional with capability signaling |