3GPP TSG RAN WG1 Meeting #106bis-e 	R1-2110422

e-Meeting, October 11th – 19th, 2021

Agenda Item:	8.1.4

Source:	Huawei, HiSilicon (Moderator)

Title:	Summary of CSI enhancements for MTRP and FDD (Round 2)

Document for:	Discussion and Decision

# Introduction

# Summary of CSI enhancement for FDD

# Summary of CSI enhancement for MTRP

Proposal 18: For CSI measurement associated with a CSI-ReportingConfig for NCJT,

- Alt 1: UE is expected that two CMRs within the same CMR pair configured for NCJT measurement hypothesis are restricted within the same CDRX active time.

Alt 1: MediaTek, OPPO, Futurewei, Spreadtrum, Qualcomm, NTT DOCOMO, Samsung, Intel, vivo, Apple

Alt 2: ZTE, Nokia

Proposal 19:

Conclusion (updated):

For a NCJT measurement hypothesis, the powerControlOffset (“Pc”) ratio associated with a CMR within a CMR pair configured for the NCJT measurement hypothesis, is defined as 10log10(P_PDSCH/P_CSIRS)  dB

Where

- P_PDSCH is the energy of PDSCH ports, which is associated with the CMR, multiplexed on one subcarrier of one OFDM symbol

- P_CSIRS is the energy of all CSI-RS ports of the CMR multiplexed on one subcarrier of one OFDM symbol

Note that whether/how to capture above conclusion is up to the editor.

Conclusion only: Oppo, Futurewei, Spreadtrum, ZTE, DOCOMO, Samsung, Intel, Vivo, LGE, CMCC, NEC, Fraunhofer

Alt 1: MTK, QC, Nokia

Alt 2: MTK, QC

Proposal 22-2: For a CSI report associated with a Multi-TRP/panel NCJT measurement hypothesis configured by single CSI reporting setting, down-select one alternative from the following:

- Alt 1: One RI restriction is configured per CodebookConfig, whereas the RI restriction is applied to both Single-TRP and NCJT measurement hypotheses.

  - If rank restriction of X is configured, reported rank is X for a Single-TRP measurement hypothesis and sum of two reported ranks is X for a Multi-TRP measurement hypothesis.

- Alt 4: Two RI restrictions can be configured per CodebookConfig, whereas one RI restriction is applied to all Single-TRP measurement hypotheses, and another one is applied to all NCJT measurement hypotheses.

  - If rank restriction of (X, Y) is configured, reported rank is X for all single-TRP measurement hypotheses and reported rank (1 out of 4 possible rank combinations) is Y for all NCJT measurement hypotheses.

Alt 1: Oppo, ZTE, DOCOMO, CMCC, Motorola, Intel

Alt 4: MTK, Futurewei, Spreadtrum, Motorola, Qualcomm, Samsung, Intel, Vivo, NEC, Nokia

Proposal 23-1: For a CSI report associated with a Multi-TRP/panel NCJT measurement hypothesis configured by single CSI reporting setting,

- Alt 1: CBSR is supported and can be applied for both single-TRP and Multi-TRP measurement hypotheses.

  - FFS detailed CBSR signalling configured as legacy or with dedicated RRC signing for Multi-TRP

Alt 1: Motorola, ZTE (2), DoCoMo, Samsung, Intel, Vivo, CMCC, NEC, Fraunhofer

Alt 2: MTK, ZTE (1)