3GPP TSG RAN WG1 Meeting #106bis-e	  R1-2110558

E-meeting, October 11th – 19th, 2021

Agenda Item:	8.3.4

Source:	Moderator (Huawei)

Title:	Feature lead summary#3 on propagation delay compensation enhancements

Document for:	Discussion and Decision

Introduction

The revised IIoT / URLLC work item description for Rel-17 [1] has enhancements for time synchronization as one of its main objectives:

- Enhancements for support of time synchronization: - RAN impacts of SA2 work on uplink time synchronization for TSN, if any. [RAN2] - Propagation delay compensation enhancements (including mobility issues, if any). [RAN2, RAN1, RAN3, RAN4]

This document summarizes the key issues discussed under agenda item 8.3.4 based on the views in [3][4][5][6][7][8][9][10][11][12][13][14], and aims to discuss a set of issues in RAN1#106bis-e. The agreements in past meetings are captured in the Appendix.

Remaining issues on error components

There are several aspects which have impact on the timing accuracy between UE and gNB. In the previous meetings, we discussed the potential error components that would have impact on the time accuracy one by one, and achieved agreements on most of the error components as shown in the Appendix. One remaining issue is how to interpret the agreed value for BS transmit timing error.

How to interpret the agreed value for BS transmit timing error

In RAN1#103-e, we have agreed to use 65 ns to represent the BS transmit timing error for the control-to-control scenario.

Agreements:

- Take 65 ns as the assumption of transmit timing error for evaluation of the overall time synchronization error for control-to-control.

In RAN1#104-e meeting, Nokia (R1-2100730) propose to clarify if this should be interpreted as a maximum (<) or a relative (±) value.

Nokia R1-2100730 The agreed number of 65ns originates from the TAE requirement from TS 38.104, where the TAE represents the relative maximum timing error between any two antenna ports (i.e. <65ns). So, our interpretation of the agreed value is to use <65ns which translates to ±32.5ns per gNB antenna port. Proposal 1: The agreed 65ns value used to represent the BS frame transmission error should be interpreted as ±32.5ns to represent a single gNB antenna port frame transmission error for the control-to-control scenario.

In RAN1#104-e meeting and RAN1#104b-e, the following was proposed based on inputs from companies with the corresponding status as below:

- errorBS,DL,TXerrorBS, DL, TX (i.e. ±32.5 ns) is included in the equation for calculating the overall time synchronization for the control-to-control scenario.

  - Support: CATT, Nokia/NSB, Vivo, ZTE, Intel, LG, Samsung, ETRI, Huawei/HiSilicon, MTK, ZTE

    - TAE represents the relative maximum timing error between any two antenna ports (i.e. <65ns), thus the safest operation is to limit the error per antenna port to ±32.5ns as to guarantee that the error between two antenna ports satisfied <65ns

  - Support ±65ns: OPPO (fine to follow the majority view for using 32.25ns if only one or two companies have concern)

  - Strong concern: Ericsson, Qualcomm

    - 65ns defined for TAE is used to represent BS transmit timing error due to lack of better standardized values, since it is expected that transmit timing error is approximated as ±65ns.

    - ±65ns is a safer assumption because there is no guarantee for the correct DL Tx timing to stay at the middle of 65ns interval

    - The assumption for the previous agreements is ±65ns.

In RAN1#106bis-e meeting, Nokia (R1-2109161) discuss this issue and propose ±32.5 ns for both TA-based PDC and RTT-based PDC.

Nokia (R1-2109161) Proposal 2: For both TA and RTT based PDC error models errorBS,DL,TXerrorBS, DL, TX should be multiplies by ½, when used in the equation for calculating the overall time synchronization for the control-to-control scenario.

Feature lead: this issue was already discussed in previous meetings, and unfortunately consensus cannot be achieved. To avoid more debate in Nov meeting based on the inputs from RAN4, it would be good if we can achieve consensus on this issue in this Oct meeting. Since both sides seem believe they are right, and seems both have some valid points, the only way I can suggest as FL is to go with the majority view. Compared to multiplying 1/2 in the equation, it seems taking 32.5 ns is better considering multiplying 1/2 may result in confusing on the meaning for each error component.

- 2.1.1 First round discussion

Based on the inputs and analysis above, the following proposal are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposal before providing your views here.

Proposal 2.1.1-1: errorBS,DL,TXerrorBS, DL, TX (i.e. ±32.5 ns) is included in the equation for calculating the overall time synchronization for the control-to-control scenario.

| Company | View |
| --- | --- |
| OPPO | This errorBS,DL,TXerrorBS, DL, TX takes time reference at baseband, not antenna connector, correct? If yes, how could 32.5ns derived from TAE using reference at antenna connector apply? |
| Nokia, NSB | Support. Either the relative value interpretation is assumed, or equivalently, the error component takes the absolute value multiplied by ½. But we agree with FL to describe as relative value of ±32.5ns to avoid confusion. |
| Vivo | Support. |
| ZTE | We can support this proposal. |
| Intel | Support. We also understand different arguments from two camps. But we think this error component is not the most limiting, thus any decision is fine to us. The most important part is to have consistent numbers between TA-based and RTT-based schemes. |
| HW/HiSi | Support |
| Qualcomm | We can not support this proposal. |
| Ericsson | Do not support. Also we do not see why FL can suggest to use ‘majority view’ to change an existing agreement. This is not RAN1 convention. >> Feature lead As discussed several meetings, different companies have different understanding on the agreement, that’s why we have been trying to discuss this issue. |
| LG | Support |

Feature lead: Exactly the same situation as previous meetings. I don’t see any further email discussion will help any more. If there is chance, we can discuss in GTW directly.

Evaluation on the achievable time synchronization accuracy over Uu interface in Rel-16

In order to evaluate whether any enhancements needed in Rel-17 to meet the requirement, we need the check the performance that can be achieved by Rel-16 mechanisms first.

The potential error components that will have impact on the time synchronization accuracy over Uu interface are as below:

- BS transmit timing error (errorBS, DL, TX):

  - For control-to-control, it was agreed to use 65 ns for the evaluation.

  - For smart grid, it was agreed to use 65ns or 200ns for the evaluation.

- Downlink frame timing error (errorUE, DL, RX):

  - Based on the reply from RAN4, it is already included in Te

- UE Initial transmit timing error (Te) :

  - The value defined in Table 7.1.2-1 for initial transmit timing error (Te) in TS 38.133

- BS detecting error (errorBS, UL, RX) :

  - 100 ns

- Asymmetry between downlink and uplink channel (errorAsymmetry):

  - Not considered

- TA indicating error (errorTA_indication): Details as shown in section 3.2.3.3 in R1-2007068

  - 8*64*Tc/2

- TA adjustment accuracy (errorTA_adjustment):

  - Not considered

- Indication error

  - 5ns, it is already included in the network part budget.

Equation to calculate the overall time synchronization error over Uu interface

In RAN1#104b-e meeting, the following 4 basic steps were made for better understanding how to get the equation to calculate the overall time synchronization error over Uu interface. It is common understanding that step 1 to step 3 are applied to both TA-based PDC and RTT-based PDC.

Step 1: gNB sends the reference time clock TBS (i.e. referenceTimeInfo-r16) to UE, and the actual time clock at the UE side should be

TUE=TBS+errorBS, DL, TX+PDL+errorUE, DL, RX

- errorBS, DL, TX: BS transmit timing error for transmitting the RRC signaling containing the reference time clock

- errorUE, DL, RX: Downlink frame timing detection error for receiving the RRC signaling contacting the reference time clock

Step 2: When the UE receives referenceTimeInfo-r16, UE obtains TBS indicated by referenceTimeInfo-r16. After UE does the propagation delay compensation, the estimated time clock at the UE side is

TUE=TBS+PDL+errorPDL

- errorPDL:  DL propagation delay estimation error, e.g. errorPDL=TA2-PDL for TA-based PDC. Note that details for errorPDL is defined in step 4 below.

Step 3: The overall time synchronization error (i.e. the difference between the actual time clock in step 1 and the estimated time clock in step 2) is

errortotal, TAbased≤errorBS, DL, TX +errorUE, DL, RX +errorPDL

Step 4: Discuss and determine error component(s) for DL propagation delay estimation (i.e. errorPDL)

For TA-based PDC, the following working assumption were achieved in RAN1#104b-e:

Working assumption:

In RAN1#105-e meeting, RAN1 received the LS [15] from RAN4 to inform that downlink frame timing detection error is already included in UE transmit timing error (i.e. Te defined in section 7.1.2 in TS 38.133). Thus it is clear that errorUE, DL,RX+errorUE, UL,TX≤Te, so the two alternatives in above WA can be updated as below:

- Alt. 1:

errortotal, TAbased≤

errorBS, DL, TX+errorUE, DL,RX+errorBS, DL,TX+Te+errorBS, UL,RX+errorTAindication2

- Alt. 2:

errortotal, TAbased≤

[12*]errorBS, DL, TX+Te+errorBS, UL,RX+errorTAindication2

  - [Note: Alt.2 assumes that gNB can coordinate the time of TA procedure and the time of PD compensation, so that the DL frame timing error and BS transmit timing error for propagation delay estimation is correlated to (e.g. the same as) that for the transmission of RRC signaling carrying the reference time clock]

In RAN1#106bis-e meeting, some companies provide further views on the equations for TA-based PDC.

Nokia (R1-2109161) Model of timing error to be assumed RAN1 has received reply LS from RAN4 regarding whether it can be assumed that DL Rx is captured in Te or not. The reply from RAN4 is clear that DL Rx is captured in Te. That means that Option 1 should be considered in the equation alternatives discussed for TA-based PDC evaluation. However, the discussion in RAN4 gives the impression that this assumption is only applicable for the test condition where the UE is tested whether it complies to Te. That is, the reference measurement probe is at the gNB (or the UE) antenna connectors and measures the time difference between an input DL time to an output UL time, which means that no matter what channel the UE is affected to (i.e. how it detects DL frame timing), it must comply to Te. That also means that while we assume that DL Rx is captured in Te here in RAN1, this is slightly artificial as the UE in practice have to rely on DL RS detection and cannot accurately determine whether it complies to Te or not. The current working assumption for TA evaluation from RAN1#104e consists of two equation alternatives with brackets and options. From the RAN4 LS reply, it is clear that Option 1 should be considered which says that errorUE,UL,TX+errorUE,DL,RX≤Te. As discussed in relation to Proposal 3, the use of Te assumes that the UE has not received any updated NTA after DRX, but as RAN1 has agreed that the UE may have an NTA update after waking up from DRX, we have a conflict that could be addressed by considering two cases; one where the UE has an updated NTA, and one where the UE does not have an updated NTA. When the UE does not have an updated NTA, Te applies (i.e. errorUE,UL,TX+errorUE,DL,RX≤Te), but when it has an updated NTA, we should assume that Te does not apply anymore. In this case, the gNB has detected the UE timing offset, signaled a correction via NTA update (subject to the error of granularity) and the UE applies the NTA update. The UE should apply the updated NTA with the timing advance adjustment error Tadjust (i.e. errorUE,UL,TX+errorUE,DL,RX≤Tadjust). Considering the above discussion, we propose to use Alt. 2 with Option 1 (as per the reply LS from RAN4) to model the total error with TA based PD and to distinguish between the case where the UE has an updated NTA after DRX and where it does not. Proposal 4: Adopt the following total error model for TA based PDC (Alt. 2 with Option 1 considering the availability of NTA update): - errortotal,TAbased≤12 errorBS,DL,TX+Te+errorBS,UL,RX+errorTAindication2, if no updated NTA after DRX is assumed, or - errortotal,TAbased≤12errorBS,DL,TX+Tadjust+errorBS,UL,RX+errorTAindication2, if it is assumed that the UE receives an updated NTA after DRX.

Feature lead: For whether to apply TA adjustment error or not, please check the analysis for issue 4.2-4 in section 4.2. Regarding [12*], as discussed in section 4.3.5 for RTT-based PDC Alt.2, it is related to the discussion in section 2.1, for further discussion I would make the proposal with the removal of the whole “[12*]” to align the proposal in section 2.1 and section 4.3.5.

ZTE (R1-2108844) Proposal 2: Both Alt. 1 and Alt. 2 can be used for further study and Alt. 2 is preferred.

Feature lead: Based on the discussions in previous meetings, it is not clear to me how to do the down-selection for TA-based PDC. My original intention is to take both, since different alternatives here reflects different assumptions, i.e. independent error components and co-related error components. If in the end we found that only with the assumption of co-related error components it can meet the budget, maybe we can provide the corresponding information to other working groups also. However, based on the discussion in RAN1#106-e, it seems people prefer to do down-selection, then let’s try again to see if any chance. Between RTT-based PDC and TA-based PDC, we may need to align the assumption when do the comparison.

- 3.1.1 First round email discussion

Based on the inputs and analysis above, the following proposal are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposal before providing your views here.

Clarifications for Alt. 2

Proposal 3.1.1-1: For Alt.2 for evaluation of the overall time synchronization error for TA-based propagation delay compensation, it is revised as below:

- Alt. 2:

errortotal, TAbased≤

[12*]errorBS, DL, TX+Te+errorBS, UL,RX+errorTAindication2

  - [Note: Alt.2 assumes that gNB can coordinate the time of TA procedure and the time of PD compensation, so that the DL frame timing error and BS transmit timing error for propagation delay estimation is correlated to (e.g. the same as) that for the transmission of RRC signaling carrying the reference time clock]

  - ±32.5 ns is used for errorBS, DL, TX

| Company | View |
| --- | --- |
| Feature lead | @all Please align your views among section 2.1.1, 3.1.1 and 4.3.5. |
| OPPO | If we correctly understand FL’s reasoning of “Regarding [12*], .... it is related to the discussion in section 2.1”, FL seems to suggest two options: either use 1/2*errorBS,DL,TX for errorBS,DL,TX=65ns or use [1/2]*errorBS,DL,TX for errorBS,DL,TX=32.5ns. This is not our understanding. In our view, the coefficient of 1/2 for 1/2*errorBS,DL,TX in Alt-2 comes from the combination of corresponding error terms in both delay estimation step and delay compensation step, not depending on the value of errorBS,DL,TX. We think the formulation for Alt-2 is errortotal, TAbased≤ 12*errorBS, DL, TX+Te+errorBS, UL,RX+errorTAindication2 with errorBS,DL,TX being decided from Proposal 2.1.1-1. BTW, it is a bit confusing to us why RAN1 should spend too much time on this for Rel-16 (given section 3 is for Rel-16, as the section 3 title suggests). RAN1 already agreed the legacy PDC can handle smart grid scenario and no company believes the legacy PDC can meet Uu interface requirement for control-to-control scenario. To change the coefficient between 1 and 1/2 for errorBS,DL,TX does not seem to change above common understanding. It would be more time-saving and efficient to focus on formula for Rel-17, and then use the same assumption of coefficient (for errorBS,DL,TX) in Rel-16 formulation. |
| Nokia, NSB | Support. |
| vivo | We can accept FL’s proposal. |
| ZTE | For Alt. 2, co-related error components are assumed, including BS transmitting time error. So it should be 1/2*errorBS,DL,TX. This should also applies in the Alt.2 for RTT-based PDC. |
| Intel | Support, aligning with 2.1 |
| HW/HiSi | Support. |
| Ericsson | Do not support. First, we cannot accept changing errorBS, DL, TX to half the agreed value. Second, even if the condition in the note is satisfied, it cannot explain why Alt 2 does not have +errorUE, DL,RX term, which is the error to receive the RRC of referenceTimeInfo. See the illustration under section 3.1 Thirdly, we cannot agree to the condition in the note that coordinate the time of TA procedure and the time of PD compensation. In fact, it is up to UE when the UE performs PD compensation. It is unreasonable to impose this condition. Also, we agree with OPPO point that it’s a waste of time to discuss this further, since it does not change the conclusion that TA-based cannot satisfy control-to-control requirement. |
| LG | Support |

Feature lead: Based on the agreement from Wednesday GTW, Alt.1 is taken. Therefore, we don’t need to discuss the above proposal again.

Down-selection between Alt.1 and Alt.2

Question 3.1.1-1: Which option do you prefer for the following evaluation of TA-based PDC?

- Option 1:  Keep both Alt.1 and Alt.2 for further evaluation for TA-based PDC, and do comparison between RTT-based PDC and TA-based PDC under the same assumption

- Option 2:  Down-select to Alt.1 for TA-based PDC

- Option 3:  Down-select to Alt.2 for TA-based PDC

| Company | View |
| --- | --- |
| OPPO | We choose Option-2. Option-1 conflicts with agreement from RAN1 #106e for RTT based PDC, which says “Take the following two alternatives as the equation for evaluation of the overall time synchronization error for RTT-based propagation delay compensation. RAN1 to select one of the alternatives in RAN1#106bis-e”. For Option-3, given the delay compensation may likely depend on referenceTimeInfo message in SIB9 (which is common to all UEs) and TA procedure is UE-specific, it is unclear to us how the gNB could coordinate TA procedure and delay compensation to make the two “close-enough” in time for all concerned UEs. In addition, we see a potential need for RAN4 to discuss the relative requirements to secure such “close-enough” assumption in RAN1, which does not seem to be an easy job for RAN4. |
| Nokia, NSB | Option 3. We prefer option 3. However, we are open to aligning the assumption with the evaluation choice made for RTT-based, i.e. TA and RTT based methods should be evaluated under the same assumption. |
| vivo | We support option 3. If majority of companies support option 1, we can also live with it. |
| ZTE | We support option 3. In addition, the procedures are provided in our paper to achieve Alt.2 for TA-based PDC as shown below. - The network sends the reference time providing the transmitting time of the ending boundary of the frame N. - Then the UE receives the downlink signal in the frame N and performs the downlink synchronization to obtain the downlink frame timing based on the received signal. In this procedure, the BS transmitting time error and UE downlink detection error in step 1 are involved. - The UE further transmits the UL signal based on the obtained downlink frame timing. It means the BS transmitting time error and UE downlink detection error in step 1 affect the UL signal transmission. - Then the network sends a new TA after receiving the UL signal so that the UE obtains the new TA value. - The UE use the new TA value to determine the time when it receives the frame N. We can see the both of the BS transmitting time error and UE downlink detection error are consistent in the step 1 and step 2 since the new TA is obtained based on the procedure in step 1. Therefore, we think Alt. 2 can also be achieved and slightly prefer to use it for evaluation since it can achieve a higher accuracy. |
| Intel | Option 1 or Option 3. |
| HW/HiSi | Option 3 or Option 1 as second preference. |
| Qualcomm | Option 2 for simplification. |
| Ericsson | Option 2. Regarding Alternative 2, we don’t think it is correct even if the correlation condition (or, steps described) by ZTE is followed. Why Alt 2 dos not have +errorUE, DL,RX term, which is the error to receive the RRC of referenceTimeInfo? It should be there even if propagation delay compensation is perfect. Also, we cannot agree to the condition in the note that coordinate the time of TA procedure and the time of PD compensation. This is unrealistic assumption. |
| LG | Option 1 or Option 3 |

Feature lead: Based on the agreement from Wednesday GTW as below, Alt.1 is taken. Therefore, we don’t need to discuss this issue again.

Agreement

For evaluation of the overall time synchronization error for TA-based propagation delay compensation,

- Alt.1 for TA-based PDC

Overall time synchronization error over Uu interface in Rel-16

According to the LS from RAN2, the single Uu interface budget for control-to-control scenario and smart grid scenario are as shown below:

| Scenario | Single Uu interface Budget |
| --- | --- |
| Control-to-Control | ±145ns to ±275ns |
| Smart Grid | ±795ns to ±845ns |

In RAN1#104bis-e meeting, the following is agreed. Then it is clear that PDC based on existing Rel-15/Rel-16 TA procedure and associated granularity, with no enhancements in RAN1, is sufficient for smart grid, and RAN1 needs to further study and specify the feasible enhancement (if any with RAN1 spec impact) for propagation delay compensation for control-to-control scenario.

Agreements:

- Observation 1: Propagation delay compensation based on existing Rel-15/Rel-16 TA procedure and associated granularity, with no enhancements in RAN1, is sufficient for meeting the Uu interface synchronicity error budget in LS R2-2010837 for the smart grid scenario.

- Observation 2: RAN1 needs to further study and specify the feasible enhancement (if any with RAN1 spec impact) for propagation delay compensation for control-to-control scenario, in order to meet the synchronicity budget of Uu interface in LS R2-2010837.

Potential enhancements for propagation delay compensation

In RAN1#102-e meeting, the following option 1 and option 2 are agreed for further study in RAN1.

- Option 1: TA-based propagation delay

  - Option 1a: Propagation delay estimation based on legacy Timing advance (potentially with enhanced TA indication granularity).

  - Option 1b: Propagation delay estimation based on timing advanced enhanced for time synchronization (as 1a but with updated RAN4 requirements to TA adjustment error and Te)

  - Option 1c: Propagation delay estimation based on a new dedicated signaling with finer delay compensation granularity (Separated signaling from TA so that TA procedure is not affected)

- Option 2: RTT based delay compensation:

  - Propagation delay estimation based on an RAN managed Rx-Tx procedure intended for time synchronization (FFS to expand or separate procedure/signaling to positioning).

Common issues for enhancements for propagation delay compensation

There are some issues that are common for both RTT-based PDC and TA-based PDC.

Issue 4.1-1: whether any additional restriction needed if Alt.2 for evaluation of the overall time synchronization error for TA-based PDC or RTT-based PDC is taken.

As shown in the agreements achieved in previous meetings in the Appendix, Alt.2 assumes that gNB can coordinate the time of TA procedure and the time of PD compensation, so that the DL frame timing error and BS transmit timing error for propagation delay estimation is correlated to (e.g. the same as) that for the transmission of RRC signaling carrying the reference time clock. Nokia R1-2109161 and OPPO R1-2109097 provide some discussions.

Nokia (R1-2109161) Two alternatives are considered where the difference is whether it is assumed that the gNB can coordinate the PD estimation events and the available DL references available for SFN boundary estimation. The gNB may deliver referenceTimeInfo using SIB9 or via RRC. When referenceTimeInfo is delivered via SIB9 the SFN boundary is specified relative to the SI-Window boundary “immediately at or after the ending boundary of which the SIB9 is transmitted” whereas for the case of delivery over RRC “ending boundary of the system frame indicated by referenceSFN”. In both cases, the gNB can timely aid the UE with the accuracy of the SFN boundary estimation. Then when it comes to the PD part, each PD estimation procedure consists of both a UL and DL reference signal component and by placing the DL RS as close to the SFN boundary estimation as possible, the channel (assuming one-shot estimation) will be the same. Additionally, as the UE applies a DL time tracking used for UL timing determining with TA and can similarly be used for SFN boundary estimation the error caused by DL time tracking induced at the UE for both SFN and PD estimation with TA will be the same. Likewise, for Rx-Tx, the same DL signal can be used for estimating the SFN boundary and for Rx-Tx measurement. This is illustrated in Figure 1. Figure 1. Illustration of a coordinated SFN boundary estimation event and PD estimation event. Observation 2: The error caused by DL time tracking at the UE will be the same for both, SFN boundary estimation and PD estimation, as a) the gNB can coordinate the time of the TA procedure and the time of PD compensation and b) UE would apply the same DL time tracking for TA/Rx-Tx as for SFN boundary estimation.

OPPO (R1-2109161) In our view, it could be difficult in practice for gNB to always ensure the two DL transmissions to be close enough to share the same run-time DL-Rx timing error, given: - One DL transmission is associated with the PHY/MAC-layer procedure and another DL transmission is associated with RRC signaling/procedure. The feasibility of tight coordination between protocol layers is not clear at this time. - To intentionally make two specific types of DL transmissions “very close to each other” indeed logically bundles the two transmissions. Then if any of two transmissions (especially the latter one) fails and the corresponding re-transmission (not necessarily HARQ re-transmission) leaves the two transmissions “not so close to each other”, the whole bundled transmission may disqualify itself and therefore need to restart as a whole. - There is no standardized criterion to define and measure “close-to-each-other enough” to ensure the same run-time DL-Rx timing error. Even if it is feasible to implement the relation of “close enough in time” of the two DL transmission for a UE, according to the existing specification, the RRC IE of ReferenceTimeInfo can be transmitted in either SIB9 or UE dedicated RRC message. For Alt-2, it could be difficult for gNB to ensure for all UEs involving in PDC that the DL transmission in TA adjustment (which is in relation to UE-dedicated DL transmission of TAC) is “close enough in time” to the UE-common SIB9. So in order to keep the same run-time DL-Rx timing error as required by Alt2, gNB needs to control which UE can use ReferenceTimeInfo in SIB9 and which UE cannot (so as to use ReferenceTimeInfo in UE-dedicated RRC signalling only). Proposal-1: If TA-based PDC is supported with Alt-2 error modeling, there should be a configuration per UE for whether or not the UE can use ReferenceTimeInfo in SIB9 for Rel-17 PDC.

Feature lead: It seems different views on whether the assumption for Alt.2 is feasible or not in practice. Since this issue was discussed much in previous meetings, and we did have achieve agreements on the assumptions, it seems not good to debate again whether the assumption is feasible or not. Instead, we can discuss whether/what additional restriction/enhancement needed if Alt.2 is taken, e.g. whether the reference time clock ReferenceTimeInfo can be transmitted in SIB9 or if transmitted whether any additional configuration needed to enable or disable the usage of the reference time clock transmitted in SIB9 for a UE. More views are needed from companies.

- 4.1.1 First round discussion for issue 4.1-1

Based on the inputs and analysis above for issue 4.1-1, the following proposals are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals before providing your views here.

Question 4.1.1-1: If Alt.2 for evaluation of the overall time synchronization error for TA-based PDC or RTT-based PDC is taken, which option should we take? Please provide your reasons also.

- Option 1: The reference time clock ReferenceTimeInfo should not be transmitted in SIB9, i.e. only transmitted in UE dedicated RRC signaling.

- Option 2: The reference time clock ReferenceTimeInfo can be transmitted in either SIB9 or UE dedicated RRC signaling,

  - Option 2-1: Introduce a UE specific configuration to indicate whether to apply the ReferenceTimeInfo transmitted in SIB9 or not

  - Option 2-2: UE can apply the reference time clock transmitted in SIB9 if it is received, and the reference time clock transmitted in UE dedicated RRC signaling can override the one transmitted in SIB9

| Company | View |
| --- | --- |
| Feature lead | The question is set based on views expressed by OPPO, more details can be seen in the above. For Alt-2, it could be difficult for gNB to ensure for all UEs involving in PDC that the DL transmission in TA adjustment (which is in relation to UE-dedicated DL transmission of TAC) is “close enough in time” to the UE-common SIB9 From feature lead perspective, one potential case to result in the issue as highlight in yellow above is that there might be no sufficient UE dedicated resource for all UEs to transmit the channel/signal needed for propagation delay compensation in order to ensure co-related time error component, e.g. to transmit the TA indication signaling for TA based PDC or transmit Rx-Tx time difference for RTT-based PDC, in which case the reference time clock transmitted in SIB9 might not be suitable. Option 2-2 above is to say by default the one transmitted in SIB9 can be applied, and if it is not suitable for some certain UEs then gNB can transmit new reference time clock information shortly in UE dedicated RRC signaling to these UEs, and these UEs can drop the one transmitted in SIB9 and use the one transmitted in UE dediated RRC signaling. |
| OPPO | We choose Option 2-1. Option 2-2 could introduce more trouble for gNB. Given SIB-9 could be (semi-)periodic, Option 2-2 actually says the PDC associated with SIB-9 can also refresh/replace the PDC outcome associated with ReferenceTimeInfo in UE-dedicated RRC signaling. Then the remedy is that the gNB should send the UE-dedicated RRC signaling “immediately/shortly after” the (semi-)periodic instance of SIB9 for all UEs that need such correction. In other words, for each involved UE, such “close-enough” timing relation is required either in PHY layer or in higher layer, but it could be more difficult to implement such in higher layer. For Option-1, does it go too far with too much signaling overhead? For RTT-based PDC, the Option-1 requires a UE-dedicated RTT indication for PD estimation followed by a UE-dedicated PD compensation for all UEs that needs clock synchronization. |
| Nokia, NSB | Option 2-2. No additional restriction needed. The origin of this Alt. 2 comes from the segmentation of the error components into an ReferenceTimeInfo part and a PD estimation part. In practice, the UE only needs to track one DL frame timing and may use this to determine its UL timing (TA) and also SFN boundaries where ReferenceTimeInfo applies. It will be up to UE implementation to also utilize a PRS / CSI-RS signal to further enhance its DL frame timing tracking which in turn can also enhance its SFN estimation and UL timing. Lets remember that it is most likely only here has been assumed the UE would do a one-shot PD estimation and SFN boundary estimation, while a real UE would apply time tracking filters (known in state of art), so lets be careful not to introduce constraints based on assumptions that might not be rooted in the reality. As we mention in our contribution highlighted above, as the UE applies a DL time tracking used for determining the UL timing with TA, the same DL time tracking can be used for SFN boundary determination. In our view, the PHY layer is the one able to determine where is a frame boundary, as well as what is the correct time to transmit in UL relative to DL frame, so we don’t see cross layer issue here. Similarly, for Rx-Tx, the UE needs to determine the received timing of DL subframe, and the same measurement can be used for estimating the SFN boundary. Therefore, the error caused by DL time tracking induced at the UE for both SFN boundary and PD estimation with TA will be the same (and with RTT as well). Therefore, we think that no additional restriction is needed, i.e. UE can apply the reference time clock transmitted in SIB9 or the reference time clock transmitted in UE dedicated RRC signalling. That should be up to UE implementation, without any restriction from specs. |
| vivo | We prefer to discuss this issue in RAN 2. |
| ZTE | From our view, as long as the UE gets the correct time information, there is no need to update the time clock during a short time. That is to say, even though the time information carried in SIB9 is transmitted periodically, it does not mean that the UE should receives SIB9 and perform PDC every time when SIB9 is transmitted. So the network does not need to transmit TA command to ALL the UE at the same time. It just transmits TA indication to a part of UEs at a time and it is up to the network implementation. Therefore, we don’t think it is an issue. We prefer option 2 |
| Intel | Option 2-2, since we don’t think it is reasonable to preclude anything in terms of gNB behavior. Reasonable implementation can take into account the error components when deciding which signaling of the reference timing info to choose. |
| HW/HiSi | No matter what alternative eventually will be selected, we prefer that SIB9 is not precluded to signal the reference time, because it is already supported in Rel-16 and can save signalling overhead. And we think this can be handled by the gNB as in option 2-2 which is preferred by us. |
| Ericsson | Option 2-2. Also we don’t see the need to make explicit agreement about this. In Rel-16 38.331, referenceTimeInfo-r16 is already transmitted in both IEs: SIB9 and DLInformationTransfer-v1610-IEs. |
| LG | We prefer Option 2-2. In general, we don’t think it is necessary to define limitation of gNB’s capabilities in here. |

Feature lead: Based on the agreement from Wednesday GTW as below, Alt.1 is taken. Therefore, we don’t need to discuss this issue again.

Issue 4.1-2: whether errorBS,DL,TX should be included in PD estimation errors?

In RAN1#106bis-e meeting, Nokia and ZTE discuss this issue below.

Nokia (R1-2109161) Similarly, it was discussed if the DL transmission error (errorBS,DL,TX) is visible in both PD estimate and in the SFN boundary estimation. The DL transmission time error is an error caused by a mismatch between the gNB-DU and the gNB-RU clocks – not necessarily due to the gNB-RU having a poor clock, but simply due to the physical distance between the two (and the bandwidth of the interfaces connecting them) which limits how accurately they can be synchronized. When it comes to how this clock mismatch impacts our PD estimation procedure and the total error, it is clear that this mismatch is directly affecting the SFN boundary estimation process, as the mismatch will shift the SFN time transmitted at the air interface relative to the SFN time at the gNB-DU. So definitely errorBS,DL,TX should be captured in the SFN estimation part of the accuracy model. However, the time synchronization error between the gNB-DU and gNB-RU will only impact the PD estimation procedure, if either: - the gNB-DU and gNB-RU splits who captures the timestamps from DL Tx and UL Rx, - if the same unit captures both DL Tx and UL Rx timestamps, but the clock of the gNB-RU is shifted between a DL and UL timestamp occasions used in the PD estimation and RTI delivery. As per our understanding, for both TA and Rx-Tx based PD estimation, it is the gNB-RU clock that affects the air interface timing (i.e. SFN boundary) and it is the gNB-RU capturing the UL detection time. As the time between a DL and UL transmission for both Rx-Tx and TA can be considered small (e.g. <10ms) and that the gNB-RU clock remains stable over this period of time, there is no need to capture errorBS,DL,TXin the PD estimation as well. Observation 3: errorBS,DL,TX should only be accounted for in the SFN boundary estimation related errors and not in the PD estimation errors.

ZTE (R1-2108844) For the DL propagation delay estimation error, whether should be included in the equation for TA based propagation delay compensation is FFS. Some companies believed that it should not be included since it reflects the error between the air interface time and the timestamp provided by the RRC signaling and the error should be accounted for in the RRC related error sources (i.e. step 1). In our understanding, the timestamp is from the time clock of the network. The UE transmits the UL signal based on the downlink timing of the DL signal transmitted by the network. When the network receives the UL signal from the UE, the TA is determined based on the time clock of the network but not the air interface time of the DL signal. Therefore, the errorBS,DL,TX should be included in the DL propagation delay estimation error. Proposal 1: should be included in the DL propagation delay estimation error.

Feature lead: In RAN1#104bis-e, we have already agreed a WA for two alternatives about the overall time synchronization error for TA based propagation delay compensation. In RAN1#106-e, we have agreed the two alternatives for RTT-based PDC. It is not clear whether any further discussion can bring us anywhere based on the previous discussion. Considering Alt.1 for both TA-based PDC and RTT-based PDC is kind of assuming independent error components, would be fine to leave it as what agreed for Alt.1 for now, while Alt.2 assumes co-related error components thus BS transmit timing error doesn't need to be considered again for propagation delay error as agreed in the previous agreement. Therefore, I would like to deprioritize this issue for now. However, if you have any thinking or suggestion on this issue, you can provide it in the following table also.

| Company | View |
| --- | --- |
| OPPO | First, RAN1 #106e agreed to make a decision in RAN1 #106b-e between Alt-1 and Alt-2 for RTT-based PDC. So it will be conflicting to this agreement to have both alternatives for both TA-based PDC and RTT-based PDC at the end of this meeting. In our view, the interpretation of errorBS,DL,TX in PD estimation is different between TA-based PDC and RTT-based PDC. - In TA-based PDC, the gNB intends to align DL-Tx and UL-Rx at the baseband where DFT is performed, instead of antenna connector. So errorBS,DL,TX needs to refer to the error term at the reference point other than antenna connector. errorBS,DL,TX=65ns is what RAN1 agreed in this case. - In RTT-based PDC, quite some companies assume the RTT measurement at gNB takes reference point at antenna connector. Then errorBS,DL,TX+errorBS,UL,RX=errorBS, RTT with all numbers taking reference point at antenna connector. RAN4 already gives some requirement forerrorBS, RTT for SRS-based RTT measurement. Once errorBS, RTT is given, it does not matter to the overall evaluation result whether errorBS,DL,TX=0 or not. In fact, it is a bit strange to say “Alt.2 assumes co-related error components thus BS transmit timing error doesn't need to be considered again for propagation delay error” because the following two conditions cannot be assumed at the same time: - A (e.g.,errorBS,DL,TX for PD estimation) and B (e.g., errorBS,DL,TX for PD compensation) take different measurement reference points. - A and B can be highly correlated, even with one fully cancelling another. So we suggest not to put this issue as “common issue” in section 4.1, if errorBS,DL,TX in PD estimation step may take different reference points anyway between TA-based PDC and RTT-based PDC . |
| Nokia, NSB | We see the point of FL for the sake of progress, but we think at least that similar assumptions should be made for RTT and TA regarding this. Therefore, current alternative Alt.2 for RTT should not consider BS TX timing error in the PD error part. |
| Intel | Agree with Nokia |

TA-based propagation delay compensation

The following agreement was achieved in RAN1#106-e:

Agreement

Send LS to RAN4 to ask for feedback on the following questions:

- Question 1: Is it feasible to support a smaller value than the current Te for the use of propagation delay compensation, assuming the existing conditions in TS 38.133 for Te requirement? If not, is it feasible under new conditions (e.g. using TRS instead of SSB)? If the answer is yes, please also provide feedback on how much it can be reduced at most.

- Question 2: Is it feasible to introduce enhanced TA command indication granularity? If the answer is yes, please also provide feedback on how much it can be reduced at most (e.g. reduced to (1/16)* (16*64*Tc/2)) similar as the granularity for Rel-16 IAB based on the Timing Delta MAC CE and related condition.

- Note 1: The alternatives in the working assumption achieved in RAN1#104bis-e together with the examples in Table 4.2-2 will be included in the LS to give some background for RAN4

- Note 2: The agreement “both SCS 15 kHz and 30 kHz are assumed for both control-to-control and smart grid for evaluation of the time synchronization” achieved in RAN1#102-e will be included in the LS for RAN4 information also.

- Note 3: Inform RAN4 that the enhancements on Te and TA command indication granularity for propagation delay compensation may or may not have impact on normal TA related procedure, depending on which candidate option for TA-based PDC is adopted. Note that this is just for RAN4 information.

- Note 4: Whether RAN1 will introduce specification enhancements is still undetermined.

In the RAN#93-e meeting, the following was concluded.

conclusion: For the objective on enhancements for support of time synchronization, RAN should provide the following guidance: - RAN4 to provide reply LS to RAN1 (e.g. in response to R1-2108635 on TA-based PDC and a potential RAN1 LS on RTT-based PDC) before the start of RAN1#107-e (Nov 11th)

Based on contributions, the following issue can be discussed.

Issue 4.2-1: Whether any additional work needed in RAN1 to support enhanced Te?

OPPO (R1-2109097) Because the RAN1 analysis assumes no further enhancements on gNB-side parameters errorBS, DL, TX and errorBS, UL, RX, Alt.2-2 may result in two solution directions: - If TA granularity is not to be changed, Te≤275×2-65-100-260=125ns<4×64Tc. It is then expected to introduce a lot of discussion in RAN4 to specify a Te that is smaller than 1/3 of existing value and even smaller than existing RAN4 parameters Tp, Tq and TA adjustment accuracy (defined in 7.3.2.2 in 38.133). The work in RAN1 for DL-Rx synchronization enhancement is also required. A back-and-forth coordination between RAN1 and RAN4 is likely necessary to make this work done, which however does not seem to be feasible based on RAN1/RAN4 timeline for Rel-17.

Feature lead: It is expected that enhanced Te would be based on TRS instead of SSB. OPPO express that the work in RAN1 for DL-Rx synchronization enhancements is also required if enhanced Te is supported. Since we don’t have much time left, it would be good to identify whether any potential additional RAN1 work needed as early as possible. Therefore, the following question is set for further discussion.

- 4.2.1 First round discussion for issue 4.2-1

Based on the inputs and analysis above for issue 4.2-1, the following proposals are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals/questions before providing your views here.

Question 4.2.1-1: For enhanced TA-based PDC if supported, any additional RAN1 work needed to support enhanced Te based on TRS, e.g. configuration of TRS/SRS for TA-based PDC?

| Company | View |
| --- | --- |
| Feature lead | My original understanding is that the main work for enhanced Te is in RAN4. From RAN1 perspective, gNB has to ensure that TRS and SRS with sufficient bandwidth is available for propagation delay estimation, which can be achieved by gNB implementation. Companies are encouraged to check and provide views whether any additional work needed in RAN1. @ OPPO Please explain more on the potential RAN1 work regarding “The work in RAN1 for DL-Rx synchronization enhancements is also required”. |
| OPPO | RAN1 only agreed TRS/PRS based PD estimation for RTT-based PDC. Nothing is in place to enhance DL-RX timing error for TA-based PDC. We see at least two issues that may need RAN1 discussion: - The RTT interval measured at UE side for PDC purpose should be based on enhanced solution, e.g., based on TRS/PRS etc, to echo reduced Te. This should be a new discussion from scratch. Meanwhile, for TA-based PDC, the measurement errors on UE side is assumed to take reference at baseband (where TA applies), so RAN4 needs to give performance requirement taking this time reference (corresponding to PRS/TRS), instead of the ones for RTT-based PDC. >> Feature lead Not sure if I get your point correctly. In my understanding, if RAN4 replied that enhanced Te is possible, then RAN4 will need to define the enhanced Te in the spec also if TA-based PDC is agreed in RAN1. But I think this work is for RAN4, not for RAN1. From RAN1 perspective, one thing we need to discuss is the configuration of TRS/SRS for TA-based PDC, similar as RTT-based PDC. With this explicit configuration, then UE would use this TRS to determine the DL synchronization, and also transmit the corresponding SRS for gNB to determine the BS detection error also I think. - The current RAN4 spec links the most reliable TA interval measurement at UE side (where Te applies) to the last TA command received by the UE (38.133 says “ (in Tc units) for other channels is the difference between UE transmission timing and the downlink timing immediately after when the last timing advance in clause 7.3 was applied” ). Then, RAN1 needs to discuss the relationship between TA command received on the DL and TRS/PRS that is also received on the DL, and this RAN1 discussion may need RAN4 input. >> Feature lead I assume gNB should transmit these channel/signal in appropriate time to avoid any serious mis-match, i.e. if there is any of this kind of problem it should be avoided by implementation. [OPPO]: People needs to keep in mind that - TRS/PRS can be UE-common and TA command is UE-specific. - TRS/PRS are pre-configured while TA command is on-demand. |
| Nokia, NSB | If RAN4 concludes that Te can be reduced, and that the possible reduction is sufficient for meeting the target based on RAN1 evaluation, and to achieve the reduction the UE will need to track DL timing and/or transmit in UL using higher bandwidth, then more work may be needed in RAN1. In our view RAN1 will need at least to study whether the enhanced Te should be active for any UL transmission (which may incur high UE processing power, even when it does not need a PD estimation), or how to determine when the enhanced Te should apply. >> Feature lead I agree this issue should be discussed. Originally I was assuming that for enhanced TA-PDC, similar as RTT-based PDC, gNB also need to configure TRS and/or SRS configuration for TA-based PDC, then enhanced Te could be applied only to the configured TRS/SRS. Maybe we can hear view from companies. |
| vivo | We also think the main work for enhanced Te is in RAN4. The bandwidths for tracking timing based on the TRS and SRS need RAN4 input. |
| ZTE | We don’t see the need of RAN1 spec work on this. |
| Intel | We let proponents of enhanced TA-based procedure to provide the details. In general, if TA needs to be measured on new/additional signals, we fail to see why RTT procedure is not acceptable with similar spec impact. |
| HW/HiSi | Initial thinking is that TRS design can be re-used which should not lead to any additional efforts in RAN1. |
| Ericsson | No need to discuss. Wait for RAN4 response. >> Feature lead The reason to discuss is we will only have one RAN1 meeting left, so better to prepare well to finish all RAN1 functions in next RAN1 meeting. Even we still don't have agreement to support TA-based PDC yet, we can discuss the aspects that RAN1 needs to discuss if supported. Similar manner is done for RTT-based PDC also. |
| LG | Our understanding is to re-use current framework with reduced Te. Main work would be conducted in RAN4. |

- 4.2.1a Second round discussion for issue 4.2-1

Based on the inputs and analysis above for issue 4.2-1 in section 4.2.1, the following proposal are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals/questions before providing your views here.

Proposal 4.2.1a-1: If enhanced TA-based PDC with reduced Te based on TRS is supported in Rel-17, a single pair of CSI-RS for tracking (TRS) and SRS configuration is configured for TA-based PDC.

| Company | View |
| --- | --- |
| OPPO | Not support. The proposal results in a new TA adjustment procedure for UE, which mandates UE to use TRS to determine the DL-Rx timing. It not only introduces an UE implementation complexity issue, but also raises a potential debate whether the UE should maintain one or two DL-Rx timing and then associated TA loops. >> Feature lead Thanks. That is the intention actually, since UE needs to determine the DL-Rx timing based on TRS for PDC, then enhanced Te can be achieved. For UEs supporting TA-based PDC with enhanced PDC, UE has to implement this I think, also whether to support is UE capability. We do not think any PDC-related enhancement can deserve an introduction of new TA behavior on UE side. Quite a few TA-related logics would be reviewed in RAN1/2/4 in that case. |
| Ericsson | Do not support Even if a new pair of TRS and SRS configuration is introduced, it should not reuse existing TA procedure, and a separate set of protocol/signaling/requirements are needed. The existing TA is for aligning UL slot with DL slot at gNB side, and this functionality should not be affected by PDC. Thus it’s important to clarify the above understanding first. >> Feature lead Thanks. Whether to have a separate set of protocol/signaling/requirements is to address in issue 4.2-2. Here, the intention is to say if enhanced Te based TRS is supported, for the purpose of PDC, better to make it clearer on the TRS/SRS used for PDC, to ensure achieve the enhanced TRS. If there is no this kind of configuration, UE may do the DL synchronization based on RS also, in which way enhanced Te is not ensured. Using a single pair of TRS and SRS will make “enhanced TA-based PDC” not distinguishable from RTT based, and RAN1 can simply agree that “a PDC method” is adopted, without calling it TA-based or RTT-based. We do not support that a single pair of TRS and SRS is configured, then two sets of protocol/signaling/requirements are specified for them. >> Feature lead The configuration on the RS may be the same, but the detailed mechanisms are still different I think, e.g. for RTT-based PDC we need to introduce the Rx-Tx time difference measurement, but for TA-based PDC we need to introduce TA indicating signaling. |
| vivo | I’m not sure whether the enhanced TA-based PDC with reduced Te requires the additional RS configuration or not. If the answer from RAN4 is Yes, the new RS configuration or procedure to obtain the enhanced TA may be needed for TA-based method. I think we should discuss the issue based on guidance from RAN4. >> Feature lead Thanks. It is expected that enhanced Te needs to rely on TRS, and we start to discuss in this meeting is because we may not have sufficient time for next meeting, and trying to address issues as much as possible and prepare well to get things done no matter what PDC enhancements is supported. I added some additional description to make it clearer. |
| Nokia, NSB | Do not support For enhanced TA-based PDC, it is not clear to us why there is any need of explicit single pairing configuration with SRS. If enhanced TA-based PDC is supported, gNB shall configure a TRS which should be tracked by a UE that supports enhanced Te, and after detecting the TRS the UE transmits at least one uplink signal complying with the enhanced Te requirement (which can be a gNB scheduled/configured SRS, that is up to gNB implementation). FFS whether the UE can get back transmitting complying with existing Te after the transmission of at least one UL signal after TRS, after transmission of a configured SRS, or after receiving the TA command (with possibly lower granularity), or other. An alternative proposal is: Proposal 4.2.1a-1: If enhanced TA-based PDC with reduced Te is supported in Rel-17, a single pair of CSI-RS for tracking (TRS) and SRS configuration is configured for enhanced TA-based PDC. - FFS whether the UE can get back transmitting complying with existing Te after the transmission of at least one UL signal after TRS, after transmission of a configured SRS, or after receiving the TA command (with possibly lower granularity), or other. >> Feature lead The BS detection error might be better if based on TRS instead of any other uplink transmission. I include single pair also, because companies think the inconsistent measurement is applied to TA-based PDC also. In addition, if it is pair, it can ensure enhanced detection error at both gNB and UE side. But anyway let’s hear more views from companies, then we could see whether the SRS part should be removed for now. |
| Samsung | We think TRS may help to improve the DL accuracy, SRS in connected mode can lead to a more accurate transmission time. In all, it can lead to a more accurate Te. Of course, this will depends on RAN 4’s feedback. Regarding on Nokia’s comment, not sure if I get your concern correctly, then why RTT based has no such issue for receiving TRS and transmitting SRS? |
| ZTE | We support this proposal. There is no doubt that TRS and SRS can be used for the network to estimate the TA according to the UL signal transmitted by the UE. In Rel-15 and Rel-16, the UE can use any downlink signal for DL synchronization for TA procedure, e.g., SSB and TRS, and so does the network. It is obvious that TRS can be configured with larger bandwidth than SSB. So we don’t think reusing the TRS and SRS may lead to UE implementation issue on TA procedure or something else. In general, the downlink frame timing is always changing as long as the UE is moving. When the UE detects a new DL RS for DL synchronization, the downlink frame timing is updated based on the new DL RS. |
| Hw/HiSi | We think that both RTT and TA based PDC should be based on the same assumptions for evaluation. If RTT based PDC uses TRS/SRS then this should also be allowed for TA-based PDC. Therefore we agree with the proposal. The impact on the existing TA procedure is already addressed in Proposal 4.2.2a, we prefer to keep this discussion separated. |
| Qualcomm | Do not support. We share the same views as Oppo and Ericsson. >> Feature lead Please see my reply to OPPO and Ericsson. |

Summary of the status for proposal 4.2.1a-1 in second round

| Support | Samsung, ZTE, Huawei, HiSilicon |
| --- | --- |
| Not support | OPPO, Ericsson, Qualcomm, Nokia, NSB |

FL recommendation: Still different views, let’s further discuss this issue.

- 4.2.1b Third round discussion for issue 4.2-1

Based on the inputs and analysis above for issue 4.2-1 in section 4.2.1a, the following proposal are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals/questions before providing your views here.

Proposal 4.2.1a-1: If enhanced TA-based PDC with reduced Te based on TRS is supported in Rel-17, a single pair of CSI-RS for tracking (TRS) and SRS configuration is configured for TA-based PDC.

| Company | View |
| --- | --- |
| Feature lead | @ OPPO @ Ericsson @ Qualcomm @ Nokia Please check my replies to your comments in section 4.2.1a, and see if you have further comments. |
| Nokia, NSB | Do not support. We don't see the need to restrict such pairing configuration with an SRS. It can be up to gNB implementation to schedule/configure SRS for UE to transmit after CSI-RS for tracking, e.g. if gNB wants to enhance detection error. (and also answering to Samsung question "why RTT based has no such issue for receiving TRS and transmitting SRS?"…) The reason is that, for RTT, what is indicated is the Rx-Tx time difference between the timing of DL and UL containing SRS associated to UE. The UE then needs to combine the indicated measurement with its Rx-Tx measurement associated with the same DL and UL events to compute PD. While TA is a continuous process (which is kicked off from first transmission of UE without TA, i.e. PRACH), then the gNB sends TA updates and the PD value can be determined in the UE directly based on current TA/2 (with some error, of course). So, in TA-based, there is no need of SRS association to UE, and no inconsistent measurements scenario, since the UE is not doing paired measurement with gNB (it is just gNB delivering TA based on UL transmission timing). Note: we don’t think this should be taken as an advantage for TA-based, we are just saying we don’t see the need for introducing such configuration restriction. >> Feature lead I think the point “It can be up to gNB implementation to schedule/configure SRS for UE to transmit after CSI-RS for tracking, e.g. if gNB wants to enhance detection error” seems feasible. But at least for DL, I think we still need to indicate UE to use TRS to measure the DL frame timing error, in order to achieve enhanced Te. Maybe let’s further check the views from companies on configuration of TRS. |
| OPPO | No. Our concern in round-2 remains. |
| Ericsson | Do not support. If a pair of TRS and SRS is configured, we do not see the justification to have two sets of measurements and reporting, one enhanced TA-based, one RTT-based. Also: to keep some resemblance to TA procedure, it makes more sense to use a PRACH of wider bandwidth instead of SRS. For example, the long sequence of length LRA = {517, 1151} developed for unlicensed band can be reused. >> Feature lead For TA-based PDC, there is no need to do the reporting, at UE side only UE needs to do the DL-Rx timing based on the TRS to achieve enhanced Te. In addition, as replied to Nokia, for uplink it might be feasible to only configure DL but leave UL to implementation. |
| Qualcomm | We cannot support and share both Ericsson and Nokia’s views. >> Feature lead Please see the replies to Nokia and Ericsson above. |
| vivo | No. We share the similar views with other companies. gNB can estimate and adjust TA based on any UL signaling or channel, e.g. PRACH/SRS/DMRS, etc. It is unnecessary with the restriction on RS pair like RTT-based method, i.e. TRS and SRS. >> Feature lead Please see the replies to Nokia and Ericsson above. |
| ZTE | We support this proposal since TRS has already used for finer synchronization in both time domain and frequency domain as QCL source. It it unfair that RS with larger bandwidth cannot be used for TA-based PDC while it has been used for RTT-based PDC. >> Feature lead Please see the replies to Nokia and Ericsson above. |
| HW/HiSi | Still support based on our views in round 2. |
| LG | No need to support We share similar views to other companies’. It would be fair to assume TRS and SRS for evaluation purpose in order compare two schemes, however, it is not necessary to restrict DL/UL signaling for TA acquisition. >> Feature lead Please see the replies to Nokia and Ericsson above. |

Feature lead: Based on the inputs above, it might be feasible to leave UL to implementation as I replied to Nokia above. For DL, it looks to me the configuration of TRS for DL detection is still needed to achieve enhanced Te. Companies are encouraged to share the view again.

- 4.2.1c 4th round discussion for issue 4.2-1

Based on the inputs and analysis above for issue 4.2-1 in section 4.2.1b, the following proposal are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals/questions before providing your views here.

Revised Proposal 4.2.1a-1: If enhanced TA-based PDC with reduced Te based on TRS is supported in Rel-17, one CSI-RS for tracking (TRS) configuration is configured for enhanced TA-based PDC.

- FFS whether/how to configure SRS for enhanced TA-based PDC

| Company | View |
| --- | --- |
| Nokia, NSB | Support. |
| HW/HiSi | Support |
| OPPO | If there is no pairing between TRS and SRS at this point, we think this TRS can come from any of the existing TRS configuration instead of one that is newly defined for enhanced PDC purpose. Then the proposal does not seem to be needed. |
| Ericsson | We are fine with main body of revised proposal. For FFS point: suggest to remove or change to: • FFS whether/how to configure UL signal for enhanced TA-based PDC |

Issue 4.2-2: Whether the normal TA procedure is impacted or not by the new procedure introduced for enhanced TA-based PDC?

In previous meetings, there was some discussion whether to the normal TA related procedure will be impacted by the potential new procedure for TA-based PDC, but no formal discussion.

OPPO (R1-2109097) - If TA granularity is to be reduced, a fair assumption is to cut TA granularity by half. Then Te≤275×2-65-100-130=255ns<8×64Tc. This means both Te and TA granularity should be reduced. It should be noted that the change of TA command granularity may have impacts to TA command bit width (to maintain the same value range) as well as the applicability to TAG, which may involve specification modifications in RAN1/RAN2/RAN4.

Huawei (R1-2109743) Assuming TA-based PDC is used, if the TA-procedure is impacted, then it means that the UE can also adjust its uplink timing based on the new TA value which is intended for PDC. From uplink transmission performance perspective, it is beneficial since the TA value for PDC is more accurate. However based on current RAN4 spec, the UE can adjust the uplink timing with a relative accuracy to the signaled timing advance value. If the UE can also adjust uplink timing according to the new TA value e.g. Tc, then it is not clear whether the timing advance adjustment accuracy defined in RAN4 also needs to be satisfied or not. If the TA-procedure is not impacted, then the uplink timing adjustment is the same as Rel-16 which is simple. And another option maybe to leave it to the UE’s implementation since anyway the PDC will be compensated no matter TA-procedure is impacted or not. Observation 5: For TA-based propagation delay compensation, it seems feasible to leave it to UE implementation about whether normal TA related procedure is impacted or not by the new procedure designed for TA-based PDC if supported.

Feature lead:

It seems good to have some discussion on this issue, since it may have impact on RAN2 design.

- 4.2.2 First round discussion for issue 4.2-2

Based on the inputs and analysis above for issue 4.2-2, the following proposals are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals/questions before providing your views here.

Question 4.2.2-1: For enhanced TA-based PDC if supported, which option do you prefer? Please provide your reasons. If you have a different option, please provide it here also.

- Option 1: The enhanced TA command indication granularity introduced for enhanced PDC is only applied for PDC, i.e. normal TA procedure will still follow the existing TA command indication granularity.

- Option 2: The enhanced TA command indication granularity introduced for enhanced PDC is also applied to normal TA procedure, if UE reports the capability to support enhanced TA-based PDC

  - FFS whether TA adjustment error need to be enhanced or not

  - FFS the detailed RAN1 specification impact

| Company | View |
| --- | --- |
| OPPO | Not sure how this Q&A can help progress in RAN1, if the applicability of enhanced TA command granularity relates to RAN4 judgment of “whether the timing advance adjustment accuracy defined in RAN4 also needs to be satisfied or not”, as Huawei mentioned. Further, our concern on TA command bit width still applies to both options. For the two options themselves, we may need more information for decision, such as whether the UE needs to be ready at any time to handle two different TA command granularities (and therefore two associated TA procedures in parallel) or the UE is ensured to just handle one granularity with one TA procedure. >> Feature lead 1. The question is also to address your concern on TA command bit width. For example, if we go option 1, that means we don’t need to touch the normal TA procedure, including the normal TA command bit width. Instead, a new MAC CE will be used to indicate the TA with the enhanced TA granularity for PDC purpose. 2. If we go to option 1, the additional thing for UE to do is to receive the TA for PDC with enhanced granularity, and UE doesn’t need to operate two TA procedure in parallel in most of the time, since UE only needs to operate the one for PDC when needed, especially if we can use the new MAC CE to directly indicate the absolute TA for PDC. If we go to option 2, I think also only one TA procedure. The difference for option 2 is that we may still need RAN4 to study whether to enhance TA adjustment accuracy. |
| Nokia, NSB | Option 1. We think that using the enhanced TA indication granularity for the normal TA procedure will be just overhead. At the same time, we think it should not be specified whether one or the other indication should be used. That should be up to gNB implementation when it will use the normal TA indication or enhanced TA indication for PDC. >> Feature lead Agree, no need to restrict whether to apply to normal TA procedure. The key point is that there is no need to couple with normal TA procedure. Maybe we can update option 1 a little bit to clarify. |
| vivo | Option 1. In our understanding, normal TA procedure is sufficient for UE without synchronization requirement |
| ZTE | Option 1 is preferred since there is no requirement on the improvement of the UL transmission. |
| Intel | Option 1 |
| HW/HiSi | Option 1 or leave it to UE implementation is fine. The enhancement is only needed for PDC. Applying the enhanced TA also to the normal TA procedure would result into unnecessary specification efforts (as seen by the FFSs for Option 2) and possibly also implementation complexity. We should avoid this at this late stage. |
| Ericsson | Option 1 |
| LG | We prefer to leave it for UE implementation. And we can also compromise to option 1 |
| Qualcomm | Option 1 |

Summary of the status for issue 4.2-2

- Option 1:

  - Support: Ericsson, Nokia, NSB, Huawei, HiSilicon, vivo, ZTE, Intel

- Option 2:

- Feature lead: It seems all companies agree with option 1, while OPPO has some questions for clarification.

- 4.2.2a Second round discussion for issue 4.2-2

Based on the inputs and analysis above for issue 4.2-2 in section 4.2.2, the following proposals are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals/questions before providing your views here.

Proposal 4.2.2a-1: If enhanced TA-based PDC with enhanced TA command indication granularity is supported in Rel-17,

- The enhanced TA command indication granularity introduced for enhanced PDC is only applied for PDC purpose, which doesn’t have impact on normal TA procedure, i.e. normal TA procedure will still follow the existing TA command indication granularity.

- Note: This doesn’t preclude UE to apply the indicated TA for PDC to uplink channel/signal not intended for PDC, if applicable. It is understood that this can be up to UE implementation.

| Company | View |
| --- | --- |
| Intel | We understand the intention of the proposal, but it seems the first sub-bullet and the note are contradicting to each other. Probably the main intention is that the measurement accuracy requirements are not affected for the legacy TA procedure when the enhanced TA procedure is active. Is that correct understanding? >> Feature lead Thanks. Firstly, yes the intention is not to change the requirements for normal TA, e.g. TA adjustment error. From specification perspective, all the enhancements are only for PDC, so we don’t need to do further specification work for normal TA procedure due to the new procedure for PDC. Of course, once UE get the latest TA from the procedure for PDC, UE can choose it use it on the channel/signal not intended for PDC or not, it is just an implementation issue. Maybe we can update the proposal to avoid this kind of misunderstanding. |
| OPPO | We are still confused by FL’s explains. On one hand, FL says “if we go option 1, that means we don’t need to touch the normal TA procedure, including the normal TA command bit width”, for which we understand as that nothing in equation of “” in normal TA procedure should be modified by this new enhanced TA command. But on the other hand, FL also says “If we go to option 1, ….., and UE doesn’t need to operate two TA procedure in parallel in most of the time”. Then could FL please explain how the new TA command works (i.e., the new TA command does not apply to a separate TA procedure but meanwhile does not change anything in existing TA loop)? >> Feature lead 1. Thanks. From my reply you could see that I use “normal TA command bit width”, which means the formula you refer doesn’t need to be changed for normal TA procedure, but for channel/signal for PDC we would need some similar formula also, depending on further discussion on how to indicate the TA, e.g. either absolute TA or relative TA. As to the description on whether to operate one or two TA procedure in my reply, it is to reply to your question whether one or two TA procedure. Maybe you can clarify more what you are exactly asking also. Happy to hear your solutions also. Then I don’t need to try hard to think about answer to your questions which you seems not like.  2. Maybe there is some misunderstanding, the conclusion “the new TA command does not apply to a separate TA procedure but meanwhile does not change anything in existing TA loop” absolutely not correct. Not sure if my above explanation in the first point would make the situation clearer or not. By the way, you can provide to me what way you would like it to work also, then I can see how to move forward. As feature lead, I am just trying to help the progress, based on questions from companies I tried to provide my thinking, it might not be that complete, the only intention is to bring us moving forward. None of the solution is mine.  |
| Ericsson | We are fine with the intention of the proposal. The main point seems to be: “A separate set of protocol/signaling/measurement/etc is applied, and the existing TA procedure is not affected in RAN1/RAN2/RAN4 specifications.” It is useful to add this to clarify. >> Feature lead Thanks. Yes the intention is similar as what you mention here. However, maybe too strong to say absolutely separate set of protocal…, in some cases maybe some common parts can be applied. We can see the situation clearer once all details done. |
| vivo | We are fine with the intention of the proposal. According to the note, it is possible for UE to use the indicated TA for PDC instead of the normal TA for uplink channel/signal. In our understanding, gNB can configure two TA command indication, i.e., one is normal TA indication granularity and another is enhanced TA indication granularity. For normal TA, the existing granularity and procedure is followed. >> Feature lead Yes that is the intention. |
| Nokia, NSB | Support On the comment by Ericsson: there may not be a need for a separate protocol / signaling / measurement here but the new protocol (e.g., finer granularity of TA signaling) could be used also for the existing TA procedure by applying the existing granularity in the TA adjustment based on the finer granularity TA signaling. |
| Samsung | We support the intention of the proposal. We think the intention is that, this will not introduce new TA adjustment requirement. |
| ZTE | We support this proposal. |
| HW/HiSi | We support the proposal and also share the view from Samsung that no new TA adjustment requirements will be introduced. |
| Qualcomm | We are fine with the proposal. |

Summary of the status for proposal 4.2.2a-1 in second round

| Support | Vivo, Nokia, NSB, Samsung, ZTE, Huawei, HiSilicon, Qualcomm |
| --- | --- |
| Not support | Intel, OPPO, Ericsson |

FL recommendation: Still different views, let’s further discuss this issue.

- 4.2.2b Third round discussion for issue 4.2-2

Based on the inputs and analysis above for issue 4.2-2 in section 4.2.2a, the following proposals are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals/questions before providing your views here.

Proposal 4.2.2a-1: If enhanced TA-based PDC with enhanced TA command indication granularity is supported in Rel-17,

- The enhanced TA command indication granularity introduced for enhanced PDC is only applied for PDC purpose, which doesn’t have impact on normal TA procedure, i.e. normal TA procedure will still follow the existing TA command indication granularity.

- Note: This doesn’t preclude UE to apply the indicated TA for PDC to uplink channel/signal not intended for PDC, if applicable. It is understood that this can be up to UE implementation.

| Company | View |
| --- | --- |
| Feature lead | @ Intel @ OPPO @ Ericsson Please check my replies to your comments in section 4.2.2a, and see if you have further comments. |
| Nokia, NSB | Support. Note: For readability, we think there should not be a strikethrough on the word applied >> Feature lead Agree. Will bring it back. |
| Intel | Carefully reading, it seems in case of PDC there will two different TA commands: one with legacy granularity and one with enhanced granularity. That was not our original understanding of the proposal, i.e., we assumed the indication is still by a single command which replaces the legacy command. Could you clarify a bit more? >> Feature lead Depends on how to enhance the TA command indication granularity, e.g. if we apply similar manner as that in IAB, an additional MAC CE will be used and keep the original TA command signaling unchanged, which is beneficial since we don’t need to touch the original normal TA procedure. |
| OPPO | Please allow me being more specific. So far we still fail to see whether there is one single TA loop or two TA loops. - In case of single TA loop, NTA_new=NTA_old+TAC where TAC can be either legacy TAC (TAClegacy) or enhanced TAC (TACenh) with smaller granularity. For this case, our question is: if UE is allowed to use TAClegacy at time t1 and to use TACenh at time t2, does the RAN1 spec would require UE to send UL SRS at a specified moment after t2, i.e., which would be a TAC-triggered AP-SRS? would the NTA_new after t2 be applicable to UL transmission only related to PDC (something like PDC-oriented SRS which is proposed to pair with TRS in another proposal) but not to other non-PDC related UL transmissions, until NTA_new is refreshed again with a legacy TAC? If yes, then PDC-related TA already impacts the non-PDC transmission on UL, opposite to what we were told before. - In case of two TA loops which is intended by proposal, we have NTAnew,legacy=NTAold,legacy+TAClegacy NTAnew,enh=NTAold,enh+TACenh The two NTA remain in UE as part of transmission state. When it comes to UL slot transmission, the proposal is not clear about which one is used because so far there is no decision on whether the UL slot is for PDC purpose or not. >> Feature lead With the proposal here, the result might not be either case 1 or case 2 above as you mentioned, depending on whether absolute TA or relative TA is indicated. If a relative TA will be indicated with the enhanced TA command indication granularity, then it would be aligned with your case 2 above. However, if an absolute TA is indicated with the enhanced TA command indication granularity, then we don’t need the formula in your case 2, i.e. NTAnew,enh=NTAold,enh+TACenh, what indicated in the signaling is already an absolute value and thus can be used directly, in which case I think it cannot be considered as two TA loops, since at the UE side UE just received the enhanced TA and apply it to do the PDC. So we see potential concerns/uncertainties in both cases if two TA granularities are allowed being used inside UE. The cleaner choice is to configure UE to use only one granularity – but this would make the enhanced granularity completely applicable to non-PDC application, which is out of scope. |
| Ericsson | We can accept the proposal. |
| Qualcomm | We are fine with this proposal since it is important to keep the existing TA process unchanged |
| Samsung | Support. |
| vivo | We are fine with this proposal. |
| ZTE | We support this proposal in order to avoid the impact of TA procedure. |
| HW/HiSi | Support |
| LG | Support. In addition, it would be better to specify that the proposal is in the perspective of RAN1. |

Summary of the status for proposal 4.2.2a-1 in third round

| Support | Vivo, Nokia, NSB, Samsung, ZTE, Huawei, HiSilicon, Qualcomm, Ericsson, LG |
| --- | --- |
| Not support | Intel, OPPO |

FL recommendation: Still different views, let’s further discuss this issue.

- 4.2.2c 4th round discussion for issue 4.2-2

Based on the inputs and analysis above for issue 4.2-2 in section 4.2.2c, the following proposals are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals/questions before providing your views here.

Revised Proposal 4.2.2a-1: If enhanced TA-based PDC with enhanced TA command indication granularity is supported in Rel-17,

- The enhanced TA command indication granularity introduced for enhanced PDC is only applied for PDC purpose, which doesn’t have impact on normal TA procedure, i.e. normal TA procedure will still follow the existing TA command indication granularity.

- Note: This doesn’t preclude UE to apply the indicated TA for PDC to uplink channel/signal not intended for PDC, if applicable. It is understood that this can be up to UE implementation.

| Company | View |
| --- | --- |
| Feature lead | @ Intel @ OPPO Please check my replies to your comments in section 4.2.2b, and see if you have further comments. |
| Nokia, NSB | Support. |
| Intel | We are fine in principle. At the same time, we think that the note may not be necessary. |
| HW/HiSi | Support |
| OPPO | We can accept the proposal (w/o note) given the key message is to have no impact to normal TA procedure, although we have some worry on the consequence of this proposal. The timing advance is defined per frame timing, as 38.211 says as in figure below. What if a uplink frame (say in the figure) contains both non-PDC signal/channel and PDC-related signal/channel? Would the TA be NTAnew,legacy or NTAnew,enh? Or the intention is to have slot timing inside this uplink subframe to change between NTAnew,legacy and NTAnew,enh back and forth by assuming the slots can be identified as non-PDC slot and PDC slot? We also share the view with Intel for the note. The note seems to say RAN1 does not need to further discuss whether the TA in above figure should be NTAnew,legacy or NTAnew,enh -- it can be UE implementation issue. We do not think it is safe to agree that point at this time. |
| Ericsson | Support |

Issue 4.2-3: Whether to apply enhanced values for BS detecting error and DL frame timing error for enhanced TA-based PDC.

In the previous meetings, the following agreements were made:

RAN1#102-e Agreements: 100 ns is assumed for BS detecting error. RAN1#104-e Agreements: Take ±100 ns as the assumption for downlink frame timing detection error (errorUE,DL,RXerrorUE, DL, RX) at the UE for evaluation of the overall time synchronization error for TA based propagation delay compensation, if downlink frame timing detection error needs to be considered separately. - Send a LS to RAN4 to ask for clarification on whether downlink frame timing detection error is included in Te or not - In the LS, to include more details about option 1 (included) & option 2 (not included); also including the necessary background - FFS whether to apply the same value to RTT-based propagation delay compensation, and the corresponding condition (if any) if the same value will be applied

Feature lead: It is expected that the above value is mainly for Rel-16 TA-based PDC. If other RS is available for TA-based PDC, e.g. TRS for evaluating downlink frame timing detection error and SRS for BS detecting error, it can be expected that larger bandwidth of RS can be used to achieve smaller detecting error. As given in Ericsson paper R1-2108833, in general the minimum timing detection error = 0.5/(RS BW), which can be a rough value in theory and can reflect the change with the bandwidth. For example, for downlink frame timing error, if 100 MHz RS bandwidth is assumed, then roughly errorUE,DL,RX= ±23 ns by using the formula. For enhanced TA-based PDC, if Te is improved with TRS with larger bandwidth, the accordingly the BS detection error and UE downlink frame timing error can be enhanced according also. For fair comparison with RTT-based PDC, same value of errorUE,DL,RX and errorBS,UL,RX should be used.

- 4.2.3 First round discussion for issue 4.2-3

Based on the inputs and analysis above for issue 4.2-3, the following proposals are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals/questions before providing your views here.

Proposal 4.2.3-1: For enhanced TA-based PDC and RTT-based PDC, the equation minimum timing detection error = 0.5/(RS BW) = 0.5/(N_PRB*12*SCS) is used to achieve errorUE,DL,RX and errorBS,UL,RX, if need to be considered separately, where N_PRB is the number of PRBs of the RS bandwidth.

| Company | View |
| --- | --- |
| OPPO | Issue 4.2-3 is called “Whether to apply enhanced values for BS detecting error and DL frame timing error for enhanced TA-based PDC”, but the proposed formula of “error = 0.5/(RS BW) = 0.5/(24*12*SCS)” gives 116ns for SCS=15kHz, even larger than 100ns that is currently assumed. So is this an enhancement or an argument against existing RAN1 assumption? It does not seem to be the case that the proposed formula is used in RAN4 to decide Rx-to-Tx measurement performance. For example, 38.133 says following for PRS-based Rx-to-Tx measurement in AWGN. It basically says the overall error of RTT measurement (including both Rx error and Tx error) can be around 40ns for 24RB of PRS, which is much smaller than the “minimum” value given by the proposed formula. Accuracy; Conditions. PRS Ês/Iot; Minimum PRS bandwidth; PRS SCS. TcNote 5; dB; RB; kHz. ± [78+]; -3; ≥[24]; 15. ± [59+]; ≥[52]. ± [30+]; >[104]. >> Feature lead Sorry there is a typo in my original proposal. I updated it accordingly as highlight in Red. In addition, my intention is not to say RAN4 used the formula to decide Rx-to-Tx measurement performance, I believe RAN4 would get values based on more complete evaluations. Here the intention of the proposal is just to provide a way to get a rough value for RAN1 to compare TA-based PDC or RTT-based PDC, since RAN1 may be not able to get these values by evaluations. Of course, if people prefer more accurate values by evaluations, it would be great also. Let’s hear more views from companies. |
| Nokia, NSB | The intention of the proposal is not clear to us. Is it the intention here to change the assumption of this error components which were already agreed? Or is it the intention to define a common equation to determine the value of these error components depending on the BW (i.e. for a fair comparison of TA and RTT under a same BW assumption for both)? In the latter case, the equation should stay as a function of the BW to be assumed for both methods – which seems to have been addressed by FL in the update already. >> Feature lead The original agreement 100 ns is agreed mainly for Rel-16 TA-based PDC. For Rel-17 enhanced TA-based PDC, a smaller value can be achieved considering larger bandwidth can be applied. The original 100 ns is mainly taken considering 20 MHz. On the other hand, the intention is also to have fair comparison of TA and RTT based PDC as you mentioned, since it is very obvious that RTT-based PDC can use smaller value in terms of Rx-Tx time difference. |
| ZTE | We are fine to reconsider the BS detection error and UE downlink frame timing error for fair comparison since DL RS/UL RS with larger bandwidth can be used in both TA-based PDC and RTT-based PDC. Regarding the value, we think it is also related to the density and the repetition. >> Feature lead Agree that bandwidth is not the only factor. Here only bandwidth is included is because it is the main factor and also can be represented by formula. For other factors, unless we do the simulations there is no any other way. As I mentioned before, if people prefer to achieve this kind of value by evaluations, I am fine also since that would be more accurate. I am just not sure if companies can be able to provide it to next RAN1 meeting since the time interval is so short. |
| Intel | Fine with the formula |
| Ericsson | We are fine to use such formula as a theoretically optimal value for estimation purpose only. As in the RAN4 discussion paper that led to Te table, performance margin should be added to the theoretical optimal. Thus, the calculated values cannot be used as requirement in implementation. >> Feature lead Sure, as clarified above it is only for evaluation purpose. |
| Qualcomm | Support. |

- 4.2.3a Second round discussion for issue 4.2-3

Based on the inputs and analysis above for issue 4.2-3 in section 4.2.3, the following proposals are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals/questions before providing your views here.

Revised Proposal 4.2.3-1: For evaluation and comparison of enhanced TA-based PDC and RTT-based PDC, the equation minimum timing detection error = 0.5/(RS BW) = 0.5/(N_PRB*12*SCS) can be used to achieve errorUE,DL,RX and errorBS,UL,RX, if needed in the evaluation equation to be considered separately, where N_PRB is the number of PRBs of the RS bandwidth used in the detection by UE and gNB, respectively.

- Note: Detection error achieved by evaluations is not precluded if available.

| Company | View |
| --- | --- |
| Intel | OK |
| OPPO | Not agree. First, for TA based case, RAN1 has no agreement so far saying the TA interval measurement on UE side should based on specific DL-RS. Instead, it is stated in 38.133 that “ (in Tc units) for other channels is the difference between UE transmission timing and the downlink timing immediately after when the last timing advance in clause 7.3 was applied.”. The wording of “immediately after” tells UE when to determine its TA interval, because this is the most reliable time for UE to know the UL-Rx timing is aligned as gNB desires. Unless the frame receiving the PRS/TRS is immediately after the frame where UE applies TA command, the formula for PRS/TRS does not apply to errorUE,DL,RX. However, PRS/TRS transmissions are pre-configured while TA command is on-demand, so this “immediately after” relationship cannot be guaranteed. Secondly, still for TA-based case, the “downlink timing” in the above-mentioned RAN4 text is currently not restricted to the timing based on specific DL signal – it could be based on SSB at least. To follow the proposal, does it mean the TA adjustment for PDC purpose has to rely on PRS/TRS but not other signal? This would be another fundamental change to TA procedure. >> Feature lead For enhanced TA-based PDC with reduced Te, it is expected that it needs to be based on TRS, we cannot only follow the normal TA procedure now, as discussed in section 4.2.2a. Thirdly, the proposed formula provides “minimum timing error”, but how could a “minimum error” come to be used in an error budget calculation which is supposed to account worst cases? >> Feature lead We can delete minimum. Here just to agree some formula for the evaluation and comparison. BTW, we wonder why the proposal still tried to include RTT-based PDC? RAN1 already agreed to use Alt-1 formula for RTT-based PDC, which only contains the concerned errorUE,DL,RX for the PD compensation step, but the compensation step has nothing to do with PRS/TRS. >> Feature lead For RTT-based PDC Alt.1, you could say we have this errorUE,DL,RX also, you can see it is there in the equation, which is used to receive the reference time clock as we discussed in several previous meetings. |
| Ericsson | Fine with the proposal with minor edits. Suggested change: “…the equation minimum timing detection error = 0.5/(RS BW) = 0.5/(N_PRB*12*SCS) can be used to achieve errorUE,DL,RX and errorBS,UL,RX, if needed in the evaluation equation to be considered separately, where N_PRB is the number of PRBs of the RS bandwidth used in the detection by UE and gNB, respectively”. This addresses some of the concern raised by OPPO. >> Feature lead Ok to update accordingly with some adjustment based on comment from OPPO. |
| vivo | Firstly, we think TA-based method and RTT-based method have the different procedure. For TA-based method, we share the similar view with OPPO that obtaining “downlink timing” is not restricted to a specific DL signal. So, we should clarify firstly whether the same RS configurations (bandwidth and density, etc.) are used for both TA-based and RTT-based method if the same equation is applied for the timing detection error calculation. >> Feature lead Please see my reply to OPPO. In addition, as discussed in previous round, the formula is just for comparison purpose, so it only consider the main factors like bandwidth. So we don’t need to really clarify all details of the RS for different PDC methods. Of course, with the note then more accurate value by evaluation can be used if available. |
| Nokia, NSB | We are fine with the proposal |
| Samsung | Fine |
| ZTE | We support the intention of this proposal that the two error components are related to the receiving RS bandwidth. But we have the concern on the assumption. Based on the equation, the error is 116ns in the case of 24 PRBs and 15kHz. In Issue 4.3-5, it says the downlink detection error is reflected by errorUE, RxTxDiff, which is 101 Tc (50.5ns) in the case of 24 PRBs and 15kHz according to TS38.133. It is obvious the two assumed values are contradictory. So we think we should use a smaller value here or a larger value in Issue 4.3-5. >> Feature lead As explained above, it is true that the value achieved by the formula is not that accurate, but since with this formula we will use the same value for both TA-based PDC and RTT-based PDC, it should be fine. Of course, as shown in this proposal, if companies can provide values based on evaluation to the next meeting, that would be great also, then we can use those more accurate value if any. It is just not clear whether companies can be able to provide evaluation during such short time. In addition, as commented before, if proposed conclusion is the common understanding, we wonder whether the margin Y reflecting errorBS, DL,TX the and margin  reflecting errorUE, UL, TX can be used in TA-based PDC as well as TA procedure. From our view, we think it can be reused at least when the network deliver the reference time. We don’t see the technique reason to use different assumptions for the two solutions. For example, Te is used in TA-based PDC while errorUE, RxTxDiff is used in RTT-based PDC. >> Feature lead Not sure I got the point correctly here. We already have agreed to take Alt.1 for both TA-based PDC and RTT-based PDC. So you are proposing to use Y and  to represent errorBS, DL,TX and errorUE, UL, TX for TA-based PDC? |
| HwHiSi | Support |
| Qualcomm | Support |

Summary of the status for revised proposal 4.2.3-1 in second round

| Support | Intel, Ericsson, Nokia, NSB, Samsung, Huawei, HiSilicon, Qualcomm |
| --- | --- |
| Not support | OPPO, vivo, ZTE |

FL recommendation: Still different views, let’s further discuss this issue.

- 4.2.3b Third round discussion for issue 4.2-3

Based on the inputs and analysis above for issue 4.2-3 in section 4.2.3a, the following proposals are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals/questions before providing your views here.

Revised Proposal 4.2.3-1: For evaluation and comparison of enhanced TA-based PDC and RTT-based PDC, the equation minimum timing detection error = 0.5/(RS BW) = 0.5/(N_PRB*12*SCS) can be used to achieve errorUE,DL,RX and errorBS,UL,RX, if needed in the evaluation equation to be considered separately, where N_PRB is the number of PRBs of the RS bandwidth used in the detection by UE and gNB, respectively.

- Note: Detection error achieved by evaluations is not precluded if available.

| Company | View |
| --- | --- |
| Feature lead | @ vivo @ OPPO @ ZTE Please check my replies to your comments in section 4.2.2a, and see if you have further comments. |
| Nokia, NSB | Support in principle. Sorry that we only noted this now, but we actually think the wording “if need to be considered separately” was preferable, in order to avoid a conflict with the agreed assumptions from Rx-Tx accuracy requirements from TS 38.133 v17.3.0 and with Potential conclusion 4.3.5-2. That is because the proposal conclusion says that errorBS, UL,RX is reflected by errorgNB,RxTxDiff and error errorUE, DL,RX is reflected by errorUE, RxTxDiff. However, it may not be possible to fit the rough value of errorUE,DL,RX and errorBS,UL,RX in the errorgNB,RxTxDiff and errorUE, RxTxDiff. So, if we don’t restrict to apply the rough error values from equation above only in case where they are considered separately, the consequence will be that we cannot use the rough values for evaluation of RTT (e.g. for the SFN boundary error part) at the same time that the Rx-Tx accuracy requirements from TS 38.133 v17.3.0 (for the PD error part). >> Feature lead That’s not the intention for sure. To avoid misunderstanding, maybe we can keep “separately” there. |
| Intel | The alignment of error bounds between RTT and TA based PDC is important. Support in principle, with potential clarification based on Nokia’s good comment. >> Feature lead See replies to Nokia above. |
| OPPO | The proposal, if agreed, should have no way to suggest RAN1 also agrees to use TRS for TA [control] procedure. So we can be neutral if a sub-bullet is added to say - This does not mean the UL-Tx timing adjustment in UE TA procedure must be based on the reception timing relative to concerned DL-RS. If company wants to bind TA control loop with TRS, please raise separate proposal. This proposal is only for one possible way for error calculation. Also please clarify whether “errorUE,DL,RX and errorBS,UL,RX” in this proposal take reference at antenna connector or baseband? We assume it is antenna connector, because none of them contains room to account antenna-to-baseband processing delay variation. But would this mean the TA interval measurement is somehow moved from baseband to antenna connect? Anyway please clarify. >> Feature lead As explained in the previous rounds, the formula here is just to find a uniform way to compare the enhanced PDC methods, the formula itself doesn’t reflect the all the potential factors that may have impact on the value of the error component but just reflect the main factor, and therefore the reference points would not necessarily have impact on the proposal here. Based on your comments in previous discussions, I know you have concern on reference point for error components for TA-based PDC and RTT-based PDC. However, the reference point for a certain error components would depend on the definition in RAN4, e.g. for Rx-Tx time different RAN4 defines that the reference point is antenna connector, but RAN4 also define the margin, which may cover the baseband to connector delay if any. [OPPO update on v023] We realized some new concerns besides the implication of this proposal on whether to use DL-RS in TA [control] procedure. Concern#1: We asked in earlier comment why this is for RTT-based PDC, and FL answers “For RTT-based PDC Alt.1, you could say we have this errorUE,DL,RX also, you can see it is there in the equation, which is used to receive the reference time clock as we discussed in several previous meetings.” Then maybe RAN1 should firstly reach a consensus that the SIB9 carrying ReferenceTimeInfo should always be made accompanied with a PRS/TRS in RTT-based PDC. This was once asked by Nokia upon implicit PDC and we answered No in order to align with RTT-based PDC. But it seems the story would not be like what we thought for RTT-based PDC if FL’s view is taken. >> Feature lead Not sure I get your point here. For RTT-based PDC, for sure either TRS or PRS will be used, no matter whether the reference time clock is transmitted in SIB9 or in UE dedicated RRC signaling. Here again the formula is not reflecting all the potential factors that may have impact on the value of the error components, and also gNB can transmit the TRS/PRS in the appropriate time instance by implementation. Maybe I miss your point. Concern#2: Given the formula 0.5/(N_PRB*12*SCS) is independent from SINR and fading profile, we wonder how to interpret the result? Again, if this formula is a certain lower bound of sync error, maybe it should not be used in budget calculation. Anyway, please clarify how the formula takes SINR and fading profile into consideration. >> Feature lead Again the formula is not reflecting all the potential factors that may have impact on the value of the error component, but just to reflect the main factors, i.e. RS bandwidth. The intention is to compare the candidate enhanced PDC methods. Of course, as shown in the proposal, if evaluates achieved by evaluations which may also reflect factors like fading channel or SINR also available, it can be used also. |
| Ericsson | Support the proposal. It’s not clear how to understand Nokia comment. For the RTT method, errorUE,DL,RX and errorBS,UL,RX are not used at the same time, only errorUE,DL,RX is used. What is the meaning of A and B in “… A is considered separately from B”? Our understanding is, in RTT equation, errorUE,DL,RX is associated with RRC ReferenceTimeInfo reception, which is separate from the measurement and reporting of RxTxTimeDiff based on TRS/PRS and SRS. >> Feature lead See replies to Nokia above. I think it should be fine to keep “separately” to avoid misunderstanding. |
| Qualcomm | We are fine with the proposal to move forward for both evaluation schemes. |
| Samsung | Support in principle. |
| vivo | Firstly, we also think it should be discussed separately whether the TA procedure should be associated with a TRS for TA-based PDC before achieving the conclusion. Secondly, we have also observed that different reference point assumption is applied for TA-based method and RTT-based method, i.e. at antenna connector for RTT based method and at baseband for TA-based method. Additional error component exists from antenna connector to the baseband. If we compare these two methods, the error component should be considered. >> Feature lead See replies to OPPO. |
| ZTE | From issue Proposal 4.2.1a-1 in the third round, it seems some companies do not support introducing TRS and SRS for TA-based PDC while support this proposal. So we think it is impossible to get the same value for TA-based PDC and RTT-based PDC as FL replied if different RS with different bandwidth are used. Therefore, we think we should agree proposal 4.2.1a-1 first, then we can accept this proposal. Alternatively, we prefer to use 100ns for synchronization error for both downlink and uplink for TA-based PDC and RTT-based PDC to follow the previous agreements. >> Feature lead In the proposal, we only say RS. We don’t really need to wait for the outcome of proposal 4.2.1a-1. |
| HW/HiSi | Support |
| LG | Support |

Summary of the status for revised proposal 4.2.3-1 in third round

| Support | Intel, Ericsson, Nokia, NSB, Samsung, Huawei, HiSilicon, Qualcomm, LG |
| --- | --- |
| Not support | OPPO, vivo, ZTE |

FL recommendation: Still different views, let’s further discuss this issue.

- 4.2.3c 4th round discussion for issue 4.2-3

Based on the inputs and analysis above for issue 4.2-3 in section 4.2.3b, the following proposals are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals/questions before providing your views here.

Revised Proposal 4.2.3-1: For evaluation and comparison of enhanced TA-based PDC and RTT-based PDC, the timing detection error = 0.5/(RS BW) = 0.5/(N_PRB*12*SCS) can be used to achieve errorUE,DL,RX and errorBS,UL,RX, if needed in the evaluation equation separately, where N_PRB is the number of PRBs of the RS bandwidth used in the detection by UE and gNB, respectively.

- Note: Detection error achieved by evaluations is not precluded if available.

| Company | View |
| --- | --- |
| Feature lead | @ OPPO @ vivo @ ZTE Please check my replies to your comments in section 4.2.2b, and see if you have further comments. |
| Nokia, NSB | Support. @Ericsson: - In Ericsson’s comment What is the meaning of A and B in “… A is considered separately from B”? In our understanding, A would be an error component (errorUE,DL,RX or errorBS,UL,RX) and B is another error component which may include A (e.g. errorUE, RxTxDiff, errorgNB,RxTxDiff or Te). - According Ericsson’s view, is the errorUE,DL,RX in this proposal different or is it the same as the errorUE,DL,RX from Potential conclusion 4.3.5-2? |
| Intel | OK |
| HW/HiSi | Ok |
| OPPO | Our thinking is the agreement of this formula would increase the chance of debate in future meeting because - it may conflict with some existing agreed numbers. - This formula is not clarified with its relation to SINR and fading profile. But if the purpose is to have a “backup” formula for the case where the value for corresponding error component is not available w/o using this backup formula, we can accept the proposal if this is what the majority wants. |
| Ericsson | We can accept the proposal. @Nokia: About ‘separately’, it reads like A is errorUE,DL,RX and B is errorBS,UL,RX since they are the only variables in the sentence. It’s hard to get that B refers to errorUE, RxTxDiff, errorgNB,RxTxDiff or Te, which are not mentioned in the sentence. This sense of ‘separately’ is taken by the equation itself in our view, i.e., two variables are both included in the same equation, hence they are needed ‘separately’. |

Issue 4.2-4: Whether we need to use timing advance adjustment accuracy instead of Te for the evaluation of TA-based PDC?

We already reached the following agreements below.

RAN1#102-e

Agreements:

The value defined in Table 7.1.2-1 for initial transmit timing error (Te) in TS 38.133 should be considered for evaluation of the time synchronization.

Agreements:

Timing advance adjustment accuracy defined in Table 7.3.2.2-1 in TS 38.133 is assumed for evaluation of the time synchronization.

RAN1#103-e

Agreements:

TA adjustment accuracy is not considered for the evaluation of time synchronization error.

RAN1#104bis-e

Agreement:

Take the following as the evaluation assumptions for both RTT-based PDC and TA-based PDC.

- The UE may acquire an up-to-date PD estimation after waking up from DRX. This implies that gNB may signal an update timing advance value or complete a Rx-Tx measurement procedure.

- errorUE,DL,RXerrorUE, DL, RX is based on other signals (e.g. CSI-RS) instead of SSB.

- errorBS, UL,RXerrorUE, DL, RX is based on other uplink signals instead of contention based PRACH, e.g. SRS.

- Further study and specify new procedure/signaling (if necessary) to ensure that the PD estimation can be acquired after DRX for the adopted PDC method.

In RAN1#106b-e meeting, Nokia (R1-2109161) propose to use TA adjustment error instead of Te for evaluation, because based on RAN1#104-bis agreement the UE may acquire an up-to-data PD estimation after waking up from DRX.

Feature lead:

This is related to the interpretation about the RAN4 spec highlight in yellow below. Nokia think that Te only applies for the first UL transmission after the UE has been in DRX and before receiving an up to date NTA, while some other companies think that Te applies to any UL transmission (and not only the first one after DRX). It seems difficult to achieve consensus in RAN1 based on the previous experience.

- 7.1.2.1 Gradual timing adjustment Requirements in this section shall apply regardless of whether the reference cell is on a carrier frequency subject to CCA or not. When the transmission timing error between the UE and the reference timing exceeds Te then the UE is required to adjust its timing to within Te. The reference timing shall be before the downlink timing of the reference cell. All adjustments made to the UE uplink timing shall follow these rules: 1) The maximum amount of the magnitude of the timing change in one adjustment shall be Tq. 2) The minimum aggregate adjustment rate shall be Tp per second. 3) The maximum aggregate adjustment rate shall be Tq per 200 ms. where the maximum autonomous time adjustment step Tq and the aggregate adjustment rate Tp are specified in Table 7.1.2.1-1. Table 7.1.2.1-1: Tq Maximum Autonomous Time Adjustment Step and Tp Minimum Aggregate Adjustment rate Frequency Range; SCS of uplink signals (kHz); Tq; Tp. 1; 15; 5.5*64*Tc; 5.5*64*Tc. 30; 5.5*64*Tc; 5.5*64*Tc. 60; 5.5*64*Tc; 5.5*64*Tc. 2; 60; 2.5*64*Tc; 2.5*64*Tc. 120; 2.5*64*Tc; 2.5*64*Tc. NOTE: Tc is the basic timing unit defined in TS 38.211 [6].

Considering that we have sent LS to RAN4 asking for checking the feasibility on enhanced Te and TA indication granularity, it would be good not to debate more later whether the enhanced Te should be used or not in Nov meeting based on inputs from RAN4, therefore it would be good for us to address the issue in Oct meeting, or if no consensus can be achieved we may have to require RAN4 again, which is not preferred considering we already requested RAN4 to do much work in one meeting already.

On the other hand, even we go to the interpretation from Nokia, it means that sometimes Te is used while some other time TA adjustment error will be used. Since Te is larger than TA adjustment error at least according to the current values, it seems ok to only check the case with utilization of Te. Therefore, a proposed conclusion in section 4.2.4 is given below for further discussion.

- 4.2.4 First round discussion for issue 4.2-4

Based on the inputs and analysis above for issue 4.2-4, the following proposals are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals/questions before providing your views here.

Potential conclusion: When evaluating enhanced TA-based PDC, there is no need to replace Te by TA adjustment error.

| Company | View |
| --- | --- |
| OPPO | Ok to the proposed conclusion. |
| Nokia, NSB | We see the point from the FL and are OK with this proposal for the sake of progress. Though it should be noted that the agreement reached in previous meeting, i.e. that UE gets an updated TA value, was pointless since in the end we will be assuming the requirement of an open loop timing advance adjustment. We remind that the uplink timing of a transmission which happens after UE receives a timing advance command and before the UE time alignment timer expires, shall fulfill timing advance adjustment requirement, and not just the more relaxed Te requirement, as per A.6.4.2 in 38.133. >> Feature lead Thank you very much for being flexible. That agreement I think still meaningful, which means that at least some error components like DL frame timing error is not necessarily based on SSB I think.  |
| vivo | Support. |
| ZTE | We are fine with this proposal to keep previous agreement. |
| Intel | Agree |
| HW/HiSi | Agree |
| Ericsson | Agree |
| LG | Agree |
| Samsung | Agree |

Feature lead: The above conclusion is stable.

Issue 4.2-5: Evaluation on the achievable time synchronization accuracy for enhanced TA-based PDC

A few contributions provide views on enhanced TA-based PDC, but most companies didn’t provide analysis on it since we are still waiting for inputs from RAN4. For easier comparison between TA-based PDC and RTT-based PDC, the required enhanced Te and TA command indication granularity as summarized in feature lead summary R1-2108634 based on the evaluations from companies submitted to RAN1#106-e meeting is copied here again, for easier potential comparison between TA-based PDC and RTT-based PDC.

Based on the table below, it can be seen that if Te and/or TA command indication granularity can be reduced, there is some chance that TA-based PDC can meet the budget for control-to-control scenarios, of course depending on how much Te and TA command indication granularity can be reduced, which needs inputs from RAN4.

Note that the values in the table here assumes no enhancements for BS uplink detection error, if it can be enhanced also as discussed in section 4.2.3, then the required Te+errorTAindication can even be smaller.

Table Sum of Te and error from TA indication granularity for TA-based PDC to meet the single Uu interface budget

|  | Te+errorTAindication |  |
| --- | --- | --- |
|  | ±275 ns single Uu interface budget | ±145 ns single Uu interface budget |
| Equation Alt. 1 | ~55 ns e.g. (1/10)*Te + (1/2)*(1/16)* (16*64*Tc/2) | N/A |
| Equation Alt. 2-1 | ~320 ns e.g. (4/5)*Te + (1/2)*(1/16)* (16*64*Tc/2) | ~60 ns e.g. (1/10)*Te + (1/2)*(1/16)* (16*64*Tc/2) |
| Equation Alt. 2-2 | ~385 ns e.g. (9/10)*Te + (1/2)*(1/16)* (16*64*Tc/2) | ~125 ns e.g. (1/4)*Te + (1/2)*(1/16)* (16*64*Tc/2) |
| Note: (1/2)*(1/16)* (16*64*Tc/2) is used here for errorTAindication because it is assumed that at least the existing work in Release-16 for IAB for Timing Delta MAC CE can be reused, which can achieve 64*Tc for FR1 for the indicating granularity. |  |  |

RTT based propagation delay compensation

This section will discuss some key issues for RTT-based propagation delay compensation.

Issue 4.3-1: TRS/PRS and SRS configuration

In RAN1#106-e meeting, the following agreements related to TRS and SRS configuration were achieved:

Agreement Support the following configurations for RTT-based propagation delay compensation, if RTT-based propagation delay compensation is supported. - At least one CSI-RS for tracking (TRS) configuration for Rx – Tx time difference estimation at UE side if PRS is not configured - At least one SRS configuration for Rx – Tx time difference estimation at gNB side Agreement If RTT-based propagation delay compensation is supported and performed at the UE side, the Rx-Tx measurement report provided from the gNB to the UE should include at least: - gNB Rx-Tx time difference at a given granularity - FFS whether to include SRS-Resource-ID

In RAN1#106b-e meeting, some companies have discussions on configurations.

Intel (R1-2109608) - Whether to consider multiple SRS / CSI-RS / PRS configurations - Single configuration should be enough in most of the cases since RTT measurement is only performed with the serving cell. However, multi-beam FR2 scenarios may need to be taken into account, which can motivate e.g., two different configurations. Proposal 3 - For RTT-based scheme, - At most two configurations of CSI-RS can be provided to a UE for UE Rx-Tx time difference measurement - At most two configurations of SRS can be provided to a UE for gNB Rx-Tx time difference measurement - Rx-Tx time difference reporting granularity is reused from positioning

Feature lead: Agree that more than one configuration may be beneficial for FR2 scenarios. However, to enable the basic support of enhanced PDC method, as several companies mentioned that one configuration should be sufficient. Considering we don't have much time left for Rel-17, and the most important thing is to enable some enhanced PDC method in Rel-17, I would recommend to only support one configuration in Rel-17.

Nokia (R1-2109161) Figure 3 illustrates three cases of Rx-Tx measurement reporting events and its relation to UL and DL transmission events. In Case 1, the relation is clear, as the Rx-Tx measurement is provided in between two DL and UL transmission events for Rx-Tx, then no additional information than the Rx-Tx measurement value is needed assuming negligible reporting delay. However, as illustrated in Case 2 and Case 3 respectively, there could be a case where there is more SRS transmission events per CSI-RS/PRS transmission event and vice-versa. That could be due to either different periodicities of CSI-RS and PRS (and not all are used for an Rx-Tx measurement report by the gNB), or due to having multiple CSI-RS/PRS or SRS configurations configured for Rx-Tx measurements. We also note here that the UE can be configured with CSI-RS/PRS and SRS configurations that are not used for the purpose of PDC, which would magnify the issue of unclarity at the UE side unless we ensure this cannot happen from specification side. Figure 3. Illustration of Rx-Tx measurement report relation to CSI-RS and SRS transmission events. First of all, it should be clear which SRS and CSI-RS/PRS configurations are used for the purpose of PDC. So, this should be configured at the UE within the context of PDC. Secondly, we do not see a strong need for more than a single configuration of CSI-RS/PRS and SRS, as PDC essentially only needs to be done when there is a PD update and when there is a referenceTimeInfo update. It should be simple for the gNB to align the Rx-Tx based PDC event with these. Our proposal is that the CSI-RS/PRS and SRS configurations are configured in PDC context and are related. The UE may then assume that the gNB always use the latest DL RS and UL RS to generate its Rx-Tx measurement report. Observation 5: For RTT based PDC, only a single pair of CSI-RS/PRS and SRS configuration is needed. Proposal 7: For RTT based PDC, to avoid possible unclarity on which CSI-RS/PRS transmission event and SRS transmission event is used by the gNB to generate the Rx-Tx measurement, the UE may assume that the latest CSI-RS / PRS and SRS transmission event of a pair of CSI-RS/PRS and SRS configuration is used. If more configurations of SRS and/or PRS/CSI-RS are desired (for example for greater gNB flexibility), a possible unclarity can be avoided by configuring a single SRS and CSI-RS/PRS per configuration in pairs. Moreover, it will not be sufficient to use the SRS ID or CSI-RS/PRS ID in the Rx-Tx measurement report in case the same SRS or CSI-RS/PRS is configured for more than one pair, but rather add an identifier of which CSI-RS/PRS and SRS configuration pair has been used to generate the Rx-Tx measurement report (e.g. in Case 3 if two pair configurations are given, one for Option 1 and other for Option 2). Observation 6: If more than one SRS and/or CSI-RS/PRS configurations are to be supported for RTT based PDC, an SRS ID or a CSI-RS/PRS ID in the Rx-Tx measurement report is insufficient to solve a possible unclarity on the UE side on which SRS and CSI-RS/PRS transmission event has been used to generate the Rx-Tx measurement. Proposal 8: If more than one SRS and/or CSI-RS/PRS configurations are to be supported for RTT based PDC, an identifier of the SRS and CSI-RS/PRS configuration pair should be included in the Rx-Tx measurement report.

Feature lead: Similar as my comment to Intel proposal, I would recommend to only support one configuration in Rel-17, i.e. a single pair of CSI-RS/PRS and SRS configuration for PDC in Rel-17. In which case it makes sense to assume the latest CSI-RS/PRS and SRS transmission event of a pair of CSI-RS/PRS and SRS configuration is used, to avoid the ambiguity on which CSI-RS/PRS transmission event and SRS transmission event is used, and to also ensure the same DL signal and UL signal pair to obtain Rx-Tx time difference as proposed by ZTE and OPPO.

CATT (R1-2109219) It was agreed in RAN1#106-e meeting that if RTT-based propagation delay compensation is supported and performed at the UE side, the Rx-Tx measurement report provided from the gNB to the UE should include at least gNB Rx-Tx time difference at a given granularity and FFS whether to include SRS-Resource-ID. In our view, SRS-Resource-ID should be included in the Rx-Tx measurement report provided from the gNB to the UE in order to pair the SRS and TRS (PRS) for a gNB Rx-Tx time difference. In addition, it is also beneficial for mitigating the Rx/Tx timing errors. Proposal: The Rx-Tx measurement report provided from the gNB includes SRS-Resource-ID.

Feature lead: As commented to Intel and Nokia above, if only one configuration is configured specifically for PDC, then seems there is no need to include SRS-Resource-ID in the report.

ZTE (R1-2110397) In addition, it was agreed that CSI-RS for tracking or PRS can be used for Rx-Tx time difference estimation at the UE side and SRS can be used for Rx-Tx time difference estimation at the network side if RTT-based propagation delay compensation is supported. Figure 1 is an example of the Alt. 2, where the UE and the network measure the same DL signal and UL signal pair (or DL subframe and UL subframe pair). Figure 1 RTT-based propagation delay estimation However, if the network and the UE measure the different DL signal and UL signal pairs at different time, then the BS transmitting time error and the BS uplink detection error are not consistent. It may lead to that the BS transmitting time error and BS uplink detection error double for the estimated propagation delay. This is very similar as the Alt. 1 in the working assumption for TA-based solution, where the BS transmitting time error and UE downlink detection error are not consistent due to two operations at the different time. An example is shown in the Figure 2 below, where the UE measures DL signal 1 and UL signal 1 to obtain UE Rx-Tx time difference while the network measures the DL signal 2 and UL signal 2 to obtain gNB Rx-Tx time difference. It can be seen that the BS transmitting time error and BS uplink detection error are not consistent in the measurement performed at the UE and the network. Similar as the discussion in section 2.1, to avoid the inconsistency, the network and the UE should measure the same DL signal and UL signal pair to obtain its Rx-Tx time difference. In other words, the same downlink subframe and uplink subframe should be measured. We believe such condition should also be applied in Alt. 1. Figure 2 RTT-based propagation delay estimation Proposal 3: The network and the UE should measure the same DL signal and UL signal pair to obtain its Rx-Tx time difference.

Feature lead: As my comment to Nokia above, if we only support one single pair of CSI-RS/PRS and SRS configuration for PDC in Rel-17, and assume the latest CSI-RS/PRS and SRS transmission event of the pair of CSI-RS/PRS and SRS configuration to use at both gNB side and UE side, it should be able to ensure same DL signal and UL signal pair to obtain Rx-Tx time difference.

OPPO (R1-2109097) There are two options to deal with the error term of ∆: - Option-1: Keep the RTT measurements in gNB and UE as inconsistent as shown in Figure 1, and meanwhile compensate the error of ∆ into one-way propagation delay estimation, i.e., TPD=RTTMSt2-RTTBSt1-∆2. In case the delay estimation is made on UE side, UE needs to know when the gNB measures the RTT (while the gNB does not need to know when UE measures the RTT). On the other hand, this option requires UE to count the change of UL-Tx timing between the two RTT measurements, which is anyhow subject to an additional error range such as Te or Tq. Therefore Option-1 is not a viable solution to completely remove effect of ∆. - Option-2: Keep the RTT measurements in gNB and UE as consistent as shown in Figure 2, where the two RTT measurements in gNB and UE are based on the same pair of uplink subframe containing the SRS and the downlink subframe containing the PRS/CSIRS. In this option, gNB and UE need to have the common understanding on how to pick the pair of downlink subframe and uplink subframe in interest, which should be a FFS point. The one-way delay estimation in Option-2 is irrelevant to how UL-Tx timing is changed over time and therefore is not impacted by ∆. This option assumes the RTT error performances determined in RAN4 still apply to the RTT measurements shown in Figure 2. Figure 2 Consistent RTT measurements to completely remove effect of ∆ Proposal-2: If RTT-based PDC is supported, the RTT measurements in gNB and UE are based on the same pair of uplink subframe and downlink subframe that may have the different subframe indices.

Feature lead: As my comment to Nokia above, if we only support one single pair of CSI-RS/PRS and SRS configuration for PDC in Rel-17, and assume the latest CSI-RS/PRS and SRS transmission event of the pair of CSI-RS/PRS and SRS configuration to use at both gNB side and UE side, it should be able to ensure same pair of uplink subframe and downlink subframe to obtain Rx-Tx time difference.

- 4.3.1 First round discussion for issue 4.3-1

Based on the inputs and analysis above for issue 4.3-1, the following proposals are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals before providing your views here.

Proposal 4.3.1-1: For RTT-based PDC, only a single pair of CSI-RS for tracking (TRS)/PRS and SRS configuration, i.e. one CSI-RS for tracking (TRS)/PRS configuration for Rx – Tx time difference estimation at UE side and one SRS configuration for Rx – Tx time difference estimation at gNB side, is configured for PDC in Rel-17, if RTT-based PDC is supported.

| Company | View |
| --- | --- |
| OPPO | We are fine to the proposal. |
| Nokia, NSB | Support |
| vivo | We are fine with the proposal. |
| ZTE | We think a single pair of CSI-RS for tracking and SRS is sufficient, especially considering repetition can be configured. |
| Intel | OK |
| HW/HiSi | Agree |
| Qualcomm | Agree. |
| Ericsson | Agree |
| LG | Agree |
| Samsung | OK |

Feature lead: The above proposal is stable.

Proposal 4.3.1-2: For one pair of CSI-RS for tracking (TRS)/PRS and SRS configuration, UE/gNB may assume that the latest CSI-RS for tracking (TRS)/PRS and SRS transmission event is used for RTT measurement.

| Company | View |
| --- | --- |
| OPPO | We support the same pairing of DL-RS and UL-RS for RTT measurements in both gNB and UE. But how to pair may need more discussion. The questions in interests at least include: - How to define “latest”? the last event before gNB sending TA command or before gNB/UE sending RTT indication? - Due to different periodicity of TRS/PRS and SRS (SRS could be aperiodic?), the time interval between the latest TRS/PRS and the latest SRS could be large – should this be a concern for MAC-CE design (for RTT indication)? >> Feature lead 1. For RTT-based PDC, we don't need to send TA command. Then the latest here my understanding is before gNB/UE do the RTT measurement or you can say before gNB/UE sending RTT indication, I assume there won’t one more RS between starting to do the measurement and indicating the measurement results. 2. I assume appropriate configuration can avoid this kind of large gap between two latest RS if there is any problem to do this way. |
| Nokia, NSB | Support |
| vivo | We share similar view with OPPO. The further clarification is needed. |
| ZTE | As commented above, the UE does not need to perform PDC every time when TRS/PRS and SRS are transmitted. With this proposal, the UE and network should measure and record the Rx-Tx time difference when the related RS is received and transmitted. We guess it is a big burden for the UE and network. Considering it may not be useful if PDC is not performed and the effort burden, we don’t think assuming the latest CSI-RS for tracking is a good choice especially considering the measurement result deliver delay. >> Feature lead 1. Agree. This should be avoided. |
| Intel | OK |
| HW/HiSi | Since the TA command is conveyed in a PDSCH, we are wondering in case of a re-transmission, how to understand the latest event. |
| Ericsson | We don’t see how this can be enforced in general. If the CSI-RS configuration and SRS configuration are aperiodic, it may be feasible to specify something like the proposal. If CSI-RS and SRS are semi-persistent or periodic, some timing relationship should be specified, for example, the measurement is based on RS sent within window T; or the measurement report can include a timestamp. |

Feature lead: It seems companies want more time to think about this, maybe we can delay the discussion to next meeting.

Issue 4.3-2: UE/gNB Rx – Tx time difference definition for RTT-based PDC

The current definition for UE Rx-Tx time difference and gNB Rx-Tx time difference defined in TS 38.215 for positioning is shown below.

5.1.30 UE Rx – Tx time difference Definition; The UE Rx – Tx time difference is defined as TUE-RX – TUE-TX Where: TUE-RX is the UE received timing of downlink subframe #i from a Transmission Point (TP) [18], defined by the first detected path in time. TUE-TX is the UE transmit timing of uplink subframe #j that is closest in time to the subframe #i received from the TP. Multiple DL PRS resources can be used to determine the start of one subframe of the first arrival path of the TP. For frequency range 1, the reference point for TUE-RX measurement shall be the Rx antenna connector of the UE and the reference point for TUE-TX measurement shall be the Tx antenna connector of the UE. For frequency range 2, the reference point for TUE-RX measurement shall be the Rx antenna of the UE and the reference point for TUE-TX measurement shall be the Tx antenna of the UE.. Applicable for; RRC_CONNECTED. 5.2.3 gNB Rx – Tx time difference Definition; The gNB Rx – Tx time difference is defined as TgNB-RX – TgNB-TX Where: TgNB-RX is the Transmission and Reception Point (TRP) [18] received timing of uplink subframe #i containing SRS associated with UE, defined by the first detected path in time. TgNB-TX is the TRP transmit timing of downlink subframe #j that is closest in time to the subframe #i received from the UE. Multiple SRS resources for positioning can be used to determine the start of one subframe containing SRS. The reference point for TgNB-RX shall be: - for type 1-C base station TS 38.104 [9]: the Rx antenna connector, - for type 1-O or 2-O base station TS 38.104 [9]: the Rx antenna (i.e. the centre location of the radiating region of the Rx antenna), - for type 1-H base station TS 38.104 [9]: the Rx Transceiver Array Boundary connector. The reference point for TgNB-TX shall be: - for type 1-C base station TS 38.104 [9]: the Tx antenna connector, - for type 1-O or 2-O base station TS 38.104 [9]: the Tx antenna (i.e. the centre location of the radiating region of the Tx antenna), - for type 1-H base station TS 38.104 [9]: the Tx Transceiver Array Boundary connector..

In RAN1#106bis-e meeting, Ericsson propose to reuse the current definition with updates to the DL RS and UL RS description for RTT-based PDC.

Ericsson (R1-2108833) Specifically, the yellow highlight sentence for UE Rx – Tx time difference need to be updated to include TRS, and the yellow highlight sentence for UE Rx – Tx time difference need to be updated to include SRS for propagation delay compensation. In terms of the reference point for measurements, the existing definition should be used, e.g., Rx antenna connector, Tx antenna connector. The reference point cannot be baseband. For example, in the latest RAN4 discussion of reference point for Te, it was agreed to include ‘antenna’ in the Te definition (see R4-2115371). Existing definitions of UE Rx – Tx time difference and gNB Rx – Tx time difference are reused with updates to the DL RS and UL RS description. TS 38.215 V16.4.0: Definition; The UE Rx – Tx time difference is defined as TUE-RX – TUE-TX Where: TUE-RX is the UE received timing of downlink subframe #i from a Transmission Point (TP) [18], defined by the first detected path in time. TUE-TX is the UE transmit timing of uplink subframe #j that is closest in time to the subframe #i received from the TP. Multiple DL PRS resources can be used to determine the start of one subframe of the first arrival path of the TP. For frequency range 1, the reference point for TUE-RX measurement shall be the Rx antenna connector of the UE and the reference point for TUE-TX measurement shall be the Tx antenna connector of the UE. For frequency range 2, the reference point for TUE-RX measurement shall be the Rx antenna of the UE and the reference point for TUE-TX measurement shall be the Tx antenna of the UE.. Applicable for; RRC_CONNECTED. Definition; The gNB Rx – Tx time difference is defined as TgNB-RX – TgNB-TX Where: TgNB-RX is the Transmission and Reception Point (TRP) [18] received timing of uplink subframe #i containing SRS associated with UE, defined by the first detected path in time. TgNB-TX is the TRP transmit timing of downlink subframe #j that is closest in time to the subframe #i received from the UE. Multiple SRS resources for positioning can be used to determine the start of one subframe containing SRS. The reference point for TgNB-RX shall be: - for type 1-C base station TS 38.104 [9]: the Rx antenna connector, - for type 1-O or 2-O base station TS 38.104 [9]: the Rx antenna (i.e. the centre location of the radiating region of the Rx antenna), - for type 1-H base station TS 38.104 [9]: the Rx Transceiver Array Boundary connector. The reference point for TgNB-TX shall be: - for type 1-C base station TS 38.104 [9]: the Tx antenna connector, - for type 1-O or 2-O base station TS 38.104 [9]: the Tx antenna (i.e. the centre location of the radiating region of the Tx antenna), - for type 1-H base station TS 38.104 [9]: the Tx Transceiver Array Boundary connector..

Feature lead: It makes sense to reuse the current definition with necessary modification. According to the agreements achieved in RAN1#106-e, CSI-RS for tracking (TRS) can be used for Rx – Tx time difference estimation at UE side, if PRS is not configured for the UE. And SRS can also be used for RTT-based PDC. In addition, per the analysis and proposals in section 4.3.1, there will be only one configuration is configured for PDC. Therefore, proposal 4.3.2-1 in section 4.3.2 below is made accordingly for further discussion.

Agreement If RTT-based propagation delay compensation is supported, - CSI-RS for tracking (TRS) can be used for Rx – Tx time difference estimation at UE side, if PRS is not configured for the UE. - PRS can be used for Rx – Tx time difference estimation at UE side, if PRS is configured for the UE.

Agreement SRS can be used for Rx – Tx time difference estimation at gNB side for RTT-based propagation delay compensation, if RTT-based propagation delay compensation is supported.

- 4.3.2 First round discussion for issue 4.3-2

Based on the inputs and analysis above for issue 4.3-2, the following proposals are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals before providing your views here.

Proposal 4.3.2-1: For RTT-based PDC, existing definitions of UE Rx – Tx time difference (i.e. section 5.1.30 in TS 38.215) and gNB Rx – Tx time difference (i.e. section 5.2.3 in TS 38.215) are reused with updates below, if RTT-based PDC is supported.

- “Multiple DL PRS resources can be used to determine the start of one subframe of the first arrival path of the TP” is changed to “One DL PRS resource or one CSI-RS for tracking resource for propagation delay compensation can be used to determine the start of one subframe of the first arrival path of the TP”.

- “Multiple SRS resources for positioning can be used to determine the start of one subframe containing SRS” is changed to “One SRS resource for propagation delay compensation can be used to determine the start of one subframe containing SRS”.

| Company | View |
| --- | --- |
| OPPO | No, the existing definition of Rx-Tx time difference cannot be reused if Proposal 4.3.1-2 is intended for agreement. Some modifications additional to RS configuration are needed. The existing Rx-Tx time measurement definition uses the relation of “that is closest in time” in establishment of DL subframe and UL subframe in each RTT measurement. This may no longer to true if Proposal 4.3.1-2 is taken. For example, in UE-side RTT measurement based on DL PRS/TRS in DL subframe I, the existing RTT measurement use a UL subframe #j that is “closest in time to subframe i” which means the time distance between the two starting edges of DL subframe and UL subframe is closest and less than one subframe; in contrast, with Proposal 4.3.1-2, the UL subframe j has to be the one containing SRS and not necessarily the one closest in time (taking into account that PRS/TRS could be UE-common and SRS is UE-specific) – consequently, the time distance between the two starting edges of DL subframe and UL subframe can be multiple subframes. To be more specific, the existing RTT measurement is given in figure below: The new RTT measurement matching proposal 4.3.1-2 is given in figure below: This proposal is better to be discussed together with Proposal 4.3.1-1. >> Feature lead My original thinking is that “closest in time” means the closest one with the latest TRS/SRS, and I assume there is no additional ones in between. But anyway since it seems companies prefer more time to think about proposal 4.3.1-2, we can discuss this issue next meeting also. |
| Nokia, NSB | Support in principle We do not see the need to introduce this restriction in the Rx-Tx measurement definition. This limitation will be introduced in RRC when configuring the PRS or CSI-RS and SRS resource configurations. As we see it, it would be OK (no unclarity) with more then one PRS resource to determine start of a subframe. For the SRS resources, the Rx-Tx definition is measuring Rx time of the SRS transmission and hence it would best that the gNB avoids having scheduling multiple SRS resources in the same subframe from the same UE (again can also be avoided by the SRS RRC configuration). Only update needed is for support CSI-RS for tracking (TRS). |
| ZTE | We are fine to reuse the current definition. But we don’t think we need to spend time to discuss the spec updates now. It can be handled during the maintain phase. |
| Intel | In our understanding, RAN4 requirements on accuracy of these measurements are defined with the assumption of > 1 measurement occasion. If we want to reuse RAN4 work, then we have to keep current definition. So, the wording “multiple” here means averaging/other processing over multiple occasions, not over multiple RS configurations. |
| HW/HiSi | Agree |
| Ericsson | Support in principle. We also see the point that exactly how to update the RS part can wait till later. |

Feature lead: Since people want to discuss it later, it should be fine also. Leave more time for companies to think about it thoroughly.

Issue 4.3-3: UE Rx – Tx time difference and gNB Rx – Tx time difference measurements accuracies for RTT-based PDC

In RAN1#106e, the following agreement was made.

Agreement Send LS to RAN4 to ask for defining the following for RTT-based propagation delay compensation, if RTT-based propagation delay compensation is supported. - UE Rx-Tx time difference measurement accuracy errorUE,RxTxDiff based on CSI-RS for tracking - gNB Rx-Tx time difference absolute accuracyerrorgNB,RxTxDiff errorUE,RxTxDiff based on SRS

Feature lead: In theory, RAN4 will define these only if RTT-based PDC is agreed, thus the original intention is to send LS to RAN4 after RAN1 makes the decision to support RTT-based PDC. However, based on the previous discussions and the contributions submitted to RAN1#106b-e, it seems some companies have concern on RTT-based PDC with Alt.1 for error model, since there are no values ready to evaluate whether it can be able to meet the budget or not. Meanwhile some other companies assumes that the values defined for positioning in TS 38.133 can be taken as reference. From feature lead perspective, in theory I think it is fine to take the values for positioning for evaluation purpose, since it is expected that the rough range of the values should be similar regardless of being based on TRS or PRS. Of course, in the end if we agree to support RTT-based PDC, RAN4 would need to evaluate and provide more accurate values for PDC again. Therefore, I made the proposals as in section 4.3.3 for further discussion. If there is still concern to use, we may need to send LS to RAN4 to check the feasibility to use those values for evaluation before making decision whether to support RTT-based PDC.

- 4.3.3 First round discussion for issue 4.3-3

Based on the inputs and analysis above for issue 4.3-3, the following proposals are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals before providing your views here.

Proposal 4.3.3-1: For evaluation of the overall time synchronization error for RTT-based propagation delay compensation with Alt.1, it is assumed that

- The UE Rx-Tx time difference measurement accuracy based on PRS defined in Table 10.1.25.2-2 in TS 38.133 is taken as the reference for the UE Rx-Tx time difference measurement accuracy based on TRS, assuming similar configuration parameters as those of PRS (e.g., minimum bandwidth, SCS, repetition, Io range) are applied to TRS

- The gNB Rx-Tx time difference accuracy based on SRS for positioning defined in Table 13.2.2.2-1 in TS 38.133 is taken as the reference for the gNB Rx-Tx time difference accuracy based on SRS for PDC, assuming similar parameters as the SRS for positioning (e.g., SRS Ês/Iot, SRS bandwidth) are applied to SRS for PDC

| Company | View |
| --- | --- |
| OPPO | We are fine to use the mentioned RAN4 references for PRS and SRS, but would like to point out the proposal would make the overall error budget SINR-dependent, which is not the case for other PDC solutions. For TRS, is there any analysis to backup the statement of “assuming similar configuration parameters as those of PRS (e.g., minimum bandwidth, SCS, repetition, Io range) are applied to TRS”? |
| Nokia, NSB | Support |
| vivo | gNB Rx-Tx time difference accuracy defined in Table 13.2.2.2-1 is for AWGN propagation conditions. It seems gNB Rx-Tx time difference accuracy for fading channel is not provided in RAN4. |
| ZTE | We have two questions on reusing the measurement accuracy defined by RAN4. It seems the understanding is that the errorUE, DL,RX and errorBS, UL,RX have been taking into account in the measurement accuracy requirement defined by RAN4. So it means the should errorUE, DL,RX and errorBS, UL,RX at least smaller than the adopted measurement accuracy given the same RS (TRS, SRS) can be used for both RTT-based PDC and TA-based PDC. Can we use the value smaller than the measurement accuracy for errorUE, DL,RX and errorBS, UL,RX for TA-based solution if this proposal is agreed. This is related to issue 4.2-3. If the errorBS, DL,TX and errorUE, UL, TX do not need to consider, since the UE/network can know the time when the signal is transmitted at the antenna connector. We wonder whether this assumption can be reused in TA-based PDC if we agree this assumption for RTT-based PDC. For example, the network estimate the TA value and indicates the reference time based on the time when the signal is transmitted at the antenna port. Of course, we think, in this case, the reference time can only be transmitted after the reference SFN is transmitted, and maybe only carried by the dedicated RRC signaling. |
| Intel | Agree |
| HW/HiSi | Agree |
| Ericsson | Support |

Feature lead: The following is agreed from Wednesday GTW.

Agreement

For evaluation of the overall time synchronization error for RTT-based propagation delay compensation with Alt.1, it is assumed that

- The UE Rx-Tx time difference measurement accuracy based on PRS defined in Table 10.1.25.2-2 in TS 38.133 v17.3.0 is taken as the reference for the UE Rx-Tx time difference measurement accuracy

- The gNB Rx-Tx time difference accuracy based on SRS for positioning defined in Table 13.2.2.2-1 in TS 38.133 v17.3.0 is taken as the reference for the gNB Rx-Tx time difference accuracy based on SRS for PDC

Issue 4.3-4: Rx-Tx time difference measurement report signaling

In RAN1#106-e meeting, the following agreements related to Rx-Tx measurement report was achieved:

Agreement If RTT-based propagation delay compensation is supported and performed at the UE side, the Rx-Tx measurement report provided from the gNB to the UE should include at least: - gNB Rx-Tx time difference at a given granularity - FFS whether to include SRS-Resource-ID

Intel provides some discussion on this:

Intel (R1-2109608) In summary, at this point there seems no evidence to adopt only one of the options, and we expect RAN1 to work on specification of the following features for the most flexible operation: - Component 1: Support of gNB-based pre-compensation and/or UE-based compensation transparent to propagation delay calculation scheme. - In case of pre-compensation, the reference time information is modified by gNB to include the necessary adjustment and to indicate to the UE that no other compensation is required. - In case of UE-based compensation, the reference time information is not modified by gNB, and gNB indicates to the UE that UE-side compensation is required. - Component 2: Measurement and indication to gNB of the UE Rx-Tx time difference based on DL signals provided outside of LPP. - Component 3: Measurement and indication to UE of the gNB Rx-Tx time difference based on UL signals provided outside of LPP. - For reporting, L1 or L2/L3 group-common signaling may be employed to avoid excessive overhead from unicast messages to UEs with similar PD. Once all components are defined, then network can have freedom to implement one of the suitable options, e.g., RTT-based pre-compensation at the gNB, non RTT-based pre-compensation at the gNB, RTT-based compensation at the UE, non RTT-based compensation at the UE, etc. As per agreements, the decision on whether to support UE-based compensation or gNB-based pre-compensation is left to RAN2. Taking this into account, the following proposal is formulated: Proposal 1 - RAN1 to agree to specify the following components to enable flexible propagation delay compensation scheme: - Component 1: Support of gNB-based pre-compensation and UE-based compensation (up to RAN2) transparent to propagation delay calculation scheme; - Component 2: Non-LPP measurement and indication to gNB of the UE Rx-Tx time difference based on DL signals; - Component 3: Non-LPP measurement and indication to UE of the gNB Rx-Tx time difference based on UL signals. Proposal 2 - For RTT-based UE side compensation, the gNB Rx-Tx time difference measurement is indicated to UE(s) using L1 group-common DCI signaling, - FFS details

Feature lead: According to agreement achieved in RAN2#115e as below, it seems RAN2 wants to have input from RAN1 on the measurement framework for gNB based RTT-based PDC also, before they make decision on whether to support UE-side PDC or gNB based pre-compensation. Therefore, a few proposes and questions are set accordingly for further discussion.

Agreements 1. RAN2 assumes that gNB can perform pre-compensation. RAN2 agrees to introduce ignaling to enable/disable UE-side PDC. 2. The gNB can enable/disable UE-side PDC via unicast-RRC ignaling for Rel-17 3. RAN2 shall wait for RAN1 to decide the measurement framework for RTT based PDC method and does not preclude UE-side PDC or gNB based pre-compensation at this point. RAN2 is expecting guidance from RAN1 on what is needed. 4. UE Assistance information from the UE which could for example be used by gNB to activate PDC is not supported 5. Implicit activation of UE-side PDC when a pre-configured threshold is met is not supported 6. UE-based trigger for TA update or RACH procedure for PDC are deprioritized for Release 17

- 4.3.4 First round discussion for issue 4.3-4

Based on the inputs and analysis above for issue 4.3-4, the following proposals are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals before providing your views here.

Proposal 4.3.4-1: If RTT-based propagation delay compensation is supported and performed at the gNB side, the Rx-Tx measurement report provided from the UE to the gNB should include at least:

- UE Rx-Tx time difference at a given granularity

| Company | View |
| --- | --- |
| OPPO | We are fine with the proposal. |
| Nokia, NSB | Agree |
| vivo | Agree. |
| ZTE | We are fine with this proposal. |
| Intel | Agree |
| HW/HiSi | Agree |
| Qualcomm | Agree |
| Ericsson | Agree |

Feature lead: Proposal 4.3.4-1 is stable.

Proposal 4.3.4-2: If RTT-based propagation delay compensation is supported, the Rx-Tx time difference is reported with similar granularity for Rx-Tx time difference reporting for positioning.

| Company | View |
| --- | --- |
| OPPO | The proposal is vague in using word of “similar granularity”. As we mentioned before, the RTT indication granularity should be determined in RAN4. What RAN1 may take interest is: whether the Rx-Tx time difference report should use MAC-CE or RRC? This should be decided first before talking about granularity. >> Feature lead Originally “similar” is used to leave some room to discuss whether any modification needed since positioning is reporting to LMF. But I agree it is vague. Let’s update. In addition, as to the way to report, the straightforward way is to do RRC I think. But let’s hear more views from companies. |
| Nokia, NSB | Agree We could even make it simpler than for Positioning and settle on a single granularity. >> Feature lead We can consider that. But in general if only one granularity in some cases it may result in unnecessary signaling overhead. Let’s discuss more. |
| vivo | Agree. |
| ZTE | We are fine with this proposal. |
| Intel | Agree, except the imprecise wording “similar”. Fix to one granularity is also OK. |
| HW/HiSi | Agree |
| Qualcomm | Agree |
| Ericsson | Agree in principle. We also see OPPO’s point about “similar granularity”. To make it clear, suggest changing to “same granularity”. Also add “corresponding” so that gNB RxTxTimeDiff (if reported) reuses that of gNB RxTxTimeDiff in positioning. The same for UE RxTxTimeDiff if RAN2 decides to have UE report to gNB. If RTT-based propagation delay compensation is supported, the Rx-Tx time difference is reported with same granularity as the corresponding Rx-Tx time difference reporting for positioning. >> Feature lead Your suggestion is ok. Some companies want to leave it to one granularity, thus I will add one FFS for further discussion also. |

- 4.3.4a Second round discussion for issue 4.3-4

Based on the inputs and analysis above for issue 4.3-4 in section 4.3.4, the following proposals are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals before providing your views here.

Revised Proposal 4.3.4-2: If RTT-based propagation delay compensation is supported, the Rx-Tx time difference is reported with same granularity as the corresponding Rx-Tx time difference reporting for positioning.

- FFS whether to restrict to one of the granularities for positioning

Revised Proposal 4.3.4-2: If RTT-based propagation delay compensation is supported, the Rx-Tx time difference is reported with granularity 2k*Tc, where k is an integer.

- FFS the value of k, e.g. 0<=k<=5 as that for positioning;

- FFS the reporting range of Rx-Tx time difference measurement for PDC

| Company | View |
| --- | --- |
| Intel | Agree |
| OPPO | The proposal is a bit vague as possibly being understood as that the UE reporting the Rx-Tx time difference for PDC purpose may firstly look at what granularity is used in positioning protocol. It also has a potential consequence that, if FFS point fails, the same full set of granularity in positioning protocol is to be cloned. >> Feature lead Thanks. Actually there is no issue to support all granularity from positioning also, which actually may have some benefits as I explained in the previous section. We can make it clearer as E// suggested below though. Given there is at least one company prefer to adopt single granularity and no companies explicitly proposed multiple candidates, maybe the proposal can be re-formulated as: Revised Proposal 4.3.4-2: If RTT-based propagation delay compensation is supported, the Rx-Tx time difference is reported with same granularity value as specified in the corresponding Rx-Tx time difference reporting for positioning. - The dependency between the granularity for RTT-based PDC propagation delay compensation and the granularity for positioning, in case UE is configured to perform both, is not supported. - FFS whether to support more than one granularity for the Rx-Tx time difference report. >> Feature lead The point is that for positioning they support multiple granularities, then for PDC whether we will also support all those values, or just pick one of them. I think nothing related to dependency of positioning. We still think the final decision of the granularity should be done in RAN4, together with the reporting range of Rx-Tx time difference. But it does not harm for RAN1 to determine the candidates for RAN4 to further work on. >> Feature lead The granularity for positioning is actually decided by RAN1, similar I think ok for RAN1 to conclude this kind of issue also. |
| Ericsson | Support in principle. We understand the goal of the purpose is, no new granularity needs to be created for PDC. Thus, we agree with the clarification points by OPPO. In this case, the proposal can be revised to the following. That is, the granularity is described directly without mentioning measurement for positioning. Thus no need to clarify that PDC is independent from positioning etc. Revised Proposal 4.3.4-2: If RTT-based propagation delay compensation is supported, the Rx-Tx time difference is reported with granularity 2k´Tc , where k is an integer and 0<=k<=5. >> Feature lead Some companies prefer to only take of the k values I think. I did some adjustment above. Moreover, the following can be added: - The reporting range of Rx-Tx time difference measurement for PDC is FFS. We expect that PDC does not need the full range of -985024Tc to 985024Tc used for positioning. The PDC measurement is for the serving cell only, whereas positioning involves measurement of neighbor cells. This then saves some signaling bit width. An LS to RAN4 can be sent to ask for the reporting range. - If and how to signal k for the granularity is FFS. If only one granularity is needed, then k can be specified without any signaling for it. If multiple k values are needed, then there is a need to signal it. >> Feature lead I think it is reasonable to add the first FFS. For the second one, once we have make decision on whether to support more than one k, we can further discuss. |
| Nokia, NSB | Support |
| ZTE | We have the same question with OPPO. We think the granularity can be configured separately. We are fine with the revised proposal. |
| Hw/HiSi | Support |
| Qualcomm | Support |

Summary of the status for revised proposal 4.3.4-2 in second round

| Support | Intel, Nokia, NSB, ZTE, Huawei, HiSilicon, Qualcomm |
| --- | --- |
| Not support | OPPO, vivo |

FL recommendation: Still different views, let’s further discuss this issue.

Proposal 4.3.4a-2: If RTT-based propagation delay compensation is supported, the Rx-Tx time difference is reported via RRC signaling.

| Company | View |
| --- | --- |
| Intel | This proposal needs to consider two cases, being discussed in RAN1 and RAN2: Case 1: gNB side pre-compensation based on RTT. For this case, UE needs to report UE Rx-Tx time difference measurement to the gNB, and RRC framework can be naturally reused (same as other UE measurements info) >> Feature lead Yes. Case 2: UE side compensation based on RTT. For this case, a gNB needs to signal gNB Rx-Tx time difference measurement to the UE. There is no evident reference to implement the measurement report from gNB to UE. As we analyzed in the tdoc, the design could be based on L1 signaling (we prefer group-common DCI), MAC, or RRC. This requires discussion, in our view. >> Feature lead Yes the intention here is to apply to both gNB-side PDC and UE-side PDC. Let me hear more views from other companies first if they prefer to further discuss for UE-side PDC. |
| OPPO | We are fine with proposal. To our understanding, the proposal itself applies to both gNB-to-UE report and UE-to-gNB report (if gNB pre-compensation is supported). We do not think the gNB Rx-Tx time difference should be reported in L1 signaling, especially group-common DCI, because the UL-Rx timing constructing the gNB Rx-Tx time difference is UE-specific. |
| Nokia NSB | Disagree We see this as a RAN2 issue how the measurement is reported, while RAN1 just need to design the measurement report content. Therefore, there is no need for RAN1 to define the way the measurement is reported / signaled. >> Feature lead If it can be agreed in RAN1, it should be good also for the overall finish of Rel-17? In addition, I think RAN1 discuss the way for a signaling often also, like we always discuss whether it is RRC or L1 signaling. |
| ZTE | We are fine with this proposal. RRC signaling is adequate. |
| HW/HiSi | Support the proposal |
| Qualcomm | Disagree. First of all, it may not work for UE side compensation. Secondly, it is more related to RAN2 scope. It is not suitable for RAN1 to define the upper layer procedure. >> Feature lead Even for UE side PDC, I think it should work. Can you clarify why it doesn’t work? As replied to Nokia, if we can agree it in RAN1, I think it is good also. In addition, RAN1 can also discuss whether it is RRC signaling or MAC signaling, of course the detailed design is up to RAN2. |
| Ericsson | Support the proposal. |

Summary of the status for revised proposal 4.3.4a-2 in second round

| Support | OPPO, ZTE, Huawei, HiSilicon, Ericsson |
| --- | --- |
| Not support | Intel, Nokia, NSB, Qualcomm |

FL recommendation: Still different views, let’s further discuss this issue.

- 4.3.4b Third round discussion for issue 4.3-4

Based on the inputs and analysis above for issue 4.3-4 in section 4.3.4a, the following proposals are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals before providing your views here.

Revised Proposal 4.3.4-2: If RTT-based propagation delay compensation is supported, the Rx-Tx time difference is reported with granularity 2k*Tc, where k is an integer.

- FFS the value of k, e.g. 0<=k<=5 as that for positioning;

- FFS the reporting range of Rx-Tx time difference measurement for PDC

| Company | View |
| --- | --- |
| Feature lead | @ OPPO @ Ericsson Please check my replies to your comments in section 4.2.2a, and see if you have further comments. |
| Nokia/NSB | Support As noted in our previous reply, we could maybe fix the value of k in the specifications for the T-sync operation (i.e. choose one of 0..5). |
| Intel | Support Fixing k to a single value may be discussed a bit later. Also, the single value is more suitable for L1/MAC CE indication while more values can be easily supported by RRC. |
| OPPO | Agree with Nokia for fixing on a single value by spec. In addition, unless RAN1 wants to make k>5 and to make some evaluations, it is better not to keep range of k as example. So our suggestion: Revised Proposal 4.3.4-2: If RTT-based propagation delay compensation is supported, the Rx-Tx time difference is reported with granularity 2k*Tc, where k is an integer satisfying 0<=k<=5. - Spec defines only one granularity for gNB Rx-Tx time difference report, and only one granularity for UE Rx-Tx time difference report (if supported) - FFS the value of k, e.g. 0<=k<=5 as that for positioning; - FFS the reporting range of Rx-Tx time difference measurement for PDC >> Feature lead Thanks for the good suggestion. As you could see from the Ericsson comment below, we would need more discussion on the first sub-bullet. |
| Ericsson | Support. Revised proposal by OPPO is fine in principle: - It is useful to move 0<=k<=5 to main sentence, i.e., out of FFS. - Regarding one granularity: this is acceptable for a given configuration (SCS, FR1 vs FR2, RS bandwidth etc), but not a single granularity for all cases. Thus, this sub-bullet can be deleted for now, or adding “• Spec defines only one granularity for gNB Rx-Tx time difference report, and only one granularity for UE Rx-Tx time difference report (if supported) for a given set of configuration including, e.g., SCS, FR1 vs FR2, RS bandwidth” |
| Qualcomm | Support the proposal with a fixed k for each scenario. |
| Samsung | Support. Can live with OPPO’s version. |
| ZTE | Regarding the value and the number of k, we prefer to keep it in the FFS. We prefer multiple granularity to provide the better flexibility and forward compatibility. In this case, the network can configure different value of k for the measurement report to cater for different scenario with different requirements as in positioning. So, at this stage, we think the details should be FFS and can only accept the proposal from FL. |
| HW/HiSi | Support, also ok to accept the revised version of Oppo. |
| LG | Support the current proposal. Regarding the value of k, we would like to keep FFS for now. |

Summary of the status for revised proposal 4.3.4-2 in third round

| Support | Intel, Nokia, NSB, ZTE, Huawei, HiSilicon, Qualcomm, Ericsson, Samsung, LG |
| --- | --- |
| Not support | OPPO |

FL recommendation: Still different views, let’s further discuss this issue.

Proposal 4.3.4a-2: If RTT-based propagation delay compensation is supported, the Rx-Tx time difference is reported via RRC signaling.

| Company | View |
| --- | --- |
| Feature lead | @ all Please check my replies to your comments in section 4.2.2a, and see if you have further comments. In addition, some companies prefer to leave it to RAN2. For companies who support this proposal, please also indicate whether you are ok to leave it to RAN2. |
| Nokia/NSB | Do not support. As mentioned already in the 2nd round, the signaling should be up to RAN2. |
| Intel | We suggest splitting the proposal for gNB-side pre-compensation and UE-side compensation. For gNB-side pre-compensation, we agree with RRC signaling. Leaving up to RAN2 is also OK. For UE-side compensation, it is important to consider signaling overhead. We think that group-common DCI-based indication is the only option which offers some signaling overhead saving, when multiple UEs have similar PD to gNB within the accuracy requirement for compensation (tens of meters, that may be typical in Indoor scenarios). If we leave this discussion to RAN2, then they may not consider L1 indication at all. |
| OPPO | Support. Whether to use RRC, MAC-CE or even L1 DCI is traditionally a decision in RAN1. |

Summary of the status for revised proposal 4.3.4a-2 in third round

| Support | OPPO, ZTE, LG |
| --- | --- |
| Not support | Intel, Nokia, NSB |

Feature lead: Based on the current situation, it seems still controversial. Maybe we can discuss further next meeting.

- 4.3.4c 4th round discussion for issue 4.3-4

Based on the inputs and analysis above for issue 4.3-4 in section 4.3.4b, the following proposals are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals before providing your views here.

Revised Proposal 4.3.4-2: If RTT-based propagation delay compensation is supported, the Rx-Tx time difference is reported with granularity 2k*Tc, where k is an integer satisfying 0<=k<=5.

- FFS the value of k

- FFS the reporting range of Rx-Tx time difference measurement for PDC

| Company | View |
| --- | --- |
| Feature lead | @ OPPO Please check my replies to your comments in section 4.2.2b, and see if you accept it. For whether |
| Nokia, NSB | Support. |
| Intel | Support |
| HW/HiSi | Support |
| OPPO | For sake of progress, we can fine to the FL proposal. We would like to encourage the group to go a step further to agree defining a single granularity that is transparent to SCS, which has been the way in positioning and Rel-16 IAB. The variation from k=0 to k=5 gives only about no more than 4ns difference in total error. |
| Ericsson | Support |

Issue 4.3-5: Equation to calculate the overall time synchronization error over Uu interface for RTT-based PDC

In RAN1#106-e meeting, two alternatives were agreed for the total error equation for RTT-based PDC, and RAN1 to select one of the alternatives in RAN1#106bis-e.

Agreement Take the following two alternatives as the equation for evaluation of the overall time synchronization error for RTT-based propagation delay compensation. RAN1 to select one of the alternatives in RAN1#106bis-e. - Alt. 1: - errorRxTxDiff, indication is to reflect the error due to indication granularity of Rx-Tx time difference - errorgNB,RxTxDiff and errorUE, RxTxDiff reflects the measurement inaccuracy of gNB Rx-Tx time difference, and the measurement inaccuracy of UE Rx-Tx time difference, respectively. - Note: The equation may be updated after clarification on the gNB TX-RX timing difference and UE TX-RX timing difference - Alt. 2: - errorRxTxDiff,indication is to reflect the error due to indication granularity of Rx-Tx time difference - Note: Alt.2 assumes that gNB can coordinate the time of TA procedure and the time of PD compensation, so that the DL frame timing error and BS transmit timing error for propagation delay estimation is correlated to (e.g. the same as) that for the transmission of RRC signaling carrying the reference time clock Note: FFS whether / how to handle inconsistent RTT measurement in gNB and UE due a change of uplink TX timing

Inconsistent RTT measurement in gNB and UE for RTT-based PDC

In RAN1#106-e meeting, there is an FFS whether / how to handle inconsistent RTT measurement in gNB and UE due a change of uplink TX timing below.

In RAN1#106bis-e meeting, some companies show views for this below.

OPPO (R1-2109097) Figure 1 RTT-based one-way propagation delay estimation with inconsistent RTT measurements Figure 1 shows the current principle of RTT-based one-way propagation delay estimation. Assume the delay estimation is performed on UE side. Because the RTT measurement on gNB side and RTT measurement on UE side are not coordinated with each other and the UE does not even know when gNB measures its RTT, there is a chance for UL-Tx timing to be changed, due to either TA command from gNB or autonomous adjustment by UE itself, between the RTT measurement on gNB side and RTT measurement on UE side. Assume such UL-Tx timing change results in a RTT interval change on UE side, from RTTMSt1=x to RTTMSt2=x+∆, as shown in Figure 1. Then the one-way propagation delay is estimated as following, including an error term of ∆: TPD=RTTMSt2-RTTBSt12=x+∆-x-2TPD2=TPD+∆2 Therefore, the overall error of RTT-based PDC is: errorRTT, total=errorDLTX+errorDLRX+errorRTT,gNB+errorRTT,UE+errorRTT,indication+∆2 where: - errorDLTX is the DL-Tx timing error. RAN1 agrees to assume it as 65ns. - errorDLRX is the DL-Rx timing detection error. RAN1 agrees to assume it as 100ns. - errorRTT,gNB is the RTT measurement error on gNB side. RAN4 allows it being as large as 117Tc or 59.5ns, under specific assumptions of SINR and PRS bandwidth. - errorRTT,UE is the RTT measurement error on UE side. RAN4 allows it being as large as 180Tc or 91.6ns, under specific assumptions of SINR and PRS bandwidth. - errorRTT,indication is the RTT indication error. The value is supposed to be determined by RAN4. If the granularity in IAB Tdelta MAC-CE is reused, it is 32Tc or 16.3ns. - ∆ is the accumulated change of UL-Tx timing relative to DL-Rx timing in UE. For a change caused by TA command, ∆ can be as small as TA granularity, i.e., 16*64Tc or 520.8ns; for a change caused by UE autonomous change of UL-Tx timing, ∆ can be as large as Tq=5.5×64Tc or 179ns, which is defined in 38.133. It can be seem the contribution from ∆ ranks on the top among all error components. The total RTT-based PDC error is 248.7ns if error term of ∆ can be avoided; otherwise, the total RTT-based PDC error could be 345ns if only UE autonomous UL-Tx timing change is taken into account. If the error term ∆ can be generated by TA command, the total RTT-based PDC error could be 509ns. Observation-3: In order to make RTT-based PDC to meet RAN2 Uu interface error budget, the error term of ∆ caused by change of UL-Tx timing between two RTT measurements in gNB and UE has to be avoided.

Nokia (R1-2109161) Proposal 6: RAN1 concludes that there is no additional error component to be added in the RTT-based PDC caused by a change in the uplink TX timing.

Huawei (R1-2109743) In the RAN1#106-e agreements, there is an FFS whether / how to handle inconsistent RTT measurement in gNB and UE due to a change of the uplink TX timing. We noticed that in TS38.133 it is defined that the UE Rx-Tx time difference measurement accuracy is not applied if the uplink transmission timing changes during the UE Rx-Tx measurement period due to the network-configured Timing Advance, and FFS whether the accuracy still apply if the uplink transmission timing changes due to the autonomous timing adjustment. The corresponding description in TS 38.133 is copied below. From the gNB perspective, the gNB should not update Timing Advance during the UE Rx-Tx measurement period for RTT-based PDC. Similar at the UE side, UE may be able to avoid the autonomous timing adjustment also if it matters. Thus, the inconsistent measurement can be avoided by gNB/UE’s implementation. TS38.133-g80 - 10.1.25.2 Measurement Accuracy Requirements The UE Rx-Tx time difference measurement accuracy requirements in this clause shall not apply, if: NTA_offset defined in Table 7.1.2-2 changes during the UE Rx-Tx measurement period or if the uplink transmission timing changes during the UE Rx-Tx measurement period due to the network-configured Timing Advance. FFS: whether UE Rx-Tx time difference measurement accuracy requirements in this clause shall also apply if the uplink transmission timing changes during the UE Rx-Tx measurement period due to the autonomous timing adjustment defined in clause 7.1.2.. Observation 3: It can be gNB/UE’s implementation to handle inconsistent RTT measurement in gNB and UE due to a change of uplink TX timing for RTT-based PDC if supported in Rel-17.

Intel (R1-2109068) - Whether / how to handle inconsistent RTT measurement in gNB and UE due to change of uplink TX timing - It seems a baseline to assume that the change in uplink TX timing is not expected when RTT procedure is being performed - Alternatively, gNB may request a UE to defer TX timing adjustments until RTT measurement procedure is completed

Feature lead: It seems companies think either there is no inconsistent RTT measurement issue, or if the inconsistent exist it should be avoided by some other way. In addition, as discussed in section 4.3.1, if the proposals in section 4.3.1 is agreed, it seems the inconsistent RTT measurement issue is not there or can be avoided. Therefore, I made a potential conclusion 4.3.5-1 as in section 4.3.5 for further discussion.

Clarifications for Alt. 1

ZTE proposes to discuss whether UE downlink detection error and/or UE transmitting time error have been taken into account when defining the UE Rx-Tx time difference measurement accuracy and whether BS transmitting time error and/or BS uplink detection error have been taken into account when defining gNB Rx-Tx time difference measurement accuracy before down-select. If no consensus, then an LS can be sent to RAN4.

ZTE (R1-2110397) Proposal 4: RAN1 should discuss and achieve a consensus on the details of gNB/UE Rx-Tx time difference measurement accuracy before further adopting Alt. 1 for evaluation. Proposal 5: If RAN1 cannot make a consensus, an LS can be sent to RAN4 for clarification on the Rx-Tx time difference measurement accuracy with the following questions. - Question 1: Whether UE downlink detection error and/or UE transmitting time error have been taken into account when defining the UE Rx-Tx time difference measurement accuracy and whether BS transmitting time error and/or BS uplink detection error have been taken into account when defining gNB Rx-Tx time difference measurement accuracy? - Question 2: Is there any other error components needed to be considered in addition to errorgNB, RxTxDiff, errorUE, RxTxDiff, error RxTxDiff,indication, if gNB Rx-Tx time difference and UE Rx-Tx time difference are used to calculate the DL propagation delay with the assumption of the same DL and UL propagation delay, where error RxTxDiff,indication is to reflect the error due to indication granularity of Rx-Tx time difference, errorgNB, RxTxDiff and errorUE, RxTxDiff reflects the measurement inaccuracy of gNB Rx-Tx time difference, and the measurement inaccuracy of UE Rx-Tx time difference, respectively.

Feature lead：This issue was discussed in RAN1#106-e meeting also. Firstly, Rx – Tx time difference is defined by the first detected path as given in TS 38.215, therefore it is expected that the measurement inaccuracy defined in TS 38.133 should have taken errorUE, DL,RX  and errorBS, UL,RX into account, respectively. Secondly, according to the definition of Rx – Tx time difference as given in TS 38.215, the reference point for transmit measurement is antenna connector, thus in this case it seems errorBS, DL,TX and errorUE, UL, TX don’t need to be considered. However, as defined in TS 38.133 section 13.2.2.1 for gNB Rx-Tx time difference measurement and section 10.1.25.2 for UE Rx-Tx time difference measurement, it can be expected that if these error components really need to be considered, it can be reflected by the margin Y for gNB Rx-Tx time different measurement and the margin   for UE Rx-Tx time difference measurement. Note that Y is very likely reported by companies and it seems RAN4 is discussing the value for . Potential conclusion 4.3.5-2 in section 4.3.5 is made for further discussion.

TS 38.133 section 13.2.2.2 #### 13.2.2.2 Requirements The accuracy requirements for gNB Rx-Tx time difference measurement shall be within ±(X+Y) Tc under the following conditions: - AWGN propagation conditions. - The measured signals are in the directions covered by RoAoA of OTA reference sensitivity requirements for gNB type 1-O and 2-O BS where - X is defined in Table 13.2.2.2-1 for gNB types 1-C, 1-H and 1-O and in Table 13.2.2.2-2 for gNB type 2-O. - Y is declared by manufacturer and can be different for different gNB types 1-C, 1-H, 1-O and 2-O. Note: The measurement accuracy requirements in Table 13.2.2.2-1 and Table 13.2.2.2-2 are defined under an assumption that gNB is not mandated to perform receive beam sweeping.

TS 38.133 section 10.1.25.2 - 10.1.25.2 Measurement Accuracy Requirements The UE Rx-Tx time difference measurement accuracy requirements in this clause shall not apply, if: NTA_offset defined in Table 7.1.2-2 changes during the UE Rx-Tx measurement period or if the uplink transmission timing changes during the UE Rx-Tx measurement period due to the network-configured Timing Advance. FFS: whether UE Rx-Tx time difference measurement accuracy requirements in this clause shall also apply if the uplink transmission timing changes during the UE Rx-Tx measurement period due to the autonomous timing adjustment defined in clause 7.1.2. The UE shall continue and complete a UE Rx-Tx measurement while meeting UE Rx-Tx measurement accuracy requirements defined in this clause when a serving cell change occurs during the UE Rx-Tx measurement provided that the serving cell change does not impact the SRS configuration for the UE Rx-Tx measurement. Note: The requriements for fading channel in this clause are derived based on TDL-A (30 ns delay spread, 5Hz) and TDL-C (60 ns delay spread, 300 Hz) channel models for FR1 and FR2 respectively. Editor’s note: In accuracy tables  is margin and is FFS The accuracy requirements in Table 10.1.25.2-1 for FR1 are valid under the following conditions: Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled. PRP¦dBm according to Annex B.2.x for a corresponding Band. AWGN propagation condition.

Clarifications for Alt. 2

Feature lead: From the agreements, it can be seen that two parts of Alt.2 is still in brackets. Based on the views in the contribution and also the previous discussions, it is not clear to me how to achieve consensus on the brackets, the views are still very divergent. As to “[12*]”, I think it is related to the proposal in section 2.1, for further discussion I would make the proposal with the removal of the whole “[12*]” to align the proposal in section 2.1. As to [errorUE, UL, TX], it is obvious that still there are diverse views. Considering that for Alt.1, we leave the room for companies to consider errorUE, UL, TX by the margin  , probably here we can compromise to keep errorUE, UL, TX also, and it can be combined with errorUE, DL,RX to go for Te, which means whatever Te is enhanced for TA-based PDC, the corresponding Te can be used here for Alt.2 for RTT-based PDC. Note that it is expected that Alt.2 would provide better performance since it is assumed co-related error components, similar as Alt.2 for TA-based PDC. I made proposal 4.3.5-1 in section 4.3.5 for further discussion.

Down-selection between Alt.1 and Alt.2

Based on inputs from companies in RAN1#106bis meeting, the company position on Alt.1 and Alt.2 are as below. Note that some companies didn’t provide very clear view here, thus I didn’t include them yet.

- Alt. 1:

  - errorRxTxDiff, indication is to reflect the error due to indication granularity of Rx-Tx time difference

  - errorgNB,RxTxDiff and errorUE, RxTxDiff reflects the measurement inaccuracy of gNB Rx-Tx time difference, and the measurement inaccuracy of UE Rx-Tx time difference, respectively.

  - Note: The equation may be updated after clarification on the gNB TX-RX timing difference and UE TX-RX timing difference

  - Support: Ericsson, Huawei, HiSilicon, Qualcomm, OPPO, vivo

- Alt. 2:

  - errorRxTxDiff,indication is to reflect the error due to indication granularity of Rx-Tx time difference

  - Note: Alt.2 assumes that gNB can coordinate the time of TA procedure and the time of PD compensation, so that the DL frame timing error and BS transmit timing error for propagation delay estimation is correlated to (e.g. the same as) that for the transmission of RRC signaling carrying the reference time clock

  - Support:

    - Nokia, NSB: Remove the bracket for [12*] (i.e. keep 12*) and delete the whole “[errorUE, UL, TX]”

    - CATT: Delete the whole “[12*]” and remove the bracket for [errorUE, UL, TX] (i.e. keep errorUE, UL, TX)

    - ZTE, LG: Remove the bracket for [12*] (i.e. keep 12*) and remove the bracket for [errorUE, UL, TX] (i.e. keep errorUE, UL, TX)

Feature lead: Based on the situation, it is not clear to me how to do the downs-selection. If we really need to do the down-selection, the only way I can suggest is to go Alt.1, since it has a little bit more companies supporting it and also no controversial brackets for Alt.1. However, let’s hear more views from companies first.

- 4.3.5 First round discussion for issue 4.3-5

Based on the inputs and analysis above for issue 4.3-5, the following proposals are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals before providing your views here.

Inconsistent RTT measurement in gNB and UE for RTT-based PDC

Proposed conclusion 4.3.5-1: there is no additional error component to be added in the RTT-based PDC caused by a change in the uplink TX timing.

| Company | View |
| --- | --- |
| OPPO | We are fine to the proposed conclusion, only if the proposal 4.3.1-2 and our comment to proposal 4.3.2-1 are addressed, i.e., the same pair of subframes containing the DL-RS and UL-RS is used by gNB and UE to measure RTT. >> Feature lead Let’s see the conclusion there first then. |
| Nokia, NSB | Support |
| vivo | Support |
| ZTE | If we can ensure the network and the UE can measure the same signal, there is no need to consider the error component caused by the change of UL Tx timing. |
| Intel | Support |
| HW/HiSi | Agree |
| Qualcomm | Agree |
| Ericsson | Agree |

Feature lead: The above proposed conclusion 4.3.5-1 is stable.

Clarifications for Alt. 1

Potential conclusion 4.3.5-2: For Alt.1 for evaluation of the overall time synchronization error for RTT-based propagation delay compensation, the following is assumed for DL propagation delay estimation error:

  - BS uplink detection error errorBS, UL,RX is reflected by errorgNB,RxTxDiff, and BS transmitting time error errorBS, DL,TX  (if needs to be considered) can be reflected by the margin Y for gNB Rx-Tx time difference measurement, where Y is declared by manufacturer;

  - UE downlink detection error errorUE, DL,RX is reflected by errorUE, RxTxDiff, and UE transmitting time error errorUE, UL, TX (if needs to be considered) can be reflected by the margin  for UE Rx-Tx time difference measurement, where the definition of   is up to RAN4.

| Company | View |
| --- | --- |
| OPPO | Is the proposal for Alt-1 only or trying to harmonize Alt-1 and Alt-2? If it is for Alt-1 only (as said in main proposal), does it mean the Alt-1 formula should use errorgNB,RxTxDiff+Y to replace errorgNB,RxTxDiff and use errorUE, RxTxDiff+δ to replace errorUE, RxTxDiff? Please clarify. In addition, the proposal seems doing something not fully endorsed by RAN4: there is an editor line in 38.133 vg80 saying “Editor’s note: In accuracy tables  is margin and is FFS”, it does not say this margin is fully dedicated to errorUE, UL, TX. Is this a different  from the one that is used in 38.133? >> Feature lead Yes for Alt.1 only. We don't need to replace the equation, but when taking the values that margin can be added if any on top of the values defined in the table. I think that should be the common understanding. It is the same  as under discussion in RAN4. The intention is not to say it is dedicated to errorUE, UL, TX, just say if errorUE, UL, TX needs to be considered then should be included in  already. I assume RAN4 would consider all possible factors and provide . No matter whether there is any other factors that RAN4 will include, whatever value RAN4 provide we need to use, since it is part of the measurement inaccuracy. |
| Nokia, NSB | Support |
| vivo | It seems errorBS,RxTx diff for fading channel is not provided in RAN4. Multipath detection error should be further included for budget calculation. We think the value for errorBS,RxTx diff for fading channel should be determined firstly. >> Feature lead For comparison perspective, I think ok to use values comes from AWGN channel. |
| ZTE | This is about the understanding of RAN4 spec. We are not sure RAN1 can achieve a consensus on this, for example, BS uplink detection error errorBS, UL,RX is reflected by errorgNB,RxTxDiff, which is quite similar as the question that whether UE downlink detection error is included in Te. We can ask RAN4 for clarification on this. >> Feature lead For detection error, I think it should be obvious based on the definition of Rx-Tx time difference. I would like to ask RAN4 too many questions, otherwise they may not be able to reply any of the questions to us in next RAN1 meeting.  |
| Intel | Support |
| HW/HiSi | Ok |
| Qualcomm | Agree |
| Ericsson | We are not sure what this trying to achieve. The Alt 1 formula is only for RAN1 to carry out the timing error estimation, so that RAN1 can proceed with method selection and related design work. RAN1 already made assumptions about what values to use for errorBS, UL,RX and errorUE, DL,RX. For actual implementation requirement, RAN4 can discuss if there is a need. >> Feature lead We discuss this because companies still have questions for clarification for Alt.1, and we have note in the agreement achieved in RAN1#106-e to clarify this. So if the clarifications are not solved, companies may question the results with Alt.1. |

Clarifications for Alt. 2

Proposal 4.3.5-1: For Alt.2 for evaluation of the overall time synchronization error for RTT-based propagation delay compensation, it is revised as below:

- Alt. 2:

errortotal,RTTbased≤

[12*]errorBS, DL, TX+12errorUE,DL,RX+errorBS,UL,RX+[errorUE,UL,TX]+errorRxTxDiff_indication

=  errorBS, DL, TX+12(errorBS, UL, RX+Te+errorRxTxDiff_ indicate)

  - ±32.5 ns is used for errorBS, DL, TX

  - Enhanced Te for TA-based PDC is applied to Te here also

  - errorRxTxDiff,indication is to reflect the error due to indication granularity of Rx-Tx time difference

  - Note: Alt.2 assumes that gNB can coordinate the time of TA procedure and the time of PD compensation, so that the DL frame timing error and BS transmit timing error for propagation delay estimation is correlated to (e.g. the same as) that for the transmission of RRC signaling carrying the reference time clock

| Company | View |
| --- | --- |
| OPPO | Not agree. If errorBS, DL, TX is zero or does not show up in PD estimation part, the necessary condition is that RTT measurements take reference point at antenna connector. Then because PD compensation takes reference point at baseband, the total error formulation (before error term combining) is: eUE,DL,RX(BB)-eBS,DL,TX(BB)+eUE,UL,TX(ANT)-eBS,UL,TX(ANT)+eBS,DL,TX(ANT)-eUE,DL,RX(ANT)+eRTT-ind2 where the first two terms come from PD compensation (“BB” stands for “baseband”) and the last term (divided by 2) comes from PD estimation (“ANT” stands for “antenna connector”, eBS,DL,TX(ANT)=0 in this case). Then the question is how come to get eUE,DL,RX(BB) and -12*eUE,DL,RX(ANT) combined to reach a single ½ coefficient? It seems Alt-2 messes up different reference points for DL-Tx timing and DL-Rx timing, which is quite strange. |
| Nokia, NSB | Support with modification - Enhanced Te for TA-based PDC may be is applied to Te here also We do not see any need to restrict enhanced Te to be used for PDC, normal TA can also be sufficiently accurate for some use cases. |
| ZTE | We prefer 1/2*errorBS,DL,RX |
| HW/HiSi | Agree |
| Ericsson | Do not support. First, we cannot accept changing errorBS, DL, TX to half the agreed value. Second, we do not understand why Alt 2 does not have +errorUE, DL,RX term, which is the error to receive the RRC of referenceTimeInfo. |

Feature lead: No need to discuss since we have agreed with Alt.1.

Down-selection between Alt.1 and Alt.2

Question 4.3.5-1: Which option do you prefer for the following evaluation of RTT-based PDC?

- Option 1:  Keep both Alt.1 and Alt.2 for further evaluation for RTT-based PDC, and do comparison between RTT-based PDC and TA-based PDC under the same assumption

- Option 2:  Down-select to Alt.1 for RTT-based PDC

- Option 3:  Down-select to Alt.2 for RTT-based PDC

| Company | View |
| --- | --- |
| OPPO | We think Option-2 is the most promising. Option-1 conflicts RAN1 agreement in last meeting. Option-3 has the issue we commented above. |
| Nokia, NSB | Option 3 In our view, Alt. 2 is easiest to align with the TA assumptions. |
| Vivo | We agree with option 2. If majority of companies support option 1, we can also live with it. |
| ZTE | We prefer option 2 to use the same error components and the same co-related assumption for fair comparison. |
| Intel | Option 1 |
| HW/HiSi | Option 2 is preferred. |
| Qualcomm | Option 2 for simplification |
| Ericsson | Option 2 |
| LG | Option 1 or Option 3 |

Feature lead: No need to discuss since we have agreed with Alt.1.

- 4.3.5a Second round discussion for issue 4.3-5

Based on the inputs and analysis above for issue 4.3-5 in section 4.3.5, the following proposals are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals before providing your views here.

Potential conclusion 4.3.5-2: For Alt.1 for evaluation of the overall time synchronization error for RTT-based propagation delay compensation, the following is assumed for DL propagation delay estimation error:

  - BS uplink detection error errorBS, UL,RX is reflected by errorgNB,RxTxDiff, and BS transmitting time error errorBS, DL,TX  (if needs to be considered) can be reflected by the margin Y for gNB Rx-Tx time difference measurement, where Y is declared by manufacturer;

  - UE downlink detection error errorUE, DL,RX is reflected by errorUE, RxTxDiff, and UE transmitting time error errorUE, UL, TX (if needs to be considered) can be reflected by the margin  for UE Rx-Tx time difference measurement, where the definition of   is up to RAN4.

  - Support: Nokia, NSB, Intel, Huawei, HiSilicon, Qualcomm

| Company | View |
| --- | --- |
| Feature lead | @ OPPO @ Vivo @ ZTE @ Ericsson Please check replies above to your questions. |
| Intel | OK |
| OPPO | The proposal seems to make things more complicated, because it establishes the interpretations of Y and  in a new way which is not backed by RAN4. It is also not clear to us why RAN1 needs to discuss how to determine errorBS, DL,TX (as equal to Y in proposal), given it is already agreed to be 65ns; and what is the point to assign  to errorUE, UL, TX? Is there any error term inside Alt-1 formula requiring use of errorUE, UL, TX? >> Feature lead I think I explained several times why this issue is discussed, we have a note in the agreements in last meeting and ZTE R1-2110397 requested to discuss it also, you can also find more detailed reasons in ZTE paper. If you have better way to address the questions they raised instead of this proposal, please share here also then we can discuss. |
| Ericsson | The conclusion is not needed. Agree with OPPO points. What does the conclusion accomplish? The evaluation formula will not be captured in any spec. It is not necessary to know how error_x is reflected by error_y. The margin discussion is RAN4 scope and there is no need of RAN1 conclusion. >> Feature lead I do agree that we won’t need to change any definition from RAN4. However, people have questions whether/how to reflect those error components as mentioned in the proposal, here I tried to say we don’t need to separately consider these error components, if they need to be considered it can be included in the margin. Any suggestion on how to address the questions raised by other companies are welcome also. |
| vivo | The conclusion is not needed. We share the similar view with OPPO and Ericsson. >> Feature lead Please see my reply to OPPO and Ericsson. |
| Nokia, NSB | Support |
| ZTE | Thanks FL so much for the clarification. We are not sure companies have the understanding on this. Please see our comments in 4.3.3a. |
| HW/HiSi | Support |

Summary of the status for potential conclusion 4.3.5-2 in second round

| Support | Nokia, NSB, Intel, Nokia, NSB, ZTE, Huawei, HiSilicon |
| --- | --- |
| Not support | OPPO, Ericsson, vivo |

FL recommendation: Still different views, let’s further discuss this issue.

- 4.3.5b Third round discussion for issue 4.3-5

Based on the inputs and analysis above for issue 4.3-5 in section 4.3.5a, the following proposals are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals before providing your views here.

Potential conclusion 4.3.5-2: For Alt.1 for evaluation of the overall time synchronization error for RTT-based propagation delay compensation, the following is assumed for DL propagation delay estimation error:

  - BS uplink detection error errorBS, UL,RX is reflected by errorgNB,RxTxDiff, and BS transmitting time error errorBS, DL,TX  (if needs to be considered) can be reflected by the margin Y for gNB Rx-Tx time difference measurement, where Y is declared by manufacturer;

  - UE downlink detection error errorUE, DL,RX is reflected by errorUE, RxTxDiff, and UE transmitting time error errorUE, UL, TX (if needs to be considered) can be reflected by the margin  for UE Rx-Tx time difference measurement, where the definition of   is up to RAN4.

| Company | View |
| --- | --- |
| Feature lead | @ OPPO @ Ericsson @vivo Please check my replies to your comments in section 4.3.5a, and see if you have further comments. |
| Nokia/NSB | Support |
| OPPO | While ZTE can argue why the discussion/proposal is needed, we concerned about the consequence if the proposal is endorsed. Our questions in the 2nd round are not answered so our concern remains. [OPPO update on v032] We wonder whether ZTE’s intention is to get a new interpretation of <errorBS, UL,RX, errorBS, DL,TX, errorUE, DL,RX, errorUE, UL, TX> for TA-based PDC (instead of RTT-based PDC). If yes, the proposal does not read that way, since the main proposal is very clear to focus on RTT-based PDC. As for the interpretation of <errorBS, UL,RX, errorBS, DL,TX, errorUE, DL,RX, errorUE, UL, TX> reflected by < errorgNB,RxTxDiff, Y, errorUE, RxTxDiff, > in ZTE’s comment, we have an earlier comment that the 2nd set of four errors take reference at antenna connector while the 1st set of four errors happens for a TA procedure in baseband. We wonder whether ZTE’s intention is to move TA procedure to antenna connector. |
| Ericsson | Thanks for explaining the intention of the conclusion. But still we do not see why the conclusions are needed or how the conclusion helps RAN1 progress. RAN1 is aware that there are two margins in RAN4 spec; RAN1 does not know the values for these values; RAN4 does not have these values (margin Y is up to manufacturer; margin is up RAN4 whenever RAN4 address their FFS); RAN1 moves forward with PDC work regardless. Regarding ZTE questions in their contribution, it may be addressed by conclude: RAN1 evaluation does not investigate the values of margin Y and margin , which are associated with requirements of gNB Rx-Tx time difference measurement and UE Rx-Tx time difference measurement. The margins are up to RAN4 study. RAN1 can recognize that the evaluation is imperfect without knowing the values of the margins. For example, RAN1 can account for this by stating that the evaluation is optimistic without the margins. In Ericsson contribution, we took the margins into account by using the largest accuracy values in 38.133 tables, i.e., 123 Tc instead of 17 Tc for gNB RxTxTimeDiff and SCS=15 kHz. |
| vivo | In my understanding, the original proposal from ZTE’s contribution supports Alt.2 for RTT-based PDC. Currently, Alt.1 have been agreed for RTT-based method. I wonder whether discussion on the issue is needed. On the other hand, for gNB RxTxTimeDiff and UE RxTxTimeDiff, the related error components included in Y and  should be discussed and determined in RAN4. I’m not sure whether RAN1 can draw the conclusion for these two sub-bullets in the proposal. |
| ZTE | Since we have agreed to use Alt.1 for evaluation for RTT-based PDC, including the gNB Rx_Tx time difference accuracy and UE Rx-Tx time difference accuracy, a common understanding of the two error components is helpful to the further evaluation. We still don’t know how companies interpret the defined time accuracy for RTT-based PDC. From the clarification from FL, we find the contradictory as explained in the second round discussion, based on the current assumption. See below. Based on the equation in 4.2.3a, the downlink detection error is 116ns in the case of 24 PRBs and 15kHz. In TS38.133, errorUE, RxTxDiff is 101 Tc (50.5ns) in the case of 24 PRBs and 15kHz according to TS38.133. So how can a error component with smaller value reflect a error component with larger value (The second bullet). Of course, FL provided her further comments. But we are still not convinced. We prefer to let RAN4 clarify this by an LS. FL’s intention is RAN1 can achieve something common on this maybe by this conclusion. We wonder how companies think about this issue? In addition, we don’t agree with Ericsson’s views that the margins can be ignored due to using the largest accuracy values. We think the largest value must be used since we cannot ensure the corresponding condition can be met in any case to achieve the smaller defined value. It should be noted this principle is also applicable for the TA-based PDC, where the largest value of Te is used even though there is smaller values defined in TS38.133. For TA-based PDC, we use the quite preserve value for safe evaluation. So we cannot ignore any factors that have negative impacts. Also the budget is (145ns, 275ns) from the LS from RAN2. The margin should be considered. The result from Ericsson is 251ns at 15kHz SCS. There is only 24 ns left for satisfying the requirement. It is difficult to say it can fully cover the delta+Y. |
| HW/HiSi | Still support |

Feature lead: Based on the discussions in several rounds, I doubt we can really get this proposal agreed. Let’s further see depending on the reported values of the margin in the next RAN1 meeting, and if needed we can discuss again.

Issue 4.3-6: Evaluation on the achievable time synchronization accuracy for RTT-based PDC

Based on inputs from companies in RAN1#106bis meeting, the total error results are summarized below.

Table 1 Overall synchronization error summary for RTT-based PDC

| Source | overall synchronization error |  | Assumptions (Alt.1 or Alt. 2) | Note |
| --- | --- | --- | --- | --- |
|  | 15 kHz | 30 kHz |  |  |
| Qualcomm (R1-2110182) | 182.3ns |  | Alt. 1 | errorgNB,RxTxDiff=45.8ns errorUE, RxTxDiff=45.8ns errorBS, DL, TX=32.5ns errorRxTxDiff_ indicate=8ns |
|  | 282ns |  | Alt.2-1 |  |
|  | 136.5ns |  | Alt.2-2 |  |
|  | 265.75 ns |  | Alt.2-3 |  |
|  | 120.25 ns |  | Alt.2-4 |  |
| Ericsson (R1-2108833) | 251ns | 158ns | Alt. 1 | errorBS, DL, TX=65ns errorUE, DL, RX=116ns for 15kHz and 58ns for 30kHz errorgNB,RxTxDiff=63ns for 15kHz and 21ns for 30kHz errorUE,RxTxDiff=70ns for 15kHz and 44ns for 30kHz errorRxTxDiff_ indicate=8ns for 15kHz and 4ns for 30kHz |
| Huawei (R1-2109743) | 245ns | 245ns | Alt. 1 | errorgNB,RxTxDiff=60ns errorUE, RxTxDiff=92ns errorBS, DL, TX=65ns errorRxTxDiff_ indicate=8ns |
|  | 179ns | 179ns | Alt. 1 | errorgNB,RxTxDiff=4ns errorUE, RxTxDiff=15ns errorBS, DL, TX=65ns errorRxTxDiff_ indicate=8ns |
|  | 314ns | 249ns | Alt.2-1 | errorBS, DL, TX=65ns errorRxTxDiff_ indicate=8ns |
|  | 169ns | 169ns | Alt.2-2 |  |
|  | 282ns | 217ns | Alt.2-3 |  |
|  | 137ns | 137ns | Alt.2-4 |  |
| ZTE (R1-2110397) | 240.75+(δ+Y)*0.25 | 217.25+(δ+Y)*0.25 | Alt.1 | errorBS, DL, TX=65ns errorUE,RxTxDiff=91.5+0.5δ for 15kHz and 85+δ for 30kHz errorgNB,RxTxDiff=59.5+0.5Y for 15kHz and 19+0.5Y for 30kHzerrorRxTxDiff_ indicate=0.5ns |
|  | 278.25 | 212.75 | Alt.2-3? | errorBS, DL, TX=65ns errorRxTxDiff_ indicate=0.5ns |
| Nokia (R1-2109161) |  |  | Alt.2-4 |  |
| vivo (R1-2108970) |  |  | Alt. 1 | errorBS, DL, TX=32.5ns errorUE,RxTxDiff=137+δ for 15kHz errorgNB,RxTxDiff is TBD in fading channel |
| LGE (R1-2109974) |  |  | Alt. 1 |  |
| CATT (R1-2109219) | 285.5ns | 220.5ns | Alt. 2-1 | errorBS, DL, TX=32.5ns errorRxTxDiff_ indicate=16ns |
| Samsung (R1-2109485) | 265.55ns |  | Alt. 1 | errorBS, DL, TX=65ns errorUE, DL,RX =116 ns errorUE,RxTxDiff=91.6ns errorgNB,RxTxDiff=59.5ns errorRxTxDiff_ indicate=16ns |
| Intel (R1-2109608) | 230ns | 178ns | Alt. 1 |  |
|  | 173ns | 173ns | Alt.2-4 |  |
| Note: Alt. 1: Alt. 2-1: errortotal,RTTbased≤ errorBS, DL, TX+12(errorUE,DL,RX+errorBS, UL, RX+errorUE, UL, TX+errorRxTxDiff_ indicate) Since errorUE, UL, TX+errorUE, DL, RX≤Te based on LS from RAN4 [15], so Alt.2-1 can be the following errortotal,RTTbased≤ errorBS, DL, TX+12(errorBS, UL, RX+Te+errorRxTxDiff_ indicate) Alt. 2-2: errortotal,RTTbased≤ errorBS, DL, TX+12(errorUE,DL,RX+errorBS, UL, RX+errorRxTxDiff_ indicate) Alt. 2-3: errortotal,RTTbased≤ 12errorBS, DL, TX+12(errorUE,DL,RX+errorBS, UL, RX+errorUE, UL, TX+errorRxTxDiff_ indicate) Since errorUE, UL, TX+errorUE, DL, RX≤Te based on LS from RAN4 [15], so Alt.2-3can be the following errortotal,RTTbased≤ 12errorBS, DL, TX+12(errorBS, UL, RX+Te+errorRxTxDiff_ indicate) Alt. 2-4: errortotal,RTTbased≤ 12errorBS, DL, TX+12(errorUE,DL,RX+errorBS, UL, RX+errorRxTxDiff_ indicate) |  |  |  |  |

Feature lead: Based on the above evaluation, it can be seen that with Alt.1, RTT-based PDC can meet the 275 ns budget in the worst case without considering the margin. With Alt.2, without enhanced Te, then in some cases RTT-based PDC cannot meet the budget. It is expected that with enhanced Te, RTT based PDC with Alt.2 can meet the budget also. I made some observation for further discussion in section 4.3.6.

- 4.3.6 First round discussion for issue 4.3-6

Based on the inputs and analysis above for issue 4.3-6, the following proposals are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals before providing your views here.

Proposed observation 4.3.6-1: With Alt.1 as the equation for evaluation of the overall time synchronization error for RTT-based PDC, RTT-based PDC can meet the upper bound (i.e. ±275 ns) of Uu interface synchronicity error budget for control-to-control scenario without considering the margin, where margin Y for gNB Rx-Tx time difference measurement is declared by manufacturer, and margin   for UE Rx-Tx time difference measurement is up to RAN4.

| Company | View |
| --- | --- |
| OPPO | We are basically fine with the observation, but having two comments on the margins: - Is the margin  the one currently mentioned in 38.133 in table 10.1.25.2-2 for UE Rx-Tx time difference measurement (for positioning purpose) or a second new one? - What we want to keep clarified is that the margins Y and  are not declared specifically for PDC purpose. >>Feature lead 1. Current one as replied to your previous questions. Details can be seen there. 2. Depending on what reference to be used for PDC. For example, for TRS it won’t be used for positioning, so the margin is only for PDC. But for PRS, the margin for positioning can be reused. 3. In addition, seems no need to exactly listing the margin in the observation, we can just delete it if there is no strong concern. |
| Nokia, NSB | Agree We believe that RTT (with Alt.1 and Alt.2) will be sufficiently accurate for the upper bound. We also believe that RAN1 should start deprioritising the importance of the equations and associated assumptions and instead select a PDC method to support. I the end, it will be RAN4 that specified the requirements to the procedure and only by then, the performance bound will be known. |
| Vivo | As mentioned above, the value for errorBS,RxTx diff for fading channel is not available. >>Feature lead Similar to your previous questions, for comparison it should be fine to only use values for AWGN. |
| ZTE | Yes, we think this observation is reasonable from the result in our contribution if proposal 4.3.3-1 is agreed. |
| Intel | Agree |
| HW/HiSi | Agree |
| Qualcomm | Agree |
| Ericsson | Agree. Additionally, another observation can be made: “Observation 4.3.6-2: RTT-based PDC can meet the upper bound (i.e. ±275 ns) of Uu interface synchronicity error budget for control-to-control scenario without considering the implementation margin” |

Proposal 4.3.6-1: Send an LS to RAN4 to ask for feedback on the following questions, if Alt.1 will be used as the equation for evaluation of the overall time synchronization error for RTT-based PDC:

- Question 1: The value range of  for UE Rx-Tx time difference measurement and the potential factors that may have impact on  ;

- Question 2: The value range of Y for gNB Rx-Tx time difference measurement and the potential factors that may have impact on Y;

| Company | View |
| --- | --- |
| Nokia, NSB | We do not seen an (absolute) need to send an LS (as per our comment to Proposed Observation 4.3.6-1) but would not object to send an LS if a strong majority of companies think an LS would be needed. >>Feature lead Yes it is not really necessary. Indeed we need to avoid sending RAN4 too many LS, otherwise they won’t be able to reply to us. |
| Vivo | Support. In addition, gNB Rx-Tx time difference accuracy for fading channel also should be provided by RAN4. >>Feature lead As my reply to Nokia above, are u ok not to send the LS, but you can report Y in your RAN1 contribution, since anyway that is declared by companies. As to delta, even we don’t send the LS, once the work in RAN4 is done, we can use it directly. |
| ZTE | If Alt.1 is used, then the two questions can be included in the LS. >>Feature lead Similar as my reply to Vivo. |
| Intel | We afraid it can delay RAN1 discussion/decision on PDC. RAN1 can assume some reasonable values (including 0) based on feedback from RAN4 colleagues. |
| Qualcomm | Support. >>Feature lead Similar as my reply to Vivo. |
| Ericsson | No need to send an LS. RAN4 will do the proper work as they see fit. |

Proposed observation 4.3.6-2: With Alt.2 as the equation for evaluation of the overall time synchronization error for RTT-based PDC, RTT-based PDC may or may not meet the upper bound (i.e. ±275 ns) of Uu interface synchronicity error budget for control-to-control scenario, depending on the value of enhanced Te.

| Company | View |
| --- | --- |
| OPPO | We have the concern on Alt-2 formula itself, as explained under issue 4.3-5. |
| Nokia, NSB | Strongly disagree. The error Te should not influence the Rx-Tx based PD estimate, not in Alt. 1 and neither in Alt. 2. There can be a margin as in Proposal 4.3.6-1, but not related to Te. |
| ZTE | We are fine with this observation, and the results depends on the value of Te. |
| Intel | Share similar understanding with Nokia. RTT procedures should be designed to compensate Te error components. |
| HW/HiSi | We support this observation. Based on the summary given in Table 1 above, the alternative 2 cannot satisfy the budget in some cases. |
| Qualcomm | Share same view as Intel and Nokia. |
| Ericsson | Alt 2 formula makes no sense to us |

Feature lead: No need to discuss any more, since Alt.1 is agreed already.

- 4.3.6a Second round discussion for issue 4.3-6

Based on the inputs and analysis above for issue 4.3-6 in section 4.3.6, the following proposals are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals before providing your views here.

Revised Proposed observation 4.3.6-1: RTT-based PDC at least under certain conditions can meet the upper bound (i.e. ±275 ns) of Uu interface synchronicity error budget for control-to-control scenario without considering the margins described in RAN4 specification.

| Company | View |
| --- | --- |
| Feature lead | 1. Since we have agreed Alt.1, I delete the sentence at the beginning in the original proposed observation 4.3.6-1. 2. Per the comment from OPPO, I delete the last part in the original proposed observation 4.3.6-1, which is not that critical I think. 3. I added “at least under certain conditions” to address OPPO concern, people can see if you can accept it. Note: To make the revised proposal easier to see, I removed all the change marks that don’t matter that much. |
| Intel | OK |
| OPPO | We would like to have a calculation of total error to be commonly understood, along with the discussion of this observation. Hope it can make the discussion easier. Alt-1 formula is: where, according to current RAN1 discussion status, - errorBS,DL,TX=65ns - errorUE,DL,RX=100ns - For the worst case in Table 10.1.25.2-2 of 38.133 for SCS=15kHz, errorUE,RxTxDiff=180Tc=91.6ns. - For the worst case in Table 13.2.2.2-1 of 38.133 for SCS=15kHz, errorBS,RxTxDiff=117Tc, but it is for AWGN. In order to use a more meaningful value for fading channel, we apply the AWGN-fading difference (=59Tc) derived on UE side, as shown in table below. So errorBS,RxTxDiff=117+59Tc=89.9ns PRS Es/IoT (dB); Accuracy under AWGN (Tc); Accuracy under FR1 fading (Tc); Accuracy difference between AWGN and fading (Tc), assuming the same δ. -3; ±78+δ; ±137+δ; 59. -13; ±101+δ; ±180+δ; 79. - errorRxTxDiff_Indication is not agreed yet, but it won’t be larger than 322Tc=8.1ns if RAN1 agrees to follow the granularity in positioning. All above gives errortotal,RTT-based≤65+100+89.9+91.6+8.12=260ns So the main observation is fine. Given this observation is made under some assumptions not covered by RAN4-specified requirements, we would like to modify the observation a bit as following: Revised Proposed observation 4.3.6-1: With RAN4-specified requirements and RAN1-agreed assumptions, RTT-based PDC can meet the upper bound (i.e. ±275 ns) of Uu interface synchronicity error budget for control-to-control scenario without considering the margins described in RAN4 specification. - Note: this observation assumes no propagation delay estimation error caused by the change of UL-Tx timing due to TA command and/or UE autonomous adjustment. >> Feature lead I think ok to go without this note, since as you can see from the discussion in section 4.3.5, people all agree that the inconsistence RTT measurement in gNB and UE due a change of uplink TX timing should be avoided, the remaining open issues is on the detailed scheme to ensure this. Since anyway we will address it, better for us to keep the observation simpler. We ask to add the above note because RAN1 is still under discussion how to treat the impact of change of UL-Tx timing, so it is not a part of existing “RAN1-agreed assumptions”. We also add “described in RAN4 specification” for the margin because otherwise different companies would have different interpretations on the margins (possibly in a different way from what RAN4 spec says), as we see in earlier discussion. >> Feature lead Thanks. I agree that it is clearer with “described in RAN4 specification”, updated accordingly. [OPPO update]: We would like to clarify why we add editing of “With RAN4-specified requirements and RAN1-agreed assumptions”. As one can see, the total error for RTT-based PDC is 260ns, which is just 15ns less than the upper-end of Uu interface budget for control-to-control. This is a completely different story from the TA-based PDC in smart grid scenario, where the total error is about 220ns less than the lower-end of Uu interface budget. This 15ns gap is quite easy to be consumed by, for example, a different SINR setting on either gNB side or UE side. To be more specific, the above error calculation takes errorBS,RxTxDiff=117Tc under condition of Es/IoT=+3dB and 24 ≤ BW ≤ 40. RAN4 does not provide an accuracy for certain lower Es/IoT (such as -3dB in table for errorUE,RxTxDiff) under the same BW. If it did happens in certain deployment, there is a high risk for the above error calculation to exceed the Uu budget. Besides the lower SINR case missed from RAN4 spec, there could be other factors such as a fading profile that is more unfriendly to synchronization than what RAN4 assumed. We (as a UE vendor) would like to point it out that, even if the UE implementation meets the RAN4 requirements for every aspect, there is still a chance for such synchronization to fail the Uu-interface budget in real deployment operation. What FL proposed here sounds a bit too strong, given it seems to say such capability of RTT-based PDC to meet the 275ns budget is unconditional. >> Feature lead Thanks. You have some points here, but as Ericsson commented below, it seems better to make it simpler, as long as there is no misunderstanding among companies. If you look at the observation, actually it is weak, since it only says meeting the upper bound of control-to-control scenario. Anyway, I tried to add “at least under certain conditions” to address your concern, I didn’t add what you exactly provide here because I think not good for all of us to spend much time debate the conditions. Hope the newly added part won’t bring more comments from companies, one thing we would need to avoid is to dig deeply on the conditions, since we don’t have much time left and better to focus on whether/how to specify a solution. |
| Ericsson | Support. The edits by OPPO are not necessary. It complicates the message too much. At most ‘margin’ can be changed to ‘margins described in RAN4 specification’. The previous observation (copied below) was trying to give similar message, and it didn’t pile up on all the assumptions and conditions. Agreements: (RAN1#104bis) - Observation 1: Propagation delay compensation based on existing Rel-15/Rel-16 TA procedure and associated granularity, with no enhancements in RAN1, is sufficient for meeting the Uu interface synchronicity error budget in LS R2-2010837 for the smart grid scenario. - Observation 2: RAN1 needs to further study and specify the feasible enhancement (if any with RAN1 spec impact) for propagation delay compensation for control-to-control scenario, in order to meet the synchronicity budget of Uu interface in LS R2-2010837.. |
| vivo | I have a question for clarification. Is the calculation based on AWGN channel only, right? I think the channel condition should be included in the proposal. In my understanding, the evaluation for fading channel is more meaningful. Actually, we share the same concern with OPPO on the possible factor for fading channel, which may lead to the deterioration of timing accuracy in some scenario. It seems early to make proposal 4.3.6-1. >> Feature lead Only for AWGN channel. As described above, only for comparison of the PDC method. For TA-based PDC, the values are based on AWGN channel also. Please also see more replied to companies above. |
| Nokia, NSB | Support in principle. Having the ‘without considering the margin’ without saying which margin we are referring to is rather unclear. Maybe we could use the wording ‘without considering the implementation margins at gNB and UE side’ instead. >> Feature lead How about the updated version? |
| Samsung | We have similar concern as vivo. There will have margin added in the end. This observation cannot lead to conclude that RTT based method can work. >> Feature lead In some sense, I agree we need to consider the margin in the end. |
| ZTE | We don’t think we need this observation since anyway the margin should be taken into account. It is easier to achieve the results if the margin is concluded. There is no need to do the analysis in two steps. >> Feature lead As replied to Samsung. |
| HW/HiSi | Support the updated version |
| Qualcomm | Support |

Summary of the status for Revised proposed observation 4.3.6-1 in second round

| Support | Intel, Ericsson, Nokia, NSB, Huawei, HiSilicon, Qualcomm |
| --- | --- |
| Not support | OPPO, vivo, Samsung, ZTE |

FL recommendation: It is true that now since margin needs to be considered, only with this observation may be not that helpful. Maybe we can see the situation in next meeting, assuming we can have better idea on the margin.

Question 4.3.6a-1: Are you ok not to send an LS to RAN4 to ask for feedback on the following questions for RTT-based PDC?

- Question 1: The value range of  for UE Rx-Tx time difference measurement and the potential factors that may have impact on  ;

- Question 2: The value range of Y for gNB Rx-Tx time difference measurement and the potential factors that may have impact on Y;

| Company | View |
| --- | --- |
| Feature lead | 1. Based on the comments in the first round email discussion, some companies prefer not to send to avoid additional work to RAN4, which I agree. 2. If we will not send the LS, companies can report your value Y in your RAN1 contribution if needed, since anyway RAN4 conclusion is that Y will be reported by manufacturer. For , even without the LS, RAN4 will work on it, then once RAN4 done, we can use it. |
| Intel | OK |
| OPPO | For Q1, RAN4 is working on . RAN1 does not need to ask. For Q2, we are neutral. >>Feature lead Yes RAN4 is working on , with LS my understanding is that RAN4 would ensure to provide feedback before RAN1 Nov meeting according to the RAN guidance. However, without the LS, not sure if RAN4 will get that done, then it would be difficult for us to conclude in the next RAN1 meeting on PDC, since companies the margin is unknown. |
| Ericsson | No need to send LS >>Feature lead As what I explained to OPPO above. |
| Nokia, NSB | OK |
| Samsung | Support LS. Otherwise, we cannot conclude whether RTT based can work or not in RAN 1. |
| ZTE | If we send an LS to RAN4 with these two questions, we are not sure if RAN4 can provide some values in their next meeting since it may not be easy to achieve a consensus on this. We also agree with the concern from the other companies. However, if the values are reported in RAN1 paper, we are afraid RAN1 cannot obtain a suitable value since it is not RAN1 field. So we slightly prefer to include the listed questions in the LS. A value range may also be helpful for RAN1 discussion if they could be provided by RAN4. If we send an LS, we suggest adding the our questions in the LS if possible. Since the questions are related to the RAN4 spec understanding, it is easy to get the answer in RAN4 without costing much time. Of course, it depends on the discussion above. Question 1: Whether UE downlink detection error and/or UE transmitting time error have been taken into account when defining the UE Rx-Tx time difference measurement accuracy and whether BS transmitting time error and/or BS uplink detection error have been taken into account when defining gNB Rx-Tx time difference measurement accuracy? Question 2: Is there any other error components needed to be considered in addition to errorgNB, RxTxDiff, errorUE, RxTxDiff, error RxTxDiff,indication, if gNB Rx-Tx time difference and UE Rx-Tx time difference are used to calculate the DL propagation delay with the assumption of the same DL and UL propagation delay, where error RxTxDiff,indication is to reflect the error due to indication granularity of Rx-Tx time difference, errorgNB, RxTxDiff and errorUE, RxTxDiff reflects the measurement inaccuracy of gNB Rx-Tx time difference, and the measurement inaccuracy of UE Rx-Tx time difference, respectively. >>Feature lead If we can conclude the proposal in section 4.3.5b, then we don’t need to include it. Personally really not good to give too much burden to RAN4. |
| HW/HiSi | We would be fine with a LS but we are also fine with the proposal. The value of Y is reported by the manufacturer anyway. And for  , we think that companies could report their values until RAN4 has made conclusion on that. |
| Qualcomm | OK |

Summary of the status for question 4.3.6a-1 in second round

| Support send LS | Samsung, ZTE, Huawei, HiSilicon, Qualcomm |
| --- | --- |
| Not support | Ericsson, Nokia, NSB, Intel |

FL recommendation: It seems still quite many companies prefer not the send the LS. The benefit to send LS is that we can get the value of the margin for further evaluate RTT-based PDC, otherwise we may not be able to make decision. Of course, on the other hand, I also think RAN1 better not to put too much burden to RAN4, then they can focus on the work and get the margin work done. Once they get it done, then RAN1 can directly use it. Let me check companies view one more round.

- 4.3.6b Third round discussion for issue 4.3-6

Based on the inputs and analysis above for issue 4.3-6 in section 4.3.6a, the following proposals are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals before providing your views here.

Proposal 4.3.6b-1: Send an LS to RAN4 to ask for feedback on the following questions for RTT-based PDC:

- Question 1: The value range of  for UE Rx-Tx time difference measurement and the potential factors that may have impact on  ;

- Question 2: The value range of Y for gNB Rx-Tx time difference measurement and the potential factors that may have impact on Y;

| Company | View |
| --- | --- |
| Feature lead | 1. As the discussion on the observation on the performance of RTT-based PDC in section 4.3.6, you could say companies think that we need the margin to decide whether RTT-based PDC can meet the budget. 2. The benefit to send LS is that we can get the value of the margin for further evaluate RTT-based PDC, otherwise we may not be able to conclude in Nov. meeting. Based on the above points, I slightly prefer to send the LS. Of course, on the other hand, I also think RAN1 better not to put too much burden to RAN4, then they can focus on the work and get the margin work done. Once they get it done, then RAN1 can directly use it as long as we can ensure that RAN4 would be able to have some progress on the margin for positioning. If people still feel better not to send the LS, I would encourage companies to talk to your RAN4 colleagues and let them know it would be good if they can make progress on the margin in their Nov meeting. Meanwhile, companies please report the margin in RAN1 paper also. |
| Nokia/NSB | We still do not see a need to send an LS. Looking at the feedback from the second round, it seemed that majority of companies (Ericsson, Intel, Nokia/NSB, QC) preferred not to send, whereas Samsung, HW/HiSi and ZTE preferred to send an LS. Seems QC input may not have been captured correctly in the 2nd round summary? >> Feature lead The proposal here is given not based on whether it is majority view or not, mainly based on whether it would be helpful for making decision in Nov. meeting. The worry from me is that I am not sure if in next meeting people will say that since these aspects are unclear we cannot make any decision on RTT-based PDC. However, if people do think it is not that necessary to send the LS, as I suggested above, hope companies try to provide inputs on the margin in RAN1 papers also if possible. |
| Intel | Similar observation as Nokia, i.e. don’t see majority or consensus to send this LS |
| Ericsson | Do not see a need to send LS. |
| Samsung | We’d like to know, without knowing  how can RAN 1 make conclusion for RRT based method? >> Feature lead I agree if there is formal inputs from RAN4, easier in RAN1. If in the end we cannot get it agreed to send the LS, companies are encouraged to talk to your RAN4 colleagues and let them know it would be good if they can make progress on the margin in their Nov meeting. Once RAN4 can get the margin  done in RAN4, even without the LS we can directly use the outcome from them in RAN4. Meanwhile, companies please report the margin in RAN1 paper also. |
| ZTE | We prefer to send an LS as explained above. From the evaluation result from Ericsson, the residual space is 24ns. It may be dangerous to cover both delta and Y. |
| HW/HiSi | We would prefer to send the LS and we are having the same question as Samsung regarding the  Anyhow, looking at the above comments it seems not likely that we will reach consensus. If the group cannot agree on sending an LS, it will be important that companies capture realistic values of and report them in their RAN1 papers. |
| LG | We are generally fine with sending LS to RAN4. Some additional information can be included to questions not to take much time of RAN4. |

Summary of the status for question 4.3.6b-1 in third round

| Support send LS | Samsung, ZTE, Huawei, HiSilicon, LG |
| --- | --- |
| Not support | Ericsson, Nokia, NSB, Intel |

Feature lead: It seems still controversial to send an LS for this. As I expressed above, there is pros and cons to send the LS. Since still several companies prefer not to send the LS, we might not be able to get it agreed. Companies are encouraged to talk to your RAN4 colleagues and let them know it would be good if they can make progress on the margin in their Nov meeting. Meanwhile, companies please report the margin in RAN1 paper also.

- 4.3.6c 4th round discussion for issue 4.3-6

Based on the inputs and analysis above for issue 4.3-6 in section 4.3.6b, the following proposals are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals before providing your views here.

Question 4.3.6b-1: Please indicate whether you will object any of the following alternatives.

Alt.1: Send an LS to RAN4 to ask for feedback on the following questions for RTT-based PDC:

- Question 1: The value range of  for UE Rx-Tx time difference measurement and the potential factors that may have impact on  ;

- Question 2: The value range of Y for gNB Rx-Tx time difference measurement and the potential factors that may have impact on Y;

Alt.2: No LS to RAN4 to ask for feedback on the following questions for RTT-based PDC:

- Question 1: The value range of  for UE Rx-Tx time difference measurement and the potential factors that may have impact on  ;

- Question 2: The value range of Y for gNB Rx-Tx time difference measurement and the potential factors that may have impact on Y;

| Objecting for Alt. 1 | Nokia/NSB, Ericsson |
| --- | --- |
| Objecting for Alt. 2 |  |

| Company | View |
| --- | --- |
| Feature lead | 1. As the discussion on the observation on the performance of RTT-based PDC in section 4.3.6, you could see companies think that we need the margin to decide whether RTT-based PDC can meet the budget. 2. The benefit to send LS is that we can get the value of the margin for further evaluate RTT-based PDC, otherwise we may not be able to conclude in Nov. meeting. As discussed in section 4.3.6b again, the views are still controversial. The number of companies prefer sending the LS and not sending the LS are almost the same. Let’s try to see if any strong concern on either of the alternatives above. As I expressed above, there is pros and cons to send the LS. If in the end we are not able to send the LS, companies are encouraged to talk to your RAN4 colleagues and let them know it would be good if they can make progress on the margin in their Nov meeting. Meanwhile, companies please report the margin in RAN1 paper also. |
| Nokia, NSB | Object to Alt. 1, support Alt.2. It is our anticipating that we will not be able to get agreed value ranges for or Y from RAN4. We will in RAN1 need to make an choice of PDC methods without having hard/fixed values for the procedure. However, we can make an effort selecting the PDC methods that can be configured to satisfy a certain accuracy budget. In this case, a higher Y (and/or ) could be compensated by a higher UL/DL reference signal bandwidth. It will only be when RAN4 starts discussing performance requirements for Rx-Tx (for PDC) that the performance requirements will be known. That is similarly as in positioning WI for example, as RAN1 did not have to wait RAN4 to agree on the exact accuracy values before RAN1 could agree on the framework that is in place. Regarding the values of  , that is under FFS in RAN4, and sending an LS will not make any difference. We don’t think we can speed up their agreement on this value with the LS. In our understanding, the RAN recommendation for RAN4 replying RAN1 before next meeting is just for the LS already sent. And for Y, it is our understanding that RAN4 will not specify Y, so it may not be possible for RAN4 to answer Question 2. Therefore, let’s save time & effort from RAN1 and RAN4 by not sending an LS. By the way: we can only ‘object’ to do or agree something, but not to ‘not to do/agree to something’ (i.e. Alt. 2). |
| Intel | Do not object any alternative |
| HW/HiSi | Same as Intel, we do not object any of the alternatives. |
| OPPO | No objection to any alternative. But given RAN4 is working on  anyway (in a positioning WI), it does not harm to ask RAN4 to provide whatever RAN4 can share with RAN1 before next RAN1 meeting; however, if RAN1 does ask this, it is important to tell RAN4 that RAN1 does not intend to have  in RAN4 response/spec to be PDC-specific in any way (it is up to RAN4 whether the number is positioning-specific). Note this is only for , not for Y since RAN4 would not work on Y. |
| Ericsson | Same view as Nokia. Support Alt 2 |

Implicit propagation delay compensation

OPPO (R1-2109097) proposes an implicit PDC method as below:

Figure 3 Implicit PDC procedure The principle of implicit PDC is to identify the clock time difference eclk between gNB clock time and UE clock time, as shown in Figure 3. It can be proved that eclk=mUL,RX+mDL,TX2-nUL,TX+nDL,RX2, which suggests the following procedure steps: - Step-1: UE sends a UL SRS to help UE and gNB to establish the UL-Tx clock time in UE (nUL,TX) and UL-Rx clock time in gNB (mUL,RX). This UL SRS can be identified as the last SRS transmitted before the DL message transmitted in Step-2. There should be no RAN1 spec impacts for Step-1. This SRS could be a subset of another configured periodic SRS. RAN2 may want to make the configuration of this SRS specifically for PDC purpose (although the SRS signal itself can be used for other purpose). - Step-2: The gNB sends to UE a ReferenceTimeInfo-alike RRC message containing a clock time information of tinfo=mUL,RX+mDL,TX2, where mDL,TX corresponds to the DL-Tx clock time for the transmission of this DL message. The existing RRC message of ReferenceTimeInfo can be reused/extended in this case. The detailed design is up to RAN2. - Step-3: The UE calculates eclk=tinfo-nUL,TX+nDL,RX2, where nDL,RX is the DL-Rx clock time corresponding to the reception of the DL message mentioned in Step-2, and nUL,TX is the UL-Tx clock time established in Step-1. For the implicit PDC in Figure 3, the total clock synchronization error is given by errortotal, implicit-PDC≤errorBS, DL,TX+Te+errorBS,UL,RX2+12Tg where Tg is clock time indication granularity in the ReferenceTimeInfo-alike RRC message. Because the implicit PDC has no explicit step of propagation delay compensation, the network error budget counted for ReferenceTimeInfo granularity of 10ns does not apply to implicit PDC, which means the total Uu interface error budget for implicit PDC is 280ns, not 275ns. Observation-4: For implicit PDC, the total Uu error budget is 280ns, instead of 275ns. Then, 65+390.6+100+Tg2≤280ns leads to Tg≤4.4ns. Proposal 3: Suggest RAN2 to adopt implicit PDC for clock synchronization, with following RAN2 specification impacts. - Design a UE-dedicated ReferenceTimeInfo-alike DL RRC message that contains (mUL,RX+mDL,TX2) with granularity no larger than 4ns, where mUL,RX is the local clock time associated with the reception of the last UL SRS before transmission of this DL RRC message, and mDL,TX is the local clock time associated with the transmission of this DL RRC message. - The UL SRS can be configured with periodicity/offset specifically for PDC purpose (which is already agreed in RAN1 in case RTT-based PDC is supported).

Feature lead: There were some initial discussions on implicit PDC in the previous meetings as captured in the feature lead summaries, mainly on the benefits and difference with explicit PDC compared to TA-based PDC and/or RTT based PDC. However, since the implicit PDC was proposed late (i.e. in RAN1#104b-e) and there was only very initial discussions, the final views from companies are still not clear yet. From feature lead perspective, some questions are set below for further discussion.

- 4.4.1 First round discussion

Based on the inputs and analysis above, the following proposals are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals before providing your views here.

Question 4.4-1: Do you have any further comment/question/views on implicit PDC proposed above?

| Company | View |
| --- | --- |
| Feature lead | A few questions from feature lead as below: 1. Is it correct understanding that implicit PDC can at most allow 280 ns synchronization budget? 2. Do we need RAN2 to check the feasibility to support Tg≤4.4ns? |
| OPPO | First of all, it is a bit surprising to see an assessment of “the implicit PDC was proposed late (i.e. in RAN1#104b-e) and there was only very initial discussions”, since the idea has been on the table for almost half year, and receiving only a few comments should not be interpreted as still being in initial discussion (we can comment in a different way that people just have no comments), not to mention all received comments in the past meetings are all answered. >> Feature lead Compared to TA-based PDC and RTT-based PDC, I think it is fair to say implicit PDC is brought out late, I think that’s why there is almost no discussion in companies contribution, and thus difficult for me to make suggestions since views from companies are not clear. But anyway, this is not critical, let’s just focus on the questions and discussion. To answer FL questions: - The error budget is not what the solution can allow, it is a requirement put onto any solution, just like 275ns for explicit PDC. But if people agree that explicit PDC can at most allow 275ns budget, then yes, implicit PDC can at most allow 280ns budget. >> Feature lead Maybe my question not clear. I am trying to understand the performance (i.e. the potential overall synchronization error) that implicit PDC can achieve, then see if any chance to make some observation for it. Based on your answer, it is still not clear to me whether my understanding is correct or not. For example, is it possible implicit PDC can meet scenarios with synchronization budget of 145 ns? [OPPO answer] Certainly No under current assumptions. But why is such question not asked for TA-based and RTT-based? Could TA-based and RTT-based solutions answer Yes? - We do not see an issue for RAN2 to support a finer clock time granularity (from 10ns down to 4ns) in RRC message. But in order to remove people’s potential concern, it is ok to us to ask RAN2 to check feasibility. |
| Nokia, NSB | In addition to FL questions we have the following: 3. Can the scheme work for compensating PD of timing provided with SIB9 and still ignore the ReferenceTimeInfo granularity from the budget? 4. Is it correctly understood that this new UE specific ReferenceTimeInfo will consist of the conventional ReferenceTimeInfo and a DL reference signal combined? 5. Do you foresee no need for an UL timestamp in the implicit scheme? |
| OPPO-2 | To answer Nokia’s question: Q3: The proposed implicit PDC scheme needs UE-dedicated RRC signalling to carry new ReferenceTimeInfo, because the time contained inside new ReferenceTimeInfo is derived based on UE-dedicated UL-Rx timing in gNB. SIB-9 would not be used. UE can receive SIB9, but the proposed implicit PDC leaves SIB9 aside. Although this implicit PDC needs to define a new RRC signalling, but it avoids the new definition of MAC-CE/RRC for new RTT indication; further, the current RAN2 signalling framework already has UE-dedicated ReferenceTimeInfo footprint anyway. Q4: No. This new UE-specific ReferenceTimeInfo differs from the conventional one in two places: - The clock time contained inside this new IE has smaller granularity, down from 10ns to 4ns. - The clock time contained inside this new IE has different interpretation (i.e., referring to the different clock time instance on gNB side) from the conventional one. This new UE-specific ReferenceTimeInfo does not require to combine with any specific DL-RS, just as conventional UE-specific ReferenceTimeInfo and SIB9 do not require the same. The UL-Rx timing detection error keeps the same as the one in conventional case. Q5: There is no need to have UL timestamp. The UL-Rx timing is associated with the “last” SRS prior to transmission of new UE-specific ReferenceTimeInfo, and the information of this UL-Rx timing is translated into the time sent inside new ReferenceTimeInfo. There is no need for explicit UL timestamp. Note that this “last” SRS can be a general SRS, i.e., not specific for PDC purpose. |

Feature lead: For implicit PDC, since the main impact is RAN2, I think whether it can work or not may have to leave it to RAN2. From RAN1 perspective, what we can try to conclude is the potential performance it can achieve I think, if the whole mechanism can work from RAN2 perspective. Frankly speaking, I don’t know what better way to handle this implicit PDC, since the interest on this solution seems low, so far only one company have strong desire to discuss it. I think I will have to check companies view first on how to handle this implicit PDC.

- 4.4.1a Second round discussion

Based on the inputs and analysis above, the following proposals are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals before providing your views here.

Question 4.4.1a-1: Which way do you prefer to handle implicit PDC?

- Option 1: Send LS to RAN2 to ask for checking on the feasibility of implicit PDC, including the feasibility to support Tg≤4.4ns. RAN1 continue to evaluate and conclude the potential performance that implicit PDC can achieve.

- Option 2: Drop implicit PDC in Rel-17.

- Option 3: Others

| Company | View |
| --- | --- |
| Feature lead | For implicit PDC, since the main impact is RAN2, I think whether it can work or not may have to leave it to RAN2. From RAN1 perspective, what we can try to conclude is the potential performance it can achieve I think, if the whole mechanism can work from RAN2 perspective. Frankly speaking, I don’t know what better way to handle this implicit PDC, since the interest on this solution seems low, so far seems only one company have strong desire to discuss it. I think I will have to check companies view first on how to handle this implicit PDC. If you have any good suggestion, please share it also. |
| OPPO | Option-1. For option-2, FL provides two reasons for dropping. - Low interest: I wonder when “no interest from a company” could be a good reason [in written] in RAN1 for the company to deny a solution. - Limited time: Given the spec impacts and WG work load for RTT-based PDC are far larger than implicit PDC (is there any different view on this?), if “limited time” is the concern, why should RAN1 go with the solution that puts all RAN1/2/4 on harder life? >> Feature lead My fault to include potential reasons in the proposal itself. That is just two potential concerns I feel that people may have, no need to include in the proposal and people may have their own thinking. The intention is just to check the temperature and then I know better which way to go. |
| Ericsson | Option 2 |
| vivo | Option 2 |
| Nokia, NSB | Option 2. Yet another concern we have with this is that it does not work for SIB9 as seen from previous reply. So even if a UE is semi-static and does not require frequent propagation delay estimation updates, it will have to exchange UL and DL to get a timing understanding by implicit PDC. While with the other PDC methods, the periodicity of PD estimation for compensating PD change and periodicity of getting timing information for compensating clock skew are decoupled, i.e. UE can get a PD estimate an apply the same estimate many times on top of timing received in SIB9 to compensate for the UE clock skew while PD has not changed. |
| Samsung | Opt 2 |
| ZTE | Option 2 |

Summary of the status for question 4.4.1a-1 in second round

| Option 1 | OPPO |
| --- | --- |
| Option 2 | Ericsson, vivo, Nokia, NSB, Samsung, ZTE |
| Option 3 |  |

FL recommendation: Based on the status, it seems impossible for us to go with option 1. If OPPO can have other better suggestion on how to move forward, please share also.

Question 4.4.1a-2: Do you have any further comment/question/views on implicit PDC proposed above?

| Company | View |
| --- | --- |

- 4.4.1b Third round discussion

Based on the inputs and analysis above, the following proposals are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals before providing your views here.

Potential conclusion: Drop implicit PDC in Rel-17.

| Company | View |
| --- | --- |
| Feature lead | Based on the status as in 4.4.1a, it seems impossible for us to go with option 1. If OPPO can have other better suggestion on how to move forward, please share also |
| Nokia/NSB | Support |
| Intel | May be a softer and more acceptable wording is “There is no consensus to support/specific implicit PDC in Rel-17” >> Feature lead Agree that your suggestion reflect the fact better. |
| OPPO | We certainly oppose the conclusion of dropping implicit PDC at this phase, because - Implicit PDC is the only solution right now being able to numerically meet the Uu budget with only RAN2 signaling impact but without worrying about margins in RAN4. In contrast, the RTT-based PDC is too greedy on the performances of errorgNB, RxTxDiff and errorUE, RxTxDiff, by following the RAN4 assumptions in establishing those requirements, such as at least Es/IoT and fading profiles, as we mentioned earlier. Consequently, we still see a risk that the current RTT-based PDC could be a conditional paperwork claim that likely fails in real deployment. Dropping another workable solution at this phase is not a good idea. - We still do not see a good technical reason to drop the solution. Note that the FL already commented the original Option-2 is not appropriate (with faulty “due to” reasons). We could argue the FL-provided conclusion is based on the earlier inappropriate proposal. >> Feature lead I meant I should not include my personal guess to the proposal directly and should see the views from companies first. Back to Nokia’s comment in round-2, which says the implicit PDC loses a benefit offered by RTT-based PDC, where “UE can get a PD estimate and apply the same estimate many times on top of timing received in SIB9 to compensate for the UE clock skew while PD has not changed”, we think this comment is a valid point but somehow still arguable, because - UE would not know whether “PD has not changed” over a certain time until it receives gNB Rx-Tx time interval from gNB and observes the newly-derived PD is the same as before; otherwise, UE just “blindly” makes a guess. >> Feature lead If UE doesn't receive any further indication from gNB on either TA or gNB Rx-Tx time interval, then UE should know that the previous one can be still valid? - The Nokia’s comment is about signaling overhead saving, it is less critical than the concern whether the solution itself can safely meet the Uu budget. So our suggestion is that RAN1 should firstly see whether RTT-based PDC can be a safe solution before dropping any other solutions. [OPPO update on v032]: To solve Nokia’s concern, we can modify our implicit PDC proposal to separate UL-Rx timing (mUL,RX) and DL-Tx timing (mDL,TX) into two fields in the UE-dedicated RRC signalling, so that the UL-Rx timing can be stored in UE and used later with ReferenceTimeInfo received in the following SIB9. The modified implicit PDC is given in following figure. The timing granularity remains the same as in current proposal. >> Feature lead If you change your proposal on implicit PDC again, it is not clear to me whether any further concerns on the proposed changes above again. [OPPO] The change is to solve Nokia’s concern, and the change does not impact the total error calculation result as well as the timing granularity asked before. |
| Ericsson | Support, with Intel phrase |
| Qualcomm | Support |
| Samsung | Intel’s wording is better |
| vivo | Support. We are fine with intel’s wording. |
| ZTE | Support. Intel’s wording is also fine to us. |
| LG | Support the proposal and Intel’s suggestion. |

- 4.4.1c 4th round discussion

Based on the inputs and analysis above, the following proposals are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals before providing your views here.

Revised Potential conclusion: There is no consensus to support/specify implicit PDC in Rel-17.

| Company | View |
| --- | --- |
| Feature lead | Based on the status as in 4.4.1b, all companies would support the conclusion except OPPO. OPPO proposed modifications on implicit PDC based on comments above. Please companies check. |
| Nokia, NSB | Support the proposal. @OPPO: In our understanding, even with the new modification on the implicit PDC, it cannot be argued that network error budget counted for ReferenceTimeInfo granularity does not apply, as in this form it has to take into account the error of SIB9 timing indication, as well as the detection error for the SFN boundary corresponding to the SIB9 timing. So, in the end then, the same error components as the other methods will apply. |
| OPPO | For Nokia’s comment above, as the figure provided in 3rd round shows, the k-th clock correction involving SIB9 ReferenceTimeInfo is based on four timing measurements on <UL-Tx timing error of SRS(nUL,TX), UL-Rx timing of SRS(mUL,RX), SFN timing of SIB9 at gNB(mDL,TX), SFN of SIB9 detection error at UE (nDL,RX)>. The total error of the implicit PDC is given by nUL,TX+mUL,RX+mDL,TX+nDL,RX+eqmUL,RX+eqmDL,TX2, where eqx is the quantization error for x (half of granularity on x). It can be seen that - ReferenceTimeInfo granularity related error (eqmDL,TX2) that was counted in RAN2 NW budget is already considered here in the Uu-interface error calculation (for a reduced granularity/error). So it should be recovered in NW budget. - Yes, the detection error for the SFN boundary corresponding to the SIB9 timing is included here, but it is subject to a coefficient equal to 1/2, instead of coefficient equal to 1 in other solutions. |
| Ericsson | Support |

Way forward on PDC in RAN1 for Rel-17

Nokia (R1-2109161) propose to send LS to RAN2 to inform the latest available status below.

Nokia (R1-2109161) On the way forward on PDC in RAN1 The meeting schedule for the RAN working groups involved in PDC in remainder of Rel-17 WI is as following: - RAN1 has one meeting remaining before the freeze deadline in RAN1: - November (RAN1#107-e 11/11-19/11). - RAN4 similarly has also only one meeting before the RAN1 deadline: - November (RAN4#101-e 01/11-12/11). - RAN2 has two meetings remaining before RAN2 freeze deadline: - November (RAN2#116-e 01/11-12/11). - February (RAN2#117-e 21/02-25-02) Currently RAN2 awaits RAN1 before they continue discussion on the remaining details on PDC. Clearly, which PDC methods to support is still pending discussion in RAN1. As RAN2 is the leading WG for PDC and given the end of Release-17 WI is near, RAN1 should target to send an LS to RAN2 with the latest status on the PDC method discussion as was also proposed by the FL in [1, Proposal 4.3.4-1]. That can be done independent on the outcome of this RAN1 meeting. From our point of view, the unclarity of which PDC methods should be supported comes when we attempt to address the most challenging use case which is the indoor control-to-control use case. However, it is already very clear from RAN1 point of view (from RAN1#104-bis-e) which option can be used for the smart grid use case and control-to-control with single Uu interface involved. Therefore one option that allows both RAN1 to continue studying PDC methods for the most strict use cases while also allowing RAN2 to progress on the signaling framework, is that RAN1 agrees to support legacy timing advance as one of the PDC methods, and sends an LS to RAN2 with firstly the latest status on PDC methods and informing RAN2 to specify the support for legacy timing advance. Proposal 1: To aid timely progress in RAN2, RAN1 should send an LS to RAN2 including: - The latest available status on PDC methods in RAN1. - Agreement that legacy timing advance based PDC can be supported for at least the less strict time sync use cases and informing that a supplementing PDC method for the most challenging time sync use cases are still being evaluated (either Rx-Tx based PDC method and/or enhanced timing advance).

Samsung (R1-2109485) propose the following.

Samsung (R1-2109485) Proposal: Adopt existing TA procedure for propagation delay compensation for smart grid scenario.

In RAN1#104bis-e meeting, RAN1 reached the following agreements.

Agreements: - Observation 1: Propagation delay compensation based on existing Rel-15/Rel-16 TA procedure and associated granularity, with no enhancements in RAN1, is sufficient for meeting the Uu interface synchronicity error budget in LS R2-2010837 for the smart grid scenario. - Observation 2: RAN1 needs to further study and specify the feasible enhancement (if any with RAN1 spec impact) for propagation delay compensation for control-to-control scenario, in order to meet the synchronicity budget of Uu interface in LS R2-2010837.

Feature lead: I do agree that it would be good for RAN1 to let RAN2 know the current status, then they can see what kind of work RAN2 can do first.

- 4.5.1 First round discussion

Based on the inputs and analysis above, the following proposals are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals before providing your views here.

Proposal 4.5.1-1: Send an LS to RAN2 with the content including:

- The latest available status on PDC methods in RAN1, e.g. key agreements achieved for TA-based PDC and RTT-based PDC.

- Inform RAN2

  - The legacy timing advance based PDC can be supported for at least the less strict time sync use cases, e.g. smart grid scenario.

  - Supplementing PDC method for the most challenging time sync use cases are still being evaluated (either Rx-Tx based PDC method and/or enhanced timing advance based PDC).

| Company | View |
| --- | --- |
| OPPO | We oppose the proposal, because it precludes implicit PDC with no technical reason and it also does not reflect the truth – implicit PDC is still being discussed and evaluated in RAN1. >> Feature lead If there is any progress for implicit PDC, we can include also. |
| Nokia, NSB | Support |
| Vivo | Support. |
| ZTE | Support |
| Intel | Support. Expect to send this LS by end of the meeting to include the latest progress |
| HW/HiSi | Fine with the LS |
| Qualcomm | Support. |
| Ericsson | We do not object to sending LS at the end of the meeting is something important from this meeting should be informed to RAN2. However, the two sub-bullets under “inform RAN2” does not seem necessary. RAN2 is already aware of this status. >> Feature lead I think it is still beneficial to include the “inform RAN2” part, to at least tell RAN2 that at least the existing TA-based PDC will be applied to some cases, then if any RAN2 work on it they can continue. For example, signalling to disable UE side PDC. But I agree it is not that critical, companies can tell RAN2 colleagues directly. If no strong concern, we can delete it. |

- 4.5.1a Second round discussion

Based on the inputs and analysis above, the following proposals are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals before providing your views here.

Revised Proposal 4.5.1-1: Send an LS to RAN2 and CC RAN4 with the content including:

- The latest available status on PDC methods in RAN1, e.g. key agreements achieved for TA-based PDC and RTT-based PDC.

- Inform RAN2

  - The legacy timing advance based PDC can be supported for at least the less strict time sync use cases, e.g. smart grid scenario.

  - Supplementing PDC method for the most challenging time sync use cases are still being evaluated (either Rx-Tx based PDC method and/or enhanced timing advance based PDC).

| Company | View |
| --- | --- |
| Intel | Agree |
| OPPO | Fine with proposal. If no other LS is sent to RAN4, this LS should cc to RAN4 as well. >> Feature lead Agree and update accordingly above for companies to check. |
| Ericsson | Support. Also agree with OPPO to cc RAN4 |
| Vivo | Agree. Also agree to cc to RAN4. |
| Nokia, NSB | We prefer the previous proposal to also ‘inform RAN2’, and agree to cc RAN4. We think it is highly beneficial for RAN2 to also get the "Inform RAN2" part. As the FL said in the comment to Ericsson, we already agree in RAN1 that legacy TA is sufficient for the less strict use cases but will need to be supplemented by an additional procedure for the strictest use cases. This information affects the activation/deactivation scheme to be designed in RAN2. We disagree that RAN2 is already aware on the status in RAN1 as there has been no official cross WG communication stating this, and per RAN2#115 agreements RAN2 is expecting guidance from RAN1. Then, based on what we are informing RAN2, RAN1 should respectfully request RAN2 to take an action at least regarding the points RAN1 has clearly agreed. It is clear that, if either enhanced TA or RTT based PDC is applied for the less strict use cases, these two more accurate methods will only be overhead compared to already available legacy TA. So, if we have a method that can already support some use cases and has no additional procedure or overhead, why not let RAN2 start enabling it while RAN1 works on the complementary method for the stricter use cases? >> Feature lead Let’s hear more views from companies, then we can see how to adjust the proposals. I do agree with you though as I explained above in section 4.5.1. Let’s try one more time with some modification. |
| Samsung | Agree to CC RAN 4. |
| ZTE | We are fine with the proposal. |
| HW/HiSi | Fine with the proposal and CC to RAN4 |
| Samsung | Support the proposal |

- 4.5.1b Third round discussion

Based on the inputs and analysis above, the following proposals are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals before providing your views here.

Revised Proposal 4.5.1-1: Send an LS to RAN2 and CC RAN4 with the content including:

- The latest available status on PDC methods in RAN1, e.g. key agreements achieved for TA-based PDC and RTT-based PDC.

- Inform RAN2

  - The legacy timing advance based PDC can be supported for at least the less strict time sync use cases, e.g. smart grid scenario.

  - Supplementing PDC method for the most challenging time sync use cases are still being evaluated (either Rx-Tx based PDC method and/or enhanced timing advance based PDC).

| Company | View |
| --- | --- |
| Nokia/NSB | Support. Note: the last sentence strike-out could be reverted (brought back) in case we are able to agree on the potential conclusion in 4.4.1b Potential conclusion: Drop implicit PDC in Rel-17. |
| Intel | Support |
| OPPO | Support |
| Ericsson | Support first bullet. The second bullet is about the observations made in RAN1#104bis (April 2021). In our understanding, RAN2 is fully aware of RAN1 observations from April (there have been two RAN plenary since then), and there is no need to send. Alternatively, we can be OK to include the old observations below. Agreements: (RAN1#104bis) - Observation 1: Propagation delay compensation based on existing Rel-15/Rel-16 TA procedure and associated granularity, with no enhancements in RAN1, is sufficient for meeting the Uu interface synchronicity error budget in LS R2-2010837 for the smart grid scenario. - Observation 2: RAN1 needs to further study and specify the feasible enhancement (if any with RAN1 spec impact) for propagation delay compensation for control-to-control scenario, in order to meet the synchronicity budget of Uu interface in LS R2-2010837.. >> Feature lead My original intention is that all key agreements we achieved so far will be included, including the observations above. In my understanding, the second bullet still provides more information to RAN2 compared to the observations above, i.e. to tell RAN2 that at least legacy timing advance based PDC can be supported. |
| Qualcomm | Support |
| Samsung | If possible, we can highlight that, whether RTT based solution can meet the requirement might depends on some requirement defined in RAN 4. And if can be agreed, we think we can ask RAN 4 for the input of and Y together. >> Feature lead The proposal on and Y is as discussed in section 4.3.6b/c, please check the discussion there. If we get the proposal there agreed, we can include all to one LS. |
| Vivo | Support. |
| ZTE | Support |
| HW/HiSi | Support |
| LG | Support |

Summary of the status for revised proposal 4.5.1-1 in third round

| Support | Nokia, NSB, OPPO, Huawei, HiSilicon, Qualcomm, vivo, ZTE, LG, Intel |
| --- | --- |
| Not support | Ericsson |

- 4.5.1c 4th round discussion

Based on the inputs and analysis above, the following proposals are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals before providing your views here.

Revised Proposal 4.5.1-1: Send an LS to RAN2 and CC RAN4 with the content including:

- The latest available status on PDC methods in RAN1, e.g. key agreements achieved for TA-based PDC and RTT-based PDC.

- Inform RAN2

  - The legacy timing advance based PDC can be supported for at least the less strict time sync use cases, e.g. smart grid scenario.

  - Supplementing PDC method for the most challenging time sync use cases are still being evaluated (either Rx-Tx based PDC method and/or enhanced timing advance based PDC).

| Company | View |
| --- | --- |
| Feature lead | @ Ericsson All companies agree with the above proposal except Ericsson. Please Ericsson check replies to your comment above and see if you can accept the proposal. |
| Nokia, NSB | Support |
| Intel | Support |
| HW/HiSi | Support |
| Ericsson | Do not support the bullets under “Inform RAN2”. With FL explanation, now the intention is clarified that Proposal above tells RAN2 to support legacy TA-based PDC. We don’t support this. The reason is, it is simpler if Rel-17 introduces only one set of method/procedure to cover a ranges of scenarios, including scenarios other than control-to-control and smart grid. In our view, the method to be decided for control-to-control case can be applied to the smart grid scenario also. For different scenarios with different accuracy requirements, different RS parameters (e.g., BW) can be configured if desired. Thus there is no need to have two sets of methods/procedures in Rel-17. |

- References

- RP-201310, Revised WID: Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR , Nokia, Nokia Shanghai Bell

- R1-2100024 Reply LS on propagation delay compensation enhancements

- R1-2108833	Propagation Delay Compensation Enhancements for Time Synchronization	Ericsson

- R1-2108844	Discussion on propagation delay compensation enhancements	ZTE

- R1-2108970	Discussion on propagation delay compensation enhancements	vivo

- R1-2109097	Enhancement for support of time synchronization	OPPO

- R1-2109161	Discussion on enhancements for propagation delay compensation	Nokia, Nokia Shanghai Bell

- R1-2109219	Discussion on propagation delay compensation enhancements	CATT

- R1-2109485	Discussion for propagation delay compensation enhancements	Samsung

- R1-2109608	Remaining issues on propagation delay compensation	Intel Corporation

- R1-2109743	Enhancements for support of time synchronization	Huawei, HiSilicon

- R1-2109974	Discussion on propagation delay compensation enhancements	LG Electronics

- R1-2110182	Enhancements for support of time synchronization for enhanced IIoT and URLLC	Qualcomm Incorporated

- R1-2110397	Discussion on propagation delay compensation enhancements	ZTE

Revision of R1-2108844

- 3GPP RAN1#105-e, R1-2104171, Reply LS on UE transmit timing error

- Appendix Agreements in the past meetings

RAN1#102-e

Agreements:

- Take the following use cases as the representative use cases for further study on propagation delay compensation enhancements in Rel-17.

| User-specific clock synchronicity accuracy level | Number of devices in one Communication group for clock synchronisation | 5GS synchronicity budget requirement (note) | Service area | Scenario |
| --- | --- | --- | --- | --- |
| 2 | Up to 300 UEs | ≤900 ns | ≤ 1000 m x 100 m | - Control-to-control communication for industrial controller |
| 4 | Up to 100 UEs | <1 µs | < 20 km2 | - Smart Grid: synchronicity between PMUs |

Agreements:

- ± 8∙64∙Tc/2μ8*64*Tc/2 as the TA indicating error is assumed in the evaluation.

Agreements:

For 5GS synchronicity budget requirement,

- One Uu interface is assumed for smart grid.

- Two Uu interfaces are assumed for control-to-control.

Agreements:

For BS transmit timing error, further study the following three options:

- Option 1: 65 ns

- Option 2:±130ns for the indoor scenario and ±200ns for the smart grid scenario

- Option 3:82.5 ns

Agreements:

The value defined in Table 7.1.2-1 for initial transmit timing error (Te) in TS 38.133 should be considered for evaluation of the time synchronization.

Agreements:

Asymmetry between downlink and uplink channel for control-to-control scenario is not considered.

Agreements:

100 ns is assumed for BS detecting error.

Agreements:

Timing advance adjustment accuracy defined in Table 7.3.2.2-1 in TS 38.133 is assumed for evaluation of the time synchronization.

Agreements:

Both 15 kHz and 30 kHz are assumed for both control-to-control and smart grid for evaluation of the time synchronization.

Agreements:

Send an LS to RAN2 with the content including

- Inform RAN2 the two representative use cases concluded in RAN1 for further study;

- Ask RAN2 for input about Uu interface error budget for each of the two use cases;

Agreements:

The following options for propagation delay compensation are further studied in RAN1

- Option 1: TA-based propagation delay

  - Option 1a: Propagation delay estimation based on legacy Timing advance (potentially with enhanced TA indication granularity).

  - Option 1b: Propagation delay estimation based on timing advanced enhanced for time synchronization (as 1a but with updated RAN4 requirements to TA adjustment error and Te)

  - Option 1c: Propagation delay estimation based on a new dedicated signaling with finer delay compensation granularity (Separated signaling from TA so that TA procedure is not affected)

- Option 2: RTT based delay compensation:

  - Propagation delay estimation based on an RAN managed Rx-Tx procedure intended for time synchronization (FFS to expand or separate procedure/signaling to positioning).

Draft LS R1-2007445 is approved, with final LS in R1-2007446.

RAN1#103-e

Agreements:

- Take 65 ns as the assumption of transmit timing error for evaluation of the overall time synchronization error for control-to-control.

- Asymmetry between downlink and uplink channel for smart grid scenario is not considered.

- TA adjustment accuracy is not considered for the evaluation of time synchronization error.

- errorBS,DL,TXerrorBS, DL, TX is included in the equation for calculating the overall time synchronization error.

Agreements:

TA adjustment accuracy is not considered for the evaluation of time synchronization error.

Agreements:

For evaluation of the overall time synchronization error for smart grid, companies can take one of the following two options as the assumption for BS transmit timing error:

- Option 1: 200 ns

- Option 2: 65 ns

RAN1#104-e

Agreements: Take ±100 ns as the assumption for downlink frame timing detection error (errorUE,DL,RXerrorUE, DL, RX) at the UE for evaluation of the overall time synchronization error for TA based propagation delay compensation, if downlink frame timing detection error needs to be considered separately.

- Send a LS to RAN4 to ask for clarification on whether downlink frame timing detection error is included in Te or not

  - In the LS, to include more details about option 1 (included) & option 2 (not included); also including the necessary background

- FFS whether to apply the same value to RTT-based propagation delay compensation, and the corresponding condition (if any) if the same value will be applied

Decision: As per email posted on feb 5th, the draft LS is endorsed. Final LS is approved in R1-2102245.

RAN1#104b-e

Agreements: If downlink frame timing detection error needs to be considered separately from propagation delay estimation error, take ±100 ns as the assumption for downlink frame timing detection error (errorUE,DL,RX) at the UE for evaluation of the overall time synchronization error for RTT based propagation delay compensation

Agreements: Take the following equation for evaluation of the DL propagation delay estimation error for TA based propagation delay compensation:

- Either option 1 or option 2 below will be applied based on the RAN4 reply to RAN1 LS R1-2102245.

- FFS whether errorBS,DL,TX in the above equation should be included or not.

Agreements:

- Observation 1: Propagation delay compensation based on existing Rel-15/Rel-16 TA procedure and associated granularity, with no enhancements in RAN1, is sufficient for meeting the Uu interface synchronicity error budget in LS R2-2010837 for the smart grid scenario.

- Observation 2: RAN1 needs to further study and specify the feasible enhancement (if any with RAN1 spec impact) for propagation delay compensation for control-to-control scenario, in order to meet the synchronicity budget of Uu interface in LS R2-2010837.

Working assumption:

Agreement:

Take the following as the evaluation assumptions for both RTT-based PDC and TA-based PDC.

- The UE may acquire an up-to-date PD estimation after waking up from DRX. This implies that gNB may signal an update timing advance value or complete a Rx-Tx measurement procedure.

- errorUE,DL,RXerrorUE, DL, RX is based on other signals (e.g. CSI-RS) instead of SSB.

- errorBS, UL,RXerrorUE, DL, RX iss based on other uplink signals instead of contention based PRACH, e.g. SRS.

- Further study and specify new procedure/signaling (if necessary) to ensure that the PD estimation can be acquired after DRX for the adopted PDC method.

Agreement:

Existing DL reference signal(s) are used for Rx – Tx time difference estimation at UE side for RTT-based propagation delay compensation, if RTT-based propagation delay compensation is supported.

- FFS whether PRS can be used for UE Rx – Tx time difference estimation or not

- FFS which DL reference signal(s) to be used if/when PRS is not used

Conclusion:

- Leave it to RAN2 to decide whether to support UE based compensation and/or gNB based compensation for any propagation delay compensation method RAN1 may adopt for Rel-17, if applicable.

RAN1#106-e

Agreement

SRS can be used for Rx – Tx time difference estimation at gNB side for RTT-based propagation delay compensation, if RTT-based propagation delay compensation is supported.

Agreement

Send LS to RAN4 to ask for feedback on the following questions:

- Question 1: Is it feasible to support a smaller value than the current Te for the use of propagation delay compensation, assuming the existing conditions in TS 38.133 for Te requirement? If not, is it feasible under new conditions (e.g. using TRS instead of SSB)? If the answer is yes, please also provide feedback on how much it can be reduced at most.

- Question 2: Is it feasible to introduce enhanced TA command indication granularity? If the answer is yes, please also provide feedback on how much it can be reduced at most (e.g. reduced to (1/16)* (16*64*Tc/2)) similar as the granularity for Rel-16 IAB based on the Timing Delta MAC CE and related condition.

- Note 1: The alternatives in the working assumption achieved in RAN1#104bis-e together with the examples in Table 4.2-2 will be included in the LS to give some background for RAN4

- Note 2: The agreement “both SCS 15 kHz and 30 kHz are assumed for both control-to-control and smart grid for evaluation of the time synchronization” achieved in RAN1#102-e will be included in the LS for RAN4 information also.

- Note 3: Inform RAN4 that the enhancements on Te and TA command indication granularity for propagation delay compensation may or may not have impact on normal TA related procedure, depending on which candidate option for TA-based PDC is adopted. Note that this is just for RAN4 information.

- Note 4: Whether RAN1 will introduce specification enhancements is still undetermined.

Agreement

If RTT-based propagation delay compensation is supported,

- CSI-RS for tracking (TRS) can be used for Rx – Tx time difference estimation at UE side, if PRS is not configured for the UE.

- PRS can be used for Rx – Tx time difference estimation at UE side, if PRS is configured for the UE.

Agreement

Send LS to RAN4 to ask for defining the following for RTT-based propagation delay compensation, if RTT-based propagation delay compensation is supported.

- UE Rx-Tx time difference measurement accuracy errorUE,RxTxDiff based on CSI-RS for tracking

- gNB Rx-Tx time difference absolute accuracyerrorgNB,RxTxDiff errorUE,RxTxDiff based on SRS

R1-2108513	Feature lead summary on propagation delay compensation enhancements	Moderator (Huawei)

Agreement

Support the following configurations for RTT-based propagation delay compensation, if RTT-based propagation delay compensation is supported.

- At least one CSI-RS for tracking (TRS) configuration for Rx – Tx time difference estimation at UE side if PRS is not configured

- At least one SRS configuration for Rx – Tx time difference estimation at gNB side

Agreement

If RTT-based propagation delay compensation is supported and performed at the UE side, the Rx-Tx measurement report provided from the gNB to the UE should include at least:

- gNB Rx-Tx time difference at a given granularity

- FFS whether to include SRS-Resource-ID

Agreement

Take the following two alternatives as the equation for evaluation of the overall time synchronization error for RTT-based propagation delay compensation. RAN1 to select one of the alternatives in RAN1#106bis-e.

- Alt. 1:

  - errorRxTxDiff, indication is to reflect the error due to indication granularity of Rx-Tx time difference

  - errorgNB,RxTxDiff and errorUE, RxTxDiff reflects the measurement inaccuracy of gNB Rx-Tx time difference, and the measurement inaccuracy of UE Rx-Tx time difference, respectively.

  - Note: The equation may be updated after clarification on the gNB TX-RX timing difference and UE TX-RX timing difference

- Alt. 2:

  - errorRxTxDiff,indication is to reflect the error due to indication granularity of Rx-Tx time difference

  - Note: Alt.2 assumes that gNB can coordinate the time of TA procedure and the time of PD compensation, so that the DL frame timing error and BS transmit timing error for propagation delay estimation is correlated to (e.g. the same as) that for the transmission of RRC signaling carrying the reference time clock

Note: FFS whether / how to handle inconsistent RTT measurement in gNB and UE due a change of uplink TX timing

R1-2108618 Draft LS on TA-based propagation delay compensation 	Moderator (Huawei)

Decision: The draft LS is endorsed with the following note

- Note: It’s pending further discussion in RAN1 whether the WA is to be confirmed including which alternative is to be selected

Final LS is approved in R1-2108635.