3GPP TSG RAN WG1 Meeting #106b-e			R1-21xxxxx

e-Meeting, October 11th – 19th, 2021

Title:	Draft LS on propagation delay compensation

Response to:	-

Release:	Rel-17

Study Item:	NR_IIOT_URLLC_enh

Source:	Huawei [TSG RAN WG1]

To:	TSG RAN WG2

Cc:	TSG RAN WG4

Contact Person:

#### Name:	Yan Cheng

E-mail Address:	chengyan.cheng@huawei.com

1. Overall Description:

RAN1 discussed propagation delay compensation enhancements and the key related agreements are copied here for informing RAN2 the latest available status on PDC methods in RAN1:

Common key agreements

Agreements The following options for propagation delay compensation are further studied in RAN1 - Option 1: TA-based propagation delay - Option 1a: Propagation delay estimation based on legacy Timing advance (potentially with enhanced TA indication granularity). - Option 1b: Propagation delay estimation based on timing advanced enhanced for time synchronization (as 1a but with updated RAN4 requirements to TA adjustment error and Te) - Option 1c: Propagation delay estimation based on a new dedicated signaling with finer delay compensation granularity (Separated signaling from TA so that TA procedure is not affected) - Option 2: RTT based delay compensation: - Propagation delay estimation based on an RAN managed Rx-Tx procedure intended for time synchronization (FFS to expand or separate procedure/signaling to positioning). Agreements Both 15 kHz and 30 kHz are assumed for both control-to-control and smart grid for evaluation of the time synchronization. Conclusion: - Leave it to RAN2 to decide whether to support UE based compensation and/or gNB based compensation for any propagation delay compensation method RAN1 may adopt for Rel-17, if applicable.

Key agreements specific for TA-based PDC

Agreements - Observation 1: Propagation delay compensation based on existing Rel-15/Rel-16 TA procedure and associated granularity, with no enhancements in RAN1, is sufficient for meeting the Uu interface synchronicity error budget in LS R2-2010837 for the smart grid scenario. - Observation 2: RAN1 needs to further study and specify the feasible enhancement (if any with RAN1 spec impact) for propagation delay compensation for control-to-control scenario, in order to meet the synchronicity budget of Uu interface in LS R2-2010837. Agreement (Note: Working assumption from RAN1#104b-e and agreements from RAN1#106b-e are merged) For evaluation of the overall time synchronization error for TA-based propagation delay compensation, - Alt.1 for TA-based PDC Agreement Send LS to RAN4 to ask for feedback on the following questions: - Question 1: Is it feasible to support a smaller value than the current Te for the use of propagation delay compensation, assuming the existing conditions in TS 38.133 for Te requirement? If not, is it feasible under new conditions (e.g. using TRS instead of SSB)? If the answer is yes, please also provide feedback on how much it can be reduced at most. - Question 2: Is it feasible to introduce enhanced TA command indication granularity? If the answer is yes, please also provide feedback on how much it can be reduced at most (e.g. reduced to (1/16)* (16*64*Tc/2)) similar as the granularity for Rel-16 IAB based on the Timing Delta MAC CE and related condition. - Note 1: The alternatives in the working assumption achieved in RAN1#104bis-e together with the examples in Table 4.2-2 will be included in the LS to give some background for RAN4 - Note 2: The agreement “both SCS 15 kHz and 30 kHz are assumed for both control-to-control and smart grid for evaluation of the time synchronization” achieved in RAN1#102-e will be included in the LS for RAN4 information also. - Note 3: Inform RAN4 that the enhancements on Te and TA command indication granularity for propagation delay compensation may or may not have impact on normal TA related procedure, depending on which candidate option for TA-based PDC is adopted. Note that this is just for RAN4 information. - Note 4: Whether RAN1 will introduce specification enhancements is still undetermined.

Key agreements specific for RTT-based PDC

Agreement (Note: Agreements from RAN1#106-e and RAN1#106b-e are merged) For evaluation of the overall time synchronization error for RTT-based propagation delay compensation, - Alt.1 for RTT-based PDC Agreement For evaluation of the overall time synchronization error for RTT-based propagation delay compensation with Alt.1, it is assumed that - The UE Rx-Tx time difference measurement accuracy based on PRS defined in Table 10.1.25.2-2 in TS 38.133 v17.3.0 is taken as the reference for the UE Rx-Tx time difference measurement accuracy - The gNB Rx-Tx time difference accuracy based on SRS for positioning defined in Table 13.2.2.2-1 in TS 38.133 v17.3.0 is taken as the reference for the gNB Rx-Tx time difference accuracy based on SRS for PDC Agreement SRS can be used for Rx – Tx time difference estimation at gNB side for RTT-based propagation delay compensation, if RTT-based propagation delay compensation is supported. Agreement If RTT-based propagation delay compensation is supported, - CSI-RS for tracking (TRS) can be used for Rx – Tx time difference estimation at UE side, if PRS is not configured for the UE. - PRS can be used for Rx – Tx time difference estimation at UE side, if PRS is configured for the UE. Agreement Support the following configurations for RTT-based propagation delay compensation, if RTT-based propagation delay compensation is supported. - At least one CSI-RS for tracking (TRS) configuration for Rx – Tx time difference estimation at UE side if PRS is not configured - At least one SRS configuration for Rx – Tx time difference estimation at gNB side Agreement If RTT-based propagation delay compensation is supported and performed at the UE side, the Rx-Tx measurement report provided from the gNB to the UE should include at least: - gNB Rx-Tx time difference at a given granularity - FFS whether to include SRS-Resource-ID Agreement If RTT-based propagation delay compensation is supported and performed at the gNB side, the Rx-Tx measurement report provided from the UE to the gNB should include at least: - UE Rx-Tx time difference at a given granularity Agreement Send LS to RAN4 to ask for defining the following for RTT-based propagation delay compensation, if RTT-based propagation delay compensation is supported. - UE Rx-Tx time difference measurement accuracy errorUE,RxTxDiff based on CSI-RS for tracking - gNB Rx-Tx time difference absolute accuracy errorUE,RxTxDiff based on SRS Agreement For RTT-based PDC, only a single pair of CSI-RS for tracking (TRS)/PRS and SRS configuration, i.e. one CSI-RS for tracking (TRS)/PRS configuration for Rx – Tx time difference estimation at UE side and one SRS configuration for Rx – Tx time difference estimation at gNB side, is configured for PDC in Rel-17, if RTT-based PDC is supported.

Note: Some agreements on detailed evaluation assumptions for PDC are not included here.

2. Actions:

To RAN2

ACTION: 	RAN1 respectfully asks RAN2 to take the above information into account.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #107-e 	11 – 19 November 2021		Electronic Meeting