3GPP TSG RAN WG1 #106bis-e			    R1-2110642

e-Meeting, October 11th – 19th, 2021

Title:	Reply LS on PUCCH and PUSCH repetition

Response to: 	R4-2114991, Reply LS on PUCCH and PUSCH transmissions

Release:	Rel-17

Work Item:	NR_cov_enh

Source:	RAN1

To:	RAN4

Cc:

Attachments:

# Contact Person:

# Name:	                Yi Huang

E-mail Address:     yihuang@qti.qualcomm.com

# 1. Overall Description:

In R1-2108703(R4-2114991) “Reply LS on PUCCH and PUSCH transmissions”, RAN 4 asked the following question to RAN1:

- What are the consequences if phase continuity cannot be maintained in the case of UL transmissions from other signals/channels in the repetition gap?

The following is the answer from RAN 1 to RAN 4, regarding the above question,

- If phase continuity cannot be maintained in the case of UL transmissions of other signals/channels in the repetition gap, then DM-RS symbols transmitted before and after the transmission of such other signals/channels cannot be part of the same bundle from UE perspective. A new DMRS bundle may or may not start after the other UL signals/channel transmission in the repetition gap. Details are still under discussion in RAN1.

# 2. Actions:

RAN1 respectfully asks RAN4 to take the above information into account in their work on NR coverage enhancement.

# 3. Date of Next RAN1 Meetings:

TSG-RAN WG1 Meeting #107-e 	11 – 19 Nov 2021		Electronic

TSG-RAN WG1 Meeting #107bis-e 	17 – 25 Jan 2022		              Electronic