3GPP TSG RAN WG1 #107-e	R1-2110825

# e-Meeting, November 11th – 19th, 2021‎

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR |  | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on CG-UCI multiplexing in TS38.212 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-Core |  |  |  |  |  | Date: |  |  | 2021-11-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | CG-UCI is always together with UL-SCH. So sub-conditional of “or if both HARQ-ACK and CG-UCI are present on the same PUSCH with UL-SCH,” of condition “if HARQ-ACK, CSI part 1 and CSI part 2 are present for transmission on the PUSCH without UL-SCH” is contraditory with each other. The multiplexing mechanism defined under conditon of “if CG-UCI is present for transmission on the PUSCH with UL-SCH and without HARQ-ACK” leaves no room for PUSCH carrying UL-SCH. On the other side, the combination of multiplexing mechanism defined with the following two conditions are sufficient to cover the above condition: - if CSI is present for transmission on the PUSCH with UL-SCH - if CG-UCI is present for transmission on the PUSCH with UL-SCH and without HARQ-ACK |  |  |  |  |  |  |  |  |
| Summary of change: |  | Delete the sub condition of “or if both HARQ-ACK and CG-UCI are present on the same PUSCH with UL-SCH,” under the condition of “if HARQ-ACK, CSI part 1 and CSI part 2 are present for transmission on the PUSCH without UL-SCH,” Delete the multiplexing mechanism for condition “if CG-UCI is present for transmission on the PUSCH with UL-SCH and without HARQ-ACK. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Error calculation of CSI-part-1 and CSI-part-2 when multiplexed with CG-UCI and UL-SCH. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 6.2.7	Data and control multiplexing

< Unchanged part is omitted >

If frequency hopping is configured for the PUSCH,

-	denote  as the OFDM symbol index of the first OFDM symbol after the first set of consecutive OFDM symbol(s) carrying DMRS in the first hop;

-	denote  as the OFDM symbol index of the first OFDM symbol after the first set of consecutive OFDM symbol(s) carrying DMRS in the second hop.

-	denote  as the OFDM symbol index of the first OFDM symbol that does not carry DMRS in the first hop;

-	denote  as the OFDM symbol index of the first OFDM symbol that does not carry DMRS in the second hop;

-	if HARQ-ACK is present for transmission on the PUSCH with UL-SCH or if both HARQ-ACK and CG-UCI are present on the same PUSCH with UL-SCH, let

-	 and ;

-	if CSI is present for transmission on the PUSCH with UL-SCH, let

-	;

-	;

-	; and

-	;

-	if CG-UCI is present for transmission on the PUSCH with UL-SCH and without HARQ-ACK, let

-	GCG-UCI1=NL∙Qm∙GCG-UCI2∙NL∙Qm and GCG-UCI2=NL∙Qm∙GCG-UCI2∙NL∙Qm

-	if only HARQ-ACK and CSI part 1 are present for transmission on the PUSCH without UL-SCH, let

-	;

-	;

-	; and

-	;

-	if HARQ-ACK, CSI part 1 and CSI part 2 are present for transmission on the PUSCH without UL-SCH, let

-	;

-	;

-	if the number of HARQ-ACK information bits is more than 2,; otherwise,

-	;

-	 if the number of HARQ-ACK information bits is no more than 2, and  otherwise; and

-	 if the number of HARQ-ACK information bits is no more than 2, and  otherwise;

GCSI-part11=minNL∙Qm∙GCSI-part12∙NL∙Qm,  M1∙NL∙Qm-GCG-UCI1

GCSI-part12= GCSI-part1-GCSI-part11

GCSI-part21=M1∙NL∙Qm-GCG-UCI1-GCSI-part11

GCSI-part22=M2∙NL∙Qm-GCG-UCI2-GCSI-part12

-	if only CSI part 1 and CSI part 2 are present for transmission on the PUSCH without UL-SCH, let

-	;

-	;

-	; and

-	;

-	let , and denote ,  as the number of OFDM symbols of the PUSCH in the first and second hop, respectively;

-	 is the number of transmission layers of the PUSCH;

-	 is the modulation order of the PUSCH;

-	;

-

-	.

< Unchanged part is omitted >