3GPP TSG RAN WG1 Meeting #107-e 	R1-2110836

e-Meeting, November 11th – 19th, 2021

Agenda Item:	8.16.10

Source:	Huawei, HiSilicon

Title:	Rel-17 UE features for IAB enhancements

Document for:	Discussion and Decision

Introduction

In RAN1#106bis-e, a list of Rel-17 IAB UE features in R1-2109916 [1] were agreed as baseline for further discussion. In this paper, we provide some further considerations on Rel-17 UE features for IAB enhancements.

UE feature on Case 7 timing

Guard symbols

In Rel-17, guard symbols enhancements are considering the adaption of the Case 6 and/or Case7 timing. Regarding the components of FG 31-1, we suggest to remove the brackets on Rel-17 ProvidedGuardSymbols and Rel-17 DesiredGuardSymbols.

Beam restriction and recommendation

Feature group 31-2 is intended for IAB-MT to receive the indication on restricted beam(s) from parent node, which the restricted beam referred to beams at the IAB-DU. From IAB-MT perspective, the name of this FG can be update as “IAB-DU beam restriction indication”.

FG 31-3 has the same issue. Feature group 31-3 is for beam recommendation indication from IAB MT. From IAB-MT perspective, the name of this FG can be updated as “IAB-MT beam recommendation indication”.

DL TX power adjustment

For FG 31-6, there are two components: 1.) Support [Desired DL TX Power Adjustment] reporting; 2) support [DL TX Power Adjustment] reception. We suggest to remove the brackets of components as it precisely describes this FG. For the consequence, “parent-node’s DL TX power adjustment reporting” should be updated as “Desired parent-node’s DL TX power adjustment reporting and DL TX power adjustment reception is not supported”.

Dynamic indication of Rel-17 or FDM soft resource availability

On FG 31-9, the consequence should be ““The IAB node cannot receive explicit availability indication for Rel-17 FDM soft resources” since the IAB node may still receive explicit availability indication for Rel-16 soft resources depending on capability.

Conclusions

Based on the discussion, we have the following proposal:

Proposal: Agree on the updated UE features for IAB in the Appendix.

- References

# R1- 2109916, Summary of UE features for IAB enhancements, Moderators (AT&T), RAN1#106bis-e, Oct. 11-19, 2021.

# Appendix

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 31. NR_IAB_enh | 31-1 | Guard symbols | 1) Support [Rel-17 DesiredGuardSymbols] reporting 2) Support [Rel-17 ProvidedGuardSymbols] reception |  | Yes | N/A | Guard symbols reporting and reception associated with Case 6 and 7 timings are not supported | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact Note: If an IAB node does not support a certain timing mode, the reported/provided values shall be ignored | Optional with capability signalling |
| 31. NR_IAB_enh | 31-2 | Restricted DU beam indication [Child IAB-DU beam restriction indication] | Support [Simultaneous Operation DU Beam Indication] reception |  | Yes | N/A | Parent-node cannot indicate restricted beams at the IAB-DU. | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling |
| 31. NR_IAB_enh | 31-3 | Recommended/[Non-preferred] MT beam indication [Child IAB-MT beam recommendation indication] | Support [Simultaneous Operation MT Beam Indication] transmission 1) [IAB-MT DL beam 2) IAB-MT UL beam |  | Yes | N/A | IAB-node cannot indicate recommended/[non-preferred] MT DL/UL beam to parent node | Per IAB-node | no | no | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling |
| 31. NR_IAB_enh | 31-4 | Timing case indication [Case 6 timing alignment] | Support [Case 6 Timing Case Indication] reception transmission] |  | Yes | N/A | Case 6 timing at the IAB-node is not supported. Switching across different timing cases (i.e., Case 1 at IAB-node, Case 6 at IAB-node, and/or Case 7 at the Parent) is not supported. | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling |
| 31. NR_IAB_enh | 31-5 | Case 7 timing [offset alignment] | Support [Case7 transmission] - support Case 7 timing; - switching between Case 7 and other timing case. |  | Yes | N/A | Parent-node cannot adopt both (and switch between) Case 1 and Case 7 timing. | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling |
| 31. NR_IAB_enh | 31-6 | DL TX power adjustment | 1.) Support [Desired DL TX Power Adjustment] reporting 2) support [DL TX Power Adjustment] reception |  | Yes | N/A | Parent-node’s Desired DL TX power adjustment reporting and reception is not supported | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling |
| 31. NR_IAB_enh | 31-8 | [Desired] UL TX power adjustment | Support [Desired IAB-MT PSD range] reporting |  | Yes | N/A | Desired MT’s UL PSD range reporting is not supported. | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling |
| 31. NR_IAB_enh | [31-9] | [Dynamic indication of Rel-17 or FDM soft resource availability] | Support monitoring DCI Format 2_5 scrambled by AI-RNTI for indication of FDM soft resource availability to an IAB node |  | Yes | N/A | Desired MT’s UL PSD range reporting is not supported The IAB-node is unable to receive explicit availability indication for Rel-17 FDM Soft resources | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling |
| 31. NR_IAB_enh | 31-10 | Simultaneous transmission and reception from multiple parent nodes | Support simultaneous transmission and reception from multiple parent nodes |  | Yes | N/A | Simultaneous transmission and reception is not supported in DC scenario | per BC | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |