3GPP TSG-RAN WG1 Meeting #107-e	R1-2110842

# e-Meeting, November 11th – 19th, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | XXX | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on processing time for simultaneous SL and UL transmissions/receptions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2021-11-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed e?planations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For simultaneous NR and E-UTRA transmission/reception, it is specified that the priorities of the channels/signals are known to both E-UTRA radio access and NR radio access at the UE T msec prior to the start of the earliest of the two transmissions to allow sufficient time to decide which RAT is prioritized. However, a similar processing time constraint has been omitted for simultaneous SL and UL transmissions/receptions. The gNB cannot know when it needs to schedule a Uu DCI and/or SL DCI for mode 1 such that UE is expected to process both in time for transmission/reception according to priority rules. The UE needs time to decode both NR DCIs and prepare the transmission/reception on the prioritized link. The minimum value of sl-DCI-ToSL-Trans is one slot, and similar for PUSCH preparation time on Uu. The maximum processing time needed is no more than 3 slots. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add that priorities of UL and SL transmissions/receptions are known at the UE T slots prior to the start of the earliest of the UL and SL transmissions/receptions, where T≤3 and is based on UE implementation |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | gNB cannot know when it needs to schedule a Uu DCI or SL DCI for mode 1 such that UE is expected to process both in time for transmission/reception according to priority rules. Control and data channel resources may be wasted on UL or SL. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.2.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged parts omitted>

#### 16.2.4.3	Simultaneous SL and UL transmissions/receptions

If a UE

-	would simultaneously transmit on the UL and on the SL in a carrier or in two respective carriers, and

-	the UE is not capable of simultaneous transmissions on the UL and on the SL in the carrier or in the two respective carriers, and

-	the priorities of both UL and SL transmissions are known at the UE T slots prior to the start of the earliest of the two transmissions, where T≤3 and is based on UE implementation.

the UE transmits only on the link, UL or SL, with the higher priority.

If a UE

-	would simultaneously transmit on the UL and receive on the SL in a carrier, or

-	would simultaneously transmit on the UL and receive on the SL in two respective carriers and the UE is not capable of simultaneous transmission on the UL and reception on the SL in the two respective carriers, and

-	the priorities of the UL transmission and SL reception are known at the UE T slots prior to the start of the earliest of the UL transmission and SL reception, where T≤3 and is based on UE implementation.

the UE transmits on UL or receives on SL, with the higher priority.

<Unchanged parts omitted>