3GPP TSG RAN WG1 Meeting #107-e	R1-2110855

e-Meeting, November 11th – 19th, 2021

Agenda Item:	8.16.5

Source:	Huawei, HiSilicon

Title:	Rel-17 UE features for positioning enhancements

Document for:	Discussion and Decision

Introduction

In RAN1#106bis-e, RAN1 agreed to the UE feature list for positioning enhancement of Rel-17 in R1-2109915 [1].

Agreement The agreements listed in Section 6 of R1-2109915 are endorsed.

In this paper, we discuss the remaining issues related to positioning UE feature of Rel-17.

Discussion

UE Rx/Tx timing error

| 27. NR_pos_enh | 27-1-x1 | Mitigation of UE Rx timing delays Maximum number of UE-RxTEGs [for UE-assisted DL TDOA and/or Multi-RTT positioning] | The maximum number of UE-RxTEG per UE, which is supported and reported by UE [for DL TDOA [and/or Multi-RTT positioning] FFS: the values (>1). FFS: whether to have a value=1 to indicate UE Rx timing errors is well calibrated FFS: whether to have separate values/FGs for DL TDOA and/or Multi-RTT positioning [If UE supports this capability with the values > 1, the UE supports including one UE Rx TEG ID for the RSTD reference time and one UE Rx TEG ID for each DL RSTD measurement (including each additional DL RSTD measurement), in a DL TDOA measurement report] | [13-3] | No |  | Mitigation of UE Rx timing delays is not supported | FFS: Per UE or per band | n/a | n/a | n/a | [The candidate values are {1,2,4,6,8,12,16,24,32}] Need for location server to know if the feature is supported. FFS: Separate row for “Support of UE-RxTEG reporting for DL-TDOA”, and “Support of UE-RxTEG reporting for M-RTT” | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

For FG 27-1-1, we think that

DL-TDOA and Multi-RTT reporting could have separate capabilities, in that for Multi-RTT, it is likely that UE only support RxTx TEG reporting instead of UE Rx TEG reporting, according to the agreement.

The reporting type could be per band. There is no need to have a per-UE capability for this if per band is agreed. The reporting Rx TEG ID in the measurement report could be numbered within the UE, i.e. the same Rx TEG ID means the same Rx TEG for different bands.

The maximum number per band should be reduced to 8.

If the reporting type is per band, we think that even if the number is 1, UE should still support Rx TEG ID reporting for the measurement in that PRS received in different bands may be associated with the same or different Rx TEG ID.

No need to have a separate row indicating support of the TEG reporting. If UE does not report the maximum number, UE does not support the feature (for a band). Perhaps we can put the number as the first component of the overall support of the feature. For example, the FG could be renamed as “Support of UE Rx TEG reporting”, and the first component is the maximum number of Rx TEGs.

| 27. NR_pos_enh | 27-x1-2 | Mitigation of UE Tx timing delays Maximum number of UE-TxTEGs [for UL TDOA and/or Multi-RTT positioning] | The maximum number of UE-TxTEG per UE, which is supported and reported by UE [for UL TDOA and/or Multi-RTT positioning] FFS: the values (>1). FFS: whether to have a value=1 to indicate UE Tx timing errors is well calibrated FFS: whether a UE supports to have different values/FGs for UL TDOA and/or Multi-RTT positioning FFS: Separate row for “Support of UE-TxTEG reporting for MRTT”, and a separate row for the “maximum number of TxTEGs for RTT” [If UE supports this capability with the values > 1, the UE supports to provide the association information of UL SRS resources for positioning with Tx TEGs to the LMF. - FFS: Whether the association information is sent directly from UE to LMF, or is first provided to gNB and then forwarded to LMF] | [13-4, 13-8] | FFS |  | Mitigation of UE Tx timing delays is not supported | FFS: Per UE or per band or per FS | n/a | n/a | n/a | [The candidate values are {1, 2, 4, 8, 16, 32}] Need for location server to know if the feature is supported. FFS: whether gNB needs to know if the feature is supported. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

For FG 27-1-2, we suggest to split the FG into two three FGs.

FG 27-1-2 deals with the UE SRS TEG reported to gNB, which can be applicable to both UL-TDOA and Multi-RTT. The reporting granularity can be per FS.

FG 27-1-2a deals with the UE SRS TEG reported to LMF for UL-TDOA. The reporting granularity can be per band for the configured CA band combination, so that LMF could request gNB with a proper number of SRS resources to cover the Tx TEG number.

FG 27-1-2b deals with the UE SRS TEG reported to LMF for Multi-RTT. The reporting granularity can be per band for the configured CA band combination, so that LMF could request gNB with a proper number of SRS resourcs to cover the Tx TEG number.

The maximum number of per band for the new three FGs should be reduced to 4.

If the reporting type is per band, we think that even if the number is 1, UE should still support Tx TEG ID reporting for the association and measurement in multi-RTT in that SRS transmitted in different bands may be associated with the same or different Tx TEG ID.

No need to have a separate row indicating support of the TEG reporting. If UE does not report the maximum number, UE does not support the feature (for a band). Perhaps we can put the number as the first component of the overall support of the feature. For example, the FG could be renamed as “Support of UE Tx TEG reporting”, and the first component is the maximum number of Tx TEGs.

| 27. NR_pos_enh | 27-x1-3 | Mitigation of UE RxTx timing delays Maximum number of UE-RxTxTEGs for Multi-RTT | The maximum number of UE-RxTxTEG per UE, which is supported and reported by UE for Multi-RTT positioning FFS: the values (>1) FFS: whether to have a value=1 to indicate UE RxTx timing errors is well calibrated [If a UE support this capability with the values > 1, the UE supports reporting of UE RxTx TEG ID with UE Rx-Tx time difference measurements for Multi-RTT positioning] | [13-4, 13-8] | No |  | Mitigation of UE RxTx timing delays is not supported | FFS: Per UE or per band | n/a | n/a | n/a | [The candidate values are {1, 2, 4, 6, 8, 12, 16, 24, 32, 64, [128]}] Need for location server to know if the feature is supported. FFS: Separate row for “Support of UE-RxTxTEG reporting for Multi-RTT” | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- For FG 27-1-3, we think that

The reporting type could be per band. There is no need to have a per-UE capability for this if per band is agreed. The reporting Rx TEG ID in the measurement report could be numbered within the UE, i.e. the same Rx TEG ID means the same Rx TEG for different bands.

The maximum number per band should be reduced to 32.

If the reporting type is per band, we think that even if the number is 1, UE should still support RxTx TEG ID reporting for the measurement in that PRS received in different bands may be associated with the same or different RxTx TEG ID.

No need to have a separate row indicating support of the TEG reporting. If UE does not report the maximum number, UE does not support the feature (for a band). Perhaps we can put the number as the first component of the overall support of the feature. For example, the FG could be renamed as “Support of UE RxTx TEG reporting for multi-RTT”, and the first component is the maximum number of RxTx TEGs.

| 27. NR_pos_enh | 27-x1-4 | The maximum Number of UE Rx TEGs for measuring the same DL PRS resource | The maximum number of different UE-RxTEGs that a UE can support to measure the same DL PRS of a TRP. FFS: The values (>1) | 27-x1 | No |  | Mitigation of UE Rx timing delays by using different Rx TEGs are not supported | FFS: Per UE or per band | n/a | n/a | n/a | [The candidate values are {1, 2, 4, 8}] Need for location server to know if the feature is supported. [Note: UE measures the same instance of the DL PRS with multiple RX TEGs] FFS: Separate row for “Support measuring the same DL PRS of a TRP with different UE-RxTEGs” FFS: Separate row for “The maximum Number of UE Rx TEGs for measuring the same DL PRS resource simultaneously” | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- For FG 27-1-4, we think that

The reporting type could be per band.

No need to have a separate row indicating support of the TEG reporting. If UE does not report the maximum number, UE does not support the feature (for a band). Perhaps we can put the number as the first component of the overall support of the feature. For example, the FG could be renamed as “Support of receiving the same DL PRS resource with multiple Rx TEGs, and the first component is the maximum number of Rx TEGs.

The prerequisite FG should be updated as FG 27-1-1.

- New FG proposals

- In addition, we propose to add the following UE feature subject to progress in 8.5.1.

| 27. NR_pos_enh | 27-xx1 | Support of MTW | Support of PRS measurement time window (PTW) configuration. |  | No |  | PRS measurement time window is not supported by UE | Per UE | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_pos_enh | 27-xx2 | Support of positioning SRS with antenna switching | Support of positioning SRS resource set configured with antenna switching 1. The number of positioning SRS resources in a positioning SRS resource set configured with antenna switching Note: The switching period follows the existing MIMO SRS antenna switching (15us as per R1-1710048). | 13-4 | Yes |  | SRS positioning configured with antenna switching is not supported | Per FS | n/a | n/a | n/a | The candidate values are {1,2,4}. | Optional with capability signaling |
| 27. NR_pos_enh | 27-xx3 | Support of PRS TEG association information for UE-based DL-TDOA | Support of reception of association between PRS and TRP Tx TEG for UE-based positioning |  | No |  | Positioning calculation assistance data containing association between PRS and TRP Tx TEG is not supported by UE | Per UE | n/a | n/a | n/a | Need for location server to know if the feature is supported. Agreement: Support the LMF to provide the association information of DL PRS resources with Tx TEGs to a UE for UE-based positioning if the TRP has multiple TEGs | Optional with capability signaling |

UL AoA

No UE feature is related to UL AoA enhancements.

DL AoD

| 27. NR_pos_enh | 27-z2-1 | Support of [UE-assisted] DL PRS RSRP of the first [or additional] path [for DL-AoD] | UE’s capability to Support providing the of [measuring and reporting the] PRS RSRP of the first [or additional] path [for DL-AoD positioning method] [Note: Applicable for DL-TDOA and Multi-RTT] | [13-2 or 13-3] | No |  |  | FFS: Per UE or per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

For FG 27-2-1, we think that

DL-TDOA, DL-AoD, and Multi-RTT reporting could have separate capabilities.

The reporting type could be per band.

| 27. NR_pos_enh | 27-z2-2 | Support of DL PRS RSRP reporting for more than 8 measurements [for UE-assisted DL-AoD positioning] | UE capability to Support reporting K> 8 DL PRS RSRP measurements per TRP. FFS: the values of K •Note: Multiple RSRPs corresponding to same or different Rx Beam index should be able to be reported for a given PRS resource for different timestamps. Note: Additional capability may be added: •FFS: Additional capability may be added to limit the maximum number of DL PRS RSRP associated with the same Rx beam index | [13-5] | No |  | UE report of more than 8 DL PRS-RSRP is not supported. | FFS: Per UE or per band | n/a | n/a FFS | n/a | [The candidate values are {12, 16, 32, 64}] Need for location server to know if the feature is supported. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

For FG 27-2-2, we think that

The reporting type could be per band.

There is no such need to indicate the K values, but rather UE only reports support of more than 8 DL PRS RSRPs. Network may indicate that more than 8 is desired, and it is up to UE to select which RSRPs are reported, and it can be more than 8 if requested by the LMF. If it is necessary to indicate the K values, we suggest to also add 24, making it  {12, 16, 24, 32, 64}

No need to have additional capability to limit the number of RSRPs per Rx beam in the capability unless we made progress on the measurement reporting enhancement.

Latency

| 27. NR_pos_enh | 27-u3-1 | Support of M-sample measurements [of DL PRS measurement on single DL PRS period/occasion] | The capability to support providing reporting a measurement based on measuring M samples (instances) of a DL PRS resource set M= [1[,2-34]. FFS: other values. If the UE does not provide the capability, the UE [is assumed to] support M=4 only. | [13-1] | No |  |  | FFS: Per UE or per band | n/a | n/a | n/a | [The candidate values are {1, 2, 3, 4}] Need for location server to know if the feature is supported. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

For FG 27-3-1, we think that

The reporting type could be per UE with FR1/FR2 differentiation.

It should be common for all positioning methods (DL-TDOA, DL-AoD, and Multi-RTT) and all positioning measurements (DL RSTD, DL PRS-RSRP, UE Rx – Tx time difference, and DL PRS-RSRPP).

| 27. NR_pos_enh | 27-u53-2 | DL PRS measurement outside MG [and in a PRS processing priority window] - processing types | 1. Supported PRS processing types subject to the UE determining that DL PRS to be higher priority for PRS measurement outside MG [and in a PRS processing priority window]. Candidate values: {Type 1A, Type 1B, Type 2} Note: - Type 1A refers to DL PRS being prioritized over other DL signals/channels in all OFDM symbols within the PRS processing priority window. The DL signals/channels from all DL CCs (per UE) are affected. - Type 21B refers to DL PRS being prioritized over other DL signals/channels in all OFDM symbols within the PRS processing priority window. The DL signals/channels from certain DL CCs are affected. - Type 2C refers to DL PRS being prioritized over other DL signals/channels only in DL PRS symbols within the PRS processing priority window. Note: When the UE determines higher priority for other DL signals/channels over the PRS measurement/processing, the UE is not expected to measure/process DL PRS which is applicable to all of the above capability options. [Note: Within a PRS processing window, UE measurement is inside the active DL BWP with PRS having the same numerology as the active DL BWP] | [13-1] | No FFS |  |  | FFS: Per UE or per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. FFS: Separate feature group for a UE to declare support of each of the Type-1A, Type-1B, Type-2” capabilities | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

For FG 27-3-2, we think that

The reporting type could be per UE with FR1/FR2 differentiation. If indeed there is need for per band reporting, different bands, on which the feature is supported, should be corresponding to a single type. It will complicate the issue if UE reports different PRS processing types on different bands, i.e. the introduction of per band granularity should only be used to identify the bands that UE does not support the feature.

No need for separate FGs for different processing types, as it results in UE supporting different processing types at the same time, which overcomplicates this issue and require network to indicate which type is applied.

| 27. NR_pos_enh | 27-u63-3 | DL PRS Processing Capability outside MG - buffering capability | 1. DL PRS buffering capability: Type 1 or Type 2 a) Type 1 – sub-slot/symbol level buffering T: [{8, 16, 20, 30, 40, 80, 160, 320, 640, 1280}] ms b) Type 2 – slot level buffering 2. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. a) Type 1 – sub-slot/symbol level buffering b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms 3. Max number of DL PRS resources that UE can process in a slot under it a) FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b) FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz | 27-u5 | No |  |  | Per band | n/a | n/a | n/a | FFS: Separate feature group for a UE to declare PRS processing capabilities of each of the Type-1A, Type-1B, Type-2” capabilities | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

For FG 27-3-3, we think that

The FG should be broken into two FGs, one for PRS being higher priority, and one for PRS being lower priority, since the UE processing capability is apparently different for two cases.

The change of the Type 1 should be component 2 instead of component 1, i.e.

1. DL PRS buffering capability: Type 1 or Type 2 a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering 2. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. a) Type 1 – sub-slot/symbol level buffering T: [{8, 16, 20, 30, 40, 80, 160, 320, 640, 1280}] ms b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms 3. Max number of DL PRS resources that UE can process in a slot under it a) FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz - b) FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz

No need for separate PRS processing types, and we think the primary focus is to introduce single type per UE.

The prerequisite FG should be updated as FG 27-3-2.

- New FG proposals

- In addition, we propose to add the following UE feature subject to progress in 8.5.4.

| 27. NR_pos_enh | 27-uu1 | Support of lower Rx beam sweeping factor | Support of the lower Rx beam sweeping factor than 8 for FR2 1. Number of Rx beam sweeping factors: {1,2,3,4,5,6,7} |  | No |  | UE only supports 8 as the Rx beam sweeping factor defined by RAN4. | Per band (FR2 only) | n/a | n/a | n/a | Need for location server to know if the feature is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_pos_enh | 27-uu2 | Support of UL MAC CE based MG activation request | 1. Support of using UL MAC CE to request measurement gap. |  | Yes |  | Using UL MAC CE to indicate PRS measurement to the gNB is not supported. | Per UE | No | No | No |  | Optional with capability signaling |
| 27. NR_pos_enh | 27-uu3 | Support of DL MAC CE based MG activation | 1. Support of preconfiguration of MGs in RRC 2. Support of using DL MAC CE to activate the MG |  | Yes |  | Using DL MAC CE to activate the preconfigured MG is not supported | Per UE | No | No | No |  | Optional with capability signaling. |
| 27. NR_pos_enh | 27-uu4 | Support of DL MAC CE based PRS processing window activation | 1. Support of preconfiguration of MGs in RRC 2. Support of using DL MAC CE to activate the PRS processing window |  | Yes |  | Using DL MAC CE to activate the preconfigured PRS processing window is not supported | Per UE | No | No | No |  | Optional with capability signaling |

MP/NLOS

| 27. NR_pos_enh | 27-v4-1 | LOS/NLOS Indicator [for UE-assisted positioning] | UE’s capability to Support reporting LoS/NLoS indicator to LMF [for RSTD and UE Rx-Tx time difference measurements to LMF for DL and DL+UL positioning] FFS: whether to have separate capability component/FG for RSTD and UE Rx-Tx time difference measurements. FFS: whether to have separate capability component for hard and soft indication |  | No |  |  | FFS: Per UE or per band | n/a | n/a | n/a | [The candidate value are [0:0.1:1]] Need for location server to know if the feature is supported. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

For FG 27-4-1, we think that

There is no strong need to have separate FGs for the soft and hard values of LoS/NLoS indicator, but we can live with being having a component indicate that.

- 1. LOS/NLOS indicator type: {softValue, hardValue, both}

There is no need to have separate FGs for RSTD and UE Rx – Tx time difference measurement, which means that this is common to all positioning methods and positioning measurements.

The reporting type could be per UE, as we do not see difference on the algorithm on the detection.

Adding another component would be helpful to indicate whether UE support resource-specific indicator or TRP specific indicator or both.

- 2. LOS/NLOS indicator granularity: {resourceSpecific, trpSpecific, both}.

- New FG proposals

- In addition, we propose to add the following UE feature subject to progress in 8.5.5.

| 27. NR_pos_enh | 27-vv1 | LOS/NLOS indicator for UE-based positioning assistance data | Support reception of the assistance data containing the LOS/NLOS indicator. 1. LOS/NLOS indicator type: {softValue, hardValue, both} 2. LOS/NLOS indicator granularity {resourceSpecific, trpSpecific, both} |  | No |  |  | Per UE | No | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_pos_enh | 27-vv2 | Additional path reporting for DL-TDOA | 1. Support of TOA reporting for more than 2 additional paths. 2. Support of path RSRP reporting for additional paths if path RSRP reporting is supported. |  | No |  |  | Per UE | No | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| 27. NR_pos_enh | 27-vv3 | Additional path reporting for DL-AoD | 1. Support of TOA reporting for more than 2 additional paths 2. Support of path RSRP reporting for additional paths if path RSRP reporting is supported. |  | No |  |  | Per UE | No | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| 27. NR_pos_enh | 27-vv4 | Additional path reporting for Multi-RTT | 1. Support of TOA reporting for more than 2 additional paths 2. Support of path RSRP reporting for additional paths if path RSRP reporting is supported. |  | No |  |  | Per UE | No | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling. |

RAN2

| 27. NR_pos_enh | 27-w5-1 | Support of [UE-initiated] on-demand PRS | UE’s capability to support UE-initiated on-demand DL PRS [request signalling] | [13-1] | No |  |  | Per UE | n/a | n/a | n/a | FFS: Need for location server to know if the feature is supported. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

For FG 27-5-1, we think that

We think that this feature can work even if the location server does not know if UE supports the feature, however LMF should anyway have control over whether UE should send the UE-initiated on-demand PRS request.

- New FG proposals

- In addition, we propose to add the following UE feature subject to progress in 8.5.6.

| 27. NR_pos_enh | 27-ww1 | Support of PRS measurement in RRC_INACTIVE | Support of PRS measurement in RRC_INACTIVE Note: UE supporting this feature may support at least one from DL RSTD, DL PRS-RSRP, or UE Rx – Tx time difference |  | Yes |  |  | Per UE | No | No | No |  | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_pos_enh | 27-ww1a | Support of PRS measurement in RRC_INACTIVE state for DL-TDOA | Support of PRS measurement in RRC_INACTIVE state for DL-TDOA Note: Other PRS capabilities follows the same as the RRC_CONNECTED state for DL-TDOA. |  | No |  |  | Per UE | No | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| 27. NR_pos_enh | 27-ww1b | Support of PRS measurement in RRC_INACTIVE state for DL-AoD | Support of PRS measurement in RRC_INACTIVE state for DL-AoD Note: Other PRS capabilities follows the same as the RRC_CONNECTED state for DL-AoD. |  | No |  |  | Per UE | No | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| 27. NR_pos_enh | 27-ww1c | Support of PRS measurement in RRC_INACTIVE state for Multi-RTT | 1. Support of PRS measurement in RRC_INACTIVE state for Multi-RTT 2. Support of positioning SRS transmission in RRC_INACTIVE state. Note: Other PRS capabilities follows the same as the RRC_CONNECTED state for Multi-RTT. |  | No |  |  | Per UE | No | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| 27. NR_pos_enh | 27-ww2 | Support of positioning SRS transmission in RRC_INACTIVE state | 1. Max number of SRS Resource Sets for positioning supported by UE per BWP. Values = {1, 2, 4, 8, 12, 16}. 2. Max number of periodic SRS Resources for positioning per BWP. Values = {1,2,4,8,16,32,64} 3. Max number of periodic SRS Resources for positioning per BWP per slot. Values = {1, 2, 3, 4, 5, 6, 8, 10, 12, 14} OLPC for SRS for positioning based on SSB from the last serving cell (the cell that releases UE from connection) is part of this FG. Note: no dedicated capability signaling is intended for this component |  | Yes |  |  | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| 27. NR_pos_enh | 27-ww2a | Support of positioning SRS transmission in RRC_INACTIVE state | 1. Max number of SRS Resource Sets for positioning supported by UE per BWP. Values = {1, 2, 4, 8, 12, 16}. 2. Max number of periodic SRS Resources for positioning per BWP. Values = {1,2,4,8,16,32,64} 3. Max number of periodic SRS Resources for positioning per BWP per slot. Values = {1, 2, 3, 4, 5, 6, 8, 10, 12, 14} | 27-ww2 | No |  |  | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported UE only reports the number on the PCell band. | Optional with capability signaling |
| 27. NR_pos_enh | 27-ww3 | Support of positioning SRS configured outside UL BWP#0 configured by system information in RRC_INACTIVE state | 1. Support positioning SRS configured outside UL BWP#0 configured by system information in RRC_INACTIVE state 2. SRS switching period: following existing SRS-SwitchingTimeNR IE. | 27-ww2 | Yes |  |  | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| 27. NR_pos_enh | 27-ww4 | Supported positioning SRS bandwidth in RRC_INACTIVE | 1. Supported bandwidth for positioning SRS transmission in RRC_INACTIVE | 27-ww2 | Yes |  |  | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| 27. NR_pos_enh | 27-ww4a | Supported positioning SRS bandwidth in RRC_INACTIVE | 1. Supported bandwidth for positioning SRS transmission in RRC_INACTIVE | 27-ww2, 27-ww2a | No |  |  | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 27. NR_pos_enh | 27-ww5 | OLPC for positioning SRS in RRC_INACTIVE state | Same as LPP OLPC-SRS-Pos-r16 RRC OLPC-SRS-Pos-r16 |  | Yes |  |  | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 27. NR_pos_enh | 27-ww6 | Spatial relation for positioning SRS in RRC_INACTIVE state | Same as LPP SpatialRelationsSRS-Pos-r16 RRC SpatialRelationsSRS-Pos-r16 |  | Yes |  |  | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |

Conclusion

In this contribution, we presented our views on Rel-17 UE feature for NR positioning enhancements. The comments and suggested changes are provided in section 2.

- Reference

RAN1, Chair's Notes RAN1#106bis-e v19, RAN1#106b-e, e-Meeting, 11th – 19th October, 2021.

R1-2109915, Summary of UE features for NR positioning enhancements, Moderator (AT&T), RAN1#106b-e, e-Meeting, 11th – 19th October, 2021.