3GPP TSG RAN WG1 #107-e	R1-2110861

e-Meeting, November 11th - 19th, 2021

Source:	Nokia, Nokia Shanghai Bell

Title:	Resource allocation for power saving

Agenda item:		8.11.1.1

Document for:		Discussion and Decision

# Introduction

2. Resource allocation enhancement:

- Specify resource allocation to reduce power consumption of the UEs [RAN1, RAN2]

  - Baseline is to introduce the principle of Rel-14 LTE sidelink random resource selection and partial sensing to Rel-16 NR sidelink resource allocation mode 2.

  - Note: Taking Rel-14 as the baseline does not preclude introducing a new solution to reduce power consumption for the cases where the baseline cannot work properly.

  - This work should consider the impact of sidelink DRX, if any.

In the description of the work item [1] for NR sidelink enhancement, the following objectives were specified for sidelink resource allocation to reduce power consumption:

In last RAN1 meeting (RAN1 #106b-e), decent progress was made on both types of partial sensing and random resource selection. In this contribution, we continue the discussion and provide our views on random resource selection, periodic-based partial sensing, contiguous partial sensing, and other enhancements to sidelink resource allocation for power saving.

In the last RAN1 meeting, RAN1 discussed one RAN2 LS [4] on the DRX impact on resource allocation. Followed the agreed WA, we provide our view on this issue with discussion in Section 3.

# Resource allocation for power saving in mode 2

The standardization of Rel-16 NR sidelink focused on vehicle-to-vehicle (V2V) communication in V2X use cases. The design is based on the assumption of “always-on” when a UE operates sidelink, since vehicular UEs are installed in vehicles with sufficient battery capacity. A UE performs blind decoding of PSCCHs in all slots except the slots in which it needs to transmit. This is for the following two purposes:

- The UE checks whether the associated PSSCH is of interest to it and hence decides whether it needs to go ahead and decode the PSSCH;

- The UE acquires information of resource allocations by other UEs to enable mode 2 (sensing-based) resource allocation.

In Rel-17, NR sidelink will support new use cases including V2P (vulnerable road users (VRUs)) in V2X use cases, public safety and commercial use cases, as identified in the WID [1]. UEs for these use cases, such as SL reception type-A/B/D UEs, are energy sensitive and therefore their power consumption needs to be minimized. Hence solutions for power saving in Rel-17 are required. In LTE V2X sidelink (Rel-14/15), random resource selection and partial sensing based resource selection were supported as power efficient RA schemes especially for the pedestrian UEs. These two RA schemes are assumed as the baseline in Rel-17 for power efficient RA schemes with potential further enhancements, as described in the WID [1]. Both resource allocation schemes, random selection and partial sensing are under discussion for specification as enhancement for SL resource allocation for power saving.

## Random resource selection

In RAN1#106-e [2], the following agreements were reached on random resource selection:

Agreement For random resource selection in a resource pool (pre-)configured with full/partial sensing and random resource selection, down-select to one of the followings in RAN1#106bis-e - Option 1: A priority threshold value or a range of priority levels is (pre-)configured for the resource pool, below or within which random resource selection is allowed - Note, lower value means higher priority - FFS whether resource pool partitioning can be additionally applied - Option 2: Increase the priority for the transmission based on random selection and indicate the new priority value in the priority field in the 1st-stage SCI - FFS: An extra field is added in SCI for indicating the original priority value associated with QoS requirement, - FFS: A 1-bit field in the SCI indicates that the UE is performing random resource selection, or - FFS: An extra field is added in SCI for indicating the mapping to the original priority value associated with QoS requirement. - Option 7: Exclude resources reserved by UE performing random selection without re-evaluation / pre-emption checking, regardless of their priorities. E.g. a 1-bit field in the SCI indicates that the UE is performing random resource selection and not performing re-evaluation and pre-emption checking - Option 12: No special consideration

With random resource selection, the transmitting UE randomly selects a resource in the resource selection window to transmit the data packet. For periodic traffic, the UE may reserve the corresponding resource in the next period for the transmission of the next TB. As there are no channel sensing operations, the random resource selection can significantly reduce the power consumption. On the other hand, the benefit of power saving is achieved at the cost of potential increased collisions with other UEs (including sensing UEs and other random selection UEs).

The random resource selection can be used under the following scenarios:

- C1: Some UEs, e.g. P-UEs, may have no or limited SL receiving capability or have no sufficient battery capacity to support SL receiving. As agreed in last two RAN1 meetings, SL UEs that are not capable of performing reception of any SL signals and channels are called reception type-A UEs, and UEs which can receive only PSFCH and S-SSB are called SL reception type-B UEs. In some scenarios, these UEs need to transmit sidelink packets to nearby UEs (e.g. to vehicle UEs for traffic safety purpose). In this case, they have to use random resource selection.

- C2: For UEs that have sensing capability (such as reception type-D UE) but still intend to keep power consumption as low as possible, they may start the sensing operations only after the arrival of data packets (especially aperiodic packets). The UEs may have to use random resource selection at least for the initial transmission if the latency requirement is stringent. Alternatively, when SL DRX mode is applied, UEs may not perform the sensing during the OFF duration. The UEs may need to use random resource selection for possible SL transmission at the beginning of the ON duration until the sensing results are available.

- C3: The UEs may use random resource selection under the condition that both random selection and (partial) sensing based RA are configured for the resource pool and at the same time, when the measured CBR of the pool is sufficiently low.

As mentioned above, the cost of random resource selection is the potential collisions with other UEs e.g. sensing UEs or other random resource selection UEs. This issue will further deteriorate if persistent collisions occur. So, enhancement to the random resource selection is desired to alleviate the potential collisions.

The resource reservation by a UE with random resource selection can be pre-empted by a UE with sensing. But this UE can’t identify the situation leading to collision. We may consider increasing the priority at PHY for a UE with random selection or adding SCI bits to indicate sensing capability (resource selection scheme).

The above issue also exists in the case that the shared resource pool is also used by Rel-16 sensing UEs. Adding SCI bits to indicate sensing capability doesn’t work since Rel-16 UEs can’t identify newly added SCI bits. On the contrary, increasing PHY priority for a UE with random selection is a feasible choice for backward compatibility with Rel-16 sensing UEs. We may use the increased priority value in the priority field in the 1st-stage SCI. This can increase the capability of a Rel-17 UE with random resource selection to protect itself from being pre-empted by Rel-16 UEs. Accordingly, to retain proper operations between Rel-17 UEs with different resource selection schemes, an extra priority field containing the original priority value associated with QoS requirements can be added to the SCI.

For backward compatibility, the collision issues with Rel-16 UEs may be also addressed by using, for example, conditions to control the UEs using random resource selection. The conditions may consider, for example, whether the UEs may use random resource selection per a TB or more than one consecutive TBs, CBR conditions, priorities of SL transmissions, uses of SL HARQ Option-2 so that the UEs using random resource selection may reselect resources based on HARQ NACKs when persistent collisions happen.

Observation 1: Random resource selection could be used under the conditions e.g. with no sensing capability, no sensing results available for urgent use or when the CBR of the pool is sufficiently low.

Observation 2: In case the shared resource pool is also used by Rel-16 sensing UEs, backward compatibility needs to be considered.

Based on the discussion, we propose to support Option 2.

Proposal 1: Consider Option 2: Increase the priority for the transmission based on random selection and indicate the new priority value in the priority field in the 1st-stage SCI.

Proposal 2: For backward compatibility with Rel-16 UEs, support of applying conditions (such as resource selection per a TB or consecutive TBs, CBR conditions, etc.) to control random resource selection may be considered.

## Partial sensing-based RA

Another RA for power saving is the partial sensing based RA which was defined in Rel-14 and is taken as baseline in Rel-17 SL enhancement [1]. To support the coexistence of periodic and aperiodic packets in the same resource pool, both periodic-based partial sensing and contiguous partial sensing are to be defined which have been intensively discussed in the last meetings. These two types of partial sensing are further discussed in this section based on the progress achieved in last meeting.

In the last meeting [3], the following agreements were reached on the periodic-based partial sensing and contiguous partial sensing:

Agreement In the agreement from RAN1#105-e, the working assumption is confirmed and the FFS bullet (in RED) is closed without any agreement. Agreement from RAN1#105-e: - For the k value in periodic-based partial sensing for resource (re)selection, - By default, the UE monitors the most recent sensing occasion for a given reservation periodicity before the resource (re)selection trigger slot n or the first slot of the set of Y candidate slots subject to processing time restriction. - If (pre-)configured, UE additionally monitors periodic sensing occasions that correspond to a set of values which can be (pre-)configured with at least one value - (Working assumption) Possible values correspond to the most recent sensing occasion for a given reservation periodicity before the resource (re)selection trigger slot n or the first slot of the set of Y candidate slots, and the last periodic sensing occasion prior to the most recent one for the given reservation periodicity are included. - FFS: whether/which other values and details of the (pre-)configuration (e.g. max number of values or sensing occasions) - FFS: whether a value denotes a specific occasion to monitor or the earliest occasion to start the monitoring. - FFS relationship between periodic-based partial sensing occasions and SL-DRX - Note: - This is for the case when the resource (re)selection triggering slot n is expected by UE. Agreement When UE performs periodic-based and contiguous partial sensing schemes in a mode 2 Tx pool with periodic reservation for another TB (sl-MultiReserveResource) enabled, - For a resource (re)selection procedure triggered by periodic transmission (Prsvp_TX≠0) in slot n, TA and TB for the CPS monitoring window is defined according to one of the followings: - n+TA is M logical slots earlier than slot ty0SL, and n+TB is Tproc,0SL+Tproc,1SL slots earlier than ty0SL, where ty0SL is the first slot of the selected Y candidate slots of PBPS, and Tproc,0SL, Tproc,1SL are in units of physical time/slots. - By default, M is 31 unless (pre-)configured with another value. Agreement For the periodic sensing occasion(s) (PSO(s)) that a UE needs to additionally monitor in PBPS, it shall be (pre-)configured jointly for all Preserve values. The UE is not required to monitor PSOs earlier than n–T0 if the UE is triggered to do resource (re)selection in slot n, where T0 is (pre-)configured.

In NR SL, both periodic and aperiodic sidelink packets need to be supported and for resource efficiency, the periodic and aperiodic packets may coexist in the same resource pool. In this case, the baseline partial sensing mechanism shall be enhanced to support coexistence of periodic and aperiodic packets.

As per the agreements in RAN1-106e meeting, the contiguous partial sensing may be used if the following conditions are all satisfied: 1) L1 [is expected to be or] is triggered by higher layer to report resources for resource (re)selection in a mode 2 Tx pool; 2) the resource pool is (pre-)configured to enable partial sensing; 3) partial sensing is configured by higher layer in the UE. Meanwhile, periodic based partial sensing may or may not be used together with the contiguous partial sensing as shown in the following cases:

If the Tx resource pool with periodic reservation for another TB (sl-MultiReserveResource) disabled,

the UE may perform only contiguous partial sensing for resource (re)selection.

If the Tx resource pool with periodic reservation for another TB (sl-MultiReserveResource) enabled, and if the resource (re)selection procedure is triggered by periodic transmission (Prsvp_TX=0), the UE will perform the periodic based partial sensing as discussed and agreed in previous meetings. If the resource (re)selection procedure is triggered by aperiodic transmission (Prsvp_TX≠0), the UE may perform PBPS between the triggering slot n and the first slot of the selected Y candidate slots within the RSW subject to processing time restriction. Alternatively depending on implementation, the UE may perform PBPS irrespective of the triggering slot n based on assumed Y candidate slots. In this case, the UE may only report the candidate resources within part of the Y candidate slots which are within the RSW based on the triggering slot n.

As discussed above, if the resource pool is (pre-)configured to enable partial sensing, and L1 is triggered by higher layer to report resources for resource (re-)selection in the resource pool, the UE may perform contiguous partial sensing with or without periodic based partial sensing. When the resource (re-)selection procedure is triggered by aperiodic transmission (P_rsvp_TX=0) at slot n, for the UE performing contiguous partial sensing, there are three open Approaches on the resource selection and the related CPS sensing window, as discussed in the last meeting (RAN1 #106b-e):

Agreement When UE performs at least contiguous partial sensing in a mode 2 Tx pool for a resource (re)selection procedure triggered by aperiodic transmission (Prsvp_TX=0) in slot n, TA and TB for CPS monitoring window and a candidate resource set (SA) is initialized according to potentially one of the following approaches (final decision in RAN1#107-e). Other approaches are not precluded and the details in each approach can still be updated. - Approach 1: (SA is initialized based on at least slots with PBPS and/or CPS results and guarantee a minimum of M slots for CPS) - The UE selects a set of Y’ candidate slots with corresponding PBPS and/or CPS results (if available) within the RSW. - FFS how to handle the case if the total number of Y’ candidate slots is less than a (pre-)configured threshold Y’min without dropping the aperiodic transmission - FFS whether the Y’ candidate slots for aperiodic transmission is the same as the Y candidate slots in PBPS for periodic transmission of another TB(s) - FFS whether/how to prioritize/select resources based on partial sensing results. - FFS: How to select Y’ in case of CPS only - Candidate resource set (SA) is initialized to the set of all single-slot candidate resources in the selected Y’ candidate slots. - For the CPS monitoring window [n+TA, n+TB]: - TA and TB are both selected such that UE has sensing results for a minimum of M consecutive logical slots before ty0, where ty0 is the first slot of the selected Y’ candidate slots. - FFS: By default, M is 31 unless (pre-)configured with another value, or M is (pre-)configured based on transmission priority - FFS the range of (pre-)configured M from a TBD lowest value up to 30 - FFS: how to handle the case when the minimum M slots for CPS cannot be guaranteed - FFS: RSW in case of CPS only - Approach 2: (SA is initialized based on all candidate single-slot resources and guarantee a minimum of M slots for CPS) - Candidate resource set (SA) is initialized to the set of all candidate single-slot resources in [n+TB+Tproc,0+Tproc,1, n+T2], where TB is selected by the UE such that length of [n+TB+Tproc,0+Tproc,1, n+T2] ≥ T2min. - Tproc,0, Tproc,1 are in units of physical time/slots - FFS whether/how to prioritize/select resources based on partial sensing results (if PBPS is performed). - For the CPS monitoring window [n+TA, n+TB]: - TA = X - FFS value X for TA including X=1 and negative value - TB is selected such that UE has sensing results for a minimum of M consecutive logical slots before the start of (n+TB+Tproc,0+Tproc,1). - FFS: By default, M is 31 unless (pre-)configured with another value, or M is (pre-)configured based on transmission priority - FFS the range of (pre-) configured M from a TBD lowest value up to 30 - FFS: how to handle the case when the minimum M slots for CPS cannot be guaranteed - Approach 3: (independent approach for different case) - When UE additionally performs periodic-based partial sensing in the resource pool, the above Approach 1 applies. - When UE does not perform periodic-based partial sensing in a resource pool that does not allow resource reservation for another TB, the above Approach 2 applies.

Approach 1 is the PBPS-based approach for the selection of the candidate resource set S_A. The S_A is initialized based on slots with sensing results (from PBPS and/or CPS) and RSW. The CPS sensing window is [n+TA, n+TB].

Approach 2 takes some modification of the RSW to [n+TB+Tproc,0+Tproc,1, n+T2], and the sensing window parameter TB is constrained with the condition that the UE has sensing results for a minimum of M consecutive logical slots.

Approach 3 uses different approaches for either periodic-based partial sensing or CPS. When the UE has extra sensing results from PBPS, Approach 1 applies; and when the UE does not have extra sensing results from PBPS and only relies CPS to provide sensing information, Approach 2 shall apply.

Among the three approaches, Approach 2 and Approach 3 are addressing the two issues of Approach 1. 1) there might be no enough CPS sensing results to determine resource slots; and 2) there might be no enough slots for resource selection because of the CPS sensing window after trigger of slot n. Approach 2 limits the choice of T_B with the minimum of M consecutive logical slots in the remaining RSW. However, Approach 1 has the choice to pick T_A and T_B for a minimum of M consecutive logical slots.

When the UE has sensing results from periodic-based partial sensing, Approach 1 won’t have the problem of insufficient sensing results. When there is no extra sensing result, Approach 1 may have the problem on the balance of CPS sensing window and minimum slots in RSW. Here Approach 2 might be helpful to ensure the minimum M slots. However, Approach 1 can pick T_A and T_B to reach the same goal for a minimum of M slots. Considering this aspect, there is no special benefit for Approach 2. Approach 3 is just a combination scheme of Approach 1 and Approach 2. Without a clear benefit of using Approach 2, Approach 1 should be sufficient to support the CPS-only case.

Proposal 3: Support Approach 1, when UE performs at least contiguous partial sensing in a mode 2 Tx pool for a resource (re)selection procedure triggered by aperiodic transmission (Prsvp_TX=0) in slot n.

# DRX impact on the restriction of resource selection

In the RAN1 #106b-e meeting, a RAN2 LS on the DRX impact on Rx UE is discussed. The output of the discussion is the working assumption, shown below:

Working Assumption When PHY layer is indicated with an active time of RX UE from MAC layer for candidate resource selection, a restriction is applied in PHY layer so that at least a subset of candidate resources reported to MAC layer is located within the indicated active time of the RX UE. The following options will be further discussed in RAN1 to restrict resources for candidate resource selection taking into account the indicated active time from MAC layer: - Option 1: PHY layer selects and reports candidate resources only within the indicated active time of the RX UE - Option 2: PHY layer selects and reports candidate resources in which at least a subset of the candidate resources is within the indicated active time of the RX UE - Option 3: PHY layer selects and reports an additional candidate resource set of candidate resources within the indicated active time of the RX UE

In the RAN2 LS (R2-2108997), RAN2 agreements were listed, indicating the L2 Tx UE can maintain a set of timers, each of which can be served as the criterion to determine the allowable transmission time for each RX UE. RAN2 asked whether such information could be applicable in RAN1 to consider certain restrictions for Rx DTX UE.

Through RAN1 discussion, it indicates that such indication of Rx UE active time can be applied in PHY layer for resource allocation, as shown in the WA. RAN1 provided three options with indication that these options “will be further discussed in RAN1”. Following this WA, we provide our further discussion on the three options.

Following the resource selection procedure (TS 38.214), either full sensing or partial sensing shall report a resource set SA to higher layers, subject to sensing results. With consideration of DRX, if the information about a Rx UE active time is available (from MAC CE, for example), the resource selection procedure may be modified to exclude the relative resources, which are not fit into the Rx UE active time, from the resource set SA. The Tx UE will report the SA resource set to higher layers so that the proper resource selection will fit the Rx UE reception. This could be a possible approach to include the Rx UE active time information in the RAN1 resource selection procedure. This approach directly relates to Option 1, where PHY layer selects and reports candidate resource only within the indicated active time of the Rx UE.

Option 2 will allow reporting resource outside of active time of the Rx UE, as long as “at least a subset of the candidate resources in within the indicated active time of the Rx UE”. Then it is up to L2 to determine the actual resource allocation based on the reporting.

Option 3 calls to report “additional” candidate resources. It implies that UE may have its “regular” report on resource based on sensing, regardless of Rx UE DRX timing. On top of this, the UE shall report “additional” candidate resource that would fit the Rx UE DRX timing.

During the RAN1 discussion, the argument to support Option 2 is based on the interpretation of “active time” of the Rx UE. If the Tx UE reports the candidate resource based on the current DRX time, provided by the higher layer, the reported resource might be too restrictive if the Rx UE extends its active time, which would provide more possible candidate resources. Reporting candidate resource outside of active time would be beneficial in this scenario.

Based on this interpretation of Rx UE “current active time” and “future active time”, RAN1 was struggling to decide whether it is beneficial to provide candidate resource outside of at least “current” active time of the Rx UE. Determination of Rx UE “current active time” and “future active time” is beyond RAN1 scope.

From RAN2 LS, the DRX timing information is provided with a set of timers:

- For unicast, the TX UE maintains a set of timers per pair of source Layer-2 ID and destination Layer-2 ID corresponding to the SL DRX timers in the RX UE. For groupcast/broadcast, the TX UE maintains a set of timers per destination Layer-2 ID corresponding to the SL DRX timers in the RX UE. The TX UE uses the timers as part of the criterion for determining the allowable transmission time for each RX UE. - For transmissions to RX UE(s) using SL DRX operation, LCP restrictions ensure that a TX UE transmits data in the active time of the RX UE(s).

Since the usage of a set of timers is the basis to determine the Rx UE DRX timing, it would imply that the DRX timing is maintained periodically. Rx UE can extend its active time; but the question is how often such active-time extension happens. Given that DRX is a power saving feature, UE will likely operate at power saving mode to maintain a regular DRX schedule. If this is the case, the benefit of Option 2 would be quite limited.

Unless we have a clear understanding on the difference of “current active time” and “future active time”, and demonstrate the benefit of Option2, Option 1 should be the simple choice in RAN1.

Proposal 4: Support Option 1, PHY layer selects and reports candidate resources only within the indicated active time of the RX UE.

# Conclusion

In this contribution, we provide our views on enhancement to sidelink resource allocation for power saving, summarized in the following observations and proposals:

On random resource selection:

Observation 1: Random resource selection could be used under the conditions e.g. with no sensing capability, no sensing results available for urgent use or when the CBR of the pool is sufficiently low.

Observation 2: In case the shared resource pool is also used by Rel-16 sensing UEs, backward compatibility needs to be considered.

Proposal 1: Consider Option 2: Increase the priority for the transmission based on random selection and indicate the new priority value in the priority field in the 1st-stage SCI.

Proposal 2: For backward compatibility with Rel-16 UEs, support of applying conditions (such as resource selection per a TB or consecutive TBs, CBR conditions, etc.) to control random resource selection may be considered.

On partial sensing:

Proposal 3: Support Approach 1, when UE performs at least contiguous partial sensing in a mode 2 Tx pool for a resource (re)selection procedure triggered by aperiodic transmission (Prsvp_TX=0) in slot n.

On DRX impact on resource selection:

Proposal 4: Support Option 1, PHY layer selects and reports candidate resources only within the indicated active time of the RX UE.

# References

- RP-202846 - WID revision: NR sidelink enhancement

- RAN1 Chair report, Meeting #106-e, Aug.16th – 27th, 2021

- RAN1 Chair report, Meeting #106bis-e, Oct.11th – 19th, 2021

- R2-2108997, LS to RAN1 on RAN2 Agreements Related to Resource Selection, RAN2 #115-e, Aug 16-27, 2021

# Appendix: Previous agreements on Rel-17 resource allocation for power saving

## RAN1 #103e

Conclusion

- SL reception Type A and Type D should be used as the reference for evaluation and designing of SL power saving features in R17.

  - Type A: UE is not capable of performing reception of any SL signals and channels, FFS with exception of performing PSFCH and S-SSB reception (aim to conclude in RAN1#104-e)

  - Type D: UE is capable of performing reception of all SL signals and channels defined in R16. It does not preclude UE to perform reception of a subset of SL signals/channels

  - If there are evaluations with assumptions other than the above reference, the detailed assumptions need to be reported

  - Note: the types and the associated capability defined here are not intended to be defined as Rel-17 UE features as is.

Agreements:

- Partial sensing based RA is supported as a power saving RA scheme

  - FFS details

- Random resource selection is supported as a power saving RA scheme

  - FFS any changes or enhancement

  - FFS on conditions to apply random resource selection

Agreements:

- In R17, a SL Mode 2 Tx resource pool can be (pre-)configured to enable full sensing only, partial sensing only, random resource selection only, or any combination(s) thereof

  - FFS details, including usage, potential restrictions, whether/how any enhancement or condition is needed for the coexistence of full sensing and power saving RA scheme(s) in a same resource pool, etc.

Agreements:

- Re-evaluation and pre-emption checking are not supported by UEs that do not perform any sensing (i.e. PSCCH reception)

- Re-evaluation and pre-emption checking are supported by UEs that perform sensing

  - FFS details and any conditions(s) in which re-evaluation and pre-emption can be performed

- FFS whether/how re-evaluation and pre-emption can be supported by UEs performing random resource selection that do perform sensing

- Note: details about sensing in this context, including when it is performed, are not decided yet.

Agreements:

- Further study congestion control based on CBR and CR for power saving RA schemes

  - Identify necessary changes from R16 CBR/CR (if any), including transmission resource selection and transmission parameters that can be adjusted and applicable to power savings RA schemes

  - Note: this is not intended to require all UEs to perform sensing for the purpose of CBR measurement

## RAN1 #104e

Agreements:

- Random resource selection is applicable to both periodic and aperiodic transmissions

  - FFS conditions for random resource selection

Conclusion:

- PSFCH reception is not included for Type A UE

- S-SSB reception is not included for Type A UE

- SL reception Type B is additionally added

  - Type B: Same as Type A with an exception of performing PSFCH and S-SSB reception

- Note: the same conditions as in RAN1#103-e regarding the context of the discussion of Type A and Type D still apply (also applicable to type B)

Agreements: In a resource pool (pre-)configured with at least partial sensing, if UE performs periodic-based partial sensing, at least when the reservation for another TB (when carried in SCI) is enabled for the resource pool and resource selection/reselection is triggered at slot n, it is up to UE implementation to determine a set of Y candidate slots within a resource selection window, where

- FFS condition(s) and timing(s) for which periodic-based partial sensing is performed by UE

- The resource selection window is [n+T1, n+T2]

  - As a baseline, T1 and T2 are defined in the same way as in R16 NR-V2X according to step 1 [TS 38.214 Sec. 8.1.4]

  - Further discuss whether or not to introduce a threshold to re-define T1 and T2 such that

    - T1 ≥ 0 (subject to processing time constraint Tproc, 1), and T2 ≤ remaining PDB

    - T2-T1 ≤ (pre-)configured threshold

- A minimum value for Y is (pre-)configured from a range of values, FFS details

- FFS any restriction to determine Y candidate slots (including its relationship with SL-DRX)

- FFS whether the resource selection window [n+T1, n+T2] should be confined within a set of periodic set of resources and its relationship with SL-DRX

- Note: The terminology “periodic-based partial sensing” is based on the “partial sensing” used in LTE-V and it is intended to be used for the design and discussion of partial sensing in Rel-17.

Agreements: In a resource pool (pre-)configured with at least partial sensing, if UE performs periodic-based partial sensing, at least when the reservation for another TB (when carried in SCI) is enabled for the resource pool and resource selection/reselection is triggered at slot n, the UE monitors slots of at least one a set of periodic sensing occasions, where a periodic sensing occasion is a set of slots according to

if tvSL is included in the set of Y candidate slots.

- Preserve is a periodicity value from the configured set of possible resource reservation periods allowed in the resource pool (sl-ResourceReservePeriodList). Down select to one:

  - Option 1:  Preserve corresponds to all values from the configured set sl-ResourceReservePeriodList

  - Option 2:  PreservePreserve Preserve corresponds to a subset of values from the configured set sl-ResourceReservePeriodList

    - FFS how to determine the subset (e.g., by (pre-)configuration, UE determination)

  - Option 3: PreservePreserve Preserve is a common divisor among values in the configured set sl-ResourceReservePeriodList

  - Option 4: FFS others

- k equals tois selected according to (down select to one)

  - Option 1: Only the most recent sensing occasion within sensing window for a given reservation periodicity before the resource (re)selection trigger or the set of Y candidate slots subject to processing time restriction

  - Option 2: The two most recent sensing occasions within sensing window for a given reservation periodicity before the resource (re)selection trigger or the set of Y candidate slots subject to processing time restriction

  - Option 3: All possible sensing occasions after n –T0

  - Option 4: Only one periodic sensing occasion for one reservation period. The k value is up to UE implementation. Max value for k is (pre-)configured.

  - Option 5: k is (pre-)configured, including multiple values

  - Option 6: (pre-)configuration of a bitmap, same as in LTE-V

  - Option 7: FFS others

- FFS relationship between periodic sensing occasions and SL-DRX

- FFS condition(s) and timing(s) for which periodic-based partial sensing is performed by UE

- Note: companies are encouraged to show performance data for the down selections

Agreements:

- In a resource pool (pre-)configured with at least partial sensing, if UE performs contiguous partial sensing and resource (re-)selection is triggered in slot n, support the following option:

  - Option 1: For the purpose of resource (re-)selection, the UE monitors slots between [n+TA, n+TB] and performs identification of candidate resources, in or after slot n+TB, based on all available sensing results, including periodic-based partial sensing results (if applicable).

    - FFS TA, TB (including the possibility of equal to zero, positive or negative) and remaining details (in particular, whether there should be exclusion of slots, changes in TA/TB values for different purposes, etc.)

    - FFS whether n can be replaced by e.g., index of some of Y candidate slots

  - FFS condition(s) in which contiguous partial sensing is performed by UE

  - FFS interaction with SL-DRX, if any

  - FFS interaction with periodic-based partial sensing, if any

  - Other options are not precluded

  - Note: This option is not to replace random resource selection only without sensing or re-evaluation and pre-emption checking

## RAN1 #104b-e

Conclusion:

- In periodic-based partial sensing,

  - It is not necessary to further discuss whether or not to introduce a threshold to re-define T1 and T2.

Agreements:

- In periodic-based partial sensing,

- For the set of Preserve values, down-select to one of the following in RAN1#105-e

    - Alt.1: Preserve corresponds to all values from the configured set sl-ResourceReservePeriodList

    - Alt.2: A set of Preserve values is (pre-)configured and includes up to the full set of values from the configured set sl-ResourceReservePeriodList

      - FFS if support multiple sets of Preserve values based on one or more metrics

      - FFS whether/how to restrict the set of values

- For the k value, down-selection to one of the following in RAN1#105-e (further refinement of each of the alternatives is possible)

        - Alt 1: Option 1 as in RAN1#104-e

        - Alt 2: A modified Option 5 as in RAN1#104-e, where the modification is such that it also includes option 1

          - FFS how to (pre-)configure (e.g. including bitmap), whether a maximum number of k values is needed, and whether it can be up to UE implementation to select a k value based on the (pre-)configuration

        - FFS details, e.g., sensing before the resource (re)selection trigger or the first slot of the set of Y candidate slots subject to processing time restriction, etc.

      - Note: companies are encouraged to provide more evaluations

Agreement:

- When periodic-based partial sensing is potentially performed by UE in a mode 2 Tx resource pool provided by higher layer, at least all of the followings are met:

  - Periodic reservation for another TB (sl-MultiReserveResource) is enabled for the resource pool

  - The resource pool is (pre-)configured to enable partial sensing

  - Partial sensing configured by higher layer in the UE

## RAN1 #105-e

Agreement:

- For the set of Preserve values in periodic-based partial sensing,

  - If no (pre-)configuration (i.e., by default), Preserve corresponds to all values from the (pre-)configured set sl-ResourceReservePeriodList.

  - Otherwise, a single set of Preserve values can be (pre-)configured, where the set of Preserve values are restricted to a subset of the (pre-)configured set sl-ResourceReservePeriodList

    - This is per mode 2 Tx resource pool (pre-)configuration

    - A UE by implementation may also monitor other sl-ResourceReservePeriodList values not part of the restricted subset

      - In particular, the UE may additionally monitor occasions corresponding to P_RSVP_Tx

        - FFS whether the monitoring can be mandatory

Agreement:

·  In periodic-based partial sensing for resource (re)selection, the UE at least monitors in periodic sensing occasion(s) for a given reservation periodicity before the first slot of the selected Y candidate slots subject to processing time restriction for the identification of candidate resources.

o   The processing time restriction includes Tproc,0SL  and Tproc,1SL.

o   Aspects relating to sensing during SL DRX are to be discussed separately

- Relationship to re-evaluation and pre-emption operation for periodic-based partial sensing to be discussed separately

  - FFS details including whether monitoring of periodic sensing occasions between triggering slot n and the first slot of the selected Y candidate slots subject to processing time restriction is performed as part of resource (re)selection or re-evaluation and pre-emption checking

Agreement:

- For the k value in periodic-based partial sensing for resource (re)selection,

  - By default, the UE monitors the most recent sensing occasion for a given reservation periodicity before the resource (re)selection trigger slot n or the first slot of the set of Y candidate slots subject to processing time restriction.

  - If (pre-)configured, UE additionally monitors periodic sensing occasions that correspond to a set of values which can be (pre-)configured with at least one value

    - (Working assumption) Possible values correspond to the most recent sensing occasion for a given reservation periodicity before the resource (re)selection trigger slot n or the first slot of the set of Y candidate slots, and the last periodic sensing occasion prior to the most recent one for the given reservation periodicity are included.

    - FFS: whether/which other values and details of the (pre-)configuration (e.g. max number of values or sensing occasions)

    - FFS: whether a value denotes a specific occasion to monitor or the earliest occasion to start the monitoring.

  - FFS relationship between periodic-based partial sensing occasions and SL-DRX

  - Note:

    - This is for the case when the resource (re)selection triggering slot n is expected by UE

Agreement:

- For random resource selection,

  - Reuse the maximum distance separation of 32 logical slots for a HARQ retransmission resource reserved by a prior SCI for the same TB, which was defined in R16 for full sensing operation.

  - SL HARQ feedback enabled transmission is supported (FFS applicable conditions if any)

    - The minimum HARQ feedback time gap (Z) shall be respected between any two selected resources of a TB where a HARQ feedback for the first of these resources is expected.

- FFS the impact of resource collision when random resource selection is performed by a UE which does not perform sensing / re-evaluation and pre-emption checking in a resource pool with mixed RA schemes (e.g. for low priority or any priority transmissions).

  - Including study potential solution(s) if the impact is not negligible (e.g. threshold based, raising priority, minimum time gap, pattern based, a priori SCI reserving initial transmissions, resource pool partitioning, and etc.).

Agreement: In contiguous partial sensing for resource (re)selection, TA and TB values can be zero, positive or negative

- TA and TB values or range depend on different operating scenarios or conditions (e.g., periodic/aperiodic traffic, predictability of triggering slot n, remaining PDB, re-evaluation/pre-emption checking, HARQ feedback, CBR/CR parameter, power saving, etc)

  - FFS details

- FFS: details of how periodic-based partial sensing and contiguous partial sensing are used for re-evaluation and pre-emption checking. Including how to reduce UE’s power consumption (caused by additional sensing operation of re-evaluation/pre-emption) after its resource selection, with the considerations of different operating scenarios or conditions (e.g., pre-emption enabled/disabled, HARQ-ACK enabled/disabled, etc).

## RAN1 #106-e

Agreement

In periodic-based partial sensing, UE monitoring of periodic sensing occasions between triggering slot n and the first slot of the selected Y candidate slots subject to processing time restriction is performed as part of resource (re)selection.

Agreement

Conditions in which contiguous partial sensing is performed by UE, when at least all of the followings are met:

- L1 [is expected to be or] is triggered by higher layer to report resources for resource (re-)selection in a mode 2 Tx pool

  - FFS: When the trigger will be received by L1

- The resource pool is (pre-)configured to enable partial sensing

- Partial sensing is configured by higher layer in the UE

Agreement

For a resource pool (pre-)configured with at least partial sensing and UE is configured by its higher layer for partial sensing,

- Periodic-based partial sensing and contiguous partial sensing schemes are supported for resource re-evaluation and pre-emption checking

  - FFS details of partial sensing for re-evaluation and pre-emption checking, including any restrictions / conditions on performing PBPS and CPS, subset of resources, timing, candidate resource set (SA) and etc

- Same as in Rel-16, the higher layer indicates a set of resources (r0,r1,r2,…) and/or a set of resources (r0',r1',r2',…) for re-evaluation and/or pre-emption checking, respectively

  - Pre-emption checking is enabled according to the Release-16 interpretation of sl-PreemptionEnable.

    - FFS: If additional enhancements are needed for enabling/disabling

- The triggering of re-evaluation and pre-emption checking is as in R16.

Agreement

When UE performs only contiguous partial sensing (CPS) in a mode 2 Tx pool with periodic reservation for another TB (sl-MultiReserveResource) disabled, and a resource (re)selection is triggered in slot n,

- The resource selection window (RSW) is [n+T1, n+T2] where T2 is defined based on step 1) of Rel-16 TS 38.214 Sec. 8.1.4

  - FFS whether the resource selection window [n+T1, n+T2] should be confined within a set of periodic set of resources and its relationship with SL-DRX

- On the sensing window [n+TA, n+TB] for CPS,

  - Details of TA and TB values based on the agreements from previous RAN1 meetings

  - FFS whether and how to define a minimum CPS window size, including (pre-)configurability and the case when TB - TA is smaller than the minimum CPS window size

  - FFS whether and how to define a maximum value / upper bound for TB with respect at least to the minimum RSW size and the remaining PDB, including (pre-)configurability

- FFS how a set of candidate resource (SA) is initialized considering candidate single-slot resources, including

  - Whether and how to define a minimum size for the RSW (e.g., Rel-16 T2min), including (pre-)configurability

  - Whether the set SA is confined within a set of Y candidate slots within the RSW

- UE performs resource exclusion from the set SA based on at least all available sensing results and based on step 6) and 7) of Rel-16 TS 38.214 Sec. 8.1.4

- Note, re-evaluation and pre-emption checking in a resource pool with periodic reservation for another TB (sl-MultiReserveResource) disabled is considered separately.

- FFS: Details on T1

Agreement

For random resource selection in a resource pool (pre-)configured with full/partial sensing and random resource selection, down-select to one of the followings in RAN1#106bis-e

- Option 1: A priority threshold value or a range of priority levels is (pre-)configured for the resource pool, below or within which random resource selection is allowed

  - Note, lower value means higher priority

  - FFS whether resource pool partitioning can be additionally applied

- Option 2: Increase the priority for the transmission based on random selection and indicate the new priority value in the priority field in the 1st-stage SCI

  - FFS: An extra field is added in SCI for indicating the original priority value associated with QoS requirement,

  - FFS: A 1-bit field in the SCI indicates that the UE is performing random resource selection, or

  - FFS: An extra field is added in SCI for indicating the mapping to the original priority value associated with QoS requirement.

- Option 7: Exclude resources reserved by UE performing random selection without re-evaluation / pre-emption checking, regardless of their priorities. E.g. a 1-bit field in the SCI indicates that the UE is performing random resource selection and not performing re-evaluation and pre-emption checking

- Option 12: No special consideration

Agreement

When UE performs periodic-based and contiguous partial sensing schemes in a mode 2 Tx pool with periodic reservation for another TB (sl-MultiReserveResource) enabled,

- For a resource (re)selection procedure triggered by aperiodic transmission (Prsvp_TX=0) in slot n,

  - The resource selection window (RSW) is [n+T1, n+T2], and T1 and T2 are defined in the same way according to step 1) of Rel-16 TS 38.214 Sec. 8.1.4

    - FFS whether UE determines a new set of Y candidate slots within the RSW and monitors corresponding periodic sensing occasions between slot n and the first slot of the new Y candidate slots subject to processing constraints

    - FFS how to initialize a set of candidate resource (SA) for the triggered resource (re)selection procedure and which partial sensing scheme(s) and results can be used for resource exclusion in the resource (re)selection procedure

    - FFS whether the resource selection window [n+T1, n+T2] should be confined within a set of periodic set of resources and its relationship with SL-DRX

- Note, re-evaluation and pre-emption checking based on periodic-based and contiguous partial sensing schemes is considered separately

Agreement

When UE performs periodic-based and contiguous partial sensing schemes in a mode 2 Tx pool with periodic reservation for another TB (sl-MultiReserveResource) enabled,

- For a resource (re)selection procedure triggered by periodic transmission (Prsvp_TX≠0) in slot n

  - A set of candidate resource (SA) is initialized to the set of selected Y candidate slots of PBPS

    - UE performs contiguous partial sensing in [n+TA, n+TB] for resource exclusion from the initialized candidate resource set (SA)

      - FFS details of TA and TB based on the agreement(s) from previous RAN1 meetings

- Note, re-evaluation and pre-emption checking based on periodic-based and contiguous partial sensing schemes is considered separately

FFS: The condition under which UE performs periodic-based and contiguous partial sensing schemes in a mode 2 Tx pool with periodic reservation for another TB (sl-MultiReserveResource) enabled

R1-2108265	FL summary for AI 8.11.1.1 – resource allocation for power saving (EOM)	Moderator (OPPO)

[106-e-NR-R17-Sidelink-02] Reply LS to R1-2106413 (LS on time gap information in SCI, RAN2) by August 20 – Kevin (OPPO)

Agreement

Regarding RAN2’s question, in RAN1’s opinion it is feasible, other than in the following exceptional cases:

- SL transmission dropping due to prioritization or congestion control

- Due to re-evaluation, a re-selected resource is earlier than a reserved resource by UE implementation in Mode 2

- If (pre-)configured with many-to-one mapping between Tx and Rx resource pools in some cases (e.g., when PSFCH is not configured)

The final LS is in R1-2108622.

R1-2108226	Moderator summary for [106-e-NR-R17-Sidelink-02] Reply LS to R1-2106413	Moderator (OPPO)

[106-e-NR-R17-Sidelink-03] Reply LS to R1-2100021 (LS to RAN1 on SL DRX design, RAN2, from RAN1#104-e) by August 25 - Yuzhou (ZTE)

Agreement

A UE can perform SL reception of PSCCH and RSRP measurement for sensing during its SL DRX inactive time.

- FFS: When such reception and measurement is performed, whether it is subject to specification, or is up to UE implementation

- FFS: Other details

Agreement

LS to RAN2 on SL DRX design is endorsed in R1-2108580.

## RAN1 #106b-e

Agreement

In the agreement from RAN1#105-e, the working assumption is confirmed and the FFS bullet (in RED) is closed without any agreement.

Agreement from RAN1#105-e: - For the k value in periodic-based partial sensing for resource (re)selection, - By default, the UE monitors the most recent sensing occasion for a given reservation periodicity before the resource (re)selection trigger slot n or the first slot of the set of Y candidate slots subject to processing time restriction. - If (pre-)configured, UE additionally monitors periodic sensing occasions that correspond to a set of values which can be (pre-)configured with at least one value - (Working assumption) Possible values correspond to the most recent sensing occasion for a given reservation periodicity before the resource (re)selection trigger slot n or the first slot of the set of Y candidate slots, and the last periodic sensing occasion prior to the most recent one for the given reservation periodicity are included. - FFS: whether/which other values and details of the (pre-)configuration (e.g. max number of values or sensing occasions) - FFS: whether a value denotes a specific occasion to monitor or the earliest occasion to start the monitoring. - FFS relationship between periodic-based partial sensing occasions and SL-DRX - Note: - This is for the case when the resource (re)selection triggering slot n is expected by UE

Agreement

When UE performs periodic-based and contiguous partial sensing schemes in a mode 2 Tx pool with periodic reservation for another TB (sl-MultiReserveResource) enabled,

- For a resource (re)selection procedure triggered by periodic transmission (Prsvp_TX≠0) in slot n, TA and TB for the CPS monitoring window is defined according to one of the followings:

  - n+TA is M logical slots earlier than slot ty0SL, and n+TB is Tproc,0SL+Tproc,1SL slots earlier than ty0SL, where ty0SL is the first slot of the selected Y candidate slots of PBPS, and Tproc,0SL, Tproc,1SL are in units of physical time/slots.

    - By default, M is 31 unless (pre-)configured with another value.

Agreement

For the periodic sensing occasion(s) (PSO(s)) that a UE needs to additionally monitored in PBPS, it shall be (pre-)configured jointly for all Preserve values.

- The UE is not required to monitor PSOs earlier than n–T0 if the UE is triggered to do resource (re)selection in slot n, where T0 is (pre-)configured

Agreement

When UE performs at least contiguous partial sensing in a mode 2 Tx pool for a resource (re)selection procedure triggered by aperiodic transmission (Prsvp_TX=0) in slot n, TA and TB for CPS monitoring window and a candidate resource set (SA) is initialized according to potentially one of the following approaches (final decision in RAN1#107-e). Other approaches are not precluded and the details in each approach can still be updated.

- Approach 1: (SA is initialized based on at least slots with PBPS and/or CPS results and guarantee a minimum of M slots for CPS)

  - The UE selects a set of Y’ candidate slots with corresponding PBPS and/or CPS results (if available) within the RSW.

    - FFS how to handle the case if the total number of Y’ candidate slots is less than a (pre-)configured threshold Y’min without dropping the aperiodic transmission

    - FFS whether the Y’ candidate slots for aperiodic transmission is the same as the Y candidate slots in PBPS for periodic transmission of another TB(s)

    - FFS whether/how to prioritize/select resources based on partial sensing results.

    - FFS: How to select Y’ in case of CPS only

  - Candidate resource set (SA) is initialized to the set of all single-slot candidate resources in the selected Y’ candidate slots.

  - For the CPS monitoring window [n+TA, n+TB]:

    - TA and TB are both selected such that UE has sensing results for a minimum of M consecutive logical slots before ty0, where ty0 is the first slot of the selected Y’ candidate slots.

      - FFS: By default, M is 31 unless (pre-)configured with another value, or M is (pre-)configured based on transmission priority

      - FFS the range of (pre-)configured M from a TBD lowest value up to 30

      - FFS: how to handle the case when the minimum M slots for CPS cannot be guaranteed

  - FFS: RSW in case of CPS only

- Approach 2: (SA is initialized based on all candidate single-slot resources and guarantee a minimum of M slots for CPS)

  - Candidate resource set (SA) is initialized to the set of all candidate single-slot resources in [n+TB+Tproc,0+Tproc,1, n+T2], where TB is selected by the UE such that length of [n+TB+Tproc,0+Tproc,1, n+T2] ≥ T2min.

    - Tproc,0, Tproc,1 are in units of physical time/slots

    - FFS whether/how to prioritize/select resources based on partial sensing results (if PBPS is performed).

  - For the CPS monitoring window [n+TA, n+TB]:

    - TA = X

      - FFS value X for TA including X=1 and negative value

    - TB is selected such that UE has sensing results for a minimum of M consecutive logical slots before the start of (n+TB+Tproc,0+Tproc,1).

      - FFS: By default, M is 31 unless (pre-)configured with another value, or M is (pre-)configured based on transmission priority

      - FFS the range of (pre-) configured M from a TBD lowest value up to 30

      - FFS: how to handle the case when the minimum M slots for CPS cannot be guaranteed

- Approach 3: (independent approach for different case)

  - When UE additionally performs periodic-based partial sensing in the resource pool, the above Approach 1 applies.

  - When UE does not perform periodic-based partial sensing in a resource pool that does not allow resource reservation for another TB, the above Approach 2 applies.

Working Assumption

In a resource pool (pre-)configured to enable partial sensing, when UE is configured with partial sensing by its higher layer, the resources for which the UE performs re-evaluation and/or pre-emption checking are for the initial transmission and retransmissions of every TB according to Rel-16 specification based on partial sensing results.

- Same as in Rel-16, for periodic transmission, re-evaluation check is not applied to the resources that have been signalled in current period or previous periods, except that it is up to UE implementation whether to apply re-evaluation check to the resources in non-initial reservation period that have been signalled neither in the immediate last nor in the current period.

- The resource in the main bullet is the set of resources (r0,r1,r2,…) and/or the set of resources (r0',r1',r2',…)  for re-evaluation and/or pre-emption checking, respectively, which has been agreed in RAN1 #106-e.