3GPP TSG RAN WG1 Meeting #107-e	R1-2110871

e-Meeting, November 11th – 19th, 2021

Agenda Item:	8.16.15

Source:	Huawei, HiSilicon

Title:	Rel-17 UE features for LTE based 5G terrestrial broadcast

Document for:	Discussion and Decision

# Introduction

UE features for LTE based 5G terrestrial broadcast were discussed in the last meeting with the summary referring to [1]. This contribution provides our views regarding the issues with TBD.

# UE feature related discussion

The UE feature is updated as follows after the last meeting discussion:

| 3. LTE_terr_bcast_bands_part1 | 3-1 | Support of new channel bandwidth for PMCH | [TBD: whether separate components are neded for different bandwidths] | Support of dedicated MBMS cells | N |  | UE cannot receive MBMS in the corresponding cell | [Per band] | N/A | N/A |  | [Optional with capability signaling] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Regarding whether separate components are needed for different bandwidths, as agreed the supported system bandwidth indicated in MIB is set to 5 MHz or 3 MHz, for which PMCH can be configured with 6/7/8 MHz.

From the perspective of UE implementation capability, a single UE capability supporting flexible bandwidth of 6/7/8 MHz is sufficient. The component of the feature could be written to support the feature of flexible bandwidth for PMCH with system bandwidth limited to 5 or 3 MHz. However, there is worry from other company that UE that is targeting a single region shall implement the bandwidths for the other regions with the corresponding potential different RF requirements if a common capability is defined. With this regard taken into account, separate UE capabilities corresponding to different bandwidth is also acceptable.

Per-band reporting and optional with capability signaling is acceptable.

Proposal 1:

- The component of the feature can be written as:

- Support flexible PMCH bandwidth (6/7/8MHz) allocation for the system bandwidth indicated in MIB set to 5 MHz or 3 MHz.

- Alternatively, define separate UE capabilities corresponding to different bandwidth (6/7/8 MHz).

Proposal 2: The capability of the feature is per band.

# Conclusions

This contribution presents our views on UE feature regarding supporting flexible PMCH bandwidth based on the preliminary input and summarizes the proposals as follows:

Proposal 1:

- The component of the feature can be written as:

- Support flexible PMCH bandwidth (6/7/8MHz) allocation for the system bandwidth indicated in MIB set to 5 MHz or 3 MHz.

- Alternatively, define separate UE capabilities corresponding to different bandwidth (6/7/8 MHz).

Proposal 2: The capability of the feature is per band.

# References

R1-2109718, Summary on UE features for LTE based 5G terrestrial broadcast, Moderator (NTT DOCOMO, INC.), October 11-19, 2021.