3GPP TSG RAN WG1 Meeting #107-e	R1-2110909

e-Meeting, November 11th – 19th, 2021

Source: 	ZTE

Title: 	Discussion on intra-UE multiplexing in Rel-16 URLLC

Agenda item:	7.2.5

Document for:	Discussion and Decision

Introduction

In RAN1#103-e, there was an LS from RAN2 to confirm if RAN1 and RAN2 have the same understanding on the UE behavior in intra-UE multiplexing. After discussion, the understanding of UE behavior is aligned between RAN1 and RAN2 in case the collision only involves LP PUSCH and HP PUSCH. However, if an additional PUCCH was involved, no consensus was reached. The agreement in RAN1#103-e is shown below[1]. In the previous meetings, this issue was discussed but no progress was achieved. In this contribution, the remaining issues are discussed, including the overlapping between SR and PUSCH with the same L1 priority, intra-UE multiplexing and PUSCH skipping in case L1 priority is configured.

Agreement Send an LS to RAN2 to convey the following: - For the collision scenario between CG and DG with same/different PHY-priority index, if there is no collision between PUCCH and the CG and there is no collision between PUCCH and the DG , the behavior mentioned in the LS is consistent with RAN1’s understanding if taking into account the TP to Rel-16 TS 38.214, i.e., revision CR in R1-2008655. - When the MAC entity is configured with lch-basedPrioritization, for the collision scenario between CG and DG with same/different PHY-priority index, and when there is collision between PUCCH and the CG with the same priority and/or there is collision between PUCCH and the DG with the same priority, RAN1 is still discussing the related PHY layer behavior. LS is endorsed in R1-2009680.

Discussion

Overlapping between SR and PUSCH with the same L1 priority

For the overlapping between SR and PUSCH with the same L1 priority, RAN2 intention is that the MAC may instruct the PHY to transmit the one with higher L2 priority. RAN1 confirm the intention only in case the overlapping involves the SR and PUSCH. For the other cases involving UCI multiplexing, RAN1 cannot make any conclusion since the UCI multiplexing may lead to that the final PUCCH resource for SR does not overlap with PUSCH any more or the final PUCCH resource for SR overlaps with the PUSCH but the original PUCCH resource for SR does not overlap with the PUSCH. There are mainly three cases for such overlapping as shown below (Case 2, 3, 4).

- Case 1: only SR overlaps with PUSCH of equal L1 priority - Case 2: other UCI(s) i.e., HARQ-ACK/CSI overlaps with SR of an equal L1 priority and the SR overlaps with the PUSCH of an equal L1 priority - Case 2-1: the final PUCCH resource after UCI multiplexing among different PUCCHs carrying HARQ-ACK/CSI and SR does not overlap with the PUSCH - Case 2-2: the final PUCCH resource after UCI multiplexing among different PUCCHs carrying HARQ-ACK/CSI and SR overlaps with the PUSCH - Case 3: other UCI(s) i.e., HARQ-ACK/CSI overlaps with a PUSCH of an equal L1 priority, SR overlaps with the PUSCH of equal L1 priority, but other UCI(s) do not overlap with the SR - Case 4: other UCI(s), i.e., HARQ-ACK/CSI overlaps with SR of an equal L1 priority, but SR does not overlap with the PUSCH of an equal L1 priority Case 1: only SR overlaps with PUSCH of equal L1 priority Case 2-1: the final PUCCH resource after UCI multiplexing does not overlap with PUSCH Case 2-2: the final PUCCH resource after UCI multiplexing overlaps with PUSCH Case 3: other UCI(s) overlaps with a PUSCH, SR overlaps with the PUSCH, SR does not overlap with other UCI(s) Case 4: other UCI(s) overlaps with SR of an equal L1 priority, but SR does not overlap with the PUSCH of an equal L1 priority

In the reply LS from RAN2 [2], it is RAN2 understanding that MAC does not use knowledge of UCI multiplexing when MAC executes LCH based prioritization and deciding when to transmit SR. It means the the MAC layer determines to generate the SR transmission or PUSCH transmission only based on the original PUCCH resource for SR. RAN2 further indicates that LCH based prioritization has higher priority than UL skipping. With this understanding, the corresponding UE behaviors under these cases are shown in Table 1.

Table 1 UE behavior

|  | L2 priority | MAC layer operation | UE behavior |
| --- | --- | --- | --- |
| Case 2-1(a) | SR>PUSCH | Generate SR transmission | SR and AN/CSI are multiplexed in the PUCCH PUSCH is not transmitted |
|  | SR<PUSCH | Generate MAC PDU | AN/CSI is multiplexed in the PUSCH |
| Case 2-1(b) | SR>PUSCH | Generate SR transmission | SR and AN/CSI are multiplexed in the PUCCH PUSCH is not transmitted |
|  | SR<PUSCH | Generate MAC PDU | AN/CSI is transmitted on the PUCCH PUSCH is transmitted |
| Case 2-2(a) | SR>PUSCH | Generate SR transmission | SR and AN/CSI are multiplexed in the PUCCH PUSCH is not transmitted |
|  | SR<PUSCH | Generate MAC PDU | AN/CSI is multiplexed in the PUSCH |
| Case 2-2(b) | SR>PUSCH | Generate SR transmission | SR and AN/CSI are multiplexed in the PUCCH PUSCH is not transmitted |
|  | SR<PUSCH | Generate MAC PDU | AN/CSI is transmitted on the PUCCH PUSCH is transmitted |
| Case 3(a) | SR>PUSCH | Generate SR transmission | AN/CSI is transmitted on the PUCCH SR is transmitted on the PUCCH |
|  | SR<PUSCH | Generate MAC PDU | AN/CSI is multiplexed in the PUSCH |
| Case 3(b) | SR>PUSCH | Generate SR transmission | AN/CSI is transmitted on the PUCCH SR is transmitted on the PUCCH |
|  | SR<PUSCH | Generate MAC PDU | AN/CSI is multiplexed in the PUSCH |
| Case 4 |  | Generate SR and MAC PDU | The SR and AN/CSI are multiplexed in the PUSCH |

It can be seen that this leads to that PUSCH cannot be transmitted in some cases even though there are UCI multiplexed in it. It violates the UL skipping agreement reached in RAN1. So the network should perform blind detection since the network can not be aware of which one is generated. Considering that all the involved channels have the same physical priority, it is better to generate the MAC PDU for the PUSCH so that UCI can be multiplexed in the PUSCH in physical layer. However, if RAN1 does not confirm the working assumption, e.g., the UL skipping has a higher priority than LCH based prioritization, it may lead to more serious problems especially when two physical priorities are configured as discussed in RAN1#105-e. Therefore, the working assumption should be changed a bit. For example, if the PUSCH is de-prioritized due to a prioritized SR and there are UCI multiplexed in the de-prioritized PUSCH, the MAC layer shall generate MAC PDU for the de-prioritized PUSCH. In other words, when the SR and PUSCH with the same physical priority overlaps, both the LCH-based prioritization and UL skipping works.

The other way is the Rel-15 rule is used when the SR and PUSCH with the same physical priority overlaps in the time domain, i.e., the SR is de-prioritized all the time.

Proposal 1: One of the following options is taken for the reply LS to RAN2.

- Option 1: RAN1 confirms the RAN2 working assumption in the LS.

- Option 2: RAN1 confirms the RAN2 working assumption in the LS with the updates that the MAC layer shall generate MAC PDU for the de-prioritized PUSCH if the PUSCH is de-prioritized due to a prioritized SR with same physical layer priority and there are UCI multiplexed in the de-prioritized PUSCH.

- Option 3: Rel-15 rule is used when the SR and PUSCH with the same physical priority overlaps in the time domain

Intra-UE multiplexing

The UE behavior is still unclear when LCH-based prioritization and uplink skipping are enabled together. The scenario is CG PUSCH and DG PUSCH overlap in the time domain with same/different priorities and one of the PUSCH further overlaps with a PUCCH with the same priority. An example is shown in Figure 1, where DG PUSCH with low priority overlaps with both of CG PUSCH with high priority and LP PUCCH in the time domain while CG PUSCH does not overlap with PUCCH.

Figure 1 An example of collision among CG PUSCH, DG PUSCH and PUCCH.

In the reply LS from RAN2, RAN2 agreed that the LCH based prioritization has higher priority than UL skipping and corresponding CR to TS38.321 has been agreed[2][3][4]. In this case, even though the LP PUSCH overlaps with the LP PUCCH in the time domain, the MAC layer still generates the MAC PDU for the HP PUSCH. It makes sense since LP PUSCH always has lower priority than HP PUSCH if the HP PUSCH has available data.

Proposal 2: RAN1 should follow the RAN2 agreement when two physical priorities are configured such that the MAC PDU is generated for the HP PUSCH as long as there is available data for the HP PUSCH even if there is a LP PUSCH with multiplexed UCI overlapping with the HP PUSCH.

If the MAC entity does not generate MAC PDU for a PUSCH, it means the PUSCH does not exist from the perspective of physical layer since there is no MAC PDU delivered to physical layer. Then, one question is whether the non-existed PUSCH participates in the subsequent physical layer procedure, e.g. UCI multiplexing/intra-UE multiplexing. In our understanding, if the physical layer has already determined there is no MAC PDU for a PUSCH, the non-existed PUSCH should not participate in the subsequent physical layer procedure since such operation is meaningless. This is also in line with the agreements for case 1-6 in RAN1#104-e, where the UCI is not multiplexed in the PUSCH without MAC PDU.

Proposal 3: If the MAC entity does not generate MAC PDU for a PUSCH, the PUSCH should not participate in the subsequent UCI multiplexing.

Regarding the transmission of the LP PUCCH overlapping with the LP PUSCH, it was discussed in RAN1#106-e and the following options were proposed [5].

In case LCH based prioritization is configured, for the case that a PUSCH i.e., PUSCH#0 overlaps with a PUCCH#0 with the same L1 priority on a same or different serving cell, a PUSCH i.e., PUSCH#1 overlaps with the PUSCH#0 on one serving cell with the same or different priorities and the PUSCH#1 does not overlap with the PUCCH#0, and if the PUSCH#0 is NOT delivered by MAC, - Option 1: Drop the PUCCH#0. - Option 2: When timeline condition is met, - If there is no other remaining PUSCH(s) on any serving cell(s) overlapping with the PUCCH#0 of the same L1 priority, the UCI is transmitted on the PUCCH. - Otherwise, the PUCCH#0 should be dropped. Note: above timeline condition is ensured by gNB, i.e. the ending symbol of UL grant for the PUSCH#1 should be at least Tproc,2mux symbols before the first symbol of the earliest PUCCH#0 or PUSCH#0. - Option 3: Transmit PUCCH#0 and the UE does not expect that there is other remaining PUSCH(s) on any serving cell(s) overlapping with the PUCCH#0 of the same L1 priority. - Option 4: - If there is no remaining PUSCH(s) on any serving cell overlapping with PUCCH#0 of the same priority, the UCI is transmitted on the PUCCH - Otherwise, the UCI is multiplexed in the PUSCH according to existing rules - Option 5: PUSCH#0 cannot be skipped by MAC since it is the PUSCH chosen by PHY to carry UCI.

First, the option 1 is not preferred since it may lead to the PUCCH resource waste. And the network has to retransmission the corresponding PDSCH, which may further reduce the resource efficiency. Therefore, the PUCCH should be transmitted as possible. However, in some cases, the PUCCH cannot be transmitted if the LP PUSCH is canceled. There are are two cases illustrated in Figure 2.

Figure 2 The collisions among CG PUSCH, DG PUSCH and PUCCH

- In case 1, the transmission of LP PUCCH is after the HP PUSCH. The physical layer can determine the LP PUSCH will be dropped and there is no MAC PDU for the LP PUSCH when processing HP PUSCH. Therefore, the UCI should not be multiplexed in LP PUSCH and will be transmitted on the PUCCH.

- In case 2, the transmission of LP PUCCH is before than the HP PUSCH. A UE may not realize the collision of HP PUSCH and LP PUSCH when the UE performs the UCI multiplexing in LP PUSCH. After that, the UE receives the PDCCH scheduling HP PUSCH, and determines that LP PUSCH should be dropped. In this case, the UE may not always have enough time to process PUCCH, especially when the first symbol of PUCCH resource is before the first symbol of LP PUSCH. If the time is not enough for the PUCCH processing, the UCI has to be dropped.

It can be seen that, the UCI cannot always be transmitted on PUCCH after determining the PUSCH to be multiplexed with UCI is canceled. It depends on whether the UE has enough time to process PUCCH. In our understanding, the timeline defined for the case 1-6 in RAN1#104-e can be reused. Therefore, if the PUSCH overlapping with a LP PUCCH is canceled due to the HP PUSCH and the time interval between LP PUCCH and the PDCCH scheduling HP PUSCH is not less than Tmuxproc,2, the PUCCH should be transmitted. Otherwise, the PUCCH should be dropped together with LP PUSCH. Therefore, we prefer option 2.

Proposal 4: For the LP PUCCH overlapping with a LP PUSCH which is canceled by a HP PUSCH,

- If the time interval between LP PUCCH and the PDCCH scheduling HP PUSCH is not less than Tmuxproc,2, the LP PUCCH should be transmitted.

- Otherwise, the LP PUCCH should be dropped.

However, it may lead to the HP PUCCH cannot be transmitted no matter which option above is adopted in another case. For example, the HP PUSCH overlaps with both HP PUCCH and LP PUSCH in the time domain as shown in Figure 3. The HP PUCCH does not overlap with LP PUSCH in the time domain.

Figure 3 The overlapping between two PUSCH and one PUCCH

If there is no available data to be mapped to the HP PUSCH, the padding PDU may be generated for the HP PUSCH according to the RAN1 UL skipping agreement since there is UCI multiplexed in the HP PUSCH. However, the L2 priority of the HP PUSCH is lower than that of the LP PUSCH according to TS38.321 as shown below [6]. Then based on the LCH based prioritization, the MAC layer generates the MAC PDU for the LP PUSCH. In this case, the HP PUCCH cannot be transmitted if the timeline is not satisfied or other options are adopted. This is not acceptable for the URLLC service. Therefore, the MAC PDU should be generated for the HP PUSCH as long as there is UCI to be multiplexed in the HP PUSCH.

TS38.321 For the MAC entity configured with lch-basedPrioritization, priority of an uplink grant is determined by the highest priority among priorities of the logical channels that are multiplexed (i.e. the MAC PDU to transmit is already stored in the HARQ buffer) or have data available that can be multiplexed (i.e. the MAC PDU to transmit is not stored in the HARQ buffer) in the MAC PDU, according to the mapping restrictions as described in clause 5.4.3.1.2. The priority of an uplink grant for which no data for logical channels is multiplexed or can be multiplexed in the MAC PDU is lower than either the priority of an uplink grant for which data for any logical channels is multiplexed or can be multiplexed in the MAC PDU or the priority of the logical channel triggering an SR.

To handle this issue, one of the potential solutions is to specify the UL skipping has higher priority than the LCH based prioritization when there is overlapping between HP PUCCH and HP PUSCH. The other one is to specify the L2 priority of HP PUSCH with multiplexed UCI is higher than the L2 priority of the LP PUSCH no matter whether there is available data to be multiplexed in the HP PUSCH. The former one will override the RAN2 agreement and make the MAC layer procedure more complicated. The latter one is better since it still complies the RAN2 agreement and the procedure.

Proposal 5: MAC PDU should be generated for the HP PUSCH as long as there is UCI to be multiplexed in the HP PUSCH.

Proposal 6: RAN1 to send an LS to RAN2 indicating that RAN1 expects the L2 priority of HP PUSCH with multiplexed UCI is higher than the L2 priority of the LP PUSCH no matter whether there is available data to be multiplexed in the HP PUSCH.

In URLLC, HP CG PUSCH with a small periodicity can be configured for the UE in order to reduce the latency of uplink transmission. When the CG HP PUSCH overlaps with DG LP PUSCH in the time domain as shown in Figure 1 and there is only available data to be mapped to the DG PUSCH, the MAC layer only generates the MAC PDU for the LP PUSCH since the priority of the PUSCH without any available data is lower than the PUSCH with available data from the perspective of MAC layer according to TS38.321 as shown above[6]. In this case, the HP PUSCH does not exist in fact. Similarly, the non-existed PUSCH should not participate in the subsequent intra-UE multiplexing, i.e. canceling the LP PUSCH or LP PUCCH.

Proposal 7: If the MAC entity does not generate MAC PDU for a HP PUSCH, the HP PUSCH should not cancel the overlapped LP PUSCH or LP PUCCH.

Based on the discussion above, we have the following text proposal for TS38.213

Proposal 8: Adopt the following text proposal for TS38.213.

---------------------------------------------------Text proposal 1 for TS38.213---------------------------------------------------

<---------------------------Other parts are omitted -------------------------------> When a UE determines overlapping for PUCCH and/or PUSCH transmissions of different priority indexes other than PUCCH transmissions with SL HARQ-ACK reports before considering limitations for UE transmission as described in clause 11.1, including repetitions if any, the UE first resolves the overlapping for PUCCH and/or PUSCH transmissions of smaller priority index as described in clauses 9.2.5 and 9.2.6. Then, - if a transmission of a first PUCCH of larger priority index scheduled by a DCI format in a PDCCH reception would overlap in time with a repetition of a transmission of a second PUSCH or a second PUCCH of smaller priority index, the UE cancels the repetition of a transmission of the second PUSCH or the second PUCCH before the first symbol that would overlap with the first PUCCH transmission - if a transmission of a first PUSCH of larger priority index with a generated transport block scheduled by a DCI format in a PDCCH reception would overlap in time with a repetition of the transmission of a second PUCCH of smaller priority index, the UE cancels the repetition of the transmission of the second PUCCH before the first symbol that would overlap with the first PUSCH transmission where - the overlapping is applicable before or after resolving overlapping among channels of larger priority index, if any, as described in clauses 9.2.5 and 9.2.6 - any remaining PUCCH and/or PUSCH transmission after overlapping resolution is subjected to the limitations for UE transmission as described in clause 11.1 - the UE expects that the transmission of the first PUCCH or the first PUSCH, respectively, would not start before Tproc,2 after a last symbol of the corresponding PDCCH reception - Tproc,2 is the PUSCH preparation time for a corresponding UE processing capability assuming d2,1= d1 [6, TS 38.214], based on μ and N2 as subsequently defined in this clause, and d1 is determined by a reported UE capability When a PUCCH of smaller priority index overlaps with a PUSCH of smaller priority index in the time domain, the PUSCH of smaller priority index overlaps with a PUSCH of larger priority index scheduled by a DCI format in a PDCCH reception in the time domain, and a UE cancels the transmission of PUSCH of the smaller priority, - the UE transmits the PUCCH if a first symbol of the PUCCH is after Tproc,2mux after a last symbol of the PDCCH reception, and - the UE does not transmit the PUCCH, otherwise. <---------------------------Other parts are omitted ------------------------------->

PUSCH skipping in case physical priority is configured

In Rel-16, a PUCCH or a PUSCH can be configured with physical priority. When the overlapped PUCCH and PUSCH are configured with the same priority, the UE behavior is the same as the reached agreement. If there are configured with different priorities, there are two cases as follows.

- HP PUSCH vs LP PUCCH

When a PUSCH with high priority overlaps with a PUCCH with low priority, the PUCCH will be canceled and only PUSCH will be transmitted. If the HP PUSCH can be skipped, then the LP PUCCH can be transmitted when there is no available data for PUSCH transmission. On the contrary, if the HP PUSCH cannot be skipped as agreed in Rel-15, the PUCCH has to be canceled. However, the transmission is useless since neither valid data nor UCI is carried in this case. Therefore, HP PUSCH can be skipped when there is no available data for PUSCH transmission in the case of collision between HP PUSCH and LP PUCCH.

- LP PUSCH vs HP PUCCH

When a PUSCH with low priority overlaps with a PUCCH with high priority, the PUSCH will be canceled and only PUCCH will be transmitted. Therefore, it does not matter whether PUSCH is skipped or not. To align with the UE behavior in the case of collision between HP PUSCH and LP PUCCH, the LP PUSCH can be skipped in this case.

Since the proposal that PUSCH can be skipped does not change the current UE behavior, it can be used for all the discussed collision cases, including when the PUSCH repetition is configured.

Proposal 9: When there is no available data for PUSCH transmission, the PUSCH can be skipped if the PUSCH overlaps with PUCCH and they are configured with different L1 priorities.

Conclusion

According to the discussion above, we have the following observations and proposals.

Proposal 1: One of the following options is taken for the reply LS to RAN2.

- Option 1: RAN1 confirms the RAN2 working assumption in the LS.

- Option 2: RAN1 confirms the RAN2 working assumption in the LS with the updates that the MAC layer shall generate MAC PDU for the de-prioritized PUSCH if the PUSCH is de-prioritized due to a prioritized SR with same physical layer priority and there are UCI multiplexed in the de-prioritized PUSCH.

- Option 3: Rel-15 rule is used when the SR and PUSCH with the same physical priority overlaps in the time domain

Proposal 2: RAN1 should follow the RAN2 agreement when two physical priorities are configured such that the MAC PDU is generated for the HP PUSCH as long as there is available data for the HP PUSCH even if there is a LP PUSCH with multiplexed UCI overlapping with the HP PUSCH.

Proposal 3: If the MAC entity does not generate MAC PDU for a PUSCH, the PUSCH should not participate in the subsequent UCI multiplexing.

Proposal 4: For the LP PUCCH overlapping with a LP PUSCH which is canceled by a HP PUSCH,

- If the time interval between LP PUCCH and the PDCCH scheduling HP PUSCH is not less than Tmuxproc,2, the LP PUCCH should be transmitted.

- Otherwise, the LP PUCCH should be dropped.

Proposal 5: MAC PDU should be generated for the HP PUSCH as long as there is UCI to be multiplexed in the HP PUSCH.

Proposal 6: RAN1 to send an LS to RAN2 indicating that RAN1 expects the L2 priority of HP PUSCH with multiplexed UCI is higher than the L2 priority of the LP PUSCH no matter whether there is available data to be multiplexed in the HP PUSCH.

Proposal 7: If the MAC entity does not generate MAC PDU for a HP PUSCH, the HP PUSCH should not cancel the overlapped LP PUSCH or LP PUCCH.

Proposal 8: Adopt the following text proposal for TS38.213.

---------------------------------------------------Text proposal 1 for TS38.213---------------------------------------------------

<---------------------------Other parts are omitted -------------------------------> When a UE determines overlapping for PUCCH and/or PUSCH transmissions of different priority indexes other than PUCCH transmissions with SL HARQ-ACK reports before considering limitations for UE transmission as described in clause 11.1, including repetitions if any, the UE first resolves the overlapping for PUCCH and/or PUSCH transmissions of smaller priority index as described in clauses 9.2.5 and 9.2.6. Then, - if a transmission of a first PUCCH of larger priority index scheduled by a DCI format in a PDCCH reception would overlap in time with a repetition of a transmission of a second PUSCH or a second PUCCH of smaller priority index, the UE cancels the repetition of a transmission of the second PUSCH or the second PUCCH before the first symbol that would overlap with the first PUCCH transmission - if a transmission of a first PUSCH of larger priority index with a generated transport block scheduled by a DCI format in a PDCCH reception would overlap in time with a repetition of the transmission of a second PUCCH of smaller priority index, the UE cancels the repetition of the transmission of the second PUCCH before the first symbol that would overlap with the first PUSCH transmission where - the overlapping is applicable before or after resolving overlapping among channels of larger priority index, if any, as described in clauses 9.2.5 and 9.2.6 - any remaining PUCCH and/or PUSCH transmission after overlapping resolution is subjected to the limitations for UE transmission as described in clause 11.1 - the UE expects that the transmission of the first PUCCH or the first PUSCH, respectively, would not start before Tproc,2 after a last symbol of the corresponding PDCCH reception - Tproc,2 is the PUSCH preparation time for a corresponding UE processing capability assuming d2,1= d1 [6, TS 38.214], based on μ and N2 as subsequently defined in this clause, and d1 is determined by a reported UE capability When a PUCCH of smaller priority index overlaps with a PUSCH of smaller priority index in the time domain, the PUSCH of smaller priority index overlaps with a PUSCH of larger priority index scheduled by a DCI format in a PDCCH reception in the time domain, and a UE cancels the transmission of PUSCH of the smaller priority, - the UE transmits the PUCCH if a first symbol of the PUCCH is after Tproc,2mux after a last symbol of the PDCCH reception, and - the UE does not transmit the PUCCH, otherwise. <---------------------------Other parts are omitted ------------------------------->

Proposal 9: When there is no available data for PUSCH transmission, the PUSCH can be skipped if the PUSCH overlaps with PUCCH and they are configured with different L1 priorities.

Reference

3GPP RAN1#103-e chairman notes

3GPP RAN1#106-e, R1-2106409, Reply LS on overlapped data and SR with equal L1 priority

3GPP RAN1#106-e, R1-2108484, LS to RAN1 on UL skipping with LCH-based prioritization

3GPP RAN1#107-e, R1-2110755, LS to RAN1 on UL skipping with LCH-based prioritization

3GPP RAN1#106-e, R1-2108461, Summary of [106-e-NR-L1enh-URLLC-05] Issue#9: Remaining issues on UL prioritization and UL skipping

3GPP TS38.321-g30, Medium Access Control (MAC) protocol specification