3GPP TSG RAN WG1 #107-e	R1-2110928

e-Meeting, November 11th – 19th, 2021

Title: 	Discussion on Rel-17 UE features for NR MBS

Source: 	ZTE

Agenda item:	8.16.12

Document for:	Discussion/Decision

Introduction

After RAN1#106b-e meeting, moderators summarized the preliminary RAN1 UE features list for Rel-17 NR [1]. In this contribution, we present our analysis and views on Rel-17 UE features for MBS.

Discussion

Rel-17 is the first release that supports broadcast and multicast for NR. To facilitate the MBS commercial process, it is beneficial to define some basic feature groups for MBS. For now, FG33-1 and FG33-2 can be set as the basic UE feature group for MBS. We can further merge or add more basic functionalities into FG33-1 and FG33-2 if needed.

Proposal 1: Define FG33-1 and FG33-2 as basic feature group for MBS.

Another issue is how to capture agreements for FG33-3-3. From our perspective, UE can share the same maximum PDSCH receptions in a slot per CC between unicast and multicast. Thus, the FG33-3-3 can be updated as following.

Proposal 2: Update the components description as following.

| 33-3-3 | Intra-slot TDM-ed unicast PDSCH and group-common PDSCH | - Support TDM between X unicast PDSCH(s) and Y group-common PDSCH(s) in a slot, where X+Y <= Z. Z is the maximum number of TDMed PDSCH receptions capability in a slot per CC according to Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b. -  Note: Group-common PDSCH(s) are counted as unicast PDSCH(s). - Support TDM-ed Type-1 HARQ-ACK codebook for multicast. |
| --- | --- | --- |

Conclusion

In this contribution, we discuss and analyse Rel-17 UE features for MBS with the following proposal.

Proposal 1: Define FG33-1 and FG33-2 as basic feature group for MBS.

Proposal 2: Update the components description as following.

| 33-3-3 | Intra-slot TDM-ed unicast PDSCH and group-common PDSCH | - Support TDM between X unicast PDSCH(s) and Y group-common PDSCH(s) in a slot, where X+Y <= Z. Z is the maximum number of TDMed PDSCH receptions capability in a slot per CC according to Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b. - Note: Group-common PDSCH(s) are counted as unicast PDSCH(s). - Support TDM-ed Type-1 HARQ-ACK codebook for multicast. |
| --- | --- | --- |

Reference

- R1-2110587, Updated RAN1 UE features list for Rel-17 NR after RAN1 #106bis-e, Moderators (AT&T, NTT DOCOMO, INC.).