3GPP TSG-RAN WG1 Meeting #107bis-e	R1-2110946

eMeeting, November 11th – 19th, 2021

Agenda Item:	8.1.2.2

Source:	Ericsson

Title:	Finalizing Multi-TRP inter-cell operation

Document for:	Discussion, Decision

# 1	Remaining issue for inter-cell mTRP

## 1.1 Discussion about Point A for nserv-cell

One open issue is when receiving a CSI-RS configured for nserv-cell, e.g. TRS, or CSI-RS for CSI reporting, the mapping of the CSI-RS is using the subcarrier 0 in common resource block 0 as reference, which is dependent on the point A configuration of the nserv-cell. Also, the DMRS sequence depends on Point A configuration. It may be so that the nserv-cell have a different point A than the serv-cell. However, we believe this is a rare case and if it happens, it can be handled by network implementation.

The UE can assume that non-serving-cell use the same Point A as the serving-cell when receiving from the non-serving-cell. Hence, no specification impact is foreseen.

# 2  Conclusion

Based on the discussion in the previous section we propose the following:

Proposal 1	The UE can assume that non-serving-cell use the same Point A as the serving-cell when receiving from the non-serving-cell. Hence, no specification impact is foreseen.