3GPP TSG RAN WG1 #107-e	R1-2110969

e-Meeting, November 11th – 19th, 2021

Title:	[Draft] Reply LS on initial state of elements controlled by MAC CEs

Response to:       R2-2109098

Release:	Release 15, Release 16

Work Item:	NR_newRAT-Core

Source:	vivo [RAN1]

To:	RAN2

CC: 	RAN4

# Contact Person:

# Name:

E-mail Address:

# 1. Overall Description:

RAN1 would like to thank RAN2 for the questions in LS in R2-2109098 on initial state of elements controlled by MAC CEs.

Please find RAN1 answers on the questions below.

Firstly, since a handover only refers to PCell change, RAN2 respectfully asks RAN1 to clarify whether the initial deactivation when using handover should be applied for both PCell change and PSCell change/addition in case of DC. Answer1: Yes, the initial deactivation is applicable for both PCell change and PSCell change/addition in case of DC.

Secondly, RAN2 respectfully asks RAN1 to clarify whether the initial deactivation when using configuration should be applied for both “initial configuration by RRC” and “reconfiguration by RRC”. Answer2: Yes, the initial deactivation when using configuration should be interpreted that it is applied for both “initial configuration by RRC” and “reconfiguration by RRC”.

Thirdly, RAN2 respectfully asks RAN1 to clarify whether the UE behavior relevant to (Enhanced) PUCCH spatial relation Activation/Deactivation MAC CE should be aligned with the other MAC CEs. Answer3: Yes, the UE behavior relevant to (Enhanced) PUCCH spatial relation initial deactivation MAC CE should also be aligned with other MAC CEs. For other activation behavior, RAN2 should further clarify the intention.

# 2. Actions:

To RAN2

ACTION:

RAN1 would like to respectfully request RAN2 to take the above answers into account.

# 3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #107b-e     17th – 25th Jan. 2022                                emeeting