3GPP TSG RAN WG1 #107-e     	                                       R1-2110982

2021 e-Meeting, November 11th - 19th, 2021

Source:	vivo

Title:	Clarification on UCI and SL HARQ-ACK

Agenda Item:	7.2.4

Document for:	Draft CR

- Introduction

In this contribution, we discuss the definition of UCI and SL HARQ-ACK.

- Discussion

SL HARQ-ACK should be one type of UCI, and all procedures defined for DL HARQ-ACK can be applied to SL HARQ-ACK unless there are stated special handlings (i.e, no multiplexing between SL HAQ-ACK and DL HARQ-ACK/SR/CSI, no priority index for SL HARQ-ACK). According to the cited text[1][2][3][4] in the appendix, SL HARQ-ACK should be treated as UCI and should reuse the procedure defined for DL HARQ-ACK, specifically:

- In 38.300[1], there is a general definition of UCI that UCI includes any control information sent in the uplink, i.e., HARQ-ACK, SR and CSI, which means that SL HARQ-ACK is also one type of UCI.

- In 38.211[2] and 38.212[3], general procedures for UCI handling, such as scrambling of PUSCH with UCI and data/UCI multiplexing on PUSCH, can be found. There is no description specifically for SL HARQ-ACK, thus it is natural to interpret it as that SL HARQ-ACK should be treated as one type of UCI.

- In subclause 7.2.1 for power control of PUCCH in 38.213[4], apparently, SL HARQ-ACK is considered as one type of UCI.

However, there are several places where SL HARQ-ACK and UCI are treated separately in clause 9 and clause 9.2.5.0 of 38.213. For example, the highlighted text below appears to indicate that UCI and SL HARQ-ACK are different concepts, which leads to ambiguity in the relationship between SL HARQ-ACK and UCI, although 'UCI' below intentionally referred only to Uu UCI (i.e., DL HARQ-ACK/SR/CSI).

Further, it may also lead to the misunderstanding that the second PUCCH is also a SL HARQ-ACK PUCCH and the 'else' statement should be executed when the 'UCI' includes SL HARQ-ACK. However, UE should determine the PUCCH to transmit based on the last DCI format 3_0 instead of comparing the priorities.

Table 1. SL HARQ-ACK and Uu HARQ-ACK/CSI/RS

38.213 9 UE procedure for reporting control information ===omitted=== When a UE determines overlapping for PUCCH transmissions with SL HARQ-ACK reports and PUSCH of smaller priority index, including repetitions if any, after resolving the overlapping PUCCH other than PUCCH transmissions with SL HARQ-ACK reports and/or PUSCH transmissions, if the PUSCH includes no UCI, the UE resolves the overlapping for PUCCH transmissions with SL HARQ-ACK reports and PUSCH of smaller priority index as described in clauses 9.2.5 and 9.2.6. 9.2.5.0 UE procedure for prioritization between SL HARQ-ACK information in a PUCCH and UCI in a PUCCH The priority value of a PUCCH transmission is as described in clause 16.2.4.3.1. For prioritization between SL HARQ-ACK information in a first PUCCH and UCI in a second PUCCH - if the second PUCCH has priority index 1, - if sl-PriorityThreshold-UL-URLLC is provided - the UE transmits the first PUCCH if a smallest priority value of the first PUCCH is smaller than sl-PriorityThreshold-UL-URLLC; otherwise, the UE transmits the second PUCCH - else - the UE transmits the second PUCCH - else - the UE transmits the first PUCCH if the smallest priority value of the first PUCCH is smaller than sl-PriorityThreshold; otherwise, the UE transmits the second PUCCH

In addition, in 38.321[5], it is specified that for a scheduled PUSCH, if there is no UL-SCH data/A-CSI to transmit and there is no UCI to be multiplexed on the PUSCH, UE does not need to transmit the PUSCH (which is the so-called ‘UL skipping’ behavior). If SL HARQ-ACK is considered as UCI, this rule is also true that UE should skip the PUSCH when there is no SL HARQ-ACK to be multiplexed on the PUSCH. If not, it should be further discussed whether UE should drop the PUSCH in the absence of Uu UCI that can be multiplexed on the PUSCH but has multiplexed SL HARQ-ACK.

Table 2. UL skipping in MAC layer

38.321 5.4.3.1.3 Allocation of resources The MAC entity shall: 1> if the MAC entity is configured with enhancedSkipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or if the MAC entity is configured with enhancedSkipUplinkTxConfigured with value true and the grant indicated to the HARQ entity is a configured uplink grant: 2> if there is no UCI to be multiplexed on this PUSCH transmission as specified in TS 38.213 [6]; and 2> if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and 2> if the MAC PDU includes zero MAC SDUs; and 2> if the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR: 3> not generate a MAC PDU for the HARQ entity.

For simplicity, it is suggested to conclude that SL HARQ-ACK is always one type of UCI and to correct the text cited in Table 1, i.e., by modifying 'UCI' in Table 1 to 'DL HARQ-ACK or SR or CSI'.

Observation 1. In various spec, 38.300, 38.211, 38.212, and clause 7.2.1 of 38.213, SL HARQ-ACK is treated as one type of UCI, but in clauses 9 and clause 9.2.5.0 of 38.213, there are several places where SL HARQ-ACK is not considered as one type of UCI, which leads to ambiguity on SL HARQ-ACK PUCCH handling and the relationship between UCI and SL HARQ-ACK.

Observation 2. If SL HARQ-ACK is not considered one type of UCI, RAN1 needs to further discuss the procedures handling SL HARQ-ACK, including whether UL skipping is allowed when UE has no UL-SCH/A-CSI to transmit and no UCI to be multiplexed but has SL HARQ-ACK to be multiplexed on a scheduled PUSCH, which also require a large spec effort.

Proposal 1. Conclude that UCI includes SL HARQ-ACK and procedures for DL HARQ-ACK can be reused for SL HARQ-ACK, unless explicitly stated for special handling otherwise.

Proposal 2. Adopt the draft CR on TS 38.213 on UCI and SL HARQ-ACK.

- Conclusion

Based on the above discussions, we have the following observations and proposals:

Observation 1. In various spec, 38.300, 38.211, 38.212, and clause 7.2.1 of 38.213, SL HARQ-ACK is treated as one type of UCI, but in clauses 9 and clause 9.2.5.0 of 38.213, there are several places where SL HARQ-ACK is not considered as one type of UCI, which leads to ambiguity on SL HARQ-ACK PUCCH handling and the relationship between UCI and SL HARQ-ACK.

Observation 2. If SL HARQ-ACK is not considered one type of UCI, RAN1 needs to further discuss the procedures handling SL HARQ-ACK, including whether UL skipping is allowed when UE has no UL-SCH/A-CSI to transmit and no UCI to be multiplexed but has SL HARQ-ACK to be multiplexed on a scheduled PUSCH, which also require a large spec effort.

Proposal 1. Conclude that UCI includes SL HARQ-ACK and procedures for DL HARQ-ACK can be reused for SL HARQ-ACK, unless explicitly stated for special handling otherwise.

Proposal 2. Adopt the draft CR on TS 38.213 on UCI and SL HARQ-ACK.

- Draft CR

38.213 9 UE procedure for reporting control information ===omitted=== When a UE determines overlapping for PUCCH transmissions with SL HARQ-ACK reports and PUSCH of smaller priority index, including repetitions if any, after resolving the overlapping PUCCH other than PUCCH transmissions with SL HARQ-ACK reports and/or PUSCH transmissions, if the PUSCH includes no DL HARQ-ACK or SR or CSI, the UE resolves the overlapping for PUCCH transmissions with SL HARQ-ACK reports and PUSCH of smaller priority index as described in clauses 9.2.5 and 9.2.6. 9.2.5.0 UE procedure for prioritization between SL HARQ-ACK information in a PUCCH and DL HARQ-ACK/SR/CSI in a PUCCH The priority value of a PUCCH transmission is as described in clause 16.2.4.3.1. For prioritization between SL HARQ-ACK information in a first PUCCH and DL HARQ-ACK or SR or CSI in a second PUCCH - if the second PUCCH has priority index 1, - if sl-PriorityThreshold-UL-URLLC is provided - the UE transmits the first PUCCH if a smallest priority value of the first PUCCH is smaller than sl-PriorityThreshold-UL-URLLC; otherwise, the UE transmits the second PUCCH - else - the UE transmits the second PUCCH - else - the UE transmits the first PUCCH if the smallest priority value of the first PUCCH is smaller than sl-PriorityThreshold; otherwise, the UE transmits the second PUCCH

- Appendix

38.300 5.3.3 Physical uplink control channel UCI multiplexing in PUSCH is supported when UCI and PUSCH transmissions coincide in time, either due to transmission of a UL-SCH transport block or due to triggering of A-CSI transmission without UL-SCH transport block, and are associated with the same priority (high/low): - UCI carrying HARQ-ACK feedback with 1 or 2 bits is multiplexed by puncturing PUSCH; - In all other cases UCI is multiplexed by rate matching PUSCH. UCI consists of the following information: - CSI; - ACK/NAK; - Scheduling request.

38.211 6.3.1 Physical uplink shared channel 6.3.1.1 Scrambling For the single codeword, the block of bits bq0,…,bqMbit(q)-1, where Mbitq is the number of bits in codeword q transmitted on the physical channel, shall be scrambled prior to modulation, resulting in a block of scrambled bits bq0,…,bqMbit(q)-1 according to the following pseudo code Set i = 0 while i<Mbitq if // UCI placeholder bits else if // UCI placeholder bits else end if end if i = i + 1 end while where x and y are tags defined in [4, TS 38.212] and where the scrambling sequence cq(i) is given by clause 5.2.1. 38.212 6.2.7 Data and control multiplexing Denote the coded bits for UL-SCH as . Denote the coded bits for HARQ-ACK, if any, as .

38.213 7.2.1 UE behaviour ===omitted=== - For a PUCCH transmission using PUCCH format 0 or PUCCH format 1, where - is a number of PUCCH format 0 symbols or PUCCH format 1 symbols for the PUCCH transmission as described in clause 9.2. - for PUCCH format 0 - for PUCCH format 1 - for PUCCH format 0 - for PUCCH format 1, where is a number of UCI bits in PUCCH transmission occasion - For a PUCCH transmission using PUCCH format 2 or PUCCH format 3 or PUCCH format 4 and for a number of UCI bits smaller than or equal to 11, , where - - is a number of HARQ-ACK information bits that the UE determines as described in clause 9.1.2.1 or 16.5.1.1 for Type-1 HARQ-ACK codebook and as described in clause 9.1.3.1 or 9.1.3.3 or 16.5.2.1 for Type-2 HARQ-ACK codebook.is the same as as described in clause 9.1.4 for Type-3 HARQ-ACK codebook. If the UE is not provided any of pdsch-HARQ-ACK-Codebook, pdsch-HARQ-ACK-Codebook-r16, or pdsch-HARQ-ACK-OneShotFeedback, if the UE includes a HARQ-ACK information bit in the PUCCH transmission; otherwise, ===omitted=== - For a PUCCH transmission using PUCCH format 2 or PUCCH format 3 or PUCCH format 4 and for a number of UCI bits larger than 11, , where - - - is a number of HARQ-ACK information bits that the UE determines as described in clause 9.1.2.1 or 16.5.1.1 for Type-1 HARQ-ACK codebook and as described in clause 9.1.3.1 or 9.1.3.3 or 16.5.2.1 for Type-2 HARQ-ACK codebook, or as described in clause 9.1.4 for Type-3 HARQ-ACK codebook. If the UE is not provided any of pdsch-HARQ-ACK-Codebook, pdsch-HARQ-ACK-Codebook-r16, or pdsch-HARQ-ACK-OneShotFeedback, if the UE includes a HARQ-ACK information bit in the PUCCH transmission; otherwise, ===omitted===

- Reference

- 38.300 g70

- 38.211 g70

- 38.212 g70

- 38.213 g70

- 38.321 g60