3GPP TSG RAN WG1 #107-e		R1-2110989

e-Meeting, November 11th – 19th, 2021

Source:	vivo

Title:	Maintenance on Rel-16 NR positioning

Agenda Item:	7.2.8

Document for:	Discussion & Decision

Introduction

In this contribution, we present our views on remaining issues for Rel-16 NR positioning.

PRS resource time gap

In TS38.214 [1], a description related to PRS resource time gap is below.

5.1.6.5 PRS reception procedure … A DL PRS resource set is configured by NR-DL-PRS-ResourceSet, consists of one or more DL PRS resources and it is defined by: … - dl-PRS-ResourceTimeGap defines the offset in number of slots between two repeated instances of a DL PRS resource with the same nr-DL-PRS-ResourceSetId within a single instance of the DL PRS resource set. The UE only expects to be configured with dl-PRS-ResourceTimeGap if dl-PRS-ResourceRepetitionFactor is configured with value greater than 1. The time duration spanned by one instance of a nr-DL-PRS-ResourceSet is not expected to exceed the configured value of DL PRS periodicity. All the DL PRS resources within one resource set have the same value of dl-PRS-ResourceTimeGap. …

However, the parameter marked yellow is not correct. Since based on previous agreement, dl-PRS-ResourceTimeGap indicates offset in units of slots between two repeated instances of a DL PRS resource with the same PRS resource ID.

Agreement (RAN1#98bis): - Parameter DL-PRS-ResourceRepetitionFactor is configured for a DL PRS Resource Set and controls how many times each DL-PRS Resource is repeated for a single instance of the DL-PRS Resource Set - Values: 1, 2, 4, 6, 8, 16, 32 - Parameter DL-PRS-ResourceTimeGap is configured for a DL-PRS Resource Set - DL-PRS-ResourceTimeGap indicates offset in units of slots between two repeated instances of a DL PRS Resource corresponding to the same DL-PRS Resource ID within a single instance of the DL PRS Resource Set - DL-PRS-ResourceTimeGap is provided only if DL-PRS-ResourceRepetitionFactor is configured and is greater than 1 - Values: 1, 2, 4, 8, 16, 32 - The time duration spanned by one DL PRS Resource set containing repeated DL PRS Resources should not exceed DL-PRS-Periodicity - Note: UE RX beam sweeping is up to UE implementation

Therefore, we should change the parameter ‘nr-DL-PRS-ResourceSetId’ to ‘nr-DL-PRS-ResourceID’

< Unchanged parts are omitted > - dl-PRS-ResourceTimeGap defines the offset in number of slots between two repeated instances of a DL PRS resource with the same nr-DL-PRS-ResourceIDSetId within a single instance of the DL PRS resource set. The UE only expects to be configured with dl-PRS-ResourceTimeGap if dl-PRS-ResourceRepetitionFactor is configured with value greater than 1. The time duration spanned by one instance of a nr-DL-PRS-ResourceSet is not expected to exceed the configured value of DL PRS periodicity. All the DL PRS resources within one resource set have the same value of dl-PRS-ResourceTimeGap. < Unchanged parts are omitted >

Thus, we propose that:

Proposal 1:

- Adopt the following modification into TS38.214 regarding PRS resource time gap.

< Unchanged parts are omitted > - dl-PRS-ResourceTimeGap defines the offset in number of slots between two repeated instances of a DL PRS resource with the same nr-DL-PRS-ResourceIDSetId within a single instance of the DL PRS resource set. The UE only expects to be configured with dl-PRS-ResourceTimeGap if dl-PRS-ResourceRepetitionFactor is configured with value greater than 1. The time duration spanned by one instance of a nr-DL-PRS-ResourceSet is not expected to exceed the configured value of DL PRS periodicity. All the DL PRS resources within one resource set have the same value of dl-PRS-ResourceTimeGap. < Unchanged parts are omitted >

Conclusion

In this contribution, we present our views on remaining issues for Rel-16 NR positioning with the following proposal:

Proposal 1:

- Adopt the following modification into TS38.214 regarding PRS resource time gap.

< Unchanged parts are omitted > - dl-PRS-ResourceTimeGap defines the offset in number of slots between two repeated instances of a DL PRS resource with the same nr-DL-PRS-ResourceIDSetId within a single instance of the DL PRS resource set. The UE only expects to be configured with dl-PRS-ResourceTimeGap if dl-PRS-ResourceRepetitionFactor is configured with value greater than 1. The time duration spanned by one instance of a nr-DL-PRS-ResourceSet is not expected to exceed the configured value of DL PRS periodicity. All the DL PRS resources within one resource set have the same value of dl-PRS-ResourceTimeGap. < Unchanged parts are omitted >

- References

- 3GPP TS38.214, V16.7.0, 2021.09