3GPP TSG RAN WG1 #107-e                             R1- 2111008

e-Meeting, November 11th – 19th, 2021

Source: 	vivo

Title:  	Remaining issues on propagation delay compensation enhancements

Agenda Item:	8.3.4

Document for:	Discussion and Decision

Introduction

In RAN1#106b e-meeting, remaining issues on TA-based and RTT-based PDC are discussed and the following agreements are achieved[1].

Agreement

If enhanced TA-based PDC with reduced Te based on TRS is supported in Rel-17, one CSI-RS for tracking (TRS) configuration is configured for enhanced TA-based PDC.

- FFS whether/how to configure UL signal for enhanced TA-based PDC

If RTT-based propagation delay compensation is supported, the Rx-Tx time difference is reported with granularity 2k*Tc, where k is an integer satisfying 0<=k<=5.

- FFS the value of k

- FFS the reporting range of Rx-Tx time difference measurement for PDC

In this contribution, we share our views on remaining issues of propagation delay compensation enhancements.

TA-based PDC

For TA-based method, if enhanced TA-based PDC with reduced Te is supported, UE can improve DL Rx timing accuracy based on TRS, e.g., with the larger bandwidth and/or the higher density. For improving uplink receiving accuracy at gNB, it is unnecessary to restrict a specific UL signal/channel for enhanced TA-based PDC. Actually, any UL channel(s)/signalling(s) can be used for the enhanced TA-based PDC with reduced Te, e.g., SRS/PUSCH. After transmitting TRS, gNB can schedule aperiodic SRS or A-CSI or PUSCH for UE to transmit for the enhanced error detection. The detailed requirement needs RAN4 input e.g., the bandwidth and density of UL channel or signalling.

Proposal 1: If enhanced TA-based PDC with reduced Te based on TRS is supported in Rel-17, UL signal for enhanced TA-based PDC is up to implementation.

Rx-Tx time difference reporting granularity for RTT-based PDC

In positioning WI, the Rx-Tx time difference supporting multiple reporting granularity, i.e., 2k*Tc, where k is an integer satisfying 0<=k<=5. For PDC for URLLC, a remaining issue is whether only one reporting granularity is sufficient. Considering PDC for URLLC support two use cases, i.e., smart grid and control to control use case, various SCS and/or RS bandwidths can be configured for these use cases. On the other hand, the operation frequency range may also be different, e.g., on FR 1 or FR 2. Supporting multiple reporting granularity can provide the better flexibility for the different scenarios and configurations. Therefore, supporting all those values is preferred. The reporting range of Rx-Tx time difference measurement for PDC reuse defined in positioning WI.

Proposal 2: If RTT-based propagation delay compensation is supported, Rx-Tx time difference is reported with granularity 2k*Tc, all k values are supported, e.g., k is an integer satisfying 0<=k<=5.

Conclusions

In this contribution, the remining issues on PDC are discussed. Based on the analysis given above, we have the following Proposals:

Proposal 1: If enhanced TA-based PDC with reduced Te based on TRS is supported in Rel-17, UL signal for enhanced TA-based PDC is up to implementation.

Proposal 2: If RTT-based propagation delay compensation is supported, Rx-Tx time difference is reported with granularity 2k*Tc, all k values are supported, e.g., k is an integer satisfying 0<=k<=5.

Reference

[1] Chairman notes RAN1#106b e-meeting