3GPP TSG RAN WG1 #107-e                                                    R1- 2111052

e-Meeting, November 11th – 19th, 2021

Source:	vivo

Title:	Discussion on UE features for IIoT and URLLC

Agenda Item:	8.16.3

Document for:	Discussion and Decision

Introductions

In RAN1#106b e-meeting, UE features for IIoT and URLLC are discussed and some agreements are achieved[1]. In this contribution, we share our view on remaining issues of UE features for IIoT and URLLC.

URLLC/IIOT UE features

FG 25-1

- Prerequisite FG

In Rel-15, downlinkSPS is configured per UE corresponding to FG 5-18 since only one SPS configuration is supported per cell group. In Rel-16, sps-r16 is defined per band i.e., FG12-2 indicates whether the UE support of up to 8 configured SPS configurations in a BWP of a serving cell and up to 32 configured SPS configurations in a cell group. So, we think prerequisite feature groups should include both FG 5-18 or FG12-2.

- whether/how to separate the capability for handling of the collision for the same HARQ process due to deferred SPS HARQ-ACK

According to the agreement on SPS HARQ-ACK deferral, when UE receives a PDSCH of a certain HARQ Process ID, the deferred SPS HARQ bit(s) for this HARQ Process ID would be dropped. Some companies have the concern that the check on the potential collision and doing the dropping would bring additional complexity. The separate UE capability is required. From UE complexity perspective, we are fine to separate the capability for handling of the collision for the same HARQ process ID due to deferred SPS HARQ-ACK. Additional UE FG, i.e., FG 25-1a, can be introduced for UE with capability to handle the collision for the same HARQ process ID due to deferred SPS HARQ-ACK. For UE only supporting 25-1 but not 25-1a, it is not expected to handle the collision for the same HARQ process due to deferred SPS HARQ-ACK, which means gNB should avoid such collision.

- Type

The prerequisite feature groups of FG 25-1 is FG 5-18 and FG 12-2. Considering type of FG 12-2 is per Band, type of FG 25-1 should be at least per band.

Proposal 1: For FG25-1, prerequisite feature groups should include FG 5-18 or FG12-2. FG 25-1a can be introduced for UE with capability to handle the collision for the same HARQ process due to deferred SPS HARQ-ACK.

| 25.NR_IIOT_URLLC_enh | 25-1 | SPS HARQ-ACK deferral in case of TDD collision | 1. Idenfify HARQ-ACK bits of active SPS configurations for deferral in the initial PUCCH slot 2. Determination of the target PUCCH slot for SPS HARQ-ACK deferral 3. Multiplexing and transmission of deferred SPS HARQ-ACK information in the target PUCCH slot FFS whether to separate capability for handling of the collision for the same HARQ process due to deferred SPS HARQ-ACK | 5-185-18or12-2 | Yes | N/A |  | Per UEBand | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25.NR_IIOT_URLLC_enh | 25-1a | Handling of the collision for the same HARQ process due to deferred SPS HARQ-ACK | Handle the collision for the same HARQ process due to deferred SPS HARQ-ACK | 25-1 | Yes | N/A |  | Per UEBand | No | No | N/A | For UE only supporting 25-1 but not 25-1a, it is not expected to handle the collision for the same HARQ process due to deferred SPS HARQ-ACK, which means gNB should avoid such collision. | Optional with capability signaling |

FG 25-3 & 25-3a

According to the latest agreements, sub-slot based PUCCH repetitions of all UCI types (incl. HARQ, SR & CSI) have been supported. Thus, FFS can be deleted.

Since prerequisite feature groups of FG 25-3 include both FG 11-3 and FG 4-23 and the type of FG 11-3 is Per FeatureSetUplink, we think the type of FG 25-3 should be per FS.

Similarly, since prerequisite feature groups of FG 25-3a include both FG 25-3 and FG 30-5 and the type of FG 25-3 should be Per FS as discussed above, the type of FG 25-3a should also be per FS.

Proposal 2: For FG 25-3, the type should be Per FS. FFS part can be deleted.

| 25.NR_IIOT_URLLC_enh | 25-3 | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots with configured K = 2, 4, 8 | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots with RRC configured repetition factor K = 2, 4, 8 FFS whether to separate the capability per UCI type | 4-23 11-3 | Yes | N/A |  | Per UEFS | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| 25.NR_IIOT_URLLC_enh | 25-3a | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots using dynamic repetition indication | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots based on dynamic repetition indication. FFS whether to separate the capability per UCI type | 25-3 30-5 | Yes | N/A |  | Per UEFS | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

FG 25-4 & FG 25-5 & FG25-6

- whether/how to separate FGs 25-4 to 25-6

From UE complexity perspective, it is unnecessity to further split UE capability into FG 25-4a, FG25-5a and 25-6a for reduced size Type 3 codebook. The reason for introducing FG25-4 and 25-5 for Type 3 codebook with full size is mainly because Type3 codebook with full size and DCI format 1_2 and PHY priority handling are specified in different WIs, due to time limitation, the two FGs are not supported in Rel-16.

According to the latest agreements, the maximum number of simultaneously configurable enhanced Type 3 CB supported by a UE is 8, which should be updated in FG 25-6.

FG 10-16 is the prerequisite feature groups of FG 25-4/5/6, which type is Per band. Thus, the type of FGs 25-4 to 25-6 should be Per band.

Proposal 3: It is unnecessary to further split UE capability FGs 25-4 to 25-6. The maximum number of configurable enhanced Type 3 CB is updated as follow.

| 25.NR_IIOT_URLLC_enh | 25-4 | One-shot HARQ ACK feedbacktriggered by DCI format 1_2 | 1. Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_2 scheduling a PDSCH 2. Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_2 without scheduling a PDSCH using a reserved FDRA value | 10-16 11-1 | Yes | N/A |  | Per UEBand | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25. NR_IIOT_URLLC_enh | 25-5 | PHY priority handling for one-shot HARQ ACK feedback | Support transmission of type 3 HARQ-ACK codebook using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI | 10-16 11-4 | Yes | N/A |  | Per UEBand | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-6 | Enhanced type 3 HARQ-ACK codebook feedback | 1. Support feedback of enhanced type 3 HARQ-ACK codebook, triggered by a DCI 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2, and 1-1) 2. Support configuration of up to X 8 enhanced type 3 HARQ-ACK codebooks. 3. Support feedback of a dynamically selected enhanced type 3 HARQ-ACK codebook based on triggering information in DCI 1_1 and DCI 1_2 (for a UE supporting DCI format 1_2, 11-1) 4. Support transmission of enhanced type 3 HARQ-ACK codebook using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4) | 10-16 | Yes | N/A |  | Per UEBand | No | No | N/A | For component 2, the UE indicates its capability in the number of enhanced type 3 HARQ-ACK codebooks: {1,...,X} For component 3, the dynamic indication is only supported if the UE for component 2 supports more than one enhanced type 3 HARQ-ACK codebook to be configured | Optional with capability signaling |

FG 25-7

For first bullet in components column of FG 25-7, we think ‘sub-slot’ should be added.

Proposal 4: For FG 25-7 ‘sub-slot’ should be added in components column.

| 25. NR_IIOT_URLLC_enh | 25-7 | Triggered HARQ-ACK codebook re-transmission | 1. Support HARQ-ACK re-transmission from an earlier PUCCH slot or sub-slot based on the triggering information in DCI format 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2, 11-1) 2. Support the related PHY priority handling in terms of HARQ-ACK codebook selection and the applicable PUCCH configuration (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4) |  | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

FG 25-9 & FG 25-10

- whether to separate the capability for different numerologies

In our opinion, PUCCH cell switch cross cells with same numerology should be the baseline and PUCCH cell switch across cells with different numerology should be deemed as a more advanced UE feature. Supporting separate UE capability provide flexible UE implementation. We suggest split FG 25-9 and FG 5-10 into separate UE capabilities.

For FG 25-9, semi-static PUCCH carrier switching is applicable to all UCI types including HARQ-ACK, SR and CSI. This should be captured in the corresponding components column.

For FG 25-10, according to the agreement, in addition to HARQ-ACK of PDSCH dynamically scheduled by a DCI indicating a PUCCH carrier, the dynamic target carrier indication also applies to:

- HARQ-ACK corresponding to the first SPS PDSCH activated by Activation DCI based on the indication in the activation DCI

- HARQ-ACK corresponding to the SPS Release DCI based on the indication in the release DCI

- triggered PUCCH for Rel-16 Type 3 CB, Rel-17 enh. Type 3 CB of smaller size and Rel-17 one-shot triggering for HARQ-Ack retransmission based on the indication in the triggering DCI

For FG 25-10, these applicable cases should be explicitly described in the components.

Proposal 5: For FG 25-9 and 25-10, support separate UE capability for different numerologies. The applicable cases should be explicitly described in the components.

| 25.NR_IIOT_URLLC_enh | 25-9 | Semi-static PUCCH cell (FFS or PUCCH carrier) switching | 1.Semi-static PUCCH cell (FFS or PUCCH carrier)switching with the same numerology using configured time-domain pattern of applicable PUCCH cell / carrier FFS whether to separate the capability for different numerologies 2.semi-static PUCCH carrier switching is applicable to all UCI types including HARQ-ACK, SR and CSI |  | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25.NR_IIOT_URLLC_enh | 25-9a | Semi-static PUCCH cell (FFS or PUCCH carrier) switching | 1.Semi-static PUCCH cell (FFS or PUCCH carrier)switching with the same or different numerologies using configured time-domain pattern of applicable PUCCH cell / carrier FFS whether to separate the capability for different numerologies 2.semi-static PUCCH carrier switching is applicable to all UCI types including HARQ-ACK, SR and CSI |  | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| 25.NR_IIOT_URLLC_enh | 25-10 | PUCCH cell (FFS or PUCCH carrier)switching based on dynamic indication | PUCCH cell (FFS or PUCCH carrier)switching with the same numerology based on dynamic indication in the DCI scheduling the PUCCH, which is applicable to - HARQ-ACK of PDSCH dynamically scheduled by a DCI indicating a PUCCH carrier - HARQ-ACK corresponding to the first SPS PDSCH activated by Activation DCI based on the indication in the activation DCI - HARQ-ACK corresponding to the SPS Release DCI based on the indication in the release DCI - triggered PUCCH for Rel-16 Type 3 CB, Rel-17 enh. Type 3 CB of smaller size and Rel-17 one-shot triggering for HARQ-Ack retransmission based on the indication in the triggering DCI FFS whether to separate the capability for different numerologies |  | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| 25.NR_IIOT_URLLC_enh | 25-10a | PUCCH cell (FFS or PUCCH carrier)switching based on dynamic indication | PUCCH cell (FFS or PUCCH carrier)switching with the same or different numerologies based on dynamic indication in the DCI scheduling the PUCCH, which is applicable to - HARQ-ACK of PDSCH dynamically scheduled by a DCI indicating a PUCCH carrier - HARQ-ACK corresponding to the first SPS PDSCH activated by Activation DCI based on the indication in the activation DCI - HARQ-ACK corresponding to the SPS Release DCI based on the indication in the release DCI - triggered PUCCH for Rel-16 Type 3 CB, Rel-17 enh. Type 3 CB of smaller size and Rel-17 one-shot triggering for HARQ-Ack retransmission based on the indication in the triggering DCI FFS whether to separate the capability for different numerologies |  | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |

FG 25-14

According to the agreements in the latest RAN1 meeting, it has been agreed to support the collision handling between HP CG PUSCH and LP DG PUSCH with condition on two MAC PDUs delivered from MAC to PHY.

Thus, the square brackets of FG 25-14 can be deleted.

Proposal 6: The square brackets and color of FG 25-14 can be removed.

| 25. NR_IIOT_URLLC_enh | [25-14] | PHY prioritization of overlapping low-priority DG-PUSCH and high-priority CG-PUSCH | Support PHY prioritization for the case where low-priority DG-PUSCH collides with high-priority CG-PUSCH | 12-1 | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

FG 25-16 & FG 25-17

For FG 25-16 and FG 25-17, we suggest to merge these two FGs. It seems no additional complexity for UE supporting HARQ-ACK multiplexing on a PUCCH and multiplexing on a PUSCH. On the other hand, it is unreasonable for UE to support transmitting HARQ-ACK with both High Priority and Low Priority only on PUCCH or PUSCH.

For UCI type for multiplexing with different priorities, the discussion is ongoing. It should be discussed after the further conclusion achieved.

For FG 25-16, the prerequisite feature groups are FG 11-3 which is per FeatureSetUplink. Thus, the type of FG 25-16 should be Per FS.

Proposal 7: Merge FG 25-17 into FG 25-16.

| 25.NR_IIOT_URLLC_enh | 25-16 | HARQ-ACK with different priorities multiplexing using a PUCCH | 1. Support multiplexing a high-priority HARQ-ACK and a low-priority HARQ-ACK into a PUCCH. Support separate coding for the two HARQ-ACKs. 2. [Support multiplexing a low-priority HARQ-ACK and a high-priority SR into a PUCCH for some HARQ-ACK/SR PF combinations (FFS applicable combinations).] 3. [Support multiplexing a low-priority HARQ-ACK, a high-priority HARQ-ACK and a high-priority SR into a PUCCH.] 4. Support multiplexing a low-priority HARQ-ACK in a high-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination. 5. Support multiplexing a high-priority HARQ-ACK in a low-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination. 6. Support multiplexing a low-priority HARQ-ACK, a high-priority PUSCH conveying UL-SCH, a high-priority HARQ-ACK and/or CSI. 7. Support multiplexing a high-priority HARQ-ACK, a low-priority PUSCH conveying UL-SCH, a low-priority HARQ-ACK and/or CSI. FFS whether to merge with FG 25-17 FFS whether to separate capability for different UCI type | 11-3 | Yes | N/A |  | Per UEFS | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25.NR_IIOT_URLLC_enh | 25-17 | HARQ-ACK piggybacked on a PUSCH of a different priority | 1. Support multiplexing a low-priority HARQ-ACK in a high-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination. 2. Support multiplexing a high-priority HARQ-ACK in a low-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination. 3. Support multiplexing a low-priority HARQ-ACK, a high-priority PUSCH conveying UL-SCH, a high-priority HARQ-ACK and/or CSI. 4. Support multiplexing a high-priority HARQ-ACK, a low-priority PUSCH conveying UL-SCH, a low-priority HARQ-ACK and/or CSI. FFS whether to merge into FG 25-16 FFS whether to separate capability for different UCI type | 11-3 12-1 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |

Conclusion

In the contribution, we have some investigations on UE feature for URLLC/IIOT, and propose that,

Proposal 1: For FG25-1, prerequisite feature groups should include FG 5-18 or FG12-2. FG 25-1a can be introduced for UE with capability to handle the collision for the same HARQ process due to deferred SPS HARQ-ACK..

| 25.NR_IIOT_URLLC_enh | 25-1 | SPS HARQ-ACK deferral in case of TDD collision | 1. Idenfify HARQ-ACK bits of active SPS configurations for deferral in the initial PUCCH slot 2. Determination of the target PUCCH slot for SPS HARQ-ACK deferral 3. Multiplexing and transmission of deferred SPS HARQ-ACK information in the target PUCCH slot FFS whether to separate capability for handling of the collision for the same HARQ process due to deferred SPS HARQ-ACK | 5-185-18or12-2 | Yes | N/A |  | Per UEBand | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25.NR_IIOT_URLLC_enh | 25-1a | Handling of the collision for the same HARQ process due to deferred SPS HARQ-ACK | Handle the collision for the same HARQ process due to deferred SPS HARQ-ACK | 25-1 | Yes | N/A |  | Per UEBand | No | No | N/A | For UE only supporting 25-1 but not 25-1a, it is not expected to handle the collision for the same HARQ process due to deferred SPS HARQ-ACK, which means gNB should avoid such collision. | Optional with capability signaling |

Proposal 2: For FG 25-3, the type should be Per FS. FFS part can be deleted.

| 25.NR_IIOT_URLLC_enh | 25-3 | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots with configured K = 2, 4, 8 | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots with RRC configured repetition factor K = 2, 4, 8 FFS whether to separate the capability per UCI type | 4-23 11-3 | Yes | N/A |  | Per UEFS | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| 25.NR_IIOT_URLLC_enh | 25-3a | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots using dynamic repetition indication | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots based on dynamic repetition indication. FFS whether to separate the capability per UCI type | 25-3 30-5 | Yes | N/A |  | Per UEFS | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 3: It is unnecessary to further split UE capability FGs 25-4 to 25-6. The maximum number of configurable enhanced Type 3 CB is updated as follow.

| 25.NR_IIOT_URLLC_enh | 25-4 | One-shot HARQ ACK feedbacktriggered by DCI format 1_2 | 1. Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_2 scheduling a PDSCH 2. Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_2 without scheduling a PDSCH using a reserved FDRA value | 10-16 11-1 | Yes | N/A |  | Per UEBand | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25. NR_IIOT_URLLC_enh | 25-5 | PHY priority handling for one-shot HARQ ACK feedback | Support transmission of type 3 HARQ-ACK codebook using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI | 10-16 11-4 | Yes | N/A |  | Per UEBand | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-6 | Enhanced type 3 HARQ-ACK codebook feedback | 1. Support feedback of enhanced type 3 HARQ-ACK codebook, triggered by a DCI 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2, and 1-1) 2. Support configuration of up to X 8 enhanced type 3 HARQ-ACK codebooks. 3. Support feedback of a dynamically selected enhanced type 3 HARQ-ACK codebook based on triggering information in DCI 1_1 and DCI 1_2 (for a UE supporting DCI format 1_2, 11-1) 4. Support transmission of enhanced type 3 HARQ-ACK codebook using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4) | 10-16 | Yes | N/A |  | Per UEBand | No | No | N/A | For component 2, the UE indicates its capability in the number of enhanced type 3 HARQ-ACK codebooks: {1,...,X} For component 3, the dynamic indication is only supported if the UE for component 2 supports more than one enhanced type 3 HARQ-ACK codebook to be configured | Optional with capability signaling |

Proposal 4: For FG 25-7 ‘sub-slot’ should be added in components column.

| 25. NR_IIOT_URLLC_enh | 25-7 | Triggered HARQ-ACK codebook re-transmission | 1. Support HARQ-ACK re-transmission from an earlier PUCCH slot or sub-slot based on the triggering information in DCI format 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2, 11-1) 2. Support the related PHY priority handling in terms of HARQ-ACK codebook selection and the applicable PUCCH configuration (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4) |  | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 5: For FG 25-9 and 25-10, support separate UE capability for different numerologies. The applicable cases should be explicitly described in the components.

| 25.NR_IIOT_URLLC_enh | 25-9 | Semi-static PUCCH cell (FFS or PUCCH carrier) switching | 1.Semi-static PUCCH cell (FFS or PUCCH carrier)switching with the same numerology using configured time-domain pattern of applicable PUCCH cell / carrier FFS whether to separate the capability for different numerologies 2.semi-static PUCCH carrier switching is applicable to all UCI types including HARQ-ACK, SR and CSI |  | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25.NR_IIOT_URLLC_enh | 25-9a | Semi-static PUCCH cell (FFS or PUCCH carrier) switching | 1.Semi-static PUCCH cell (FFS or PUCCH carrier)switching with the same or different numerologies using configured time-domain pattern of applicable PUCCH cell / carrier FFS whether to separate the capability for different numerologies 2.semi-static PUCCH carrier switching is applicable to all UCI types including HARQ-ACK, SR and CSI |  | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| 25.NR_IIOT_URLLC_enh | 25-10 | PUCCH cell (FFS or PUCCH carrier)switching based on dynamic indication | PUCCH cell (FFS or PUCCH carrier)switching with the same numerology based on dynamic indication in the DCI scheduling the PUCCH, which is applicable to - HARQ-ACK of PDSCH dynamically scheduled by a DCI indicating a PUCCH carrier - HARQ-ACK corresponding to the first SPS PDSCH activated by Activation DCI based on the indication in the activation DCI - HARQ-ACK corresponding to the SPS Release DCI based on the indication in the release DCI - triggered PUCCH for Rel-16 Type 3 CB, Rel-17 enh. Type 3 CB of smaller size and Rel-17 one-shot triggering for HARQ-Ack retransmission based on the indication in the triggering DCI FFS whether to separate the capability for different numerologies |  | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| 25.NR_IIOT_URLLC_enh | 25-10a | PUCCH cell (FFS or PUCCH carrier)switching based on dynamic indication | PUCCH cell (FFS or PUCCH carrier)switching with the same or different numerologies based on dynamic indication in the DCI scheduling the PUCCH, which is applicable to - HARQ-ACK of PDSCH dynamically scheduled by a DCI indicating a PUCCH carrier - HARQ-ACK corresponding to the first SPS PDSCH activated by Activation DCI based on the indication in the activation DCI - HARQ-ACK corresponding to the SPS Release DCI based on the indication in the release DCI - triggered PUCCH for Rel-16 Type 3 CB, Rel-17 enh. Type 3 CB of smaller size and Rel-17 one-shot triggering for HARQ-Ack retransmission based on the indication in the triggering DCI FFS whether to separate the capability for different numerologies |  | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |

Proposal 6: The square brackets and color of FG 25-14 can be removed.

| 25. NR_IIOT_URLLC_enh | [25-14] | PHY prioritization of overlapping low-priority DG-PUSCH and high-priority CG-PUSCH | Support PHY prioritization for the case where low-priority DG-PUSCH collides with high-priority CG-PUSCH | 12-1 | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 7: Merge FG 25-17 into FG 25-16.

| 25.NR_IIOT_URLLC_enh | 25-16 | HARQ-ACK with different priorities multiplexing using a PUCCH | 1. Support multiplexing a high-priority HARQ-ACK and a low-priority HARQ-ACK into a PUCCH. Support separate coding for the two HARQ-ACKs. 2. [Support multiplexing a low-priority HARQ-ACK and a high-priority SR into a PUCCH for some HARQ-ACK/SR PF combinations (FFS applicable combinations).] 3. [Support multiplexing a low-priority HARQ-ACK, a high-priority HARQ-ACK and a high-priority SR into a PUCCH.] 4. Support multiplexing a low-priority HARQ-ACK in a high-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination. 5. Support multiplexing a high-priority HARQ-ACK in a low-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination. 6. Support multiplexing a low-priority HARQ-ACK, a high-priority PUSCH conveying UL-SCH, a high-priority HARQ-ACK and/or CSI. 7. Support multiplexing a high-priority HARQ-ACK, a low-priority PUSCH conveying UL-SCH, a low-priority HARQ-ACK and/or CSI. FFS whether to merge with FG 25-17 FFS whether to separate capability for different UCI type | 11-3 | Yes | N/A |  | Per UEFS | No | No | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25.NR_IIOT_URLLC_enh | 25-17 | HARQ-ACK piggybacked on a PUSCH of a different priority | 1. Support multiplexing a low-priority HARQ-ACK in a high-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination. 2. Support multiplexing a high-priority HARQ-ACK in a low-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination. 3. Support multiplexing a low-priority HARQ-ACK, a high-priority PUSCH conveying UL-SCH, a high-priority HARQ-ACK and/or CSI. 4. Support multiplexing a high-priority HARQ-ACK, a low-priority PUSCH conveying UL-SCH, a low-priority HARQ-ACK and/or CSI. FFS whether to merge into FG 25-16 FFS whether to separate capability for different UCI type | 11-3 12-1 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |

References

R1-2110587 Updated RAN1 UE features list for Rel-17 NR after RAN1 #106bis-e, Moderators (AT&T, NTT DOCOMO, INC.) RAN WG1 #106b-e