3GPP TSG RAN WG1 #107-e	R1-2111080

e-Meeting, November 11th – 19th, 2021

Source: 	ZTE, Sanechips

Title: 	Discussion on RRC parameters for 52.6 to 71 GHz

Agenda item:	8.2.7

Document for:	Discussion and Decision

Introduction

In this contribution, we will share our views on RRC parameters for 52.6 to 71 GHz. Wherein, the latest approved parameters list can be found in [1].

Discussion

Per cell for the indication of LBT mode

In RAN1 #106bis e-meeting, the RRC parameters for Rel-17 above 52.6GHz were further discussed and updated. Wherein, for channel access part, LBT mode has been captured in the latest RRC excel sheet [1], but specific LBT mode is per beam or per cell has not been determined yet. According to the agreement of last meeting, there is no consensus to support per beam LBT mode or No LBT mode UE specific gNB indication. Based on this, we share our views in section 2.5 of our company’s contribution [2] and propose to configure LBT mode or No LBT mode as per cell. Besides, specific RRC parameters for UE specific and/or cell specific to configure LBT mode can be decided by RAN2.

| Sub-feature group | RAN1 specification | Section | RAN2 Parant IE | RAN2 ASN.1 name | Parameter name in the spec | New or existing? | Parameter name in the text | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | Specification | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel access |  |  | FFS. For cell-specific configuration |  | LBT-Mode | New |  | To switch between LBT and no-LBT mode | FFS |  | Per cell | Cell-specific |  | Agreement: For regions where LBT is not mandated, gNB should indicate to the UE this gNB-UE connection is operating in LBT mode or no-LBT mode • Support both cell specific (common for all UEs in a cell as part of system information or dedicated RRC signalling or both) and UE specific (can be different for different UEs in a cell as part of UE-specific RRC configuration) gNB indication Conclusion (RAN1 #106bis e-meeting): There is no consensus to support per beam LBT mode or no-LBT mode UE specific gNB indication. |
| Channel access |  |  | FFS. For UE-specific configuration |  | LBT-Mode | New |  | To switch between LBT and no-LBT mode | FFS |  | Per cell | UE-specific |  | Agreement: For regions where LBT is not mandated, gNB should indicate to the UE this gNB-UE connection is operating in LBT mode or no-LBT mode • Support both cell specific (common for all UEs in a cell as part of system information or dedicated RRC signalling or both) and UE specific (can be different for different UEs in a cell as part of UE-specific RRC configuration) gNB indication Conclusion (RAN1 #106bis e-meeting): There is no consensus to support per beam LBT mode or no-LBT mode UE specific gNB indication. |

Proposal 1: If per cell is agreed for the indication of LBT mode and No LBT mode, it is suggested to capture this  feature in RRC parameters list on cell specific and UE specific configuration.

Conclusion

In this contribution, we share some our views on RRC parameters for 52.6 GHz to 71GHz and provide the following proposals:

Proposal 1: If per cell is agreed for the indication of LBT mode and No LBT mode, it is suggested to capture this  feature in RRC parameters list on cell specific and UE specific configuration.

Reference

R1-2110573, Consolidated higher layers parameter list for Rel-17 NR

R1-2111079, Discussion on the channel access for 52.6 to 71GHz, ZTE, Sanechips, RAN1#107-e, 11th - 19th November, 2021