3GPP TSG-RAN WG1 Meeting #107-e	R1-2111128

# e-Meeting, November 11th - 19th, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR |  | rev |  | Current version: | 15.14.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on collision between UL transmission scheduled by RAR UL grant and DL reception configured by high layer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2021-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In 38.213, it is stated that when PDSCH/PDCCH/RS configured by high layer collides with PUCCH/PUSCH/PRACH/SRS scheduled by a DCI format, the PDSCH/PDCCH/RS reception is canceled, otherwise, UE should receive the PDSCH/PDCCH/RS. However, it is not specified whether UE should drop the DL reception when the PDSCH/PDCCH/RS configured by high layer collides with PUSCH scheduled by a RAR UL grant (e.g., Msg3 initial transmission, PUSCH scheduled by a RAR UL grant for CFRA case). Since Msg3 retransmission is already covered by PUSCH scheduled by a DCI format 0_0, it is natural to align UE behavior between Msg3 initial transmission and Msg3 retransmission. Besides, for CFRA case, PUSCH scheduled by a RAR UL grant should be treated in the same way as normal PUSCH scheduled by a DCI format. However, the current spec can lead to a misunderstanding that the collision between PDSCH/PDCCH/RS configured by high layer and PUSCH scheduled by a RAR UL grant is covered by the ‘if the UE does not detect a DCI format 0_0, DCI format 0_1, DCI format 1_0, DCI format 1_1, or DCI format 2_3’ statement thus leading to PDSCH/PDCCH/RS reception and cancellation of UL transmission. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that UE should perform UL transmission scheduled by a RAR UL grant when the PUSCH transmission scheduled by a RAR UL grant is overlapped with PDSCH/PDCCH/RS configured by the high layer. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The misinterpretation that UL transmission scheduled by a RAR UL grant should be dropped when it collides with PDSCH/PDCCH/RS configured by high layer remains. Misaligned UE behavior between Msg3 initial transmission and Msg3 retransmission. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: This CR aligns with RAN1 common understanding, with no isolated impact. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

11.1	Slot configuration

For operation on a single carrier in unpaired spectrum, if a UE is configured by higher layers to receive a PDCCH, or a PDSCH, or a CSI-RS in a set of symbols of a slot, the UE receives the PDCCH, the PDSCH, or the CSI-RS if the UE does not detect a RAR UL grant, a DCI format 0_0, DCI format 0_1, DCI format 1_0, DCI format 1_1, or DCI format 2_3 that indicates to the UE to transmit a PUSCH, a PUCCH, a PRACH, or a SRS in at least one symbol of the set of symbols of the slot; otherwise, the UE does not receive the PDCCH, or the PDSCH, or the CSI-RS in the set of symbols of the slot.